IEEE P802.11
Wireless LANs

| TGac Functional Requirements and Evaluation Methodology Rev. 10 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-01-15 |  |  |  |  |
| Authors and Contributors |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Peter Loc | Ralink Technology | 20833 Stevens Creek Blvd, Suite 200., Cupertino CA 95014 | 408 807-0868 | peterloc@gmail.com |
| Minho Cheong | ETRI | 161 Gajeong-dong, Yuseong-gu, Daejeon, Korea | +82 42 860 5635 | minho@etri.re.kr |

ABSTRACT

This document describes the functional requirements and the evaluation methodology for Tgac. These requirements are derived from the document “11-08-0807-04-0vht-below-6-ghz-par-nescom-form-plus-5cs”. The amendment must address the functional requirements in this document.

# Contributors

This will grow to reflect those providing explicit contributions / review comments of this document. (Please let either Peter or Minho know if  we have missed any name)

| Name | Company | Address | Phone | Email |
| --- | --- | --- | --- | --- |
| Eldad Perahia | Intel Corporation |  |  | eldad.perahia@intel.com |
| Robert Stacey | Intel Corporation |  |  | 1robert.j.stacey@intel.com |
| Michelle X. Gong | Intel Corporation |  |  | michelle.x.gong@intel.com |
| Vinko Erceg | Broadcom |  |  | verceg@broadcom.com |
| Matthew Fischer | Broadcom |  |  | mfischer@broadcom.com |
| VK Jones | Qualcomm Inc. |  |  | vkjones@qualcomm.com |
| Santosh Abraham | Qualcomm Inc. |  |  | sabraham@qualcomm.com |
| Allan Zhu | Samsung |  |  | c.zhu@samsung.com |
| Kai Shi | Atheros Comm. Inc. |  |  | kai.shi@atheros.com |
| Yashusi Takadori | NTT |  |  | takatori.yasushi@lab.ntt.co.jp |
| Yusuke Asai | NTT |  |  | Asai.yusuke@lab.ntt.co.jp |
| Sudheer Grandhi | InterDigital Comm. |  |  | sagrandhi802@gmail.com |
| Yuichi Morioka | Sony |  |  | Yuichi.morioka@jp.sony.com |
| Brian Hart | Cisco Systems |  |  | brianh@cisco.com |
| Andrew Myles | Cisco Systems |  |  | Amyles@cisco.com |
| Darwin Engwer | Self |  |  | evoles_2001@yahoo.com |

# Revision History

| Revision | Comments | Date |
| --- | --- | --- |
| 0304r0 | Peter and Minho initial contribution on Functional Requirements | 13 March 2009 |
| R0 | Evaluation Methodology and Simulation Scenario are included | 10 April 2009 |
| R1 | Revised by conference call discussions on April 9 and April 23. | 09 May 2009 |
| R2 | Revised by Montreal Interim discussions on May 11-15. | 28 May 2009 |
| R3 | Revised by conference call discussions on May 28 | 13 July 2009 |
| R4 | Revised by San Francisco Plenary discussions on July 14 | 16 July 2009 |
| R5 | Shadowing term in PHY channel is TBD for further discussions | 16 July 2009 |
| R6 | Minor editorial changes | 16 July 2009 |
| R7 | Some changes in Abstract | 16 July 2009 |
| R8 | Added enterprise simulation scenario with OBSS | 17 November 2009 |
| R9 | Added in-home entertainment scenario with OBSS | 18 November 2009 |
| R10 | In-home entertainment scenario revised and wall peneration loss defined in scenario #4 | 15 January 2010 |

# 1. Overview

This document specifies the functional requirements and the evaluation methodology for TGac as stated in the VHT below 6 GHz PAR Plus 5C’s. The functional requirements as stated in this document cover the following aspects of TGac

- System performance

- Backward compatibility with 802.11a/n devices operating in 5 GHz

- Coexistence with 802.11a/n devices operating in 5 GHz

- Compliance to PAR

# 2. Functional Requirements

# 2.1 System Performance

2.1.1 Multi-STA throughput measured at the MAC SAP to be at least 1 Gbps.

TGac R1 – The TGac amendment shall provide at least a mode of operation capable of achieving a maximum Multi-Station aggregate throughput of more than 1 Gbps as measured at the MAC data service access point (SAP), utilizing no more than 80 MHz of channel bandwidth in 5 GHz band.

2.1.2 Single-STA throughput measured at the MAC SAP to be at least 500 Mbps.

TGac R2 – The TGac amendment shall provide at least a mode of operation capable of achieving a maximum Single-Station throughput of more than 500 Mbps as measured at the MAC data service access point  (SAP), utilizing no more than 80 MHz of channel bandwidth in 5GHz band.

2.2 Backward Compatibility with 802.11a/n devices operating in 5 GHz

Refer to the IEEE Std. 802.15.2-2003, section 3.1 for the definitions of backward compatible.

TGac R3- The TGac admendment shall provide backward compatibility with IEEE802.11a devices operating in the 5 GHz frequency band.

TGac R4- The TGac admendment shall provide backward compatibility with IEEE802.11n devices operating in the 5 GHz frequency band.

# 2.3 Coexistence with 802.11a/n devices operating in 5 GHz

Refer to the IEEE Std. 802.15.2-2003, section 3.1 for the definitions of coexistence.

TGac R5 – The TGac amendment shall provide mechanisms that ensure coexistence between TGac and legacy IEEE802.11a/n devices.

# 2.4 Compliance to PAR

TGac R6 - The proposal complies with the PAR and 5 Criteria [1].

# 3. Evaluation Methodology

The evaluation methodology defines PHY performance, conditions for PAR compliance and a limited set of simulation scenarios and comparison criteria for TGac evaluatation.

As TGac agreed on the approach outlined in the 802.11/09/0376r1, the evaluation methodology for TGac can be build up based on 802.11n one by some modifications.

# 3.1 PHY Performance

3.1.1 PHY channel model

Channel models defined in 802.11n channel model document [8] shall be used. Some modifications to 802.11n channel model are described in [11].

3.1.2 PHY impairments

PHY impairments are updated from ones desribed in 802.11n comparison criteria document [7].

Table 1. PHY impairments

| Number | Name | Definition | Comments |
| --- | --- | --- | --- |
| IM1 | PA non-linearity | Simulation should be run at an oversampling rate of at least 2x. To perform convolution of the 2x oversampled transmit waveform with the channel, the channel may be resampled by rounding each channel tap time value to the nearest integer multiple of a sample interval of the oversampled transmit waveform. Use RAPP power amplifier model as specified in document 00/294 with p = 3. Calculate backoff as the output power backoff from full saturation: PA Backoff = 10 log10(Average TX Power/Psat). Total TX power shall be limited to no more than 17 dBm. Disclose: (a) EIRP and how it was calculated, (b) PA Backoff, and (c) Psat per PA. Note: the intent of this IM is to allow different proposals to choose different output power operating points. Note: the value Psat = 25dBm is recommended. | Added comments for higher sampling rate for channel |
| IM2 | Carrier frequency offset | Single-user simulations for all comparisons except Offset Compensation shall be run using a fixed carrier frequency offset of –13.675 ppm at the receiver, relative to the transmitter. The symbol clock shall have the same relative offset as the carrier frequency offset. Simulations shall include timing acquisition on a per-packet basis. Multi-user simulations for all comparisons except offset compensation shall be run using a fixed carrier frequency offset selected from the array [N(1) ,N(2),……,N(16) ], relative to the transmitter, where N(j) corresponds to the frequency offset of the j-th client and is randomly chosen from [-20,20] ppm with a uniform distribution. | Added a set of possible offsets to be used for several STAs. 802.11n specified a single offset of -13.67 ppm |
| IM3 | Phase noise | The phase noise will be specified with a pole-zero model. PSD(f)=PSD(0)[1+(ffz)2][1+(ffp)2] PSD(0) = -100 dBc/Hz pole frequency fp = 250 kHz zero frequency fz = 7905.7 kHz Note, this model results in PSD(infinity) = -130 dBc/Hz Note, this impairment is modeled at both transmitter and receiver. | Unchanged from 802.11n |
| IM4 | Noise figure | Input referred total noise figure from antenna to output of the A/D will be 10dB. | Unchanged from 802.11n |
| IM5 | Antenna Configuration | The TGn antenna configuration at both ends of the radio link shall be a uniform linear array of isotropic antennas with separation of one-half wavelength, with an antenna coupling coefficient of zero. The TGac antennas can be assumed to either be all vertically polarized or a mix of vertical and horizontal polarizations or dual polarization at ±45 degree, as specified in the TGac channel model addendum document [11] | Mix of vertically and horizontally polarized antennas or dual polarization at ±45 degree is also considered for TGac devices |
| IM6 | Fluoroscent Light Effects | The fluoroscent light effects specifed in the TGn Channel model shall not be considered for the simulation scenarios. |  |

3.1.3 Comparison criteria

- PER vs. SNR curves

- all MCS’s

- Simulate all of channel models

- Simulation must include:

- updated PHY impairments

- timing acquisition on a per-packet basis

- preamble detection on a per-packet basis

# 3.2 Traffic Models

TGac evaluation shall consider traffic models defined 802.11n usage model documents [3] including high-quality videos for VHT defined in [12] and high-speed file transfer.

Table 2. Traffic models

| Num. | Application | Offered Load (Mbps) | Protocol | MSDU Size (B) | Max. PLR | Max. Delay (ms) | Source [ref] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Lightly-compressed video | 150 | UDP | 1500 | 10^-7 | 10 | Motion JPEG2000 |
| 2 | Lightly-compressed video | 200 | UDP | 1500 | 10^-7 / 8 | 20 | H.264 |
| 3 | Compressed video | 50Mbps | UDP | 1500 | 10^-7 | 20 | Blu-rayTM |
| 4 | Compressed video | 20Mbps | UDP | 1500 | 3x10^-7 | 20 | HD-MPEG2 |
| 5 | VoD control channel | 0.06 | UDP | 64 | 10^-2 | 100 | Guess |
| 6 | Video Conf | 0.128 - 2 | UDP | 512 | 10^-4 | 100 | 1 |
| 7 | Internet Streaming video/audio | 0.1 – 4 | UDP | 512 | 10^-4 | 200 | 1 |
| 8 | Internet Streaming audio | 0.064~0.256 | UDP | 418 | 10^-4 | 200 | Group guess |
| 9 | VoIP | 0.096 | UDP | 120 | 5% | 30 | ITU-T G.114 300ms round-trip delay G.711 Codec |
| 10 | Reserved |  |  |  |  |  |  |
| 11 | Reserved |  |  |  |  |  |  |
| 12 | MP3 Audio Other formats are taking over (AAC/MPEG-4, OggVorbis, etc) | 0.064 – 0.32 | UDP | 418 | 10^-4 | 200 | 1 |
| 13 | Reserved |  |  |  |  |  |  |
| 14 | Content download (photo camera) | Max. 10Mbps | TCP | 1500 | N/A |  | Corresponds to USB and flash speed |
| 15 | Internet File transfer (email, web, chat) | Max. 10Mbps | TCP | 300 | N/A |  |  |
| 16 | Local File transfer, printing | Max. 1Gbps | TCP | 1500 | N/A |  | Aps guess |
| 17 | Reserved |  |  |  |  |  |  |
| 18 | Reserved |  |  |  |  |  |  |
| 19 | Reserved |  |  |  |  |  |  |
| 20 | Video phone | 0.5 | UDP | 512 | 10^-2 | 100 | Aps guess |
| 21 | Remote user interface (X11, Terminal Server Client) (remote display/keyboard/mouse) | 0.5-1.5 (peak) | UDP | 700 | N/A | 100 | 11-03-0696r0 |
| 22 | Clicking on web link | 0.256 | TCP | 64 | N/A |  | desribed in 11-03-0802r23 (pp. 27) |
| 23 | Infinite Source Model | Infinite (transmit buffer always full) | TCP | 1500 or 1000 or 300 | N/A |  | Popular model in network analysis |

# 4. Simulation Scenarios

Simulation scenarios for TGac evaluation are summarized as:

Table 3. Simulation scenarios

| Scenario Number | Purpose | Note |
| --- | --- | --- |
| 1 | Test compliance to PAR. | Single STA 500Mbps throughput at the MAC SAP |
| 2 | Test compliance to PAR. | Multi STA 1Gbps throughput at the MAC SAP |
| 3 | In-home entertainment application without OBSS | Multiple flows with varied QoS requirements. Includes several lightly-compressed video flows. Aligns with Category 1 and Category 2 applications. |
| 4 | In-home entertainment application with OBSS | Multiple flows with varied QoS requirements. Includes several lightly-compressed video flows. Aligns with Category 1 and Category 2 applications. |
| 5 | Enterprise network without OBSS | Stress test for TGac operation. Scenario with large number of flows. Aligns with Category 2 and Category 3 applications. |
| 6 | Enterprise network with OBSS | Stress test for TGac operation. Scenario with large number of flows. Aligns with Category 2 and Category 3 applications. |

# 4.1 Test for Compliance to PAR

4.1.1 Point-to-point link test (scenario #1)

Synthetic test case to demonstrate single STA 500Mbps throughput at the MAC SAP.

This scenario is derived from scenario #19 defind in 802.11n usage model document.

Two stations

One TGac AP is source.

One TGac STA is sink.

Traffic from AP to STA

Protocol: UDP

Offered load : infinite

MSDU size: 1500

PHY channel model

Model B

Locations of stations

Fixed locations: (0,0) meters for AP and (0,5) meters for STA

Meet requirements in [functional requirements Sections 2.1.2]

4.1.2 Point-to-multi-point link test (scenario #2)

Synthetic test case to demonstrate multi STA aggregated 1Gbps throughput at the MAC SAP.

This scenario is also derived from scenario #19 defind in 802.11n usage model document.

Number of stations (AP + STAs): at least 3

One TGac AP is source

Number of TGac STAs which are sinks : at least 2

Traffic from AP to STA

Protocol: UDP

Offered load : infinite

MSDU size: 1500

PHY channel model

Model D

Locations of stations

Fixed locations

Meet requirements in [functional requirements Sections 2.1.1]

Table 4. Flows in scenario #2

| Flow No. | Source | Source Location (meters) | Sink | Channel Model | Sink Location (meters) | Application | Application Load (Mbps) | Rate Distribution | MSDU Size (B) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Downlink Flows |  |  |  |  |  |  |  |  |
| 1 | AP | (0,0) | STA1 | D | (0,10) | Data |  | Infinite Backlog , UDP | 1500 |
| 2 | AP | (0,0) | STA2 | D | (10,0) | Data |  | Infinite Backlog , UDP | 1500 |
|  | : : | : : |  | : : | : : | : : |  | : : | : : |
| N | AP | (0,0) | STAN | D | (-6,-8) | Data |  | Infinite Backlog , UDP | 1500 |

Note) Different bit rate can be offered to each flow.

# 4.2 In-Home Entertainment Application

# 4.2.1 In-Home Entertainment Application without OBSS (scenario #3)

Test case to demonstrate in-home entertainment application with multiple flows with varied QoS requirements including lightly-compressed video.

This scenario is derived from scenario #1 defind in 802.11n usage model document and modified in order to consider usage model 2a and 2b (PVR’s in residential) defined in [12] as well.

13 stations

One TGac AP is source and sink.

12 STAs are souces and sinks.

Traffic from AP to STAs

Lightly-compressed video, compressed video, Internet file, Internet streaming video, MP3 audio, VoIP

Traffic from STAs to AP

Compressed video, VoD control channel, video console, Internet entertainment, VoIP

Traffic STAs to STAs

Local file transfer, video phone, controller to console, compressed video

PHY channel for each link

Channel model type applied : Model C

Channel model break point between LOS and NLOS: unchanged from 802.11n

Shadowing term is captured from LOS or NLOS shadowing case in [8].

For example, it is 5 dB (standard deviation) for NLOS channel model C.

In 802.11n channel model document [8], shadow fading std. dev. is specified for each channel model between 3dB and 6dB at page 7.

But,

in 802.11n usage model document [3], shadowing term applied to all the 802.11n simulation scenarios is set to 0dB for generating a channel realization at page 22.

Locations of stations

Fixed locations (unchagend from 802.11n scenario #1)

Meet requirements specified in PLR and Max. Delay per each flow

Total throughput condition

It is available to offer infinite load for this scenario with TCP flows.

Table 5. Flows in scenario #3

| Flow No. | STAs (Source/Sink) | Source Location (meters) | Sink Location (meters) | Channel Model | Application (Forward Traffice / Backward Traffic) | Application Load (Mbps) (Forward / Backward) | Rate Distribution (Forward / Backward) | MSDU Size (B) (Forward / Backward) | Max. Delay (ms) (Forward / Backward) | Max. PLR (Forward / Backward) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | AP / STA1 | (0,0) | (0,5) | C | LC Video / VoD control channel | 150.00 / 0.06 | Constant, UDP / Constant UDP | 1500 / 64 | 10 / 100 | 10^-7 / 10^-2 |
| 2note | STA7 / STA2 | (7,-7) | (-10,-10) | C | HD MPEG2 / control channel | 20.00 / 0.06 | Constant UDP / Constant UDP | 1500 / 64 | 20 / 100 | 3x10^-7 / 10^-2 |
| 4 note | STA9/ STA 4 | (0,-10) | (-7,7) | C | Blu-rayTM/ control channel | 50.00 / 0.06 | Constant, UDP / Constant UDP | 1500 / 64 | 20 / 100 | 10^-7 / 10^-2 |
| 5 | AP / STA4 | (0,0) | (-7,7) | C | Internet file / clicking on web link | Max. 10Mbps / 0.256 | TCP / TCP | 300 / 64 | Inf. / Inf. | N/A / N/A |
| 6 | AP / STA10 | (0,0) | (10,10) | C | HD MPEG2 / video console + Internet entertainment | 20.00 / 1.00 | Constant UDP / Constant UDP | 1500 / 512 | 20 / 50 | 3x10^-7 / 10^-4 |
| 7 | AP / STA11 | (0,0) | (10,5) | C | MP3 audio | 0.13 / | UDPConstant, UDP | 418 | 200 | 10^-4 |
| 8 | STA11/STA10 | (10,5) | (10,10) | C | Controller to Console | 0.50 | Constant, UDP | 50 | 16 | 10^-4 |
| 9 | AP / STA12 | (0,0) | (20,0) | C | VoIP / VoIP | 0.096 / 0.096 | UDP / UDP | 120 / 120 | 30 / 30 | 5% / 5% |
| 10 | AP / STA13 | (0,0) | (0,20) | C | VoIP / VoIP | 0.096 / 0.096 | UDP / UDP | 120 / 120 | 30 / 30 | 5% / 5% |
| 11 | AP / STA14 | (0,0) | (0,-20) | C | VoIP / VoIP | 0.096 / 0.096 | UDP / UDP | 120 / 120 | 30 / 30 | 5% / 5% |
| 12 | STA4 / STA10 | (-7,7) | (10,10) | C | Local file transfer | Max. 1Gbps | TCP | 1500 | Inf. | N/A |
| 13 | STA5 / STA6 | (-15,0) | (0,-15) | C | Video Phone / Video Phone | 0.50 / 0.50 | Constant, UDP / Constant UDP | 512 / 512 | 100 / 100 | 10^-2 / 10^-2 |
|  |  |  |  |  | Total Throughput |  |  |  |  |  |

Note) Display/video flows are specified as logical links so that they may either be simulated as direct link setup or relay via the AP.

Note) Logical link in this scenario can be seen with topology in Support document.

### 4.2.2 In-Home Entertainment Application with OBSS (scenario #4)

Test case to demonstrate in-home entertainment application with multiple flows with varied QoS requirements including lightly-compressed video and OBSS.

3 BSSs: A, B and C

BSS A is identical to the BSS in TGac scenario #3 with one AP and 12 associated STAs

BSS B is an 802.11ac BSS with one AP and 3 associated STAs

BSS C is lower 40MHz 802.11n BSS with one AP and 3 associated STAs

Each AP is source and sink for its associated STAs

Scenario #4 includes two cases of spectrum positions for BSSs A, B, and C, which are shown in Figs. 1 and 2.

Figure 1: Spectrum positions for BSSs A, B, and C (Case 1).

Figure 2: Spectrum positions for BSSs A, B, and C (Case 2).

Traffic from AP to STAs

Lightly-compressed video, compressed video, Internet file, Internet streaming video, MP3 audio, VoIP

Traffic from STAs to AP

Compressed video, VoD control channel, video console, Internet entertainment, VoIP

Traffic STAs to STAs

Local file transfer, video phone, controller to console, compressed video

PHY channel for each link within a BSS or between BSSs

Channel model type applied : Model C for BSS A and BSS B. TGn Model C for BSS C.

Channel model type for link between different BSSs : Model C between BSS A and BSS B. TGn Model C between BSS C and BSS A.

Channel model break point between LOS and NLOS: unchanged from 802.11n

Shadowing term is captured from LOS or NLOS shadowing case in [8].

For example, it is 5 dB (standard deviation) for NLOS channel model C.

In 802.11n channel model document [8], shadow fading std. dev. is specified for each channel model between 3dB and 6dB at page 7.

But, in 802.11n usage model document [3], shadowing term applied to all the 802.11n simulation scenarios is set to 0dB for generating a channel realization at page 22.

PHY channel for each link between BSS

Channel model break point between LOS and NLOS: unchanged from 802.11n

Locations of stations

Locations for BSS A are unchagend from TGac scenario #3.

Relative locations for BSS B are taken from TGac scenario #3 that all STAs except for STA1, STA4, and STA10 are deleted and re-numbered. Locations of BSS B are translated by (xb, yb) = (40,0) from those of BSS A.

Relative locations for BSS C are taken from TGac scenario #3 that all STAs except for STA1, STA4, and STA10 are deleted and re-numbered. Locations for BSS C are translated by (xc, yc) = (-40,0) from those of BSS A.

Walls

Room walls are introduced at x=-39 and x=41.

Attenuation factor of each room wall is set to 3 dB.

Attenuation factor of outside wall is set to 20 dB.

Meet requirements specified in PLR and Max. Delay per each flow

Total throughput condition

It is available to offer infinite load for this scenario with TCP flows.

Table 6. Flows at BSS B in scenario #4

| Flow No. | STAs (Source/Sink) | Source Location (meters) | Sink Location (meters) | Channel Model | Application (Forward Traffice / Backward Traffic) | Application Load (Mbps) (Forward / Backward) | Rate Distribution (Forward / Backward) | MSDU Size (B) (Forward / Backward) | Max. Delay (ms) (Forward / Backward) | Max. PLR (Forward / Backward) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14 | AP B / STA15 | (xb,yb) | (xb,5+yb) | C | Blu-rayTM/ control channel | 50.00 / 0.06 | Constant, UDP / Constant UDP | 1500 / 64 | 20 / 100 | 10^-7 / 10^-2 |
| 15 | STA16 / STA17 | (-7+xb,7+yb) | (10+xb,10+yb) | C | Local file transfer | Max. 1Gbps | TCP | 1500 | Inf. | N/A |
|  |  |  |  |  | Total Throughput |  |  |  |  |  |

Table 7. Flows at BSS C in scenario #4

| Flow No. | STAs (Source/Sink) | Source Location (meters) | Sink Location (meters) | Channel Model | Application (Forward Traffice / Backward Traffic) | Application Load (Mbps) (Forward / Backward) | Rate Distribution (Forward / Backward) | MSDU Size (B) (Forward / Backward) | Max. Delay (ms) (Forward / Backward) | Max. PLR (Forward / Backward) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16 | AP C / STA18 | (xc,yc) | (xc,5+yc) | C | HD-MPEG2 / VoD control channel | 20/0.06 | Constant UDP / Constant UDP | 1500/64 | 20/100 | 3x10^-7 / 10^-2 |
| 17 | STA19 / STA20 | (-7+xc,7+yc) | (10+xc,10+yc) | C | Local file transfer | Max. 1Gbps | TCP | 1500 | Inf. | N/A |
|  |  |  |  |  | Total Throughput |  |  |  |  |  |

Figure 3: AP and STA locations for scenario #4.

# 4.3 Enterprise Network

### 4.3.1 Enterprise Network without OBSS (scenario #5)

Test case to demonstrate enterprise network application with large number of flows (over 30).

This scenario is derived from scenario #4 defind in 802.11n usage model document.

21 stations

One TGac AP is source and sink.

20 STAs are souces and sinks.

Traffic from AP to STAs

Internet file, video conferencing, Internet streaming video + MP3, local file transfer

VoIP

Traffic from STAs to AP

Clicking on web link, file upload, video conferencing, VoIP

PHY channel for each link

Channel model type applied : Model D

Channel model break point between LOS and NLOS : unchanged from 802.11n

Shadowing term is captured from LOS or NLOS shadowing case in [8].

For example, it is 5 dB (standard deviation) for NLOS channel model D.

In 802.11n channel model document [8], shadow fading std. dev. is specified for each channel model between 3dB and 6dB at page 7.

But in 802.11n usage model document [3], shadowing term applied to all the 802.11n simulation scenarios is set to 0dB for generating a channel realization at page 22.

Locations of stations

Locations are unchagend from 802.11n scenario #4, excepting that STAs 3, 6, 9, … , 30 are deleted (see Figure 4) and the surviving STAs are not renumbered.

Meet requirements specified in PLR and Max. Delay per each flow

Total throughput condition

It is available to offer infinite load for this scenario with TCP flows.

Figure 4: STA locations for scenario #5.

85

| . |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Flow No. | Source | Source Location (meters) | Sink | Sink Location (meters) | Channel | Application | Application Load (Mbps) | Rate Distribution | MSDU Size (B) | Max. Delay (ms) | Max. PLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Downlink Flows |  |  |  |  |  |  |  |  |  |  |
| 1 | AP | (0,0) | STA1 | (5,-9.5) | D | Internet file | Max. 10Mbps | TCP | 300 | Inf. | N/A |
| 2 | AP | (0,0) | STA2 | (3.5,7.5) | D | Internet file | Max. 10Mbps | TCP | 300 | Inf. | N/A |
| 3 | AP | (0,0) | STA4 | (-4.5,0.5) | D | Internet file | Max. 10Mbps | TCP | 300 | Inf. | N/A |
| 4 | AP | (0,0) | STA5 | (-1.5,6) | D | Internet file | Max. 10Mbps | TCP | 300 | Inf. | N/A |
| 5 | AP | (0,0) | STA7 | (-9,-5) | D | Video conferencing | 1.00 | Constant, UDP | 512 | 100 | 10^-4 |
| 6 | AP | (0,0) | STA8 | (-8.5,8.5) | D | Video conferencing | 1.00 | Constant, UDP | 512 | 100 | 10^-4 |
| 7 | AP | (0,0) | STA10 | (-3,0.5) | D | Internet Streaming video + MP3 audio | 2.00 | UDP | 512 | 200 | 10^-4 |
| 8 | AP | (0,0) | STA11 | (-0.5,8) | D | Local File transfer | Max. 1Gbps | TCP | 1500 | Inf. | N/A |
| 9 | AP | (0,0) | STA13 | (-4,-4) | D | Local File transfer | Max. 1Gbps | TCP | 1500 | Inf. | N/A |
| 10 | AP | (0,0) | STA14 | (7.5,-1) | D | Local File transfer | Max. 1Gbps | TCP | 1500 | Inf. | N/A |
| 11 | AP | (0,0) | STA16 | (8,-6) | D | Local File transfer | Max. 1Gbps | TCP | 1500 | Inf. | N/A |
| 12 | AP | (0,0) | STA17 | (0,-7.5) | D | Local File transfer | Max. 1Gbps | TCP | 1500 | Inf. | N/A |
| 13 | AP | (0,0) | STA19 | (-2.5,-4.5) | D | Local File transfer | Max. 1Gbps | TCP | 1500 | Inf. | N/A |
| 14 | AP | (0,0) | STA20 | (0.5,-2) | D | Local File transfer | Max. 1Gbps | TCP | 1500 | Inf. | N/A |
| 15 | AP | (0,0) | STA25 | (3.5,-5) | D | VoIP | 0.10 | Constant, UDP | 120 | 30 | 5% |
| 16 | AP | (0,0) | STA26 | (9,9.5) | D | VoIP | 0.10 | Constant, UDP | 120 | 30 | 5% |
| 17 | AP | (0,0) | STA28 | (-8,-5.5) | D | VoIP | 0.10 | Constant, UDP | 120 | 30 | 5% |
| 18 | AP | (0,0) | STA29 | (1.5,3.5) | D | VoIP | 0.10 | Constant, UDP | 120 | 30 | 5% |
| Flow No. | Source | Source Location (meters) | Sink | Sink Location (meters) |  | Application | Application Load (Mbps) | Rate Distribution | MSDU Size (B) | Max Delay (ms) | PLR |
|  | Uplink Flows |  |  |  |  |  |  |  |  |  |  |
| 19 | STA1 | (5,-9.5) | AP | (0,0) | D | Clicking on web link | 0.256 | TCP | 64 | Inf. | N/A |
| 20 | STA2 | (3.5,7.5) | AP | (0,0) | D | Clicking on web link | 0.256 | TCP | 64 | Inf. | N/A |
| 21 | STA4 | (-4.5,0.5) | AP | (0,0) | D | File Upload | Max. 1Gbps | TCP | 1000 | Inf. | N/A |
| 22 | STA5 | (-1.5,6) | AP | (0,0) | D | File Upload | Max. 1Gbps | TCP | 1500 | Inf. | N/A |
| 23 | STA7 | (-9,-5) | AP | (0,0) | D | Video conferencing | 1.00 | Constant, UDP | 512 | 100 | 10^-4 |
| 24 | STA8 | (-8.5,8.5) | AP | (0,0) | D | Video conferencing | 1.00 | Constant, UDP | 512 | 100 | 10^-4 |
| 25 | STA22 | (0,-4.5) | AP | (0,0) | D | File Upload | Max. 1Gbps | TCP | 1500 | Inf. | N/A |
| 26 | STA23 | (-1.5,7) | AP | (0,0) | D | File Upload | Max. 1Gbps | TCP | 1500 | Inf. | N/A |
| 27 | STA25 | (3.5,-5) | AP | (0,0) | D | VoIP | 0.10 | Constant, UDP | 120 | 30 | 5% |
| 28 | STA26 | (9,9.5) | AP | (0,0) | D | VoIP | 0.10 | Constant, UDP | 120 | 30 | 5% |
| 29 | STA28 | (-8,-5.5) | AP | (0,0) | D | VoIP | 0.10 | Constant, UDP | 120 | 30 | 5% |
| 30 | STA29 | (1.5,3.5) | AP | (0,0) | D | VoIP | 0.10 | Constant, UDP | 120 | 30 | 5% |
|  |  |  |  |  |  | Total Measured Throughput |  |  |  |  |  |

Note) Logical link in this scenario can be seen with topology in Support document.

### 4.3.2 Enterprise Network with OBSS (scenario #6)

Test case to demonstrate enterprise network application with large number of flows and OBSS.

This scenario is derived from scenario #4 defined in 802.11n usage model document.

3 BSSs: A, B and C

BSS A is identical to the BSS in 802.11ac scenario #5 with one AP and 20 associated STAs

BSS B is an 802.11ac BSS using the same bandwidth as BSS A with one AP and 5 associated STAs

BSS C is a 20/40MHz 802.11n BSS with one AP and 5 associated STAs

Each AP is source and sink for its associated STAs

Traffic from AP to STAs

Internet file, video conferencing, Internet streaming video + MP3, local file transfer

VoIP

Traffic from STAs to AP

Clicking on web link, file upload, video conferencing, VoIP

PHY channel for each link within a BSS or between BSSs

Channel model type applied: Model D for BSS A and B. TGn Model D for BSS C.

Channel model type among different BSSs : Model D between BSS A and BSS B. TGn Model D between BSS C and BSS A.

Channel model break point between LOS and NLOS : unchanged from 802.11n

Shadowing term is captured from LOS or NLOS shadowing case in [8] .

For example, it is 5 dB (standard deviation) for NLOS channel model D.

In 802.11n channel model document [8], shadow fading std. dev. is specified for each channel model between 3dB and 6dB at page 7.

But in 802.11n usage model document [3], shadowing term applied to all the 802.11n simulation scenarios is set to 0dB for generating a channel realization at page 22.

Locations of stations (see Figure 5)

Locations for BSS A are unchanged from 802.11n scenario #4, excepting that STAs 3,6,9, … 30 are deleted (i.e. BSS A is identical to scenario #4)

Locations for BSS B are taken from 802.11n scenario #4, with all STAs except 3,9,15,… 27 deleted and the AP and surviving STAs translated by (xb,yb) = (40,20).

Locations for BSS C are taken from 802.11n scenario #4, with all STAs except 6,1218, … 30 deleted and the AP and surviving STAs translated by (xc,yc) = (-40,-20).

Bandwidth sensitivity

If BSS A is 40 MHz, then BSS C is deleted, leaving BSSs A and B.

If BSS A is 80 MHz, then the scenario includes BSSs A, B and C

Parameters used by STAs within the BSS in 802.11ac scenario #5 should be reused in this scenario

Meet requirements specified in PLR and Max. Delay per each flow

Total throughput condition

It is available to offer infinite load for this scenario with TCP flows.

Figure 5: STA locations for scenario #6.

Table 9. Flows in scenario #6

| Flow No. | Source | Source Location (meters) | Sink | Sink Location (meters) | Channel | Application | Application Load (Mbps) | Rate Distribution | MSDU Size (B) | Max. Delay (ms) | Max. PLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Downlink Flows in BSS A |  |  |  |  |  |  |  |  |  |  |  |
| 1 | AP A | (0,0) | STA1 | (5,-9.5) | D | Internet file | Max. 10Mbps | TCP | 300 | Inf. | N/A |
| 2 | AP A | (0,0) | STA2 | (3.5,7.5) | D | Internet file | Max. 10Mbps | TCP | 300 | Inf. | N/A |
| 3 | AP A | (0,0) | STA4 | (-4.5,0.5) | D | Internet file | Max. 10Mbps | TCP | 300 | Inf. | N/A |
| 4 | AP A | (0,0) | STA5 | (-1.5,6) | D | Internet file | Max. 10Mbps | TCP | 300 | Inf. | N/A |
| 5 | AP A | (0,0) | STA7 | (-9,-5) | D | Video conferencing | 1.00 | Constant, UDP | 512 | 100 | 10^-4 |
| 6 | AP A | (0,0) | STA8 | (-8.5,8.5) | D | Video conferencing | 1.00 | Constant, UDP | 512 | 100 | 10^-4 |
| 7 | AP A | (0,0) | STA10 | (-3,0.5) | D | Internet Streaming video + MP3 audio | 2.00 | UDP | 512 | 200 | 10^-4 |
| 8 | AP A | (0,0) | STA11 | (-0.5,8) | D | Local File transfer | Max. 1Gbps | TCP | 1500 | Inf. | N/A |
| 9 | AP A | (0,0) | STA13 | (-4,-4) | D | Local File transfer | Max. 1Gbps | TCP | 1500 | Inf. | N/A |
| 10 | AP A | (0,0) | STA14 | (7.5,-1) | D | Local File transfer | Max. 1Gbps | TCP | 1500 | Inf. | N/A |
| 11 | AP A | (0,0) | STA16 | (8,-6) | D | Local File transfer | Max. 1Gbps | TCP | 1500 | Inf. | N/A |
| 12 | AP A | (0,0) | STA17 | (0,-7.5) | D | Local File transfer | Max. 1Gbps | TCP | 1500 | Inf. | N/A |
| 13 | AP A | (0,0) | STA19 | (-2.5,-4.5) | D | Local File transfer | Max. 1Gbps | TCP | 1500 | Inf. | N/A |
| 14 | AP A | (0,0) | STA20 | (0.5,-2) | D | Local File transfer | Max. 1Gbps | TCP | 1500 | Inf. | N/A |
| 15 | AP A | (0,0) | STA25 | (3.5,-5) | D | VoIP | 0.10 | Constant, UDP | 120 | 30 | 5% |
| 16 | AP A | (0,0) | STA26 | (9,9.5) | D | VoIP | 0.10 | Constant, UDP | 120 | 30 | 5% |
| 17 | AP A | (0,0) | STA28 | (-8,-5.5) | D | VoIP | 0.10 | Constant, UDP | 120 | 30 | 5% |
| 18 | AP A | (0,0) | STA29 | (1.5,3.5) | D | VoIP | 0.10 | Constant, UDP | 120 | 30 | 5% |
| Flow No. | Source | Source Location (meters) | Sink | Sink Location (meters) |  | Application | Application Load (Mbps) | Rate Distribution | MSDU Size (B) | Max Delay (ms) | PLR |
| Uplink Flows in BSS A |  |  |  |  |  |  |  |  |  |  |  |
| 19 | STA1 | (5,-9.5) | AP A | (0,0) | D | Clicking on web link | 0.256 | TCP | 64 | Inf. | N/A |
| 20 | STA2 | (3.5,7.5) | AP A | (0,0) | D | Clicking on web link | 0.256 | TCP | 64 | Inf. | N/A |
| 21 | STA4 | (-4.5,0.5) | AP A | (0,0) | D | File Upload | Max. 1Gbps | TCP | 1000 | Inf. | N/A |
| 22 | STA5 | (-1.5,6) | AP A | (0,0) | D | File Upload | Max. 1Gbps | TCP | 1500 | Inf. | N/A |
| 23 | STA7 | (-9,-5) | AP A | (0,0) | D | Video conferencing | 1.00 | Constant, UDP | 512 | 100 | 10^-4 |
| 24 | STA8 | (-8.5,8.5) | AP A | (0,0) | D | Video conferencing | 1.00 | Constant, UDP | 512 | 100 | 10^-4 |
| 25 | STA22 | (0,-4.5) | AP A | (0,0) | D | File Upload | Max. 1Gbps | TCP | 1500 | Inf. | N/A |
| 26 | STA23 | (-1.5,7) | AP A | (0,0) | D | File Upload | Max. 1Gbps | TCP | 1500 | Inf. | N/A |
| 27 | STA25 | (3.5,-5) | AP A | (0,0) | D | VoIP | 0.10 | Constant, UDP | 120 | 30 | 5% |
| 28 | STA26 | (9,9.5) | AP A | (0,0) | D | VoIP | 0.10 | Constant, UDP | 120 | 30 | 5% |
| 29 | STA28 | (-8,-5.5) | AP A | (0,0) | D | VoIP | 0.10 | Constant, UDP | 120 | 30 | 5% |
| 30 | STA29 | (1.5,3.5) | AP A | (0,0) | D | VoIP | 0.10 | Constant, UDP | 120 | 30 | 5% |
| Downlink Flows in BSS B |  |  |  |  |  |  |  |  |  |  |  |
| 31 | AP B | (xb,xb) | STA3 | (7.5+xb, -9.5+yb) | D | Internet file | Max. 10Mbps | TCP | 300 | Inf. | N/A |
| 32 | AP B | (xb,xb) | STA9 | (7+xb, -7.5+yb) | D | Internet Streaming video + MP3 audio | 2.00 | UDP | 512 | 200 | 10^-4 |
| 33 | AP B | (xb,xb) | STA15 | (3+xb, -0.5+yb) | D | Local File transfer | Max. 1Gbps | TCP | 1500 | Inf. | N/A |
| 34 | AP B | (xb,xb) | STA27 | (-6+xb, 2.5+yb) | D | VoIP | 0.10 | Constant, UDP | 120 | 30 | 5% |
| Uplink Flows in BSS B |  |  |  |  |  |  |  |  |  |  |  |
| 35 | STA3 | (7.5+xb, -9.5+yb) | AP B | (xb,xb) | D | Clicking on web link | 0.256 | TCP | 64 | Inf. | N/A |
| 36 | STA21 | (-6.5+xb, -3+yb) | AP B | (xb,xb) | D | File Upload | Max. 1Gbps | TCP | 1500 | Inf. | N/A |
| 37 | STA27 | (-6+xb, 2.5+yb) | AP B | (xb,xb) | D | VoIP | 0.10 | Constant, UDP | 120 | 30 | 5% |
| Downlink Flows in BSS C |  |  |  |  |  |  |  |  |  |  |  |
| 38 | AP C | (xc,yc) | STA6 | (-5.5+xc,4.5+yc) | D | Internet file, downloading large email attachments | Max. 10Mbps | TCP | 300 | Inf. | N/A |
| 39 | AP C | (xc,yc) | STA12 | (7+xc,7+yc) | D | Local File transfer | Max. 1Gbps | TCP | 1500 | Inf. | N/A |
| 40 | AP C | (xc,yc) | STA18 | (10+xc,0.5+yc) | D | Local File transfer | Max. 1Gbps | TCP | 1500 | Inf. | N/A |
| 41 | AP C | (xc,yc) | STA30 | (9.5+xc,3.5+yc) | D | VoIP | 0.10 | Constant, UDP | 120 | 30 | 5% |
| Uplink Flows in BSS C |  |  |  |  |  |  |  |  |  |  |  |
| 42 | STA6 | (-5.5+xc,4.5+yc) | AP C | (xc,yc) | D | Clicking on web link | 0.256 | TCP | 64 | Inf. | N/A |
| 43 | STA24 | (3+xc,2.5+yc) | AP C | (xc,yc) | D | File Upload | Max. 1Gbps | TCP | 1500 | Inf. | N/A |
| 44 | STA30 | (9.5+xc,3.5+yc) | AP C | (xc,yc) | D | VoIP | 0.10 | Constant, UDP | 120 | 30 | 5% |
|  |  |  |  |  |  | Total Measured Throughput |  |  |  |  |  |

# 4.4 Comparison Criteria

MAC SAP throughput (goodput)

Aggregated throughput (downlink + uplink + STA-to-STA)

Throughput of each flow in the simulation scenario

Packet Loss Rate for each flow in the simulation scenario

Lost packets include

Packets that exceed maximum latency requirement

Packets that are dropped after exceeding maximum number of re-tries

Specify number of antennas for each STA used in the simulation scenario

Specify number of 20 MHz channels used in the simulation scenario

# 4.5 Results Presentation Format

e.g.) for scenario #3

Table 10. Results presentation format

| Flow No. | STAs (Source/Sink) | Application (Forward Traffice / Backward Traffic) | Application Load (Mbps) (Forward / Backward) | Rate Distribution (Forward / Backward) | Max. Delay (ms) (Forward / Backward) | PLR (Forward / Backward) | Pass or Fail (Forward / Backward) | Latency Compliant (%) (Forward / Backward) | Measured Throughput (Mbps) (Forward / Backward) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | AP / STA1 | LC Video / VoD control channel | 150.00 / 0.06 | Constant, UDP / Constant UDP | 10 / 100 |  |  |  |  |
| 2note | STA7 / STA2 | HD MPEG2 / control channel | 20.00 / 0.06 | Constant UDP / Constant UDP | 20 / 100 |  |  |  |  |
| 4 note | STA9/ STA 4 | Blu-rayTM/ control channel | 50.00 / 0.06 | Constant, UDP / Constant UDP | 20 / 100 |  |  |  |  |
| 5 | AP / STA4 | Internet file / clicking on web link | Max. 10Mbps / 0.256 | TCP / TCP | Inf. / Inf. |  |  |  |  |
| 6 | AP / STA10 | HD MPEG2 / video console + Internet entertainment | 20.00 / 1.00 | Constant UDP / Constant UDP | 20 / 50 |  |  |  |  |
| 7 | AP / STA11 | MP3 audio | 0.13 / | UDP | 200 |  |  |  |  |
| 8 | STA11/STA10 | Controller to Console | 0.50 | Constant, UDP | 16 |  |  |  |  |
| 9 | AP / STA12 | VoIP / VoIP | 0.096 / 0.096 | UDP / UDP | 30 / 30 |  |  |  |  |
| 10 | AP / STA13 | VoIP / VoIP | 0.096 / 0.096 | UDP / UDP | 30 / 30 |  |  |  |  |
| 11 | AP / STA14 | VoIP / VoIP | 0.096 / 0.096 | UDP / UDP | 30 / 30 |  |  |  |  |
| 12 | STA4 / STA10 | Local file transfer | Max. 1Gbps | TCP | Inf. |  |  |  |  |
| 13 | STA5 / STA6 | Video Phone / Video Phone | 0.50 / 0.50 | Constant, UDP / Constant UDP | 100 / 100 |  |  |  |  |
|  |  | Total Given Throughput |  |  |  |  |  | Total Measured Throughput |  |

5. Summary of Functional Requirements

Table 11. Summary of functional requirements

| Requirement Number | Description | Requirement Statement | Status of Requirement | Notes (informative) |
| --- | --- | --- | --- | --- |
| R1 | Maximum multi-STA throughput | Support at least 1 Gbps at the top of the MAC SAP utilizing no more than 80MHz of channel bandwidth in 5GHz band |  |  |
| R2 | Maximum single link throughput | Support 500 Mbps throughput at the top of the MAC SAP utilizing no more than 80MHz of channel bandwidth in 5GHz band |  |  |
| R3 | 802.11a backward compatibility | TGac amendment shall provide backward compatibility with IEEE802.11a devices operating in the 5 GHz frequency band. . |  |  |
| R4 | 802.11n backward compatibility | TGac amendment shall provide backward compatibility with IEEE802.11n devices operating in the 5 GHz frequency band |  |  |
| R5 | Coexistence with 802.11a/n devices operating in 5GHz | TGac amendment shall provide mechanisms to enable coexistence between TGac and legacy IEEE802.11a/n devices. |  |  |
| R6 | Compliance to PAR | The proposal complies with the PAR and 5 Criteria [1]. |  |  |

# 6. Support Documents

11-09-0838-02-00ac-tgac-supporting-document-for-tgac-evaluation-methodology.ppt

11-10-0078-00-00ac-tgac-supporting-document-for-wall-penetration-loss.ppt

7. References

- 11-08-0807-04-0vht-below-6-ghz-par-nescom-form-plus-5cs

- 11-03-0813-13-000n-functional-requirements

- 11-03-0802-23-000n-usage-models

- 11-08-0307-01-0vht-on-the-feasibility-of-1gbps-for-various-mac-phy-architectures

- 11-08-0535-00-0vht-phy-and-mac-throughput-analysis-with-80-mhz-for-vht-below-6-ghz

- 11-09-0071-01-00ac-discussion-on-functional-requirements

- 11-03-0814-31-000n-comparison-criteria

- 11-03-0940-04-000n-tgn-channel-models

- 11-09-0059-04-00ac-tgac-802.11ac-proposed-selection-procedure

- 11-09-0376-01-00ac-proposal-for-tgac-evaluation-methodology

- 11-09-0308-08-00ac-tgac-channel-model-addendum-document

- 11-09-0161-02-00ac-802.11ac-usage-model-document

- 11-09-0980-01-00ac-change-edits-for-enterprise-simulation-scenario

- 11-09-1076-00-00ac-home-network-simulation-scenario-with-obss

Note, this corresponds to a loss of a 1024B MSDU per hour.   The TS PDU PLR is higher than this.  It is not known what is the effect to the decoder of giving it burst packet losses.

Note, a PLR of 10^-7 will not be measurable in our simulation technologies.