IEEE P802.11
Wireless LANs

| Protecting Action Frames and Resolutions Related to Action Frame Protection CIDs |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-04-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Kapil Sood | Intel | 2111 NE 25th Ave JF3-206, Hillsboro OR 97006 | +1 503 264 3759 | Kapil.sood@intel.com |
| Jesse Walker | Intel | 2111 NE 25th Ave JF3-206, Hillsboro OR 97006 | +1 503 712 1849 | Jesse.walker@intel.com |
| Nancy Cam-Winget | Cisco | 225 E Tasman Dr, San Jose CA 95134 | +1 408 853 0532 | ncamwing@cisco.com |
| Jouni Malinen | Atheros |  | +1 408 426 5459 | jouni.malinen@atheros.com |
| Henry Ptasinski | Broadcom | 150 Mathilda Place Sunnyvale, CA 94086 USA | +1-408-543-3316 | henryp@broadcom.com |
| Mike Montemurro | RIM | 4701 Tahoe Dr, Mississauga, ON. L4W 5W4 | +1-905-629-4746 x14999 | mmontemurro@rim.com |
| Paul Lambert | Marvell | 5488 Marvell Lane, Santa Clara CA 95054 | +1 650 787 9141 | Paul@marvell.com |

Abstract

This document contains proposed resolutions to the following comments from SB on D7.0

- CID # 19

- CID # 20

- CID # 70

- CID # 49

All changes apply to TGw draft D8.0.

Resolution to CID 19 as “Principle”, “See Resolution to CID 70”

Resolution to CID 20 as “Principle”, “See Resolution to CID 70”

Resolution to CID 49 as “Principle”, “See Resolution to CID 70”

Resolution for CID 70 is given below:

Insert a new definition in 3, page 1, line 64

Robust Action frame: Robust Action frames are defined as Action frames with category values specified in 7.3.1.11 Table 7-24 with “Yes” in the “Robust” column.

Change 5.4.3.8, page 4, line 42

The Robust Management frames are Disassociation and Deauthentication frames, and Robust Action Frames.  Action Frames specified with “No” in the “Robust” column are not Robust Management frames and shall not be protected.

Insert in Table 7-24 from IEEE 802.11-2007, Clause 7.3.1.11., page 96, line 1

A new Column “Robust”

In the row with Code value 0, Spectrum Management, insert “Yes” in “Robust” column

In the row with Code value 1, QoS, insert “Yes” in “Robust” column

In the row with Code value 2, DLS, insert “Yes” in “Robust” column

In the row with Code value 3, Block Ack, insert “Yes” in “Robust” column

In the row with Code value 127, Vendor Specific, insert “No” in “Robust” column

Insert a new row “Code=<ANA> (suggested value 126), Meaning=Vendor Specific Protected, See subclause=7.4.5, Robust=Yes”

Insert in Table 7-24 from IEEE 802.11k-2007, Clause 7.3.1.11

In the row with Code value 4, Public, insert “No” in “Robust” column

In the row with Code value 5, Radio Measurement, insert “Yes” in “Robust” column

Insert in Table 7-24 from IEEE 802.11r-2007, Clause 7.3.1.11

In the row with Code value 6, Fast BSS Transition, insert “Yes” in “Robust” column

Insert in 7.4.5 in IEEE 802.11-2007, page 151, lines 13 (in a new line at end of first paragraph in 7.4.5)

Note: If protection for management frame is negotiated, then Vendor-Specific Protected Action frames (See 7-24) are protected, otherwise, treated similar to the unprotected Vendor Specific Action frames with Category Code=127.

Change 8.4.11, page 34, lines 8-16

Robust Management frame protection cannot be applied until the PTK and IGTK has been established with the STA..  A STA shall not transmit Robust Action frames until it has installed the PTK for the peer STA, or, in the case of broadcast/multicast, has installed the IGTK. The STA shall discard any Robust Action frames received before the PTK and IGTK are installed. Action Frames with “No” in the “Robust” column in 7.3.1.11 Table 7-24 shall not be protected.

Insert in Table 7-24, Clause 7.3.1.11 page 12 line 27

In the row with Code value 8, SA Query, insert “Yes” in “Robust” column

Change Clause 8.7.2.1a,  page 48 line 10

…|| not Robust Action Frame)

Change Clause 8.7.2.1a,  page 48 line 55

…|| not Robust Action Frame)

Change Clause 8.7.2.3a,  page 52 line 2

…|| not Robust Action Frame)