IEEE P802.11
Wireless LANs

| TGn Proposal on the protection of action frames |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-04-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Matthew Fischer | Broadcom | 190 Mathilda Place, Sunnyvale, CA 94086 | +1 408 543 3370 | mfischer@broadcom.com |

Abstract

This submission contains proposed resolutions for MAC comments from the TGn sponsor ballot 2 (i.e. third sponsor ballot).

# Revision notes

R1:

CID 2003 – added a sentence to resolution, changed to green

CID 2004 – added a sentence to resolution, changed to green

CID 2008 – minor modification to wording, but no substantive change to resolution, changed to green

CID 2009 – changed to green

CID 2010 – slight modification to proposed change text to include the word “defined”, changed to green

CID 2011 – selected one of two options for resolution, updated that option

R0: Initial revision

| CID | Commenter | Page | Line | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2003 | Worstell, Harry |  |  |  | The TGn amendment is incompatible with the scope of the document it is amending. IEEE Std 802.11-2007 1.1 states "...to define one medium access control (MAC) and several physical layer (PHY) specification...". The distinction between "STA" and "HT STA", as applied to MAC functions, violates this scope statement. | change all occurrences of "HT STA" and "HT AP" in the document to STA and AP, respectively. | Disagree –There is nothing in the stated scope of the standard that disallows a single MAC from having multiple optional features, and so, from the first version of the 802.11 standard, the MAC has always had optional features. It is convenient and appropriate to create specific terminology related to such optional features in order to create a more readable standard. Such terminology can also apply to sets of optional or mandatory features. An example of the previous use of such terminology is the term PC which refers to the Point Coordinator that performs the point coordination function, which is an optional feature of the first version of the 802.11 standard. There are instances of AP and PC used throughout the standard, and this is perfectly acceptable, as the terms exist solely to note when an optional feature is being employed. Also see HC, and QOS STA, which are later additions to the standard. The uses of HT STA and HT AP are similar to all of these examples, in that they too, simply provide a convenient way to express requirements for STAs that implement an optional set of features. In addition, the comment is procedurally disallowed because it refers to text that has not changed between draft 8.0 and draft 9.0. |
| 2004 | Worstell, Harry |  |  |  | The TGn amendment is incompatible with the scope of the document it is amending. IEEE Std 802.11-2007 1.1 states "to define one medium access control (MAC) and several physical layer (PHY) specification". The use of the HT Capabilities information element to advertise the support of various MAC features violates this principle. | move the indications of support for MAC features from the HT Capabilities element to the Extended Capabilities element. Specifically, move the indication of support of Block Ack, A-MSDU, RD, and PCO. | Disagree – (this resolution reads differently from the resolution to CID 2003) There is nothing in the stated scope of the standard that disallows a single MAC from having multiple optional features, and so, from the first version of the 802.11 standard, the MAC has always had optional features. It is convenient and appropriate to create specific terminology related to such optional features in order to create a more readable standard. Such terminology can also apply to sets of optional or mandatory features. There is nothing in the scope that prevents the coupling of multiple optional features, which is what has been done in this instance. An example of coupling of multiple features in the 802.11 standard is: Block Acknowledgement, which can only be employed by STAs that also support the QOS feature. The case cited in the comment is similar, in that some optional features of the amendment are only allowed to exist in an implementation when coupled with another optional feature. This practice is not new and it is not out of scope. In addition, the comment is procedurally disallowed because it refers to text that has not changed between draft 8.0 and draft 9.0. |
| 2008 | O'Hara, Robert | 43.00 | 64 | 7.2.3.13 | The new statement of behavior in response to receipt of an Action NoAck frame does not belong in clause 7, which is supposed to be reserved for frame format descriptions. This statement belongs in clause 9, if it is not there already. | Delete the sentence "No frame is transmitted as an acknowledgement in response to the receipt of an Action No Ack frame." and move it to clause 9 or 11. This will not result in any change to the functionality, but does put the behavioral description where it belongs. | Principle – The statement is not normative, but only informative, and this information is needed in order to allow the proper interpretation of the normative language that already exists in 9.2.8. However, the TGn amendment also already has included this same information in a new sentence to be added to 9.2.8, so the information found in 7.2.3.13 is redundant. TGn editor to delete the sentence cited by the commenter in CID 2008 within subclause 7.2.3.13. (Note however, that the normative behavior in 9.2.8 of draft 9.0 speaks only of transmitter behavior and not receiver behavior – see CID 2009 which fixes this issue.) |
| 2009 | O'Hara, Robert | 115.00 | 65 | 9.2.8 | The newly added second "paragraph" does not state any requirement of an implementation. | Replace the new "paragraph" with the following: "On receipt of a frame of type Management, subtype Action NoAck, a STA shall not send an Acknowledge frame in response." | Agree |
| 2010 | O'Hara, Robert | 123.00 | 37 | 9.6.0e.5.2 | The term "primary rate" is used here to define a requirement for the rate selected for the transmission of a response frame. Unfortunately, "primary rate" is not defined anywhere in the draft. | Define "primary rate" properly. The assumption is that all this complex text above line 37 is the definition. If that is so, play start this paragraph with ""the primary rate is...". | Principle – TGn editor to make changes shown in document 11-09-0455r1 under any heading that includes CID 2010. Note that it is extremely difficult to determine a wording that allows the language to flow properly in creating the description of the determination of the response rate. |

CID 2010

TGn editor: change the first paragraph of 9.6.0e.5.2 on page 123 line 123 of TGn draft 9.0 as shown:

9.6.0e.5.2 Selection of a rate or MCS

To allow the transmitting STA to calculate the contents of the Duration/ID field, a STA responding to a received frame transmits its control response frame at a primary rate, or at an alternate rate or at an MCS, each of which is defined according to the following rules:

| 2011 | Fischer, Matthew | 59.00 | 29 | 7.3.2 | It seems difficult to provide TXBF feedback information simultaneously with MCS feedback. The spec should accommodate this desire. | Add an information element "MCS feedback" with a body format similar to the Link Adaptation control field of the HT Control field, so that MCS feedback (perhaps unsolicited) can be included in a TXBF feedback mgmt action frame. | Principle – make changes to include the new element as suggested, or modify the management frame frame format shown in 7.2.3 to show that management frames can include the HTC field. |
| --- | --- | --- | --- | --- | --- | --- | --- |

CID 2011

NOTE that 7.1.3.5a, 7.1.3.1.9 and 9.19.3 already include explicit indication that +HTC field is allowed within a mgmt frame.

TGn editor: In TGn draft 9.0 within subclause 7.2.3, in table 7.26, on page 39 at about line 19, insert a diagram of the management frame showing a modification of the diagram from the one that appears in the baseline, with the modified diagram to include the HT Control field inbetween the existing Sequence Control field and Frame Body field, and add an accompanying new paragraph preceeding the paragraph that begins with “The frame body” as shown:

The HT Control field is defined in 7.1.3.5a (HT Control field). The presence of the HT Control field is determined by the Order subfield of the Frame Control field, as specified in 7.1.3.1.9 (Order field).

TGn editor: In subclause 9.18.2 Link adaptation using the HT Control field of TGn draft 9.0, on page 174 at about line 42, insert the following new paragraph:

A STA whose most recently transmitted MCS Feedback field of the HT Extended capabilities field of the HT Capabilities element is set to 2 or 3 may transmit unsolicited MCS feedback in any frame that contains a +HTC field.

| 2012 | Fischer, Matthew | 90.00 | 22 | 7.4.10 | Move the HT action frames to Public so that they are not required to be encrypted. | Move the HT action frames to Public so that they are not required to be encrypted. | Principle – TGn editor to make the changes shown in document 11-09-0455r0 under any heading that includes CID 2012. |
| --- | --- | --- | --- | --- | --- | --- | --- |

CID 2012

7.3.1.11 Action field

TGn editor: change Table 7-24 of 7.3.1.11 Action field on page 45 line 10 of TGn draft 9.0 by adding “No” in the “Robust” column of the row that contains “HT” in the Meaning column.

| 2013 | Fischer, Matthew | 93.00 | 13 | 7.4.10.6 | It is difficult to provide simultaneous TXBF channel feedback or matrix feedback information while also providing MCS feedback information. | Add an item to the body of the CSI feedback action frame format which is a new element containing a link adaptation control field, just like the one in the HT control field. As in another comment, create the new element. Also modify the body of the compressed and uncompressed beamforming frame formats. | Principle – see CID 2011. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2017 | Van Zelst, Allert | 192.00 | 41 | 9.21.2 | There is no need to limit MM NDPs to two or more spatial streams. I can see why it is needed for GF NDPs, simply because then the signal fields need to be processed very fast to be able to see that no symbols are following the signal fields. For MM NDPs with 1 spatial stream, the HT-STF and HT-LTF are following the signal fields so there is enough time there to decode and parse the signal fields. | Change "MCS shall indicate two or more spatial streams" to "When the TXVECTOR parameter FORMAT is set to HT_GF, MCS shall indicate two or more spatial streams". | Transferring to PHY |
| 2018 | Chu, Liwen | 109.00 | 27 | 9.2.3.0b | Here the draft says "RIFS shall not be used between frames with different RA values, except within a PSMP sequence as explivitly indicated in 9.16.1.2 and 9.16.1.3". But RIFS between frames with different RA values is never allowed in 9.16.1.2 (P164, L21 This means that PPDU to different RA are separated by at least SIFS). In 9.16.1.3, aIUStime can be used between frames with different RA values when RIFS is allowed but aIUstime (8us) is not equal to FIFS(2us). Change this sentence to reflect what is defined in 9.16.1.2 and 9.16.1.3. | As proposed. | Agree – TGn editor to remove the phrase “, except within a PSMP sequence as explicitly indicated in 9.16.1.2 (PSMP Down link transmission (PSMP-DTT)) and 9.16.1.3 (PSMP Up link transmission (PSMP-UTT))” From the second paragraph of 9.2.3.0b |
| 2019 | Chu, Liwen | 107.00 | 1 | 9 | The 11n MAC also includes beamforming, antenna selection etc. which is not shown in Figure 9. Change Figure 9 to include them. | As proposed. | Disagree – the purpose of figure 9-1 is to show the different access methods of the MAC. Beamforming, antenna selection, etc. are not access methods and therefore do not need to appear in the diagram. The introductory sentence to the diagram begins with “The MAC architecture can be described as shown in Figure 9-1” – the implication is that the architecture can also be described in other ways, each of which depends on which aspects of the MAC are being emphasized in the diagram. In this case, the purpose of the diagram is to show access methods. |
| 2020 | Chu, Liwen | 247.00 | 63 | 11.18 | Here the draft says "A STA that is a member of a BSS that transmit a Management frame of Subtype Action, Category Public with a unicast value in the Address 1 field corresponding to a STA that is a member of the same BSS". This violates the definition of Public action frame. The definition of the Public action frame is "The Public action frame is defined to allow inter-BSS and AP to unasociated-STA communicaitons". | Change the definition of Public action frame accordingly, or delete this bullet. | Principle – TGn editor to change the modifications to 7.4.7.1 Public Action frame in TGn draft 9.0 by including editing instructions and modifications to the baseline’s first paragraph so that the first paragraph of the baseline will read as follows: “The Public Action frame is defined to allow inter-BSS and AP to unassociated-STA communications in addition to intra-BSS communication.” |
| 2025 | Banerjea, Raja | 115.00 | 41 | 9.2.7 | Text has been added to Draft 9.0 to clarify the BSSID value used in public action frames. In section 9.2.7 while describing the Broadcast and Multicast PDU transfer procedure it says "The broadcast/multicast message shall be distributed into the BSS, or to any BSS if the frame is a Management frame with a wildcard BSSID value in the BSSID field." This could lead to a public action frame storm where an AP1 receiving a public action frame will broadcast it to all BSS. AP2 on receiving the same public action frame will broadcast it to all BSS including AP1's BSS. AP1 will receive the public action frame it had originally transmitted. Also I am not sure why a public action frame should be re-broadcast by a receiving STA. The public action frame indicates the availability of a certain service provided by a STA or an AP-STA. Re-broadcasting the public action frame indicates that other BSSs which are simply re-broadcasting a received public action frame is providing this service. | On page 115, line 41, remove the section of the sentence "or to any BSS if the frame is a Management frame with a wildcard BSSID value in the BSSID field." | Principle - TGn editor shall replace the change shown in TGn draft 9.0 for the first paragraph of 9.2.7, not for the reason given by the commenter, but instead, because the case described in the paragraph cannot happen – the frame in question is one that has RA=AP’s UCAST address, TA=STA’s UCAST address and address3=DA, which is BCAST/MCAST. There is no place for a wildcard BSSID in this frame. The new change to this paragraph is to split the paragraph after the second sentence to avoid confusion of the two cases of a BCAST/MCAST frame. Only frames with ToDS=1 are redistributed by the AP. Whenever an AP transmits an MCAST/BCAST frame, it typically has FromDS=1, (or possibly 0) and ToDS=0 (except for the WDS case), so a receiving AP will NOT forward the frame. |
| 2026 | Banerjea, Raja | 123.00 | 53 | 9.6.0.e5.2 | Text has been added to Draft 9.0 to indicate that the immidiate BA in response to a BAR (non HT-PPDU) shall be sent using the same rate as the BAR. The improvement in throughput achieved is minimal. It further instructs the transmitter to transmit at a certain rate based on the received packet which is not desirable given that most wireless links have assymetric links due to the higher transmit power and different antenna configurations of the Access Point as compared to the stations. | On page 123, line 53, modify to " If a BlockAck frame is sent as an immediate response to a BlockAckReq frame that was carried in a non-HT PPDU and the BlockAck frame is carried in a non-HT PPDU, the STA may transmit the BlockAck frame at the same rate and modulation class as the BlockAckReq frame." | Disagree – The cited bullet item is for a specific case which is the existing baseline condition, and we do not want to modify it at the risk of making existing implementations possibly non-interoperable with new ones. |