IEEE P802.11
Wireless LANs

| TGn SB2 PHY Comments |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-04-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Vinko Erceg | Broadcom | 16340 W. Bernardo Dr. San Diego, CA 92127 | +1 858 521 5885 | verceg@broadcom.com |

Abstract

This submission suggests resolutions of SB2 PHY comments. The following CIDs are addressed: CID 2001, 2005, 2017, 2027, and 2028.

The changes marked in this document are based on TGn Draft 9.0

Revision History:

Rev 0: Initial version.

CID        Sec.            Pg.          Ln.             Comment                        Proposal                      Proposed Resolution

| 2001 |  |  |  | Include option for protocol-assisted switched diversity to enable single-stream handheld devices (e.g. phones) to use multiple antennas and concatenated spread-coded bursts to achieve reduced packet loss using simple receiver and transmitter archtectures. Handheld devices are more likely to experience fades during packets because of local movement. These devices will also be more challenged on power use and cost, mandating simpler processing architectures. | See November 2007 contribution regarding PASD. Include implementation language and capability bit to allow multiple bursts of same MSDU to be sent, but received using different antennas with intermediate storage of soft symbols between bursts separated by RIFs using the same space-time coding as 2x2 MIMO implementation, but with diversity switch action between 1st and 2nd burst. The bursts received using two switched antennas emulate reception of a single burst with reception at two simultaneous antennas. The PASD option is to be applied only to a single-stream mobile device, not to the AP (except that the AP provides additional packet redundancy to such devices). This allows the mobile device to use its two antennas more effectively than would be the case with simple switched diversity. The increased overhead that results from redundant transmission is compensated for by the reduction in repeat overhead to achieve transmission success, which uses more radio resource than PASD. The technique also reduces dropouts (timeouts) and latency for VoIP clients, which are inherently challenged by their streaming media focus, operation while users are moving, battery life limitations, and poorer antenna performance. Note: The PASD option may be invoked only when a device declares that it has the capability and that its PER has become unacceptable. | Disagree. Explanation: TGn specification mandates support of 2 spatial streams at an AP and 1 spatial stream at a STA. As a result, all TGn specification compliant APs must have at least two antennas for the reception/transmission. This means that STBC or Beamforming may be used during the transmission. Therefore, it is not necessary to mandate alternate diversity techniques as proposed in the comment that require buffering at the receiver, an additional antenna and a switch. In addition, the proposed technique may yield overhead since the same burst has to be transmitted twice when compared to the STBC transmissions (STBC transmissions at the two transmit AP antennas are simultaneous, no overhead). In the presentation 07/2796r0 no comparison was shown to alternate techniques such as STBC and Beamforming, and also overhead due to retransmissions was not accounted for in the results. The STBC and Beamforming techniques do not require an additional STA antenna, an antenna switch and burst buffering at STAs. To add PASD option on the top of STBC and Beamforming may not be trivial, and uplink/downlink transmissions may become unbalanced (possibly 4th order diversity on the downlink and 2 order diversity on the uplink direction) assuming 2 AP antennas and 2 switched diversity STA antennas employing PASD. |
| --- | --- | --- | --- | --- | --- | --- |

CID        Sec.            Pg.          Ln.             Comment                        Proposal                      Proposed Resolution

| 2005 |  |  |  | STBC modes should be mandatory, as they improve range and robustness | Remove TxSTBC from the HT Capabilities element, and change to "Reserved" the value 0 of RxSTBC in the HT Capabilities element. Change the statement regarding STBC in 20.1.1 (page 245 line 37) from being optional to mandatory. | Disagree. For the complexity reasons, STBC technique was defined as optional. Same was defined for other features like Beamforming and LDPC, for example. Some of the implementations may require only basic set of features because of the power, cost and chip area optimization reasons. |
| --- | --- | --- | --- | --- | --- | --- |

CID        Sec.            Pg.          Ln.             Comment                        Proposal                      Proposed Resolution

| 2017 | 9.21.2 | 192 | 41 | There is no need to limit MM NDPs to two or more spatial streams. I can see why it is needed for GF NDPs, simply because then the signal fields need to be processed very fast to be able to see that no symbols are following the signal fields. For MM NDPs with 1 spatial stream, the HT-STF and HT-LTF are following the signal fields so there is enough time there to decode and parse the signal fields. | Change "MCS shall indicate two or more spatial streams" to "When the TXVECTOR parameter FORMAT is set to HT_GF, MCS shall indicate two or more spatial streams". | Disagree. For an ease and consistency of implementation it is better to have same restriction for MM and GF packets. |
| --- | --- | --- | --- | --- | --- | --- |

CID        Sec.            Pg.          Ln.             Comment                        Proposal                      Proposed Resolution

| 2027 | 20.3.22.6 | 335 | 20 | RCPI specification indicates +/- 5dB accuracy. This extremely poor measurement accuracy is used in PHY clauses 15, 17, 18 and 19 in order to permit older (circa 2003) PHY chip implementations to provide a standardized power measurement. The +/- 5db accuracy specified in 2003 was based on the limited and antiquated capabilities of these older legacy PHY chips. Many older implementations only measured signal power during preamble acquisition. When frame power measurement is extended over much longer periods much more accuracy may be achieved. RCPI has been defined to measure power over the data portion of the received frame and over all the active receive chains; this greatly improves RCPI measurement accuracy. For newer PHYs, like 11n, with recent chip level implementations (2007 and later), a more reasonable and more useful accuracy specification is needed. Since most manufacturers already have first generation 11n chips on the market, this improved RCPI accuracy should be optional. | P335L20 change " range of the receiver." to " range of the receiver. RCPI measurement may be made with improved accuracy (see Annex A)." | Disagree. PICS is not intended for providing performance measures, it indicates conformance and should not be used otherwise. In addition, preamble and other fields of 2003 devices in many cases are very similar to 2007 and later devices. Averaging over multiple antennas may increase accuracy, but how to distinguish accuracy between single antenna and multiple antenna TGn devices? One may argue that receiver sensitivity of newer devices is better and therefore PICS should accommodate this also. How about better accuracy EVM, energy detect threshold, CCA, clock, temperature range, etc. options in the PICS? |
| --- | --- | --- | --- | --- | --- | --- |

CID        Sec.            Pg.          Ln.             Comment                        Proposal                      Proposed Resolution

| 2028 | Annex A | 399 | 59 | TGn must specify improved RCPI accuracy as an option. Optional accuracy is specified over a limited power range. | Insert 4 new rows in table as follows: HTP2.12.3 ¦ RCPI improved accuracy: +/-4dB in range -30dBm to -90dBm ¦ 20.3.22.6 ¦ CF16:O HTP2.12.4 ¦ RCPI improved accuracy: +/-3dB in range -30dBm to -90dBm ¦ 20.3.22.6 ¦ HTP2.12.3:O HTP2.12.5 ¦ RCPI improved accuracy: +/-2dB in range -30dBm to -90dBm ¦ 20.3.22.6 ¦ HTP2.12.4:O HTP2.12.6 ¦ RCPI improved accuracy: +/-1dB in range -30dBm to -90dBm ¦ 20.3.22.6 ¦ HTP2.12.5:O | Disagree. PICS is not intended for providing performance measures, it indicates conformance and should not be used otherwise. In addition, preamble and other fields of 2003 devices in many cases are very similar to 2007 and later devices. Averaging over multiple antennas may increase accuracy, but how to distinguish accuracy between single antenna and multiple antenna TGn devices? One may argue that receiver sensitivity of newer devices is better and therefore PICS should accommodate this also. How about better accuracy EVM, energy detect threshold, CCA, clock, temperature range, etc. options in the PICS? |
| --- | --- | --- | --- | --- | --- | --- |