IEEE P802.11
Wireless LANs

| TGp LB 144 Clause 11 Resolutions of Comments |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-04-07 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Justin McNew | Kapsch TrafficCom ¦ TechnoCom Mobility Solutions | 2035 Corte del Nogal, Suite 105, Carlsbad, CA 92011 | 760-438-5115 x 175 | justin.mcnew@kapsch.net |
| Francois Simon | US DOT | ARINC, Inc. 2551 Riva Road Annapolis, MD 21401 | 410-266-4531 | fsimon@arinc.com |
| Alastair Malarky | Mark IV Industries | 6030 Ambler Drive, Mississaugua, Ontario, Canada L4W 2P1 | 905-624-3020 | amalarky@ivhs.com |

Abstract

This submission proposes the resolutions to comments for Clause 11 (and one for Clause 4).

|  | LB144 Comment Resolution |
| --- | --- |

- COMMENT:

| CID | Commenter | Clause | Pg | Ln | Ty | Comment | Suggested Remedy | Comment resolution. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 179 | Stephenson, Dave | 11.19 | 17 | 3 | TR | The text should state when it is permitted for a STA to transmit a data frame (even if it is always permitted to do so) and when it is not permitted. | Clarify the text. | Resolution TBD. Can state data frames can be transmitted at any time to be explicitly clear. |
| 180 | Stephens, Adrian | 11.19 | 17 | 5 | TR | "the STA shall use only one EDCA parameter set for accessing the medium on the current channel." This is unnecessary. The EDCA should use the values in the MIB. This standard can define initial values, but there is no need to define how they are used or updated by the SME. | Remove this bullet. | Accepted as per Suggested Remedy |
| 181 | Perahia, Eldad | 11.19 | 17 | 11 | TR | "When dot11OCBEnabled is false a STA shall not transmit data frames outside the context of a BSS." This statement seems to conflict with concept of class 1 frames. | delete sentence | Counter, see CID 182 |
| 182 | Stephens, Adrian | 11.19 | 17 | 11 | TR | "When dot11OCBEnabled is false a STA shall not transmit data frames outside the context of a BSS." "Outside the context of a BSS" is not defined. Elsewhere it has been replaced with "when dot11OCBEnabled is true", which makes no sense here. | Remove the cited text - it specifies nothing. | Counter. In 5.2.11, the phrase “transmit data frames outside the context of a BSS" is a synonym for a STA that is not a member of a BSS transmitting a data frame. But will replace the text with that synonym. |
| 183 | Roy, Richard | 11.19 | 17 | 12 | TR | The ability to exchange data frames (and all other frames for that matter) is a potentially useful capability in many 802.11 WLAN deployments regardless of the state of other links currently in use. To date, no valid technical reason for prohibitiing the use of this very generic capability in all conditions has been tendered. In fact, a recent poll of knowledgeable members of the WG concluded that such a capability could successfully coexist with all other legacy 802.11 link states (cf. BSS, IBSS links). As written, the material in this subclause intends to prohibit the general use of this very useful functionality. It should be rewritten to allow coexistence with current legacy link states. | Rewrite the clause to permit simultaneous operation of BSS and non-BSS links. | Declined. |
| 184 | Stephens, Adrian | 11.19 | 17 | 12 | TR | "When a STA is a member of a BSS" - there is no normative definition of what it means to be "a member of a BSS" | Relate to normative behaviour, or remove the cited text. | Resolution TBD. Can change text to ‘after a STA has joined a BSS’. |
| 185 | Stephens, Adrian | 11.19 | 17 | 13 | TR | "shall be regarded". Ah, Jim me lad, I left my regarding irons behind - can I borrow yours? How do you test "regarding". What do you really mean in terms of testable normative behaviour? With Best Regards, A.N.Commenter | Specify something that can be tested, or remove the cited language. | Counter. Delete the text. The procedure for how to “regard” a STA when dot11OCBEnabled when isn’t defined is already described in 5.2.11. |
| 186 | Stephens, Adrian | 11.19 | 17 | 15 | TR | "The SME may change the parameters of the MAC and PHY sublayers by setting MIB attributes as well as by using the MLME-Reset.request primitive as described in clause 10.3.9." - true. And it was true before this normative statement was made. | Turn into an informative statement or remove. | Accepted. Remove the text. |
| 187 | Stephens, Adrian | 11.19 | 17 | 17 | TR | "Whenever MAC and PHY sublayer parameters are changed, MAC and PHY sublayer operation shall resume with the appropriate MIB attributes in less than 2 TU." This statement has two problems: 1. It is made, independent of OCBEnabled - i.e. applies to all future and past implementations of non-11p devices. i.e. it may make existing devices non-compliant. 2. It is too general. Which MIB variables are being changed? What does "shall resume" mean? This is way too lazy. If you need specific behaviour, specify it. | Limit to .11p devices. Specify which MIB variables are included in this. Specify what behaviour is implied by "shall resume". Or remove the cited text. | Counter: limit to dot11OCBEnabled set to TRUE. |
| 188 | Seok, Yongho | 11.19 | 17 | 20 | TR | In order transmit the data frame to peer STA, the STA shall know the capability information of the peer STA, e.g., supported rate set. To this end, a STA may send a requet for receiving Timing Advertisement frame having the capability information from peer STA. Add a request frame of Timing Advertisement frame. | Add a request frame of Timing Advertisement frame. | Resolution TBD. Frame could be added, but need feedback from task group. |
| 189 | Kenney, John | 11.20 | 17 | 22 | E | The word "to" is missing between "According" and "11.1.2". | Insert "to". | Accepted as per Suggested Remedy |
| 190 | Lauer, Joseph | 11.20 | 17 | 22 | E | First sentence is missing a "to" after "According". | Add "to" after "According". | Accepted as per Suggested Remedy |
| 191 | Roy, Richard | 11.20 | 17 | 22 | E | As written, the text is confusing. Thhe first sentence deals with the TSF timer, the following sentences with the timestamp field in a TA frame. It should be rewritten in two paragraphs, or the timestamp related stuff moved into the description in 11.20.1. | make one of the suggested changes. | Counter: the sentence provides an introduction to the mention of the TSF timer in following sentences. |
| 192 | Malarky, Alastair | 11.20 | 17 | 26 | T | The description doesn't identify Local Time which is also critical to the estimate. | Add text to indicate Local Time is the value of a station's TSF timer at the start of reception of the first octet of the timestamp field of the received Timing Advertisement frame. | Accepted as per Suggested Remedy |
| 193 | Cypher, David | 11.20.1 | 17 | 28 | TR | Marking the clause heading as informative is not following IEEE Standards Style Manual 10.1, page 11; "Interspersed normative and informative text is not allowed. As such, neither clauses nor subclauses shall be labeled as informative. Identification of normative or informative text shall be reviewed during the ballot of a document; therefore, it is important that the working group consult an IEEE Standards project editor early with any questions." http://standards.ieee.org/guides/style/ | Delete entire clause 11.20.1 | Declined, see CID 194. |
| 194 | Cypher, David | 11.20.1 | 17 | 28 | TR | Having an addition level subclause heading without a second is not following IEEE Standards Style Manual 11.1 page 18; "Clauses and subclauses should be divided into further subclauses only when there is to be more than one subclause. In other words, clauses and subclauses should not be broken down into further subclauses if another subclause of the same level does not exist. For example, Clause 1 should not have a subclause 1.1 unless there is also a subclause 1.2." http://standards.ieee.org/guides/style/ | Delete clause 11.20.1 heading, just make it part of 11.20 | Accepted as per Suggested Remedy |
| 195 | Kenney, John | 11.20.1 | 17 | 31 | E | (pp 31-50 ) This very useful section will be more helpful if it is made more clear that including the Timing IE is optional. The current text includes the word "optionally" in the last sentence of the 2nd paragraph, in the context of the MLME-TimingAdvertisement.indication primitive. However, the Timing IE information is not actually optional in that primitive. If the received Timing Advertisement frame included the Timing IE then the MLME must report it to the SME in the indication primitive (I think). However, it would be helpful to insert the word "optional" once in each paragraph as a reminder to the reader that while the timestamp is not optional the Timing IE is. | In the first sentence of the first paragraph of this subclause, insert "optional" between "including the" and "Timing information element." In the third sentence of the second paragraph (starts "In order for …") change "and includes the Timing information element" to ", and this primtive optionally includes the Timing information element." Finally, in the last sentence of the second paragraph omit the word "optionally" and add "(if included in the frame)" at the end of the sentence. | Accepted in principle |
| 196 | Malarky, Alastair | 11.20.1 | 17 | 34 | E | Move "(using the TTOE field)" to after the word "difference" in the previous line. | As per comment | Accepted as per Suggested Remedy |
| 197 | Malarky, Alastair | 11.20.1 | 17 | 38 | T | The description doesn't identify Local Time which is also critical to the estimate. | Change the last two sentences to read "When a Timing Advertisement frame is received by a STA its MLME reports the transmitted Timestamp and Timing information element along with the Local Time, the receiving STA TSF Timer value when the frame was received, to the SME. The receiving STA's SME can use the Local Time, Timestamp and Timing information element to align its estimate of the time standard to the transmitting STA's estimate of the corresponding time standard." | Accepted as per Suggested Remedy |
| 198 | Stephenson, Dave | 11.19 | 17 | 42 | E | The first sentence is informative and adds no value. Rather the text should describe how to use these new MLME primitives. | Clarify the text. | Counter: delete the first sentence. How to use the primitives is an implementation issue. |
| 199 | Malarky, Alastair | 11.20.1 | 17 | 43 | E | There is no need for the parentheses on the sentence. | Delete the parentheses | Counter: delete the first sentence. |
| 200 | Stephenson, Dave | 11.19 | 17 | 44 | T | The term "TTOE" should be added to clause 4. | Add the text. | Accepted as per Suggested Remedy |
| 201 | Stephenson, Dave | 11.19 | 17 | 45 | T | The term "reasonably bounded" is imprecise and can mean different things to different readers. | Define the required value of "reasonably bounded" along with measurement accuracy. | Counter: delete the first sentence. |
| 202 | Malarky, Alastair | 11.20.1 | 17 | 49 | E | Timestamp should be initial caps | As per comment | Accepted as per Suggested Remedy |
| 203 | Cypher, David | 11.20.1 | 17 | 50 | TR | If 11.20.1 is not otherwise removed, the use of optionally here is the only place that states that this element is optional for MLME-TIMING_ADVERTISEMENT. Is it optional or is it mandatory? | If 11.20.1 is not otherwise removed, make it clear in the normative text that the Timing information element is optional or mandatory in the MLME-TIMING_ADVERTISEMENT.indication. Table 7-18a states that the timing is optional in the frame, but states nothing about the primitive. The primitive defines it (at least lists it). The final determination of it presence for the primitive is the received frame that causes the primitive's generation. | Accepted in principle. See CID 171 |
| 207 | Fischer, Matthew | 11.26 | 21 | 24 | E | Missing preposition and comma | change "according 11.1.2" to "according to 11.1.2," | Accepted as per Suggested Remedy It assumed that the commenter meant 11.20, P 17, L22 |
| 213 | Stephens, Adrian | 11.22.1 | 26 | 42 | ER | The style guide does not allow the labelling of subclauses as "(informative)". | Remove this labelling. Or move to an informative annex. | Counter: see CID 194. |

- Background

This submission proposes the resolutions to comments for Clause 11 (and one for Clause 4).

- Recommended Resolution of the Comments:

Editor to add the following to the appropriate position in Clause 4 (CID 200):

TTOE		TSF timer offset estimate

Editor to make changes (deletions, if any, in yellow background; insertions, if any, in green background) as follows (CID 180, 181, 182, 185, 186, 187):

11. MLME

Insert the following new subclauses, 11.19 and 11.20, after the last subclause in 11 inserting the

appropriate subclause number:

11.19 STAs communicating data frames outside the context of a BSS

When dot11OCBEnabled is true in a STA:

- synchronization, authentication, and association as defined in Clause 11.1 and Clause 11.3 do not apply,

- if the STA transmits a data frame, the STA shall set the BSSID field of the data frame to the wildcard BSSID value, and

- the STA shall use only one EDCA parameter set for accessing the medium on the current channel. The SME of the STA may initially configure the PHY with parameters (frequency, channel width, etc.) known in advance via mechanisms outside the scope of this standard (see 5.2.11). The EDCA parameter set is either the default EDCA parameter set specified in Table 7-37a or the EDCA parameter set that the SME sets in dot11EDCATable according to these mechanisms.

When dot11OCBEnabled is falseFALSE a STA that is not a member of a BSS shall not transmit data frames outside the context of a BSS. When a STA is a member of a BSS or is the point coordinator within a BSS, dot11OCBEnabled shall be set to falseFALSE. If a STA does not have dot11OCBEnabled defined, the attribute shall be regarded as false.

The SME may change the parameters of the MAC and PHY sublayers by setting MIB attributes as well as by using the MLME-Reset.request primitive as described in clause 10.3.9. Whenever MAC and PHY sublayer parameters are changed in a STA with dot11OCBEnabled set to TRUE, MAC and PHY sublayer operation shall resume with the appropriate MIB attributes in less than 2 TU.

Editor to make changes (deletions, if any, in yellow background; insertions, if any, in green background) as follows (CID 189, 190, 191, 192, 193, 194, 195, 196, 197, 198, 199, 201, 202, 203, 207, 213):

11.20 Time Advertisement

According to 11.1.2, each STA maintains a TSF Timer. The Timestamp field is included in the Timing Advertisement frame, and when a STA transmits the Timing Advertisement frame the Timestamp shall be set to the value of the STA's TSF timer at the time that the data symbol containing the first bit of the Timestamp is transmitted to the PHY plus the transmitting STA's delays through its local PHY from the MAC-PHY interface to its interface with the WM (e.g., antenna, light emitting diode (LED) emission surface).  For a STA receiving a Timing Advertisement frame, Local Time is the value of the STA's TSF timer at the start of reception of the first octet of the Timestamp field of the frame.

11.20.1 Advertising a time standard using the TSF Timer, Timing Advertisement frame, and

Timing information element (informative)

A STA can advertise a time standard when transmitting the Timing Advertisement frame by including the optional Timing information element. As defined in clause 7.3.2.80 the Timing information element contains an estimate of the difference (using the TTOE field) between a time standard, which can be derived from an external time source, and the Timestamp included in the frame (using the TTOE field) as well as the standard deviation of the error in that estimate (using the TTOE-STD field). A STA with an external time source can implement an estimator in a variety of ways, which are beyond the scope of this standard. The SME provides the Timing information element to the MLME when it requests the MLME to send a Timing Advertisement frame. When a Timing Advertisement frame is received by a STA its MLME reports the Timestamp and Timing information element to the SME. The receiving STA's SME can use the Timestamp and Timing information element to align its estimate of the time standard to the transmitting STA's estimate of the corresponding time standard.

In order to support this Timing Advertisement frame and Timing information element, the MLME-GETTSFTIME.request and MLME-GETTSFTIME.confirm primitives are included in clause 10. (In order to accurately estimate the TTOE the MLME must be able to provide the current value of its TSF timer to the SME in a known or reasonably bounded time between the instant the SME requests the current value of the TSF timer and the instant the MLME reports it.) In order for the SME to request the transmission of a Timing Advertisement frame the MLME-TIMING_ADVERTISEMENT.request primitive is also included in clause 10 and includes the Timing information element . Finally, the MLME-TIMING_ADVERTISEMENT.indication primitive is included in clause 10 for reception of the frame and includes both the timestamp and optionally the Timing information element. When a Timing Advertisement frame is received by a STA its MLME reports the transmitted Timestamp and Timing information element (if included in the frame) along with the Local Time to the SME. The receiving STA's SME can use the Local Time, Timestamp and optionally the Timing information element to align its estimate of the time standard to the transmitting STA's estimate of the corresponding time standard.

- Motion (if technical and/or significant):

(And instructions to the editor.)

Move to accept the Recommended Resolutions to these comments and the Recommended changes to P802.11p D6.0 noted above and instruct the editor to make these changes to P802.11p D6.01.

- Motion by: ___Justin McNew_________________Date: ___________

- Second:  ______________________

| Approve: | Disapprove: | Abstain: |
| --- | --- | --- |

- References: