IEEE P802.11
Wireless LANs

| LB146 STA Statistics Comment Resolutions Normative Text |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-May-01 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Ganesh Venkatesan | Intel Corporation | 2111NE 25th Ave Hillsboro, OR 97124 | 503-264-0637 | ganesh.venkatesan@intel.com |

- Abstract

This document contains normative text changes to resolve comment IDs 11, 12 and 43.

The submission is referenced to draft D5.01 and augments to all editor instructions as well as the actual base draft modifications.

All new text relative to the draft D4.0 is identified in blue color and underlined and all deleted text is identified by strikethrough.

# 7. Frame formats

7.3.2.21.8 STA Statistics Request

Change the paragraph starting with “The Peer MAC Address field is the RA or TA...” as follows:

The Peer MAC Address field is the RA or TA MAC address for the frame statistics of this measurement. If the Peer MAC Address field is the wildcard address, then the value of the STA counters reported in the STA Statistics Report is the sum of the respective values for every peer of the reporting STA. For other MAC addresses in the Peer MAC Address field, the value of STA counters reported in the STA Statistics Report corresponds to frames where the RA or the TA matches the MAC address in the Peer MAC Address field.

Change Table 7-29c in 7.3.2.21.8 as shown below:

| Table 7-29c—Group Identity for a STA Statistics Request |  |
| --- | --- |
| Statistics Group Name | Group Identity |
| RSNA Counters | 16 |
| Reserved | 1617 – 255 |

EDITORIAL NOTE—Table 7-29c is defined in IEEE 802.11k-2008.

Change the paragraph starting with “The Measurement Duration field...” as follows:

The Measurement Duration field is set to the duration of the requested measurement in TUs except when triggered reporting is used. When triggered reporting is used, the measurement duration is set to 0.

Change Table 7-29k as follows:

| Table 7-29k—Optional Subelement IDs for STA Statistics Request |  |  |  |
| --- | --- | --- | --- |
| Subelement ID | Name | Length field (octets) | Extensible |
| 0 | Reserved |  |  |
| 1 | Triggered Reporting | variable |  |
| 02-220 | Reserved |  |  |
| 221 | Vendor Specific | 1 to 239 |  |
| 222-255 | Reserved |  |  |

Insert the following before the paragraph that reads "The Vendor Specific subelements  ":

The Triggered Reporting subelement is used to specify trigger conditions and thresholds for triggered STA Statistics measurements. It is present when setting up triggered reporting from STA Counters, QoS STA Counters, or RSNA Counters, see 11.10.8.5.

The format of the Triggered Reporting subelement for STA Counters is shown in Figure 7-62i1. The fields marked as optional are the fields that are only present if the appropriate bit in the STA Counter Trigger Condition field is set to 1.

|  | Measurement Count | Trigger Timeout | STA Counter Trigger Condition | dot11FailedCount Threshold (optional) | dot11FCS ErrorCount Threshold (optional) |
| --- | --- | --- | --- | --- | --- |
| Octets: | 4 | 2 | 2 | 0 or 4 | 0 or 4 |
|  | dot11Multiple RetryCount Threshold (optional) | dot11Frame DuplicateCount Threshold (optional) | dot11RTS FailureCount Threshold (optional) | dot11ACK FailureCount Threshold (optional) | dot11RetryCount Threshold (optional) |
| Octets: | 0 or 4 | 0 or 4 | 0 or 4 | 0 or 4 | 0 or 4 |
| Figure 7-62i1—Triggered Reporting subelement for STA Counters |  |  |  |  |  |

EDITORIAL NOTE—Figure 7-62i is defined in IEEE 802.11k-2008.

The value in the Measurement Count field specifies the number of MSDUs or MPDUs transmitted and/or received by the STA, which are to be used to determine if one or more of the trigger conditions have been met.

The Trigger Timeout field contains a value in units of 100 TUs during which a measuring STA does not generate further triggered STA Statistics Reports after a trigger condition has been met.

The STA Counter Trigger Condition field specifies trigger values used when requesting triggered STA Statistics reporting. The format of the STA Counter Trigger Condition field is shown in Figure 7-62i2.

|  | B0 | B1 | B2 | B3 |
| --- | --- | --- | --- | --- |
|  | dot11FailedCount | dot11FCSErrorCount | dot11Multiple RetryCount | dot11Frame DuplicateCount |
| Bits | 1 | 1 | 1 | 1 |
|  | B4 | B5 | B6 | B7-B15 |
|  | dot11RTS FailureCount | dot11ACK FailureCount | dot11RetryCount | Reserved |
| Bits | 1 | 1 | 1 | 9 |
| Figure 7-62i2—STA Counter Trigger Condition field |  |  |  |  |

For each bit in the STA Counter Trigger Condition field that is set to 1, a corresponding threshold value (defined in Figure 7-62i1) in the Triggered Reporting subelement for STA Counters. With this, the STA Statistics Request indicates that a STA Statistics Report be generated when the corresponding STA counter defined in Figure 7-68j and Figure 7-68j (in 7.3.2.22.8) exceeds the value of the the specfied threshold, within the total number of MSDUs or MPDUs indicated in the Measurement Count field.  See 11.10.8.5. One or more trigger conditions are set with specified thresholds. In the triggered STA Statistics request, the value of the Group Identity field is set to either 0 or 1. When the Group Identity field value of the triggered STA Statistics request is set to 0, B2 – B6 in the STA Counter Trigger Condition field are set to 0. When the group identity of the triggered STA Statistics request is set to 1, B0 and B1 in the STA Counter Trigger Condition field are set to 0.

The format of the Triggered Reporting subelement for QoS STA Counters is shown in Figure 7-62i3. The fields marked as optional are the fields that are only present if the appropriate bit in the QoS STA Counter Trigger Condition is set to 1.

|  | Measurement Count | Trigger Timeout | QoS STA Counter Trigger Condition | dot11QoSFailedCount Threshold (optional) | dot11QoSRetryCount Threshold (optional) |
| --- | --- | --- | --- | --- | --- |
| Octets: | 4 | 2 | 2 | 0 or 4 | 0 or 4 |
|  | dot11QoSMultiple RetryCount Threshold (optional) | dot11QoSFrame DuplicateCount Threshold (optional) | dot11QoSRTSCount Failure Threshold (optional) | dot11QoSACK FailureCount Threshold (optional) | dot11QoSDiscardedCount Threshold (optional) |
| Octets: | 0 or 4 | 0 or 4 | 0 or 4 | 0 or 4 | 0 or 4 |
| Figure 7-62i3—Triggered Reporting subelement for QoS STA Counters |  |  |  |  |  |

The UP of the QoS STA Counters for triggered QoS Statistics measurement is determined by the group identity of the measurement request field for a STA Statistics Request frame as defined in Figure 7-29j.

The value in the Measurement Count field specifies the number of MSDUs or MPDUs transmitted and/or received by the STA, which are to be used to determine if one or more of the trigger conditions have been met.

The Trigger Timeout field contains a value in units of 100 TUs during which a measuring STA does not generate further triggered STA Statistics Reports after a trigger condition has been met.

The QoS STA Counter Trigger Condition field specifies reporting triggers when requesting triggered STA Statistics reporting. The format of the QoS STA Counter Trigger Condition field is shown in Figure 7-62i4.

|  | B0 | B1 | B2 | B3 |
| --- | --- | --- | --- | --- |
|  | dot11QoS FailedCount | dot11QoS RetryCount | dot11QoSMultiple RetryCount | dot11QoSFrame DuplicateCount |
| Bits: | 1 | 1 | 1 | 1 |
|  | B4 | B5 | B6 | B7-B15 |
|  | dot11QoSRTS FailureCount | dot11QoSACK FailureCount | dot11QoS DiscardedCount | Reserved |
| Bits: | 1 | 1 | 1 | 9 |

Figure 7-62i4—QoS STA Counter Trigger Condition field

For each bit in the QoS STA Counter Trigger Condition field that is set to 1, a corresponding threshold value (defined in Figure 7-62i3) in the Triggered Reporting subelement for QoS STA Counters. With this, the STA Statistics Request indicates that a STA Statistics Report be generated when the corresponding QoS STA counter defined in Figure 7-85j (in 7.3.2.22.8) exceeds the value of the the specfied threshold, within the total number of MSDUs or MPDUs indicated in the Measurement Count field.. See 11.10.8.5. One or more trigger conditions are set with specified thresholds.

The format of the Triggered Reporting subelement for RSNA Counters is shown in Figure 7-62i5. The fields marked as optional are the fields that are only present if the appropriate bit in the RSNA Trigger Condition is set to 1.

|  | Measurement Count | Trigger Timeout | RSNA Counter Trigger Condition | dot11RSNAStats CMACICVErrors Threshold (optional) | dot11RSNA StatsCMACReplays Threshold (optional) |
| --- | --- | --- | --- | --- | --- |
| Octets: | 4 | 2 | 2 | 0 or 4 | 0 or 4 |
|  | dot11RSNA StatsRobustMgmt CCMPReplays Threshold (optional) | dot11RSNAStats TKIPICVErrors Threshold (optional) | dot11RSNAStats TKIPReplays Threshold (optional) | dot11RSNAStats CCMPDecryptErrors Threshold (optional) | dot11RSNAStats CCMPReplays Threshold (optional) |
| Octets: | 0 or 4 | 0 or 4 | 0 or 4 | 0 or 4 | 0 or 4 |
| Figure 7-62i5—Triggered Reporting subelement for RSNA Counters |  |  |  |  |  |

The value in the Measurement Count field specifies the number of MPDUs transmitted and/or received by the STA, which are to be used to determine if one or more of the trigger conditions have been met.

The Trigger Timeout field contains a value in units of 100 TUs during which a measuring STA does not generate further triggered STA Statistics Reports after a trigger condition has been met.

The RSNA Counter Trigger Condition field specifies reporting triggers when requesting triggered STA Statistics reporting. The format of the RSNA Trigger Condition field is shown in Figure 7-62i6.

|  | B0 | B1 | B2 | B3 |
| --- | --- | --- | --- | --- |
|  | dot11RSNAStats CMACICVErrors | dot11RSNA StatsCMACReplays | Dot11RSNAStats RobustMgmt CCMPReplays | dot11RSNAStats TKIPICVErrors |
| Bits: | 1 | 1 | 1 | 1 |
|  | B4 | B5 | B6 | B7-B15 |
|  | dot11RSNAStats TKIPReplays | dot11RSNAStats CCMPDecryptErrors | dot11RSNAStats CCMPReplays | Reserved |
| Bits: | 1 | 1 | 1 | 9 |
| Figure 7-62i6—RSNA Trigger Condition field |  |  |  |  |

For each bit in the RSNA Trigger Condition field that is set to 1, a corresponding threshold value (defined in Figure 7-62i5) in the Triggered Reporting subelement for RSNA Counters. With this, the STA Statistics Request indicates that a STA Statistics Report be generated when the corresponding RSNA counter defined in Figure 7-68l1 (in 7.3.2.22.8) exceeds the value of the the specfied threshold, within the total number of MPDUs indicated in the Measurement Count field.  See 11.10.8.5. One or more trigger conditions are set with specified thresholds.

7.3.2.22 Measurement Report element

7.3.2.22.8 STA Statistics Report

Change the paragraph starting with “The Measurement Duration field...” as follows:

The Measurement Duration is set to the duration over which the change in Statistics Group Data was measured and reported, expressed in units of TUs. A Measurement Duration value of 0 indicates a report of the current values of the Statistics Group Data.

Insert a new row and change the None row accordingly in Table 7-31f:

| Table 7-31f—Group Identity for a STA Statistics Report |  |  |
| --- | --- | --- |
| Group Identity Requested | Statistics Group Data field length (octets) | Statistics Returned |
| 16 | 28 | dot11RSNAStatsCMACICVErrors (Counter32) dot11RSNAStatsCMACReplays (Counter32) dot11RSNAStatsRobustMgmtCCMPReplays(Counter32) dot11RSNAStatsTKIPICVErrors (counter32) dot11RSNAStatsTKIPReplays (counter32) dot11RSNAStatsCCMPDecryptErrors (counter32) dot11RSNAStatsCCMPReplays (counter32 |
| 1716– 255 | Reserved | Reserved |

Change Table 7-31g of IEEE 802.11k-2008 as follows:

| Table 7-31g—Optional Subelement IDs for STA Statistics Report |  |  |  |
| --- | --- | --- | --- |
| Subelement ID | Name | Length field (octets) | Extensible |
| 0 | Reserved |  |  |
| 1 | Reporting Reason | variable |  |
| 02-220 | Reserved |  |  |
| 221 | Vendor Specific | 1 to 239 |  |
| 222-255 | Reserved |  |  |

Insert the following before the paragraph that reads "The Vendor Specific subelements  ":

The format of the Measurement Report field for RSNA Counters Group is shown in Figure 7-68l1.

|  | dot11RSNAStats CMACICVErrors | dot11RSNAStatsCMACReplays | dot11RSNAStats RobustMgmt CCMPReplays | dot11RSNAStats TKIPICVErrors |
| --- | --- | --- | --- | --- |
| Octets: | 4 | 4 | 4 | 4 |
|  | dot11RSNAStats TKIPReplays | dot11RSNAStats CCMPDecryptErrors | dot11RSNAStats CCMPReplays |  |
| Octets | 4 | 4 | 4 |  |
| Figure 7-68l1—Measurement Report field format for RSNA Counters Group |  |  |  |  |

EDITORIAL NOTE—Figure 7-68l is defined in IEEE 802.11k-2008.

The Reporting Reason subelement indicates the reason why the measuring STA sent the STA Statistics report. It is present if Statistics Group Name is from STA Counters, QoS STA Counters, or RSNA Counters (see 11.10.8.5).

The Reporting Reason subelement for STA Statistics Group Identities 0 or 1 (STA Counters) is shown in Figure 7-68l2.

|  | B0 | B1 | B2 | B3 |
| --- | --- | --- | --- | --- |
|  | dot11Failed | dot11FCS Error | dot11Multiple Retry | dot11Frame Duplicate |
| Bits | 1 | 1 | 1 | 1 |
|  | B4 | B5 | B6 | B7 |
|  | dot11RTS Failure | dot11ACK Failure | dot11Retry | Reserved |
| Bits | 1 | 1 | 1 | 1 |
| Figure 7-68l2—Reporting Reason subelement for STA Counters |  |  |  |  |

The Reporting Reason subelement for STA Statistics Group Identity 2 through 9 (QoS STA Counters) is shown in Figure 7-62i3.

|  | B0 | B1 | B2 | B3 |
| --- | --- | --- | --- | --- |
|  | dot11QoS Failed | Dot11QoS Retry | dot11QoSMultiple Retry | dot11QoSFrame Duplicate |
| Bits | 1 | 1 | 1 | 1 |
|  | B4 | B5 | B6 | B7 |
|  | dot11QoSRTS Failure | dot11QoSACK Failure | dot11QoS Discarded | Reserved |
| Bits | 1 | 1 | 1 | 1 |
| Figure 7-68l3—Reporting Reason subelement for QoS STA Counters |  |  |  |  |

The Reporting Reason subelement for STA Statistics Group Identity 16 (RSNA Counters) is shown in Figure 7-68l4.

|  | B0 | B1 | B2 | B3 |
| --- | --- | --- | --- | --- |
|  | dot11RSNAStats CMACICVErrors | dot11RSNA StatsCMACReplays | dot11RSNAStats RobustMgmt CCMPReplays | dot11RSNAStats TKIPICVErrors |
| Bits: | 1 | 1 | 1 | 1 |
|  | B4 | B5 | B6 | B7 |
|  | dot11RSNAStats TKIPReplays | dot11RSNAStats TKIPReplays | dot11RSNAStats CCMPReplays | Reserved |
| Bits: | 1 | 1 | 1 | 1 |
| Figure 7-68l4—Reporting Reason subelement for RSNA Counters |  |  |  |  |

In a non-triggered STA Statistics Report, all fields in the Reporting Reason subelement are set to 0.

11.10.8 Specific measurement usage

11.10.8.5 STA statistics report

Insert the following at the end of 11.10.8.5:

A STA may request that a STA Statistics report be sent when statistics of interest reach a threshold as defined in the Measurement Request element of the STA Statistics Request frame (see 7.3.2.21.8). This is termed a triggered STA Statistics measurement. Implementation of Triggered STA Statistics Reporting is optional for a WNM STA. A STA that implements Triggered STA Statistics Reporting has the MIB attribute dot11MgmtOptionTriggerSTAStatisticsImplemented set to true. When dot11MgmtOptionTriggerSTAStatisticsImplemented is true, dot11WirelessManagementImplemented shall be true.

A triggered STA Statistic measurement shall be requested by setting the Enable and Report bits to 1 within a Measurement Request element containing the STA Statistics Measurement Type. The Measurement Request field shall contain a STA Statistics Request with the trigger conditions specified in the Triggered Reporting subelement, as defined in 7.3.2.21.8. One or more trigger conditions may be set with specified thresholds. See 11.10.7. To prevent generation of too many triggered reports, the value of the Trigger Timeout field shall be set to a value greater or equal to the value of dot11MinTriggerTimeout. If the value of the Trigger Timeout field is less than the value of dot11MinTriggerTimout, the STA shall reject the measurement request and respond with a report where the Measurement Report Mode field is set to "Incapable".

A STA accepting a triggered STA Statistics measurement shall measure the requested statistics. If a trigger condition occurs, the measuring STA shall send a STA Statistics measurement report to the requesting STA. The measuring STA shall not send further triggered STA Statistics reports for that trigger condition to the requesting STA until the Trigger Timeout period specified in the request frame has expired, or new trigger conditions have been requested. The STA Statistics measurement report is defined in 7.3.2.22.8. If the number of MPDUs or MSDUs indicated in the Measurement Count field are transmitted or received without any of the counted statistics meeting the corresponding trigger threshold then the measuring STA shall reset all counted statistics accumulated in the current measuring window and shall restart measuring for another measurement count window.

If a STA receives a STA Statistics measurement request from the same STA for which a triggered STA Statistics measurement is in progress, triggered measurement shall be terminated and the counted statistics shall be reset.

STA Statistics reported in a triggered STA Statistics report shall be the values accumulated over the number of transmitted or received MSDUs or MPDUs before the trigger condition is met. Measurement duration shall not be specified when requesting a triggered STA statistics measurement and the Measurement Duration field in the corresponding Measurement Request shall be set to 0.

All triggered STA Statistics measurements shall be terminated at a measuring STA by receiving a STA Statistics measurement request with the Enable bit set to 1 and the Report bit set to 0. A STA requesting a triggered STA Statistics measurement may update the trigger conditions by sending a STA Statistics measurement request specifying the new trigger conditions.

Once accepted by a measuring STA, a triggered STA Statistics measurement continues to be active until the measurement request is superseded by a STA Statistics measurement request from the requesting STA or the measuring STA disassociates or reassociates.