IEEE P802.11
Wireless LANs

| Proposed Normative Text for CID 99 and CID 100 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-05-05 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Emily Qi | Intel Corporation | 2111 NE 25th Ave, Hillsboro OR | 503-264-7799 | Emily.h.qi@intel.com |

Abstract

This document contains proposed edits to TGv draft 5.01 for CID 99 and CID100.

7.2.3.8 Probe Request frame format

Insert order 10 and 11 information field into Table 7-14:

| Table 7-14—Probe Request frame body |  |  |
| --- | --- | --- |
| Order | Information | Notes |
| 10 | SSID List | The SSID List element is optionally present if dot11MgmtOptionSSIDListEnabled is set to true. |
| 11 | Channel Usage | The Channel Usage element is optionally present if dot11MgmtOptionChannelUsageEnabled is true. |

7.2.3.9 Probe Response frame format

Change Table 7-15 as follows:

| Table 7-15—Probe Response frame body |  |  |
| --- | --- | --- |
| Order | Information | Notes |
| 24 | Multiple BSSID | One or more Multiple BSSID elements are present Included if dot11RRMMeasurementPilotCapability is set to a value between 2 and 7 and the AP is a member of a Multiple BSSID Set (see 11.10.11) with two or more members, or if dot11MgmtOptionMultiBSSIDEnabled is set to true. |
| 40 | QoS Traffic Capability | The QoS Traffic Capability element is optionally present if dot11MgmtOptionACStationCountEnabled is set to true. |
| 41 | Channel Usage | The Channel Usage element is present if the Channel Usage element is present in the Probe Request frame and dot11MgmtOptionChannelUsageEnabled is true. |

7.3.2.81 Channel Usage element

The Channel Usage element defines the channel usage information for non-infrastructure networks. The format of the Channel Usage element is shown in Figure v74.

|  |  |  |  | zero or more entries |
| --- | --- | --- | --- | --- |
|  | Element ID | Length (2xn+1) | Usage Mode | Channel Entry |
| Octets: | 1 | 1 | 1 | 2xn |
| Figure v74—Channel Usage element format |  |  |  |  |

The Element ID field is equal to the Channel Usage value in Table 7-26.

The Length field is set to one plus twice the number of Regulatory Class/Channel Number pairs.

The Usage Mode field is set to a number that identifies the usage of the recommended channels listed in the Regulatory Class/Channel Number pair fields. The Usage Mode definitions are shown in Table v37.

| Table v37—Usage Mode definitions |  |
| --- | --- |
| Value | Usage Mode |
| 0 | Reserved |
| 1 | Non-infrastructure IEEE 802.11 network |
| 2-255 | Reserved |

The Channel Entry field includes zero or more Regulatory Class and Channel pairs. The format of Channel Entry field is shown in Figure v47.

The Regulatory Class/Channel pair is not present if the Channel Usage element is included in the Probe Request frame and Channel Usage Request frame.

The Channel Usage element may be included in Probe Request frames, as described in 7.2.3.8, Probe Response frames, as described in 7.2.3.9, Channel Usage Request frames, as described in 7.4.11.27, and Channel Usage Response frames, as described in 7.4..11.28. The use of the Channel Usage element and frames is described in 11.20.14.

7.4.7.1 Public Action frames

Insert the following rows to Table 7-57e:

| Table 7-57e— Public Action field values |  |
| --- | --- |
| Action field value | Description |
| <ANA> | Location Track Notification |
| <ANA>+1-255 | Reserved |

7.4.7.

7.4.11.1 WNM Action fields

Several Action frame formats are defined for Wireless Network Management (WNM) purposes. An Action field, in the octet field immediately after the Category field, differentiates the formats. The Action field values associated with each frame format are defined in Table v39.

| Table v39—WNM Action field values |  |
| --- | --- |
| Action field value | Description |
| 0 | Event Request |
| 1 | Event Report |
| 2 | Diagnostic Request |
| 3 | Diagnostic Report |
| 4 | Location Configuration Request |
| 5 | Location Configuration Response |
| 6 | BSS Transition Management Query |
| 7 | BSS Transition Management Request |
| 8 | BSS Transition Management Response |
| 9 | FMS Request |
| 10 | FMS Response |
| 11 | Collocated Interference Request |
| 12 | Collocated Interference Response |
| 13 | TFS Request |
| 14 | TFS Response |
| 15 | TFS Notify |
| 16 | WNM-Sleep Mode Request |
| 17 | WNM-Sleep Mode Response |
| 18 | TIM Broadcast Request |
| 19 | TIM Broadcast Response |
| 20 | TIM |
| 21 | QoS Traffic Capability Update |
| 22 | Timing Measurement |
| 23 | DMS Request |
| 24 | DMS Response |
| 25 | Channel Usage Request |
| 26 | Channel Usage Response |
| 27-255 | Reserved |

7.4.11.27 Channel Usage Request frame format

The Channel Usage Request frame is sent by a non-AP STA to the AP to request the specified Channel Usage information. The format of the Channel Usage Request frame is defined in Figure v76.

|  | Category | Action | Dialog Token | Channel Usage Elements | Supported Regulatory Classes Element |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | variable | variable |
| Figure v76—Channel Usage Request frame format |  |  |  |  |  |

The Category field is set to the value indicating the WNM category, as specified in 7.3.1.11.

The Action field is set to the value indicating Channel Usage Request frame, as specified in 7.4.11.1.

The Dialog Token field is set to a non-zero value chosen by the non-AP STA sending the Channel Usage Request frame to identify the request/response transaction.

The Channel Usage Element field includes one or more Channel Usage elements described in 7.3.2.81 to identify the request Usage Mode.

The Supported Regulatory Classes Element field includes a Supported Regulatory Classes element to indicate the supported regulatory classes for the requested network type, consistent with the Country element advertised by the AP. The Supported Regulatory Classes is described in 7.3.2.54.

EDITORIAL NOTE—7.3.2.56 is defined in IEEE 802.11y-2008.

7.4.11.28 Channel Usage Response frame format

The Channel Usage Response frame is sent by an AP STA in response to a Channel Usage Request frame, or autonomously. The format of the Channel Usage Response frame is shown in Figure v77.

|  | Category | Action | Dialog Token | Channel Usage Elements | Country String | Power Constraint Element (Optional) | EDCA Parameter Set Element (optional) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | variable | 3 | 0 or 3 | 0 or 20 |
| Figure v77—Channel Usage Response frame format |  |  |  |  |  |  |  |

The Category field is set to the value indicating the WNM category, as specified in 7.3.1.11.

The Action field is set to the value indicating Channel Usage Response frame, as specified in 7.4.11.1.

The Dialog Token field is set to the nonzero value received in the Channel Usage Request frame if the Channel Usage Response frame is being transmitted in response to a Channel Usage Request frame. The Dialog Token field is set to zero if the Channel Usage Response frame is being transmitted other than in response to a Channel Usage Request frame.

The Channel Usage Element field includes zero or more Channel Usage elements described in 7.3.2.81.

The Country String field is set to the value contained in the dot11CountryString attribute.

The Power Constraint Element field includes zero or one Power Constraint elements described in 7.3.2.16. The use of the Power Constraint element included in the Power Constraint Element field is described in 11.20.14.

The EDCA Parameter Set Element field includes zero or one EDCA Parameter Set elements described in 7.3.2.29. The use of the EDCA Parameter Set element included in the EDCA Parameter Set Element field is described in 11.20.18.

10.3.2.1.2 Semantics of the service primitive

Change the primitive parameter list as shown:

MLME-SCAN.request(
BSSType,
BSSID,
SSID,
ScanType,
ProbeDelay,
ChannelList,
MinChannelTime,
MaxChannelTime,
RequestInformation,
SSIDList,
ChannelUsage,
VendorSpecificInfo)

Insert the following new rows before VendorSpecificInfo in the parameter table:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| SSID List | Octet string | Variable | A list of one or more SSID elements, which may be present when dot11MgmtOptionSSIDListEnabled is true. |
| ChannelUsage | A set of Channel Usage element | A set of Channel Usage element | Specifies request types for the Channel Usage request. |

10.3.2.2 MLME-SCAN.confirm

10.3.2.2.2 Semantics of the service primitive

Insert the following new row in the BSSDescription parameter table:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| ChannelUsage | A set of Channel Usage element | A set of Channel Usage element | Specifies parameters for the Channel Usage. |

11.20.14 Channel usage procedures

Channel Usage information is a set of channels provided by an AP to non-AP STAs for operation of a non-infrastructure network. The Channel Usage information provided by the AP to the non-AP STA is to advise the STA on how to co-exist with the infrastructure network.

Implementation of Channel Usage is optional for a WNM STA. A STA that implements Channel Usage has the MIB attribute dot11MgmtOptionChannelUsageImplemented set to true. When dot11MgmtOptionChannelUsageImplemented is true, dot11WirelessManagementImplemented shall be true. A STA that has a value of true for the MIB attribute dot11MgmtOptionChannelUsagEnabled is defined as a STA that supports Channel Usage. A STA for which the MIB attribute dot11MgmtOptionChannelUsageEnabled is true shall set the Channel Usage field of the Extended Capabilities information element to 1.

A non-AP STA that supports Channel Usage and is not associated to an AP prior to using a non-infrastructure network may transmit a Probe Request frame including both Supported Regulatory Classes and Channel Usage elements. A non-AP STA supporting Channel Usage may send a Channel Usage Request frame at any time after association to the AP that supports the use of Channel Usage to request the Channel Usage information for supported regulatory classes.

Upon the receipt of Channel Usage element in the Probe Request frame, the AP supporting Channel Usage shall send a Probe Response frame including one or more Channel Usage elements. Upon receiving a Channel Usage Request frame, the AP supporting Channel Usage shall send a Channel Usage Response frame including one or more Channel Usage elements. Channel Usage elements shall only include channels that are valid for the regulatory domain in which the AP transmitting the element is operating and consistent with the Country element in the Beacon or Probe Response frame.

The AP may send an unsolicited group addressed or individually addressed Channel Usage Response frame to the STAs that have requested Channel Usage information if the corresponding Channel Usage information needs to be updated. The Country information element shall be included in the unsolicited and/or group addressed Channel Usage Response frame. The Power Constraint information and EDCA Parameter elements may be included in the Channel Usage Response frame. The values of the fields in the Power Constraint and EDCA Parameter Set information elements included in the Channel Usage Response frame shall be the same values of the fields in the Power Constraint and EDCA Parameter Set information elements that are transmitted by the AP.

On the receipt of Channel Usage element in the Probe Response or Channel Usage Response frame, the recipient may use

—	the channel usage information as part of channel selection processing to start a non-infrastructure network.

—	the Power Constraint element, if present, as part of determining its maximum transmit power for transmissions for the non-infrastructure network.

—	the EDCA Parameter Set element, if present, as part of determining its EDCA parameters for transmissions for the non-infrastructure network.

If either a recommended regulatory class, or a recommended channel, or both are not supported or understood by the recipient, or if the operating country of the sender is unknown, the recipient shall discard the corresponding channel usage recommendation. A STA that has not requested Channel Usage information shall discard an unsolicited group addressed Channel Usage Response frame.

A.4.21 WNM extensions

| *WNM18 | Channel Usage | 11.20.14 | CFv:O | Yes, No, N/A |
| --- | --- | --- | --- | --- |
| WNM18.1 | Channel Usage Request frame | 7.3.2.81, 7.4.11.27 | WNM18:M | Yes, No, N/A |
| WNM18.2 | Channel Usage Response frame | 7.3.2.81, 7.4.11.28 | WNM18:M | Yes, No, N/A |