IEEE P802.11
Wireless LANs

| Notice of Session Expiry |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-05-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Dave Stephenson | Cisco Systems, Inc. | 170 W. Tasman Dr. San Jose, CA 95134 | +1 408 527 7991 | daves@cisco.com |
| Michael Montemurro | Research in Motion | 4701 Tahoe Blvd. Mississauga, ON. L4W 0B4 | +1 905 629 4746 | mmontemurro@rim.com |
| Stephen McCann | Research in Motion | 200 Bath Road, Slough Berkshire, SL1 3XE, UK | +44 1753 667099 | smccann@rim.com |
| Allan Thomson | Cisco Systems | 170 W Tasman Drive, San Jose, CA 95134 | +1 408 853 5570 | allant@cisco.com |
| Dorothy Stanley | Aruba Networks | 1322 Crossman Ave Sunnyvale CA 94089 | +1 630 363 1389 | dstanley@arubanetworks.com |

Abstract

Hotspot operators have reported that subscribers, who have purchased a time-restricted plan (e.g., Internet access for 2 hours), would like a notification that their session is about to expire so they can quickly finish what they are doing (e.g., send out emails).  Likewise, SSPNs would like to sell additional time to those subscribers needing it.

This submission provides a notice of session expiry to a subscriber.  It also provides an optional URL to that the non-AP STA can autonomously launch a browser to the website at which they can purchase additional time.

This document is based on 802.11v-d5.01.

2. Normative references

Insert the following new references into 2 maintaining the ordering in the base spec:

IETF RFC 3986, Uniform Resource Identifier (URI): Generic Syntax, January 2005.

7	Frame formats

7.4 Action frame format details

7.4.11 WNM Action details

Change the text as shown below

7.4.11.9 BSS Transition Management Request frame format

The BSS Transition Management Request frame uses the Action frame body format and is transmitted by an AP STA in response to a BSS Transition Management Query frame, or autonomously. The format of the BSS Transition Management Request frame body is shown in Figure v85.

|  | Category | Action | Dialog Token | Request mode | Dis-association Timer | Validity Interval | BSS Termination Duration (optional) | Session Information URL (optional) | BSS Transition Candidate List Entries |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 2 | 1 | 0 or 12 | variable | variable |  |

Figure v85—BSS Transition Management Request frame body format

The Category field is set to the value indicating the WNM category, as specified in Table 7-24 in 7.3.1.11.

The Action field is set to the value indicating BSS Transition Management Request frame, as specified in Table v39 in 7.4.11.1.

The Dialog Token field is set to the nonzero value received in the BSS Transition Management Query frame if the BSS Transition Management Request frame is being transmitted in response to a BSS Transition Management Query frame. If the BSS Transition Management Request frame is being transmitted other than in response to a BSS Transition Management Query frame, then the Dialog Token field is set to a nonzero value chosen by the AP STA sending the BSS Transition Management Request frame to identify the request/response transaction.

The Request mode field is defined in Figure v86.

|  | Preferred Candidate List Included | Abridged | Disassociation Imminent | BSS Termination Included | ESS Disassociation Imminent | Reserved |
| --- | --- | --- | --- | --- | --- | --- |
| Bit: | 0 | 1 | 2 | 3 | 4 | 5- 7 |

Figure v86—Request Mode field

—	The Preferred Candidate List Included (bit 0) field indicates whether the BSS transition candidate list included in this frame is a preferred candidate list or a list of known BSS transition candidates. See 11.20.6.3.

—	The Abridged (bit 1) field indicates to the recipient of the frame the intended treatment of all BSSIDs not listed in the BSS Transition Candidate List. See 11.20.6.3.

—	The Disassociation Imminent (bit 2) field indicates whether the STA will be disassociated from the current AP. See 11.20.6.3.

- The BSS Termination Included (bit 3) field indicates that the BSS Termination Duration field is included, the BSS is shutting down and the STA will be disassociated.

- The ESS Disassociation Imminent (bit 4) field indicates that the Session Information URL field is included, and that the STA will be disassociated from the ESS. See 11.20.6.3.

The Disassociation Timer field is set to the number of beacon transmission times (TBTTs) until the serving AP sends a Disassociation frame to this STA. If the Disassociation Imminent field is set to 0, the Disassociation Timer field is reserved.  The format of the Disassociation Timer field is shown in Figure v86a.

|  | B0 | B15 |
| --- | --- | --- |
|  | Disassociation Timer |  |
| Octets: | 2 |  |

Figure v86a—Disassociation Timer field

The Validity Interval field is set to the number of beacon transmission times (TBTTs) until this recommendation of this BSS transition candidate list is no longer valid. A value of 0 is reserved.

The BSS Termination Duration field contains the BSS Termination Duration subelement (see 7.3.2.37) for the current BSS and is present only when the BSS Termination Included field is set to 1 in the Request mode field.

The format of the optional Session Information URL field is shown in Figure v86b.  This field is present when the ESS Disassociation Imminent field is set to 1.

|  | URL Length | URL (optional) |
| --- | --- | --- |
| Octets: | 1 | variable |

Figure v86b—Session Information URL field format

- The URL Length field is a 1-octet field whose value is the length of the URL field.  The URL Length field is set to 0 when the optional URL subfield is not present..

- The URL field is a variable-length field formatted in accordance with RFC-3986.

The BSS Transition Candidate List Entries field contains one or more Neighbor Report elements described in 7.3.2.37. If the STA has no information in response to the BSS Transition Management Query frame, the Neighbor Report Elements are omitted. The length of the BSS Transition Candidate List Entries in a BSS Transition Management Request frame is limited to 2304 octets.

10.	Layer management

10.3	MLME SAP interface

10.3.52 BSS Transition Management

10.3.52.4 MLME-BTM.request

10.3.52.4.2 Semantics of the Service Primitive

Change the text as shown below

The primitive parameters are as follows:

MLME-BTM.request	(
PeerMACAddress
DialogToken,
RequestMode,
DisassociationTimer,
ValidityInterval,
BSSTerminationDuration,
SessionInformationURL,
BSSTransitionCandidateList)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual MAC Address | The address of the peer MAC entity to which the BSS Transition Management Request frame is sent. |
| DialogToken | Integer | 1 – 255 | The Dialog Token to identify the BSS Transition Management transaction. Set to 0 for an autonomous BSS Transition Management Request frame. |
| RequestMode | Integer | As specified in 7.4.11.9 | Contains the Disassociation Imminent, ESS Disassociation Imminent and Abridged bits for the BSS Transition Management Request. |
| Disassociation Timer | Integer | 0 – 65535 | Specifies the number of TBTTs until the AP shall disassociate the non-AP STA. A value of 0 indicates time of disassociation has not yet been determined and a value of 1 indicates disassociation shall occur before the next TBTT. |
| ValidityInterval | Integer | 1 – 255 | Specifies the number of beacon transmission times (TBTTs) until this recommendation of this BSS transition candidate list is no longer valid. |
| BSSTerminationDuration | BSS Termination Duration subelement | BSS Termination Duration subelement | Contains the BSS Termination Duration subelement (see 7.3.2.37) for the current BSS and is present only when the BSS Termination Included field is set to 1 in the Request mode field. |
| SessionInformationURL | URL | n/a | Optionally contains a URL formatted per RFC-3986 where additional information pertaining to the user’s accounting session may be found. |
| BSSTransition CandidateList | Set of Neighbor Report Elements | Set of Neighbor Report Elements as defined in the Neighbor Report Element in 7.3.2.37 | Contains the description of candidate BSS transition APs and their capabilities as described in section 7.3.2.37. |

10.3.52.5 MLME-BTM.indication

10.3.52.5.2 Semantics of the Service Primitive

Change the text as shown below

The primitive parameters are as follows:

MLME-BTM.indication	(
ResultCode,
PeerMACAddress,
DialogToken,
RequestMode,
DisassociationTimer,
ValidityInterval,
BSSTerminationDuration,
SessionInformationURL,
BSSTransitionCandidateList)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID_ PARAMETERS, TIMEOUT, TRANSMISSION_FAILURE, UNSPECIFIED_ FAILURE | Indicates the result of the corresponding MLME-BTMQUERY.request. The ResultCode field is set to SUCCESS if the received BSS Transition Request frame is an unsolicited frame. |
| Peer MAC Address | MACAddress | Any valid individual MAC Address | The address of the MAC entity from which a BSS Transition Management Request frame was received. |
| DialogToken | Integer | 1 – 255 | The Dialog Token to identity this BSS Transition Management transaction as received in the BSS Transition Management Request frame. |
| RequestMode | Integer | As specified in 7.4.11.9 | Contains the Disassociation Imminent, ESS Disassociation Imminent and Abridged bits for the BSS Transition Management Request. |
| Disassociation Timer | Integer | 0 – 65535 | Specifies the number of TBTTs until the AP shall disassociate the non-AP STA. A value of 0 indicates time of disassociation has not been determined yet and a value of 1 indicates disassociation shall occur before the next TBTT. |
| ValidityInterval | Integer | 1 – 255 | Specifies the number of beacon transmission times (TBTTs) until this recommendation of this BSS transition candidate list is no longer valid. |
| BSSTerminationDuration | BSS Termination Duration subelement | BSS Termination Duration subelement | Contains the BSS Termination Duration subelement (see 7.3.2.37) for the current BSS and is present only when the BSS Termination Included field is set to 1 in the Request mode field. |
| SessionInformationURL | URL | n/a | Optionally contains a URL formatted per RFC-3986 where additional information pertaining to the user’s accounting session may be found. |
| BSSTransition CandidateList | Set of Neighbor Report Elements | Set of Neighbor Report Elements as defined in the Neighbor Report Element in 7.3.2.37 | Contains the description of candidate BSS transition APs and their capabilities as described in 7.3.2.37. |

11	MLME

11.20.6 BSS transition management for network load balancing

11.20.6.1 BSS transition capability

Change the text as shown below

The BSS Transition capability can enable the improved throughput, effective data rate and/or QoS for the aggregate of STAs in a network by shifting (via transition) individual STA traffic loads to more appropriate points of association within the ESS.  In addition, the BSS Transition capability can provide accounting session control information to a non-AP STA which can be used to provide an alert to the non-AP STA’s user that their session is almost over and the STA will be disassociated from the ESS.

11.20.6.2 BSS transition management query

11.20.6.3 BSS transition management request

An AP that supports BSS transition management shall respond to a BSS Transition Management Query frame with a BSS Transition Management Request frame. The AP may send an unsolicited BSS Transition Management Request frame to a non-AP STA at any time if the non-AP STA indicates that it supports such capability in the Extended Capabilities information element.

The AP shall include the BSS Transition Candidate List Entries field in the BSS Transition Management Request frame if the AP has information in response to the BSS Transition Management Query frame. The BSS Transition Candidate List Entries field contains one or more Neighbor Report elements describing the preferences for target AP candidates. Preference field value of 0 indicates that the AP to be listed is an excluded AP, and the STA shall not attempt to associate to the listed AP unless the STA is unable to associate with any non-excluded APs that are compatible with the STA’s choice of SSID and, if the STA is associated with the AP that sent the request, the STA has transmitted a BSS Transition Candidate Response frame to the associated AP with the reason of reject. The Preference field values are used only to establish the relative order of entries within the given list at the given time, and for the given AP.

The values between 1 and 255 provide the indication of order, with 255 indicating the most preferred AP within the given candidate list, decreasing numbers representing decreasing preference relative only to entries with lower values of the Preference field, and equal numbers representing equal preference. The preference value is only valid before the Validity Interval has expired. Zero or more subelements may be included in the BSS Transition Candidate List Entries field. Load Subelements contain additional information describing the BSS load and QoS BSS Admission Control Capacity. The subelement is used by a non-AP QoS STAs to select a QoS AP that is likely to accept future admission control requests, but it does not represent a guarantee that the HC shall admit these requests.

The Preferred Candidate List Included bit set to 0 indicates that the receiving STA may ignore the Preferred Candidate List. The Preferred Candidate List Included bit set to 1 indicates that the sender expects the receiving STA to process this frame.

The AP sets the Abridged bit in the Request Mode field to 1 when it wishes to assign a preference value of 0 to all BSSIDs that do NOT appear in the BSSID list. The non-AP STA that receives the abridged bit with a value of 1 shall treat every considering BSSID as if it were present in the BSS Transition Candidate List with a Preference value of 0. The AP sets the Abridged bit in the Request Mode field to 0 when the AP has no recommendation for or against any BSSID not present in the BSS Transition Candidate List Entries field.

The Disassociation Imminent bit in the Request Mode field set to 1 indicates that STA is to be disassociated from the current AP. The Disassociation Imminent field set to 0 indicates that disassociation from the AP is not imminent.

The AP sets the BSS Termination Included bit in the Request mode field to 1 to indicate that the BSS is shutting down. The BSS Termination Included bit is set to 0 if no BSS Termination Duration information is included in the BSS Transition Management Request frame.

The ESS Disassociation Imminent bit in the Request Mode field set to 1 indicates that STA is to be disassociated from the ESS. When the ESS Disassociation Imminent bit is set to 1, a Session Information URL field is included in the BSS Transition Management Request frame. The ESS Disassociation Imminent field set to 0 indicates that disassociation from the ESS is not imminent.

The Disassociation Timer indicates the time after which the AP will issue a Disassociation frame to this STA. A value of 0 indicates that the serving AP has not determined when it will send a Disassociation frame to this STA.

If the AP sets the BSS Termination Included (bit 3) field in the Request mode field to 1 the AP shall include the BSS Termination Duration field to indicate the expected time interval for which its BSS will be shutdown.

The AP may also include one BSS Termination Duration subelement for each BSS in the BSS Transition Candidate List entry, including the BSS Termination Duration value and a BSS Termination TSF value. The BSS Termination duration may be different for each BSS.

An AP’s SME may have accounting session control information, such as a notice of session expiry.  The means by which the AP’s SME obtains accounting session control information is out of scope of this specification.  Accounting session control information can include a time duration after which the non-AP STA will be disassociated from the ESS and an optional session information URL at which information may be obtained to extend the accounting session.  When an AP’s SME has accounting session control information, it shall issue a MLME-BTM.request to the AP’s MLME and shall encode the time to session expiry in the DisassociationTimer parameter and shall encode the URL, if available, in the SessionInformationURL parameter.  A non-AP STA’s SME receiving a MLME-BTM.indication  forwards the MLME-BTM.indication parameters to the appropriate entity within the device (e.g., web-browser) to inform the end-user; the means and protocol by which the SME accomplishes this is outside the scope of this specification.

A STA receiving a MLME-BTM.indication containing the BSS Transition Candidate List Entries field may use this list of candidates, with their individual transition preference values, to make BSS transition decisions. Upon receiving a MLME-BTM.indication, the STA shall disregard any previous MLME-BTM.indication received from the same AP. The STA shall not use the corresponding BSS Transition Candidate List Entries field information after the indicated Validity Interval. The STA may send a BSS Transition Management Query frame at any time to obtain an updated BSS Transition Candidate List Entries field.

A STA receiving a MLME-BTM.indication containing a non-zero value of the Disassociation Timer field shall attempt to re-associate with some other AP before the indicated disassociation time. If the receiving STA cannot find a suitable AP with which to associate, the STA shall send a MLME-BTM.response containing a Status Code indicating reject before the indicated disassociation time.