IEEE P802.11
Wireless LANs

| LB-148: Roaming Consortium Update |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-05-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Dave Stephenson | Cisco Systems, Inc. | 170 W. Tasman Dr. San Jose, CA 95134 | +1 408 527 7991 | daves@cisco.com |

Abstract

This submission is to resolve the following comments from LB-148: CIDs 6056, 6057, 6058 and 6090.

This submission is based on IEEE 802.11u-d6.01.

7	Frame formats

7.3	Management frame body components

- 7.3.2	Information elements

Change the text below as shown:

7.3.2.87 Roaming Consortium information element

The Roaming Consortium Information element contains information identifying the roaming consortium and/or SSP whose security credentials can be used to authenticate with the AP transmitting this element. The element’s format is shown in Figure 7-95o117.

|  | Element ID | Length | Number of Native-GAS OIs | OI #1 | OI #2 (optional) | OI #3 (optional) |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 3 or 5 | 3 or 5 | 3 or 5 |

Figure 7-95o117—Roaming Consortium element format

The Length is a one-octet field whose value is between 4 and 16 inclusive depending on the number and size of OI fields present.

The Number of Native-GAS OIs field’s format is one-octet unsigned integer whose value is the number of additional roaming consortium OIs obtainable via Native Query protocol. A value of zero means that no additional OIs will be returned in response to a Native GAS query for the Roaming Consortium List. A value of 255 means that 255 or more additional OIs are obtainable via Native Query protocol.

The OI field is a public OI assigned by the IEEE ANA as defined in 7.3.1.21. Each OI identifies a roaming consortium (group of SSPs with inter-SSP roaming agreement) or a single SSP. A non-AP STA in possession of security credentials for the SSPN(s) identified by the OI should be able to successfully authenticate to this AP.  The value of the OI(s) in this table are equal to the value of the first 3 OIs in the dot11RoamingConsortiumTable.  If fewer than 3 values are defined in the dot11RoamingConsortiumTable, then only as many OIs as defined in the table are populated in this element.

7.3.4 Native Query Protocol elements

Change the text below as shown:

7.3.4.5 Roaming Consortium List

The Roaming Consortium List provides a list of information about the Roaming Consortium and/or SSPs whose networks are accessible via this AP. This list may be returned in response to a Native GAS Query Request. The format of the Roaming Consortium List is provided in 7-95o126.

|  | Info ID | Length | OI #1 (optional) | OI #2 (optional) | … | OI #N (optional) |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 2 | 2 | 3 or 5 | 3 or 5 |  | 3 or 5 |

Figure 7-95o126—Roaming Consortium List format

The Info ID field is equal to the value in Table 7-43by corresponding to the Roaming Consortium List.

The Length is a 2-octet field whose value is dependent on the number of and size OIs present in the element.

Each OI field is a public OI assigned by the IEEE as defined in 7.3.1.21. Each OI identifies a roaming consortium (group of SSPs with inter-SSP roaming agreement) or a single SSP. There are zero or more OIs in this list. A non-AP STA in possession of security credentials for the SSPN(s) identified by the OI should be able to authenticate with this AP. OIs contained within the Roaming Consortium element (see 7.3.2.87) are also included in this list.  The value of the OI(s) in this list are equal to the values of the OIs in the dot11RoamingConsortiumTable.

Change the following text as shown below:

- Annex D

dot11RoamingConsortiumOui OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(5))

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This attribute contains the 6 or 9 hexadecimal

characters which correspond to the IEEE defined OUI

per 7.3.1.21”.

::= { dot11RoamingConsortiumEntry 1 }