IEEE P802.11
Wireless LANs

| LB 151 – P802.11p/D 7.0 Comment Submission – Annex J Tables |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-06-02 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| George Vlantis | STMicroelectronics | 1060 East Brokaw Road, San Jose, CA 95131 | 408-451-8109 (office) 408-893-9357 (mobile) | George.Vlantis@st.com |

Abstract

Submission to align the Annex J Table J.1 and Table J.2 of 802.11p Draft 7.0 with the 802.11 baseline specification in the 5.4 GHz band and and with the usages in the United States and Europe in the 5.9 GHz band.

|  | LB151 Comment Resolution |
| --- | --- |

- COMMENT:  [From Spreadsheet]  INSERT Original Comment Here:

| ID | Commenter | Clause | Pg | Ln | Type | Comment | Suggested Remedy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |

- Background, Explanation, Discussion, etc.:

3.	Recommended Resolution of the Comments:

See the right column above for the resolutions of the individual comments.

- Recommended Changes to P802.11p D7.0 (recommended by Alastair Malarkey, Mark IV):

Table J.1—Regulatory classes in the United States

| Regulatory Class | Channel starting frequency (GHz) | Channel spacing (MHz) | Channel set | Transmit power limit (mW) | Transmit power limit (EIRP) | Emissions limits set | Behavior limits set |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 161 | 5.0025 | 5 | 170-184 | 760 | 44.8 dBm | 7 | 17, 18 |
| 171 | 5 | 10 | 171-184 (172, 174, 176, 178, 180, 182, and 184) | 760 | 44.8 dBm | 7 | 17, 18 |
| 181 | 5 | 20 | 172-183 (175 and 181) | 100 | 23 dBm | 7 | 17, 18 |
| 19-255 | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved |

Table J.2—Regulatory classes for 5 GHz bands in Europe

| Regulatory Class | Channel starting frequency (GHz) | Channel spacing (MHz) | Channel set | Transmit power limit (mW) | Transmit power limit (EIRP) | Emissions limits set | Behavior limits set |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 131 | 5.0025 | 5 | 170-184 | - | 33 dBm | 7 | 17, 18 |
| 141 | 5 | 10 | 172, 174, 176, 178, 180, 182, and 184 171-184 | - | 33 dBm | 7 | 17, 18 |
| 151 | 5 | 20 | 172-183 | - | 23 dBm | 7 | 17, 18 |
| 161 | 5 | 10 | 100, 102, 104, 106, 108, 110, 112, 114, 116, 118, 120, 122, 124, 126, 128, 130, 132, 134, 136, 138, and 140 | - | 30 dBm | 7 | 17, 18 |
| - 16 (no superscript) | 5 | 20 | 100-140 100, 104, 108, 112, 116, 120, 124, 128, 132, 136, and 140 | - | 30 dBm | 7 | 1, 3, 4, 17, 18 |
| 18 17-255 | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved |

- Recommended Changes to P802.11p D7.0 (recommended by Eldad Perahia, Intel):

Table J.1—Regulatory classes in the United States

| Regulatory Class | Channel starting frequency (GHz) | Channel spacing (MHz) | Channel set | Transmit power limit (mW) | Transmit power limit (EIRP) | Emissions limits set | Behavior limits set |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 161 | 5.0025 | 5 | 170-184 | 760 | 44.8 dBm | 7 | 17, 18 |
| 171 | 5 | 10 | 171-184 172, 174, 176, 178, 180, 182, and 184 | 760 | 44.8 dBm | 7 | 17, 18 |
| 181 | 5 | 20 | 172-183 175 and 181 | 100 | 23 dBm | 7 | 17, 18 |
| 19-255 | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved |

Table J.2—Regulatory classes for 5 GHz bands in Europe

| Regulatory Class | Channel starting frequency (GHz) | Channel spacing (MHz) | Channel set | Transmit power limit (mW) | Transmit power limit (EIRP) | Emissions limits set | Behavior limits set |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 131 | 5.0025 | 5 | 170-184 | - | 33 dBm | 7 | 17, 18 |
| 141 | 5 | 10 | 172, 174, 176, 178, 180, 182, and 184 | - | 33 dBm | 7 | 17, 18 |
| 151 | 5 | 20 | 172-183 173 and 175 | - | 23 dBm | 7 | 17, 18 |
| 161 | 5 | 10 | 100, 102, 104, 106, 108, 110, 112, 114, 116, 118, 120, 122, 124, 126, 128, 130, 132, 134, 136, 138, and 140 | - | 30 dBm | 7 | 17, 18 |
| 171 16 (no superscript) | 5 | 20 | 100-140 100, 104, 108, 112, 116, 120, 124, 128, 132, 136, and 140 | - | 30 dBm | 7 | 1, 3, 4, 17, 18 |
| 18 17-255 | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved |