IEEE P802.11
Wireless LANs

| LB150 Timing Measurement Comment Resolutions Normative Text |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-Jume-30 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Ganesh Venkatesan | Intel Corporation | 2111NE 25th Ave Hillsboro, OR 97124 | 503-264-0637 | ganesh.venkatesan@intel.com |
| Robert Stacey | Intel Corporation | 2111NE 25th Ave Hillsboro, OR 97124 | 503 264 2823 | Robert.J.Stacey@intel.com |

- Abstract

This document contains normative text changes to resolve LB150 comment IDs: 90, 91, 92, 94, 96, 97, and 100.

The submission is referenced to draft D6.0 and augments to all editor instructions as well as the actual base draft modifications.

All new text relative to the draft D6.0 is identified in blue color and underlined and all deleted text is identified by strikethrough.

### 7.4.13.3 Timing Measurement frame format

. The format of the Timing Measurement frame is shown in Figure v115.

Editor’s note: In Figure v115, delete “(option)” from TOD, TOA, Max TOD Error and Max TOA Error fields and set the octet length to 4, 4, 1 and 1 respectively.

|  | Category | Action | Dialog Token | Follow Up Dialog Token | TOD |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 4 |
|  | TOA | Max TOD Error | Max TOA Error |  |  |
| Octets: | 4 | 1 | 1 |  |  |
| Figure v115—Timing Measurement frame format |  |  |  |  |  |

The Category field is set to the value indicating the Unprotected WNM category, as specified in Table 7-24 in 7.3.1.11.

The Action field is set to the value indicating Timing Measurement as specified in Table v44 in 7.4.13.1.

The Dialog Token field is set to a non-zero value chosen by the sending STA  to identify the Timing Measurement frame as the first of a pair, with the second or follow up Timing Measurement frame to be sent later. The Dialog Token is set to zero indicate that the Timing Measurement frame will not be followed by a subsequent follow up Timing Measurement frame.

The Follow On Dialog Token is set to the non-zero value of the Dialog Token field of the previously transmitted Timing Measurement frame to indicate that it is the follow up Timing Measurement frame and that the TOD, TOA, Max TOD Error and Max TOA Error fields contain the values of the timestamps captured with the first Timing Measurement frame of the pair. The Follow On Dialog Token is set to zero to indicate that the Timing Measurement frame is not a follow on to a previously transmitted Timing Measurement frame. A zero value in this field also indicates that the TOD, TOA, Max TOD Error and Max TOA Error fields are reserved. See 11.21.5.

The TOD, TOA, Max TOD Error, and Max TOA Error fields are expressed in units of 10 nanoseconds.

The TOD field contains a timestamp that represents the time at which the start of the preamble of the previously transmitted Timing Measurement frame appeared at the transmit antenna port.

The TOA field contains a timestamp that represents the time at which the start of the preamble of the ACK response to the previously transmitted Timing Measurement frame arrived at the receive antenna port.

NOTE — The values specified in the TOD and TOA fields are described in 10.3.54.

The Max TOD Error field contains an upper bound for the error in the value specified in the TOD field. For instance, a value of 2 in the Max TOD Error field indicates that the value in the TOD field has a maximum error of +/- 20 nanoseconds.

The Max TOA Error field contains an upper bound for the error in the value specified in the TOA field. For instance, a value of 2 in the Max TOA Error field indicates that the value in the TOA field has a maximum error of +/- 20 nanoseconds.

A value of zero for the Max TOD Error or the Max TOA Error field indicates that the upper bound on the error in the corresponding TOD or TOA value is unknown. A value of 255 indicates that the upper bound on the error is greater than or equal to 2.55 microseconds.

# 10. Layer management

## 10.3 MLME SAP Interface

## 10.3.54 Timing Measurement

10.3.54 Timing Measurement

The following set of primitives supports exchange of timing measurement information from one SME to another. The informative diagram in Figure v120 depicts various points in time that are of interest to the timing measurement procedure.

Figure v120—Timing Measurement Information Exchange

NOTE 1 — In Figure v120, t1 and t3 correspond to the point in time at which the start of the preamble for the transmitted frame appears at the transmit antenna port. An implementation may capture a timestamp during the transmit processing earlier or later than the point at which it actually occurs and offset the value to compensate for the time difference.

NOTE 2— In Figure v120, t2 and t4 correspond to the point in time at which the start of the preamble for the incoming frame arrives at the receive antenna port. Because time is needed to detect the frame and synchronize with its logical structure, an implementation will necessarily determine when the start of the preamble for the incoming frame arrived at the receive antenna port by capturing a timestamp some time after it occurred  and compensating for the delay by subtracting an offset from the captured value.

10.3.54.1 MLME-TIMINGMSMT.request

10.3.54.1.1 Function

This primitive requests the transmission of Timing Measurement frame to a peer entity.

10.3.54.1.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-TIMINGMSMT.request	(
Peer MAC Address,
Dialog Token,
Follow Up Dialog Token,
TOD,
				TOA,
Max TOD Error,
Max TOA Error,
VendorSpecific)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual or group addressed MAC Address | The address of the peer MAC entity to which the Timing Measurement frame shall be sent. |
| Dialog Token | Integer | 1-255 | The dialog token to identify the Timing Measurement transaction. |
| Follow Up Dialog Token | Integer | 0-255 | The dialog token of a Timing Measurement frame which the current frame follows. See 11.21.5. |
| TOD | Integer |  | This parameter is present only if the Follow Up Dialog Token field is non-zero. See 7.4.13.3. It is set to value of t1. See Figure v120. |
| TOA | Integer |  | This parameter is present only if the Follow Up Dialog Token field is non-zero. See 7.4.13.3. It is set to value of t4. See Figure v120. |
| Max TOD Error | Integer | 0-255 | This parameter is present only if the Follow Up Dialog Token field is non-zero. See 7.4.13.3. |
| Max TOA Error | Integer | 0-255 | This parameter is present only if the Follow Up Dialog Token field is non-zero. See 7.4.13.3. |
| VendorSpecific | A set of information elements | As defined in 7.3.2.26 | Zero or more information elements. |

10.3.54.1.3 When generated

This primitive is generated by the SME to request that a Timing Measurement frame be sent to a peer entity.

10.3.54.1.4 Effect of receipt

On receipt of this primitive, the MLME constructs a Timing Measurement frame with the specified parameters. This frame is then scheduled for transmission.

10.3.54.2 MLME-TIMINGMSMT.confirm

10.3.54.2.1 Function

This primitive indicates that a Timing Measurement frame has been successfully received by the peer STA to which it was sent.

10.3.54.2.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-TIMINGMSMT.confirm	(

Peer MAC Address,

Dialog Token,

TOD,

Max TOD Error,

TOA,

Max TOA Error)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual or group addressed MAC Address | The address of the peer MAC entity to which acknowledges the receipt of the Timing Measurement frame |
| Dialog Token | Integer | 1-255 | The dialog token to identify the Timing Measurement transaction. |
| TOD | 32 bit unsigned Integer | 0 - 232-1 | Set to the value of t1 (see Figure v120) expressed in 10 nanosecond units. |
| Max TOD Error | Integer | 0-255 | Maximum error in the TOD value expressed in 10 nanosecond units. |
| TOA | 32 bit unsigned Integer | 0 - 232-1 | Set to the value of t4 (see Figure v120) expressed in 10 nanosecond units. |
| Max TOA Error | Integer | 0-255 | Maximum error in TOA value expressed in 10 nanosecond units. |

10.3.54.2.3 When generated

This primitive is generated by the MLME when an ACK corresponding to the Timing Measurement frame is received from the peer STA.

10.3.54.2.4 Effect of receipt

On receipt of this primitive, the SME uses the information contained within the notification.

10.3.54.3 MLME-TIMINGMSMT.indication

10.3.54.3.1 Function

This primitive indicates that a Timing Measurement frame has been received and the corresponding ACK has been transmitted.

10.3.54.3.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-TIMINGMSMT.indication	(

Peer MAC Address,

Dialog Token,

Follow Up Dialog Token,

TOD,

Max TOD Error,

TOA,

Max TOA Error,

Timing Measurement Frame Time of Arrival,

Max Error in Timing Measurement Frame Time of Arrival,

ACK TOD,

Max ACK TOD Error,

VendorSpecific)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual or group addressed MAC Address | The address of the peer MAC entity to which the Timing Measurement frame shall be sent. |
| Dialog Token | Integer | 1-255 | The dialog token to identify the Timing Measurement transaction. |
| Follow Up Dialog Token | Integer | 1-255 | The dialog token of a Timing Measurement frame which the current frame follows. See 11.21.5. |
| TOD | Integer |  | Set to the value of TOD field in the incoming Timing Measurement action frame. Otherwise set to zero. |
| Max TOD Error | Integer | 0-255 | Set to the value of Max TOD Error field in the incoming Timing Measurement action frame. Otherwise set to zero. |
| TOA | Integer |  | Set to the value of TOA Error field in the incoming Timing Measurement action frame. Otherwise set to zero. |
| Max TOA Error | Integer | 0-255 | Set to the value of Max TOA Error field in the incoming Timing Measurement action frame. Otherwise set to zero. |
| Timing Measurement Frame Time of Arrival | 32 bit unsigned Integer | 0 - 232-1 | Set to the value of t2 (See Figure v120) expressed in 10 nanosecond units. |
| Max Error in Timing Measurement Time of Arrival | Integer | 0-255 | Maximum error in the Time of Arrival value expressed in 10 nanosecond units. |
| ACK TOD | 32 bit unsigned Integer | 0 - 232-1 | Set to the value of t3 (See Figure v120) expressed in 10 nanosecond units. |
| Max ACK TOD | Integer | 0-255 | Maximum error in the value specified in the ACK TOD field, expressed in 10 nanosecond units. |
| VendorSpecific | A set of information elements | As defined in 7.3.2.26 | Zero or more information elements. |

10.3.54.3.3 When generated

This primitive is generated by the MLME when a valid Timing Measurement frame is received.

10.3.54.3.4 Effect of receipt

On receipt of this primitive, the SME uses the information contained within the notification.

## 11.21.5 Timing measurement procedure

Implementation of Timing Measurement is optional for a WNM STA. A STA that implements Timing Measurement has the MIB attribute dot11MgmtOptionTimingMsmtImplemented set to true. When

dot11MgmtOptionTimingMsmtImplemented is true, dot11WirelessManagementImplemented shall be true.

If dot11MgmtOptionTimingMsmtEnabled is set to true, the Timing Measurement field in the Extended Capabilities information element shall be set to 1 and the STA supports the timing measurement procedure. If dot11MgmtOptionTimingMsmtEnabled is false the STA shall set the Timing Measurement field in the Extended Capabilities information element to 0 and STA does not support the timing measurement procedure. ASTA that does not support the timing measurement procedure shall ignore a received Timing Measurement frame.

A STA that supports the timing measurement procedure may transmit Timing Measurement actions frames addressed to a peer STA that also supports the timing measurement procedure. The means by which a STA determines that it should transmit Timing Measurement frames to a peer STA is beyond the scope of this standard.

A sending STA transmits Timing Measurement action frames in pairs. The first Timing Measurement action frame of a pair contains a non-zero Dialog Token. The follow up Timing Measurement action frame contains a Follow Up Dialog Token set the value of the Dialog Token in the first frame of the pair. With the first Timing Measurement action frame, both STAs capture timestamps. The sending STA captures the time at which the Timing Measurement frame is transmitted (t1). The receiving STA captures the time at which the Timing Measurement frame arrives (t2) and the time at which the ACK response is transmitted (t3). The sending STA captures the time at which the ACK response arrives (t4). In the follow up Timing Measurement action frame, the sending STA transfers the timestamp values it captured (t1 and t4) to the receiving STA.

The offset of the clock at the receiving STA with respect to the clock at the sending STA is calculated using the equation below (assuming a symmetric wireless channel). See Figure v120 in 10.3.54.

Clock offset at receiving STA relative to sending STA = [(t2 -t1) - (t4 -t3)]/2

The acknowledgement procedure for Timing Measurement action frames is the same as that for regular management frames (see 9.2.8). If the ACK for a transmitted Timing Measurement frame is not received, the sending STA may retransmit the frame. The sending STA shall capture a new set of timestamps for the retransmitted frame and its ACK response

On receiving a Timing Measurement action frame with a Dialog Token for which timestamps have previously been captured, the receiving STA shall discard previously captured timestamps and capture a new set of timestamps.

Figure v124—Timing Measurement Procedure