IEEE P802.11
Wireless LANs

| Liaison Request to IETF GEOPRIV |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-07-13 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Dorothy Stanley | Aruba Networks | 1322 Crossman Ave Sunnyvale, CA | +1 630-363-1389 | dstanley@arubanetworks.com |
| Allan Thomson | Cisco Systems | 170 W. Tasman Drive, San Jose, CA95134 | +1-408-853-5570 | allant@cisco.com |

Abstract

This document contains a liaison to the IETF GEOPRIV Working Group, providing the rationale for and requesting extensions to the CIVIC location binary encoding specifications to support indoor location applications.

.

- From: Bruce Kraemer (bkraemer@marvell.com ), Chair IEEE 802.11 Working Group

To: Richard Barnes (rbarnes@bbn.com ), Alissa Cooper (acooper@cdt.org )

cc: Robert Sparks ( rjsparks@nostrum.com ), Cullen Jennings (fluffy@cisco.com ), Russ Housley (housley@vigilsec.com )

- Title: Request to IETF GEOPRIV for Civic Binary encoding extensions to support the use of location in WLANs

- Purpose: To request Civic Binary encoding extensions to support the use of location in WLAN deployments

Dear Richard and Alissa,

IEEE Std 802.11k-2008 defines a protocol for the exchange of geospatial location data in wireless LANs. The IEEE 802.11v Draft 6.0 amendment, currently under development extends the formats of location data that can be exchanged using the IEEE Std 802.11k defined Location Configuration Request and Report to include both the Civic and Location by Reference formats. It is our preference to re-use IETF defined formats for location data, to minimize specification duplication and provide common formats across layer 2 protocols and upper layer protocols.

RFCs 4119 and 5139 provide an xml format for the representation of civic location. They enable use of civic location information by location-aware devices, including WLAN devices, inside buildings or indoor regions where WLAN networks provide the primary access network.

In WLAN networks, efficient over-the-air transmission of location data is essential. The existing full-featured, text-based XML representation does not meet the required level of efficiency, and thus IEEE 802.11v Draft 6.0 currently uses/refers to RFC 4776 for a binary representation of Civic location information. From a WLAN location requirements perspective however, RFC 4776 does not support full location description where the combination of civic and shape information is required. The following functionality is required:

- A binary representation that supports the Point, Polygon, Arc Band, Rectangle, Circle, Ellipse, Cube, Cuboid, Sphere, and Ellipsoid shapes is required.

- The binary representation should extend or be combinable with civic information already defined in RFC 4776 so that a single payload message can convey the location of a WLAN entity that combines Building Address, Floor and a determined/known reference point and reference location as represented by a shape.

- We request that IETF GEOPRIV develop extensions to existing Civic binary encodings as described above to support WLAN location applications. These extensions enable a common format to be used across layer 2 and upper layer protocols.

Please contact Bruce Kraemer, IEEE 802.11 Working Group chair, together with Dorothy Stanley, IEEE 802.11 to IETF Liaison, and Chair IEEE 802.11 TGv with any questions.

Best Regards,

- Bruce Kraemer

Contact information:

Bruce Kraemer

bkraemer@marvell.com

+1 321 427 4098

Dorothy Stanley

dstanley@arubanetworks.com

+1 630 363 1389