IEEE P802.11
Wireless LANs

| Stream Classification Service |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-07-06 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Alex Ashley | NDS Ltd | One London Road, Staines, Middlesex, TW18 4EX, UK | +44 1784 848770 | aashley@nds.com |

Abstract

This document contains the normative text of a proposal to fulfil two of the PAR objectives of the 11aa task group:

- Graceful degradation of audio video streams when there is insufficient channel capacity, by enabling packet discarding without any requirement for deep packet inspection,

- Intra-Access Category prioritization of transport streams by modifying EDCA timing and parameter selection without any requirement for deep packet inspection

This proposal allows the creation of up to 4 extra EDCA parameter sets to facilitate intra-access category prioritization and a method to subdivide AIFS.

This proposal also creates a new service called the “stream classification service” that allows the establishment of streams with layer 2 and/or layer 3 signalling of packet drop eligibility and allows classification of streams in to an existing or one of the new access categories.

This document is based upon

- IEEE P802.11-2007

- IEEE P802.11y-2008

- IEEE P802.11r-2008

- IEEE P802.11k-2008

- IEEE P802.11n draft 10.0

- IEEE P802.11p draft 6.0

- IEEE P802.11s draft 3.0

- IEEE P802.11u draft 7.0

- IEEE P802.11v draft 6.0

- IEEE P802.11w draft 9.0

- IEEE P802.11z draft 4.0

- IEEE P802.11aa draft 0.01

4. Abbreviations and acronyms

Add the following abbreviations:

AIFSF	arbitration interframe space fraction

DEI	drop eligibility indicator

SCS	Stream Classification Service

SCSID	Stream Classification Identifier

5. General description

5.2 Components of the IEEE 802.11 architecture

Insert the following clauses after 5.2.11:

5.2.12 Robust Audio Video Streaming

5.2.12.1 Overview

Robust Audio Video Streaming enables audio video streams to be transported over 802.11 for consumer/enterprise applications.

5.2.12.3 Stream Classification Service

The stream classification service enables the establishment of a stream that is classified using layer 2 and/or layer 3 signalling. Once classified, packets belonging to a stream are assigned to an Access Category and are tagged with their packet drop eligibility.

Move “5.2.10 More Reliable Groupcast” to 5.2.12.2

6. MAC service definition

6.1 Overview of MAC services

6.1.1 Data service

6.1.1.1 Determination of UP

Modifiy 6.1.1.1 as indicated.

The QoS facility supports eight priority values, referred to as UPs. The values a UP may take are the integer values from 0 to 7 and are identical to the IEEE 802.1D priority tags. An MSDU with a particular UP is said to belong to a traffic category (TC) with that UP. The UP is provided with each MSDU at the medium access control service access point (MAC_SAP) either directly, in the UP parameter, or indirectly, in a TSPEC or SCS Descriptor designated by the UP parameter.

7. Frame formats

7.1 MAC frame formats

7.1.3 Frame fields

7.1.3.5 QoS Control field

Add the following to 7.1.3.5.

Editorial note: The exact format of the information in the AVT field is to be defined and will be updated as the 11aa signalling requirements become clearer. The intention is to remove the AVT field by placing its subfields in existing MAC fields.

|  | B0 | B1 – B7 |  |
| --- | --- | --- | --- |
|  | DEI | Reserved |  |
| Bits: | 1 | 6 |  |
| Figure 7-aa17 —AVT field |  |  |  |

7.1.3.5.8 DEI subfield

The DEI subfield is 1 bit in length and is used by the STA to indicate that the corresponding MSDU, or fragment thereof, is eligible for discarding if there are insufficient resources at the destination STA.

How the destination STA determines that there are insufficient resources or when it decides to discard the MSDU is beyond the scope of this standard.

7.2 Format of individual frame types

7.2.3 Management frames

7.2.3.1 Beacon frame format

Insert order 49 information field into Table 7-8 as shown:.

| Table 7-8—Beacon frame body |  |  |
| --- | --- | --- |
| Order | Information | Notes |
| 49 | Alternate EDCA Parameter Set | The Alternate EDCA Parameter Set element is present if dot11RobustAVStreamingAlternateEDCAEnabled is set to true. |

7.2.3.5 Association Response frame format

Insert order 21 information field into Table 7-11:

| Table 7-11—Association Response frame body |  |  |
| --- | --- | --- |
| Order | Information | Notes |
| 21 | Alternate EDCA Parameter Set | The Alternate EDCA Parameter Set element is present if dot11RobustAVStreamingAlternateEDCAEnabled is set to true. |

7.2.3.7 Reassociation Response frame format

Insert order 25 information field into Table 7-13:

| Table 7-13—Reassociation Response frame body |  |  |
| --- | --- | --- |
| Order | Information | Notes |
| 25 | Alternate EDCA Parameter Set | The Alternate EDCA Parameter Set element is present if dot11RobustAVStreamingAlternateEDCAEnabled is set to true. |

7.2.3.9 Probe Response frame format

Insert order 44 information field into Table 7-15 as follows:

| Table 7-15—Probe Response frame body |  |  |
| --- | --- | --- |
| Order | Information | Notes |
| 44 | Alternate EDCA Parameter Set | The Alternate EDCA Parameter Set element is present if dot11RobustAVStreamingAlternateEDCAEnabled is set to true. |

7.3 Management frame body components

7.3.1 Fields that are not information elements

7.3.1.9 Status Code field

Insert status codes <ANA> and change the Reserved status code row in Table 7-23 as follows (note that the entire table is not shown here):

| Table 7-23—Status codes |  |
| --- | --- |
| Status code | Meaning |
| <ANA> | Requested TCLAS processing has been terminated by the AP |
| (<ANA>+1) 31— 65535 | Reserved |

Add the following sub-clause to the end of 7.3.1

Editorial note: 11n adds entries up to 7.3.1.30, 11u adds 7.3.1.31, 11v adds another entry.

7.3.1.33aa EDCA AC Parameter Record field

The EDCA AC Parameter Record field is defined in Figure 7-79.

Move Figure 7-79 from 7.3.2.29 to here.

The format of the ACI/AIFSN field is illustrated in Figure 7-80.

Move Figure 7-80 from 7.3.2.29 to here and modify as indicated.

|  | B0 – B3 | B4 | B5 – B6 | B7 |  |
| --- | --- | --- | --- | --- | --- |
|  | AIFSN | ACM | ACI | Reserved AIFSF |  |
| Bits: | 4 | 1 | 2 | 1 |  |
| Figure 7-80aa —ACI/AIFSN field |  |  |  |  |  |

Move to here the sixth paragraph onwards until the end of the subclause from 7.3.2.29 and modify as indicated.

The value of the AC index (ACI) references the AC to which all parameters in this record correspond. The mapping between ACI and AC is defined in Table 7-36aa. The ACM (admission control mandatory) subfield indicates that admission control is required for the AC. If the ACM subfield is set to 0, then there is no admission control for the corresponding AC. If the ACM subfield is set to 1, admission control has to be used prior to transmission using the access parameters specified for this AC. The AIFSN and AIFSF subfields indicates the number of slots after a SIFS duration a non-AP STA should defer before either invoking a backoff or starting a transmission. The minimum value for the AIFSN subfield is 2.

Move Figure 7-36 from 7.3.2.29 to here.

The ECWmin and ECWmax fields are illustrated in Figure 7-81

Move Figure 7-81 from 7.3.2.29 to here.

The ECWmin and ECWmax fields encode the values of CWmin and CWmax, respectively, in an exponent form. The ECWmin and ECWmax values are defined so that

CWmin = 2ECWmin – 1

CWmax = 2ECWmax – 1

Hence the minimum encoded value of CWmin and CWmax is 0, and the maximum value is 32767.

The value of the TXOP Limit field is specified as an unsigned integer, with the least significant octet transmitted first, in units of 32 μs. A TXOP Limit field value of 0 indicates that a single MSDU or MMPDU, in addition to a possible RTS/CTS exchange or CTS to itself, may be transmitted at any rate for each TXOP.

The default values used by non-AP STAs for the parameters in the EDCA Parameter Set element are defined in Table 7-37

Move table 7-37 from clause 7.3.2.29 and modify as indicated:

| Table 7-37—Default EDCA Parameter Set element parameter values |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | TXOP limit |  |  |  |
| AC | CWmin | CWmax | AIFSN | AIFSF | For PHYs defined in Clause 15 and Clause 18 | For PHYs defined in Clause 17 and, Clause 19 and Clause 20 |  | Other PHYs |
| AC_BE | aCWmin | aCWmax | 7 | 0 | 0 | 0 |  | 0 |
| AC_BK | aCWmin | aCWmax | 3 | 0 | 0 | 0 |  | 0 |
| AC_VI | (aCWmin+1)/2 - 1 | aCWmin | 2 | 0 | 6.016 ms | 3.008 ms |  | 0 |
| AC_VO | (aCWmin+1)/4 - 1 | (aCWmin+1)/2 - 1 | 2 | 0 | 3.264 ms | 1.504 ms |  | 0 |

Move table 7-37a from clause 7.3.2.29 and modify as shown:

Editorial note: Table 7-37a is defined in 11p

| Table 7-37a—Default EDCA parameter set for STA operation if dot11OCBEnabled is true |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| AC | CWmin | CWmax | AIFSN | AIFSF | TXOP Limit OFDM/CCKOFDM PHY |  |
| AC_BK | aCWmin | aCWmax | 9 | 0 | 0 |  |
| AC_BE | (aCWmin+1)/2-1 | aCWmin | 6 | 0 | 0 |  |
| AC_VI | (aCWmin+1)/4 - 1 | (aCWmin+1)/2-1 | 3 | 0 | 0 |  |
| AC_VO | (aCWmin+1)/4 - 1 | (aCWmin+1)/2 - 1 | 2 | 0 | 0 |  |

7.3.2 Information elements

Insert the following additional Element IDs <ANA> rows before the “Reserved” entry of Table 7-26 and adjust the “Reserved” entries as appropriate (note that the entire table is not shown here):

| Table 7-26—Element IDs |  |  |  |
| --- | --- | --- | --- |
| Information Element | Element ID | Length (in octets) | Extensible |
| Alternate EDCA Parameter Set element (see 7.3.2.89aa) | <ANA> | 18 | Subelements |
| SCS Request (see 7.3.2.91aa) | <ANA> | 3 to 257 | Yes |
| SCS Response (see 7.3.2.92aa) | <ANA> | 2 to 256 | Yes |
| Reserved | (<ANA>+1) 74 — 220 |  |  |

Change 7.3.2.27 as follows:

7.3.2.27 Extended Capabilities information element

Insert Bit(s) <ANA> and change the Reserved row in Table 7-35a as follows (note that the entire table is not shown here):

| Table 7-35a—Capabilities field |  |  |
| --- | --- | --- |
| Bit(s) | Information | Notes |
| <ANA> | SCS | The STA sets the SCS field to 1 when the MIB attribute dot11RobustAVStreamingSCSEnabled is set to true, and sets it to 0 otherwise. See 11.21.2 (SCS Procedures) |
| <ANA> | Alternate EDCA | The STA sets the Alternate EDCA field to 1 when the MIB attribute dot11RobustAVStreamingAlternateEDCAEnabled is set to true, and sets it to 0 otherwise. See 9.1.3.1 (HCF contention-based channel access) |

7.3.2.29 EDCA Parameter Set element

Modify clause 7.3.2.29 as indicated.

The EDCA Parameter Set element provides information needed by non-AP STAs for proper operation of the QoS facility during the CP. The format of the EDCA Parameter Set element is defined in Figure 7-78.

|  | Element ID (12) | Length (18) | QoS Info | Reserved | AC_BE Parameter Record | AC_BK Parameter Record | AC_VI Parameter Record | AC_VO Parameter Record |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 4 | 4 | 4 | 4 |  |
| Figure 7-78 —EDCA Parameter Set element |  |  |  |  |  |  |  |  |  |

The EDCA Parameter Set element is used by the AP to establish policy (by changing default MIB attribute values), to change policies when accepting new STAs or new traffic, or to adapt to changes in offered load. The most recent EDCA parameter set element received by a non-AP STA is used to update the appropriate MIB values.

The format of the QoS Info field is defined in 7.3.1.17. The QoS Info field contains the EDCA Parameter Set Update Count subfield, which is initially set to 0 and is incremented each time any of the AC parameters changes. This subfield is used by non-AP STAs to determine whether the EDCA parameter set has changed and requires updating the appropriate MIB attributes.

The formats of AC_BE, AC_BK, AC_VI, and AC_VO Parameter Record fields are identical and are illustrated in Figure 7-79 defined in 7.3.1.32aa.

Editorial note: All the text and figures from here to the end of 7.3.29 has been moved to 7.3.1.33aa

|  | ACI/AIFSN | ECWmin/ECWmax | TXOP Limit |  |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 |  |
| Figure 7-79 — AC_BE, AC_BK, AC_VI, and AC_VO Parameter Record field format |  |  |  |  |

The format of the ACI/AIFSN field is illustrated in Figure 7-80.

|  | B0 – B3 | B4 | B5 – B6 | B7 |  |
| --- | --- | --- | --- | --- | --- |
|  | AIFSN | ACM | ACI | Reserved |  |
| Bits: | 4 | 1 | 2 | 1 |  |
| Figure 7-80—ACI/AIFSN field |  |  |  |  |  |

The value of the AC index (ACI) references the AC to which all parameters in this record correspond. The mapping between ACI and AC is defined in Table 7-36. The ACM (admission control mandatory) subfield indicates that admission control is required for the AC. If the ACM subfield is set to 0, then there is no admission control for the corresponding AC. If the ACM subfield is set to 1, admission control has to be used prior to transmission using the access parameters specified for this AC. The AIFSN subfield indicates the number of slots after a SIFS duration a non-AP STA should defer before either invoking a backoff or starting a transmission. The minimum value for the AIFSN subfield is 2.

| Table 7-36—ACI-to-AC coding |  |  |  |
| --- | --- | --- | --- |
| ACI | AC | Description |  |
| 00 | AC_BE | Best effort |  |
| 01 | AC_BK | Background |  |
| 10 | AC_VI | Video |  |
| 11 | AC_VO | Voice |  |

The ECWmin and ECWmax fields are illustrated in Figure 7-81.

|  | B0 – B3 | B4 – B7 |  |
| --- | --- | --- | --- |
|  | ECWmin | ECWmax |  |
| Bits: | 4 | 4 |  |
| Figure 7-81aa —ECWmin and ECWmax fields |  |  |  |

The ECWmin and ECWmax fields encode the values of CWmin and CWmax, respectively, in an exponent form. The ECWmin and ECWmax values are defined so that

CWmin = 2ECWmin – 1

CWmax = 2ECWmax – 1

Hence the minimum encoded value of CWmin and CWmax is 0, and the maximum value is 32767.

The value of the TXOP Limit field is specified as an unsigned integer, with the least significant octet transmitted first, in units of 32 μs. A TXOP Limit field value of 0 indicates that a single MSDU or MMPDU, in addition to a possible RTS/CTS exchange or CTS to itself, may be transmitted at any rate for each TXOP.

The default values used by non-AP STAs for the parameters in the EDCA Parameter Set element are defined in Table 7-37.

Add the following clauses to the end of section 7.3.2:

Editorial note: 11v adds entries up to 7.3.2.88. 11u adds another 5 entries

7.3.2.94aa Alternate EDCA Parameter Set element

The Alternate EDCA Parameter Set element provides information needed by non-AP STAs for proper operation of the QoS facility during the CP. The format of the Alternate EDCA Parameter Set element is defined in Figure 7-aa7

|  | Element ID | Length (18) | QoS Info | Alternate Priority Info | Alternate AC_BE Parameter Record | Alternate AC_BK Parameter Record | Alternate AC_VI Parameter Record | Alternate AC_VO Parameter Record |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 4 | 4 | 4 | 4 |  |
| Figure 7-aa7 — Alternate EDCA Parameter Set element |  |  |  |  |  |  |  |  |  |

Editorial note: Figure 7-aa7 is the same as Figure 7-78, except the reserved field is replaced with the Alternate Priority Info field.

The format of the QoS Info field is defined in 7.3.1.17. The QoS Info field contains the EDCA Parameter Set Update Count subfield, which is initially set to 0 and is incremented each time any of the Alternate AC Parameter Records or Alternate Priority Info fields change.

The format of the Altenate Priority Info field is defined in Figure 7-aa8.

|  | B0 | B1 | B2 | B3 | B4-B7 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Alternate AC_BE Present | Alternate AC_BK Present | Alternate AC_VI Present | Alternate AC_VO Present | Reserved |  |
| Bits: | 1 | 1 | 1 | 1 | 4 |  |
| Figure 7-aa8—Alternate Priority Info field |  |  |  |  |  |  |

The Alternate AC_BE Present field is set to one to indicate that an Alternative AC_BE Parameter Record is present in this Alternate EDCA Parameter Set element. When the Alternate AC_BE Present field is zero, the Alternative AC_BE Parameter Record field is reserved.

The Alternate AC_BK Present field is set to one to indicate that an Alternative AC_BK Parameter Record is present in this Alternate EDCA Parameter Set element. When the Alternate AC_BK Present field is zero, the Alternative AC_BK Parameter Record field is reserved.

The Alternate AC_VI Present field is set to one to indicate that an Alternative AC_VI Parameter Record is present in this Alternate EDCA Parameter Set element. When the Alternate AC_VI Present field is zero, the Alternative AC_VI Parameter Record field is reserved.

The Alternate AC_VO Present field is set to one to indicate that an Alternative AC_VO Parameter Record is present in this Alternate EDCA Parameter Set element. When the Alternate AC_VO Present field is zero, the Alternative AC_VO Parameter Record field is reserved.

The formats of Alternate AC_BE, AC_BK, AC_VI, and AC_VO Parameter Record fields are identical and are defined in 7.3.1.32aa.

7.3.2.90aa Intra-Access Category Priority element

The Intra-Access Category Priority element provides information from a non-AP STA to an AP on the relative priorities of streams within an AC. This element may be included in ADDTS Request, QoS Map Configure or SCS Request frames. The Intra-Access Category Priority element is defined in Figure 7-aa9.

|  | Element ID | Length (1) | Intra-Access Priority |  |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 |  |
| Figure 7-aa9 — Intra-Access Category Priority element |  |  |  |  |

The Intra-Access Priority field is defined in Figure 7-aa10.

|  | B0 – B2 | B3 | B4 | B5-B7 |  |
| --- | --- | --- | --- | --- | --- |
|  | User Priority | Alternate Priority | Drop Eligibilty | Reserved |  |
| Bits: | 3 | 1 | 1 | 3 |  |
| Figure 7-aa10— Intra-Access Priority field |  |  |  |  |  |

The User Priority subfield indicates the value of the UP of the stream to which this stream relates.

The Alternate Priority subfield indicates which EDCA parameter set is used for this stream. When set to zero, the EDCA Parameter Set (see 7.3.2.29) is used. When the Alternate Priority subfield is set to one, the Alternate EDCA Parameter Set (7.3.2.89aa) is used.

The Drop Eligibility subfield is set to one to indicate that MPDUs for this stream may be dropped if there are insufficient resources to transmit all SCS streams. The Drop Eligibility subfield is set to zero to indicate that this stream does not support drop eligibility.

7.3.2.91aa SCS Request element

The SCS Request element defines information about the stream that is being classified using the procedures defined in 11.21.2. The format of the SCS Request element is shown in Figure 7-aa11.

|  | Element ID | Length | Request Type | SCS Descriptor List |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | variable |
| Figure 7-aa11—SCS Request element format |  |  |  |  |

The Element ID field is set to the SCS Request value in Table 7-26.

The value of the Length field is set to 1+n, where n indicates the total length of the SCS Descriptor List field.

The Request Type field is set to a number to identify the type of SCS request. The Request Types are shown in Table 7-aa7.

| Table 7-aa7—Request Type definitions |  |
| --- | --- |
| Name | Usage Mode |
| Add | 0 |
| Remove | 1 |
| Change | 2 |
| Reserved | 3-255 |

The SCS Descriptor List field contains one or more SCS Descriptors. The format of the SCS Descriptor is defined in Figure 7-aa12.

|  |  |  |  | zero or more TCLAS Elements |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | SCSID | Length | Intra-Access Category Priority Element (optional) | TCLAS Elements (optional) | TCLAS Processing Element (optional) |  |
| Octets: | 1 | 1 | 0 or 3 | variable | 0 or 3 |  |
| Figure 7-aa12 —SCS Descriptor |  |  |  |  |  |  |

The SCSID field is set to a non-zero value chosen by the non-AP STA identifying the SCS stream specified in the SCS Descriptor List field.

The Length field is set to the total length of presented Intra-Access Category Priority Element, TCLAS elements and presented TCLAS Processing element.

The Intra-Access Category Priority Element is present when Request Type is set to “Add” or “Change” and is defined in 7.3.2.90aa.

The TCLAS Elements field contains zero or more TCLAS information elements to specify how incoming MSDUs are classified as part of this SCS stream, as defined in 7.3.2.31. One or more TCLAS information elements are present when Request Type is set to “Add” or “Change” and no TCLAS information elements are present when Request Type is set to “Remove”.

The TCLAS Processing Element field is present when more than one TCLAS elements are present and defines how multiple TCLAS information elements are to be processed, as defined in 7.3.2.33.

The SCS Request element is included in SCS Request frames, as described in 7.4.2.5aa, and Reassociation Request frames, as described in 7.2.3.6. The use of the SCS Request element and frames is described in 11.21.2.

7.3.2.92aa SCS Response element

The SCS Response element provides the status information about the requested stream classification. The format of the SCS Response element is shown in Figure 7-aa13.

|  |  |  | one or more SCS Status |
| --- | --- | --- | --- |
|  | Element ID | Length | SCS Status List |
| Octets: | 1 | 1 | variable |
| Figure 7-aa13—SCS Response element format |  |  |  |

The Element ID field is set to the SCS Response value in Table 7-26.

The value of the Length field is the total length of the SCS Status List field.

The SCS Status List field contains one or more SCS Status duples. The format of the SCS Status duple is defined in Figure 7-aa14.

|  | SCSID | Status |
| --- | --- | --- |
| Octets: | 1 | 1 |
| Figure 7-aa14—SCS Status duple format |  |  |

The SCSID field is set to the value of the SCSID field in the SCS Descriptor received in the SCS Request frame.

The Status field indicates the status returned by the AP responding to the STA's requested SCSID, as indicated in Table 7-23.

The SCS Response element is included in SCS Response frames, as described in 7.4.2.6aa, and Reassociation Response frames, as described in 7.2.3.7. The use of the SCS Response element and frames is described in 11.21.2.

7.4 Action frame format details

7.4.1 Spectrum management action details

7.4.1.5 QoS Map Configure frame format

Editorial note: Clause 7.4.1.5 and Table 7-49a is defined in 11u

Modify Table 7-49a as follows.

| Table 7-49a—QoS Map configure frame body |  |  |
| --- | --- | --- |
| Order | Information |  |
| 0 | Category |  |
| 1 | Action |  |
| 2 | Qos Map Set |  |
| 3 – n | Intra-Access Category Priority elements (optional) |  |

Add the following to the end of 7.4.1.5

There may be zero or more Intra-Access Category Priority elements, which are defined in 7.3.2.90aa.

7.4.2 QoS Action frame details

Insert the following additional Action field values <ANA> rows before the “Reserved” entry of Table 7-45 and adjust the “Reserved” entries as appropriate (note that the entire table is not shown here):

| Table 7-45—QoS Action field values |  |  |
| --- | --- | --- |
| Action field value | Meaning |  |
| <ANA> | SCS Request |  |
| <ANA> | SCS Response |  |
| <ANA>+2 – 255 | Reserved |  |

7.4.2.1 ADDTS Request frame format

Change Table 7-46 by inserting new row as shown below.

Editorial note: 11u added Expedited bandwidth request element

| Table 7-46—ADDTS Request frame body |  |  |
| --- | --- | --- |
| Order | Information |  |
| 1 | Category |  |
| 2 | Action |  |
| 3 | Dialog token |  |
| 4 | TSPEC |  |
| 5 – n | TCLAS (optional) |  |
| n + 1 | TCLAS processing (optional) |  |
| n + 2 | Expedited bandwidth request element (optional) |  |
| n + 3 | Intra-Access Category Priority element (optional) |  |

Add the following to the last paragraph in 7.4.2.1.

There may be one Intra-Access Category Priority element, which is defined in 7.3.2.90aa.

Add the following clause to the end of section 7.4.2:

7.4.2.5aa SCS Request frame format

SCS Request frames are used to request the creation, modification or deletion of a stream classification using the procedures defined in 11.21.2.

The frame body of the SCS Request frame contains the information shown in Figure 7-aa15

|  | Category | Action | Dialog Token | SCS Request |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | variable |
| Figure 7-aa15—SCS Request frame body format |  |  |  |  |

The Category field is set to 1 (representing QoS).

The Action field is set to <ANA> (representing SCS request).

The Dialog Token field is set to a nonzero value that is unique among the SCS Request frames sent to the AP for which a corresponding SCS Response frame has not been received.

The SCS Request field is defined in 7.3.2.91aa.

7.4.2.6aa SCS Response frame format

The SCS Response frame is sent in response to an SCS Request frame using the procedures defined in 11.21.2. The frame body of an SCS Response frame contains the information shown in Figure 7-aa16.

|  | Category | Action | Dialog Token | SCS Response |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | variable |
| Figure 7-aa16—SCS Response frame body format |  |  |  |  |

The Category field is set to 1 (representing QoS).

The Action field is set to <ANA> (representing SCS response).

The Dialog Token field is set to the nonzero value of the corresponding SCS Request frame. If the SCS Report frame is being transmitted other than in response to an SCS Request frame, then the Dialog token is set to zero.

The SCS Response field is defined in 7.3.2.92aa.

9. MAC sublayer functional description

9.1 MAC architecture

9.1.3 Hybrid coordination function (HCF)

9.1.3.1 HCF contention-based channel access (EDCA)

Modify clause 9.1.3.1 as follows:

The EDCA mechanism provides differentiated, distributed access to the WM for STAs using eight different UPs. The EDCA mechanism defines at least four access categories (ACs) that provide support for the delivery of traffic with UPs at the STAs. The AC is derived from the UPs as shown in Table 9-1.

| Table 9-1 – UP-to-AC mappings |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Priority | UP (Same as 802.1D user priority) | 802.1D designation | AC | Alternate AC (if supported) | Designation (informative) |
| Lowest | 1 | BK | AC_BK | AC_BK | Background |
|  | 2 | - | AC_BK | AAC_BK | Background (alternate) |
|  | 0 | BE | AC_BE | AC_BE | Best Effort |
|  | 3 | EE | AC_BE | AAC_BE | Best Effort (alternate) |
|  | 4 | CL | AC_VI | AAC_VI | Video (alternate) |
|  | 5 | VI | AC_VI | AC_VI | Video |
|  | 6 | VO | AC_VO | AC_VO | Voice |
| Highest | 7 | NC | AC_VO | AAC_VO | Voice (alternate) |

For each AC, an enhanced variant of the DCF, called an enhanced distributed channel access function (EDCAF), contends for TXOPs using a set of EDCA parameters. When communicating outside the context of a BSS (dot11OCBEnabled is true), the EDCA parameters are the corresponding default values or are as set by the SME in the MIB attribute table dot11EDCATable. When communicating within a BSS, the EDCA parameters used are from the EDCA Parameter Set element, Alternate EDCA Parameter Set element, or from the default values for the parameters when no EDCA Parameter Set elements are is received from the AP of the BSS with which the STA is associated or when the STA is a mesh STA; the parameters used by the EDCAF to control its operation are defined by MIB attribute table dot11QAPEDCATable at the AP and by MIB attribute table dot11EDCATable at the non-AP STA.

Modify the third paragraph of 9.1.3.1 as indicated:

The QoS AP announces the EDCA parameters in selected Beacon frames and in all Probe Response and (Re) Association Response frames by the inclusion of the EDCA Parameter Set information element and if dot11RobustAVStreamingAlternateEDCAEnabled is true, the Alternate EDCA Parameter Set element. If no such element is received, the STAs shall use the default values for the parameters. The EDCA AC Parameter Record fields following the QoS Info field in the EDCA Parameter Set information element and Alternate EDCA Parameter Set element (if dot11RobustAVStreamingAlternateEDCAEnabled is true) shall be included in all Beacon frames occurring within two or more delivery traffic indication message (DTIM) periods following a change in AC parameters to assure that all STAs are able to receive the updated EDCA parameters. A QoS STA shall update its MIB values of the EDCA parameters within an interval of time equal to one beacon interval after receiving an updated EDCA parameter set. QoS STAs update the MIB attributes and store the EDCA Parameter Set update count value in the QoS Info field. An AP may change the EDCA access parameters by changing the EDCA Parameter Set element in the Beacon and Probe Response frames. However, the AP should change them only rarely. A QoS STA shall use the EDCA Parameter Set Update Count Value subfield in the QoS Capability element of all Beacon frames to determine whether the STA is using the current EDCA Parameter Values. If the EDCA Parameter Set update count value in the QoS Capability element is different from the value that has been stored, the QoS STA shall query the updated EDCA parameter values by sending a Probe Request frame to the AP.

9.2 DCF

9.2.3 IFS

9.2.3.4 AIFS

Modify the last paragraph of 9.2.3.4 as follows.

A non-AP QoS STA computes the time periods for each AIFS[AC] from the dot11EDCATableAIFSN and dot11EDCATableAIFSF attributes in the MIB. In an infrastructure BSS, QoS STAs update their dot11EDCATableAIFSN and dot11EDCATableAIFSF values using information in the most recent EDCA Parameter Set and Alternate EDCA Parameter Set elements of Beacon frames received from the AP of the BSS (see 7.3.2.2829 and 7.3.2.94aa). A QoS AP computes the time periods for each AIFS[AC] from the dot11QAPEDCATableAIFSN and dot11QAPEDCATableAIFSF attributes in its MIB.

9.2.4 Random backoff time

Modify the third paragraph as indicated.

The CW shall be reset to aCWmin after every successful attempt to transmit an MPDU or MMPDU, when SLRC reaches dot11LongRetryLimit for frames where the DEI subfield is zero or the QoS Control field is not present, or when SSRC reaches dot11ShortRetryLimit for frames where the DEI subfield is zero or the QoS Control field is not present, SLRC reaches dot11LongDEIRetryLimit for frames where the DEI subfield is one, or when SSRC reaches dot11ShortDEIRetryLimit for frames where the DEI subfield is one. The SSRC shall be reset to 0 when a CTS frame is received in response to an RTS frame, when an ACK frame is received in response to an MPDU or MMPDU transmission, or when a frame with a group address in the Address1 field is transmitted. The SLRC shall be reset to 0 when an ACK frame is received in response to transmission of an MPDU or MMPDU of length greater than dot11RTSThreshold, or when a frame with a group address in the Address1 field is transmitted.

9.2.5 DCF access procedure

9.2.5.3 Recovery procedures and retransmit limits

Modify the fith paragraph as indicated.

For frames where the DEI subfield is zero or the QoS Control field is not present, rRetries for failed transmission attempts shall continue until the SRC for the MPDU of type Data or MMPDU is equal to dot11ShortRetryLimit or until the LRC for the MPDU of type Data or MMPDU is equal to dot11LongRetryLimit. For frames where the DEI subfield is one, retries for failed transmission attempts shall continue until the SRC for the MPDU of type Data or MMPDU is equal to dot11ShortDEIRetryLimit or until the LRC for the MPDU of type Data or MMPDU is equal to dot11LongDEIRetryLimit. When either of these limits is reached, retry attempts shall cease, and the MPDU of type Data (and any MSDU of which it is a part) or MMPDU shall be discarded.

9.9 HCF

9.9.1 HCF contention-based channel access (EDCA)

9.9.1.2 EDCA TXOPs

Modify second paragraph in clause 9.9.1.2 as follows:

The TXOP limit duration values are advertised by the AP in the EDCA Parameter Set information and Alternate EDCA Parameter Set information elements in Beacon, Association Response, Ressociation Response and Probe Response frames transmitted by the AP. A TXOP limit value of 0 indicates that a single MSDU or MMPDU, in addition to a possible RTS/CTS exchange or CTS to itself, may be transmitted at any rate for each TXOP.

9.9.1.3 Obtaining an EDCA TXOP

Modify clause 9.9.1.3 as follows:

Each channel access timer shall maintain a backoff function (timer), which has a value measured in backoff slots.

The duration AIFS[AC] is a duration derived from the value AIFSN[AC] by the relation

AIFS[AC] = (AIFSN[AC] + (AIFSF[AC] & dot11QosTransmittedFrameCount [AC])) × aSlotTime + aSIFSTime.

The value of AIFSN[AC] shall be greater than or equal to 2 for non-AP STAs. In an infrastructure BSS

AIFSN[AC] is advertised by the AP in the EDCA Parameter Set information element in Beacon and Probe Response frames transmitted by the AP. A STA may also advertise AIFSN[AC] using the

EDCA Parameter Set information element in a Timing Advertisement frame. The value of AIFSN[AC] shall be greater than or equal to 1 for APs. An EDCA TXOP is granted to an EDCAF when the EDCAF determines that it shall initiate the transmission of a frame exchange sequence. Transmission initiation shall be determined according to the following rules:

On specific slot boundaries, each EDCAF shall make a determination to perform one and only one of the following functions:

— Initiate the transmission of a frame exchange sequence for that access function.

— Decrement the backoff timer for that access function.

— Invoke the backoff procedure due to an internal collision.

— Do nothing for that access function.

The specific slot boundaries at which exactly one of these operations shall be performed are defined as follows, for each EDCAF:

- Following AIFSN[AC] × aSlotTime - aSIFSTime – aRxTxTurnaroundTime of idle medium after SIFS (not necessarily idle medium during the SIFS duration) after the last busy medium on the antenna that was the result of a reception of a frame with a correct FCS.

- Following EIFS – DIFS + AIFSN[AC] × aSlotTime + aSIFSTime – aRxTxTurnaroundTime of idle medium after the last indicated idle medium as determined by the physical CS mechanism that was the result of a frame reception that has resulted in FCS error, or PHY-RXEND.indication (RXERROR) primitive where the value of RXERROR is not NoError.

- When any other EDCAF at this STA transmitted a frame requiring acknowledgment, the earlier of

  - The end of the ACK-Timeout interval timed from the PHY_TXEND.confirm primitive, followed by AIFSN[AC] x aSlotTime + aSIFSTime  – aRxTxTurnaroundTime of idle medium, and

  - The end of the first AIFSN[AC] × aSlotTime - aSIFSTime – aRxTxTurnaroundTime of idle medium after SIFS (not necessarily medium idle during the SIFS duration, the start of the SIFS duration implied by the length in the PLCP header of the previous frame) when a PHYRXEND. indication primitive occurs as specified in 9.2.8.

- Following AIFSN[AC] × aSlotTime - aSIFSTime – aRxTxTurnaroundTime of idle medium after SIFS (not necessarily medium idle during the SIFS duration) after the last busy medium on the antenna that was the result of a transmission of a frame for any EDCAF and which did not require an acknowledgment.

- Following AIFSN[AC] × aSlotTime + aSIFSTime – aRxTxTurnaroundTime of idle medium after the last indicated idle medium as indicated by the CS mechanism that is not covered by a) through d).

- Following aSlotTime of idle medium, which occurs immediately after any of these conditions, a) through f), is met for the EDCAF.

9.9.1.5 EDCA backoff procedure

Modify the fifth paragraph of 9.9.1.5 as indicated.

If the backoff procedure is invoked because of a failure event [either reason c) or d) above], the value of CW[AC] shall be updated as follows before invoking the backoff procedure:

- If the QSRC[AC] or the QLRC[AC] for the QoS STA or mesh STA has reached dot11ShortRetryLimit /dot11ShortDEIRetryLimit or dot11LongRetryLimit/dot11LongDEIRetryLimit respectively, CW[AC] shall be reset to CWmin[AC].

- Otherwise,

- If CW[AC] is less than CWmax[AC], CW[AC] shall be set to the value (CW[AC] + 1)*2 – 1.

- If CW[AC] is equal to CWmax[AC], CW[AC] shall remain unchanged for the remainder of any retries.

9.9.3 Admission Control at the HC

9.9.3.1 Contention-based admission control procedures

Modify the second paragraph of 9.9.3.1 as indicated.

The AP uses the ACM (admission control mandatory) subfields advertised in the EDCA Parameter Set and Alternate EDCA Parameter Set elements to indicate whether admission control is required for each of the ACs. While the CWmin, CWmax, AIFS, TXOP limit parameters may be adjusted over time by the AP, the ACM bit shall be static for the duration of the lifetime of the BSS. An ADDTS Request frame shall be transmitted by a non-AP STA to the HC in order to request admission of traffic in any direction (i.e., uplink, downlink, direct, or bidirectional) employing an AC that requires admission control. The ADDTS Request frame shall contain the UP associated with the traffic and shall indicate EDCA as the access policy. The AP shall associate the received UP of the ADDTS Request frame with the appropriate AC per the UP-to-AC mappings described in 9.1.3.1. The non-AP STA may transmit unadmitted traffic for the ACs for which the AP does not require admission control. If a STA desires to send data without admission control using an AC that mandates admission control, the STA shall use EDCA parameters that correspond to a lower priority and do not require admission control. All ACs with priority higher than that of an AC with an ACM flag equal to 1 should have the ACM flag set to 1.

10. Layer management

10.3 MLME SAP Interface

10.3.6 Associate

10.3.6.2 MLME-ASSOCIATE.confirm

10.3.6.2.2 Semantics of the service primitive

Change the primitive parameter list in 10.3.6.2.2 as shown:

MLME-ASSOCIATE.confirm (

ResultCode,

CapabilityInformation,

AssociationID,

SupportedRates,

EDCAParameterSet,

RCPI.request,

RSNI.request,

RCPI.response,

RSNI.response,

RRMEnabledCapabilities

Content of FT Authentication Information Elements,

SupportedRegulatoryClasses,

HT Capabilities,

Extended Capabilities,

20/40 BSS Coexistence,

BSSMaxIdlePeriod,

TIMBroadcastResponse,

TimeStamp,

QosMapSet,

Alternate EDCAParameterSet,

VendorSpecificInfo

)

Insert the following row before the VendorSpecificInfo row of the untitled table defining the primitive parameters in 10.3.6.1.2:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Alternate EDCAParameterSet | As defined in frame format | As defined in frame format | Specifies the Alternate EDCA parameter set that the STA should use. The parameter shall be present only if the MIB attribute dot11RobustAVStreamingAlternateEDCAEnabled is true. |

10.3.6.4 MLME-ASSOCIATE.response

10.3.6.4.2 Semantics of the service primitive

Change the primitive parameter list in 10.3.6.4.2 as shown:

MLME-ASSOCIATE.response(

PeerSTAAddress 

ResultCode,

CapabilityInformation,

AssociationID,

Content of FT Authentication Information Elements

SupportedRegulatoryClasses,

HT Capabilities,

Extended Capabilities, 

20/40 BSS Coexistence,

BSSMaxIdlePeriod,

TIMBroadcastResponse,

QoSMapSet,

Alternate EDCAParameterSet,

VendorSpecificInfo

)

Insert the following row before the VendorSpecificInfo row of the untitled table defining the primitive parameters in 10.3.6.4.2:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Alternate EDCAParameterSet | As defined in frame format | As defined in frame format | Specifies the Alternate EDCA parameter set that the STA should use. The parameter shall be present only if the MIB attribute dot11RobustAVStreamingAlternateEDCAEnabled is true. |

10.3.7.2 MLME-REASSOCIATE.confirm

10.3.7.2.2 Semantics of the service primitive

Change the primitive parameter list in 10.3.7.2.2 as shown:

MLME-REASSOCIATE.confirm(

ResultCode,

CapabilityInformation,

AssociationID,

SupportedRates,

EDCAParameterSet,

Content of FT Authentication Information Elements, 

SupportedRegulatoryClasses,

HT Capabilities,

Extended Capabilities, 

20/40 BSS Coexistence,

BSSMaxIdlePeriod,

TIMBroadcastResponse,

QoSMapSet,

Alternate EDCAParameterSet,

VendorSpecificInfo

)

Insert the following row before the VendorSpecificInfo row of the untitled table defining the primitive parameters in 10.3.7.2.2:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Alternate EDCAParameterSet | As defined in frame format | As defined in frame format | Specifies the Alternate EDCA parameter set that the STA should use. The parameter shall be present only if the MIB attribute dot11RobustAVStreamingAlternateEDCAEnabled is true. |

10.3.24 TS management interface

10.3.24.1 MLME- ADDTS.request

10.3.24.1.2 Semantics of the service primitive

Change the primitive parameter list in 10.3.24.1.2 as shown:

MLME-ADDTS.request(

DialogToken,

TSPEC,

TCLAS,

TCLASProcessing,

ADDTSFailureTimeout,

EBR,

IntraAccessCategoryPriority,

VendorSpecificInfo

)

Insert the following row before the VendorSpecificInfo row of the untitled table defining the primitive parameters in 10.3.24.1.2:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| IntraAccessCategoryPriority | As defined in frame format | As defined in 7.3.2.90aa | Specifies the intra-AC priorities the STA should use. The parameter shall be present only if the MIB attribute dot11RobustAVStreamingSCSEnabled is true. |

10.3.64 QoS Map Set element management

10.3.64.1 MLME-QoSMap.request

10.3.64.1.2 Semantics of the service primitive

Change the primitive parameter list in 10.3.64.1.2 as shown:

MLME-QoSMap.request (

Non-APSTAAddress,

QoSMapSet,

Intra-Access Category Priority elements

)

Insert the following row before the VendorSpecificInfo row of the untitled table defining the primitive parameters in 10.3.64.1.2:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Intra-Access Category Priority elements | As defined in frame format | As defined in 7.3.2.90aa | Specifies the intra-AC priorities the STA should use. The parameter shall be present only if the MIB attribute dot11RobustAVStreamingSCSEnabled is true. |

10.3.64.2 MLME-QoSMap.indication

10.3.64.2.2 Semantics of the service primitive

Change the primitive parameter list in 10.3.64.2.2 as shown:

MLME-QoSMap.indication (

QoSMapSet,

Intra-Access Category Priority elements

)

Insert the following row before the VendorSpecificInfo row of the untitled table defining the primitive parameters in 10.3.64.2.2:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Intra-Access Category Priority elements | As defined in frame format | As defined in 7.3.2.90aa | Specifies the intra-AC priorities the STA should use |

Add the following clause at the end of 10.3.

10.3.67 SCS request and response procedure

The following MLME primitives support the signaling of SCS request and response procedure. The informative diagram shown in Figure 10-aa1 depicts the SCS request and response process and is not meant to be exhaustive of all possible protocol uses.

Figure 10-aa1 – Example SCS Setup and Termination Protocol Exchange

10.3.67.1 MLME-SCS.request

10.3.67.1.1 Function

This primitive requests the transmission of an SCS Request frame be sent to an AP.

10.3.67.1.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-SCS.request	(
			PeerSTAAddress,
			Dialog Token,
			SCSRequest)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MAC Address | Any valid individual MAC Address | Specifies the address of the peer MAC entity with which to perform the SCS process. |
| Dialog Token | Integer | 1– 255 | The Dialog Token to identify the SCS request and response transaction. |
| SCSRequest | SCS Request element | SCS Request element, as defined in 7.3.2.91aa | Specifies frame classifiers and priority for the requested SCS stream. |

10.3.67.1.3 When Generated

This primitive is generated by the SME to request that a SCS Request frame be sent to the AP with which the STA is associated.

10.3.67.1.4 Effect of Receipt

On receipt of this primitive, the MLME constructs a SCS Request action management frame. The STA then attempts to transmit this to the AP with which the STA is associated.

10.3.67.2 MLME-SCS.confirm

10.3.67.2.1 Function

This primitive reports the result of a SCS procedure.

10.3.67.2.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-SCS.confirm(
			ResultCode,
			PeerSTAAddress,
			Dialog Token, 
			SCSResponse)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, DECLINED, TCLAS_PROCESSING_NOT_SUPPORTED, INSUFFICIENT_TCLAS_PROCESSING, TIMEOUT | Reports the outcome of a request to send a SCS Request frame. |
| PeerSTAAddress | MAC Address | Any valid individual MAC Address | Specifies the address of the peer MAC entity with which to perform the SCS process. |
| Dialog Token | Integer | 1 – 255 | The Dialog Token to identify the SCS request and response transaction. |
| SCSResponse | SCS Response element | SCS Response element, as defined in 7.3.2.92aa | Specifies the status returned by the AP responding to the STA's requested SCSID. |

10.3.67.2.3 When Generated

This primitive is generated by the MLME as a result of an MLME-SCS.request and indicates the results of the request.

This primitive is generated when the MLME-SCS.request contains invalid parameters, when a timeout or failure occurs, or when the STA receives a SCS Response frame from the AP.

10.3.67.2.4 Effect of Receipt

On receipt of this primitive the SME should operate according to the procedure in 11.21.2.

10.3.67.3 MLME-SCS.indication

10.3.67.3.1 Function

This primitive indicates that an SCS Request frame was received from a non-AP STA.

10.3.67.3.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-SCS.indication(
PeerSTAAddress,
Dialog Token,
SCSRequest)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MACAddress | Any valid individual MAC Address | The address of the non-AP STA MAC entity from which an SCS Request frame was received. |
| Dialog Token | Integer | 1 – 255 | The Dialog Token to identify the SCS request and response transaction. |
| SCSRequest | SCS Request element | SCS Request element, as defined in 7.3.2.91aa | Specifies frame classifiers and priority for the requested SCS stream. |

10.3.67.3.3 When Generated

This primitive is generated by the MLME when a valid SCS Request frame is received.

10.3.67.3.4 Effect of Receipt

On receipt of this primitive the SME should operate according to the procedure in 11.21.2.

10.3.67.4 MLME-SCS.response

10.3.67.4.1 Function

This primitive is generated in response to an MLME-SCS.indication requesting an SCS Response frame is sent to a non-AP STA.

10.3.67.4.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-SCS.response(
PeerSTAAddress,
Dialog Token, 
SCSResponse)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MACAddress | Any valid individual MAC Address | The address of the non-AP STA MAC entity from which a SCS Request frame was received. |
| Dialog Token | Integer | 1 – 255 | The Dialog Token to identify the SCS request and response transaction. |
| SCSResponse | SCS Response element | SCS Response element, as defined in 7.3.2.92aa | Specifies the status returned by the AP responding to the STA's requested SCSID. |

10.3.67.4.3 When Generated

This primitive is generated by the SME in response to an MLME-SCS.indication requesting an SCS Response be sent to a non-AP STA.

10.3.67.4.4 Effect of Receipt

On receipt of this primitive, the MLME constructs a SCS Response frame. The STA then attempts to transmit this to the non-AP STA indicated by the PeerSTAAddress parameter.

10.3.67.5 MLME-SCS.termrequest

10.3.67.5.1 Function

This primitive requests the transmission of a SCS Response frame to a non-AP STA to terminate an established SCS stream.

10.3.67.5.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-SCS.termrequest(
PeerSTAAddress,
Dialog Token, 
SCSResponse)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MACAddress | Any valid individual MAC Address | The address of the non-AP STA MAC entity to which the SCS Response frame is to be sent. |
| Dialog Token | Integer | 0 | Set to 0 for an autonomous SCS Response frame. |
| SCSResponse | SCS Response element | SCS Response element, as defined in 7.3.2.92aa | Specifies the requested SCSID that is cancelled by the AP. |

10.3.67.5.3 When Generated

This primitive is generated by the SME to terminate an SCS stream.

10.3.67.5.4 Effect of Receipt

On receipt of this primitive, the MLME constructs an SCS Response frame. The STA then attempts to transmit this to the non-AP STA indicated by the PeerSTAAddress parameter.

10.3.67.6 MLME-DMS.termindication

10.3.67.6.1 Function

This primitive is generated by the MLME when a valid unsolicated SCS Response frame is received.

10.3.67.6.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-SCS.termindication(
			ResultCode,
			Dialog Token, 
			SCSResponse)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, FAILURE | Indicates the result of the MLME-SCS.termrequest primitive. |
| Dialog Token | Integer | 0 | Set to 0 for an autonomous SCS Response frame. |
| SCSResponse | SCS Response element | SCS Response element, as defined in 7.3.2.92aa | Specifies the requested SCSID that is cancelled by the AP. |

10.3.67.6.3 When Generated

This primitive is generated when the STA receives an unsoliciated SCS Response frame from the AP.

10.3.64.6.4 Effect of Receipt

On receipt of this primitive the SME should operate according to the procedure in 11.21.2.

11. MLME

11.4 TS operation

11.4.1 Introduction

Modify the second and third paragraphs of 11.4.1 as indicated.

A TS may have one or more TCLAS (within the discretion of the STA that sets up the stream) associated with it. The AP uses the parameters in the TCLAS elements to filter the MSDUs belonging to this TS so that they can be delivered with the QoS parameters that have been set up for the TS. An Intra-Access Category Priority element may be associated with a TS. The User Priority subfield of the Intra-Access Priority element shall be set to the same User Priority as specified in the User Priority subfield of the TS Info field. The Alternate Priority subfield is used to select the appropriate EDCA parameter set if the Access Policy subfield of the TS Info field in the TSPEC is EDCA or HEMM. The Drop Eligibility subfield is used to tag the drop eligibility of the MSDUs of this TS.

TSPEC and the optional TCLAS and Intra-Access Priority elements are transported on the air by the ADDTS, in the corresponding QoS Action frame and across the MLME SAP by the MLME-ADDTS primitives.

11.4.6 Data transfer

Add the following to the end of 11.4.6.

If an Intra-Access Priority element was provided when the TS was created and the Access Policy subfield of the TS Info field in the TSPEC is EDCA or HEMM, the value from the Alternate Priority subfield is used to select the appropriate EDCA parameter set.

If an Intra-Access Priority element was provided when the TS was created, the Drop Eligibility subfield is used to tag the drop eligibility of the MSDU.

11.21 Robust AV Streaming

11.21.2 SCS Procedures

The Stream Classification Service (SCS) is a service that may be provided by an AP to its associated STAs that support the SCS service, where the AP classifies incoming MSDUs based upon parameters provided by the non-AP STA.

The classification allows User Priority, Drop Eligibility and EDCA Parameter Set to be selected for all MSDUs matching the classification.

Implementation of SCS is optional for a STA. A STA that implements SCS has the MIB attribute dot11RobustAVStreamingSCSImplemented set to true. A STA that has a value of true for the MIB attribute dot11RobustAVStreamingSCSEnabled is defined as a STA that supports Stream Classification. A STA for which MIB attribute dot11RobustAVStreamingSCSEnabled is true shall set the SCS field of the Extended Capabilities information element to 1.

A non-AP STA that supports SCS may request use of SCS by sending an SCS Request frame frame that includes SCS Request element with the Request Type field set to “Add” or “Change”. The SCS Descriptor List field in the SCS Request element identifies how MSDUs are classified and the priority to assign to MSDUs that match this classification.

Each SCS stream is identified by an SCSID. This SCSID is used by a non-AP STA to request creation, modification or deletion of an SCS stream. The SCSID is used by an AP to identify an SCS stream in SCS responses.

Upon receipt of an SCS Request frame from an associated non-AP STA, the AP shall respond with a corresponding SCS Response frame. A status value of “Successful” shall be set in the corresponding Status field of the SCS Status in the SCS Response element when the AP accepts the SCS request for the requested SCSID. A status value of “The request has been declined”, “Requested TCLAS processing is not supported by the AP”, or “The AP has insufficient TCLAS processing resources to satisfy the request” shall be set in the corresponding SCS Status field of the SCS Status in the SCS Response element when the AP denies the SCS request for the requested SCSID.

If the AP declines a request to change a previously accepted SCSID, the previously accepted classification for this SCSID shall continue to operate.

If the requested SCS is accepted by the AP, the AP shall process subsequent incoming unicast MSDUs from the DS or WM that match the classifier specified in the SCS Descriptor. The processing of matching MSDUs depends upon the access policy assigned to the MSDU:

- For matching MSDUs that are not part of a TS (as described in 11.4), the User Priority subfield of the Intra-Access Priority element is used as the UP of these MSDUs.

- For matching MSDUs that are not part of a TS (as described in 11.4), or MSDUs that are part of a TS that uses EDCA or HEMM as the Access Policy, the Alternate Priority subfield of the Intra-Access Priority element is used to select the EDCA Parameter Set for these MSDUs, if the MIB attribute dot11RobustAVStreamingAlternateEDCAEnabled is true.

- All matching MSDUs are tagged with their drop eligibility using the value from the Drop Eligibility subfield of the Intra-Access Priority element in the DEI subfield of the AVT field, as defined in 7.1.3.5.8.

A non-AP STA may request the termination of an accepted SCS stream by sending an SCS Request frame with the Request Type set to “Remove” and the requested SCSIDs in the SCS Request element. The length field of the SCS Descriptor is set to 0 and no Intra-Access Priority, TCLAS or TCLAS Processing elements shall be included in the SCS Descriptor.

Upon reception of a request to terminate a previously accepted SCS stream, the AP shall cease to apply the classifier related to this SCSID. The AP shall send an SCS Response frame to confirm the termination of the SCS stream identified by the SCSID, by including the SCSID and a Status value of “Terminate” in the SCS Status field of an SCS Response element and the dialog token in the SCS Response frame set to the value from the SCS Request frame that requested termination.

The AP may send an unsolicited SCS Response frame at any time to cancel a granted SCS stream identified by the SCSID, by including the SCSID and a Status value of “Terminate” in the SCS Status field of an SCS Response element and the dialog token in the SCS Response frame set to 0.

Annex A

(normative)

Protocol Implementation Conformance Statement (PICS) proforma

A.2 Abbreviations and special symbols

A.2.2 General abbreviations for Item and Support columns

Insert one new list item at the end of A.2.2 as indicated below:

AVT	audio video transport

A.4 PICS proforma–IEEE Std. 802.11, 2007 Edition

A.4.3 IUT configuration

Insert this entry to the end of the IUT configuration table:

| Item | IUT configuration | References | Status | Support |
| --- | --- | --- | --- | --- |
| *CFaa | Is RobustAVT supported? | 5.2.12 | (CF12):O | Yes, No, N/A |

Insert this new clause after A.4.22:

EDITORIAL NOTE—last clause in 11u is A.4.22.

A.4.23 RobustAVT extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| Item | Protocol Capability | References | Status | Support |
| AVT1 | Extended Capabilities information element | 7.3.2.27 | CFaa:M | Yes, No, N/A |
| AVT2 | More Reliable Groupcast | 9.2.7.3 | CFaa: <TBD> | Yes, No, N/A |
| AVT3 | Alternate EDCA Parameter Set | 9.1.3.1 | CFaa:M | Yes, No, N/A |
| AVT4 | Stream Classification Service | 11.21.2 | CFaa:O | Yes, No, N/A |
| AVT4.1 | Event Request frame | 7.4.2.5aa | AVT4:M | Yes, No, N/A |
| AVT4.2 | Event Report frame | 7.4.2.6aa | AVT4:M | Yes, No, N/A |

Annex D

(normative)

ASN.1 encoding of the MAC and PHY MIB

Change the “Dot11StationConfigEntry” of the “dotStationConfig TABLE” as follows:

-- *********************************************************************

-- * dotStationConfig TABLE

-- *********************************************************************

Dot11StationConfigEntry ::=
SEQUENCE {
	dot11StationID 						MacAddress,
	dot11MediumOccupancyLimit 					INTEGER,
	dot11CFPollable 						TruthValue,
	dot11CFPeriod 						INTEGER,
	dot11CFPMaxDuration 					INTEGER,
	dot11AuthenticationResponseTimeOut 				Unsigned32,
	dot11PrivacyOptionImplemented 				TruthValue,
	dot11PowerManagementMode 					INTEGER,
	dot11DesiredSSID 						OCTET STRING,
	dot11DesiredBSSType 					INTEGER,
	dot11OperationalRateSet 					OCTET STRING,
	dot11BeaconPeriod 						INTEGER,
	dot11DTIMPeriod 						INTEGER,
	dot11AssociationResponseTimeOut 				Unsigned32,
	dot11DisassociateReason 					INTEGER,
	dot11DisassociateStation 					MacAddress,
	dot11DeauthenticateReason 					INTEGER,
	dot11DeauthenticateStation 					MacAddress,
	dot11AuthenticateFailStatus 					INTEGER,
	dot11AuthenticateFailStation 				MacAddress,
	dot11MultiDomainCapabilityImplemented 			TruthValue,
	dot11MultiDomainCapabilityEnabled 				TruthValue,
	dot11CountryString 					OCTET STRING,
	dot11SpectrumManagementImplemented 				TruthValue,
	dot11SpectrumManagementRequired 				TruthValue,
	dot11RSNAOptionImplemented 					TruthValue,
	dot11RSNAPreauthenticationImplemented 			TruthValue,
	dot11RegulatoryClassesImplemented 				TruthValue,
	dot11RegulatoryClassesRequired 				TruthValue,
	dot11QosOptionImplemented 					TruthValue,
	dot11ImmediateBlockAckOptionImplemented 			TruthValue,
	dot11DelayedBlockAckOptionImplemented 			TruthValue,
	dot11DirectOptionImplemented 				TruthValue,
	dot11APSDOptionImplemented 					TruthValue,
	dot11QAckOptionImplemented 					TruthValue,
	dot11QBSSLoadOptionImplemented 				TruthValue,
	dot11QueueRequestOptionImplemented 				TruthValue,
	dot11TXOPRequestOptionImplemented 				TruthValue,
	dot11MoreDataAckOptionImplemented 				TruthValue,
	dot11AssociatedinNQBSS 					TruthValue,
	dot11DLSAllowdInQBSS 					TruthValue,
	dot11DLSAllowed 						TruthValue,
	dot11AssociateStation 					MacAddress,
	dot11AssociateID 						INTEGER,
	dot11AssociateFailStation 					MacAddress,
	dot11AssociateFailStatus 					INTEGER,
	dot11ReassociateStation 					MacAddress,
	dot11ReassociateID 					INTEGER,
	dot11ReassociateFailStation 					MacAddress,
	dot11ReassociateFailStatus 					INTEGER,
	dot11RadioMeasurementCapable 				TruthValue,
	dot11RadioMeasurementEnabled 				TruthValue,
	dot11RadioMeasurementProbeDelay 				INTEGER,
	dot11MeasurementPilotReceptionEnabled 			TruthValue,
	dot11MeasurementPilotTransmissionEnabled 			TruthValue,
	dot11MeasurementPilotTransmissionVirtualApSetEnabled 		TruthValue,
	dot11MeasurementPilotPeriod 					INTEGER,
	dot11LinkMeasurementEnabled 					TruthValue,
	dot11NeighborReportEnabled 					TruthValue,
	dot11ParallelMeasurementsEnabled 				TruthValue,
	dot11TriggeredMeasurementsEnabled 				TruthValue,
	dot11RepeatedMeasurementsEnabled 				TruthValue,
	dot11MeasurementPauseEnabled 				TruthValue,
	dot11QuietIntervalEnabled 					TruthValue,
	dot11PassiveBeaconMeasurementEnabled 				TruthValue,
	dot11ActiveBeaconMeasurementEnabled 				TruthValue,
	dot11TableBeaconMeasurementEnabled 				TruthValue,
	dot11ReportingConditionsEnabled 				TruthValue,
	dot11FrameMeasurementEnabled 				TruthValue,
	dot11ChannelLoadEnabled 					TruthValue,
	dot11NoiseHistogramEnabled 					TruthValue,
	dot11StatisticsReportEnabled 				TruthValue,
	dot11LCIReportEnabled 					TruthValue,
	dot11TransmitStreamMeasurementEnabled 			TruthValue,
	dot11APChannelReportEnabled 					TruthValue,
	dot11AnnexQMIBSupportEnabled 				TruthValue,
	dot11NonOperatingChannelMeasurementsEnabled 			TruthValue,
	dot11MaximumMeasurementDuration 				Unsigned32,
	dot11MeasurementPilotSupport 				Unsigned32,
	dot11FastBSSTransitionImplemented 				TruthValue,
	dot11LCIDSEImplemented 					TruthValue, 
	dot11LCIDSERequired 					TruthValue, 
	dot11DSERequired 						TruthValue, 
	dot11ExtendedChannelSwitchEnabled 				TruthValue, 
	dot11HighThroughputOptionImplemented 				TruthValue,
	dot11WirelessManagementImplemented 				TruthValue, 
	dot11MaxIdlePeriod 					INTEGER,
	dot11TIMBroadcastInterval     				INTEGER,
	dot11TIMBroadcastOffset       				INTEGER,
	dot11MinTriggerTimeout     				INTEGER,
	dot11RRMCivicMeasurementEnabled 				TruthValue,
	dot11RRMIdentifierMeasurementEnabled 			TruthValue,
dot11DMSMaxSTAS       					INTEGER,
dot11DMSMaxChannelLoadForNewService 				INTEGER,
dot11DMSMaxChannelLoad 					INTEGER,
dot11UTCTSFDTIMInterval 					INTEGER,
dot11UTCTSFOffsetTimeError 					INTEGER,
dot11UTCTSFOffsetTimeValue 					INTEGER,

dot11RobustAVStreaming 					TruthValue,

dot11RobustAVStreamingAdvancedMRG 				TruthValue,

dot11RobustAVStreamingAlternateEDCAEnabled			TruthValue,

dot11RobustAVStreamingSCSImplemented			TruthValue,

dot11RobustAVStreamingSCSEnabled				TruthValue

}

Change the “dot11EDCATable” of the “SMT EDCA Config TABLE” as shown:

dot11EDCATable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11EDCAEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Conceptual table for EDCA default parameter values at a non-AP

STA. This table shall contain the foureight entries of the EDCA

parameters corresponding to foureight possible ACs. Index 1 corresponds

to AC_BK, index 2 to AC_BE, index 3 to AC_VI, and index 4 to AC_VO.

Index 5 corresponds to AAC_BK, index 6 to AAC_BE,

index 7 to AAC_VI, and index 8 to AAC_VO."

REFERENCE

"IEEE 802.11-2007, 9.1.3.1"

::= { dot11mac 4 }

Modify “Dot11EDCAEntry” as shown:

Dot11EDCAEntry ::=

SEQUENCE { dot11EDCATableIndex INTEGER,

dot11EDCATableCWmin INTEGER,

dot11EDCATableCWmax INTEGER,

dot11EDCATableAIFSN INTEGER,

dot11EDCATableTXOPLimit INTEGER,

dot11EDCATableMSDULifetime INTEGER,

dot11EDCATableMandatory TruthValue,

dot11EDCATableAIFSF INTEGER

}

Modify “dot11EDCATableIndex” as shown:

dot11EDCATableIndex OBJECT-TYPE

SYNTAX INTEGER (1..48)

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The auxiliary variable used to identify instances of the columnar

objects in the EDCA Table. The value of this variable is

1) 1, if the value of the AC is AC_BK.

2) 2, if the value of the AC is AC_BE.

3) 3, if the value of the AC is AC_VI.

4) 4, if the value of the AC is AC_VO.

5) 5, if the value of the AC is AAC_BK.

6) 6, if the value of the AC is AAC_BE.

7) 7, if the value of the AC is AAC_VI.

8) 8, if the value of the AC is AAC_VO.

"

::= { dot11EDCAEntry 1 }

Insert the following element after “dot11EDCATableMandatory”:

dot11EDCATableAIFSF OBJECT-TYPE

SYNTAX INTEGER (0..1)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the number of fractional slots, after a SIFS

duration, that the STA, for a particular AC, shall sense the

medium idle either before transmitting or executing a backoff. The

default value for this attribute is 0."

::= { dot11EDCAEntry aa3 }

Change the “dot11QAPEDCATable” of the “SMT AP EDCA Config TABLE” as shown:

dot11QAPEDCATable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11QAPEDCAEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Conceptual table for EDCA default parameter values at the AP.

This table shall contain the foureight entries of the EDCA parameters

corresponding to foureight possible ACs. Index 1 corresponds to AC_BK,

index 2 to AC_BE, index 3 to AC_VI, and index 4 to AC_VO.

Index 5 corresponds to AAC_BK, index 6 to AAC_BE,

index 7 to AAC_VI, and index 8 to AAC_VO."

REFERENCE

"IEEE 802.11-2007, 9.9.1"

::= { dot11mac 5 }

Modify “dot11QAPEDCAEntry” as shown:

Dot11QAPEDCAEntry ::=

SEQUENCE { 	dot11QAPEDCATableIndex INTEGER,

dot11QAPEDCATableCWmin INTEGER,

dot11QAPEDCATableCWmax INTEGER,

dot11QAPEDCATableAIFSN INTEGER,

dot11QAPEDCATableTXOPLimit INTEGER,

dot11QAPEDCATableMSDULifetime INTEGER,

dot11QAPEDCATableMandatory TruthValue,

dot11QAPEDCATableAIFSF INTEGER }

Modify “dot11QAPEDCATableIndex” as shown:

dot11QAPEDCATableIndex OBJECT-TYPE

SYNTAX INTEGER (1..48)

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The auxiliary variable used to identify instances of the columnar

objects in the EDCA Table. The value of this variable is

1) 1, if the value of the AC is AC_BK.

2) 2, if the value of the AC is AC_BE.

3) 3, if the value of the AC is AC_VI.

4) 4, if the value of the AC is AC_VO.

5) 5, if the value of the AC is AAC_BK.

6) 6, if the value of the AC is AAC_BE.

7) 7, if the value of the AC is AAC_VI.

8) 8, if the value of the AC is AAC_VO.

"

::= { dot11QAPEDCAEntry 1 }

Insert the following element after “dot11QAPEDCATableMandatory”:

dot11QAPEDCATableAIFSF OBJECT-TYPE

SYNTAX INTEGER (0..1)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the number of fractional slots, after a SIFS

duration, that the AP, for a particular AC, shall sense the

medium idle either before transmitting or executing a backoff. The

default value for this attribute is 0."

::= { dot11EDCAEntry aa4 }

Insert the following elements at the end of the dot11StationConfigTable element definitions:

dot11RobustAVStreamingAlternateEDCAEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This attribute, when TRUE, indicates that the station

implementation supports the Alternate EDCA Parameter Set.”
		DEFVAL { false }

::= { dot11StationConfigEntry aa5 }

dot11RobustAVStreamingSCSImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This attribute, when TRUE, indicates that the station

implementation supports the stream classification service.”
		DEFVAL { false }

::= { dot11StationConfigEntry aa6 }

dot11RobustAVStreamingSCSEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This attribute, when TRUE, indicates that the station

implementation supports the stream classification service

and that this has been enabled.”
		DEFVAL { false }

::= { dot11StationConfigEntry aa7 }

Modify the “Dot11OperationEntry” in the “dot11Operation TABLE” as shown:

Dot11OperationEntry ::=

SEQUENCE { 	dot11MACAddress MacAddress,

dot11RTSThreshold INTEGER,

dot11ShortRetryLimit INTEGER,

dot11LongRetryLimit INTEGER,

dot11FragmentationThreshold INTEGER,

dot11MaxTransmitMSDULifetime Unsigned32,

dot11MaxReceiveLifetime Unsigned32,

dot11ManufacturerID DisplayString,

dot11ProductID DisplayString,

dot11CAPLimit INTEGER,

dot11HCCWmin INTEGER,

dot11HCCWmax INTEGER,

dot11HCCAIFSN INTEGER,

dot11ADDBAResponseTimeout INTEGER,

dot11ADDTSResponseTimeout INTEGER,

dot11ChannelUtilizationBeaconInterval INTEGER,

dot11ScheduleTimeout INTEGER,

dot11DLSResponseTimeout INTEGER,

dot11QAPMissingAckRetryLimit INTEGER,

dot11EDCAAveragingPeriod INTEGER,

dot11ShortDEIRetryLimit INTEGER,

dot11LongDEIRetryLimit INTEGER }

Add the following two entries after the “dot11EDCAAveragingPeriod” entry:

dot11ShortDEIRetryLimit OBJECT-TYPE

SYNTAX INTEGER (1..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall indicate the maximum number of

transmission attempts of a frame where the DEI subfield is set

to one, the length of which is less than or equal to dot11RTSThreshold,

that shall be made before a failure condition is indicated.

The default value of this attribute shall be 7."

::= { dot11OperationEntry aa8 }

dot11LongDEIRetryLimit OBJECT-TYPE

SYNTAX INTEGER (1..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall indicate the maximum number of

transmission attempts of a frame where the DEI subfield is

set to one, the length of which is

greater than dot11RTSThreshold, that shall be made before a

failure condition is indicated. The default value of this

attribute shall be 4."

::= { dot11OperationEntry aa9 }

Modify “dot11MACbase2” in the “Groups - units of conformance” section as shown:

dot11MACbase2 OBJECT-GROUP

OBJECTS { 	dot11MACAddress, dot11Address,

dot11GroupAddressesStatus,

dot11RTSThreshold, dot11ShortRetryLimit,

dot11LongRetryLimit, dot11FragmentationThreshold,

dot11MaxTransmitMSDULifetime,

dot11MaxReceiveLifetime, dot11ManufacturerID,

dot11ProductID, dot11CAPLimit, dot11HCCWmin,

dot11HCCWmax, dot11HCCAIFSN,

dot11ADDBAResponseTimeout, dot11ADDTSResponseTimeout,

dot11ChannelUtilizationBeaconInterval, dot11ScheduleTimeout,

dot11DLSResponseTimeout, dot11QAPMissingAckRetryLimit,

dot11EDCAAveragingPeriod,

dot11ShortDEIRetryLimit, dot11LongDEIRetryLimit }

STATUS current

DESCRIPTION

"The MAC object class provides the necessary support for the

access control, generation, and verification of frame check

sequences (FCSs), and proper delivery of valid data to upper

layers."

::= { dot11Groups 31 }

References:

The default values for TXOP limit are expressed in milliseconds and are multiples of 32 µs.

Support for an alternate AC is defined as dot11RobustAVStreamingAlternateEDCAEnabled set to true and the appropriate Alternate AC_?? Present bit it is set in the Alternate Priority Info field of the Alternate EDCA Parameter Set element (see 7.3.2.94aa).