IEEE P802.11
Wireless LANs

| LB150: Time Advertisement Comments |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-JUL- |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Joe Kwak | InterDigital Communications | 482 Degas Bolingbrook, IL 60440 | 630-739-4159 | joekwak@sbcglobal.net |

Abstract

This document presents normative text to correct 2 problems with time advertisement:

1.  Since 99.999% of 802.11 STAs do not have a timebase with greater resolution than 1 microsecond,  it is wasteful of radio resources to encode time error in nanoseconds.  The solution described here reserves the 5 byte time error for the case when Time Advertisement Capability is set to 1 (for the TGp application).  A more simple time error code is then used within the Time Value field to describe the time error.

2.  The normative reference ITU-R TF.460-4 used in the TGv draft to define UT0 clearly indicates that UT0 is not UTC:

"Universal time (UT) is the general designation of time scales based on the rotation of the Earth.

In applications in which an imprecision of a few hundredths of a second cannot be tolerated, it is

necessary to specify the form of UT which should be used:

UT0 is the mean solar time of the prime meridian obtained from direct astronomical observation;

UT1 is UT0 corrected for the effects of small movements of the Earth relative to the axis of

rotation (polar variation);

UT2 is UT1 corrected for the effects of a small seasonal fluctuation in the rate of rotation of

the Earth;

UT1 is used in this Recommendation, since it corresponds directly with the angular position of

the Earth around its axis of diurnal rotation…….

The UTC scale is adjusted by the insertion or deletion of seconds (positive or negative leapseconds)

to ensure approximate agreement with UT1."

The UTC UT0 time reference in Table 37b should be changed to Universal Time UT0

7.3.2.65 Time Advertisement information element

The Time Advertisement information element, shown in Figure 7-95a1, specifies fields describing the source of time corresponding to a time standard, which may correspond with an external clock (external time source), an estimate of the offset between that time standard and the TSF timer, and the standard deviation of the error in the offset estimate. This information may be used by a receiving STA to align its own estimate of the time standard based on that of another STA.

|  | Element ID | Length | Timing capabilities | Time Value (optional) | Time Error (optional) |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 10 | 5 |
| Figure 7-95a1—Time Advertisement element format |  |  |  |  |  |

The Timing capabilities field specifies the STA's source and encoding of the Time Value field. The encoding of the Timing capabilities field is specified in Table 7-37b.

| Table 7-37b—Encoding of the Timing Capabilities field |  |
| --- | --- |
| Value | Usage |
| 0 | No standardized external time source (arbitrary) |
| 1 | Timestamp offset based on UT0 [See clause 2 - ITU-R TF.460-4] |
| 2 | Universal Time UT0 at which the TSF timer is 0 |
| 23-255 | Reserved |

When the Timing Capabilities field is set to 0, the Time Value and Time Error fields are not present in the Time Advertisement information element. When Timing Capabilities is set to 1, the Time Value field specifies a two's complement integer in nanoseconds which, when added to the Timestamp present in the same transmitted frame gives the receiving STA an estimate of the time standard at the time the frame was transmitted. The Timestamp is derived from the TSF Timer as defined in 11.20. When Timing Capabilities is set to 2, the Time Value field is set to the UTC UT0 time at which the TSF Timer is 0. The format, including all subfields is shown in Table 7-37c.

| Table 7-37c—Time Value field format when Timing Capabilities is 2 |  |
| --- | --- |
| Octet | Description |
| 0 - 1 | Year (0-65534) |
| 2 | Month (1-12) |
| 3 | Day of month (1-31) |
| 4 | Hours (0-23) |
| 5 | Minutes (0-59) |
| 6 | Seconds (0-59) |
| 7-8 | Milliseconds (0-999) |
| 9 | Time Error Code (see Table 7-37x) |

| Table 7-37x—Time Error Code format when Timing Capabilities is 2 |  |
| --- | --- |
| Value | Time Error (see definition below) |
| 0 | Time error unknown |
| 1 | Time error > 10 hourss) |
| 2 | Time error <= 10 hours) |
| 3 | Time error <= 1 hour) |
| 4 | Time error <= 10 minutes) |
| 5 | Time error <= 1 minute) |
| 6 | Time error <= 10 seconds) |
| 7 | Time error <= 1 second) |
| 8 | Time error <= 100 milliseconds) |
| 9 | Time error <= 10 milliseconds) |
| 10 | Time error <= 1 millisecond) |
| 11 | Time error <= 100 microseconds) |
| 12 | Time error <= 10 microseconds) |
| 13 | Time error <= 1 microseconds) |
| 14-255 | Reserved |

When the Timing Capabilities field is set to 1, the Time Error field is present in the Time Advertisement information element; it is not present otherwise. The Time Error field contains the time error specified as an unsigned integer in nanoseconds that defines the standard deviation of the error in the Time Value estimate. When the Timing Capabiities feld is set to 2, the time error is specified by the value of the Time Error Code included in the Time Value field. The time error  is the error in the estimate of the external time standard (e.g. UT0) compared with the true value of time corresponding to that external time standard, as well as any additional errors introduced when computing the Time Value.