IEEE P802.11
Wireless LANs

| Mesh Compatible MLME-Scan and MLME-Join |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-07-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Jarkko Kneckt | Nokia | Itämerenkatu 11 – 13, 00180 Helsinki, Finland |  | Jarkko.Kneckt (at) Nokia.com |

Abstract

The document describes mesh compatible MLME-Scan and MLME-Start primitives. The document solves CID 34.

Summary of Changes

1 . MESH is added to BSSType element

2. The Mesh ID is added as parameter to MLME-Scan.request, to be able to specify the MESH Id for scanned MBSS networks

3. The Beacon information handling in MLME-SCAN.response is made MBSS compatible

4. The Mesh ID and Mesh Configuration are added to MLME-Start, to be able to specify the mesh specific parameters.

With these changes the MLME specified in 802.11 reaff2007 is compatible with 802.11s D3.02.

TGs Editor: modify the selected clauses as shown below.

Additions to clauses are Underlined and deleted text is strikethrough:

10.3.2.1 MLME-SCAN.request

10.3.2.1.1 Function

This primitive requests a survey of potential BSSs that the STA can later elect to try to join.

10.3.2.1.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-SCAN.request(

BSSType,

BSSID,

SSID,

ScanType,

ProbeDelay,

ChannelList,

MinChannelTime,

MaxChannelTime,

MeshID,

VendorSpecificInfo

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| BSSType | Enumeration | INFRASTRUCTURE, INDEPENDENT, MESH, ANY_BSS | Determines whether infrastructure BSS,IBSS, MBSS, or both all, are included in the scan. |
| BSSID | MACAddress | Any valid individual or broadcast MAC address | Identifies a specific or wildcard BSSID. |
| SSID | Octet string | 0 – 32 octets | Specifies the desired SSID or the wildcard SSID |
| ScanType | Enumeration | ACTIVE, PASSIVE | Indicates either active or passive scanning. |
| ProbeDelay | Integer | N/A | Delay (in microseconds) to be used prior to transmitting a Probe frame during active scanning. |
| ChannelList | Ordered set of integers | Each channel will be selected from the valid channel range from the valid channel range for the appropriate PHY and carrier set. | Specifies a list of channels that are examined when scanning for a BSS. |
| MinChannelTime | Integer | ≥ ProbeDelay | The minimum time (in TU) to spend on each channel when scanning. |
| MaxChannelTime | Integer | ≥ MinChanneTime | The maximum time (inTU) to spend on each channel when scanning. |
| Mesh ID | Octet string | 0 – 32 octets | Only present if BSSType = MESH or BSSType = ANY_BSS. Specifies the desired Mesh ID or wildcard Mesh ID. |
| VendorSpecificInfo | A set of information elements | As defined in 7.3.2.26 | Zero or more information elements. |

10.3.2.1.3 When generated

This primitive is generated by the SME for a STA to determine if there are other BSSs that it can join.

10.3.2.1.4 Effect of receipt

This request initiates the scan process when the current frame exchange sequence is completed.

10.3.2.2 MLME-SCAN.confirm

10.3.2.2.1 Function

This primitive returns the descriptions of the set of BSSs detected by the scan process.

10.3.2.2.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-SCAN.confirm(

BSSDescriptionSet,

ResultCode,

VendorSpecificInfo

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| BSSDescriptionSet | Set of BSSDescriptions | N/A | The BSSDescriptionSet is returned to indicate the results of the scan request. It is a set containing zero or more instances of a BSSDescription. |
| ResultCode | Enumeration | SUCCESS. INVALID_PARAMETERS, NOT_SUPPORTED | Indicates the result of the MLMESCAN.confirm. |
| VendorSpecificInfo | A set of information elements | As defined in 7.3.2.26 | Zero or more information elements. |

Each BSSDescription consists of the following elements:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| BSSID | MACAddress | N/A | The BSSID of the found BSS or the MAC address of the responding mesh STA. |
| SSID | Octet string | 1 - 32 octets | The SSID of the found BSS. |
| BSSType | Enumeration | INFRASTRUCTURE, INDEPENDENT, MESH | The type of the found BSS. |
| Beacon Period | Integer | N/A | Present only is BSSType = INFRASTRUCTURE or INDEPENDENT. The Beacon period of the found BSS (in TU). |
| DTIM Period | Integer | As defined in frame format | Present only is BSSType = INFRASTRUCTURE or INDEPENDENT. The DTIM period of the BSS (in beacon periods). |
| Timestamp | Integer | N/A | Present only is BSSType = INFRASTRUCTURE or INDEPENDENT. The timestamp of the received frame (probe response/beacon) from the found BSS. |
| Local Time | Integer | N/A | The value of the STA’s TSF timer at the start of reception of the first octet of the timestamp field of the received frame (probe response or beacon) from the found BSS. |
| PHY Parameter Set | As defined in frame format or according to the relevant PHY clause. | As defined in frame format or according to the relevant PHY clause. | The parameter sets relevant to the PHY from the received Beacon or Probe Response frame. If no PHY Parameter Set information element is present int he received frame, this parameter contains the channel number on which the frame was received. Valid channel numbers are defined in the relevant PHY clause. |
| CF Parameter Set | As defined in frame format. | As defined in frame format. | The parameter set for the CF periods, if found BSS supports CF mode. |
| IBSS Parameter Set | As defined in frame format. | As defined in frame format. | The parameter set for the IBSS, if found BSS is an IBSS. |
| CapabilityInformation | As defined in frame format. | As defined in frame format. | The advertised capabilities of the BSS. |
| BSSBasicRateSet | Set of integers | 1–127 inclusive (for each integer in the set) | The set of data rates that must be supported by all STAs that desire to join this BSS. The STAs must be able to receive and transmit at each of the data rates listed in the set. |
| OperationalRateSet | Set of integers | 1–127 inclusive (for each integer in the set) | The set of data rates that the STA the BSS. The STA must be able to receive at each of the data rates listed in the set. This set is a superset of the rates contained in the BSSBasicRateSet parameter. |
| Country element | As defined in the Country element. | The information required to identify the regulatory domain in which the STA is located and to configure its PHY for operation in that regulatory domain. | Present only when TPC functionality is required, as specified in 11.8, or when dot11MultiDomainCapabilityEnabled is true. |
| IBSS DFS Recovery Interval | Integer 1–255 Only present if BSSType = INDEPENDENT | The time interval that is used for DFS recovery. | Present only when DFS functionality is required, as specified in 11.9. |
| RSN | RSN information element | As defined in frame format | A description of the cipher suites and AKM suites supported in the BSS. |
| Load | As defined in frame format | As defined in frame format | The values from the BSS Load information element if such an element was present in the probe response or Beacon frame, else null. |
| EDCAParameterSet | As defined in frame format | As defined in frame format | The values from the EDCA Parameter Set information element if such an element was present in the probe response or Beacon frame, else null. |
| QoSCapability | As defined in frame format | As defined in frame format | The values from the QoS Capability information element if such an element was present in the probe response or Beacon frame, else null. |
| MeshID | 0-32 Octets | As defined in frame format | The value of MeshId element if such element was present in the probe response or Beacon frame, else null. |
| MeshConfiguration | As defined in frame format | As defined in frame format | The values from the Mesh Configuration information element if such an element was present in the probe response or Beacon frame, else null. |

10.3.2.2.3 When generated

This primitive is generated by the MLME as a result of an MLME-SCAN.request to ascertain the operating environment of the STA.

10.3.2.2.4 Effect of receipt

The SME is notified of the results of the scan procedure.

10.3.10.1 MLME-START.request

10.3.10.1.1 Function

This primitive requests that the MAC entity start a new BSS.

10.3.10.1.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-START.request(

SSID,

BSSType,

BeaconPeriod,

DTIMPeriod,

CF parameter set,

PHY parameter set,

IBSS parameter set,

ProbeDelay.

CapabilityInformation,

BSSBasicRateSet,

OperationalRateSet,

Country,

IBSS DFS Recovery Interval,

EDCAParameterSet,

MeshID,

MeshConfiguration,

VendorSpecificInfo

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| SSID | Octet string | 1 - 32 octets | The SSID of the BSS. |
| BSSType | Enumeration | INFRASTRUCTURE, INDEPENDENT, MESH | The type of the BSS. |
| Beacon Period | Integer | ≥1 | The Beacon period of the BSS (in TU). |
| DTIM Period | Integer | As defined in 7.3.2.6 | The DTIM period of the BSS (in beacon periods). |
| CF Parameter Set | As defined in frame format. | As defined in 7.3.2.5 | The parameter set for CF periods, if the BSS supports CF mode. aCFPPeriod is modified as a side effect of the issuance of an MLMESTART.request primitive. |
| PHY Parameter Set | As defined in frame format. | As defined in 7.3.2.3 or 7.3.2.4. | The parameter sets relevant to the PHY. |
| IBSS Parameter Set | As defined in frame format. | As defined in 7.3.2.7 | The parameter set for the IBSS, if BSS is an IBSS. |
| ProbeDelay | Integer | N/A | Delay (in microseconds) to be used prior to transmitting a Probe frame during active scanning. |
| CapabilityInformation | As defined in frame format. | As defined in 7.3.1.4 | The capabilities to be advertised for the BSS. |
| BSSBasicRateSet, | Set of integers | 1–127 inclusive (for each integer in the set) | The set of data rates that must be supported by all STAs to join this BSS. The STA that is creating the BSS must be able to receive and transmit at each of the data rates listed in the set. |
| OperationalRateSet, | Set of integers | 1–127 inclusive (for each integer in the set) | The set of data rates that the STA desires to use for communication within the BSS. The STA must be able to receive at each of the data rates listed in the set. This set is a superset of the rates contained in the BSSBasicRateSet parameter. |
| Country | As defined in the Country element | As defined in the Country element | The information required to identify the regulatory domain in which the STA is located and to configure its PHY for operation in that regulatory domain. Present only when TPC functionality is required, as specified in 11.8 or when dot11MultiDomainCapabilityEnabled is true. |
| IBSS DFS Recovery Interval, | Integer | 1 - 255 | Present only if BSSType = INDEPENDENT. The time interval that is used for DFS recovery. Present only when DFS functionality is required, as specified in 11.9. |
| EDCAParameterSet (QoS only) | As defined in frame format. | As defined in frame format. | Present when BSSType = INFRASTRUCTURE.The initial EDCA parameter set values to be used in the BSS. The parameter shall be present only if the MIB attribute dot11QosOptionImplemented is true |
| MeshID | 0-32 Octets | As defined in frame format | The MeshId of the found MBSS. |
| MeshConfiguration | As defined in frame format | As defined in frame format | The values from the Mesh Configuration information element if such an element was present in the probe response or Beacon frame, else null. |
| VendorSpecificInfo | A set of information elements | As defined in 7.3.2.26 | Zero or more information elements. |

10.3.10.1.3 When generated

This primitive is generated by the SME to start either an infrastructure BSS (with the MAC entity within an AP), an MBSS (with the MAC entity acting as the first STA in the MBSS) or an IBSS (with the MAC entity acting as the first STA in the IBSS).

The MLME-START.request primitive must be generated after an MLME-RESET.request primitive has been used to reset the MAC entity and before an MLME-JOIN.request primitive has been used to successfully join an existing infrastructure BSS MBSS or IBSS.

The MLME-START.request primitive must not be used after successful use of the MLME-START.request primitive or successful use of the MLME-JOIN.request without generating an intervening MLMERESET.request primitive.

10.3.10.1.4 Effect of receipt

This primitive initiates the BSS initialization procedure once the current frame exchange sequence is complete. The MLME subsequently issues an MLME-START.confirm that reflects the results of the creation procedure.

10.3.10.2 MLME-START.confirm

10.3.10.2.1 Function

This primitive reports the results of a BSS creation procedure.

10.3.10.2.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-START.confirm(

ResultCode,

VendorSpecificInfo

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID_PARAMETERS, BSS_ALREADY_STARTED_OR_JOINED, RESET_REQUIRED_BEFORE_ START, NOT_SUPPORTED | Indicates the result of the MLME-START.request. |
| VendorSpecificInfo | A set of information elements | As defined in 7.3.2.26 | Zero or more information elements. |

10.3.10.2.3 When generated

This primitive is generated by the MLME as a result of an MLME-START.request to create a new BSS.

10.3.10.2.4 Effect of receipt

The SME is notified of the results of the BSS creation procedure.

References: