IEEE P802.11
Wireless LANs

| MLME primitives |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-06-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Menzo Wentink | Qualcomm | Straatweg 66, Breukelen, the Netherlands | +31 65 183 6231 | mwentink@qualcomm.com |
| Brian Hart | Cisco |  |  |  |

Abstract

This document specifies two additional MLME primitives for TDLS.

TGz editor make the following changes to TGz draft 5.0.

Replace Fig 10.6a with the following figure:

Add two new primitives:

#### 10.3.44.1 MLME-TDLSPOTENTIALTDLSPEERSTA.request

##### 10.3.44.1.1 Function

This primitive requests for information about a potential TDLS peer STA.

##### 10.3.44.1.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-TDLSPOTENTIALPEERSTA.request(

MACAddress

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| MACAddress | MAC Address | Any valid individual MAC Address | Specifies the MAC address of the potential TDLS peer STA. |

##### 10.3.44.1.3 When Generated

This primitive is generated by the SME to request the MLME to prpovide information about a potential TDLS peer STA.

##### 10.3.44.1.4 Effect of Receipt

On receipt of this primitive, the MLME responds with the requested information about the identified STA.

#### 10.3.44.1 MLME-TDLSPOTENTIALTDLSPEERSTA.response

##### 10.3.44.1.1 Function

This primitive informs the SME about potential TDLS peer STAs.

##### 10.3.44.1.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-TDLSPOTENTIALPEERSTA.response(

MACAddress,

RSSI,

VendorSpecificInfo

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| MACAddress | MAC Address | Any valid individual MAC Address | Specifies the MAC address of the STA for which information is requested. |
| STA RSSI | Integer | 0-RSSI Max | Specifies the RSSI from the STA. 0 indicates that the STA is not present. |
| VendorSpecificInfo | Vendor Specific | Vendor Specific | Specifies vendor specific information about the STA identified in the MACAddress field. |

##### 10.3.44.1.3 When Generated

This primitive is generated by the MLME to indicate to the SME that a potential TDLS peer STA has been detected.

##### 10.3.44.1.4 Effect of Receipt

On receipt of this primitive, the SME can decide to attempt to set up a TDLS direct link by issueing an MLME-TDLSSETUPREQUEST.request primitive to the MLME.