IEEE P802.11
Wireless LANs

| Alternate EDCA Parameter Set |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-01-20 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Alex Ashley | NDS Ltd | One London Road, Staines, Middlesex, TW18 4EX, UK | +44 1784 848770 | aashley@nds.com |

Abstract

This document contains the normative text of a proposal to fulfil the PAR objective of the 11aa task group:

- Intra-Access Category prioritization of transport streams by modifying EDCA timing and parameter selection without any requirement for deep packet inspection

This document is based upon

- IEEE P802.11-2007

- IEEE P802.11k-2008

- IEEE P802.11r-2008

- IEEE P802.11y-2008

- IEEE P802.11w-2009

- IEEE P802.11n-2009

- IEEE P802.11p draft 9.0

- IEEE P802.11s draft 3.0

- IEEE P802.11u draft 8.0

- IEEE P802.11v draft 7.0

- IEEE P802.11z draft 6.0

- IEEE P802.11aa draft 0.03

7. Frame formats

7.3 Management frame body components

7.3.2 Information elements

Change 7.3.2.27 as follows:

7.3.2.27 Extended Capabilities information element

Insert Bit(s) <ANA> and change the Reserved row in Table 7-35a as follows (note that the entire table is not shown here):

| Table 7-35a—Capabilities field |  |  |
| --- | --- | --- |
| Bit(s) | Information | Notes |
| <ANA> | Alternate EDCA | The STA sets the Alternate EDCA field to 1 when the MIB attribute dot11RobustAVStreamingAlternateEDCAEnabled is set to true, and sets it to 0 otherwise. See 9.1.3.1 (HCF contention-based channel access) |

7.3.2.aa85 Intra-Access Category Priority element

Modify figure 7-aa10 as shown.

|  | B0 – B2 | B3 | B4 | B5-B7 |  |
| --- | --- | --- | --- | --- | --- |
|  | User Priority | Reserved Alternate Priority | Drop Eligibilty | Reserved |  |
| Bits: | 3 | 1 | 1 | 3 |  |
| Figure 7-aa10— Intra-Access Priority field |  |  |  |  |  |

Insert the following paragraph immediately after the paragraph that defines the User Priority subfield.

The Alternate Priority subfield indicates which EDCA parameter set is used for this stream. When set to zero, the EDCA queues (see 7.3.2.29) are used. When the Alternate Priority subfield is set to one, the Alternate EDCA queues (7.3.2.89aa) are used.

9. MAC sublayer functional description

9.1 MAC architecture

9.1.3 Hybrid coordination function (HCF)

9.1.3.1 HCF contention-based channel access (EDCA)

Modify clause 9.1.3.1 as follows:

The EDCA mechanism provides differentiated, distributed access to the WM for STAs using eight different UPs. The EDCA mechanism defines four access categories (ACs) that provide support for the delivery of traffic with UPs at the STAs. The AC is derived from the UPs as shown in Table 9-1 and Table 9-1aa. Table 9-1 is used when dot11RobustAVStreamingAlternateEDCAEnabled is false. Table 9-1aa is used when dot11RobustAVStreamingAlternateEDCAEnabled is true.

| Table 9-1 – UP-to-AC mappings (dot11RobustAVStreamingAlternateEDCAEnabled false) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Priority | UP (Same as 802.1D user priority) | 802.1D designation | AC Queue | Designation (informative) |
| Lowest | 1 | BK | AC_BK | Background |
|  | 2 | - | AC_BK | Background |
|  | 0 | BE | AC_BE | Best Effort |
|  | 3 | EE | AC_BE | Video |
|  | 4 | CL | AC_VI | Video |
|  | 5 | VI | AC_VI | Video |
|  | 6 | VO | AC_VO | Voice |
| Highest | 7 | NC | AC_VO | Voice |

| Table 9-1aa – UP-to-AC mappings (dot11RobustAVStreamingAlternateEDCAEnabled true) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Priority | UP (Same as 802.1D user priority) | 802.1D designation | AC Queue | Designation (informative) |
| Lowest | 1 | BK | AC_BK | Background |
|  | 2 | - | AC_BK | Background |
|  | 0 | BE | AC_BE | Best Effort |
|  | 3 | EE | AAC_VI | Video (alternate) |
|  | 4 | CL | AAC_VI | Video (alternate) |
|  | 5 | VI | AC_VI | Video |
|  | 6 | VO | AC_VO | Voice |
| Highest | 7 | NC | AC_VO | Voice |

For each AC, an There are four enhanced variants of the DCF, called an enhanced distributed channel access function (EDCAF), that contends for TXOPs using a set of EDCA parameters. When communicating data frames outside the context of a BSS (dot11OCBEnabled is true), the EDCA parameters are the corresponding default values or are as set by the SME in the MIB attribute table dot11EDCATable (except for TXOP limit values, which shall be set to zero for each AC). When communicating within a BSS, the EDCA parameters used are from the EDCA Parameter Set element or from the default values for the parameters when no EDCA Parameter Set element is received from the AP of the BSS with which the STA is associated or when the STA is a mesh STA. The parameters used by the EDCAF to control its operation are defined by MIB attribute table dot11QAPEDCATable at the AP and by MIB attribute table dot11EDCATable at the non-AP STA.

The AAC_VI and AAC_VO queues share the same EDCAF as AC_VI and AC_VO respectively. There is a scheduling function above the VI and VO EDCAFs that selects an MSDU or an MMPDU from either the AAC_VI/AAC_VO or AC_VI/AC_VO queues such that the queue with the higher UP is provided with a greater probability of being selected. The algorithm to prioritize between MSDU/MMPDU in the AAC_VI and AC_VI queues, and between MSDU/MMPDU in the AAC_VO and AC_VO queues, is not specified. MSDU/MMPDU assigned to AC_VI and AAC_VI use the same over-the-air EDCA parameters of AC_VI. MSDU/MMPDU assigned to AC_VO and AAC_VO use the same over-the-air EDCA parameters of AC_VO.

9.9 HCF

9.9.1 HCF contention-based channel access (EDCA)

9.9.1.1 Reference implementation

Modify the second paragraph of 9.9.1.1 as follows.

A model of the reference implementation is shown in Figure 9-17 (Reference implementation model) and illustrates a mapping from frame type or UP to AC: the four AC transmit queues and the four independent EDCAFs, one for each queue. The mapping of UP to the AC is described in 9.1.3.1 (HCF contention-based channel access (EDCA)) and Table 9-1 (UP-to-AC mappings). The mapping of AC to EDCAF is described in 9.1.3.1 (HCF contention-based channel access (EDCA)), Table 9-1 (UP-to-AC mappings), and Table 9-1aa (UP-to-AC mappings). The mapping of frame types to ACs is described in 9.1.3.1 (HCF contention-based channel access (EDCA)).

9.9.1.2 EDCA TXOPs

Insert the following figure after Figure 9-17:

VO

VI

BE

BK

Mapping to

Access Category

Transmit queues

for ACs

EDCA functions with internal collision resolution

(MSDU, UP)

AC_BK

AC_BE

AC_VI

AAC_VI

AC_VO

AAC_VO

Figure 9-17aa—Reference implementation model (dot11RobustAVStreamingAlternateEDCAEnabled)

11. MLME

11.4 TS operation

11.4.1 Introduction

Modify the second paragraph of 11.4.1 as indicated.

A TS may have one or more TCLAS (within the discretion of the STA that sets up the stream) associated with it. The AP uses the parameters in the TCLAS elements to filter the MSDUs belonging to this TS so that they can be delivered with the QoS parameters that have been set up for the TS. An Intra-Access Category Priority element may be associated with a TS. The User Priority subfield of the Intra-Access Priority element shall be set to the same User Priority as specified in the User Priority subfield of the TS Info field. If dot11RobustAVStreamingAlternateEDCAEnabled is true, the Alternate Priority subfield is also used to select the appropriate EDCA transmit queue when the Access Policy subfield of the TS Info field in the TSPEC is EDCA or HEMM. The Drop Eligibility subfield is used to tag the drop eligibility of the MSDUs of this TS.

11.4.6 Data transfer

Add the following to the end of 11.4.6.

If dot11RobustAVStreamingAlternateEDCAEnabled is true, an Intra-Access Priority element was provided when the TS was created and the Access Policy subfield of the TS Info field in the TSPEC is EDCA or HEMM, the value from the Alternate Priority subfield is used to select the appropriate EDCA transmit queue.

11.21 Robust AV Streaming

11.aa21.2 SCS Procedures

Modify the 2nd  paragraph of 11.aa21.2 as shown.

The classification allows User Priority and, Drop Eligibility and EDCA transmit queue to be selected for all MSDUs matching the classification.

Modify the 7th paragraph of 11.aa21.2 as shown

If the requested SCS is accepted by the AP, the AP shall process subsequent incoming unicast MSDUs from the DS or WM that match the classifier specified in the SCS Descriptor. The processing of matching MSDUs depends upon the access policy assigned to the MSDU:

- For matching MSDUs that are not part of a TS (as described in 11.4), the User Priority subfield of the Intra-Access Priority element is used as the UP of these MSDUs.

- If dot11RobustAVStreamingAlternateEDCAFEnabled is true, matching MSDUs that are not part of a TS (as described in 11.4), or MSDUs that are part of a TS that uses EDCA or HEMM as the Access Policy, the Alternate Priority subfield of the Intra-Access Priority element is used to select whether the EDCA transmit queue or Alternate EDCA transmit queue is used for these MSDUs.

- All matching MSDUs are tagged with their drop eligibility using the value from the Drop Eligibility subfield of the Intra-Access Priority element in the DEI subfield of the AVT field, as defined in 7.1.3.5.8.

Annex A

(normative)

Protocol Implementation Conformance Statement (PICS) proforma

A.4 PICS proforma–IEEE Std. 802.11, 2007 Edition

A.4.3 IUT configuration

A.4.23 RobustAVT extensions

Insert the following items in to the unnumbered table in A.4.23

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| AVT3 | Alternate EDCA transmit queues | 7.3.2.aa90 | CFaa:O | Yes, No, N/A |

Annex D

(normative)

ASN.1 encoding of the MAC and PHY MIB

Change the “Dot11StationConfigEntry” of the “dotStationConfig TABLE” as follows:

-- *********************************************************************

-- * dotStationConfig TABLE

-- *********************************************************************

Dot11StationConfigEntry ::=
SEQUENCE {
	dot11StationID 						MacAddress,
	dot11MediumOccupancyLimit 					INTEGER,
	dot11CFPollable 						TruthValue,
	dot11CFPeriod 						INTEGER,
	dot11CFPMaxDuration 					INTEGER,
	dot11AuthenticationResponseTimeOut 				Unsigned32,
	dot11PrivacyOptionImplemented 				TruthValue,
	dot11PowerManagementMode 					INTEGER,
	dot11DesiredSSID 						OCTET STRING,
	dot11DesiredBSSType 					INTEGER,
	dot11OperationalRateSet 					OCTET STRING,
	dot11BeaconPeriod 						INTEGER,
	dot11DTIMPeriod 						INTEGER,
	dot11AssociationResponseTimeOut 				Unsigned32,
	dot11DisassociateReason 					INTEGER,
	dot11DisassociateStation 					MacAddress,
	dot11DeauthenticateReason 					INTEGER,
	dot11DeauthenticateStation 					MacAddress,
	dot11AuthenticateFailStatus 					INTEGER,
	dot11AuthenticateFailStation 				MacAddress,
	dot11MultiDomainCapabilityImplemented 			TruthValue,
	dot11MultiDomainCapabilityEnabled 				TruthValue,
	dot11CountryString 					OCTET STRING,
	dot11SpectrumManagementImplemented 				TruthValue,
	dot11SpectrumManagementRequired 				TruthValue,
	dot11RSNAOptionImplemented 					TruthValue,
	dot11RSNAPreauthenticationImplemented 			TruthValue,
	dot11RegulatoryClassesImplemented 				TruthValue,
	dot11RegulatoryClassesRequired 				TruthValue,
	dot11QosOptionImplemented 					TruthValue,
	dot11ImmediateBlockAckOptionImplemented 			TruthValue,
	dot11DelayedBlockAckOptionImplemented 			TruthValue,
	dot11DirectOptionImplemented 				TruthValue,
	dot11APSDOptionImplemented 					TruthValue,
	dot11QAckOptionImplemented 					TruthValue,
	dot11QBSSLoadOptionImplemented 				TruthValue,
	dot11QueueRequestOptionImplemented 				TruthValue,
	dot11TXOPRequestOptionImplemented 				TruthValue,
	dot11MoreDataAckOptionImplemented 				TruthValue,
	dot11AssociatedinNQBSS 					TruthValue,
	dot11DLSAllowdInQBSS 					TruthValue,
	dot11DLSAllowed 						TruthValue,
	dot11AssociateStation 					MacAddress,
	dot11AssociateID 						INTEGER,
	dot11AssociateFailStation 					MacAddress,
	dot11AssociateFailStatus 					INTEGER,
	dot11ReassociateStation 					MacAddress,
	dot11ReassociateID 					INTEGER,
	dot11ReassociateFailStation 					MacAddress,
	dot11ReassociateFailStatus 					INTEGER,
	dot11RadioMeasurementCapable 				TruthValue,
	dot11RadioMeasurementEnabled 				TruthValue,
	dot11RadioMeasurementProbeDelay 				INTEGER,
	dot11MeasurementPilotReceptionEnabled 			TruthValue,
	dot11MeasurementPilotTransmissionEnabled 			TruthValue,
	dot11MeasurementPilotTransmissionVirtualApSetEnabled 		TruthValue,
	dot11MeasurementPilotPeriod 					INTEGER,
	dot11LinkMeasurementEnabled 					TruthValue,
	dot11NeighborReportEnabled 					TruthValue,
	dot11ParallelMeasurementsEnabled 				TruthValue,
	dot11TriggeredMeasurementsEnabled 				TruthValue,
	dot11RepeatedMeasurementsEnabled 				TruthValue,
	dot11MeasurementPauseEnabled 				TruthValue,
	dot11QuietIntervalEnabled 					TruthValue,
	dot11PassiveBeaconMeasurementEnabled 				TruthValue,
	dot11ActiveBeaconMeasurementEnabled 				TruthValue,
	dot11TableBeaconMeasurementEnabled 				TruthValue,
	dot11ReportingConditionsEnabled 				TruthValue,
	dot11FrameMeasurementEnabled 				TruthValue,
	dot11ChannelLoadEnabled 					TruthValue,
	dot11NoiseHistogramEnabled 					TruthValue,
	dot11StatisticsReportEnabled 				TruthValue,
	dot11LCIReportEnabled 					TruthValue,
	dot11TransmitStreamMeasurementEnabled 			TruthValue,
	dot11APChannelReportEnabled 					TruthValue,
	dot11AnnexQMIBSupportEnabled 				TruthValue,
	dot11NonOperatingChannelMeasurementsEnabled 			TruthValue,
	dot11MaximumMeasurementDuration 				Unsigned32,
	dot11MeasurementPilotSupport 				Unsigned32,
	dot11FastBSSTransitionImplemented 				TruthValue,
	dot11LCIDSEImplemented 					TruthValue, 
	dot11LCIDSERequired 					TruthValue, 
	dot11DSERequired 						TruthValue, 
	dot11ExtendedChannelSwitchEnabled 				TruthValue, 
	dot11HighThroughputOptionImplemented 				TruthValue,
	dot11WirelessManagementImplemented 				TruthValue, 
	dot11MaxIdlePeriod 					INTEGER,
	dot11TIMBroadcastInterval     				INTEGER,
	dot11TIMBroadcastOffset       				INTEGER,
	dot11MinTriggerTimeout     				INTEGER,
	dot11RRMCivicMeasurementEnabled 				TruthValue,
	dot11RRMIdentifierMeasurementEnabled 			TruthValue,
dot11DMSMaxSTAS       					INTEGER,
dot11DMSMaxChannelLoadForNewService 				INTEGER,
dot11DMSMaxChannelLoad 					INTEGER,
dot11UTCTSFDTIMInterval 					INTEGER,
dot11UTCTSFOffsetTimeError 					INTEGER,
dot11UTCTSFOffsetTimeValue 					INTEGER,

dot11RobustAVStreaming 					TruthValue,

dot11RobustAVStreamingAdvancedMRG 				TruthValue,

dot11RobustAVStreamingSCSImplemented			TruthValue,

dot11RobustAVStreamingSCSEnabled				TruthValue,

dot11RobustAVStreamingAlternateEDCAEnabled			TruthValue

}

Insert the following elements at the end of the dot11StationConfigTable element definitions:

dot11RobustAVStreamingAlternateEDCAEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This attribute, when TRUE, indicates that the station

implementation supports the Alternate EDCA transmit queues.”
		DEFVAL { false }

::= { dot11StationConfigEntry aa5 }

References: