IEEE P802.11
Wireless LANs

| Normative Texts for MCF Issues |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-07-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Liwen Chu | STMicroelectronics, Inc. | 1060 East Brokaw Rd. San Jose, CA 95131 | +1-408-467-8436 | Liwen.chu@st.com |
| George Vlantis | STMicroelectronics, Inc. | 1060 East Brokaw Rd. San Jose, CA 95131 | +1 408 451-8109 | George.vlantis@st.com |

Abstract

This submission provides text to resolve the issues identified in 11-09/562r2

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Background

This contribution provides text to address the issues presented in 11-09/562r2:

Transmitting frames from multiple access categories in a MCCAOP to save power;

Making MCCAOP stable in neighboring mesh STAs.

The following is normative text proposed as an amendment to P802.11s/D3.02.

7.3.1.38 MCCAOP Reservation field

Change the first paragraph of 7.3.1.38 as follows:

The MCCAOP Reservation field is a variable-length field specifying the starts of a series of MCCAOPs, relative to the DTIM interval of the reporting mesh STA, and their (common) duration. The MCCAOP Reservation field consists of variable subfields and its format is illustrated in Figure s12 (MCCAOP Reservation field).

The MCCAOP Duration field specifies the duration of the MCCAOPs in multiples of 32μs.

| MCCAOP Duration | MCCAOP Periodicity | MCCAOP Offset 1 | …… | MCCAOP Offset n |
| --- | --- | --- | --- | --- |

Octet: 1                    1                        2                                                  2

The MCCAOP Periodicity field is an eight bit unsigned number. Bit 7 is used to indicate if variable offset is used. If Bit 7 is 0, the same MCCA Offset is used for all MCCAOP in a DTIM beacon interval. If Bit 7 is 1, the different MCCA offsets are used for MCCAOPs in a DTIM beacon interval. A positive Periodicity field specifies the number of MCCAOPs in the DTIM interval. These MCCAOPs are repeated in all DTIM intervals until the MCCAOP Reservation is torn down. A value of zero indicates a non-repeated MCCAOP in the DTIM interval following the setup. The MCCAOP Offset field specifies the beginning of the MCCAOP relative to the beginning of each subinterval. The value is specified in multiples of 32 μs. An example of periodicity, duration, and offset values for an MCCAOP Reservation field is shown in Figure s13 (Values for Periodic MCCAOP Info field for an example MCCAOP Reservation). In this particular example, the periodicity equals four, so that there are four MCCAOPs within the DTIM interval. As further illustrated in the figure, the offset value indicates the start of the MCCAOPs relative to the beginning of these subintervals.

7.3.2.95 MCCAOP Setup Reply element

Change Figure s31 as follows:

| Element ID | Length | MCCAOP Reservation ID | MCCAOP Reservation |
| --- | --- | --- | --- |

Octet: 1                    1                        1                       variable

7.3.2.96 MCCAOP Setup Request element

Change Figure s32 as follows:

| Element ID | Length | MCCAOP Reservation ID | MCCA Reply Code | MCCAOP Reservation |
| --- | --- | --- | --- | --- |

Octet: 1                    1                        1                       1                       variable

7.3.2.97.1 MCCAOP Reservation Report field

Change Figure s34 as follows:

| Number of Reported MCCAOP Reservations | MCCAOP Reservation | … | MCCAOP Reservation |
| --- | --- | --- | --- |

Octet: 1                      variable                                      variable

9.9.1.7 Truncation of TXOP

Change the 4th  paragraph of 9.9.1.7 as follows:

A non-AP STA that is not the TXOP holder shall not transmit a CF-End frame in a non-mesh BSS. In a mesh BSS, after receiving a CF-End frame, a TXOP receiver may respond by transmitting a CF-End frame after SIFS.

9.9a.2.2 MCCA opportunity (MCCAOP)

Change the first paragraph of 9.9a.2.2 as follows:

An MCCAOP is a period of time, defined by a target transmission starting time and a duration, set up between an MCCAOP owner and one (for individually addressed frames) or more (for group addressed frames) MCCAOP responders for frame transmissions. To make the MCCAOP position stable in neighbor’s DTIM beacon interval, each mesh STA that uses MCCA shall use the default DTIM beacon interval as its DTIM beacon interval. MCCAOPs are setup by means of the procedure defined in 9.9a.2.7 (MCCAOP setup procedure). Once an MCCAOP is setup,

9.9a.2.10.1 Access by MCCAOP Owners

Change 9.9a.2.10.1 as follows:

An MCCAOP owner shall attempt to access the channel during the time set up for the MCCAOP and obtain the MCCAOP using EDCA contention and backoff parameters as defined in 9.9.1.3. The MCCAOP owner can foreshorten any reservation period by transmitting frames available under EDCA, see 9.9.1, that set the NAV at a receiving stations. These frames not only set the NAV of the receiving station, but also the time period during which the receiving station is blocked from accessing the medium due to the reservation. A mesh STA may transmit frames from any access category in a MCCAOP.

Note: A mesh STA can use any acess category to contend the MCCAOP.

A mesh STA successfully obtains the MCCAOP when the mesh STA completes a frame exchange with the receiver of the MSDU. If the mesh STA successfully obtains the MCCAOP, it may transmit until the end of the MCCAOP.

If a mesh STA accesses the channel during the MCCAOP but fails to obtain MCCAOP, the mesh STA shall perform the backoff procedure specified in 9.9.1.5.

After a mesh STA successfully obtains the MCCAOP, if there are multiple MSDUs to be transmitted, the transmission and retransmission rules for multiple MSDU transmission should use the rules specified in 9.9.1.4 and 9.9.1.6.