| Resolutions to a number of MCCA related comments |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-07-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Dee Denteneer | Philips | HTC 37; 5656 AE Eindhoven The Netherlands |  | Dee.Denteneer@philips.com |
| GuidoHiertz | Philips | HTC 37; 5656 AE Eindhoven The Netherlands |  | Guido.Hiertz@Riedel.com |

Abstract

The document implements the comment resolutions in 837r2, except for the resolutions to CIDs 258 and 360.

- Normative References

Insert the following new normative references:

IETF RFC 5297, Synthetic Initialization Vector (SIV) Authenticated Encryption Using the Advanced Encryption Standard (AES), D. Harkins, October 2008 (status: informational).

IETF RFC 2409, The Internet Key Exchange (IKE), D. Harkins, D. Carrel, November 1998. (status: Standards Track)

- Definitions

Change definition 3.42 Distribution system service (DSS) as indicated:

- Distribution system service (DSS): The set of services provided by the distribution system (DS) that

- enable the medium access control (MAC) to transport MAC service data units (MSDUs) between stations

- (STAs) that are not in direct communication with each other over a single instance of the wireless medium

- (WM). These services include transport of MSDUs between the access points (APs) of basic service sets

- (BSSs) within an extended service set (ESS), transport of MSDUs between portals and BSSs within an ESS, transport of MSDUs between mesh stations (mesh STAs) of mesh basic services sets (MBSSs), and transport of MSDUs between STAs in the same BSS in cases where the MSDU has a multicast or

- broadcast destination address or where the destination is an individual address and the STA is associated

- with an AP. DSSs are provided between pairs of IEEE 802.11 MACs.

Delete definition 3.170 wireless distribution system (WDS)

Insert the following new definitions:

- The subclause numbering of definition is of the form “3.s<x>” where <x> is an increasing number. These numbers need to be assigned before the sponsor ballot.

- candidate peer mesh station (STA): A neighbor mesh STA to which a peering has not been established but meets eligibility requirements to become a peer mesh STA.

- destination mesh STA: A mesh STA where a frame leaves the MBSS. A destination mesh STA may be a mesh STA that is the final destination of a frame or a proxy mesh STA that receives a frame from the MBSS and translates and forwards the frame to an entity outside of the MBSS.

- DTIM interval: The value, in Time Units, that is equivalent to the product of the Beacon Interval and the DTIM Period subfields specified within the TIM element in Beacon frames.

- link metric: A criterion used to characterize the performance/quality/eligibility of a link.

- mesh: A network consisting of two or more mesh stations (STAs) communicating via mesh services.

- mesh access point: A mesh station (STA) that is collocated with one or more access point(s).

- Mesh Awake Window: A period of time where the mesh STA operates in awake state after its Beacon or Probe Response frame transmission that contained the Mesh Awake Window element.

- mesh basic service set (MBSS): A basic service set (BSS) that forms a self-contained network of mesh stations (mesh STAs), and which may be used as a distribution system (DS).

- mesh BSS service area (MBSSSA): The area within which members (mesh stations) of a mesh basic service set (MBSS) may communicate.

- 3.xy mesh coordination function (MCF): A coordination function that combines aspects of the contention-based and reduced contention access methods. The MCF includes the functionality provided by both enhanced distributed channel access (EDCA) and mesh controlled channel access (MCCA).

- Mesh Data Frame: A data frame with the FromDS and ToDS bits set to one that is transmitted from a mesh STA to a peer mesh STA.

- mesh coordinated channel access (MCCA): A coordination function for the MBSS.

- mesh coordinated channel access opportunity (MCCAOP): A period of time that is set up between a transmitter and a receiver.

- mesh link: A link from one mesh station (STA) to a neighbor mesh STA that has been established with the peering management protocol.

- mesh neighborhood: The set of all neighbor mesh STAs relative to a particular mesh STA.

- mesh path: A concatenated set of mesh links from a source mesh STA to a destination mesh STA.

- mesh path selection: The process of selecting a mesh path.

- mesh power mode: a mesh STA can select its power mode seen by neighboring peer mesh STA among active, light sleep and deep sleep modes. For peer mesh STAs the mesh power mode is peering specific.

- mesh services: The set of services that enable the creation and operation of an MBSS.

- mesh station (mesh STA): A station (STA) that implements the mesh facility. A mesh STA that operates in the Mesh BSS (MBSS) may provide the distribution services for other mesh STAs.

- neighbor mesh station (STA): A mesh STA that has a link with another mesh STA. Not all neighbor mesh STAs are peer mesh STAs.

- neighbor peer mesh station (STA): A mesh station (STA) to which a peering has been established and that has a link with its peer mesh STA.

- neighbor station (STA): From the point of view of a given STA, the set of neighbor STAs includes all STAs that it has a link with. In an MBSS, neighbor mesh STAs might not be peer mesh STAs.

- path metric: An aggregate multi-hop criterion used to characterize the performance/quality/eligibility of a mesh path.

- peering: A logical relationship from one mesh STA to another mesh STA that has been established with the mesh peering management protocol.

- peer mesh station (STA): A mesh STA to which a peering has been established. Not all peer mesh STAs are neighbor mesh STAs.

- peer service period (PSP): A peer service period is a contiguous period of time during which one or more individually addressed frames are transmitted between two peer mesh STAs and at least one mesh STA operates in power save mode. A peer service period is directional and may contain one or more TXOPs. One mesh STA transmits frames and initiates the termination of the peer service period. A mesh STA may have multiple peer service periods ongoing in parallel. At most one peer service period can be set up in each direction with each peer mesh STA.

- protocol instance: An execution of a particular protocol that consists of the state of the communicating parties as well as the messages exchanged.

- proxy mesh station (STA): A mesh STA that represents 802 entities outside the mesh basic service set (MBSS).

- source mesh STA: A mesh STA where a frame enters the MBSS. A source mesh STA may be a mesh STA that is the original source of a frame or a proxy mesh STA that receives a frame from an entity outside of the MBSS and translates and forwards the frame on a mesh path.

- Frame formats

- MCCAOP Reservation field

The MCCAOP Reservation field is a 4 octet field specifying the starts of a series of MCCAOPs, relative to the DTIM interval of the reporting mesh STA, and their (common) duration. The MCCAOP Reservation field consists of 3 subfields and its format is illustrated in MCCAOP Reservation field.

| MCCAOP Duration | MCCAOP Periodicity | MCCAOP Offset |
| --- | --- | --- |
| Octets: 1 | 1 | 2 |
| - MCCAOP Reservation field |  |  |

The MCCAOP Duration field specifies the duration of the MCCAOPs in multiples of 32µs.

The MCCAOP Periodicity field is an eight bit unsigned number. A non-zero Periodicity field specifies the number of MCCAOPs in the DTIM interval. These MCCAOPs are repeated in all DTIM intervals until the MCCAOP Reservation is torn down. A value of zero indicates a non-repeated MCCAOP in the DTIM interval following the setup.

The MCCAOP Offset field specifies the beginning of the MCCAOP relative to the beginning of each subinterval. The value is specified in multiples of 32 µs.

An example of periodicity, duration, and offset values for an MCCAOP Reservation field is shown in Example MCCAOP Reservation. In this particular example, the periodicity equals four, so that there are four MCCAOPs within the DTIM interval. As further illustrated in the figure, the offset value indicates the start of the MCCAOPs relative to the beginning of these subintervals.

- Example MCCAOP Reservation

- Information elements

- MCCAOP Setup Request element

The MCCAOP Setup Request element is used to setup one or more MCCAOPs. This information element is transmitted in individually addressed MCCA action frames. The mesh STA transmitting the MCCA Setup Request element is the MCCAOP owner of the MCCAOPs defined in this setup. The receivers of the MCCAOP Setup Request are the MCCAOP responders.The format of the information element is as shown in MCCAOP Setup Request element.

| Element ID | Length | MCCAOP Reservation ID | MCCAOP Reservation |
| --- | --- | --- | --- |
| Octets: 1 | 1 | 1 | 4 |
| - MCCAOP Setup Request element |  |  |  |

The Element ID is set to the value given in Error: Reference source not found for this information element. The Length is set to 5 octets.

The MCCAOP Reservation ID field is an eight bit unsigned number that represents the ID for the MCCAOP reservation. It is determined by the MCCAOP owner. When used in combination with the MAC address of the MCCAOP owner, the MCCAOP Reservation ID uniquely identifies the MCCAOP Reservation.

If this MCCAOP Setup Request is for an individually addressed transmission, the MCCAOP Reservation ID is between 0 and 127 and the Setup Request is transmitted in an individually addressed frame to the intended responder. If this MCCAOP Setup Request is for a group addressed transmission, the MCCAOP Reservation ID is between 128 and 254 and the Setup Request is transmitted in individually addressed frames to each of the intended responders.

The MCCAOP Reservation field specifies the starts and duration of a series of MCCAOPs, as described in MCCAO.

- MCCAOP Setup Reply element

The MCCAOP Setup Reply element is used to reply to an MCCAOP Setup Request. Its format is as shown in Error: Reference source not found.

The Element ID is set to the value given in Error: Reference source not found for this information element. The length is set to 2 or 6 octets.

| Element ID | Length | MCCAOP Reservation ID | MCCA Reply Code | MCCAOP Reservation |
| --- | --- | --- | --- | --- |
| Octets: 1 | 1 | 1 | 1 | 4 |
| - MCCAOP Setup Reply element |  |  |  |  |

The MCCAOP Reservation ID field is an eight bit unsigned number that represents the ID for the requested series of MCCAOPs. It is determined by the MCCAOP owner. When used in combination with the MAC address of the MCCAOP owner, the MCCAOP Reservation ID uniquely identifies one or more MCCAOPs. If this MCCAOP Setup Request is for an individually addressed transmission, the MCCAOP Reservation ID is between 0 and 127. If this MCCAOP Setup Request is for a group addressed transmission, the MCCAOP Reservation ID is between 128 to 254.

The MCCA reply Code field is defined in Error: Reference source not found.

The MCCAOP Reservation field includes an alternative to the MCCAOP Reservation specified in the MCCAOP Setup Request message. Its format is described in MCCAO. This field is an optional field and is only included and interpreted when the reply code indicates rejection.

- MCCA Reply Code field

The MCCA Reply Code field is a one Octet field that contains the responses used in an MCCAOP Setup Reply element. The reply codes are defined in MCCA Reply codesinput.doc

| - MCCA Reply codesinput.doc |  |
| --- | --- |
| MCCA Reply code | Meaning |
| 0 | Accept |
| 1 | Reject: MCCAOP Reservation conflict |
| 2 | Reject: MAF limit exceeded |
| Other | Reserved |

- MCCAOP Advertisements element

The MCCAOP Advertisements element is used by a mesh STA to advertise its MCCA state to its neighbors. This information element can be carried in selected Beacon frames with a chosen frequency. This information element is also transmitted in an MCCA action frame. The format of the information element is as shown in MCCAOP Advertisements element.

| Element ID | Length | MCCA Information | TX-RX Times Report | Broadcast Times Report | Interfering Times Report |
| --- | --- | --- | --- | --- | --- |
| Octets: 1 | 1 | 2 | variable | variable | variable |
| - MCCAOP Advertisements element |  |  |  |  |  |

The Element ID is set to the value given in Error: Reference source not found for this information element. The Length is set to 1 to 255 octets.

The MCCA Information field is a field of 2 octets. It is described in MCCA Information field.

The TX-RX Times Report field is a variable length field that contains an MCCAOP Reservation Report field as described in MCCAOP Reservation Report field. Through the TX-RX Times Report field, a mesh STA advertises the MCCAOPs in which it is involved as an MCCAOP owner or an MCCAOP responder and in which individually addressed frames are transmitted.

The Broadcast Times Report field is a variable length field that contains an MCCAOP Reservation Report field as described in MCCAOP Reservation Report field. Through the Broadcast Times Report field, a mesh STA advertises the MCCAOPs in which it is involved as an MCCAOP owner or an MCCAOP responder and in which group addressed frames are transmitted. These reports optionally include known and otherwise un-advertised transmission and reception times of the mesh STA besides MCCAOPs, for example, its HCCA times.

The Interfering Times Report field is a variable length field that contains an MCCAOP Reservation Report field as described in MCCAOP Reservation Report field. Through the Interfering Times Report field, a mesh STA reports the TX-RX and Broadcast Times of its neighbors, excluding its own TX-RX and Broadcast times.

- MCCAOP Reservation Report field

The MCCAOP Reservation Report field is a variable length field that is used to report a number of MCCAOP Reservations. The field consists of a variable number of subfields and its format is illustrated in MCCAOP Reservation Report field

| Number of Reported MCCAOP Reservations | MCCAOP Reservation 1 | ... | MCCAOP Reservation n |
| --- | --- | --- | --- |
| Octets: 1 | 4 |  | 4 |
| - MCCAOP Reservation Report field |  |  |  |

The Number of Reported MCCAOP Reservations is a field of one octet with an unsigned integer that specifies the number, n, of MCCAOP Reservations reported in this field.

The MCCAOP Reservation 1 through MCCAOP Reservation n fields specify the MCCAOP Reservations reported. Each field is of length 4 Octets and contains an MCCAOP Reservation field, whose format is shown in MCCAOP Reservation field in MCCAO.

- MCCA Information field

The MCCA Information field is a field of length 2 Octets that is used to provide information on the MCCAOP Reservations. The field consists of six subfields and its format is illustrated in .

| B0-B7 | B8-B11 | B12 | B13 | B14 | B15 |
| --- | --- | --- | --- | --- | --- |
| MCCA Access Fraction | MCCA Access Fraction Limit | TX-RX Report Present | Broadcast Report Present | Interfering Report Present | Partial Report |
| Bits: 8 | 4 | 1 | 1 | 1 | 1 |
| - MCCA Information field |  |  |  |  |  |

The MCCA Access Fraction is an eight bit unsigned number field and the MCCA Access Fraction Limit field is a 4 bit unsigned number field. The MCCA Access Fraction denotes a positive fraction of the MCCA Access Fraction Limit expressed in units of 1/255. The MCCA Access Fraction Limit denotes a positive fraction of the DTIM interval length expressed in units of (1/16). The MCCA Access Fraction field represents the current value of MCCA Access Fraction at the mesh STA rounded down (floor) to the nearest multiple of (1/256). The MCCA Access Fraction Limit field represents the maximum MCCA access fraction allowed at the mesh STA. This number is always a multiple of (1/16).

The TX-RX Report Present is a one bit field. It is set to 1 if a TX-RX Report is present in the MCCAOP Advertisement and set to 0 if no TX-RX Report is present.

The Broadcast Report Present is a one bit field. It is set to 1 if a Broadcast TX-RX Report is present in the MCCAOP Advertisement and set to 0 if no Broadcast TX-RX Report is present.

The Interfering Report Present is a one bit field. It is set to 1 if an Interfering Times Report is present in the MCCAOP Advertisement and set to 0 if no Interfering Times Report is present.

The Partial Report field is a one bit field. It is set to 0 if the Advertisement element includes all known reservations and it is set to 1 if the Advertisement element does not include all known reservations.

- MCCAOP Reservation Teardown element

The MCCAOP Reservation Teardown element is used to announce the teardown of an MCCAOP Reservation. Its format is shown in MCCAOP Reservation Teardown element.

| Element ID | Length | MCCAOP Reservation ID | MCCAOP Owner |
| --- | --- | --- | --- |
| Octets: 1 | 1 | 1 | 6 |
| - MCCAOP Reservation Teardown element |  |  |  |

The Element ID is set to the value given in Error: Reference source not found for this information element. The Length is variable and set to 1 or 7 octets.

An MCCAOP Reservation Teardown element is transmitted by either the MCCAOP owner or the MCCAOP responder of a MCCAOP Reservation to tear down the MCCAOP Reservation. The MCCAOP Owner field is an optional field that indicates the MAC address of the MCCAOP owner. This field is only included if the information element is transmitted by the MCCAOP responder. The MCCAOP Reservation Teardown element is transmitted in Beacon frames or group addressed MCCA action frames.

- Action frame format details

.

- Mesh Resource Coordination action frame details

Action frame formats for management of Mesh Resource Coordination are defined in this subclause.

An Action field, in the octet field immediately after the Category field, differentiates the frame format. The Action field values associated with each frame format are defined in Mesh Resource Coordination Action field valuesinput.doc.

| - Mesh Resource Coordination Action field valuesinput.doc |  |
| --- | --- |
| Action field value | Description |
|  | Congestion Control Notification |
|  | MCCA Setup Request |
|  | MCCA Setup Reply |
|  | MCCAOP Advertisement Request |
|  | MCCAOP Advertisements |
|  | MCCAOP Reservation Teardown |
|  | TBTT Adjustment Request |
|  | Mesh Channel Switch Announcement |
| 8-255 | Reserved |

- Congestion Control Notification frame format

The Congestion Control Notification frame uses the Action frame body format and is sent by a mesh STA to its neighbor peer mesh STA(s) to indicate its congestion status. The body is shown in Congestion Control Notification frame bodyinput.doc.

| - Congestion Control Notification frame bodyinput.doc |  |  |
| --- | --- | --- |
| Order | Information | Notes |
|  | Category |  |
|  | Action |  |
|  | Congestion Notification element |  |
| Last | Vendor Specific | Optionally present: one or more vendor-specific information elements. This information element follows all other information elements. |

The Category field is set to the value in Error: Reference source not found for category Mesh Resource Coordination.

The Action field is set to the value in Mesh Resource Coordination Action field valuesinput.doc for this action frame type.

The Congestion Notification element field is set as described in Error: Reference source not found.

- MCCA Setup Request frame format

The MCCA Setup Request frame is used to request the setup of a set of MCCAOPs. It is transmitted by a mesh STA with dot11MCCAEnabled set to a neighbor peer mesh STA with dot11MCCAEnabled set. This frame is transmitted using individual addresses. The frame body of an MCCA Setup Request frame contains the information shown in MCCA Setup Request frame bodyinput.doc.

| - MCCA Setup Request frame bodyinput.doc |  |  |
| --- | --- | --- |
| Order | Information | Notes |
|  | Category |  |
|  | Action |  |
|  | MCCA Setup Request element |  |
| Last | Vendor Specific | Optionally present: one or more vendor-specific information elements. This information element follows all other information elements. |

The Category field is set to the value in Error: Reference source not found for category Mesh Resource Coordination.

The Action field is set to the value in Mesh Resource Coordination Action field valuesinput.doc for this action frame type.

The MCCA Setup Request element is described in Error: Reference source not found.

- MCCA Setup Reply frame format

The MCCA Setup Reply frame is used to reply to an MCCAOP Setup Request. It is transmitted by a mesh STA with dot11MCCAEnabled set to a neighbor peer mesh STA with dot11MCCAEnabled set. This frame is transmitted using individual addresses. The frame body of an MCCA Setup Reply frame contains the information shown in MCCA Setup Reply frame bodyinput.doc.

| - MCCA Setup Reply frame bodyinput.doc |  |  |
| --- | --- | --- |
| Order | Information | Notes |
|  | Category |  |
|  | Action |  |
|  | MCCA Setup Reply element |  |
| Last | Vendor Specific | Optionally present: one or more vendor-specific information elements. This information element follows all other information elements. |

The Category field is set to the value in Error: Reference source not found for category Mesh Resource Coordination.

The Action field is set to the value in Mesh Resource Coordination Action field valuesinput.doc for this action frame type.

The MCCA Setup Reply element is described in Error: Reference source not found.

- MCCAOP Advertisement Request frame format

The MCCAOP Advertisement Request frame is transmitted by a mesh STA with dot11MCCAEnabled set to request MCCA advertisements from neighbor peer mesh STAs. The frame body of an MCCAOP Advertisement Request frame is shown in MCCAOP Advertisement Request frame bodyinput.doc.

| - MCCAOP Advertisement Request frame bodyinput.doc |  |  |
| --- | --- | --- |
| Order | Information | Notes |
|  | Category |  |
|  | Action |  |
| Last | Vendor Specific | Optionally present: one or more vendor-specific information elements. This information element follows all other information elements. |

The Category field is set to the value in Error: Reference source not found for category Mesh Resource Coordination.

The Action field is set to the value in Mesh Resource Coordination Action field valuesinput.doc for this action frame type.

- MCCAOP Advertisements frame format

The MCCAOP Advertisements frame is transmitted by a mesh STA with dot11MCCAEnabled set to one or more neighbor peer mesh STAs with dot11MCCAEnabled set. This frame is transmitted using group addresses or individual addresses. The frame body of an MCCAOP Advertisements frame contains the information shown in MCCAOP Advertisements frame bodyinput.doc.

| - MCCAOP Advertisements frame bodyinput.doc |  |  |
| --- | --- | --- |
| Order | Information | Notes |
|  | Category |  |
|  | Action |  |
|  | MCCA Advertisements element |  |
| Last | Vendor Specific | Optionally present: one or more vendor-specific information elements. This information element follows all other information elements. |

The Category field is set to the value in Error: Reference source not found for category Mesh Resource Coordination.

The Action field is set to the value in Mesh Resource Coordination Action field valuesinput.doc for this action frame type.

The MCCAOP Advertisements element is described in Error: Reference source not found.

- MCCAOP Reservation Teardown frame format

The MCCAOP Reservation Teardown frame is transmitted by a mesh STA with dot11MCCAEnabled set to one or more neighbor peer mesh STAs with dot11MCCAEnabled set. This frame is transmitted using group addresses or individual addresses. The frame body of an MCCAOP Reservation Teardown frame contains the information shown in MCCAOP Reservation Teardown frame bodyinput.doc.

| - MCCAOP Reservation Teardown frame bodyinput.doc |  |  |
| --- | --- | --- |
| Order | Information | Notes |
|  | Category |  |
|  | Action |  |
|  | MCCA Reservation Teardown element |  |
| Last | Vendor Specific | Optionally present: one or more vendor-specific information elements. This information element follows all other information elements. |

The Category field is set to the value in Error: Reference source not found for category Mesh Resource Coordination.

The Action field is set to the value in Mesh Resource Coordination Action field valuesinput.doc for this action frame type.

The MCCAOP Reservation Teardown element is described in Error: Reference source not found.

Change the first paragraph of Clause 9 as following:

- MAC sublayer functional description

The MAC functional description is presented in this clause. The architecture of the MAC sublayer, including the distributed coordination function (DCF), the point coordination function (PCF), the hybrid coordination function (HCF), the Mesh Coordination Function (MCF), and their coexistence in an IEEE 802.11 LAN are introduced in 9.1. These functions are expanded on in 9.2 (DCF), 9.3 (PCF), and 9.9 (HCF), and 9.9a (MCF). Fragmentation and defragmentation are defined in 9.4 and 9.5. Multirate support is addressed in 9.6. A number of additional restrictions to limit the cases in which MSDUs are reordered or discarded are described in 9.7. Operation across regulatory domains is defined in 9.8. The Block Ack mechanism is described in 9.10. The No Ack mechanism is described in 9.11. The protection mechanism is described in 9.13.

- MAC architecture

Insert the following text to the end of 9.1:

Due to the distributed nature of the MBSS, only the MCF is present in a mesh STA.

- Hybrid coordination function (HCF)

- HCF contention-based channel access (EDCA)

Change the second paragraph in 9.1.3.1 as shown:

- The following text is based on the amendment by 11p.

For each AC, an enhanced variant of the DCF, called an enhanced distributed channel access function (EDCAF), contends for TXOPs using a set of EDCA parameters. When communicating outside the context of a BSS (dot11OCBEnabled is true), the EDCA parameters are the corresponding default values or are as set by the SME in the MIB attribute table dot11EDCATable. When communicating within a BSS, the EDCA parameters used are from the EDCA Parameter Set element, or from the default values for the parameters when no EDCA Parameter Set element is received from the AP of the BSS with which the STA is associated or when the STA is a mesh STA. The parameters used by the EDCAF to control its operation are defined by MIB attribute table dot11QAPEDCATable at the AP and by MIB attribute table dot11EDCATable at the non-AP STA or at the mesh STA.

Change the second to last paragraph in 9.1.3.1 as shown:

The management frames shall be sent using the access category AC_VO without being restricted by admission control procedures. A QoS STA or mesh STA shall also send management frames using the access category AC_VO before associating with any BSS, even if there is no QoS facility available in that BSS. BlockAckReq and BlockAck control frames shall be sent using the same QoS parameters as the corresponding QoS data frames. PS-Poll control frames shall be sent using the access category AC_BE to reduce the likelihood of collision following a Beacon frame. For the purpose of determining the proper AC for an RTS frame, the RTS frame shall inherit the UP of the data or management frame(s) that are included in the frame exchange sequence where the RTS is the first frame.

Insert the following new subclause after the 9.1.3.2:

- Mesh coordination function (MCF)

The mesh facility includes an additional coordination function called MCF that is usable only in an MBSS. Mesh STAs shall implement the MCF only. The MCF uses both a contention-based channel access method, defined by the enhanced distributed channel access (EDCA) mechanism for contention-based transfer and a controlled channel access, referred to as the Mesh Coordinated Channel Access (MCCA) mechanism, that optimizes frame exchanges in the MBSS. The operation rules of the EDCA are defined in 9.9.1. The operation rules of the MCCA are defined in MCF controlled channel access (MCCA).

- Multirate support

- The subclause numbering is based on the amendment by 11n.

Insert the following new subclause after the 9.6.0c.

- Basic Rate Set and Basic MCS Set for mesh STA

Because a mesh STA does not establish a peering with a mesh STA using a different BSSBasicRateSet, all mesh STAs in an MBSS use the same BSSBasicRateSet. See Error: Reference source not found.

Mesh STAs should adopt the mandatory PHY rates as the default BSSBasicRateSet to reduce the risk that candidate peer mesh STA utilizes different BSSBasicRateSet. If the mesh STA is also a HT STA, it should adopt the MCSs of mandatory MCSs as the default BSSBasicMCSSet.

Once the mesh STA establishes a peering with a mesh STA, it shall change neither the BSSBasicRateSet nor the BSSBasicMCSSet parameters.

- HCF

- HCF contention-based channel access (EDCA)

- EDCA TXOPs

Change the fifth paragraph in 9.9.1.2 as shown:

It should be noted, that when transmitting multiple frames in a TXOP using acknowledgment mechanisms other than Normal Ack, a protective mechanism should be used (such as RTS/CTS or the protection mechanism described in 9.13). A QoS AP may send broadcast/multicast group addressed frames without using any protection mechanism. In a QoS IBSS, or in an MBSS, or when dot11OCBEnabled is true, broadcast/multicast group addressed frames shall be sent one at a time, and backoff shall be performed after the transmission of each of the broadcast/multicast group addressed frames.

- EDCA backoff procedure

Change the second to last dashed list item in 9.9.1.5 as shown:

- The lettered list is changed to dashed list based on the amendment by 11n.

- If the QSRC[AC] or the QLRC[AC] for the QoS STA or mesh STA has reached dot11ShortRetryLimit or dot11LongRetryLimit respectively, CW[AC] shall be reset to CWmin[AC].

- Retransmit procedures

Change the text in 9.9.1.6 as shown:

QoS STAs and mesh STAs shall maintain a short retry counter and a long retry counter for each MSDU, A-MSDU or MMPDU that belongs to a TC requiring acknowledgment. The initial value for the short and long retry counters shall be zero. QoS STAs and mesh STAs also maintain a short retry counter and a long retry counter for each AC. They are defined as QSRC[AC] and QLRC[AC], respectively, and each is initialized to a value of zero.

After transmitting a frame that requires acknowledgment, the STA shall perform either of the acknowledgment procedures as appropriate and as defined in 9.2.8 and 9.10.3. The short retry count for an MSDU, A-MSDU that is not part of a Block Ack agreement or for an MMPDU shall be incremented every time transmission of a frame of length less than or equal to dot11RTSThreshold fails for that MSDU, A-MSDU or MMPDU. QSRC[AC] shall be incremented every time transmission of an A-MPDU or frame of length less than or equal to dot11RTSThreshold fails. This short retry count and the QoS STA QSRC[AC] and mesh STAs OSRC[AC] shall be reset when an A-MPDU or frame of length less than or equal to dot11RTSThreshold succeeds. The long retry count for an MSDU or A-MSDU that is not part of a Block Ack agreement or for an MMPDU shall be incremented every time transmission of a MAC frame of length greater than dot11RTSThreshold fails for that MSDU, A-MSDU or MMPDU. QLRC[AC] shall be incremented every time transmission of an A-MPDU or frame of length greater than or equal to dot11RTSThreshold fails. This long retry count and the QLRC[AC] shall be reset when an A-MPDU or frame of length greater than dot11RTSThreshold succeeds. All retransmission attempts for an MPDU that is not sent under a Block Ack agreement and that has failed the acknowledgment procedure one or more times shall be made with the Retry field set to 1 in the data or management frame.

Retries for failed transmission attempts shall continue until the short retry count for the MSDU, A-MSDU or MMPDU is equal to dot11ShortRetryLimit or until the long retry count for the MSDU, A-MSDU or MMPDU is equal to dot11LongRetryLimit. When either of these limits is reached, retry attempts shall cease, and the MSDU, A-MSDU or MMPDU shall be discarded.

For internal collisions occurring with the EDCA access method, the appropriate retry counters (short retry counter for MSDU, A-MSDU or MMPDU and QSRC[AC] or long retry counter for MSDU, A-MSDU or MMPDU and QLRC[AC]) are incremented. For transmissions that use Block Ack, the rules in 9.10.3 also apply. STAs shall retry failed transmissions until the transmission is successful or until the relevant retry limit is reached.

With the exception of a frame belonging to a TID for which Block Ack is set up, a QoS STA or a mesh STA shall not initiate the transmission of any management or data frame to a specific RA while the transmission of another management or data frame with the same RA and having been assigned its sequence number from the same sequence counter has not yet completed to the point of success, retry fail, or other MAC discard (e.g., lifetime expiry).

QoS STAs and mesh STAs shall maintain a transmit MSDU timer for each MSDU passed to the MAC. The MIB attribute dot11EDCATableMSDULifetime specifies the maximum amount of time allowed to transmit an MSDU for a given AC. The transmit MSDU timer shall be started when the MSDU is passed to the MAC. If the value of this timer exceeds the appropriate entry in dot11EDCATableMSDULifetime, then the MSDU, or any remaining, undelivered fragments of that MSDU, shall be discarded by the source STA without any further attempt to complete delivery of that MSDU.

When A-MSDU aggregation is used, the HT STA maintains a single timer for the whole A-MSDU. The timer is restarted each time an MSDU is added to the A-MSDU. This ensures that no MSDU in the A-MSDU is discarded before a period of dot11EDCATableMSDULifetime has elapsed.

Insert the following new subclause after 9.9:

- MCF

Under MCF, the basic unit of allocation of the right to transmit onto the WM is the TXOP. Each TXOP is defined by a starting time and a defined maximum length.

Under MCF, there are two types of TXOPs: EDCA TXOPs and MCCAOPs. The EDCA TXOP is obtained by a mesh STA winning an instance of EDCA contention (see 9.9.1). The MCCAOP is an advance reservation for the time interval for its transmission by means of an MCCA Opportunity Reservation  (see 9.9a.2). An EDCA TXOP and an MCCAOP shall not exceed dot11MaxDwellTime (if using an FH PHY).

- MCF contention-based channel access

In an MBSS, EDCA implements the contention-based channel access. 9.9.1 defines EDCA.

- MCF controlled channel access (MCCA)

Mesh Coordinated Channel Access (MCCA) is an optional access method that allows mesh STAs that support MCCA to access the WM at selected times with lower contention than would otherwise be possible. This standard does not require all mesh STAs to use MCCA. MCCA can be used by a subset of mesh STAs in an MBSS. However, MCCA connections can only be setup among mesh STAs that operate on the same channel and that are in MCCA enabled mode. The performance of MCCA may be impacted by devices that do not respect MCCAOP reservations.

MCCA capable mesh STAs use management frames to make an advance reservation for transmissions. Via MCCA reports, they advertise these advance reservations to their neighbors.  The MCCA capable neighbor mesh STAs that could cause interference to transmissions during these reserved time periods, or that would experience interference from them, shall shall not initiate a transmission during these reserved time periods.. The MCCAOP owner and the MCCAOP responders access the WM during these MCCAOPs using HCF contention-based channel access (EDCA) described in HCF contention-based channel access (EDCA).

In order to use MCCA, a mesh STA maintains synchronization with its neighboring mesh STAs. Additionally, a mesh STA shall track the reservations of its neigbhoring mesh STAs..

- MCCA operation process

The mesh STA that decides to use MCCA shall scan the current operating channel for dot11MCCAScanDuration. During the scan operation, the mesh STA shall not initiate or accept new MCCAOPs, but the mesh STA shall set the MCCA Enabled bit to 1 in Beacon and Probe Response frames it transmits. The mesh STA shall transmit a MCCAOP Advertisements frame to neighbor mesh STAs with dot11MCCAEnabled set and do not listen to the Beacons from the mesh STA. After the mesh STA has scanned the channel for dot11MCCAScanDuration it may initiate and accept MCCAOP Reservations.

A mesh STA with dot11MCCAEnabled setmay discover other neighboring mesh STAs with dot11MCCAEnabled set by receiving a Beacon, Probe Response, or MCCAOP Advertisements frames from the neighboring mesh STAs.

- MCCA opportunity (MCCAOP)

An MCCAOP is a period of time, defined by a target transmission starting time and a duration, set up between an MCCAOP owner and one (for individually addressed frames) or more (for group addressed frames) MCCAOP responders for frame transmissions. MCCAOPs are setup by means of the procedure defined in MCCAOP setup procedure. Once an MCCAOP is setup,

- Access to the channel by MCCA-supporting mesh STAs is governed by the procedures in Access during MCCAOP.

- The MCCAOP is advertised according to the procedures in MCCAOP Advertisements .

- MCCAOP Reservations

An MCCAOP Reservation specifies one or more MCCAOPs by means of the MCCAOP Reservation field defined in MCCAO. The target transmission starting times of these MCCAOPs are specified relative to the start of the DTIM interval of the reporting mesh STA. These MCCAOPs have a common duration.

The Periodicity subfield of the MCCAOP Reservation field indicates the periodicity of the MCCAOPs re-served via the reservation. If the Periodicity field equals 0 then the MCCAOP Reservation specifies one MCCAOP located in the DTIM interval of the MCCAOP owner following the successful completion of the MCCAOP setup procedure described in MCCAOP setup procedure.

If the Periodicity field is positive then the MCCAOP Reservation specifies one or more MCCAOPs repeated in each DTIM intervals of the MCCAOP owner following the successful completion of the MCCAOP setup procedure until the MCCAOP Reservation is torn down.

The locations within each DTIM interval of the MCCAOPs specified in the MCCAOP Reservation are specified relative to the start of the DTIM interval of the reporting mesh STA, as described in MCCAO. If a mesh STA adjusts its TBTT, e.g., in response to a TBTT Adjustment Request, it shall adjust the Reports accordingly by modifying the MCCAOP Offset of each of the reported MCCAOP Reservations.

The set of MCCAOP Reservations in which a mesh STA is involved as an MCCAOP owner or an MCCAOP responder are referred to as the TX-RX times of this mesh STA.

An MCCAOP Reservation is identified by an MCCAOP Reservation ID. The MCCAOP owner shall select an MCCAOP Reservation ID that is unique among all of its MCCAOP Reservations. The MCCAOP Reservation ID and MCCAOP owner’s MAC address uniquely identify the MCCAOP Reservation in the mesh BSS.

The MCCAOP Reservation ID is an 8-bit unsigned number. If this MCCAOP Setup Request is for the individually addressed transmission, the MCCAOP Reservation ID is between 0 and 127. If this MCCAOP Setup Request is for the group addressed transmission, the MCCAOP Reservation ID is between 128 and 254. The special value of MCCAOP Reservation ID, when all bits are set to 1, is used to indicate all MCCAOPs.

- Neighborhood MCCAOP times at a mesh STA

In a mesh STA’s mesh neighborhood, all the TX-RX Times and Broadcast Times reported by its neighbors, as described in MCCAOP Advertisements , form a set of MCCAOPs that are already being used in the neighborhood. These times are referred to as Neighborhood MCCAOP times for the mesh STA. In effect, Neighborhood MCCAOP Times at a mesh STA include all MCCAOPs for which the mesh STA or one of its neighbors, including neighbors from other MBSSs, are either transmitters or receivers.

A mesh STA shall track at least dot11MCCATrackStates of these MCCAOP Reservations, including its own. No new MCCAOPs may be set up by the mesh STA during these times as specified in MCCAOP setup procedure. Access during MCCAOP specifies the access to the medium during the tracked Neighborhood MCCAOP Times.

- Interfering Times for a mesh STA

Through the Interfering Times Report in its MCCAOP Advertisements, a mesh STA reports MCCAOP Reservations that its neighbors have advertised in their TX-RX Reservation Reports and in their Broadcast Reservation Reports and in which it is not involved itself. These times shall not be used for a new MCCAOP with the reporting mesh STA as they may experience interference as specified in MCCAOP setup procedure.

The Interfering Times are directly derived from neighbor peer mesh STAs’ TX-RX Times Report. The Interfering Times Report reflects the latest TX-RX Times Reports from the neighbor peer mesh STAs.

- MCCA access fraction (MAF)

The MCCA Access Fraction at a mesh STA is the ratio of the total duration of its Neighborhood MCCAOP Times, defined in Neighborhood MCCAOP times at a mesh STA, in a DTIM interval to the duration of the DTIM interval. This parameter is reported in the MCCA Information field in the MCCAOP Advertisements element. The maximum value for the MAF that is allowed at a mesh STA is specified by the dot11MAFlimit. The dot11MAFlimit is copied into the MCCA Access Fraction Limit field of the MCCA Information field in the MCCAOP Advertisements element.

The MAF and MAF Limit may be used to limit the use of MCCA in a mesh STAs mesh neighborhood, as specified in MCCAOP setup procedure. Before attempting to set up an MCCAOP Reservation with a peer mesh STA, a mesh STA is required to ensure that the new MCCAOP Reservation does not cause the MAF of itself or any of its neighbor mesh STAs to exceed the MAF Limit. An MCCAOP Setup Request shall be refused by the intended MCCAOP responder if the MAF limit of its own neighbors is exceeded due to the new setup.

- MCCAOP setup procedure

The setup of an MCCAOP Reservation is initiated by an MCCAOP owner, and is accepted/rejected by an MCCAOP responder. The setup procedure for an MCCAOP Reservation is as follows:

- The MCCAOP owner builds a map of Neighborhood MCCAOP times in the DTIM interval after hearing Advertisements from all of its neighbor mesh STAs with dot11MCCAActivated is true. If no advertisement was heard from a neighbor mesh STA in the last dot11MCCAdvertPeriodMax mesh DTIM intervals, the mesh STA may request for an MCCAOP Advertisement from the neighbor mesh STA.

- Based on traffic characteristics, it then chooses MCCAOP starting times and duration in the DTIM interval. The MCCAOP parameters shall be chosen so that they do not overlap with either the Neighborhood MCCAOP Times of the MCCAOP owner or the Interfering Times of the intended MCCAOP responder or responders. It also avoids using times that are known to it as being used by itself or one of its neighbor peer mesh STAs for other activities such as Beacon transmissions.

- It then verifies that the new MCCAOP Reservation will not cause the MAF limit to be exceeded for either itself or its neighbor mesh STAs. If the dot11MCCAOPTrackTimes is exceeded or if any of these MAF limit would be exceeded due to the new MCCAOP Reservation, it cancels the setup procedure.

- If the conditions in b) and c) are satisfied and if the MCCA Reservation is intended for individually addressed frames, the MCCAOP ownertransmits an MCCAOP Setup Request element to the intended MCCAOP responder with chosen MCCAOP starting times and duration.

- If the conditions in b) and c) are satisfied and if the MCCA Reservation is intended for group ad-dressed frames, the MCCAOP owner transmits an MCCAOP Setup Request to the intended MCCAOP responders. Such MCCAOP Setup Requests may be sent in individually addressed frames or group addressed frames.

- The MCCAOP responder verifies that the MCCAOP Reservation does not overlap with its Neighborhood MCCAOP Times. The MCCAOP responder verifies that the MCCAOP Reservation does not overlap with other times that it knows are set to be used by itself or its neighbor mesh STAs for activities such as Beacon transmissions. The MCCAOP responder also verifies that the new MCCAOP Reservation will not cause the MAF limit to be crossed for itself or its neighbor mesh STAs. If the conditions are not met, the responder sends an MCCAOP Setup Reply frame to the MCCAOP owner with the MCCA Reply Code set accordingly to indicate rejection of the MCCAOP Setup Request. The MCCAOP owner then cancels the MCCAOP Setup operation.

- If the MCCAOP request is for individually addressed transmissions, the MCCAOP responder verifies that the MCCAOP Reservation does not overlap with its Neighborhood MCCAOP Times. If the condition is satisfied, it transmits a MCCAOP Setup Reply frame to the MCCAOP owner, with the MCCA Reply Code set to Accept. If the condition is not satisfied, it may calculate an alternative MCCAOP Reservation by including an optional alternative reservation in the MCCAOP Reservation field of the MCCAOP Setup Reply element.

- If the MCCAOP request is for group addressed transmissions, the MCCAOP responder verifies that the MCCAOP Reservation does not overlap with its Neighborhood MCCAOP Times excluding those for which the intended MCCAOP owner itself is the owner of the overlapped times, i.e., it declares the times of the requested MCCAOP Reservation by transmitting the Broadcast MCCAOP advertisement. If the condition is satisfied, it transmits a MCCAOP Setup Reply frame to the MCCAOP owner, with the MCCA Reply Code set to Accept.

- If the MCCAOP responder accepts a Setup Request for a Broadcast MCCAOP, it includes the related times into its MCCAOP advertisements only after the MCCAOP advertisement from the owner of the Broadcast MCCAOP is received.

- If the MCCAOP owner receives an MCCA Setup Reply frame with MCCAOP Setup Reply code equal to Accept, the MCCAOP Reservation is established. Otherwise, the  mesh STA may repeat the MCCAOP Setup procedure using a modified MCCAOP Setup Request. If an alternative MCCAOP Reservation is included in the MCCAOP Setup Reply, the mesh STA may consider this alternative in its modified MCCAOP Setup Request.

- MCCAOP Advertisements

A mesh STA with dot11MCCAEnabled set shall advertise TX-RX, Broadcast, and Interfering Times using the MCCAOP Advertisements element, at least once in dot11MCCAdvertPeriodMax mesh DTIM intervals. It need not report all reservations in each MCCAOP Advertisements element. If it reports only a subset of the reservations, then this shall be indicated by setting the Partial Report field in the MCCAOP Information field to 1.

These advertisements shall be transmitted in Beacon frames or MCCA action frames.. The advertised times may include:

- TX-RX Times Report:

- All MCCAOP times for individually addressed frames for which the mesh STA is the MCCAOP owner or the MCCAOP responder.

- Other times that are reserved for individually addressed transmissions for which the mesh STA is either the transmitter or the receiver..

- Broadcast Times Report:

- All MCCAOP times for group addressed frames for which the mesh STA is the MCCAOP owner or the MCCAOP responder.

- Other times that are reserved for group addressed transmissions for which the mesh STA is either the transmitter or the receiver.  A non exhaustive list includes expected HCCA times for a mesh AP and its own or neighbor peer mesh STA’s expected beacon times.

- Interfering Times Report:

- TX-RX and Broadcast times reported by the mesh STA’s neighbor peer mesh STAs excluding those times for which the mesh STA is either the MCCAOP owner or the MCCAOP responder.

Each of the reports shall consist of a number of reported MCCAOP Reservations, as described in Error: Reference source not found. If a mesh STA adjusts its TBTT, e.g., in response to a TBTT Adjustment Request, it shall adjust the Reports accordingly by modifying the MCCAOP Offset of each of the reported MCCAOP Reservations.

- MCCAOP Reservation teardown

Either the MCCAOP owner or the MCCAOP responder may initiate a teardown by transmitting the MCCAOP Reservation Teardown frame to the other communicating end. A teardown shall be initiated when either MCCAOP owner or MCCAOP responder learn of a conflicting MCCAOP Reservation from an advertisement by a neighbor with a lower MAC address. After a teardown, the MCCAOP owner and MCCAOP responder stop advertising the MCCAOP Reservation in their TX-RX times.

The transmitter of the teardown assumes a successful teardown and stops advertising the MCCAOP Reservation in its TX-RX Times Report if any of the following happens:

- Its MCCAOP Reservation Teardown frame is successfully Acked.

- The advertisement of the receiver of the teardown does not include the MCCAOP Reservation.

- The receiver of the teardown is unreachable for greater than dot11MCCAOPtimeout time.

The receiver of the teardown assumes a successful teardown and stops advertising the MCCAOP Reservation in its TX-RX Times Report if any of the following happens:

- Its MCCAOP Reservation Teardown frame is successfully Acked.

- The advertisement of the transmitter of the teardown does not include the MCCAOP Reservation.

- The transmitter of the teardown is inactive for greater than dot11MCCAOPtimeout time.

- Access during MCCAOP

The access behavior for mesh STAs with dot11MCCAEnabled set during the Neighborhood MCCAOP times is described in Access by non-owners of MCCAOP.

- Access by MCCAOP Owners

An MCCAOP owner shall attempt to access the channel during the time set up for the MCCAOP and obtain a TXOP using EDCA contention and backoff parameters for the Access Category of the MSDU as defined in 9.9.1.3. The MCCAOP owner can foreshorten any reservation period by transmitting frames available under EDCA, see 9.9.1, that set the NAV at a receiving stations. These frames not only set the NAV of the receiving station, but also the time period during which the receiving station is blocked from accessing the medium due to the reservation.

A mesh STA successfully obtains a TXOP when the mesh STA completes a frame exchange with the receiver of the MSDU. If the mesh STA successfully obtains a TXOP, it may transmit until the EDCA TXOP limit for the Access Category of the MSDU is reached. If the mesh STA reaches the TXOP limit before the end of the MCCAOP, the mesh STA may transmit additional MSDU(s), if any are ready to be transmitted, using EDCA..

If a mesh STA accesses the channel during the MCCAOP but fails to obtain either an initial or a subsequent TXOP, the mesh STA shall perform the backoff procedure specified in 9.9.1.5.

After a mesh STA successfully obtains the TXOP, if there are multiple MSDUs to be transmitted, the transmission and retransmission rules for multiple MSDU transmission shall use the rules specified in 9.9.1.4 and 9.9.1.6.

- Access by non-owners of MCCAOP

A mesh STA with dot11MCCAEnabled is true shall maintain a Reservation Allocation Vector (RAV) for each reservation in its Interfering Times. A RAV is set when a corresponding MCCAOP starts and it is initialized to the duration of the MCCAOP given in the Duration field of the MCCAOP Reservation.. The RAVs are implemented as timers. They expire when they reach 0.

RAVs are set in addition to the NAV. At any time, both the NAV and the non-zero RAVs inhibit access to the medium.

A mesh STA may update its RAV upon receipt of a NAV setting frame that originates from either the owner or the responder of the reservation that is associated with the RAV. The rules for this are the same as for standard NAV resetting operation and are as described in 9.9.2.1. The RAV is not adjusted upon receipt of any NAV setting frame by other stations than the reservation owner or responder.

An EDCA TXOP of a mesh STA with dot11MCCAEnabled set shall not extend across any MCCAOP in its neigborhood MCCAOP times.

- MLME

\

- DFS procedures

- Selecting and advertising a new channel

Insert the following new subclause after 11.9.7.2:

- clause numbering based on 11n adding 11.9.7.3. The following clause should appear immediately after 11.9.7.2 (i.e., before 11n subclause 11.9.7.3)

- Selecting and advertising a new channel in an MBSS

If a mesh STA detects the need to switch the channel (e.g., due to regulatory requirement for radar avoidance), the mesh STA should inform peer mesh STAs to which a mesh link has been established.

Once the mesh STA identifies the candidate channel to switch its channel to, it shall initiate the Channel Switch protocol described in Error: Reference source not found.

A 20/40 MHz MBSS may be changed to a 20 MHz MBSS and a 20 MHz MBSS may be changed to a 20/40 MHz MBSS.

When an MBSS switches from a 20/40 MHz MBSS to a 20 MHz MBSS, some MCCAOP may need to be deleted to make sure that the MAF of each mesh STA does not exceed its MAF Limit.

When an MBSS switches from a 20 MHz MBSS to a 20/40 MHz MBSS or switches from a 20/40 MHz MBSS to a 20 MHz MBSS, a mesh STA may need to do path maintenance to find an optimized path.

- The following clause numbering based on 11u ending with 11B.

- (normative) ASN.1 encoding of the MAC and PHY MIB

Insert the following objects to the Major sections:

-- dot11MeshStationConfigTable 						::= { dot11smt 18 }

-- dot11MeshHWMPConfigTable 						::= { dot11smt 19 }

Change the “Dot11StationConfigEntry” of the “dotStationConfigTable” as follows:

Dot11StationConfigEntry ::=

SEQUENCE {

dot11StationID 								MacAddress,

dot11MediumOccupancyLimit 								INTEGER,

dot11CFPollable 								TruthValue,

dot11CFPeriod								INTEGER,

dot11CFPMaxDuration 								INTEGER,

dot11AuthenticationResponseTimeOut 								Unsigned32,

dot11PrivacyOptionImplemented 								TruthValue,

dot11PowerManagementMode 								INTEGER,

dot11DesiredSSID OCTET 								STRING,

dot11DesiredBSSType 								INTEGER,

dot11OperationalRateSet 								OCTET STRING,

dot11BeaconPeriod 								INTEGER,

dot11DTIMPeriod 								INTEGER,

dot11AssociationResponseTimeOut 								Unsigned32,

dot11DisassociateReason 								INTEGER,

dot11DisassociateStation 								MacAddress,

dot11DeauthenticateReason 								INTEGER,

dot11DeauthenticateStation 								MacAddress,

dot11AuthenticateFailStatus 								INTEGER,

dot11AuthenticateFailStation 								MacAddress,

dot11MultiDomainCapabilityImplemented 								TruthValue,

dot11MultiDomainCapabilityEnabled 								TruthValue,

dot11CountryString 								OCTET STRING,

dot11SpectrumManagementImplemented 								TruthValue,

dot11SpectrumManagementRequired 								TruthValue,

dot11RSNAOptionImplemented 								TruthValue,

dot11RSNAPreauthenticationImplemented 								TruthValue,

dot11RegulatoryClassesImplemented	 							TruthValue,

dot11RegulatoryClassesRequired 								TruthValue,

dot11QosOptionImplemented 								TruthValue,

dot11ImmediateBlockAckOptionImplemented 								TruthValue,

dot11DelayedBlockAckOptionImplemented 								TruthValue,

dot11DirectOptionImplemented 								TruthValue,

dot11APSDOptionImplemented 								TruthValue,

dot11QAckOptionImplemented 								TruthValue,

dot11QBSSLoadOptionImplemented 								TruthValue,

dot11QueueRequestOptionImplemented 								TruthValue,

dot11TXOPRequestOptionImplemented 								TruthValue,

dot11MoreDataAckOptionImplemented 								TruthValue,

dot11AssociatedinNQBSS 								TruthValue,

dot11DLSAllowdInQBSS 								TruthValue,

dot11DLSAllowed 								TruthValue,

dot11AssociateStation 								MacAddress,

dot11AssociateID 								INTEGER,

dot11AssociateFailStation 								MacAddress,

dot11AssociateFailStatus 								INTEGER,

dot11ReassociateStation 								MacAddress,

dot11ReassociateID 								INTEGER,

dot11ReassociateFailStation 								MacAddress,

dot11ReassociateFailStatus 								INTEGER,

dot11RadioMeasurementCapable 								TruthValue,

dot11RadioMeasurementEnabled 								TruthValue,

dot11RRMMeasurementProbeDelay 								INTEGER,

dot11RRMMeasurementPilotPeriod 								INTEGER,

dot11RRMLinkMeasurementEnabled 								TruthValue,

dot11RRMNeighborReportEnabled 								TruthValue,

dot11RRMParallelMeasurementsEnabled								TruthValue,

dot11RRMRepeatedMeasurementsEnabled								TruthValue,

dot11RRMBeaconPassiveMeasurementEnabled								TruthValue,

dot11RRMBeaconActiveMeasurementEnabled								TruthValue,

dot11RRMBeaconTableMeasurementEnabled 								TruthValue,

dot11RRMBeaconMeasurementReportingConditionsEnabled								TruthValue,

dot11RRMFrameMeasurementEnabled 								TruthValue,

dot11RRMChannelLoadMeasurementEnabled 								TruthValue,

dot11RRMNoiseHistogramMeasurementEnabled 								TruthValue

dot11RRMStatisticsMeasaurementEnabled 								TruthValue,

dot11RRMLCIMeasurementEnabled 								TruthValue,

dot11RRMLCIAzimuthEnabled 								TruthValue,

dot11RRMTransmitStreamCategoryMeasurementEnabled								TruthValue,

dot11RRMTriggeredTransmitStreamCategoryMeasurementEnabled TruthValue,

dot11RRMAPChannelReportEnabled 								TruthValue,

dot11RRMMIBEnabled 								TruthValue,

dot11RRMMaxMeasurementDuration 								Unsigned32,

dot11RRMNonOperatingChannelMaxMeasurementDuration								Unsigned32,

dot11RRMMeasurementPilotTransmissionInformationEnabled TruthValue,

dot11RRMMeasurementPilotCapability 								Unsigned32,

dot11RRMNeighborReportTSFOffsetEnabled 								TruthValue,

dot11RRMRCPIMeasurementEnabled 								TruthValue,

dot11RRMRSNIMeasurementEnabled 								TruthValue,

dot11RRMBSSAverageAccessDelayEnabled 								TruthValue,

dot11RRMBSSAvailableAdmissionCapacityEnabled 								TruthValue,

dot11RRMAntennaInformationEnabled 								TruthValue,

dot11FastBSSTransitionImplemented 								TruthValue,

dot11LCIDSEImplemented 								TruthValue,

dot11LCIDSERequired 								TruthValue,

dot11DSERequired 								TruthValue,

dot11ExtendedChannelSwitchEnabled 								TruthValue,

dot11RSNAProtectedManagementFramesEnabled 								TruthValue,

dot11RSNAUnprotectedManagementFramesAllowed 								TruthValue,

dot11AssociationPingResponseTimeout 								Unsigned32,

dot11AssociationMaximumPingAttempts 								INTEGER,

dot11HighThroughputOptionImplemented 								TruthValue,

dot11RSNAPBACRequired 								TruthValue},

dot11MeshActivated								TruthValue }

Insert “dot11MeshActivated” to the end of  “dotStationConfigTable”:

dot11MeshActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This attribute shall specify whether or not mesh services are supported by a station.”

::= { dot11StationConfigEntry 94 }

Change the dot11OperationEntity as shown:

dot11OperationEntry ::=

SEQUENCE {dot11MACAddress 							MacAddress,

dot11RTSThreshold 						INTEGER,

dot11ShortRetryLimit 						INTEGER,

dot11LongRetryLimit 						INTEGER,

dot11FragmentationThreshold 						INTEGER,

dot11MaxTransmitMSDULifetime 						Unsigned32,

dot11MaxReceiveLifetime 						Unsigned32,

dot11ManufacturerID 						DisplayString,

dot11ProductID 						DisplayString,

dot11CAPLimit 						INTEGER,

dot11HCCWmin 						INTEGER,

dot11HCCWmax 						INTEGER,

dot11HCCAIFSN 						INTEGER,

dot11ADDBAResponseTimeout 						INTEGER,

dot11ADDTSResponseTimeout 						INTEGER,

dot11ChannelUtilizationBeaconInterval 						INTEGER,

dot11ScheduleTimeout 						INTEGER,

dot11DLSResponseTimeout 						INTEGER,

dot11QAPMissingAckRetryLimit 						INTEGER,

dot11EDCAAveragingPeriod 						INTEGER, }

dot11SAERetransPeriod						INTEGER,

dot11SAEThresh						INTEGER }

Insert the following to the end of dot11OperationEntry Table in Annex D:

dot11SAERetransPeriod OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This object specifies the initial retry timeout, in millisecond units, used by the SAE authentication and key establishment protocol."

DEFVAL {40}

::= { dot11OperationEntry 21}

dot11SAEThresh OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This object specifies the maximum number of SAE protocol instances allowed to simultaneously be in either Commit or Confirmed state."

DEFVAL {5}

::= { dot11OperationEntry 22}

Insert the following dot11 MeshSTATable to the end of Annex D:

**********************************************************************

* dot11MeshSTAConfig TABLE

**********************************************************************

dot11MeshStationConfigTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11MeshStationConfigEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"MeshStation Configuration attributes. In tabular form to allow for multiple instances on an agent."

dot11MeshStationConfigEntry  OBJECT-TYPE

SYNTAX Dot11MeshStationConfigEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11MeshStationConfigTable. It is possible for there to be multiple IEEE 802.11 interfaces on one agent, each with its unique MAC address. The relationship between an IEEE 802.11 interface and an interface in the context of the Internet-standard MIB is one-to-one. As such, the value of an ifIndex object instance can be directly used to identify corresponding instances of the objects defined herein.

ifIndex - Each IEEE 802.11 interface is represented by an ifEntry. Interface tables in this MIB module are indexed by ifIndex."

INDEX { ifIndex }

::= { dot11MeshStationConfigTable 1 }

Dot11MeshStationConfigEntry ::=

SEQUENCE {

dot11MeshID							OCTET STRING,

dot11MeshSecurityActivated							TruthValue,

dot11MeshHWMPActivated							TruthValue,

dot11MeshForwarding							TruthValue,

dot11MeshMaxRetries							INTEGER,

dot11MeshRetryTimeout							INTEGER,

dot11MeshTTL							INTEGER,

dot11MeshPortalAnnouncementProtocol 							TruthValue,

dot11MeshPortalAnnouncementInterval							INTEGER,

dot11MeshConfirmTimeout							INTEGER,

dot11MeshHoldingTimeout							INTEGER,

dot11MeshNbrOffsetActivated							TruthValue,

dot11MeshNbrOffsetMaxNeighbor							INTEGER,

dot11MeshBeaconTimingReportInterval							INTEGER,

dot11MeshBeaconTimingReportMaxNum							INTEGER,

dot11MeshTSFAdjustmentActivated							TruthValue,

dot11MeshTbttSelectionActivated							TruthValue,

dot11MeshDelayedBeaconTxInterval							INTEGER,

dot11MeshDelayedBeaconDelayAmount							INTEGER,

dot11MeshAverageBeaconFrameDuration							INTEGER,

dot11MeshSTAMissingAckRetryLimit							INTEGER,

dot11MCCAImplemented 							TruthValue,

dot11MCCAActivated 							TruthValue,

dot11MAFlimit 							INTEGER,

dot11MCCAScanDuration							INTEGER,

dot11MCCAdvertPeriodMax							INTEGER,

dot11MCCATrackStates OBJECT-TYPE							INTEGER,

dot11MCCAOPtimeout							INTEGER

}

dot11MeshID OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..32))

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute reflects the Mesh ID configured in this entity."

dot11MeshSecurityActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This object shall specify whether or not the station is security enabled."

dot11MeshHWMPActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This object shall specify whether or not the station is HWMP enabled."

dot11MeshForwarding OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the ability of a mesh STA to forward frames. The default value for this attribute is TRUE."

dot11MeshMaxRetries OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This object specifies the maximum number of Peering Open retries that can be sent to establish a new peering instance in a mesh. The default value for this attribute is 2.”

dot11MeshRetryTimeout OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This object specifies the initial retry timeout, in millisecond units, used by the Peering Open message. The default value for this attribute is 40.”

dot11MeshTTL OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the value of TTL field set at a source mesh STA. The default value for this attribute is 31."

dot11MeshPortalAnnouncementProtocol OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute shall specify whether or not the mesh portal is using the Portal Announcement protocol. The default value for this attribute is FALSE.”

dot11MeshPortalAnnouncementInterval OBJECT-TYPE

SYNTAX INTEGER (1..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The Portal Announcement Interval is the number of seconds between the transmission of two Portal Announcements. The default value for this attribute is 10."

dot11MeshConfirmTimeout OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This object specifies the initial retry timeout, in millisecond units, used by the Peering Open message. The default value for this attribute is 40."

dot11MeshHoldingTimeout OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This object specifies the confirm timeout, in millisecond units, used by the peering management to close a peering. The default value for this attribute is 40."

dot11MeshNbrOffsetActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This object shall specify whether or not the station is neighbor offset protocol activated. The default value for this attribute is TRUE. "

dot11MeshNbrOffsetMaxNeighbor OBJECT-TYPE

SYNTAX INTEGER  (1..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This object shall specify the maximum number of neighbor STAs with which the mesh STA maintains the synchronization using the neighbor offset protocol. The default value for this attribute is 16. "

dot11MeshBeaconTimingReportInterval OBJECT-TYPE

SYNTAX INTEGER  (0..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This object shall specify when the Beacon Timing element is present in Beacon frames. When set to 0, Beacon Timing element is present in neither the Beacon frame nor the Probe Response frames. In other cases, Beacon Timing element is present when the DTIM Count value in the Beacon frame is zero or equal to integer multiply of set value. The default value for this attribute is 0. "

dot11MeshBeaconTimingReportMaxNum OBJECT-TYPE

SYNTAX INTEGER  (0..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This object shall specify the maximum number of beacon timing information contained in a Beacon Timing element in the transmitting Beacon frames. The default value for this attribute is 16. "

dot11MeshTSFAdjustmentActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This object shall specify whether or not the station activates TSF adjustment functions. When set to TRUE, the mesh STA adjusts its local TSF timer according to the TSF offset value among neighbor STAs with which the STA maintains synchronization. The default value for this attribute is FALSE. "

dot11MeshTbttSelectionActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This object shall specify whether or not the station activates Mesh TBTT selection functions, which is utilized to avoid beacon collisions. When set to TRUE, the mesh STA selects its TBTT not to collide with neighbor’s and neighbor’s neighbor’s TBTT. The default value for this attribute is FALSE. "

dot11MeshDelayedBeaconTxInterval OBJECT-TYPE

SYNTAX INTEGER  (0..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute shall specify the interval for the delayed beacon transmission from TBTT, for MBCA purpose. The value is expressed in unit of beacon interval, whereas the value 0 indicates that the delayed beacon transmission is disabled exceptionally. The default value for this attribute is 0."

dot11MeshDelayedBeaconDelayAmount OBJECT-TYPE

SYNTAX INTEGER  (0..4023)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute shall specify the delay amount for the delayed beacon transmission from TBTT, for MBCA purpose. The value is expressed in units of microseconds. The default value for this attribute is 0."

dot11MeshAverageBeaconFrameDuration OBJECT-TYPE

SYNTAX INTEGER  (0..4023)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute shall specify the expected averaged duration of last 16 Beacon frames received by the neighboring mesh STAs. The value is expressed in units of microseconds."

dot11MeshSTAMissingAckRetryLimit OBJECT-TYPE

SYNTAX INTEGER (0..100)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute indicates the number of times the mesh STA may retry a frame for which it does not receive an ACK for a STA in power-save mode after the mesh STA does not receive an ACK to a directed MPDU sent with the EOSP set to 1."

dot11MCCAImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This object shall specify whether or not the MCCA is implemented in this station."

dot11MCCAActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This object shall specify whether or not the station is MCCA enabled."

dot11MAFlimit OBJECT-TYPE

SYNTAX INTEGER (0..16)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This object shall specify the maximum MCCA access fraction allowed at the mesh STA. This number is always a multiple of (1/16)."

dot11MCCAScanDuration OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute specifies the duration in TUs that a mesh STA scans on the current operating channel before it may accept or setup MCCAOPs."

dot11MCCAdvertPeriodMax OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This object shall specify the maximum interval that a mesh STA with dot11MCCAEnabled set waits for an MCCAOP Advertisement update.  It is expressed in number of DTIM intervals."

dot11MCCATrackStates OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This object shall specify the minimum number of MCCAOP reservations which an MCCA active mesh STA must track; it equals 83, which is the maximum number of MCCAOP reservations that can be reported in one MCCAOP Advertisement element"

dot11MCCAOPtimeout OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This object specifies the timeout value for an MCCAOP teardown. It is expressed in TU units."

**********************************************************************

* End of dot11MeshSTAConfig TABLE

**********************************************************************

**********************************************************************

* dot11MeshHWMPConfig TABLE

**********************************************************************

dot11MeshHWMPConfigTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11MeshHWMPConfigEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"MeshStation HWMP Configuration attributes. In tabular form to allow for multiple instances on an agent."

dot11MeshHWMPConfigEntry  OBJECT-TYPE

SYNTAX Dot11MeshHWMPConfigEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11MeshHWMPConfigTable. It is possible for there to be multiple IEEE 802.11 interfaces on one agent, each with its unique MAC address. The relationship between an IEEE 802.11 interface and an interface in the context of the Internet-standard MIB is one-to-one. As such, the value of an ifIndex object instance can be directly used to identify corresponding instances of the objects defined herein.

ifIndex - Each IEEE 802.11 interface is represented by an ifEntry. Interface tables in this MIB module are indexed by ifIndex."

INDEX { ifIndex }

::= { dot11MeshStationConfigTable 1 }

Dot11MeshHWMPConfigEntry ::=

SEQUENCE {

dot11MeshHWMPmaxPREQretries							INTEGER,

dot11MeshHWMPnetDiameter							INTEGER,

dot11MeshHWMPnetDiameterTraversalTime							INGETER,

dot11MeshHWMPpreqMinInterval							INTEGER,

dot11MeshHWMPperrMinInterval							INTEGER,

dot11MeshHWMPactiveRootTimeout							INTEGER,

dot11MeshHWMPactivePathTimeout							INTEGER,

dot11MeshHWMProotMode							INTEGER,

dot11MeshHWMPpathToRootTimeout							INTEGER,

dot11MeshHWMProotInterval							INTEGER,

dot11MeshHWMPrannInterval							INTEGER,

dot11MeshHWMPtargetOnly							INTEGER,

dot11MeshHWMPreplyAndForward							INTEGER,

dot11MeshHWMPmaintenanceInterval							INTEGER,

dot11MeshHWMPconfirmationInterval							INTEGER

}

dot11MeshHWMPmaxPREQretries OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This object shall specify the number of action frames containing a PREQ that an originator mesh STA can send to a particular path target for a specific path discovery. The default value of this attribute is 3"

::= { dot11MeshHWMPConfigEntry 1}

dot11MeshHWMPnetDiameter OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This object shall specify the estimate of the maximum number of hops that it takes for an HWMP information element to propagate across the mesh. The default value of this attribute is 31"

::= { dot11MeshHWMPConfigEntry 2}

dot11MeshHWMPnetDiameterTraversalTime OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This object shall specify the estimate of the interval of time (in TUs) that it takes for an HWMP information element to propagate across the mesh. The default value of this attribute is 500"

::= { dot11MeshHWMPConfigEntry 3}

dot11MeshHWMPpreqMinInterval OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This object shall specify the minimum interval of time (in TUs) during which a mesh STA can send only one action frame containing a PREQ element. The default value of this attribute is 100"

::= { dot11MeshHWMPConfigEntry 4}

dot11MeshHWMPperrMinInterval OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This object shall specify the minimum interval of time (in TUs) during which a mesh STA can send only one action frame containing a PERR element. The default value of this attribute is 100"

::= { dot11MeshHWMPConfigEntry 5}

dot11MeshHWMPactiveRootTimeout OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This object shall specify the time (in TUs) for which mesh STAs receiving a PREQ to individual targets(s) shall consider the forwarding information from the root to be valid. The default value of this attribute is 5000"

::= { dot11MeshHWMPConfigEntry 6}

dot11MeshHWMPactivePathTimeout OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This object shall specify the time (in TUs) for which mesh STAs receiving a PREQ to individual target(s) shall consider the forwarding information to be valid. The default value of this attribute is 5000"

::= { dot11MeshHWMPConfigEntry 7}

dot11MeshHWMProotMode OBJECT-TYPE

SYNTAX INTEGER {

noRoot(0),

proactivePREQnoPREP(2),

proactivePREQwithPREP(3),

rann(4) }

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute controls the configuration of a mesh STA as root mesh STA. A mesh STA is configured as a root mesh STA if dot11MeshHWMProotMode is set to 2, 3 or 4. Different values correspond to different modes of the root mesh STA. The mesh STA is not a root mesh STA when the attribute is set to 0. The default value of this attribute is 0"

::= { dot11MeshHWMPConfigEntry 8}

dot11MeshHWMPpathToRootTimeout OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This object shall specify the time (in TUs) for which mesh STAs receiving a proactive PREQ shall consider the forwarding information to the root mesh STA to be valid; it must be greater than dot11MeshHWMProotInterval. The default value of this attribute is 5000"

::= { dot11MeshHWMPConfigEntry 9}

dot11MeshHWMProotInterval OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This object shall specify the minimum interval of time (in TUs) during which a root mesh STA can send only one action frame containing a proactive PREQ element. The default value of this attribute is 2000"

::= { dot11MeshHWMPConfigEntry 10}

dot11MeshHWMPrannInterval OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This object shall specify the minimum interval of time (in TUs) during which a mesh STA can send only one action frame containing a RANN element. The default value of this attribute is 1000"

::= { dot11MeshHWMPConfigEntry 11}

dot11MeshHWMPtargetOnly OBJECT-TYPE

SYNTAX INTEGER { intermediateMSTA(0), targetOnly(1)}

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when set to 0, shall allow intermediate mesh STAs to respond with a PREP to a PREQ if they have valid forwarding information to the requested target. When set to 1, only the target MSTA is allowed to respond with a PREP to a PREQ. The default value of this attribute is 1"

::= { dot11MeshHWMPConfigEntry 12}

dot11MeshHWMPreplyAndForward OBJECT-TYPE

SYNTAX INTEGER { noPropagation(0), propagation(1)}

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when set to 1, shall require intermediate mesh STAs that respond with a PREP to a PREQ to propagate the PREQ further towards the target MSTA. The default value of this attribute is 1"

::= { dot11MeshHWMPConfigEntry 13}

dot11MeshHWMPmaintenanceInterval OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This object shall specify the minimum interval of time (in TUs) during which a mesh STA can send only one action frame containing a PREQ element for path maintenance. The default value of this attribute is 2000"

::= { dot11MeshHWMPConfigEntry 14}

dot11MeshHWMPconfirmationInterval OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This object shall specify the minimum interval of time (in TUs) during which a mesh STA can send only one action frame containing a PREQ element for root path confirmation. The default value of this attribute is 2000"

::= { dot11MeshHWMPConfigEntry 15}