IEEE P802.11
Wireless LANs

| EAS retrieval |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-07-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Gabor Bajko | Nokia | 313 Fairchild dr. Mountain View, CA, 94043 | +1 858 525 3693 | gabor.bajko@nokia.com |

Abstract

This proposal addresses CID #7018

This proposal is based on draft TGu version D7.0

- Make the following changes to the text in section 11.21.6:

Upon receiving an Emergency Alert element for an EAS Message which has not already been retrieved from the network, a non-AP STA having dot11EASEnabled set to TRUE shall retrieve the Emergency Alert Server URI (see 7.3.4.13) using a Native GAS query according to the procedures in 11.21.2.1. Then the STA shall form the EAS Message URI by concatenating the Emergency Alert Server URI with the hexadecimal numerals of the Alert Identifier Hash converted to UTF-8 and the “.xml” file extension.

Example: If the Emergency Alert Server URI is http://eas.server.org, the Alert Identifier Hash is 0x1234567890abcdef, then the EAS Message URI is http://eas.server.org/1234567890abcdef.xml.

A non-AP STA in the un-associated state having dot11EASEnabled set to TRUE shall retrieve the EAS message using non-native GAS procedures defined in 11.21.2.2 with Advertisement Protocol ID set to the value for EAS (see Table 7-43bw). A non-AP STA in the associated state having dot11EASEnabled set to TRUE shall retrieve the EAS message using either non-native GAS procedures defined in 11.21.2.2 with Advertisement Protocol ID set to the value for EAS (see Table 7-43bw) or HTTP using Internet Protocols (the latter being preferred).

Upon receiving an Emergency Alert element for an EAS Message which has not already been retrieved from the network, a non-AP STA in the un-associated state having dot11EASEnabled set to TRUE shall retrieve either the EAS message itself using non-native GAS procedures defined in 11.21.2.2 with Advertisement Protocol ID set to the value for EAS (see Table 7-43bw), or retrieve the Emer gency Alert Server URI (see 7.3.4.13) using a Native GAS query according to the procedures in 11.21.2.1 and then form the EAS Message URI by concatenating the Emergency Alert Server URI with the hexadecimal numerals of the Alert Identifier Hash converted to UTF-8 and the “.xml” file extension.

Example: If the Emergency Alert Server URI is http://eas.server.org, the Alert Identifier Hash is 0x1234567890abcdef, then the EAS Message URI is http://eas.server.org/1234567890abcdef.xml. If the interface of the non-AP STA is IP configured, the non-AP STA can download the EAS message using HTTP from the URI created.