IEEE P802.11
Wireless LANs

| Editorial Changes to Annex D and Annex Q for correct MIB compilation |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-08-04 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Stephen McCann | Research in Motion (RIM) UK Ltd | 200 Bath Road, Slough, Berkshire, SL1 3XE, UK | +44 1753 667099 | smccann@rim.com |
| Adrian Stephens | Intel Corporation UK Ltd | Pipers Way, Swindon SN3 1RJ, UK | +44 1954 204 609 | adrian.p.stephens@ieee.org |

Abstract

This submission highlights editorial changes which were applied to the MIB (Annex D and Annex Q) to allow correct compilation.

This document is based on IEEE 802.11mb-D1.01

-- Annex D

-- (normative)

-- ASN.1 encoding of the MAC and PHY MIB

-- Editor’s Note: The <year> tag, which occurs throughout this Clause, will be replaced by the year of pub-

-- lication during publication by the IEEE-SA editorial staff.

-- **********************************************************************

-- * IEEE 802.11 MIB

-- **********************************************************************

IEEE802dot11-MIB DEFINITIONS ::= BEGIN

IMPORTS

MODULE-IDENTITY, OBJECT-TYPE,

NOTIFICATION-TYPE,Integer32, Counter32,

iso, Unsigned32							FROM SNMPv2-SMI

DisplayString , MacAddress, RowStatus,

TruthValue							FROM SNMPv2-TC

MODULE-COMPLIANCE, OBJECT-GROUP,

NOTIFICATION-GROUP						FROM SNMPv2-CONF

ifIndex							FROM RFC1213-MIB;

TimeTicks							FROM RFC1155-SMI;

-- **********************************************************************

-- *  MODULE IDENTITY

-- **********************************************************************

ieee802dot11 MODULE-IDENTITY

LAST-UPDATED "0610200000Z"

ORGANIZATION "IEEE 802.11"

CONTACT-INFO

"    Chair: Bruce Kraemer

Postal: 517 La Costa Court

Melbourne, FL 32940, USA

Tel: +1 321 427 4098

Fax: +1 321 751 3988

E-mail: bkraemer@ieee.org

Editor: Terry L. Cole

E-mail: terry.cole+ieee@gmail.com"

DESCRIPTION

"The MIB module for IEEE 802.11 entities.

iso(1).member-body(2).us(840).ieee802dot11(10036)"

::= { us 10036 }

-- **********************************************************************

-- *  Tree Definition

-- **********************************************************************

member-body     OBJECT IDENTIFIER ::= { iso 2 }

us              OBJECT IDENTIFIER ::= { member-body 840 }

-- **********************************************************************

-- *  Major sections

-- **********************************************************************

--  Station ManagemenT (SMT) Attributes

--  DEFINED AS "The SMT object class provides the necessary support

--  at the station to manage the processes in the station such that

--  the station may work cooperatively as a part of an IEEE 802.11

--  network."

dot11smt OBJECT IDENTIFIER ::= { ieee802dot11 1 }

--  dot11smt GROUPS

--  dot11StationConfigTable										::= { dot11smt 1 }

--  dot11AuthenticationAlgorithmsTable										::= { dot11smt 2 }

--  dot11WEPDefaultKeysTable										::= { dot11smt 3 }

--  dot11WEPKeyMappingsTable										::= { dot11smt 4 }

--  dot11PrivacyTable										::= { dot11smt 5 }

--  dot11SMTnotification										::= { dot11smt 6 }

--  dot11MultiDomainCapabilityTable										::= { dot11smt 7 }

--  dot11SpectrumManagementTable										::= { dot11smt 8 }

--  dot11RSNAConfigTable							::= { dot11smt 9 }

--  dot11RSNAConfigPairwiseCiphersTable							::= { dot11smt 10 }

--  dot11RSNAConfigAuthenticationSuitesTable							::= { dot11smt 11 }

--  dot11RSNAStatsTable							::= { dot11smt 12 }

--  dot11RegulatoryClassesTable							::= { dot11smt 13 }

--  dot11RadioResourceMeasurement						 	::= { dot11smt 14 }

--  dot11FastBSSTransitionConfigTable							::= { dot11smt 15 }

--  dot11LCIDSETable       							::= { dot11smt 16 }

--  MAC Attributes

--  DEFINED AS "The MAC object class provides the necessary support

--  for the access control, generation, and verification of frame

--  check sequences (FCSs), and proper delivery of valid data to

--  upper layers."

dot11mac OBJECT IDENTIFIER ::= { ieee802dot11 2 }

--  MAC GROUPS

--  reference IEEE Std 802.1f-1993

--  dot11OperationTable							::= { dot11mac 1 }

--  dot11CountersTable							::= { dot11mac 2 }

--  dot11GroupAddressesTable							::= { dot11mac 3 }

--  dot11EDCATable							::= { dot11mac 4 }

--  dot11QAPEDCATable							::= { dot11mac 5 }

--  dot11QosCountersTable							::= { dot11mac 6 }

--  dot11PeerStatsTable 							::= { dot11mac 7 }

--  Resource Type ID

dot11res 			  OBJECT IDENTIFIER 	::= { ieee802dot11 3 }

dot11resAttribute 	OBJECT IDENTIFIER 	::= { dot11res 1 }

--  PHY Attributes

--  DEFINED AS "The PHY object class provides the necessary support

--  for required PHY operational information that may vary from PHY

--  to PHY and from STA to STA to be communicated to upper layers."

dot11phy OBJECT IDENTIFIER ::= { ieee802dot11 4 }

--  PHY GROUPS

--  dot11PhyOperationTable 	 						 ::= { dot11phy 1 }

--  dot11PhyAntennaTable   							 ::= { dot11phy 2 }

--  dot11PhyTxPowerTable    	 						 ::= { dot11phy 3 }

--  dot11PhyFHSSTable        	 						 ::= { dot11phy 4 }

--  dot11PhyDSSSTable         							 ::= { dot11phy 5 }

--  dot11PhyIRTable           							 ::= { dot11phy 6 }

--  dot11RegDomainsSupportedTable							 ::= { dot11phy 7 }

--  dot11AntennasListTable							 ::= { dot11phy 8 }

--  dot11SupportedDataRatesTxTable							 ::= { dot11phy 9 }

--  dot11SupportedDataRatesRxTable							 ::= { dot11phy 10 }

--  dot11PhyOFDMTable							 ::= { dot11phy 11 }

--  dot11PhyHRDSSSTable							 ::= { dot11phy 12 }

--  dot11EHCCHoppingPatternTable							 ::= { dot11phy 13 }

--  dot11PhyERPTable							 ::= { dot11phy 14 }

-- **********************************************************************

-- *  Textual conventions from 802 definitions

-- **********************************************************************

WEPKeytype ::= OCTET STRING (SIZE (5))

TSFType    ::= OCTET STRING (SIZE (8))

-- **********************************************************************

-- *  MIB attribute OBJECT-TYPE definitions follow

-- **********************************************************************

-- **********************************************************************

-- *  dotStationConfig  TABLE

-- **********************************************************************

dot11StationConfigTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11StationConfigEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Station Configuration attributes.  In tabular form to

allow for multiple instances on an agent."

::= { dot11smt 1 }

dot11StationConfigEntry OBJECT-TYPE

SYNTAX Dot11StationConfigEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11StationConfigTable.  It is

possible for there to be multiple IEEE 802.11 interfaces

on one agent, each with its unique MAC address. The

relationship between an IEEE 802.11 interface and an

interface in the context of the Internet-standard MIB is

one-to-one.  As such, the value of an ifIndex object

instance can be directly used to identify corresponding

instances of the objects defined herein.

ifIndex - Each IEEE 802.11 interface is represented by an

ifEntry.  Interface tables in this MIB module are indexed

by ifIndex."

INDEX { ifIndex }

::= { dot11StationConfigTable 1 }

Dot11StationConfigEntry ::=

SEQUENCE {

dot11StationID	MacAddress,

dot11MediumOccupancyLimit	INTEGER,

dot11CFPollable	TruthValue,

dot11CFPPeriod	INTEGER,

dot11CFPMaxDuration	INTEGER,

dot11AuthenticationResponseTimeOut	Unsigned32,

dot11PrivacyOptionImplemented	TruthValue,

dot11PowerManagementMode	INTEGER,

dot11DesiredSSID	OCTET STRING,

dot11DesiredBSSType	INTEGER,

dot11OperationalRateSet	OCTET STRING,

dot11BeaconPeriod	INTEGER,

dot11DTIMPeriod	INTEGER,

dot11AssociationResponseTimeOut	Unsigned32,

dot11DisassociateReason	INTEGER,

dot11DisassociateStation	MacAddress,

dot11DeauthenticateReason	INTEGER,

dot11DeauthenticateStation	MacAddress,

dot11AuthenticateFailStatus	INTEGER,

dot11AuthenticateFailStation	MacAddress,

dot11MultiDomainCapabilityImplemented	TruthValue,

dot11MultiDomainCapabilityEnabled	TruthValue,

dot11CountryString	OCTET STRING,

dot11SpectrumManagementImplemented	TruthValue,

dot11SpectrumManagementRequired	TruthValue,

dot11RSNAOptionImplemented	TruthValue,

dot11RSNAPreauthenticationImplemented	TruthValue,

dot11RegulatoryClassesImplemented	TruthValue,

dot11RegulatoryClassesRequired	TruthValue,

dot11QosOptionImplemented	TruthValue,

dot11ImmediateBlockAckOptionImplemented	TruthValue,

dot11DelayedBlockAckOptionImplemented	TruthValue,

dot11DirectOptionImplemented	TruthValue,

dot11APSDOptionImplemented	TruthValue,

dot11QAckOptionImplemented	TruthValue,

dot11QBSSLoadOptionImplemented	TruthValue,

dot11QueueRequestOptionImplemented	TruthValue,

dot11TXOPRequestOptionImplemented	TruthValue,

dot11MoreDataAckOptionImplemented	TruthValue,

dot11AssociateInNQBSS	TruthValue,

dot11DLSAllowedInQBSS	TruthValue,

dot11DLSAllowed	TruthValue,

dot11AssociateStation	MacAddress,

dot11AssociateID 	INTEGER,

dot11AssociateFailStation 	MacAddress,

dot11AssociateFailStatus 	INTEGER,

dot11ReassociateStation 	MacAddress,

dot11ReassociateID 	INTEGER,

dot11ReassociateFailStation 	MacAddress,

dot11ReassociateFailStatus 	INTEGER,

dot11RadioMeasurementCapable 	TruthValue,

dot11RadioMeasurementEnabled 	TruthValue,

dot11RMMeasurementProbeDelay 	INTEGER,

dot11RMMeasurementPilotPeriod 	INTEGER,

dot11RMLinkMeasurementEnabled 	TruthValue,

dot11RMNeighborReportEnabled 	TruthValue,

dot11RMParallelMeasurementsEnabled 	TruthValue,

dot11RMRepeatedMeasurementsEnabled 	TruthValue,

dot11RMBeaconPassiveMeasurementEnabled 	TruthValue,

dot11RMBeaconActiveMeasurementEnabled 	TruthValue,

dot11RMBeaconTableMeasurementEnabled 	TruthValue,

dot11RMBeaconMeasurementReportingConditionsEnabled 	TruthValue,

dot11RMFrameMeasurementEnabled 	TruthValue,

dot11RMChannelLoadMeasurementEnabled 	TruthValue,

dot11RMNoiseHistogramMeasurementEnabled 	TruthValue,

dot11RMStatisticsMeasurementEnabled 	TruthValue,

dot11RMLCIMeasurementEnabled 	TruthValue,

dot11RMLCIAzimuthEnabled 	TruthValue,

dot11RMTransmitStreamCategoryMeasurementEnabled 	TruthValue,

dot11RMTriggeredTransmitStreamCategoryMeasurementEnabled

TruthValue,

dot11RMAPChannelReportEnabled 	TruthValue,

dot11RMMIBEnabled 	TruthValue,

dot11RMMaxMeasurementDuration 	Unsigned32,

dot11RMNonOperatingChannelMaxMeasurementDuration 	Unsigned32,

dot11RMMeasurementPilotTransmissionInformationEnabled	TruthValue,

dot11RMMeasurementPilotCapability 	Unsigned32,

dot11RMNeighborReportTSFOffsetEnabled 	TruthValue,

dot11RMRCPIMeasurementEnabled 	TruthValue,

dot11RMRSNIMeasurementEnabled 	TruthValue,

dot11RMBSSAverageAccessDelayEnabled 	TruthValue,

dot11RMBSSAvailableAdmissionCapacityEnabled 	TruthValue,

dot11RMAntennaInformationEnabled 	TruthValue,

dot11FastBSSTransitionImplemented 	TruthValue,

dot11LCIDSEImplemented 	TruthValue,

dot11LCIDSERequired  	TruthValue,

dot11DSERequired 	TruthValue,

dot11ExtendedChannelSwitchEnabled 	TruthValue }

dot11StationID OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-write

STATUS deprecated

DESCRIPTION

"The purpose of dot11StationID is to allow a manager to

identify a station for its own purposes.  This attribute

provides for that eventuality while keeping the true MAC

address independent.  Its syntax is MAC address, and the

default value is the station's assigned, unique

MAC address."

::= { dot11StationConfigEntry 1 }

dot11MediumOccupancyLimit OBJECT-TYPE

SYNTAX INTEGER (0..1000)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall indicate the maximum amount of time,

in TU, that a point coordinator (PC) may control the usage

of the wireless medium (WM) without relinquishing control

for long enough to allow at least one instance of DCF access

to the medium.  The default value of this attribute shall

be 100, and the maximum value shall be 1000."

::= { dot11StationConfigEntry 2 }

dot11CFPollable OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"When this attribute is true, it shall indicate that

the STA is able to respond to a CF-Poll with a data frame

within a SIFS time. This attribute shall be false if

the STA is not able to respond to a CF-Poll with a data

frame within a SIFS time."

::= { dot11StationConfigEntry 3 }

dot11CFPPeriod OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The attribute shall describe the number of DTIM intervals

between the start of CFPs.  It is modified by

MLME-START.request primitive."

::= { dot11StationConfigEntry 4 }

dot11CFPMaxDuration OBJECT-TYPE

SYNTAX INTEGER (0..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The attribute shall describe the maximum duration of

the CFP in TU that may be generated by the PCF. It is

modified by MLME-START.request primitive."

::= { dot11StationConfigEntry 5 }

dot11AuthenticationResponseTimeOut OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the number of time units (TUs)

that a responding STA should wait for the next frame in the

authentication sequence."

::= { dot11StationConfigEntry 6 }

dot11PrivacyOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, shall indicate that the IEEE

802.11 WEP option is implemented.  The default value of

this attribute shall be false."

::= { dot11StationConfigEntry 7 }

dot11PowerManagementMode OBJECT-TYPE

SYNTAX INTEGER { active(1), powersave(2) }

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the power management

mode of the STA. When set to active, it shall

indicate that the station is not in power-save

(PS) mode. When set to powersave, it shall indicate

that the station is in power-save mode. The power

management mode is transmitted in all frames

according to the rules in  7.1.3.1.7."

::= { dot11StationConfigEntry 8 }

dot11DesiredSSID OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..32))

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute reflects the Service Set ID (SSID)

used in the DesiredSSID parameter of the most recent

MLME-SCAN.request.  This value may be modified

by an external management entity and used by the

local SME to make decisions about the Scanning

process."

::= { dot11StationConfigEntry 9 }

dot11DesiredBSSType OBJECT-TYPE

SYNTAX INTEGER { infrastructure(1), independent(2), any(3) }

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the type of BSS the

station shall use when scanning for a BSS with

which to synchronize. This value is used to filter

Probe Response frames and Beacon frames. When set to

infrastructure, the station shall synchronize only

with a BSS who’s Capability Information field has

the ESS subfield set to 1. When set to independent,

the station shall synchronize only with a BSS whose

Capability Information field has the IBSS subfield

set to 1. When set to any, the station may

synchronize to either type of BSS."

::= { dot11StationConfigEntry 10 }

dot11OperationalRateSet OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(1..126))

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the set of data

rates at which the station may transmit data.

Each octet contains a value representing a rate.

Each rate shall be within the range from 2 to 127,

corresponding to data rates in increments of

500 kbit/s from 1 Mb/s to 63.5 Mb/s, and shall

be supported (as indicated in the supported rates

table) for receiving data. This value is reported in

transmitted Beacon, Probe Request, Probe Response,

Association Request, Association Response,

Reassociation Request, and Reassociation Response

frames, and is used to determine whether a BSS

with which the station desires to synchronize is

suitable. It is also used when starting a BSS,

as specified in  10.3."

::= { dot11StationConfigEntry 11 }

dot11BeaconPeriod OBJECT-TYPE

SYNTAX INTEGER (1..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the number of TUs that

a station shall use for scheduling Beacon

transmissions. This value is transmitted in Beacon

and Probe Response frames."

::= { dot11StationConfigEntry 12 }

dot11DTIMPeriod OBJECT-TYPE

SYNTAX INTEGER(1..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the number of beacon

intervals that shall elapse between transmission of

Beacon frames containing a TIM element whose DTIM

Count field is 0. This value is transmitted in

the DTIM Period field of Beacon frames."

::= { dot11StationConfigEntry 13 }

dot11AssociationResponseTimeOut OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the number of TU that a

requesting STA should wait for a response to a

transmitted association-request MMPDU."

::= { dot11StationConfigEntry 14 }

dot11DisassociateReason OBJECT-TYPE

SYNTAX INTEGER(0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the most recently

transmitted Reason Code in a Disassociation

frame.  If no Disassociation frame has been

transmitted, the value of this attribute shall

be 0."

REFERENCE "IEEE Std 802.11-<year>, 7.3.1.7"

::= { dot11StationConfigEntry 15 }

dot11DisassociateStation OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the MAC address from the

Address 1 field of the most recently transmitted

Disassociation frame.  If no Disassociation

frame has been transmitted, the value of this

attribute shall be 0."

::= { dot11StationConfigEntry 16 }

dot11DeauthenticateReason OBJECT-TYPE

SYNTAX INTEGER(0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the most recently

transmitted Reason Code in a Deauthentication

frame.  If no Deauthentication frame has been

transmitted, the value of this attribute shall

be 0."

REFERENCE "IEEE Std 802.11-<year>, 7.3.1.7"

::= { dot11StationConfigEntry 17 }

dot11DeauthenticateStation OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the MAC address from the

Address 1 field of the most recently transmitted

Deauthentication frame.  If no Deauthentication

frame has been transmitted, the value of this

attribute shall be 0."

::= { dot11StationConfigEntry 18 }

dot11AuthenticateFailStatus OBJECT-TYPE

SYNTAX INTEGER(0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the most recently

transmitted Status Code in a failed

Authentication frame.  If no failed

Authentication frame has been transmitted, the

value of this attribute shall be 0."

REFERENCE "IEEE Std 802.11-<year>, 7.3.1.9"

::= { dot11StationConfigEntry 19 }

dot11AuthenticateFailStation OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the MAC address from the

Address 1 field of the most recently transmitted

failed Authentication frame.  If no failed

Authentication frame has been transmitted, the

value of this attribute shall be 0."

::= { dot11StationConfigEntry 20 }

dot11MultiDomainCapabilityImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the

station implementation is capable of supporting

multiple regulatory domains.  The capability is

disabled, otherwise.  The default value of this

attribute is false."

::= { dot11StationConfigEntry 21 }

dot11MultiDomainCapabilityEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the

capability of the station to operate in multiple

regulatory domains, is enabled.  The capability is

disabled, otherwise.  The default value of this

attribute is false."

::= { dot11StationConfigEntry 22 }

dot11CountryString OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(3))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute identifies the country or non-country

entity in which the station is operating. If it is a

country, the first two octets of this

string is the two character country code as described

in document ISO/IEC 3166-1.  The third octet shall

be one of the following:

1. an ASCII space character, if the regulations under

which the station is operating encompass all

environments in the country,

2. an ASCII 'O' character, if the regulations under

which the station is operating are for an Outdoor

environment only, or

3. an ASCII 'I' character, if the regulations under

which the station is operating are for an Indoor

environment only.

4. an ASCII 'X' character, if the station is operating

under a non-country entity. The first two octets of the

non-country entity shall be two ASCII 'XX' characters."

::= { dot11StationConfigEntry 23 }

dot11SpectrumManagementImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the station

implementation is capable of supporting spectrum management.

The capability is disabled otherwise. The default value of

this attribute is false."

::= { dot11StationConfigEntry 24 }

dot11SpectrumManagementRequired OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"A STA will use the defined TPC and DFS procedures if this

attribute is true; otherwise it will not use the defined TPC and

DFS procedures. The default value of this attribute is false."

::= { dot11StationConfigEntry 25 }

dot11RSNAOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This variable indicates whether the entity is RSNA-capable."

::= { dot11StationConfigEntry 26 }

dot11RSNAPreauthenticationImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This variable indicates whether the entity supports RSNA

preauthentication. This cannot be true unless

dot11RSNAOptionImplemented is true."

::= { dot11StationConfigEntry 27 }

dot11RegulatoryClassesImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the station implemen

tation is capable of supporting regulatory classes. The capabil

ity is disabled otherwise. The default value of this attribute

is false."

::= { dot11StationConfigEntry 28 }

dot11RegulatoryClassesRequired OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"A STA will use the defined regulatory classes procedures if

this attribute is true; otherwise it will not use the defined

regulatory classes’ procedures. The default value of this

attribute is false."

::= { dot11StationConfigEntry 29 }

dot11QosOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the station

implementation is capable of supporting QoS. The capability

is disabled, otherwise. The default value of this attribute

is false."

::= { dot11StationConfigEntry 30}

dot11ImmediateBlockAckOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the station

implementation is capable of supporting Immediate Block Ack.

The capability is disabled, otherwise. The default value of

this attribute is false."

::= { dot11StationConfigEntry 31}

dot11DelayedBlockAckOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the station

implementation is capable of supporting Delayed Block Ack.

The capability is disabled, otherwise. The default value of

this attribute is false."

::= { dot11StationConfigEntry 32 }

dot11DirectOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the station

implementation is capable of sending and receiving frames

from a non-AP STA in the BSS. The capability is disabled,

otherwise. The default value of this attribute is false."

::= { dot11StationConfigEntry 33 }

dot11APSDOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute is available only at an AP. When true,

this attribute indicates that the AP implementation is

capable of delivering data and polls to stations using

APSD."

::= { dot11StationConfigEntry 34 }

dot11QAckOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the station

implementation is capable of interpreting the CF-Ack bit

in a received frame of type data even if the frame is not

directed to the QoS station. The capability is disabled,

otherwise. A STA is capable of interpreting the CF-Ack bit

in a received data frame if that station is the recipient

of the data frame, regardless of the value of this MIB

variable. The default value of this attribute is false."

::= { dot11StationConfigEntry 35 }

dot11QBSSLoadOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute is available only at an AP. This attribute,

when true, indicates that the AP implementation is capable

of generating and transmitting the BSS load element in the

Beacon and Probe Response frames. The capability is

disabled, otherwise. The default value of this attribute is

false."

::= { dot11StationConfigEntry 36 }

dot11QueueRequestOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute is available only at an AP. This attribute,

when true, indicates that the AP is capable of processing

the Queue Size field in QoS Control field of QoS Data type

frames. The capability is disabled, otherwise. The default

value of this attribute is false."

::= { dot11StationConfigEntry 37 }

dot11TXOPRequestOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute is available only at an AP. This attribute,

when true, indicates that the AP is capable of processing

the TXOP Duration requested field in QoS Control field of

QoS Data type frames. The capability is disabled, otherwise.

The default value of this attribute is false."

::= { dot11StationConfigEntry 38 }

dot11MoreDataAckOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the station

implementation is capable of interpreting the More Data

bit in the ACK frames. The capability is disabled,

otherwise. The default value of this attribute is false."

::= { dot11StationConfigEntry 39 }

dot11AssociateInNQBSS OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the station may

associate in a non-QoS BSS. When false, this attribute

indicates that the station shall not associate in a non-QoS

BSS.

The default value of this attribute is true."

::= { dot11StationConfigEntry 40 }

dot11DLSAllowedInQBSS OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute available at the AP, when true, indicates

that non-AP STAs may set up DLS and communicate with other

non-AP STAs in the BSS. When false, this attribute

indicates that the stations shall not set up DLS nor

communicate with other non-AP STAs in the BSS. The default

value of this attribute is true."

::= { dot11StationConfigEntry 41 }

dot11DLSAllowed OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute available at the non-AP STAs, when true,

indicates that STA may set up DLS or accept DLS Requests,

and communicate with other non-AP STAs in the BSS. When

false, this attribute indicates that the STA shall not set

up DLS nor accept DLS, nor communicate with other non-AP

STAs in the BSS. The default value of this attribute is

true."

::= { dot11StationConfigEntry 42}

dot11AssociateStation OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the MAC address from the Address 1

field of the most recently transmitted association response

frame. If no association response

frame has been transmitted, the value of this attribute shall be

0."

::= { dot11StationConfigEntry 43 }

dot11AssociateID OBJECT-TYPE

SYNTAX INTEGER(0..2007)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the Association ID from the most

recently transmitted association response frame. If no associa-

tion response frame has been transmitted, the value of this

attribute shall be 0."

::= { dot11StationConfigEntry 44 }

dot11AssociateFailStation OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the MAC address from the Address 1

field of the most recently transmitted failed association

response frame. If no failed association response frame has been

transmitted, the value of this attribute shall be 0."

::= { dot11StationConfigEntry 45 }

dot11AssociateFailStatus OBJECT-TYPE

SYNTAX INTEGER(0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the most recently transmitted Status

Code in a failed association response frame. If no failed asso-

ciation response frame has been transmitted, the value of this

attribute shall be 0."

::= { dot11StationConfigEntry 46 }

dot11ReassociateStation OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the MAC address from the Address 1

field of the most recently transmitted reassociation response

frame. If no reassociation response frame has been transmitted,

the value of this attribute shall be 0."

::= { dot11StationConfigEntry 47 }

dot11ReassociateID OBJECT-TYPE

SYNTAX INTEGER(0..2007)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the Association ID from the most

recently transmitted reassociation response frame. If no

reassociation response frame has been transmitted, the value of

this attribute shall be 0."

::= { dot11StationConfigEntry 48 }

dot11ReassociateFailStation OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the MAC address from the Address 1

field of the most recently transmitted failed reassociation

response frame. If no failed reassociation response frame has

been transmitted, the value of this attribute shall be 0."

::= { dot11StationConfigEntry 49 }

dot11ReassociateFailStatus OBJECT-TYPE

SYNTAX INTEGER(0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the most recently transmitted Status

Code in a failed reassociation response frame. If no failed

reassociation response frame has been transmitted, the value of

this attribute shall be 0."

::= { dot11StationConfigEntry 50 }

dot11RadioMeasurementCapable OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the station implemen-

tation is capable of supporting Radio Measurement. Otherwise it

is not capable of performing Radio Measurement. The default

value of this attribute is false."

::= { dot11StationConfigEntry 51 }

dot11RadioMeasurementEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when true, indicates that one or more of the

Radio Measurement Enabled Capabilities MIB attributes

(dot11RMxxxEnabled), listed in 7.3.2.45, are set to true. A STA

may use the defined Radio Measurement procedures if this

attribute is true. The default value of this attribute is

false."

::= { dot11StationConfigEntry 52 }

dot11RMMeasurementProbeDelay OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The value of ProbeDelay to be used when making a beacon type

measurement with measurement mode active when

dot11RMActiveBeaconMeasurementEnabled is true."

::= { dot11StationConfigEntry 53 }

dot11RMMeasurementPilotPeriod OBJECT-TYPE

SYNTAX INTEGER (1..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the number of TUs that a station

shall use for scheduling Measurement Pilot transmissions. This

value is transmitted in Measurement Pilot frames. The default

period is 25% of dot11BeaconPeriod."

::= { dot11StationConfigEntry 54 }

dot11RMLinkMeasurementEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when true, indicates that

dot11RadioMeasurementEnabled is true and that the station capa-

bility for Link Measurement is enabled. false indicates the sta-

tion has no Link Measurement capability or that the capability

is present but is disabled. The default value of this attribute

is false."

::= { dot11StationConfigEntry 55 }

dot11RMNeighborReportEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when true, indicates that

dot11RadioMeasurementEnabled is true and that the station capa-

bility for Neighbor Report is enabled. false indicates the sta-

tion has no Neighbor Report capability or that the capability is

present but is disabled. The default value of this attribute is

false."

::= { dot11StationConfigEntry 56 }

dot11RMParallelMeasurementsEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when true, indicates that

dot11RadioMeasurementEnabled is true and that the station capa-

bility for Parallel Measurements is enabled. false indicates the

station has no Parallel Measurements capability or that the

capability is present but is disabled. The default value of this

attribute is false."

::= { dot11StationConfigEntry 57 }

dot11RMRepeatedMeasurementsEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when true, indicates that

dot11RadioMeasurementEnabled is true and that the station capa-

bility for Repeated Measurements is enabled. false indicates the

station has no Repeated Measurements capability or that the capa-

bility is present but is disabled. The default value of this

attribute is false."

::= { dot11StationConfigEntry 58 }

dot11RMBeaconPassiveMeasurementEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when true, indicates that

dot11RadioMeasurementEnabled is true and that the station capa-

bility for Beacon Passive Measurement is enabled. false indi-

cates the station has no Beacon Passive Measurement capability

or that the capability is present but is disabled. The default

value of this attribute is false."

::= { dot11StationConfigEntry 59 }

dot11RMBeaconActiveMeasurementEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when true, indicates that

dot11RadioMeasurementEnabled is true and that the station capa-

bility for Beacon Active Measurement is enabled. false indicates

the station has no Beacon Active Measurement capability or that

the capability is present but is disabled. The default value of

this attribute is false."

::= { dot11StationConfigEntry 60 }

dot11RMBeaconTableMeasurementEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when true, indicates that

dot11RadioMeasurementEnabled is true and that the station capa-

bility for Beacon Table Measurement is enabled. false indicates

the station has no Beacon Table Measurement capability or that

the capability is present but is disabled. The default value of

this attribute is false."

::= { dot11StationConfigEntry 61 }

dot11RMBeaconMeasurementReportingConditionsEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when true, indicates that

dot11RadioMeasurementEnabled is true and that the station capa-

bility for Beacon Measurement Reporting Conditions is enabled.

false indicates the station has no Beacon Measurement Reporting

Conditions capability or that the capability is present but is

disabled. The default value of this attribute is false."

::= { dot11StationConfigEntry 62 }

dot11RMFrameMeasurementEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when true, indicates that

dot11RadioMeasurementEnabled is true and that the station capa-

bility for Frame Measurement is enabled. false indicates the

station has no Frame Measurement capability or that the capabil-

ity is present but is disabled. The default value of this

attribute is false."

::= { dot11StationConfigEntry 63 }

dot11RMChannelLoadMeasurementEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when true, indicates that

dot11RadioMeasurementEnabled is true and that the station capa-

bility for Channel Load Measurement is enabled. false indicates

the station has no Channel Load Measurement capability or that

the capability is present but is disabled. The default value of

this attribute is false."

::= { dot11StationConfigEntry 64 }

dot11RMNoiseHistogramMeasurementEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when true, indicates that

dot11RadioMeasurementEnabled is true and that the station capa-

bility for Noise Histogram Measurement is enabled. false indi-

cates the station has no Noise Histogram Measurement capability

or that the capability is present but is disabled. The default

value of this attribute is false."

::= { dot11StationConfigEntry 65 }

dot11RMStatisticsMeasurementEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when true, indicates that

dot11RadioMeasurementEnabled is true and that the station capa-

bility for Statistics Measurement is enabled. false indicates

the station has no Statistics Measurement capability or that the

capability is present but is disabled. The default value of this

attribute is false."

::= { dot11StationConfigEntry 66 }

dot11RMLCIMeasurementEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when true, indicates that

dot11RadioMeasurementEnabled is true and that the station capa-

bility for LCI Measurement is enabled. false indicates the sta-

tion has no LCI Measurement capability or that the capability is

present but is disabled. The default value of this attribute is

false."

::= { dot11StationConfigEntry 67 }

dot11RMLCIAzimuthEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when true, indicates that

dot11RadioMeasurementEnabled is true and that the station capa-

bility for LCI Azimuth Measurement is enabled. false indicates

the station has no LCI Azimuth Measurement capability or that

the capability is present but is disabled. The default value of

this attribute is false."

::= { dot11StationConfigEntry 68 }

dot11RMTransmitStreamCategoryMeasurementEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when true, indicates that

dot11RadioMeasurementEnabled is true and that the station capa-

bility for Transmit Stream/Category Measurement is enabled.

false indicates the station has no Transmit Stream/Category Mea-

surement capability or that the capability is present but is

disabled. The default value of this attribute is false."

::= { dot11StationConfigEntry 69 }

dot11RMTriggeredTransmitStreamCategoryMeasurementEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when true, indicates that

dot11RadioMeasurementEnabled is true and that the station capa-

bility for Triggered Transmit Stream/Category Measurement is

enabled. false indicates the station has no Triggered Transmit

Stream/Category Measurement capability or that the capability is

present but is disabled. The default value of this attribute is

false."

::= { dot11StationConfigEntry 70 }

dot11RMAPChannelReportEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when true, indicates that

dot11RadioMeasurementEnabled is true and that the station capa-

bility for AP CHannel Report is enabled. false indicates the

station has no AP CHannel Report capability or that the capabil-

ity is present but is disabled. The default value of this

attribute is false."

::= { dot11StationConfigEntry 71 }

dot11RMMIBEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when true, indicates that

dot11RadioMeasurementEnabled is true and that the station capa-

bility for RM MIB is enabled. false indicates the station has no

RM MIB capability or that the capability is present but is dis-

abled. The default value of this attribute is false. See RM MIB

details in Annex Q."

::= { dot11StationConfigEntry 72 }

dot11RMMaxMeasurementDuration OBJECT-TYPE

SYNTAX Unsigned32(0 .. 7)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute indicates the maximum measurement duration for

operating channel measurements, where

Max Measurement Duration in TUs = 2<superscript>

(dot11RMMaxMeasurementDuration - 4)</superscript> * BeaconInter-

val

Further details are provided in 11.10.3"

::= { dot11StationConfigEntry 73 }

dot11RMNonOperatingChannelMaxMeasurementDuration OBJECT-TYPE

SYNTAX Unsigned32(0 .. 7)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute indicates the maximum measurement duration for

non-operating channel measurements, where

Non-OpMax Measurement Duration in TUs = 2<superscript>

(dot11RMNonOperatingChannelMaxMeasurementDuration - 4)

</superscript> * BeaconInterval

Further details are provided in 11.10.3"

::= { dot11StationConfigEntry 74 }

dot11RMMeasurementPilotTransmissionInformationEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when true, indicates that

dot11RadioMeasurementEnabled is true and that the station capa-

bility for Measurement Pilot Transmission information is

enabled. false indicates the station has no Measurement Pilot

Transmission information capability or that the capability is

present but is disabled. The default value of this attribute is

false."

::= { dot11StationConfigEntry 75 }

dot11RMMeasurementPilotCapability OBJECT-TYPE

SYNTAX Unsigned32(0 .. 7)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute indicates the station capability for Measurement

Pilot. 0 indicates the station has no Measurement Pilot capabil-

ity or that the capability is present but is disabled. Capabil-

ity values 1-7 are defined in Table 11-11 (Measurement Pilot

Capability definition). The default value of this attribute is

0."

::= { dot11StationConfigEntry 76 }

dot11RMNeighborReportTSFOffsetEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when true, indicates that

dot11RadioMeasurementEnabled is true and that the station capa-

bility for Neighbor Report TSF Offset is enabled. false indi-

cates the station has no Neighbor Report TSF Offset capability

or that the capability is present but is disabled. The default

value of this attribute is false."

::= { dot11StationConfigEntry 77 }

dot11RMRCPIMeasurementEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when true, indicates that

dot11RadioMeasurementEnabled is true and that the station capa-

bility for RCPI Measurement is enabled. false indicates the sta-

tion has no RCPI Measurement capability or that the capability

is present but is disabled. The default value of this attribute

is false."

::= { dot11StationConfigEntry 78 }

dot11RMRSNIMeasurementEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when true, indicates that

dot11RadioMeasurementEnabled is true and that the station capa-

bility for RSNI Measurement is enabled. false indicates the sta-

tion has no RSNI Measurement capability or that the capability

is present but is disabled. The default value of this attribute

is false."

::= { dot11StationConfigEntry 79 }

dot11RMBSSAverageAccessDelayEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when true, indicates that

dot11RadioMeasurementEnabled is true and that the station capa-

bility for BSS Average Access Delay is enabled. false indicates

the station has no BSS Average Access Delay capability or that

the capability is present but is disabled. The default value of

this attribute is false."

::= { dot11StationConfigEntry 80 }

dot11RMBSSAvailableAdmissionCapacityEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when true, indicates that

dot11RadioMeasurementEnabled is true and that the station capa-

bility for BSS Available Admission Capacity is enabled. false

indicates the station has no BSS Available Admission Capacity

capability or that the capability is present but is disabled.

The default value of this attribute is false."

::= { dot11StationConfigEntry 81 }

dot11RMAntennaInformationEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when true, indicates that

dot11RadioMeasurementEnabled is true and that the station capa-

bility for Antenna information is enabled. false indicates the

station has no Antenna information capability or that the capa-

bility is present but is disabled. The default value of this

attribute is false."

::= { dot11StationConfigEntry 82 }

dot11FastBSSTransitionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This object indicates if the entity is fast BSS transition

capable."

::= { dot11StationConfigEntry 83 }

dot11LCIDSEImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the station implemen-

tation is capable of supporting LCI DSE. The capability is dis-

abled, otherwise."

DEFVAL { false }

::= { dot11StationConfigEntry 84 }

dot11LCIDSERequired OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"A STA will use the Dependent Station Enablement procedures if

this attribute is true."

DEFVAL { false }

::= { dot11StationConfigEntry 85 }

dot11DSERequired OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the station operation

is dependent on enablement from enabling STAs. This attribute,

when false, indicates that the station operation does not depend

on enablement from STAs."

DEFVAL { false }

::= { dot11StationConfigEntry 86 }

dot11ExtendedChannelSwitchEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the station implemen-

tation is capable of supporting Extended Channel Switch

Announcement. The capability is disabled, otherwise."

DEFVAL { false }

::= { dot11StationConfigEntry 87 }

-- **********************************************************************

-- *    End of dot11StationConfig  TABLE

-- **********************************************************************

-- **********************************************************************

-- *    dot11AuthenticationAlgorithms  TABLE

-- **********************************************************************

dot11AuthenticationAlgorithmsTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11AuthenticationAlgorithmsEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This (conceptual) table of attributes shall be a set of

all the authentication algorithms supported by the

stations.  The following are the default values and the

associated algorithm:

Value = 1: Open System

Value = 2: Shared Key

Value = 3: Fast BSS Transition (FT)"

REFERENCE "IEEE Std 802.11-<year>, 7.3.1.1 (Authentication Algorithm Number

field)"

::= { dot11smt 2 }

dot11AuthenticationAlgorithmsEntry OBJECT-TYPE

SYNTAX Dot11AuthenticationAlgorithmsEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An Entry (conceptual row) in the Authentication

Algorithms Table.

ifIndex - Each IEEE 802.11 interface is represented by an

ifEntry.  Interface tables in this MIB module are indexed

by ifIndex."

INDEX { ifIndex,

dot11AuthenticationAlgorithmsIndex }

::= { dot11AuthenticationAlgorithmsTable  1 }

Dot11AuthenticationAlgorithmsEntry ::=

SEQUENCE { dot11AuthenticationAlgorithmsIndex										Integer32,

dot11AuthenticationAlgorithm      										INTEGER,

dot11AuthenticationAlgorithmsEnable										TruthValue }

dot11AuthenticationAlgorithmsIndex OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The auxiliary variable used to identify instances

of the columnar objects in the Authentication Algorithms Table."

::= { dot11AuthenticationAlgorithmsEntry 1 }

dot11AuthenticationAlgorithm OBJECT-TYPE

SYNTAX INTEGER { openSystem(1), sharedKey(2), fastBSSTransition(3) }

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute shall be the authentication algorithm described by

this entry in the table. The following values can be used here

Value = 1: Open System

Value = 2: Shared Key

Value = 3: Fast BSS Transition (FT)"

::= { dot11AuthenticationAlgorithmsEntry 2 }

dot11AuthenticationAlgorithmsEnable  OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when true at a station, shall enable the acceptance

of the authentication algorithm described in the corresponding table

entry in authentication frames received by the station that have odd

authentication sequence numbers.  The default value of this attribute

shall be true when the value of dot11AuthenticationAlgorithm is

Open System and false otherwise."

::= { dot11AuthenticationAlgorithmsEntry 3 }

-- **********************************************************************

-- *    End of dot11AuthenticationAlgorithms  TABLE

-- **********************************************************************

-- **********************************************************************

-- *    dot11WEPDefaultKeys  TABLE

-- **********************************************************************

dot11WEPDefaultKeysTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11WEPDefaultKeysEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Conceptual table for WEP default keys.  This table shall

contain the four WEP default secret key values

corresponding to the four possible KeyID values.  The WEP

default secret keys are logically WRITE-ONLY.  Attempts to

read the entries in this table shall return unsuccessful

status and values of null or zero.  The default value of

each WEP default key shall be null."

REFERENCE "IEEE Std 802.11-<year>, 8.3.2"

::= { dot11smt 3 }

dot11WEPDefaultKeysEntry OBJECT-TYPE

SYNTAX Dot11WEPDefaultKeysEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An Entry (conceptual row) in the WEP Default Keys Table.

ifIndex - Each IEEE 802.11 interface is represented by an

ifEntry.  Interface tables in this MIB module are indexed

by ifIndex."

INDEX { ifIndex,

dot11WEPDefaultKeyIndex }

::= { dot11WEPDefaultKeysTable  1 }

Dot11WEPDefaultKeysEntry ::=

SEQUENCE { dot11WEPDefaultKeyIndex     INTEGER,

dot11WEPDefaultKeyValue     WEPKeytype }

dot11WEPDefaultKeyIndex OBJECT-TYPE

SYNTAX INTEGER (1..4)

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The auxiliary variable used to identify instances

of the columnar objects in the WEP Default Keys Table.

The value of this variable is equal to the WEPDefaultKeyID + 1"

::= { dot11WEPDefaultKeysEntry 1 }

dot11WEPDefaultKeyValue OBJECT-TYPE

SYNTAX WEPKeytype

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"A WEP default secret key value."

::= { dot11WEPDefaultKeysEntry 2 }

-- **********************************************************************

-- *    End of dot11WEPDefaultKeys  TABLE

-- **********************************************************************

-- **********************************************************************

-- *    dot11WEPKeyMappings  TABLE

-- **********************************************************************

dot11WEPKeyMappingsTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11WEPKeyMappingsEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Conceptual table for WEP Key Mappings.  The MIB supports

the ability to share a separate WEP key for each RA/TA

pair.  The Key Mappings Table contains zero or one entry

for each MAC address and contains two fields for each

entry: WEPOn and the corresponding WEP key.  The WEP key

mappings are logically WRITE-ONLY.  Attempts to read the

entries in this table shall return unsuccessful status and

values of null or zero.  The default value for all WEPOn

fields is false."

REFERENCE "IEEE Std 802.11-<year>, 8.3.2"

::= { dot11smt 4 }

dot11WEPKeyMappingsEntry OBJECT-TYPE

SYNTAX Dot11WEPKeyMappingsEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An Entry (conceptual row) in the WEP Key Mappings Table.

ifIndex - Each IEEE 802.11 interface is represented by an

ifEntry.  Interface tables in this MIB module are indexed

by ifIndex."

INDEX { ifIndex,

dot11WEPKeyMappingIndex }

::= { dot11WEPKeyMappingsTable  1 }

Dot11WEPKeyMappingsEntry ::=

SEQUENCE { dot11WEPKeyMappingIndex								Integer32,

dot11WEPKeyMappingAddress								MacAddress,

dot11WEPKeyMappingWEPOn								TruthValue,

dot11WEPKeyMappingValue								WEPKeytype,

dot11WEPKeyMappingStatus								RowStatus }

dot11WEPKeyMappingIndex OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The auxiliary variable used to identify instances

of the columnar objects in the WEP Key Mappings Table."

::= { dot11WEPKeyMappingsEntry 1 }

dot11WEPKeyMappingAddress OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The MAC address of the STA for which the values from this

key mapping entry are to be used."

::= { dot11WEPKeyMappingsEntry 2 }

dot11WEPKeyMappingWEPOn OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"Boolean as to whether WEP is to be used when communicating

with the dot11WEPKeyMappingAddress STA."

::= { dot11WEPKeyMappingsEntry 3 }

dot11WEPKeyMappingValue OBJECT-TYPE

SYNTAX WEPKeytype

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"A WEP secret key value."

::= { dot11WEPKeyMappingsEntry 4 }

dot11WEPKeyMappingStatus OBJECT-TYPE

SYNTAX RowStatus

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The status column used for creating, modifying, and

deleting instances of the columnar objects in the WEP key

mapping Table."

DEFVAL { active }

::= { dot11WEPKeyMappingsEntry 5 }

-- **********************************************************************

-- *    End of dot11WEPKeyMappings  TABLE

-- **********************************************************************

-- **********************************************************************

-- *    dot11Privacy TABLE

-- **********************************************************************

dot11PrivacyTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11PrivacyEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group containing attributes concerned with IEEE 802.11

Privacy.  Created as a table to allow multiple

instantiations on an agent."

::= { dot11smt 5 }

dot11PrivacyEntry OBJECT-TYPE

SYNTAX Dot11PrivacyEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11PrivacyTable Table.

ifIndex - Each IEEE 802.11 interface is represented by an

ifEntry.  Interface tables in this MIB module are indexed

by ifIndex."

INDEX { ifIndex }

::= { dot11PrivacyTable 1 }

Dot11PrivacyEntry ::=

SEQUENCE { dot11PrivacyInvoked										TruthValue,

dot11WEPDefaultKeyID										INTEGER,

dot11WEPKeyMappingLength										Unsigned32,

dot11ExcludeUnencrypted										TruthValue,

dot11WEPICVErrorCount										Counter32,

dot11WEPExcludedCount										Counter32,

dot11RSNAEnabled								TruthValue,

dot11RSNAPreauthenticationEnabled								TruthValue }

dot11PrivacyInvoked OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"When this attribute is true, it shall indicate that some level of security

is invoked for transmitting data frames. For WEP-only clients, the

security mechanism used is WEP.

For RSNA-capable clients, an additional variable dot11RSNAEnabled indicates

whether RSNA is enabled. If dot11RSNAEnabled is false or the MIB

variable does not exist, the security mechanism invoked is WEP; if

dot11RSNAEnabled is true, RSNA security mechanisms invoked are con-

figured in the dot11RSNAConfigTable. The default value of this at-

tribute shall be false."

::= { dot11PrivacyEntry 1 }

dot11WEPDefaultKeyID  OBJECT-TYPE

SYNTAX INTEGER (0..3)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall indicate the use of the first,

second, third, or fourth element of the WEPDefaultKeys

array when set to values of zero, one, two, or three.  The

default value of this attribute shall be 0."

REFERENCE "IEEE Std 802.11-<year>, 8.3.2"

::= { dot11PrivacyEntry 2 }

dot11WEPKeyMappingLength  OBJECT-TYPE

SYNTAX Unsigned32 (10..4294967295)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The maximum number of tuples that dot11WEPKeyMappings can hold."

REFERENCE "IEEE Std 802.11-<year>, 8.3.2"

::= { dot11PrivacyEntry 3 }

dot11ExcludeUnencrypted  OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"When this attribute is true, the STA shall not indicate at the MAC service

interface received MSDUs that have the Protected Frame subfield of

the Frame Control field equal to zero. When this attribute is false,

the STA may accept MSDUs that have the Protected Frame subfield of

the Frame Control field equal to zero. The default value of this at-

tribute shall be false."

::= { dot11PrivacyEntry 4 }

dot11WEPICVErrorCount  OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when a frame is received with the Protected

Frame subfield of the Frame Control field set to one and the value

of the ICV as received in the frame does not match the ICV value that

is calculated for the contents of the received frame. ICV errors for

TKIP are not counted in this variable but in

dot11RSNAStatsTKIPICVErrors."

::= { dot11PrivacyEntry 5 }

dot11WEPExcludedCount  OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when a frame is received with the Protected

Frame subfield of the Frame Control field set to zero and the value

of dot11ExcludeUnencrypted causes that frame to be discarded."

::= { dot11PrivacyEntry 6 }

dot11RSNAEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"When this object is true, this shall indicate that RSNA is enabled

on this entity. The entity will advertise the RSN  element in its

Beacon and Probe Response frames. Configuration variables for RSNA

operation are found in the dot11RSNAConfigTable.

This object requires that dot11PrivacyInvoked also be set to true."

::= { dot11PrivacyEntry 7 }

dot11RSNAPreauthenticationEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"When this object is true, this shall indicate that RSNA preauthen-

tication is enabled on this entity.

This object requires that dot11RSNAEnabled also be set to true."

::= { dot11PrivacyEntry 8 }

-- **********************************************************************

-- *    End of dot11Privacy  TABLE

-- **********************************************************************

-- **********************************************************************

-- *    dot11SMTnotification Objects

-- **********************************************************************

dot11SMTnotification OBJECT IDENTIFIER ::= { dot11smt 6 }

dot11Disassociate NOTIFICATION-TYPE

OBJECTS { ifIndex, dot11DisassociateReason, dot11DisassociateStation }

STATUS current

DESCRIPTION

"The disassociate notification shall be sent when the STA

sends a Disassociation frame. The value of the notification

shall include the MAC address of the MAC to which the Disassociation

frame was sent and the reason for the disassociation.

ifIndex - Each IEEE 802.11 interface is represented by an

ifEntry.  Interface tables in this MIB module are indexed

by ifIndex."

::= { dot11SMTnotification 0 1 }

dot11Deauthenticate NOTIFICATION-TYPE

OBJECTS { ifIndex, dot11DeauthenticateReason,

dot11DeauthenticateStation }

STATUS current

DESCRIPTION

"The deauthenticate notification shall be sent when the STA

sends a Deauthentication frame. The value of the notification

shall include the MAC address of the MAC to which the

Deauthentication frame was sent and the reason for the

deauthentication.

ifIndex - Each IEEE 802.11 interface is represented by an

ifEntry.  Interface tables in this MIB module are indexed

by ifIndex."

::= { dot11SMTnotification 0 2 }

dot11AuthenticateFail NOTIFICATION-TYPE

OBJECTS { ifIndex, dot11AuthenticateFailStatus,

dot11AuthenticateFailStation }

STATUS current

DESCRIPTION

"The authenticate failure notification shall be sent when the STA

sends an Authentication frame with a status code other than

'successful'. The value of the notification

shall include the MAC address of the MAC to which the Authentication

frame was sent and the reason for the authentication failure.

ifIndex - Each IEEE 802.11 interface is represented by an

ifEntry.  Interface tables in this MIB module are indexed

by ifIndex."

::= { dot11SMTnotification 0 3 }

dot11Associate NOTIFICATION-TYPE

OBJECTS { ifIndex, dot11AssociateStation, dot11AssociateID}

STATUS current

DESCRIPTION

"The associate notification shall be sent when the STA sends an

Association Response frame with a status code equal to ‘success-

ful.’ The value of the notification shall include the MAC address

of the MAC to which the Association Response frame was sent and

the Association ID. ifIndex - Each 802.11 interface is repre-

sented by an ifEntry. Interface tables in this MIB module are

indexed by ifIndex."

::= { dot11SMTnotification 0 4 }

dot11AssociateFailed NOTIFICATION-TYPE

OBJECTS { ifIndex, dot11AssociateFailStatus,

dot11AssociateFailStation }

STATUS current

DESCRIPTION

"The associate failed notification shall be sent when the STA

sends an Association Response frame with a status code other than

‘successful.’ The value of the notification shall include the MAC

address of the MAC to which the Association Response frame was

sent and the reason for the association failure. ifIndex - Each

802.11 interface is represented by an ifEntry. Interface tables in

this MIB module are indexed by ifIndex."

::= { dot11SMTnotification 0 5 }

dot11Reassociate NOTIFICATION-TYPE

OBJECTS { ifIndex, dot11ReassociateStation, dot11ReassociateID}

STATUS current

DESCRIPTION

"The reassociate notification shall be sent when the STA sends an

Reassociation Response frame with a status code equal to ‘success-

ful.’ The value of the notification shall include the MAC address

of the MAC to which the Reassociation Response frame was sent and

the Reassociation ID. ifIndex - Each 802.11 interface is repre-

sented by an ifEntry. Interface tables in this MIB module are

indexed by ifIndex."

::= { dot11SMTnotification 0 6 }

dot11ReassociateFailed NOTIFICATION-TYPE

OBJECTS { ifIndex, dot11ReassociateFailStatus,

dot11ReassociateStation }

STATUS current

DESCRIPTION

"The reassociate failed notification shall be sent when the STA

sends an Reassociation Response frame with a status code other

than ‘successful.’ The value of the notification shall include the

MAC address of the MAC to which the Reassociation Response frame

was sent and the reason for the reassociation failure. ifIndex -

Each 802.11 interface is represented by an ifEntry. Interface

tables in this MIB module are indexed by ifIndex."

::= { dot11SMTnotification 0 7 }

-- **********************************************************************

-- *    End of dot11SMTnotification Objects

-- **********************************************************************

-- **********************************************************************

-- * dot11MultiDomainCapability TABLE

-- **********************************************************************

dot11MultiDomainCapabilityTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11MultiDomainCapabilityEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This (conceptual) table of attributes for

cross-domain mobility."

::= { dot11smt 7 }

dot11MultiDomainCapabilityEntry OBJECT-TYPE

SYNTAX Dot11MultiDomainCapabilityEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry (conceptual row) in the Multiple Domain

Capability Table.

IfIndex - Each IEEE 802.11 interface is represented

by an ifEntry. Interface tables in this MIB are

indexed by ifIndex."

INDEX { ifIndex,

dot11MultiDomainCapabilityIndex }

::= { dot11MultiDomainCapabilityTable 1 }

Dot11MultiDomainCapabilityEntry ::=

SEQUENCE { dot11MultiDomainCapabilityIndex									Integer32,

dot11FirstChannelNumber									Integer32,

dot11NumberofChannels									Integer32,

dot11MaximumTransmitPowerLevel									Integer32 }

dot11MultiDomainCapabilityIndex OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The auxiliary variable used to identify instances of

the columnar objects in the Multi Domain Capability Table."

::= { dot11MultiDomainCapabilityEntry 1 }

dot11FirstChannelNumber OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall indicate the value of the lowest

channel number in the subband for the associated domain

country string. The default value of this attribute

shall be zero."

::= { dot11MultiDomainCapabilityEntry 2 }

dot11NumberofChannels OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall indicate the value of the total

number of channels allowed in the subband for the

associated domain country string. The default value of

this attribute shall be zero."

::= { dot11MultiDomainCapabilityEntry 3 }

dot11MaximumTransmitPowerLevel OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall indicate the maximum transmit power,

in dBm, allowed in the subband for the associated domain

country string. The default value of this attribute shall

be zero."

::= { dot11MultiDomainCapabilityEntry 4 }

-- ********************************************************************

-- * End of dot11MultiDomainCapability TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11SpectrumManagement TABLE

-- ********************************************************************

dot11SpectrumManagementTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11SpectrumManagementEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"(Conceptual) table of attributes for spectrum management"

::= {dot11smt 8}

dot11SpectrumManagementEntry OBJECT-TYPE

SYNTAX Dot11SpectrumManagementEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry (conceptual row) in the Spectrum Management Table.

IfIndex - Each 802.11 interface is represented by an ifEntry.

Interface tables in this MIB are indexed by ifIndex."

INDEX {ifIndex, dot11SpectrumManagementIndex}

::= { dot11SpectrumManagementTable 1 }

Dot11SpectrumManagementEntry ::=

SEQUENCE {

dot11SpectrumManagementIndex								Integer32,

dot11MitigationRequirement								Integer32,

dot11ChannelSwitchTime								Integer32,

dot11PowerCapabilityMax								Integer32,

dot11PowerCapabilityMin								Integer32 }

dot11SpectrumManagementIndex OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The auxiliary variable used to identify instances of the columnar

objects in the Spectrum Management Table."

::= { dot11SpectrumManagementEntry 1 }

dot11MitigationRequirement OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall indicate the mitigation requirement in dB

required. The default value of this attribute shall be 3 dB."

::= { dot11SpectrumManagementEntry 2 }

dot11ChannelSwitchTime OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall indicate assumed channel switch time, measured

in TUs. The unit of this attribute is TUs. The default value of this

attribute shall be 2 TUs. The minimum value shall be 1 TU."

::= { dot11SpectrumManagementEntry 3 }

dot11PowerCapabilityMax OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute shall indicate the maximum transmit Power Capability

of this station. The unit of this attribute is dBm. The default value

of this attribute shall be 0 dBm."

::= { dot11SpectrumManagementEntry 4 }

dot11PowerCapabilityMin OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute shall indicate the minimum transmit Power Capability

of this station. The unit of this attribute is dBm. The default value

of this attribute shall be -100 dBm."

::= { dot11SpectrumManagementEntry 5 }

-- ******************************************************************

-- * End of dot11SpectrumManagement TABLE

-- ******************************************************************

-- **********************************************************************

-- * dot11RegulatoryClasses TABLE

-- **********************************************************************

dot11RegulatoryClassesTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11RegulatoryClassesEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"(Conceptual) table of attributes for regulatory classes"

::= {dot11smt 13}

dot11RegulatoryClassesEntry OBJECT-TYPE

SYNTAX Dot11RegulatoryClassesEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry (conceptual row) in the Regulatory Classes Table.

IfIndex - Each 802.11 interface is represented by an ifEntry. Interface

tables in this MIB are indexed by ifIndex."

INDEX {ifIndex, dot11RegulatoryClassesIndex}

::= { dot11RegulatoryClassesTable 1 }

Dot11RegulatoryClassesEntry ::=

SEQUENCE {

dot11RegulatoryClassesIndex 						Integer32,

dot11RegulatoryClass 						INTEGER,

dot11CoverageClass 						INTEGER }

dot11RegulatoryClassesIndex OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The auxiliary variable used to identify instances of the columnar

objects in the Regulatory Classes Table."

::= { dot11RegulatoryClassesEntry 1 }

dot11RegulatoryClass OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall indicate the regulatory class to be used. The

default value of this attribute shall be zero."

::= { dot11RegulatoryClassesEntry 2 }

dot11CoverageClass OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall indicate the coverage class to be used. The

default value of this attribute shall be zero."

::= { dot11RegulatoryClassesEntry 3 }

-- ********************************************************************

-- * End of dot11RegulatoryClasses TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11FastBSSTransitionConfig TABLE

-- ********************************************************************

dot11FastBSSTransitionConfigTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11FastBSSTransitionConfigEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The table containing fast BSS transition configuration objects."

::= { dot11smt 15 }

dot11FastBSSTransitionConfigEntry OBJECT-TYPE

SYNTAX Dot11FastBSSTransitionConfigEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11FastBSSTransitionConfigTable."

INDEX { ifIndex }

::= { dot11FastBSSTransitionConfigTable 1 }

Dot11FastBSSTransitionConfigEntry ::=

SEQUENCE {

dot11FastBSSTransitionEnabled	TruthValue,

dot11FTMobilityDomainID	OCTET STRING,

dot11FTOverDSEnabled	TruthValue,

dot11FTResourceRequestSupported	TruthValue,

dot11FTR0KeyHolderID	OCTET STRING,

dot11FTR0KeyLifetime	Unsigned32,

dot11FTR1KeyHolderID	OCTET STRING,

dot11FTReassociationDeadline	Unsigned32 }

dot11FastBSSTransitionEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"When this object is true, this shall indicate that fast BSS tran-

sition (FT) is enabled on this entity. The entity will advertise

the FT-related elements in its Beacon and Probe Response frames.

This object requires that dot11FastBSSTransitionImplemented also

be set to true."

::= { dot11FastBSSTransitionConfigEntry 1 }

dot11FTMobilityDomainID OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(2))

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the Mobility Domain identifier

(MDID) of this entity.

The MDID is used to indicate a group of APs, within an ESS,

between which a STA can use fast BSS transition services. Fast BSS

transitions are allowed only between APs that have the same MDID

and are within the same ESS. They are not allowed between APs with

different MDIDs or in different ESSs.

Since fast BSS transition services are defined only within the

scope of an ESS, there is no requirement that MDIDs be unique

across ESSs."

::= { dot11FastBSSTransitionConfigEntry 2 }

dot11FTOverDSEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"When this object is true, this shall indicate that fast BSS tran-

sition via the over-the-DS protocol as described in Clause 11A

(Fast BSS transition) is enabled on this AP entity."

::= { dot11FastBSSTransitionConfigEntry 3 }

dot11FTResourceRequestSupported OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"When this object is true, this shall indicate that the fast BSS

transition (FT) resource request procedures of 11A.10 (Remote

request broker (RRB) communication) are supported on this AP

entity."

::= { dot11FastBSSTransitionConfigEntry 4 }

dot11FTR0KeyHolderID OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(1..48))

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the PMK-R0 key holder identifier

(R0KH-ID) of the Authenticator of this AP.

NOTE: Backend protocol may allow longer NAS Client identifiers

(e.g., RADIUS allows up to 253-octet NAS-Identifier), but when

used with fast BSS transition, the maximum length is limited to 48

octets. The same value must be used for the NAS Client identifier

and dot11FTR0KeyHolderID."

::= { dot11FastBSSTransitionConfigEntry 5 }

dot11FTR0KeyLifetime OBJECT-TYPE

SYNTAX Unsigned32 (60..4294967295)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the default lifetime of the PMK-R0,

in seconds, when a Session-Timeout attribute is not provided dur-

ing the EAP authentication. This attribute shall also apply when

the PMK-R0 is derived from a PSK."

DEFVAL { 1209600 }

::= { dot11FastBSSTransitionConfigEntry 6 }

dot11FTR1KeyHolderID OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(6))

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the PMK-R1 key holder identifier

(R1KH-ID) of the Authenticator of this AP. It shall be set to a

MAC address of the entity holding the PMK-R1 in the Authentica-

tor."

::= { dot11FastBSSTransitionConfigEntry 7 }

dot11FTReassociationDeadline OBJECT-TYPE

SYNTAX Unsigned32 (1000..4294967295)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the number of time units (TUs) that

this target AP entity shall retain a PTKSA and reserve any speci-

fied resources for a STA while waiting for a reassociation from

that STA. It is assumed that this value will be administered con-

sistently across the mobility domain"

DEFVAL { 1000 }

::= { dot11FastBSSTransitionConfigEntry 8 }

-- *********************************************************************

-- * End of dot11FastBSSTransitionConfig TABLE

-- *********************************************************************

-- ********************************************************************

-- * dot11LCIDSE TABLE

-- ********************************************************************

dot11LCIDSETable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11LCIDSEEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains conceptual table of attributes for Dependent Sta-

tion Enablement."

::= { dot11smt 16 }

dot11LCIDSEEntry OBJECT-TYPE

SYNTAX Dot11LCIDSEEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11LCIDSETable Indexed by dot11LCIDSEIndex."

INDEX { dot11LCIDSEIndex }

::= { dot11LCIDSETable 1 }

Dot11LCIDSEEntry ::=

SEQUENCE {

dot11LCIDSEIndex                         Unsigned32,

dot11LCIDSEIfIndex                       INTEGER,

dot11LCIDSECurrentRegulatoryClass        INTEGER,

dot11LCIDSELatitudeResolution            Unsigned32,

dot11LCIDSELatitudeInteger               Integer32,

dot11LCIDSELatitudeFraction              Integer32,

dot11LCIDSELongitudeResolution           Unsigned32,

dot11LCIDSELongitudeInteger              Integer32,

dot11LCIDSELongitudeFraction             Integer32,

dot11LCIDSEAltitudeType                  INTEGER,

dot11LCIDSEAltitudeResolution            Unsigned32,

dot11LCIDSEAltitudeInteger               Integer32,

dot11LCIDSEAltitudeFraction              Integer32,

dot11LCIDSEDatum                         INTEGER,

dot11RegLocAgreement                     TruthValue,

dot11RegLocDSE                           TruthValue,

dot11DependentSTA                        TruthValue,

dot11DependentEnablementIdentifier       Integer32,

dot11DSEEnablementTimeLimit              Unsigned32,

dot11DSEEnablementFailHoldTime           Unsigned32,

dot11DSERenewalTime                      Unsigned32,

dot11DSETransmitDivisor                  Unsigned32 }

dot11LCIDSEIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for LCI DSE elements in dot11LCIDSETable, greater than 0."

::= { dot11LCIDSEEntry 1 }

dot11LCIDSEIfIndex OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Each IEEE 802.11 interface is represented by an ifEntry. Inter-

face Tables in this MIB are indexed by ifIndex."

::= { dot11LCIDSEEntry 2 }

dot11LCIDSECurrentRegulatoryClass OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Current Regulatory Class is 8 bits indicating the particular Reg-

ulatory Class in use by the radio."

::= { dot11LCIDSEEntry 3 }

dot11LCIDSELatitudeResolution OBJECT-TYPE

SYNTAX Unsigned32 (0..63)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Latitude resolution is 6 bits indicating the number of valid bits

in the fixed-point value of Latitude. This field is derived from

IETF RFC-3825, and is accessed big-endian."

::= { dot11LCIDSEEntry 4 }

dot11LCIDSELatitudeInteger OBJECT-TYPE

SYNTAX Integer32 (-359..359)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Latitude is a twos-complement 34-bit fixed point value consist-

ing of 9 bits of integer and 25 bits of fraction. This field con-

tains the 9 bits of integer portion of Latitude. This field is

derived from IETF RFC-3825, and is accessed big-endian."

::= { dot11LCIDSEEntry 5 }

dot11LCIDSELatitudeFraction OBJECT-TYPE

SYNTAX Integer32 (-16777215..16777215)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Latitude is a twos-complement 34-bit fixed point value consist-

ing of 9 bits of integer and 25 bits of fraction. This field con-

tains the 25 bits of fraction portion of Latitude. This field is

derived from IETF RFC-3825, and is accessed big-endian."

::= { dot11LCIDSEEntry 6 }

dot11LCIDSELongitudeResolution OBJECT-TYPE

SYNTAX Unsigned32 (0..63)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Longitude resolution is 6 bits indicating the number of valid

bits in the fixed-point value of Longitude. This field is derived

from IETF RFC-3825, and is accessed big-endian."

::= { dot11LCIDSEEntry 7 }

dot11LCIDSELongitudeInteger OBJECT-TYPE

SYNTAX Integer32 (-359..359)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Longitude is a twos-complement 34-bit fixed point value consist-

ing of 9 bits of integer and 25 bits of fraction. This field con-

tains the 9 bits of integer portion of Longitude. This field is

derived from IETF RFC-3825, and is accessed big-endian."

::= { dot11LCIDSEEntry 8 }

dot11LCIDSELongitudeFraction OBJECT-TYPE

SYNTAX Integer32 (-16777215..16777215)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Longitude is a twos-complement 34-bit fixed point value consist-

ing of 9 bits of integer and 25 bits of fraction. This field con-

tains the 25 bits of fraction portion of Longitude. This field is

derived from IETF RFC-3825, and is accessed big-endian."

::= { dot11LCIDSEEntry 9 }

dot11LCIDSEAltitudeType OBJECT-TYPE

SYNTAX INTEGER {

meters(1),

floors(2),

hagm(3) }

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Altitude Type is 4 bits encoding the type of altitude.

Codes defined are:

meters : in 2s-complement fixed-point 22-bit integer part with 8-

bit fraction

floors : in 2s-complement fixed-point 22-bit integer part with 8-

bit fraction

hagm : Height Above Ground in meters, in 2s-complement fixed-point

22-bit integer part with 8-bit fraction. This field is derived

from IETF RFC-3825, and is accessed big-endian."

DEFVAL { 3 }

::= { dot11LCIDSEEntry 10 }

dot11LCIDSEAltitudeResolution OBJECT-TYPE

SYNTAX Unsigned32 (0..63)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Altitude resolution is 6 bits indicating the number of valid bits

in the altitude. This field is derived from IETF RFC-3825, and is

accessed big-endian."

::= { dot11LCIDSEEntry 11 }

dot11LCIDSEAltitudeInteger OBJECT-TYPE

SYNTAX Integer32 (-2097151..2097151)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Altitude is a 30-bit value defined by the Altitude type field.

The field is encoded as a 2s-complement fixed-point 22-bit inte-

ger Part with 8-bit fraction. This field contains the fixed-point

Part of Altitude. This field is derived from IETF RFC-3825, and is

accessed big-endian."

::= { dot11LCIDSEEntry 12 }

dot11LCIDSEAltitudeFraction OBJECT-TYPE

SYNTAX Integer32 (-127..127)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Altitude is a 30-bit value defined by the Altitude type field.

The field is encoded as a 2s-complement fixed-point 22-bit inte-

ger Part with 8-bit fraction. This field contains the fraction

part of Altitude. This field is derived from IETF RFC-3825, and is

accessed big-endian."

::= { dot11LCIDSEEntry 13 }

dot11LCIDSEDatum OBJECT-TYPE

SYNTAX INTEGER (1..3)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Datum is an 8-bit value encoding the horizontal and vertical ref-

erences used for the coordinates given in this LCI. IETF RFC-3825

defines the values of Datum. Type 1 is WGS-84, the coordinate sys-

tem used by GPS. Type 2 is NAD83 with NAVD88 vertical reference.

Type 3 is NAD83 with Mean Lower Low Water vertical datum. All

other types are reserved. This field is derived from IETF RFC-

3825, and is accessed big-endian."

DEFVAL { 1 }

::= { dot11LCIDSEEntry 14 }

dot11RegLocAgreement OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"RegLocAgreement reports the Enabling STA's Agreement status.

False indicates it is operating away from national borders and

outside national policy zones. True indicates it is operating by

agreement near national boarders or inside national policy zones."

DEFVAL { false }

::= { dot11LCIDSEEntry 15 }

dot11RegLocDSE OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"RegLocDSE reports the Enabling STA's DSE status. False indicates

Dependent STAs are not enabled. True indicates Dependent STA oper-

ation is enabled."

DEFVAL { false }

::= { dot11LCIDSEEntry 16 }

dot11DependentSTA OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute reports the Dependent STA status of the STA that

sent the beacon or Probe Response with this information. False

indicates that STA is not operating as a Dependent STA. True indi-

cates that STA is operating as a Dependent STA."

DEFVAL { true }

::= { dot11LCIDSEEntry 17 }

dot11DependentEnablementIdentifier OBJECT-TYPE

SYNTAX Integer32 (0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute reports the Dependent STA identifier assigned by

the enabling STA to the dependent station."

DEFVAL { 0 }

::= { dot11LCIDSEEntry 18 }

dot11DSEEnablementTimeLimit OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"dot11DSEAssociateTimeLimit indicates the maximum number of sec-

onds that a dependent STA may transmit in a DSE frequency band

while attaining enablement with an enabling STA. Unless another

value is mandated by regulatory authorities, the value shall be 32

seconds."

DEFVAL { 32 }

::= { dot11LCIDSEEntry 19 }

dot11DSEEnablementFailHoldTime OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"dot11DSEAssociateFailHoldTime indicates the number of seconds

that a dependent STA must not transmit in a DSE frequency band

when it fails to attain enablement with an enabling STA within

dot11DSEEnablementTimeLimit seconds. Unless another value is man-

dated by regulatory authorities, the value shall be 512 seconds."

DEFVAL { 512 }

::= { dot11LCIDSEEntry 20}

dot11DSERenewalTime OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"dot11DSERenewalTime indicates the maximum number of seconds that

a dependent STA may operate in a DSE frequency band without

receiving and decoding an enabling signal from its enabling STA.

Unless another value is mandated by regulatory authorities, the

value shall be 60 seconds."

DEFVAL { 60 }

::= { dot11LCIDSEEntry 21}

dot11DSETransmitDivisor OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"dot11DSETransmitDivisor indicates the value used by a dependent

STA when operating in a DSE frequency band and transmitting. The

dependent STA sends an Action frame when the sum of

dot11TransmittedFragmentCount, dot11MulticastTransmittedFrameCount

and dot11ReceivedFragmentCount modulo dot11DSETransmitDivisor

equals zero. Unless another value is mandated by regulatory

authorities, the default value shall be 256."

DEFVAL { 256 }

::= { dot11LCIDSEEntry 22}

-- ********************************************************************

-- * End of dot11LCIDSE TABLE

-- ********************************************************************

-- **********************************************************************

-- * MAC Attribute Templates

-- **********************************************************************

-- **********************************************************************

-- * dot11Operation TABLE

-- **********************************************************************

dot11OperationTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11OperationEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains MAC attributes pertaining to the operation

of the MAC.  This has been implemented as a table in order

to allow for multiple instantiations on an agent."

::= { dot11mac 1 }

dot11OperationEntry OBJECT-TYPE

SYNTAX Dot11OperationEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11OperationEntry Table.

ifIndex - Each IEEE 802.11 interface is represented by an

ifEntry.  Interface tables in this MIB module are indexed

by ifIndex."

INDEX { ifIndex }

::= { dot11OperationTable 1 }

Dot11OperationEntry ::=

SEQUENCE { dot11MACAddress										MacAddress,

dot11RTSThreshold										INTEGER,

dot11ShortRetryLimit										INTEGER,

dot11LongRetryLimit										INTEGER,

dot11FragmentationThreshold										INTEGER,

dot11MaxTransmitMSDULifetime										Unsigned32,

dot11MaxReceiveLifetime										Unsigned32,

dot11ManufacturerID										DisplayString,

dot11ProductID										DisplayString,

dot11CAPLimit										INTEGER,

dot11HCCWmin										INTEGER,

dot11HCCWmax										INTEGER,

dot11HCCAIFSN										INTEGER,

dot11ADDBAResponseTimeout										INTEGER,

dot11ADDTSResponseTimeout										INTEGER,

dot11ChannelUtilizationBeaconInterval										INTEGER,

dot11ScheduleTimeout										INTEGER,

dot11DLSResponseTimeout										INTEGER,

dot11QAPMissingAckRetryLimit										INTEGER,

dot11EDCAAveragingPeriod										INTEGER }

dot11MACAddress OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Unique MAC Address assigned to the STA."

::= { dot11OperationEntry 1 }

dot11RTSThreshold OBJECT-TYPE

SYNTAX INTEGER (0..3000)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall indicate the number of octets in an MPDU,

below which an RTS/CTS handshake shall not be performed, except as

RTS/CTS is used as a cross modulation protection mechanism as

defined in 9.13. An RTS/CTS handshake shall be performed at the

beginning of any frame exchange sequence where the MPDU is of type

Data or Management, the MPDU has an individual address in the

Address1 field, and the length of the MPDU is greater than this

threshold. Setting this attribute to be larger than the maximum

MSDU size shall have the effect of turning off the RTS/CTS

handshake for frames of Data or Management type transmitted by

this STA. Setting this attribute to zero shall have the effect

of turning on the RTS/CTS handshake for all frames of Data or

Management type transmitted by this STA. The default value of

this attribute shall be 3000."

::= { dot11OperationEntry 2 }

dot11ShortRetryLimit OBJECT-TYPE

SYNTAX INTEGER (1..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall indicate the maximum number of

transmission attempts of a frame, the length of which is less

than or equal to dot11RTSThreshold, that shall be made before a

failure condition is indicated. The default value of this

attribute shall be 7."

::= { dot11OperationEntry 3 }

dot11LongRetryLimit OBJECT-TYPE

SYNTAX INTEGER (1..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall indicate the maximum number of

transmission attempts of a frame, the length of which is

greater than dot11RTSThreshold, that shall be made before a

failure condition is indicated. The default value of this

attribute shall be 4."

::= { dot11OperationEntry 4 }

dot11FragmentationThreshold OBJECT-TYPE

SYNTAX INTEGER (256..3000)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the current maximum size, in

octets, of the MPDU that may be delivered to the PHY. An MSDU

shall be broken into fragments if its size exceeds the value

of this attribute after adding MAC headers and trailers. An

MSDU or MMPDU shall be fragmented when the resulting frame has

an individual address in the Address1 field, and the length

of the frame is larger than this threshold. The default value

for this attribute shall be the lesser of 3000 or the

aMPDUMaxLength of the attached PHY and shall never exceed the

lesser of 3000 or the aMPDUMaxLength of the attached PHY.

The value of this attribute shall never be less than 256."

::= { dot11OperationEntry 5 }

dot11MaxTransmitMSDULifetime OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The MaxTransmitMSDULifetime shall be the elapsed time in TU,

after the initial transmission of an MSDU, after which further

attempts to transmit the MSDU shall be terminated. The default

value of this attribute shall be 512."

::= { dot11OperationEntry 6 }

dot11MaxReceiveLifetime OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The MaxReceiveLifetime shall be the elapsed time in TU,

after the initial reception of a fragmented MMPDU or MSDU,

after which further attempts to reassemble the MMPDU or

MSDU shall be terminated. The default value shall be

512."

::= { dot11OperationEntry 7 }

dot11ManufacturerID OBJECT-TYPE

SYNTAX DisplayString (SIZE(0..128))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The ManufacturerID shall include, at a minimum, the name

of the manufacturer.  It may include additional

information at the manufacturer's discretion.  The default

value of this attribute shall be null."

::= { dot11OperationEntry 8 }

dot11ProductID OBJECT-TYPE

SYNTAX DisplayString (SIZE(0..128))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The ProductID shall include, at a minimum, an identifier

that is unique to the manufacturer.  It may include

additional information at the manufacturer's discretion.

The default value of this attribute shall be null."

::= { dot11OperationEntry 9 }

dot11CAPLimit OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the maximum number of TUs a

Controlled access phase(CAP) can last."

::= { dot11OperationEntry 10 }

dot11HCCWmin OBJECT-TYPE

SYNTAX INTEGER -- (0..aCWmin)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the value of the minimum size of the

window that shall be used by the HC for generating a random number

for the backoff. The value of this attribute shall be such that it

could always be expressed in the form of 2^X - 1, where X is an

integer. The default value of this attribute shall be 0."

::= { dot11OperationEntry 11 }

dot11HCCWmax OBJECT-TYPE

SYNTAX INTEGER -- (0..aCWmax)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the value of the maximum size of the

window that shall be used by the HC for generating a random number

for the backoff. The value of this attribute shall be such that it

could always be expressed in the form of 2^X - 1, where X is an

integer. The default value of this attribute shall be 0."

::= { dot11OperationEntry 12 }

dot11HCCAIFSN OBJECT-TYPE

SYNTAX INTEGER (1..15)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the number of slots, after a SIFS

duration, that the HC shall sense the medium idle either before

transmitting or executing a backoff. The default value of this

attribute shall be 1."

::= { dot11OperationEntry 13 }

dot11ADDBAResponseTimeout OBJECT-TYPE

SYNTAX INTEGER (1..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the maximum number of seconds a BA is

to be responded. The default value of this attribute shall be 1."

::= { dot11OperationEntry 14 }

dot11ADDTSResponseTimeout OBJECT-TYPE

SYNTAX INTEGER (1..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the maximum number of seconds an

ADDTS request is to be responded. The default value of this

attribute shall be 1."

::= { dot11OperationEntry 15 }

dot11ChannelUtilizationBeaconInterval OBJECT-TYPE

SYNTAX INTEGER (1..100)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall indicate the number of beacon intervals over

which the channel busy time should be averaged. The default value

for this parameter shall be 50."

::= { dot11OperationEntry 16 }

dot11ScheduleTimeout OBJECT-TYPE

SYNTAX INTEGER (1..100)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall indicate the duration in TUs after which a

STA could go into power-save mode. The default value for this

parameter shall be 10."

::= { dot11OperationEntry 17 }

dot11DLSResponseTimeout OBJECT-TYPE

SYNTAX INTEGER (1..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the maximum number of seconds a

direct link request is to be responded. The default value of this

attribute shall be 10."

::= { dot11OperationEntry 18 }

dot11QAPMissingAckRetryLimit OBJECT-TYPE

SYNTAX INTEGER (1..100)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute indicates the number of times the AP may retry a

frame for which it does not receive an ACK for a STA in power-save

mode after receiving a PS-Poll and sending a directed response or

after the AP does not receive an ACK to a directed MPDU sent with

the EOSP set to 1."

::= { dot11OperationEntry 19 }

dot11EDCAAveragingPeriod OBJECT-TYPE

SYNTAX INTEGER (1..100)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall indicate the number of seconds over which

the admitted_time and used_time are computed. The default value

for this parameter shall be 5."

::= { dot11OperationEntry 20 }

-- **********************************************************************

-- *    End of dot11Operation TABLE

-- **********************************************************************

-- **********************************************************************

-- *    dot11Counters TABLE

-- **********************************************************************

dot11CountersTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11CountersEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group containing attributes that are MAC counters.

Implemented as a table to allow for multiple

instantiations on an agent."

::= { dot11mac 2 }

dot11CountersEntry OBJECT-TYPE

SYNTAX Dot11CountersEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11CountersEntry Table.

ifIndex - Each IEEE 802.11 interface is represented by an

ifEntry.  Interface tables in this MIB module are indexed

by ifIndex."

INDEX { ifIndex }

::= { dot11CountersTable 1 }

Dot11CountersEntry ::=

SEQUENCE { dot11TransmittedFragmentCount	Counter32,

dot11MulticastTransmittedFrameCount	Counter32,

dot11FailedCount	Counter32,

dot11RetryCount	Counter32,

dot11MultipleRetryCount	Counter32,

dot11FrameDuplicateCount	Counter32,

dot11RTSSuccessCount	Counter32,

dot11RTSFailureCount	Counter32,

dot11ACKFailureCount	Counter32,

dot11ReceivedFragmentCount	Counter32,

dot11MulticastReceivedFrameCount	Counter32,

dot11FCSErrorCount	Counter32,

dot11TransmittedFrameCount	Counter32,

dot11WEPUndecryptableCount	Counter32,

dot11QosDiscardedFragmentCount	Counter32,

dot11AssociatedStationCount	Counter32,

dot11QosCFPollsReceivedCount	Counter32,

dot11QosCFPollsUnusedCount	Counter32,

dot11QosCFPollsUnusableCount	Counter32,

dot11QoSCFPollsLostCount	Counter32}

dot11TransmittedFragmentCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall be incremented for an acknowledged MPDU

with an individual address in the address 1 field or an MPDU

with a multicast address in the address 1 field of type Data

or Management."

::= { dot11CountersEntry 1 }

dot11MulticastTransmittedFrameCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment only when the multicast bit

is set in the destination MAC address of a successfully

transmitted MSDU.  When operating as a STA in an ESS, where

these frames are directed to the AP, this implies having

received an acknowledgment to all associated MPDUs."

::= { dot11CountersEntry 2 }

dot11FailedCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when an MSDU is not transmitted

successfully due to the number of transmit attempts exceeding

either the  dot11ShortRetryLimit or dot11LongRetryLimit."

::= { dot11CountersEntry 3 }

dot11RetryCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when an MSDU is successfully

transmitted after one or more retransmissions."

::= { dot11CountersEntry 4 }

dot11MultipleRetryCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when an MSDU is successfully

transmitted after more than one retransmission."

::= { dot11CountersEntry 5 }

dot11FrameDuplicateCount  OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when a frame is received

that the Sequence Control field indicates is a

duplicate."

::= { dot11CountersEntry 6 }

dot11RTSSuccessCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when a CTS is received in

response to an RTS."

::= {  dot11CountersEntry 7 }

dot11RTSFailureCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when a CTS is not received in

response to an RTS."

::= { dot11CountersEntry 8 }

dot11ACKFailureCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when an ACK is not received

when expected."

::= {  dot11CountersEntry 9 }

dot11ReceivedFragmentCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall be incremented for each successfully

received MPDU of type Data or Management."

::= { dot11CountersEntry 10 }

dot11MulticastReceivedFrameCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when a MSDU is received

with the multicast bit set in the destination

MAC address."

::= { dot11CountersEntry 11 }

dot11FCSErrorCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when an FCS error is

detected in a received MPDU."

::= { dot11CountersEntry 12 }

dot11TransmittedFrameCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment for each successfully transmitted

MSDU."

::= { dot11CountersEntry 13 }

dot11WEPUndecryptableCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when a frame is received with the

Protected Frame subfield of the Frame Control field set to one and

the WEPOn value for the key mapped to the transmitter's MAC address

indicates that the frame should not have been encrypted or that frame

is discarded due to the receiving STA not implementing the privacy

option."

::= { dot11CountersEntry 14 }

dot11QosDiscardedFragmentCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment for each QoS Data MPDU that has been

discarded."

::= { dot11CountersEntry 15 }

dot11AssociatedStationCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter, only available at AP, shall increment when a

station associates or reassociates. This counter shall decrement

when a station disassociates."

::= { dot11CountersEntry 16 }

dot11QosCFPollsReceivedCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment for each QoS (+)CF-Poll that has been

received."

::= { dot11CountersEntry 17 }

dot11QosCFPollsUnusedCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment for each QoS (+)CF-Poll that has been

received but not used."

::= { dot11CountersEntry 18 }

dot11QosCFPollsUnusableCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment for each QoS (+)CF-Poll that has been

received but could not be used due to the TXOP size being smaller

than the time that is required for one frame exchange sequence."

::= { dot11CountersEntry 19 }

dot11QoSCFPollsLostCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment for each QoS (+)CF-Poll that has

been issued where there was no response from the QoS STA."

::= { dot11CountersEntry 20 }

-- **********************************************************************

-- * End of dot11Counters TABLE

-- **********************************************************************

-- **********************************************************************

-- * dot11GroupAddresses  TABLE

-- **********************************************************************

dot11GroupAddressesTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11GroupAddressesEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"A conceptual table containing a set of MAC addresses

identifying the multicast-group addresses for which this STA

will receive frames.  The default value of this attribute

shall be null."

::= { dot11mac 3 }

dot11GroupAddressesEntry OBJECT-TYPE

SYNTAX Dot11GroupAddressesEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An Entry (conceptual row) in the Group Addresses Table.

ifIndex - Each IEEE 802.11 interface is represented by an

ifEntry.  Interface tables in this MIB module are indexed

by ifIndex."

INDEX { ifIndex,

dot11GroupAddressesIndex }

::= { dot11GroupAddressesTable  1 }

Dot11GroupAddressesEntry ::=

SEQUENCE { dot11GroupAddressesIndex    Integer32,

dot11Address                MacAddress,

dot11GroupAddressesStatus   RowStatus }

dot11GroupAddressesIndex OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The auxiliary variable used to identify instances

of the columnar objects in the Group Addresses Table."

::= { dot11GroupAddressesEntry 1 }

dot11Address OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"MAC address identifying multicast-group addresses

from which this STA will receive frames."

::= { dot11GroupAddressesEntry 2 }

dot11GroupAddressesStatus OBJECT-TYPE

SYNTAX RowStatus

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The status column used for creating, modifying, and

deleting instances of the columnar objects in the Group

Addresses Table."

DEFVAL { active }

::= { dot11GroupAddressesEntry 3 }

-- **********************************************************************

-- *    End of dot11GroupAddresses TABLE

-- **********************************************************************

-- **********************************************************************

-- *    SMT EDCA Config TABLE

-- **********************************************************************

dot11EDCATable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11EDCAEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Conceptual table for EDCA default parameter values at a non-AP

STA. This table shall contain the four entries of the EDCA

parameters corresponding to four possible ACs. Index 1 corresponds

to AC_BK, index 2 to AC_BE, index 3 to AC_VI, and index 4 to AC_VO."

REFERENCE

"IEEE 802.11-<year>, 9.1.3.1"

::= { dot11mac 4 }

dot11EDCAEntry OBJECT-TYPE

SYNTAX Dot11EDCAEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An Entry (conceptual row) in the EDCA Table.

ifIndex - Each IEEE 802.11 interface is represented by an ifEntry.

Interface tables in this MIB module are indexed by ifIndex."

INDEX { ifIndex,

dot11EDCATableIndex }

::= { dot11EDCATable  1 }

Dot11EDCAEntry ::=

SEQUENCE { dot11EDCATableIndex    								INTEGER,

dot11EDCATableCWmin								INTEGER,

dot11EDCATableCWmax								INTEGER,

dot11EDCATableAIFSN								INTEGER,

dot11EDCATableTXOPLimit								INTEGER,

dot11EDCATableMSDULifetime								INTEGER,

dot11EDCATableMandatory								TruthValue }

dot11EDCATableIndex OBJECT-TYPE

SYNTAX INTEGER (1..4)

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The auxiliary variable used to identify instances of the columnar

objects in the EDCA Table. The value of this variable is

1)			1, if the value of the AC is AC_BK.

2)			2, if the value of the AC is AC_BE.

3)			3, if the value of the AC is AC_VI.

4)			4, if the value of the AC is AC_VO."

::= { dot11EDCAEntry 1 }

dot11EDCATableCWmin OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the value of the minimum size of the

window that shall be used by a STA for a particular AC for

generating a random number for the backoff. The value of this

attribute shall be such that it could always be expressed in the

form of 2^X - 1, where X is an integer. The default value for this

attribute is

1)			aCWmin, if dot11EDCATableIndex is 1 or 2.

2)			(aCWmin+1)/2 - 1, if dot11EDCATableIndex is 3.

3)			(aCWmin+1)/4 - 1, if dot11EDCATableIndex is 4."

::= { dot11EDCAEntry 2 }

dot11EDCATableCWmax OBJECT-TYPE

SYNTAX INTEGER (0..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the value of the maximum size of the

window that shall be used by a STA for a particular AC for

generating a random number for the backoff. The value of this

attribute shall be such that it could always be expressed in the

form of 2^X - 1, where X is an integer. The default value for this

attribute is

1)			aCWmax, if dot11EDCATableIndex is 1 or 2.

2)			aCWmin, if dot11EDCATableIndex is 3.

3)			(aCWmin+1)/2 - 1, if dot11EDCATableIndex is 4."

::= { dot11EDCAEntry 3 }

dot11EDCATableAIFSN OBJECT-TYPE

SYNTAX INTEGER (2..15)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the number of slots, after a SIFS

duration, that the STA, for a particular AC, shall sense the

medium idle either before transmitting or executing a backoff. The

default value for this attribute is

1)			7, if dot11EDCATableIndex is 1,

2)			3, if dot11EDCATableIndex is 2

3)			2, otherwise."

::= { dot11EDCAEntry 4 }

dot11EDCATableTXOPLimit OBJECT-TYPE

SYNTAX INTEGER (0..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the maximum number of microseconds of

an EDCA TXOP for a given AC at a non-AP STA. The default value for

this attribute is

1)			0 for all PHYs, if dot11EDCATableIndex is 1 or 2; this

implies that the sender can send one MSDU in an EDCA TXOP,

2)			3008 microseconds for Clause 17 and Clause 19 PHY and 6016

microseconds for Clause 18 PHY, if dot11EDCATableIndex is 3,

3)			1504 microseconds for Clause 17 and Clause 19 PHY and 3264

microseconds for Clause 18 PHY, if dot11EDCATableIndex is 4."

::= { dot11EDCAEntry 5 }

dot11EDCATableMSDULifetime OBJECT-TYPE

SYNTAX INTEGER (0..500)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify (in TUs) the maximum duration an

MSDU, for a given AC, would be retained by the MAC before it is

discarded. The default value for this parameter shall be 500."

::= { dot11EDCAEntry 6 }

dot11EDCATableMandatory OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when true, indicates that admission control is

mandatory for the given AC. When False, this attribute indicates

that the admission control is not mandatory for the given AC. The

default value for this parameter shall be false."

::= { dot11EDCAEntry 7 }

-- **********************************************************************

-- *    End of SMT EDCA Config TABLE

-- **********************************************************************

-- **********************************************************************

-- *    SMT AP EDCA Config TABLE

-- **********************************************************************

dot11QAPEDCATable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11QAPEDCAEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Conceptual table for EDCA default parameter values at the AP.

This table shall contain the four entries of the EDCA parameters

corresponding to four possible ACs. Index 1 corresponds to AC_BK,

index 2 to AC_BE, index 3 to AC_VI, and index 4 to AC_VO."

REFERENCE

"IEEE 802.11-<year>, 9.9.1"

::= { dot11mac 5 }

dot11QAPEDCAEntry OBJECT-TYPE

SYNTAX Dot11QAPEDCAEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An Entry (conceptual row) in the EDCA Table.

ifIndex - Each IEEE 802.11 interface is represented by an

ifEntry. Interface tables in this MIB module are indexed

by ifIndex."

INDEX { ifIndex,

dot11QAPEDCATableIndex }

::= { dot11QAPEDCATable  1 }

Dot11QAPEDCAEntry ::=

SEQUENCE { dot11QAPEDCATableIndex   		 								INTEGER,

dot11QAPEDCATableCWmin										INTEGER,

dot11QAPEDCATableCWmax										INTEGER,

dot11QAPEDCATableAIFSN										INTEGER,

dot11QAPEDCATableTXOPLimit										INTEGER,

dot11QAPEDCATableMSDULifetime										INTEGER,

dot11QAPEDCATableMandatory										TruthValue }

dot11QAPEDCATableIndex OBJECT-TYPE

SYNTAX INTEGER (1..4)

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The auxiliary variable used to identify instances of the columnar

objects in the EDCA Table. The value of this variable is

1)			1, if the value of the AC is AC_BK.

2)			2, if the value of the AC is AC_BE.

3)			3, if the value of the AC is AC_VI.

4)			4, if the value of the AC is AC_VO."

::= { dot11QAPEDCAEntry 1 }

dot11QAPEDCATableCWmin OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the value of the minimum size of the

window that shall be used by an AP for a particular AC for

generating a random number for the backoff. The value of this

attribute shall be such that it could always be expressed in the

form of 2^X - 1, where X is an integer. The default value for this

attribute is

1)			aCWmin, if dot11QAPEDCATableIndex is 1 or 2.

2)			(aCWmin+1)/2 - 1, if dot11QAPEDCATableIndex is 3.

3)			(aCWmin+1)/4 - 1, if dot11QAPEDCATableIndex is 4."

::= { dot11QAPEDCAEntry 2 }

dot11QAPEDCATableCWmax OBJECT-TYPE

SYNTAX INTEGER (0..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the value of the maximum size of the

window that shall be used by an AP for a particular AC for

generating a random number for the backoff. The value of this

attribute shall be such that it could always be expressed in the

form of 2^X - 1, where X is an integer. The default value for this

attribute is

1)			aCWmax, if dot11QAPEDCATableIndex is 1.

2)			4*(aCWmin+1) - 1, if dot11QAPEDCATableIndex is 2.

3)			aCWmin, if dot11QAPEDCATableIndex is 3.

4)			(aCWmin+1)/2 - 1, if dot11QAPEDCATableIndex is 4."

::= { dot11QAPEDCAEntry 3 }

dot11QAPEDCATableAIFSN OBJECT-TYPE

SYNTAX INTEGER (1..15)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the number of slots, after a SIFS

duration, that the AP, for a particular AC, shall sense the

medium idle either before transmitting or executing a backoff. The

default value for this attribute is

1)			7, if dot11QAPEDCATableIndex is 1,

2)			3, if dot11QAPEDCATableIndex is 2

3)			1, otherwise."

::= { dot11QAPEDCAEntry 4 }

dot11QAPEDCATableTXOPLimit OBJECT-TYPE

SYNTAX INTEGER (0..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the maximum number of microseconds of

an EDCA TXOP for a given AC at the AP. The default value for

this attribute is

1)			0 for all PHYs, if dot11QAPEDCATableIndex is 1 or 2; this

implies that the sender can send one MSDU in an EDCA TXOP,

2)			3008 microseconds for Clause 17 and Clause 19 PHY and 6016

microseconds for Clause 18 PHY, if dot11QAPEDCATableIndex is 3,

3)			1504 microseconds for Clause 17 and Clause 19 PHY and 3264

microseconds for Clause 18 PHY, if dot11QAPEDCATableIndex is 4."

::= { dot11QAPEDCAEntry 5 }

dot11QAPEDCATableMSDULifetime OBJECT-TYPE

SYNTAX INTEGER (0..500)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify (in TUs) the maximum duration an

MSDU, for a given AC, would be retained by the MAC at the AP before

it is discarded. The default value for this parameter shall be

500."

::= { dot11QAPEDCAEntry 6 }

dot11QAPEDCATableMandatory OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when true, indicates that admission control is

mandatory for the given AC. When False, this attribute indicates

that the admission control is not mandatory for the given AC. The

default value for this parameter shall be false."

::= { dot11QAPEDCAEntry 7 }

-- **********************************************************************

-- *    End of SMT AP EDCA Config TABLE

-- **********************************************************************

-- **********************************************************************

-- *    dot11QosCounters TABLE

-- **********************************************************************

dot11QosCountersTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11QosCountersEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group containing attributes that are MAC counters implemented as a

table to allow for multiple instantiations on an agent."

::= { dot11mac 6 }

dot11QosCountersEntry OBJECT-TYPE

SYNTAX Dot11QosCountersEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An Entry (conceptual row) in the EDCA Table.

ifIndex - Each IEEE 802.11 interface is represented by an

ifEntry. Interface tables in this MIB module are indexed

by ifIndex."

INDEX { ifIndex,

dot11QosCountersIndex }

::= { dot11QosCountersTable 1 }

Dot11QosCountersEntry ::=

SEQUENCE { dot11QosCountersIndex    											INTEGER,

dot11QosTransmittedFragmentCount											Counter32,

dot11QosFailedCount											Counter32,

dot11QosRetryCount											Counter32,

dot11QosMultipleRetryCount											Counter32,

dot11QosFrameDuplicateCount											Counter32,

dot11QosRTSSuccessCount											Counter32,

dot11QosRTSFailureCount											Counter32,

dot11QosACKFailureCount											Counter32,

dot11QosReceivedFragmentCount											Counter32,

dot11QosTransmittedFrameCount											Counter32,

dot11QosDiscardedFrameCount											Counter32,

dot11QosMPDUsReceivedCount											Counter32,

dot11QosRetriesReceivedCount											Counter32}

dot11QosCountersIndex OBJECT-TYPE

SYNTAX INTEGER (1..16)

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The auxiliary variable used to identify instances of the columnar

objects in the QoSCounter Table. The value of this variable is

equal to TID + 1."

::= { dot11QosCountersEntry 1 }

dot11QosTransmittedFragmentCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall be incremented for an acknowledged MPDU, for a

particular UP, with an individual address in the address 1 field or

an MPDU with a multicast address in the address 1 field, either

belonging to a particular TID. This counter has relevance only for

TIDs between 0 and 7."

::= { dot11QosCountersEntry 2 }

dot11QosFailedCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when an MSDU, for a particular UP, is

not transmitted successfully due to the number of transmit attempts

exceeding either the dot11ShortRetryLimit or dot11LongRetryLimit.

This counter has relevance only for TIDs between 0 and 7."

::= { dot11QosCountersEntry 3 }

dot11QosRetryCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when an MSDU, of a particular UP, is

successfully transmitted after one or more retransmissions. This

counter has relevance only for TIDs between 0 and 7."

::= { dot11QosCountersEntry 4 }

dot11QosMultipleRetryCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when an MSDU, of a particular UP, is

successfully transmitted after more than one retransmissions. This

counter has relevance only for TIDs between 0 and 7."

::= { dot11QosCountersEntry 5 }

dot11QosFrameDuplicateCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when a frame, of a particular UP, is

received that the Sequence Control field indicates is a duplicate.

This counter has relevance only for TIDs between 0 and 7."

::= { dot11QosCountersEntry 6 }

dot11QosRTSSuccessCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when a CTS is received in response to

an RTS that has been sent for the transmission of an MPDU of a

particular UP. This counter has relevance only for TIDs between 0

and 7."

::= { dot11QosCountersEntry 7 }

dot11QosRTSFailureCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when a CTS is not received in

response to an RTS that has been sent for the transmission of an

MPDU of a particular UP. This counter has relevance only for TIDs

between 0 and 7."

::= { dot11QosCountersEntry 8 }

dot11QosACKFailureCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when an ACK is not received in

response to an MPDU of a particular UP. This counter has relevance

only for TIDs between 0 and 7."

::= { dot11QosCountersEntry 9 }

dot11QosReceivedFragmentCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall be incremented for each successfully received

MPDU of type Data of a particular UP. This counter has relevance

only for TIDs between 0 and 7."

::= { dot11QosCountersEntry 10 }

dot11QosTransmittedFrameCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment for each successfully transmitted

MSDU of a particular UP. This counter has relevance only for TIDs

between 0 and 7."

::= { dot11QosCountersEntry 11 }

dot11QosDiscardedFrameCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment for each Discarded MSDU of a

particular UP. This counter has relevance only for TIDs between 0

and 7."

::= { dot11QosCountersEntry 12 }

dot11QosMPDUsReceivedCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment for each received MPDU of a

particular TID."

::= { dot11QosCountersEntry 13 }

dot11QosRetriesReceivedCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment for each received MPDU of a

particular TID with the retry bit set to 1."

::= { dot11QosCountersEntry 14 }

-- **********************************************************************

-- *    End of dot11QosCounters TABLE

-- **********************************************************************

-- **********************************************************************

-- *    Resource Type Attribute Templates

-- **********************************************************************

dot11ResourceTypeIDName OBJECT-TYPE

SYNTAX DisplayString (SIZE(4))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Contains the name of the Resource Type ID managed object.

The attribute is read-only and always contains the value

RTID.  This attribute value shall not be used as a naming

attribute for any other managed object class."

REFERENCE "IEEE Std 802.1F-1993,  A.7"

DEFVAL { "RTID" }

::= { dot11resAttribute 1 }

-- **********************************************************************

-- *    dot11ResourceInfo  TABLE

-- **********************************************************************

dot11ResourceInfoTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11ResourceInfoEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Provides a means of indicating, in data readable from a

managed object, information that identifies the source of

the implementation."

REFERENCE "IEEE Std 802.1F-1993,  A.7"

::= { dot11resAttribute 2 }

dot11ResourceInfoEntry OBJECT-TYPE

SYNTAX Dot11ResourceInfoEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11ResourceInfo Table.

ifIndex - Each IEEE 802.11 interface is represented by an

ifEntry.  Interface tables in this MIB module are indexed

by ifIndex."

INDEX { ifIndex }

::= { dot11ResourceInfoTable 1 }

Dot11ResourceInfoEntry ::=

SEQUENCE { dot11manufacturerOUI             OCTET STRING,

dot11manufacturerName            DisplayString,

dot11manufacturerProductName     DisplayString,

dot11manufacturerProductVersion  DisplayString }

dot11manufacturerOUI OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(3))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Takes the value of an organizationally unique identifier."

::= { dot11ResourceInfoEntry 1 }

dot11manufacturerName OBJECT-TYPE

SYNTAX DisplayString (SIZE(0..128))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"A printable string used to identify the manufacturer of the

resource.  Maximum string length is 128 octets."

::= { dot11ResourceInfoEntry 2 }

dot11manufacturerProductName OBJECT-TYPE

SYNTAX DisplayString (SIZE(0..128))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"A printable string used to identify the manufacturer's product

name of the resource.  Maximum string length is 128 octets."

::= { dot11ResourceInfoEntry 3 }

dot11manufacturerProductVersion OBJECT-TYPE

SYNTAX DisplayString (SIZE(0..128))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Printable string used to identify the manufacturer's product

version of the resource.  Maximum string length is 128 octets."

::= { dot11ResourceInfoEntry 4 }

-- **********************************************************************

-- * End of dot11ResourceInfo  TABLE

-- **********************************************************************

-- **********************************************************************

-- * PHY Attribute Templates

-- **********************************************************************

-- **********************************************************************

-- * dot11PhyOperation  TABLE

-- **********************************************************************

dot11PhyOperationTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11PhyOperationEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"PHY level attributes concerned with

operation.  Implemented as a table indexed on

ifIndex to allow for multiple instantiations on an

Agent."

::= { dot11phy 1 }

dot11PhyOperationEntry OBJECT-TYPE

SYNTAX Dot11PhyOperationEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11PhyOperation Table.

ifIndex - Each IEEE 802.11 interface is represented by an

ifEntry.  Interface tables in this MIB module are indexed

by ifIndex."

INDEX { ifIndex }

::= { dot11PhyOperationTable 1 }

Dot11PhyOperationEntry ::=

SEQUENCE { dot11PHYType            INTEGER,

dot11CurrentRegDomain   Integer32,

dot11TempType           INTEGER }

dot11PHYType OBJECT-TYPE

SYNTAX INTEGER { fhss(1), dsss(2), irbaseband(3), ofdm(4),

hrdsss(5), erp(6)}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is an 8-bit integer value that identifies the PHY type

supported by the attached PLCP and PMD. Currently defined

values and their corresponding PHY types are:

FHSS 2.4 GHz = 01, DSSS 2.4 GHz = 02, IR Baseband = 03,

OFDM 5GHz = 04, HRDSSS = 05, ERP = 06"

::= { dot11PhyOperationEntry 1 }

dot11CurrentRegDomain OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The current regulatory domain this instance of the PMD is

supporting.  This object corresponds to one of the

RegDomains listed in dot11RegDomainsSupported."

::= { dot11PhyOperationEntry 2 }

dot11TempType OBJECT-TYPE

SYNTAX INTEGER { tempType1(1), tempType2(2) }

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"There are different operating temperature requirements

dependent on the anticipated environmental conditions. This

attribute describes the current PHY's operating temperature

range capability. Currently defined values and their

corresponding temperature ranges are:

Type 1 = X'01'-Commercial range of 0 to 40 degrees C,

Type 2 = X'02'-Industrial range of -30 to 70 degrees C."

::= { dot11PhyOperationEntry 3 }

-- **********************************************************************

-- * End of dot11PhyOperation  TABLE

-- **********************************************************************

-- **********************************************************************

-- * dot11PhyAntenna  TABLE

-- **********************************************************************

dot11PhyAntennaTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11PhyAntennaEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group of attributes for PhyAntenna.  Implemented as a

table indexed on ifIndex to allow for multiple instances on

an agent."

::= { dot11phy 2}

dot11PhyAntennaEntry OBJECT-TYPE

SYNTAX Dot11PhyAntennaEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11PhyAntenna Table.

ifIndex - Each IEEE 802.11 interface is represented by an

ifEntry.  Interface tables in this MIB module are indexed

by ifIndex."

INDEX { ifIndex }

::= { dot11PhyAntennaTable 1 }

Dot11PhyAntennaEntry ::=

SEQUENCE { dot11CurrentTxAntenna  Integer32,

dot11DiversitySupport  INTEGER,

dot11CurrentRxAntenna  Integer32 }

dot11CurrentTxAntenna OBJECT-TYPE

SYNTAX Integer32 (1..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The current antenna being used to transmit.  This value

is one of the values appearing in dot11SupportedTxAntenna. This

may be used by a management agent to control which antenna is

used for transmission. "

::= { dot11PhyAntennaEntry 1 }

dot11DiversitySupport OBJECT-TYPE

SYNTAX INTEGER { fixedlist(1), notsupported(2), dynamic(3) }

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This implementation's support for diversity, encoded as:

X'01'-diversity is available and is performed over the fixed

list of antennas defined in dot11DiversitySelectionRx.

X'02'-diversity is not supported.

X'03'-diversity is supported and control of diversity is also

available, in which case the attribute

dot11DiversitySelectionRx can be dynamically modified by

the LME."

::= { dot11PhyAntennaEntry 2 }

dot11CurrentRxAntenna OBJECT-TYPE

SYNTAX Integer32 (1..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The current antenna being used to receive, if the dot11

DiversitySupport indicates that diversity is not supported.

The selected antenna shall be one of the antennae marked

for receive in the dot11AntennasListTable."

::= { dot11PhyAntennaEntry 3 }

-- **********************************************************************

-- * End of dot11PhyAntenna  TABLE

-- **********************************************************************

-- **********************************************************************

-- * dot11PhyTxPower  TABLE

-- **********************************************************************

dot11PhyTxPowerTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11PhyTxPowerEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group of attributes for dot11PhyTxPowerTable.  Implemented

as a table indexed on STA ID to allow for multiple

instances on an Agent."

::= { dot11phy 3}

dot11PhyTxPowerEntry OBJECT-TYPE

SYNTAX Dot11PhyTxPowerEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11PhyTxPower Table.

ifIndex - Each IEEE 802.11 interface is represented by an

ifEntry.  Interface tables in this MIB module are indexed

by ifIndex."

INDEX { ifIndex }

::= { dot11PhyTxPowerTable 1 }

Dot11PhyTxPowerEntry ::=

SEQUENCE { dot11NumberSupportedPowerLevels  INTEGER,

dot11TxPowerLevel1               INTEGER,

dot11TxPowerLevel2               INTEGER,

dot11TxPowerLevel3               INTEGER,

dot11TxPowerLevel4               INTEGER,

dot11TxPowerLevel5               INTEGER,

dot11TxPowerLevel6               INTEGER,

dot11TxPowerLevel7               INTEGER,

dot11TxPowerLevel8               INTEGER,

dot11CurrentTxPowerLevel         INTEGER }

dot11NumberSupportedPowerLevels OBJECT-TYPE

SYNTAX INTEGER (1..8)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The number of power levels supported by the PMD.

This attribute can have a value of 1 to 8."

::= { dot11PhyTxPowerEntry 1 }

dot11TxPowerLevel1 OBJECT-TYPE

SYNTAX INTEGER (0..10000)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The transmit output power for LEVEL1 in mW.

This is also the default power level."

::= { dot11PhyTxPowerEntry 2 }

dot11TxPowerLevel2 OBJECT-TYPE

SYNTAX INTEGER (0..10000)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The transmit output power for LEVEL2 in mW."

::= { dot11PhyTxPowerEntry 3 }

dot11TxPowerLevel3 OBJECT-TYPE

SYNTAX INTEGER (0..10000)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The transmit output power for LEVEL3 in mW."

::= { dot11PhyTxPowerEntry 4 }

dot11TxPowerLevel4 OBJECT-TYPE

SYNTAX INTEGER (0..10000)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The transmit output power for LEVEL4 in mW."

::= { dot11PhyTxPowerEntry 5 }

dot11TxPowerLevel5 OBJECT-TYPE

SYNTAX INTEGER (0..10000)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The transmit output power for LEVEL5 in mW."

::= { dot11PhyTxPowerEntry 6 }

dot11TxPowerLevel6 OBJECT-TYPE

SYNTAX INTEGER (0..10000)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The transmit output power for LEVEL6 in mW."

::= { dot11PhyTxPowerEntry 7 }

dot11TxPowerLevel7 OBJECT-TYPE

SYNTAX INTEGER (0..10000)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The transmit output power for LEVEL7 in mW."

::= { dot11PhyTxPowerEntry 8 }

dot11TxPowerLevel8 OBJECT-TYPE

SYNTAX INTEGER (0..10000)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The transmit output power for LEVEL8 in mW."

::= { dot11PhyTxPowerEntry 9 }

dot11CurrentTxPowerLevel OBJECT-TYPE

SYNTAX INTEGER (1..8)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The TxPowerLevel N currently being used to transmit data.

Some PHYs also use this value to determine the receiver

sensitivity requirements for CCA."

::= { dot11PhyTxPowerEntry 10 }

-- **********************************************************************

-- * End of dot11PhyTxPower  TABLE

-- **********************************************************************

-- **********************************************************************

-- * dot11PhyFHSS  TABLE

-- **********************************************************************

dot11PhyFHSSTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11PhyFHSSEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group of attributes for dot11PhyFHSSTable.  Implemented as a

table indexed on STA ID to allow for multiple instances on

an Agent."

::= { dot11phy 4 }

dot11PhyFHSSEntry OBJECT-TYPE

SYNTAX Dot11PhyFHSSEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11PhyFHSS Table.

ifIndex - Each IEEE 802.11 interface is represented by an

ifEntry.  Interface tables in this MIB module are indexed

by ifIndex."

INDEX { ifIndex }

::= { dot11PhyFHSSTable 1 }

Dot11PhyFHSSEntry ::=

SEQUENCE { dot11HopTime											INTEGER,

dot11CurrentChannelNumber											INTEGER,

dot11MaxDwellTime											INTEGER,

dot11CurrentDwellTime											INTEGER,

dot11CurrentSet											INTEGER,

dot11CurrentPattern											INTEGER,

dot11CurrentIndex											INTEGER,

dot11EHCCPrimeRadix											Integer32,

dot11EHCCNumberofChannelsFamilyIndex											Integer32,

dot11EHCCCapabilityImplemented											TruthValue,

dot11EHCCCapabilityEnabled											TruthValue,

dot11HopAlgorithmAdopted											INTEGER,

dot11RandomTableFlag											TruthValue,

dot11NumberofHoppingSets											Integer32,

dot11HopModulus											Integer32,

dot11HopOffset											Integer32 }

dot11HopTime OBJECT-TYPE

SYNTAX INTEGER (224)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The time in microseconds for the PMD to change from

channel 2 to channel 80."

::= { dot11PhyFHSSEntry 1 }

dot11CurrentChannelNumber OBJECT-TYPE

SYNTAX INTEGER (0..200)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The current channel number of the frequency output by the RF

synthesizer."

::= { dot11PhyFHSSEntry 2 }

dot11MaxDwellTime OBJECT-TYPE

SYNTAX INTEGER (1..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The maximum time in TU that the transmitter

is permitted to operate on a single channel."

::= { dot11PhyFHSSEntry 3 }

dot11CurrentDwellTime OBJECT-TYPE

SYNTAX INTEGER (1..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The current time in TU that the transmitter shall operate

on a single channel, as set by the MAC.  Default is 19 TU."

::= { dot11PhyFHSSEntry 4 }

dot11CurrentSet OBJECT-TYPE

SYNTAX INTEGER (1..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The current set of patterns the PLME

is using to determine the hopping sequence. "

::= { dot11PhyFHSSEntry 5 }

dot11CurrentPattern OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The current pattern the PLME is

using to determine the hop sequence."

::= { dot11PhyFHSSEntry 6 }

dot11CurrentIndex OBJECT-TYPE

SYNTAX INTEGER (1..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The current index value the PLME is using to determine

the CurrentChannelNumber."

::= { dot11PhyFHSSEntry 7 }

dot11EHCCPrimeRadix OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute indicates the value to be

used as the prime radix (N) in the HCC and

EHCC algorithms."

::= { dot11PhyFHSSEntry 8 }

dot11EHCCNumberofChannelsFamilyIndex OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute indicates the value to be

used as the maximum for the family index (a)

in the HCC and EHCC algorithms. The value of

this field shall not be less than the prime

radix minus 3 (N - 3). The valid range of

allowed values is (N - 1), (N - 2), and (N - 3)."

::= { dot11PhyFHSSEntry 9 }

dot11EHCCCapabilityImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the

station implementation is capable of generating

the HCC or EHCC algorithms for determining Hopping

patterns.  The capability is disabled, otherwise.

The default value of this attribute is false."

::= { dot11PhyFHSSEntry 10 }

dot11EHCCCapabilityEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the

capability of the station to operate using the HCC

or EHCC algorithms for determining Hopping Patterns

is enabled.  The capability is disabled, otherwise.

The default value of this attribute is false."

::= { dot11PhyFHSSEntry 11 }

dot11HopAlgorithmAdopted OBJECT-TYPE

SYNTAX INTEGER { crnt(1), hopindex(2), hcc(3) }

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, indicates which of the algorithms

will be used to generate the Hopping Patterns.

Valid values are:

1 - hopping patterns as defined in Clause 14 (Frequency-Hopping

spread spectrum (FHSS) PHY specification for the 2.4 GHz

industrial, scientific, and medical (ISM) band)

2 - hop index method (with or without table)

3 - HCC/EHCC method"

::= { dot11PhyFHSSEntry 12 }

dot11RandomTableFlag OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, indicates that a Random Table is

present when the value is true. When the value is

false it indicates that a Random Table is not

present and that the hop index method is to be

used to determine the hopping sequence. The default

value of this attribute is true."

::= { dot11PhyFHSSEntry 13 }

dot11NumberofHoppingSets OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The Number of Sets field indicates the total

number of sets within the hopping patterns."

::= { dot11PhyFHSSEntry 14 }

dot11HopModulus OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The number of allowed channels for the hopping

set. This is defined by the governing regulatory

agency for the country code of the country

in which this device is operating."

::= { dot11PhyFHSSEntry 15 }

dot11HopOffset OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The next position in the hopping set."

::= { dot11PhyFHSSEntry 16 }

-- **********************************************************************

-- * End of dot11PhyFHSS  TABLE

-- **********************************************************************

-- **********************************************************************

-- * dot11PhyDSSSEntry  TABLE

-- **********************************************************************

dot11PhyDSSSTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11PhyDSSSEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Entry of attributes for dot11PhyDSSSEntry.  Implemented as a

table indexed on ifIndex to allow for multiple instances on

an Agent."

::= { dot11phy 5 }

dot11PhyDSSSEntry OBJECT-TYPE

SYNTAX Dot11PhyDSSSEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11PhyDSSSEntry Table.

ifIndex - Each IEEE 802.11 interface is represented by an

ifEntry.  Interface tables in this MIB module are indexed

by ifIndex."

INDEX { ifIndex }

::= { dot11PhyDSSSTable 1 }

Dot11PhyDSSSEntry ::=

SEQUENCE { dot11CurrentChannel    INTEGER,

dot11CCAModeSupported  INTEGER,

dot11CurrentCCAMode    INTEGER,

dot11EDThreshold       Integer32 }

dot11CurrentChannel OBJECT-TYPE

SYNTAX INTEGER (1..14)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The current operating frequency channel of the DSSS

PHY. Valid channel numbers are as defined in 15.4.6.2"

::= { dot11PhyDSSSEntry 1 }

dot11CCAModeSupported OBJECT-TYPE

SYNTAX INTEGER (1..7)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"dot11CCAModeSupported is a bit-significant value,

representing all of the CCA modes supported by the PHY.

Valid values are:

energy detect only (ED_ONLY) = 01,

carrier sense only (CS_ONLY) = 02,

carrier sense and energy detect (ED_and_CS)= 04

or the logical sum of any of these values.  This

attribute shall not be used to indicate the CCA modes

supported by a higher rate extension PHY.  Rather, the

dot11HRCCAModeSupported attribute shall be used to

indicate the CCA modes of the higher rate extension PHY."

::= { dot11PhyDSSSEntry 2 }

dot11CurrentCCAMode OBJECT-TYPE

SYNTAX INTEGER { edonly(1), csonly(2), edandcs(4), cswithtimer(8),

hrcsanded(16) }

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The current CCA method in operation.   Valid values are:

energy detect only (edonly) = 01,

carrier sense only (csonly) = 02,

carrier sense and energy detect (edandcs)= 04

carrier sense with timer (cswithtimer)= 08

high rate carrier sense and energy detect (hrcsanded)=16."

::= { dot11PhyDSSSEntry 3 }

dot11EDThreshold OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The current Energy Detect Threshold being used by the DSSS PHY."

::= { dot11PhyDSSSEntry 4 }

-- **********************************************************************

-- * End of dot11PhyDSSSEntry  TABLE

-- **********************************************************************

-- **********************************************************************

-- * dot11PhyIR  TABLE

-- **********************************************************************

dot11PhyIRTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11PhyIREntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group of attributes for dot11PhyIRTable.  Implemented as a

table indexed on ifIndex to allow for multiple instances on

an Agent."

::= { dot11phy 6 }

dot11PhyIREntry OBJECT-TYPE

SYNTAX Dot11PhyIREntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11PhyIR Table.

ifIndex - Each IEEE 802.11 interface is represented by an

ifEntry.  Interface tables in this MIB module are indexed

by ifIndex."

INDEX { ifIndex }

::= { dot11PhyIRTable 1 }

Dot11PhyIREntry ::=

SEQUENCE { dot11CCAWatchdogTimerMax       Integer32,

dot11CCAWatchdogCountMax       Integer32,

dot11CCAWatchdogTimerMin       Integer32,

dot11CCAWatchdogCountMin       Integer32 }

dot11CCAWatchdogTimerMax OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This parameter, together with CCAWatchdogCountMax,

determines when energy detected in the channel can be

ignored."

::= { dot11PhyIREntry 1 }

dot11CCAWatchdogCountMax OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This parameter, together with CCAWatchdogTimerMax,

determines when energy detected in the channel can be

ignored."

::= { dot11PhyIREntry 2 }

dot11CCAWatchdogTimerMin OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The minimum value to which CCAWatchdogTimerMax can be

set."

::= { dot11PhyIREntry 3 }

dot11CCAWatchdogCountMin OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The minimum value to which CCAWatchdogCount can be set."

::= { dot11PhyIREntry 4 }

-- **********************************************************************

-- * End of dot11PhyIR  TABLE

-- **********************************************************************

-- **********************************************************************

-- * dot11RegDomainsSupported  TABLE

-- **********************************************************************

dot11RegDomainsSupportedTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11RegDomainsSupportedEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"There are different operational requirements dependent on

the regulatory domain.  This attribute list describes the

regulatory domains the PLCP and PMD support in this

implementation.  Currently defined values and their

corresponding Regulatory Domains are:

FCC (USA) = X'10', DOC (Canada) = X'20', ETSI (most of

Europe) = X'30', Spain = X'31', France = X'32',

Japan = X'40', China = X'50', Other = X'00' "

::= { dot11phy 7}

dot11RegDomainsSupportedEntry OBJECT-TYPE

SYNTAX Dot11RegDomainsSupportedEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11RegDomainsSupportedTable.

ifIndex - Each IEEE 802.11 interface is represented by an

ifEntry.  Interface tables in this MIB module are indexed

by ifIndex."

INDEX { ifIndex,

dot11RegDomainsSupportedIndex }

::= { dot11RegDomainsSupportedTable 1 }

Dot11RegDomainsSupportedEntry ::=

SEQUENCE { dot11RegDomainsSupportedIndex    Integer32,

dot11RegDomainsSupportedValue    INTEGER }

dot11RegDomainsSupportedIndex OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The auxiliary variable used to identify instances

of the columnar objects in the RegDomainsSupport Table."

::= { dot11RegDomainsSupportedEntry 1 }

dot11RegDomainsSupportedValue OBJECT-TYPE

SYNTAX INTEGER { fcc(16), doc(32), etsi(48), spain (49), france(50),

japan (64), other (0)}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"There are different operational requirements dependent on

the regulatory domain.  This attribute list describes the

regulatory domains the PLCP and PMD support in this

implementation.  Currently defined values and their

corresponding Regulatory Domains are:

FCC (USA) = X'10', DOC (Canada) = X'20', ETSI (most of

Europe) = X'30', Spain = X'31', France = X'32',

Japan = X'40', China = X'50' "

::= { dot11RegDomainsSupportedEntry 2 }

-- **********************************************************************

-- * End of dot11RegDomainsSupported  TABLE

-- **********************************************************************

-- **********************************************************************

-- * dot11AntennasList  TABLE

-- **********************************************************************

dot11AntennasListTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11AntennasListEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This table represents the list of antennae.  An antenna can be

marked to be capable of transmitting, receiving, and/or for

participation in receive diversity.  Each entry in this table

represents a single antenna with its properties.  The maximum

number of antennae that can be contained in this table is 255."

::= { dot11phy 8 }

dot11AntennasListEntry OBJECT-TYPE

SYNTAX Dot11AntennasListEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11AntennasListTable, representing the properties

of a single antenna.

ifIndex - Each IEEE 802.11 interface is represented by an

ifEntry.  Interface tables in this MIB module are indexed

by ifIndex."

INDEX { ifIndex,

dot11AntennaListIndex }

::= { dot11AntennasListTable 1 }

Dot11AntennasListEntry ::=

SEQUENCE { dot11AntennaListIndex     Integer32,

dot11SupportedTxAntenna   TruthValue,

dot11SupportedRxAntenna   TruthValue,

dot11DiversitySelectionRx TruthValue }

dot11AntennaListIndex OBJECT-TYPE

SYNTAX Integer32 (1..255)

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The unique index of an antenna which is used to identify the columnar

objects in the dot11AntennasList Table."

::= { dot11AntennasListEntry 1 }

dot11SupportedTxAntenna OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"When true, this object indicates that the antenna represented by

dot11AntennaIndex can be used as a transmit antenna."

::= { dot11AntennasListEntry 2 }

dot11SupportedRxAntenna OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"When true, this object indicates that the antenna represented by

the dot11AntennaIndex can be used as a receive antenna."

::= { dot11AntennasListEntry 3 }

dot11DiversitySelectionRx OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"When true, this object indicates that the antenna represented by

dot11AntennaIndex can be used for receive diversity.  This object

shall be true only if the antenna can be used as a receive antenna,

as indicated by dot11SupportedRxAntenna."

::= { dot11AntennasListEntry 4 }

-- **********************************************************************

-- * End of dot11AntennasList  TABLE

-- **********************************************************************

-- **********************************************************************

-- * dot11SupportedDataRatesTx  TABLE

-- **********************************************************************

dot11SupportedDataRatesTxTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11SupportedDataRatesTxEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The Transmit bit rates supported by the PLCP and PMD,

represented by a count from X'02-X'7f, corresponding to data

rates in increments of 500kbit/s from 1 Mb/s to 63.5 Mb/s subject

to limitations of each individual PHY."

::= { dot11phy 9 }

dot11SupportedDataRatesTxEntry OBJECT-TYPE

SYNTAX Dot11SupportedDataRatesTxEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An Entry (conceptual row) in the dot11SupportedDataRatesTx

Table.

ifIndex - Each IEEE 802.11 interface is represented by an

ifEntry.  Interface tables in this MIB module are indexed

by ifIndex."

INDEX { ifIndex,

dot11SupportedDataRatesTxIndex }

::= { dot11SupportedDataRatesTxTable  1 }

Dot11SupportedDataRatesTxEntry ::=

SEQUENCE { dot11SupportedDataRatesTxIndex  Integer32,

dot11SupportedDataRatesTxValue  Integer32 }

dot11SupportedDataRatesTxIndex OBJECT-TYPE

SYNTAX Integer32 (1..255)

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index object that identifies which data rate to access.

Range is 1..255."

::= { dot11SupportedDataRatesTxEntry 1 }

dot11SupportedDataRatesTxValue OBJECT-TYPE

SYNTAX Integer32 (2..127)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The Transmit bit rates supported by the PLCP and PMD,

represented by a count from X'02-X'7f, corresponding to data

rates in increments of 500kbit/s from 1 Mb/s to 63.5 Mb/s subject

to limitations of each individual PHY."

::= { dot11SupportedDataRatesTxEntry 2 }

-- **********************************************************************

-- * End of dot11SupportedDataRatesTx  TABLE

-- **********************************************************************

-- **********************************************************************

-- * dot11SupportedDataRatesRx  TABLE

-- **********************************************************************

dot11SupportedDataRatesRxTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11SupportedDataRatesRxEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The receive bit rates supported by the PLCP and PMD,

represented by a count from X'002-X'7f, corresponding to data

rates in increments of 500kbit/s from 1 Mb/s to 63.5 Mb/s."

::= { dot11phy 10 }

dot11SupportedDataRatesRxEntry OBJECT-TYPE

SYNTAX Dot11SupportedDataRatesRxEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An Entry (conceptual row) in the dot11SupportedDataRatesRx Table.

ifIndex - Each IEEE 802.11 interface is represented by an

ifEntry.  Interface tables in this MIB module are indexed

by ifIndex."

INDEX { ifIndex,

dot11SupportedDataRatesRxIndex }

::= { dot11SupportedDataRatesRxTable  1 }

Dot11SupportedDataRatesRxEntry ::=

SEQUENCE { dot11SupportedDataRatesRxIndex  Integer32,

dot11SupportedDataRatesRxValue  Integer32 }

dot11SupportedDataRatesRxIndex OBJECT-TYPE

SYNTAX Integer32 (1..255)

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index object that identifies which data rate to access.

Range is 1..255."

::= { dot11SupportedDataRatesRxEntry 1 }

dot11SupportedDataRatesRxValue OBJECT-TYPE

SYNTAX Integer32 (2..127)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The receive bit rates supported by the PLCP and PMD,

represented by a count from X'02-X'7f, corresponding to data

rates in increments of 500kbit/s from 1 Mb/s to 63.5 Mb/s."

::= { dot11SupportedDataRatesRxEntry 2 }

-- **********************************************************************

-- * End of dot11SupportedDataRatesRx  TABLE

-- **********************************************************************

-- **********************************************************************

-- * dot11PhyOFDM TABLE

-- **********************************************************************

dot11PhyOFDMTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11PhyOFDMEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group of attributes for dot11PhyOFDMTable. Implemented as a

table indexed on ifindex to allow for multiple instances on

an Agent."

::= { dot11phy 11 }

dot11PhyOFDMEntry OBJECT-TYPE

SYNTAX Dot11PhyOFDMEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11PhyOFDM Table.

ifIndex - Each IEEE 802.11 interface is represented by an

ifEntry. Interface tables in this MIB module are indexed

by ifIndex."

INDEX { ifIndex }

::= { dot11PhyOFDMTable 1 }

Dot11PhyOFDMEntry ::=

SEQUENCE { dot11CurrentFrequency        											INTEGER,

dot11TIThreshold             											Integer32,

dot11FrequencyBandsSupported 											INTEGER,

dot11ChannelStartingFactor 											Integer32,

dot11FiveMHzOperationImplemented 											TruthValue,

dot11TenMHzOperationImplemented 											TruthValue,

dot11TwentyMHzOperationImplemented											TruthValue,

dot11PhyOFDMChannelWidth 											INTEGER,

dot11OFDMCCAEDImplemented 											TruthValue,

dot11OFDMCCAEDRequired 											TruthValue,

dot11OFDMEDThreshold 											Unsigned32 }

dot11CurrentFrequency OBJECT-TYPE

SYNTAX INTEGER (0..200)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The number of the current operating frequency channel of the OFDM

PHY."

::= { dot11PhyOFDMEntry 1 }

dot11TIThreshold OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS deprecated

DESCRIPTION

"The Threshold being used to detect a busy medium (frequency).

CCA shall report a busy medium upon detecting the RSSI above

this threshold."

::= { dot11PhyOFDMEntry 2 }

dot11FrequencyBandsSupported OBJECT-TYPE

SYNTAX INTEGER (1..127)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The capability of the OFDM PHY implementation to operate in the

4.9 GHz and 5 GHz bands. Coded as an integer value with bit 0 LSB as

follows:

bit 0 .. capable of operating in the 5.15-5.25 GHz band

bit 1 .. capable of operating in the 5.25-5.35 GHz band

bit 2 .. capable of operating in the 5.725-5.825 GHz band

bit 3 .. capable of operating in the 5.47-5.725 GHz band

bit 4 .. capable of operating in the lower Japanese (5.15-

5.25 GHz) band

bit 5 .. capable of operating in the 5.03-5.091 GHz band

bit 6 .. capable of operating in the 4.94-4.99 GHz band

For example, for an implementation capable of operating in the

5.15-5.35 GHz bands this attribute would take the value 3."

::= { dot11PhyOFDMEntry 3 }

dot11ChannelStartingFactor OBJECT-TYPE

SYNTAX Integer32 (8000..10000)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The base factor from which channel center frequencies are calculat-

ed. This number is multiplied by 500 kHz to form the base frequency

to be added to the channel number x 5 MHz. The default value of this

attribute shall be 10 000."

::= { dot11PhyOFDMEntry 4 }

dot11FiveMHzOperationImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, shall indicate that the 5 MHz

Operation is implemented. The default value of this attribute is

false."

::= { dot11PhyOFDMEntry 5 }

dot11TenMHzOperationImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, shall indicate that the 10 MHz

Operation is implemented. The default value of this attribute is

false."

::= { dot11PhyOFDMEntry 6 }

dot11TwentyMHzOperationImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, shall indicate that the 20 MHz

Operation is implemented. The default value of this attribute is

true."

::= { dot11PhyOFDMEntry 7 }

dot11PhyOFDMChannelWidth OBJECT-TYPE

SYNTAX INTEGER { width5MHz(1), width10MHz(2), width20MHz(3)}

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is an 8-bit integer value that identifies the OFDM

PHY channel width. Currently defined values and their corresponding

Channel widths are:

5MHz = 01, 10MHz = 02, 20MHz = 03"

::= { dot11PhyOFDMEntry 8 }

dot11OFDMCCAEDImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates that the OFDM PHY is capable of CCA-

Energy Detect."

::= { dot11PhyOFDMEntry 9 }

dot11OFDMCCAEDRequired OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates that the PHY CCA-Energy Detect function-

ality is enabled."

::= { dot11PhyOFDMEntry 10 }

dot11OFDMEDThreshold OBJECT-TYPE

SYNTAX Unsigned32 (0..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The current Energy Detect Threshold being used by the OFDM PHY."

::= { dot11PhyOFDMEntry 11 }

-- **********************************************************************

-- * End of dot11PhyOFDM TABLE

-- **********************************************************************

-- **********************************************************************

-- * dot11PhyHRDSSSEntry TABLE

-- **********************************************************************

dot11PhyHRDSSSTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11PhyHRDSSSEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Entry of attributes for dot11PhyHRDSSSEntry.

Implemented as a table indexed on ifIndex to allow for

multiple instances on an Agent."

::= { dot11phy 12 }

dot11PhyHRDSSSEntry OBJECT-TYPE

SYNTAX Dot11PhyHRDSSSEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11PhyHRDSSSEntry Table.

ifIndex - Each IEEE 802.11 interface is represented by an

ifEntry. Interface tables in this MIB module are indexed

by ifIndex."

INDEX { ifIndex }

::= { dot11PhyHRDSSSTable 1 }

Dot11PhyHRDSSSEntry ::=

SEQUENCE { dot11ShortPreambleOptionImplemented												TruthValue,

dot11PBCCOptionImplemented												TruthValue,

dot11ChannelAgilityPresent												TruthValue,

dot11ChannelAgilityEnabled												TruthValue,

dot11HRCCAModeSupported												INTEGER }

dot11ShortPreambleOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, shall indicate that the short preamble

option as defined in 18.2.2.2 (Short PPDU format) is implemented.

The default value of this attribute shall be false."

::= { dot11PhyHRDSSSEntry 1 }

dot11PBCCOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, shall indicate that the PBCC modulation

option as defined in  18.4.6.6 (DSSS/PBCC data modulation and modu-

lation rate (optional)) is implemented.  The default value of this

attribute shall be false."

::= { dot11PhyHRDSSSEntry 2 }

dot11ChannelAgilityPresent OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates that the PHY is capable of

channel agility."

::= { dot11PhyHRDSSSEntry 3 }

dot11ChannelAgilityEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates that the PHY channel agility

functionality is enabled."

::= { dot11PhyHRDSSSEntry 4 }

dot11HRCCAModeSupported OBJECT-TYPE

SYNTAX INTEGER (1..31)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"dot11HRCCAModeSupported is a bit-significant value,

representing all of the CCA modes supported by the PHY.

Valid values are:

energy detect only (ED_ONLY) = 01,

carrier sense only (CS_ONLY) = 02,

carrier sense and energy detect (ED_and_CS)= 04,

carrier sense with timer (CS_and_Timer)= 08,

high rate carrier sense and energy detect

(HRCS_and_ED)= 16

or the logical sum of any of these values.  In

the high rate extension PHY, this attribute shall

be used in preference to the dot11CCAModeSupported

attribute."

::= { dot11PhyHRDSSSEntry 5 }

-- **********************************************************************

-- * End of dot11PhyHRDSSSEntry TABLE

-- **********************************************************************

-- **********************************************************************

-- * dot11HoppingPattern TABLE

-- **********************************************************************

dot11HoppingPatternTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11HoppingPatternEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The (conceptual) table of attributes necessary for

a frequency hopping implementation to be able to

create the hopping sequences necessary to operate

in the subband for the associated domain country string."

::= { dot11phy 13 }

dot11HoppingPatternEntry OBJECT-TYPE

SYNTAX Dot11HoppingPatternEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry (conceptual row) in the Hopping Pattern Table

that indicates the random hopping sequence to be followed.

IfIndex - Each IEEE 802.11 interface is represented

by an ifEntry. Interface tables in this MIB are indexed

by ifIndex."

INDEX { ifIndex,

dot11HoppingPatternIndex }

::= { dot11HoppingPatternTable 1 }

Dot11HoppingPatternEntry ::=

SEQUENCE { dot11HoppingPatternIndex											Integer32,

dot11RandomTableFieldNumber											Integer32 }

dot11HoppingPatternIndex OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The auxiliary variable used to identify instances of

the columnar objects in the Hopping Pattern Table."

::= { dot11HoppingPatternEntry 1 }

dot11RandomTableFieldNumber OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall indicate the value of the

starting channel number in the hopping sequence of

the subband for the associated domain country string.

The default value of this attribute shall be zero."

::= { dot11HoppingPatternEntry 2 }

-- **********************************************************************

-- * End of dot11HoppingPattern TABLE

-- **********************************************************************

-- **********************************************************************

-- * dot11PhyERP TABLE

-- **********************************************************************

dot11PhyERPTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11PhyERPEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Entry of attributes for dot11PhyERPEntry. Implemented as a

table indexed on ifIndex to allow for multiple instances on an

Agent."

::= { dot11phy 14 }

dot11PhyERPEntry OBJECT-TYPE

SYNTAX Dot11PhyERPEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11PhyERPEntry Table.

ifIndex - Each 802.11 interface is represented by an ifEntry.

Interface tables in this MIB module are indexed by ifIndex."

INDEX {ifIndex}

::= { dot11PhyERPTable 1 }

Dot11PhyERPEntry ::= SEQUENCE {

dot11ERPPBCCOptionImplemented								TruthValue,

dot11ERPBCCOptionEnabled								TruthValue,

dot11DSSSOFDMOptionImplemented								TruthValue,

dot11DSSSOFDMOptionEnabled								TruthValue,

dot11ShortSlotTimeOptionImplemented								TruthValue,

dot11ShortSlotTimeOptionEnabled								TruthValue }

dot11ERPPBCCOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, shall indicate that the ERP-PBCC modu-

lation option as defined in 19.6 (ERP-PBCC operation specifica-

tions) is implemented. The default value of this attribute is

false."

::= { dot11PhyERPEntry 1 }

dot11ERPBCCOptionEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when true, shall indicate that the ERP-PBCC

option as defined in 19.6 (ERP-PBCC operation specifications) is

enabled. The default value of this attribute is false."

::= { dot11PhyERPEntry 2 }

dot11DSSSOFDMOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, shall indicate that the DSSS-OFDM

option as defined in 19.7 (DSSS-OFDM operation specifications) is

implemented. The default value of this attribute is false."

::= { dot11PhyERPEntry 3 }

dot11DSSSOFDMOptionEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when true, shall indicate that the DSSS-OFDM

option as defined in 19.7 (DSSS-OFDM operation specifications) is

enabled. The default value of this attribute is false."

::= { dot11PhyERPEntry 4 }

dot11ShortSlotTimeOptionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when true, shall indicate that the Short Slot

Time option as defined in 7.3.1.4 (Capability Information field)

is implemented. The default value of this attribute is false."

::= { dot11PhyERPEntry 5}

dot11ShortSlotTimeOptionEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when true, shall indicate that the Short Slot

Time option as defined in 7.3.1.4 (Capability Information field)

is enabled. The default value of this attribute is false."

::= { dot11PhyERPEntry 6 }

-- **********************************************************************

-- * End of dot11PhyERP TABLE

-- **********************************************************************

-- **********************************************************************

-- * Conformance Information

-- **********************************************************************

dot11Conformance				OBJECT IDENTIFIER ::= { ieee802dot11 5 }

dot11Groups 				OBJECT IDENTIFIER ::= { dot11Conformance 1 }

dot11Compliances				OBJECT IDENTIFIER ::= { dot11Conformance 2 }

-- **********************************************************************

-- * Compliance Statements

-- **********************************************************************

dot11Compliance MODULE-COMPLIANCE

STATUS  current

DESCRIPTION

"The compliance statement for SNMPv2 entities

that implement the IEEE 802.11 MIB."

MODULE  -- this module

MANDATORY-GROUPS {

dot11SMTbase9,

dot11MACbase2, dot11CountersGroup2,

dot11SmtAuthenticationAlgorithms,

dot11ResourceTypeID, dot11PhyOperationComplianceGroup }

GROUP dot11PhyDSSSComplianceGroup

DESCRIPTION

"Implementation of this group is required when object

dot11PHYType is dsss.  This group is

mutually exclusive with the groups dot11PhyIRComplianceGroup,

dot11PhyFHSSComplianceGroup2, dot11PhyOFDMComplianceGroup3

and dot11PhyHRDSSSComplianceGroup."

GROUP dot11PhyIRComplianceGroup

DESCRIPTION

"Implementation of this group is required when object

dot11PHYType is irbaseband.  This group is

mutually exclusive with the groups dot11PhyDSSSComplianceGroup,

dot11PhyFHSSComplianceGroup2, dot11PhyOFDMComplianceGroup3

and dot11PhyHRDSSSComplianceGroup."

GROUP dot11PhyFHSSComplianceGroup2

DESCRIPTION

"Implementation of this group is required when object

dot11PHYType is fhss.  This group is

mutually exclusive with the groups dot11PhyDSSSComplianceGroup,

dot11PhyIRComplianceGroup, dot11PhyOFDMComplianceGroup3

and dot11PhyHRDSSSComplianceGroup."

GROUP dot11PhyOFDMComplianceGroup3

DESCRIPTION

"Implementation of this group is required when object

dot11PHYType is ofdm. This group is

mutually exclusive with the groups dot11PhyDSSSComplianceGroup,

dot11PhyIRComplianceGroup, dot11PhyFHSSComplianceGroup2

and dot11PhyHRDSSSComplianceGroup."

GROUP dot11PhyHRDSSSComplianceGroup

DESCRIPTION

"Implementation of this group is required when object

dot11PHYType is hrdsss. This group is

mutually exclusive with the groups

dot11PhyDSSSComplianceGroup, dot11PhyIRComplianceGroup,

dot11PhyFHSSComplianceGroup2 and dot11PhyOFDMComplianceGroup3."

GROUP dot11PhyERPComplianceGroup

DESCRIPTION

"Implementation of this group is required when object

dot11PHYType is ERP. This group is mutually

exclusive with the groups dot11PhyIRComplianceGroup and

dot11PhyFHSSComplianceGroup2."

-- OPTIONAL-GROUPS { dot11SMTprivacy, dot11MACStatistics,

--    dot11PhyAntennaComplianceGroup, dot11PhyTxPowerComplianceGroup,

--    dot11PhyRegDomainsSupportGroup,

--    dot11PhyAntennasListGroup, dot11PhyRateGroup,

--    dot11MultiDomainCapabilityGroup,

--    dot11PhyFHSSComplianceGroup2, dot11RSNAadditions,

--    dot11RegulatoryClassesGroup, dot11Qosadditions,

--    dot11RMCompliance, dot11FTComplianceGroup  }

--

::= { dot11Compliances 1 }

-- **********************************************************************

-- *   Groups - units of conformance

-- **********************************************************************

dot11SMTbase OBJECT-GROUP

OBJECTS { dot11StationID, dot11MediumOccupancyLimit,

dot11CFPollable,

dot11CFPPeriod,

dot11CFPMaxDuration,

dot11AuthenticationResponseTimeOut,

dot11PrivacyOptionImplemented,

dot11PowerManagementMode,

dot11DesiredSSID, dot11DesiredBSSType,

dot11OperationalRateSet,

dot11BeaconPeriod, dot11DTIMPeriod,

dot11AssociationResponseTimeOut }

STATUS deprecated

DESCRIPTION

"The SMT object class provides the necessary support at the

STA to manage the processes in the STA such that the STA may

work cooperatively as a part of an IEEE 802.11 network."

::= { dot11Groups 1 }

dot11SMTprivacy OBJECT-GROUP

OBJECTS { dot11PrivacyInvoked,

dot11WEPKeyMappingLength, dot11ExcludeUnencrypted,

dot11WEPICVErrorCount , dot11WEPExcludedCount ,

dot11WEPDefaultKeyID,

dot11WEPDefaultKeyValue,

dot11WEPKeyMappingWEPOn,

dot11WEPKeyMappingValue , dot11WEPKeyMappingAddress,

dot11WEPKeyMappingStatus }

STATUS current

DESCRIPTION

"The SMTPrivacy package is a set of attributes that shall be

present if WEP is implemented in the STA."

::= { dot11Groups 2 }

dot11MACbase OBJECT-GROUP

OBJECTS { dot11MACAddress, dot11Address,

dot11GroupAddressesStatus,

dot11RTSThreshold, dot11ShortRetryLimit,

dot11LongRetryLimit, dot11FragmentationThreshold,

dot11MaxTransmitMSDULifetime,

dot11MaxReceiveLifetime, dot11ManufacturerID,

dot11ProductID }

STATUS deprecated

DESCRIPTION

"The MAC object class provides the necessary support for the

access control, generation, and verification of frame check

sequences (FCSs), and proper delivery of valid data to upper

layers."

::= { dot11Groups 3 }

dot11MACStatistics OBJECT-GROUP

OBJECTS { dot11RetryCount, dot11MultipleRetryCount,

dot11RTSSuccessCount, dot11RTSFailureCount,

dot11ACKFailureCount, dot11FrameDuplicateCount }

STATUS current

DESCRIPTION

"The MACStatistics package provides extended statistical

information on the operation of the MAC.  This

package is completely optional."

::= { dot11Groups 4 }

dot11ResourceTypeID OBJECT-GROUP

OBJECTS { dot11ResourceTypeIDName, dot11manufacturerOUI,

dot11manufacturerName, dot11manufacturerProductName,

dot11manufacturerProductVersion }

STATUS current

DESCRIPTION

"Attributes used to identify a STA, its manufacturer,

and various product names and versions."

::= { dot11Groups 5 }

dot11SmtAuthenticationAlgorithms OBJECT-GROUP

OBJECTS { dot11AuthenticationAlgorithm,

dot11AuthenticationAlgorithmsEnable }

STATUS current

DESCRIPTION

"Authentication Algorithm Table."

::= { dot11Groups 6 }

dot11PhyOperationComplianceGroup OBJECT-GROUP

OBJECTS { dot11PHYType, dot11CurrentRegDomain, dot11TempType }

STATUS current

DESCRIPTION

"PHY layer operations attributes."

::= { dot11Groups 7 }

dot11PhyAntennaComplianceGroup OBJECT-GROUP

OBJECTS { dot11CurrentTxAntenna, dot11DiversitySupport,

dot11CurrentRxAntenna }

STATUS current

DESCRIPTION

"Attributes for Data Rates for IEEE 802.11."

::= { dot11Groups 8 }

dot11PhyTxPowerComplianceGroup OBJECT-GROUP

OBJECTS { dot11NumberSupportedPowerLevels, dot11TxPowerLevel1,

dot11TxPowerLevel2, dot11TxPowerLevel3, dot11TxPowerLevel4,

dot11TxPowerLevel5, dot11TxPowerLevel6, dot11TxPowerLevel7,

dot11TxPowerLevel8, dot11CurrentTxPowerLevel }

STATUS current

DESCRIPTION

"Attributes for Control and Management of transmit power."

::= { dot11Groups 9 }

dot11PhyFHSSComplianceGroup OBJECT-GROUP

OBJECTS { dot11HopTime, dot11CurrentChannelNumber,

dot11MaxDwellTime, dot11CurrentDwellTime, dot11CurrentSet,

dot11CurrentPattern, dot11CurrentIndex }

STATUS deprecated

DESCRIPTION

"Attributes that configure the Frequency Hopping for IEEE

802.11."

::= { dot11Groups 10 }

dot11PhyDSSSComplianceGroup OBJECT-GROUP

OBJECTS { dot11CurrentChannel, dot11CCAModeSupported,

dot11CurrentCCAMode, dot11EDThreshold }

STATUS current

DESCRIPTION

"Attributes that configure the DSSS for IEEE 802.11."

::= { dot11Groups 11 }

dot11PhyIRComplianceGroup OBJECT-GROUP

OBJECTS { dot11CCAWatchdogTimerMax, dot11CCAWatchdogCountMax,

dot11CCAWatchdogTimerMin, dot11CCAWatchdogCountMin }

STATUS current

DESCRIPTION

"Attributes that configure the baseband IR for IEEE 802.11."

::= { dot11Groups 12 }

dot11PhyRegDomainsSupportGroup OBJECT-GROUP

OBJECTS { dot11RegDomainsSupportedTable }

STATUS current

DESCRIPTION

"Attributes that specify the supported Regulation Domains."

::= { dot11Groups 13 }

dot11PhyAntennasListGroup OBJECT-GROUP

OBJECTS { dot11SupportedTxAntenna,

dot11SupportedRxAntenna, dot11DiversitySelectionRx }

STATUS current

DESCRIPTION

"Attributes that specify the supported Regulation Domains."

::= { dot11Groups 14 }

dot11PhyRateGroup OBJECT-GROUP

OBJECTS { dot11SupportedDataRatesTxValue,

dot11SupportedDataRatesRxValue }

STATUS current

DESCRIPTION

"Attributes for Data Rates for IEEE 802.11."

::= { dot11Groups 15 }

dot11CountersGroup OBJECT-GROUP

OBJECTS { dot11TransmittedFragmentCount,

dot11MulticastTransmittedFrameCount,

dot11FailedCount, dot11ReceivedFragmentCount,

dot11MulticastReceivedFrameCount,

dot11FCSErrorCount,

dot11WEPUndecryptableCount,

dot11TransmittedFrameCount }

STATUS deprecated

DESCRIPTION

"Attributes from the dot11CountersGroup that are not described

in the dot11MACStatistics group.  These objects are

mandatory."

::= { dot11Groups 16 }

dot11NotificationGroup NOTIFICATION-GROUP

NOTIFICATIONS { dot11Disassociate,

dot11Deauthenticate,

dot11AuthenticateFail }

STATUS current

DESCRIPTION

"IEEE 802.11 notifications"

::= { dot11Groups 17 }

dot11SMTbase2 OBJECT-GROUP

OBJECTS { dot11MediumOccupancyLimit,

dot11CFPollable,

dot11CFPPeriod,

dot11CFPMaxDuration,

dot11AuthenticationResponseTimeOut,

dot11PrivacyOptionImplemented,

dot11PowerManagementMode,

dot11DesiredSSID, dot11DesiredBSSType,

dot11OperationalRateSet,

dot11BeaconPeriod, dot11DTIMPeriod,

dot11AssociationResponseTimeOut,

dot11DisassociateReason,

dot11DisassociateStation,

dot11DeauthenticateReason,

dot11DeauthenticateStation,

dot11AuthenticateFailStatus,

dot11AuthenticateFailStation }

STATUS deprecated

DESCRIPTION

"The SMTbase2 object class provides the necessary support at the

STA to manage the processes in the STA such that the STA may

work cooperatively as a part of an IEEE 802.11 network."

::= { dot11Groups 18 }

dot11PhyOFDMComplianceGroup OBJECT-GROUP

OBJECTS { dot11CurrentFrequency,

dot11TIThreshold,

dot11FrequencyBandsSupported,

dot11ChannelStartingFactor }

STATUS deprecated

DESCRIPTION

"Attributes that configure the OFDM for IEEE 802.11."

::= { dot11Groups 19 }

dot11SMTbase3 OBJECT-GROUP

OBJECTS { dot11MediumOccupancyLimit,

dot11CFPollable,

dot11CFPPeriod,

dot11CFPMaxDuration,

dot11AuthenticationResponseTimeOut,

dot11PrivacyOptionImplemented,

dot11PowerManagementMode,

dot11DesiredSSID, dot11DesiredBSSType,

dot11OperationalRateSet,

dot11BeaconPeriod, dot11DTIMPeriod,

dot11AssociationResponseTimeOut,

dot11DisassociateReason,

dot11DisassociateStation,

dot11DeauthenticateReason,

dot11DeauthenticateStation,

dot11AuthenticateFailStatus,

dot11AuthenticateFailStation,

dot11MultiDomainCapabilityImplemented,

dot11MultiDomainCapabilityEnabled,

dot11CountryString }

STATUS deprecated

DESCRIPTION

"The SMTbase3 object class provides the necessary support at the

STA to manage the processes in the STA such that the STA may

work cooperatively as a part of an IEEE 802.11 network, when the STA

is capable of multi-domain operation. This object group should be

implemented when the multi-domain capability option is implemented."

::= { dot11Groups 20 }

dot11MultiDomainCapabilityGroup OBJECT-GROUP

OBJECTS { dot11FirstChannelNumber,

dot11NumberofChannels,

dot11MaximumTransmitPowerLevel }

STATUS current

DESCRIPTION

"The dot11MultiDomainCapabilityGroup object class provides

the objects necessary to manage the channels usable by a STA,

when the multi-domain capability option is implemented."

::= { dot11Groups 21 }

dot11PhyFHSSComplianceGroup2 OBJECT-GROUP

OBJECTS { dot11HopTime, dot11CurrentChannelNumber,

dot11MaxDwellTime, dot11CurrentDwellTime, dot11CurrentSet,

dot11CurrentPattern, dot11CurrentIndex, dot11EHCCPrimeRadix,

dot11EHCCNumberofChannelsFamilyIndex,

dot11EHCCCapabilityImplemented, dot11EHCCCapabilityEnabled,

dot11HopAlgorithmAdopted, dot11RandomTableFlag,

dot11NumberofHoppingSets, dot11HopModulus,

dot11HopOffset, dot11RandomTableFieldNumber }

STATUS current

DESCRIPTION

"Attributes that configure the Frequency Hopping for IEEE

802.11 when multi-domain capability option is implemented."

::= { dot11Groups 22 }

dot11PhyHRDSSSComplianceGroup OBJECT-GROUP

OBJECTS { dot11CurrentChannel, dot11CCAModeSupported,

dot11CurrentCCAMode, dot11EDThreshold,

dot11ShortPreambleOptionImplemented,

dot11PBCCOptionImplemented, dot11ChannelAgilityPresent,

dot11ChannelAgilityEnabled, dot11HRCCAModeSupported }

STATUS current

DESCRIPTION

"Attributes that configure the HRDSSS for IEEE 802.11."

::= { dot11Groups 23 }

dot11PhyERPComplianceGroup OBJECT-GROUP

OBJECTS { dot11CurrentChannel,

dot11ShortPreambleOptionImplemented,

dot11ChannelAgilityPresent,

dot11ChannelAgilityEnabled,

dot11DSSSOFDMOptionImplemented,

dot11DSSSOFDMOptionEnabled,

dot11PBCCOptionImplemented,

dot11ERPPBCCOptionImplemented,

dot11ShortSlotTimeOptionImplemented,

dot11ShortSlotTimeOptionEnabled }

STATUS current

DESCRIPTION

"Attributes that configure the ERP."

::= { dot11Groups 24 }

dot11RSNAadditions OBJECT-GROUP

OBJECTS { dot11RSNAEnabled,

dot11RSNAPreauthenticationEnabled }

STATUS current

DESCRIPTION

"This object class provides the objects from the IEEE 802.11 MIB

required to manage RSNA functionality. Note that additional objects

for managing this functionality are located in the IEEE 802.11 RSN

MIB."

::= { dot11Groups 25 }

dot11SMTbase4 OBJECT-GROUP

OBJECTS { dot11MediumOccupancyLimit,

dot11CFPollable,

dot11CFPPeriod,

dot11CFPMaxDuration,

dot11AuthenticationResponseTimeOut,

dot11PrivacyOptionImplemented,

dot11PowerManagementMode,

dot11DesiredSSID, dot11DesiredBSSType,

dot11OperationalRateSet,

dot11BeaconPeriod, dot11DTIMPeriod,

dot11AssociationResponseTimeOut,

dot11DisassociateReason,

dot11DisassociateStation,

dot11DeauthenticateReason,

dot11DeauthenticateStation,

dot11AuthenticateFailStatus,

dot11AuthenticateFailStation,

dot11MultiDomainCapabilityImplemented,

dot11MultiDomainCapabilityEnabled,

dot11CountryString,

dot11RSNAOptionImplemented }

STATUS deprecated

DESCRIPTION

"The SMTbase4 object class provides the necessary support at the

IEEE STA to manage the processes in the STA so that the STA may work

cooperatively as a part of an IEEE 802.11 network."

::= { dot11Groups 26 }

dot11RegulatoryClassesGroup OBJECT-GROUP

OBJECTS { dot11RegulatoryClass,

dot11CoverageClass }

STATUS current

DESCRIPTION

"Attributes that configure the OFDM for IEEE 802.11 in many regula-

tory domains."

::= { dot11Groups 1029 }

-- Note:  dot11Groups 29 is used by dot11RSNSMKcachingGroup.  This number is temporary.

dot11SMTbase5 OBJECT-GROUP

OBJECTS { dot11MediumOccupancyLimit,

dot11CFPollable,

dot11CFPPeriod,

dot11CFPMaxDuration,

dot11AuthenticationResponseTimeOut,

dot11PrivacyOptionImplemented,

dot11PowerManagementMode,

dot11DesiredSSID, dot11DesiredBSSType,

dot11OperationalRateSet,

dot11BeaconPeriod, dot11DTIMPeriod,

dot11AssociationResponseTimeOut,

dot11DisassociateReason,

dot11DisassociateStation,

dot11DeauthenticateReason,

dot11DeauthenticateStation,

dot11AuthenticateFailStatus,

dot11AuthenticateFailStation,

dot11MultiDomainCapabilityImplemented,

dot11MultiDomainCapabilityEnabled,

dot11CountryString,

dot11SpectrumManagementImplemented,

dot11SpectrumManagementRequired,

dot11RSNAOptionImplemented,

dot11RegulatoryClassesImplemented,

dot11RegulatoryClassesRequired }

STATUS current

DESCRIPTION

"The SMTbase5 object class provides the necessary support at the STA

to manage the processes in the STA so that the STA may work cooper-

atively as a part of an IEEE 802.11 network, when the STA is capable

of multidomain operation. This object group should be implemented

when the multidomain capability option is implemented."

::= { dot11Groups 30 }

dot11MACbase2 OBJECT-GROUP

OBJECTS { dot11MACAddress, dot11Address,

dot11GroupAddressesStatus,

dot11RTSThreshold, dot11ShortRetryLimit,

dot11LongRetryLimit, dot11FragmentationThreshold,

dot11MaxTransmitMSDULifetime,

dot11MaxReceiveLifetime, dot11ManufacturerID,

dot11ProductID, dot11CAPLimit, dot11HCCWmin,

dot11HCCWmax, dot11HCCAIFSN,

dot11ADDBAResponseTimeout, dot11ADDTSResponseTimeout,

dot11ChannelUtilizationBeaconInterval, dot11ScheduleTimeout,

dot11DLSResponseTimeout, dot11QAPMissingAckRetryLimit,

dot11EDCAAveragingPeriod }

STATUS current

DESCRIPTION

"The MAC object class provides the necessary support for the

access control, generation, and verification of frame check

sequences (FCSs), and proper delivery of valid data to upper

layers."

::= { dot11Groups 31 }

dot11CountersGroup2 OBJECT-GROUP

OBJECTS { dot11TransmittedFragmentCount,

dot11MulticastTransmittedFrameCount,

dot11FailedCount, dot11ReceivedFragmentCount,

dot11MulticastReceivedFrameCount,

dot11FCSErrorCount,

dot11WEPUndecryptableCount,

dot11TransmittedFrameCount,

dot11QosDiscardedFragmentCount,

dot11AssociatedStationCount,

dot11QosCFPollsReceivedCount,

dot11QosCFPollsUnusedCount,

dot11QosCFPollsUnusableCount }

STATUS current

DESCRIPTION

"Attributes from the dot11CountersGroup that are not described

in the dot11MACStatistics group. These objects are mandatory."

::= { dot11Groups 32 }

dot11Qosadditions OBJECT-GROUP

OBJECTS { dot11EDCATable, dot11QAPEDCATable,

dot11QosCountersTable }

STATUS current

DESCRIPTION

"This object class provides the objects from the IEEE 802.11 MIB

required to manage QoS functionality."

::= { dot11Groups 33 }

dot11SMTbase6 OBJECT-GROUP

OBJECTS { dot11MediumOccupancyLimit,

dot11CFPollable,

dot11CFPPeriod,

dot11CFPMaxDuration,

dot11AuthenticationResponseTimeOut,

dot11PrivacyOptionImplemented,

dot11PowerManagementMode,

dot11DesiredSSID, dot11DesiredBSSType,

dot11OperationalRateSet,

dot11BeaconPeriod, dot11DTIMPeriod,

dot11AssociationResponseTimeOut,

dot11DisassociateReason,

dot11DisassociateStation,

dot11DeauthenticateReason,

dot11DeauthenticateStation,

dot11AuthenticateFailStatus,

dot11AuthenticateFailStation,

dot11MultiDomainCapabilityImplemented,

dot11MultiDomainCapabilityEnabled,

dot11CountryString,

dot11RSNAOptionImplemented,

dot11RegulatoryClassesImplemented,

dot11RegulatoryClassesRequired,

dot11QosOptionImplemented,

dot11ImmediateBlockAckOptionImplemented,

dot11DelayedBlockAckOptionImplemented,

dot11DirectOptionImplemented,

dot11APSDOptionImplemented,

dot11QAckOptionImplemented,

dot11QBSSLoadOptionImplemented,

dot11QueueRequestOptionImplemented,

dot11TXOPRequestOptionImplemented,

dot11MoreDataAckOptionImplemented,

dot11AssociateInNQBSS,

dot11DLSAllowedInQBSS,

dot11DLSAllowed }

STATUS deprecated

DESCRIPTION

"The SMTbase6 object class provides the necessary support at the

STA to manage the processes in the STA such that the STA may work

cooperatively as a part of an IEEE 802.11 network."

::= { dot11Groups 34 }

dot11PhyOFDMComplianceGroup2 OBJECT-GROUP

OBJECTS { dot11CurrentFrequency,

dot11TIThreshold,

dot11FrequencyBandsSupported,

dot11ChannelStartingFactor,

dot11FiveMHzOperationImplemented,

dot11TenMHzOperationImplemented,

dot11TwentyMHzOperationImplemented,

dot11PhyOFDMChannelWidth }

STATUS deprecated

DESCRIPTION

"Attributes that configure the OFDM for IEEE 802.11."

::= { dot11Groups 35}

dot11SMTbase7 OBJECT-GROUP

OBJECTS{ dot11MediumOccupancyLimit,

dot11CFPollable,

dot11CFPPeriod,

dot11CFPMaxDuration,

dot11AuthenticationResponseTimeOut,

dot11PrivacyOptionImplemented,

dot11PowerManagementMode,

dot11DesiredSSID,

dot11DesiredBSSType,

dot11OperationalRateSet,

dot11BeaconPeriod,

dot11DTIMPeriod,

dot11AssociationResponseTimeOut,

dot11DisassociateReason,

dot11DisassociateStation,

dot11DeauthenticateReason,

dot11DeauthenticateStation,

dot11AuthenticateFailStatus,

dot11AuthenticateFailStation,

dot11MultiDomainCapabilityImplemented,

dot11MultiDomainCapabilityEnabled,

dot11CountryString,

dot11SpectrumManagementImplemented,

dot11SpectrumManagementRequired,

dot11RSNAOptionImplemented,

dot11RegulatoryClassesImplemented,

dot11RegulatoryClassesRequired,

dot11QosOptionImplemented,

dot11ImmediateBlockAckOptionImplemented,

dot11DelayedBlockAckOptionImplemented,

dot11DirectOptionImplemented,

dot11APSDOptionImplemented,

dot11QAckOptionImplemented,

dot11QBSSLoadOptionImplemented,

dot11QueueRequestOptionImplemented,

dot11TXOPRequestOptionImplemented,

dot11MoreDataAckOptionImplemented,

dot11AssociateInNQBSS,

dot11DLSAllowedInQBSS,

dot11DLSAllowed,

dot11AssociateStation,

dot11AssociateID,

dot11AssociateFailStation,

dot11AssociateFailStatus,

dot11ReassociateStation,

dot11ReassociateID,

dot11ReassociateFailStation,

dot11ReassociateFailStatus,

dot11RadioMeasurementCapable,

dot11RadioMeasurementEnabled,

dot11RMMeasurementProbeDelay,

dot11RMMeasurementPilotPeriod,

dot11RMLinkMeasurementEnabled,

dot11RMNeighborReportEnabled,

dot11RMParallelMeasurementsEnabled,

dot11RMRepeatedMeasurementsEnabled,

dot11RMBeaconPassiveMeasurementEnabled,

dot11RMBeaconActiveMeasurementEnabled,

dot11RMBeaconTableMeasurementEnabled,

dot11RMBeaconMeasurementReportingConditionsEnabled,

dot11RMFrameMeasurementEnabled,

dot11RMChannelLoadMeasurementEnabled,

dot11RMNoiseHistogramMeasurementEnabled,

dot11RMStatisticsMeasurementEnabled,

dot11RMLCIMeasurementEnabled,

dot11RMLCIAzimuthEnabled,

dot11RMTransmitStreamCategoryMeasurementEnabled,

dot11RMTriggeredTransmitStreamCategoryMeasurementEnabled,

dot11RMAPChannelReportEnabled,

dot11RMMIBEnabled,

dot11RMMaxMeasurementDuration,

dot11RMNonOperatingChannelMaxMeasurementDuration,

dot11RMMeasurementPilotTransmissionInformationEnabled,

dot11RMMeasurementPilotCapability,

dot11RMNeighborReportTSFOffsetEnabled,

dot11RMRCPIMeasurementEnabled,

dot11RMRSNIMeasurementEnabled,

dot11RMBSSAverageAccessDelayEnabled,

dot11RMBSSAvailableAdmissionCapacityEnabled,

dot11RMAntennaInformationEnabled}

STATUS deprecated

DESCRIPTION

"The SMTbase7 object class provides the necessary support at the

STA to manage the processes in the STA such that the STA may work

cooperatively as a part of an IEEE 802.11 net-work, when the STA

is capable of multi-domain operation. This object group should be

implemented when the multi-domain capability option is imple-

mented."

::= { dot11Groups 36 }

dot11FTComplianceGroup OBJECT-GROUP

OBJECTS { dot11FastBSSTransitionEnabled }

STATUS current

DESCRIPTION

"This object class provides the objects from the IEEE 802.11 MIB

required to manage fast BSS transition functionality. Note that

additional objects for managing this functionality are located in

the dot11FastBSS TransitionConfigTable."

::= { dot11Groups 40}

dot11SMTbase8 OBJECT-GROUP

OBJECTS {	dot11MediumOccupancyLimit,

dot11CFPollable,

dot11CFPPeriod,

dot11CFPMaxDuration,

dot11AuthenticationResponseTimeOut,

dot11PrivacyOptionImplemented,

dot11PowerManagementMode,

dot11DesiredSSID, dot11DesiredBSSType,

dot11OperationalRateSet,

dot11BeaconPeriod, dot11DTIMPeriod,

dot11AssociationResponseTimeOut,

dot11DisassociateReason,

dot11DisassociateStation,

dot11DeauthenticateReason,

dot11DeauthenticateStation,

dot11AuthenticateFailStatus,

dot11AuthenticateFailStation,

dot11MultiDomainCapabilityImplemented,

dot11MultiDomainCapabilityEnabled,

dot11CountryString,

dot11SpectrumManagementImplemented,

dot11SpectrumManagementRequired,

dot11RSNAOptionImplemented,

dot11RegulatoryClassesImplemented,

dot11RegulatoryClassesRequired,

dot11QosOptionImplemented,

dot11ImmediateBlockAckOptionImplemented,

dot11DelayedBlockAckOptionImplemented,

dot11DirectOptionImplemented,

dot11APSDOptionImplemented,

dot11QAckOptionImplemented,

dot11QBSSLoadOptionImplemented,

dot11QueueRequestOptionImplemented,

dot11TXOPRequestOptionImplemented,

dot11MoreDataAckOptionImplemented,

dot11AssociateInNQBSS,

dot11DLSAllowedInQBSS,

dot11DLSAllowed,

dot11AssociateStation,

dot11AssociateID,

dot11AssociateFailStation,

dot11AssociateFailStatus,

dot11ReassociateStation,

dot11ReassociateID,

dot11ReassociateFailStation,

dot11ReassociateFailStatus,

dot11RadioMeasurementCapable,

dot11RadioMeasurementEnabled,

dot11RMMeasurementProbeDelay,

dot11RMMeasurementPilotPeriod,

dot11RMLinkMeasurementEnabled,

dot11RMNeighborReportEnabled,

dot11RMParallelMeasurementsEnabled,

dot11RMRepeatedMeasurementsEnabled,

dot11RMBeaconPassiveMeasurementEnabled,

dot11RMBeaconActiveMeasurementEnabled,

dot11RMBeaconTableMeasurementEnabled,

dot11RMBeaconMeasurementReportingConditionsEnabled,

dot11RMFrameMeasurementEnabled,

dot11RMChannelLoadMeasurementEnabled,

dot11RMNoiseHistogramMeasurementEnabled,

dot11RMStatisticsMeasurementEnabled,

dot11RMLCIMeasurementEnabled,

dot11RMLCIAzimuthEnabled,

dot11RMTransmitStreamCategoryMeasurementEnabled,

dot11RMTriggeredTransmitStreamCategoryMeasurementEnabled,

dot11RMAPChannelReportEnabled,

dot11RMMIBEnabled,

dot11RMMaxMeasurementDuration,

dot11RMNonOperatingChannelMaxMeasurementDuration,

dot11RMMeasurementPilotTransmissionInformationEnabled,

dot11RMMeasurementPilotCapability,

dot11RMNeighborReportTSFOffsetEnabled,

dot11RMRCPIMeasurementEnabled,

dot11RMRSNIMeasurementEnabled,

dot11RMBSSAverageAccessDelayEnabled,

dot11RMBSSAvailableAdmissionCapacityEnabled,

dot11RMAntennaInformationEnabled,

dot11FastBSSTransitionImplemented }

STATUS deprecated

DESCRIPTION

"The SMTbase8 object class provides the necessary support at the

STA to manage the processes in the STA so that the STA may work

cooperatively as a part of an IEEE 802.11 network, when the STA is

capable of multidomain operation. This object group should be

implemented when the multidomain capability option is imple-

mented."

::= { dot11Groups 41 }

dot11PhyOFDMComplianceGroup3 OBJECT-GROUP

OBJECTS { dot11CurrentFrequency,

dot11FrequencyBandsSupported,

dot11ChannelStartingFactor,

dot11FiveMHzOperationImplemented,

dot11TenMHzOperationImplemented,

dot11TwentyMHzOperationImplemented,

dot11PhyOFDMChannelWidth,

dot11OFDMCCAEDImplemented,

dot11OFDMCCAEDRequired,

dot11OFDMEDThreshold }

STATUS current

DESCRIPTION

"Attributes that configure the OFDM for IEEE 802.11."

::= { dot11Groups 42}

dot11SMTbase9 OBJECT-GROUP

OBJECTS { dot11MediumOccupancyLimit,

dot11CFPollable,

dot11CFPPeriod,

dot11CFPMaxDuration,

dot11AuthenticationResponseTimeOut,

dot11PrivacyOptionImplemented,

dot11PowerManagementMode,

dot11DesiredSSID, dot11DesiredBSSType,

dot11OperationalRateSet,

dot11BeaconPeriod, dot11DTIMPeriod,

dot11AssociationResponseTimeOut,

dot11DisassociateReason,

dot11DisassociateStation,

dot11DeauthenticateReason,

dot11DeauthenticateStation,

dot11AuthenticateFailStatus,

dot11AuthenticateFailStation,

dot11MultiDomainCapabilityImplemented,

dot11MultiDomainCapabilityEnabled,

dot11CountryString,

dot11RSNAOptionImplemented,

dot11RegulatoryClassesImplemented,

dot11RegulatoryClassesRequired,

dot11QosOptionImplemented,

dot11ImmediateBlockAckOptionImplemented,

dot11DelayedBlockAckOptionImplemented,

dot11DirectOptionImplemented,

dot11APSDOptionImplemented,

dot11QAckOptionImplemented,

dot11QBSSLoadOptionImplemented,

dot11QueueRequestOptionImplemented,

dot11TXOPRequestOptionImplemented,

dot11MoreDataAckOptionImplemented,

dot11AssociateInNQBSS,

dot11DLSAllowedInQBSS,

dot11DLSAllowed,

dot11RadioMeasurementCapable,

dot11RadioMeasurementEnabled,

dot11FastBSSTransitionImplemented,

dot11LCIDSEImplemented,

dot11LCIDSERequired,

dot11DSERequired,

dot11ExtendedChannelSwitchEnabled }

STATUS current

DESCRIPTION

"The SMTbase9 object class provides the necessary support at the

STA to manage the processes in the STA such that the STA may work

cooperatively as a part of an IEEE 802.11 network, when the STA is

capable of multi-domain operation. This object group should be

implemented when the multi-domain capability option is imple-

mented."

::= { dot11Groups 43 }

-- ********************************************************************

-- * dot11RSNAConfig TABLE (RSNA and TSN)

-- ********************************************************************

dot11RSNAConfigTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11RSNAConfigEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The table containing RSNA configuration objects."

::= { dot11smt 9 }

dot11RSNAConfigEntry OBJECT-TYPE

SYNTAX Dot11RSNAConfigEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11RSNAConfigTable."

INDEX { ifIndex }

::= { dot11RSNAConfigTable 1 }

Dot11RSNAConfigEntry ::=

SEQUENCE {

dot11RSNAConfigVersion				 			Integer32,

dot11RSNAConfigPairwiseKeysSupported						 	Unsigned32,

dot11RSNAConfigGroupCipher				 			OCTET STRING,

dot11RSNAConfigGroupRekeyMethod					 		INTEGER,

dot11RSNAConfigGroupRekeyTime					 		Unsigned32,

dot11RSNAConfigGroupRekeyPackets					 		Unsigned32,

dot11RSNAConfigGroupRekeyStrict				 			TruthValue,

dot11RSNAConfigPSKValue				 			OCTET STRING,

dot11RSNAConfigPSKPassPhrase					 		DisplayString,

dot11RSNAConfigGroupUpdateCount					 		Unsigned32,

dot11RSNAConfigPairwiseUpdateCount						 	Unsigned32,

dot11RSNAConfigGroupCipherSize					 		Unsigned32,

dot11RSNAConfigPMKLifetime				 			Unsigned32,

dot11RSNAConfigPMKReauthThreshold					 		Unsigned32,

dot11RSNAConfigNumberOfPTKSAReplayCounters						 	INTEGER,

dot11RSNAConfigSATimeout				 			Unsigned32,

dot11RSNAAuthenticationSuiteSelected						 	OCTET STRING,

dot11RSNAPairwiseCipherSelected				 			OCTET STRING,

dot11RSNAGroupCipherSelected					 		OCTET STRING,

dot11RSNAPMKIDUsed			 				OCTET STRING,

dot11RSNAAuthenticationSuiteRequested						 	OCTET STRING,

dot11RSNAPairwiseCipherRequested					 		OCTET STRING,

dot11RSNAGroupCipherRequested					 		OCTET STRING,

dot11RSNATKIPCounterMeasuresInvoked				 			Unsigned32,

dot11RSNA4WayHandshakeFailures				 			Unsigned32,

dot11RSNAConfigNumberOfGTKSAReplayCounters						 	INTEGER,

dot11RSNAConfigSTKKeysSupported							Unsigned32,

dot11RSNAConfigSTKCipher 							OCTET STRING,

dot11RSNAConfigSTKRekeyTime 							Unsigned32,

dot11RSNAConfigSMKUpdateCount 							Unsigned32,

dot11RSNAConfigSTKCipherSize 							Unsigned32,

dot11RSNAConfigSMKLifetime 							Unsigned32,

dot11RSNAConfigSMKReauthThreshold 							Unsigned32,

dot11RSNAConfigNumberOfSTKSAReplayCounters 							INTEGER,

dot11RSNAPairwiseSTKSelected 							OCTET STRING,

dot11RSNASMKHandshakeFailures 							Unsigned32 }

-- dot11RSNAConfigEntry 1 has been deprecated.

dot11RSNAConfigVersion OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The highest RSNA version this entity supports. See 7.3.2.9."

::= { dot11RSNAConfigEntry 2 }

dot11RSNAConfigPairwiseKeysSupported OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This object indicates how many pairwise keys the entity supports

for RSNA."

::= { dot11RSNAConfigEntry 3 }

dot11RSNAConfigGroupCipher OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(4))

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This object indicates the group cipher suite selector the entity

must use. The group cipher suite in the RSN  element shall take its

value from this variable. It consists of an OUI (the first 3

octets) and a cipher suite identifier (the last octet)."

::= { dot11RSNAConfigEntry 4 }

dot11RSNAConfigGroupRekeyMethod OBJECT-TYPE

SYNTAX INTEGER { disabled(1), timeBased(2), packetBased(3), timepacket-

Based(4) }

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This object selects a mechanism for rekeying the RSNA GTK. The

default is time-based, once per day. Rekeying the GTK is only

applicable to an entity acting in the Authenticator role (an AP in

an ESS)."

DEFVAL { timeBased }

::= { dot11RSNAConfigEntry 5 }

dot11RSNAConfigGroupRekeyTime OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

UNITS "seconds"

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The time in seconds after which the RSNA GTK shall be refreshed.

The timer shall start at the moment the GTK was set using the MLME-

SETKEYS.request primitive."

DEFVAL { 86400 } -- once per day

::= { dot11RSNAConfigEntry 6 }

dot11RSNAConfigGroupRekeyPackets OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

UNITS "1000 packets"

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"A packet count (in 1000s of packets) after which the RSNA GTK

shall be refreshed. The packet counter shall start at the moment

the GTK was set using the MLME-SETKEYS.request primitive and it

shall count all packets encrypted using the current GTK."

::= { dot11RSNAConfigEntry 7 }

dot11RSNAConfigGroupRekeyStrict OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This object signals that the GTK shall be refreshed whenever a STA

leaves the BSS that possesses the GTK."

::= { dot11RSNAConfigEntry 8 }

dot11RSNAConfigPSKValue OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(32))

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The PSK for when RSNA in PSK mode is the selected AKM suite. In

that case, the PMK will obtain its value from this object.

This object is logically write-only. Reading this variable shall

return unsuccessful status or null or zero."

::= { dot11RSNAConfigEntry 9 }

dot11RSNAConfigPSKPassPhrase OBJECT-TYPE

SYNTAX DisplayString

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The PSK, for when RSNA in PSK mode is the selected AKM suite, is

configured by dot11RSNAConfigPSKValue.

An alternative manner of setting the PSK uses the password-to-key

algorithm defined in H.4. This variable provides a means to enter a

pass-phrase. When this object is written, the RSNA entity shall use

the password-to-key algorithm specified in H.4 to derive a pre-

shared and populate dot11RSNAConfigPSKValue with this key.

This object is logically write-only. Reading this variable shall

return unsuccessful status or null or zero."

::= { dot11RSNAConfigEntry 10 }

-- dot11RSNAConfigEntry 11 and dot11RSNAConfigEntry 12 have been

-- deprecated.

dot11RSNAConfigGroupUpdateCount OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The number of times Message 1 in the RSNA Group Key Handshake will

be retried per GTK Handshake attempt."

DEFVAL { 3 }

::= { dot11RSNAConfigEntry 13 }

dot11RSNAConfigPairwiseUpdateCount OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The number of times Message 1 and Message 3 in the RSNA 4-Way Hand-

shake will be retried per 4-Way Handshake attempt."

DEFVAL { 3 }

::= { dot11RSNAConfigEntry 14 }

dot11RSNAConfigGroupCipherSize OBJECT-TYPE

SYNTAX Unsigned32 (0..4294967295)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This object indicates the length in bits of the group cipher key."

::= { dot11RSNAConfigEntry 15 }

dot11RSNAConfigPMKLifetime OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

UNITS "seconds"

MAX-ACCESS	read-write

STATUS current

DESCRIPTION

"The maximum lifetime of a PMK in the PMK cache."

DEFVAL { 43200 }

::= { dot11RSNAConfigEntry 16 }

dot11RSNAConfigPMKReauthThreshold OBJECT-TYPE

SYNTAX Unsigned32 (1..100)

UNITS "percentage"

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The percentage of the PMK lifetime that should expire before an

IEEE 802.1X reauthentication occurs."

DEFVAL { 70 }

::= { dot11RSNAConfigEntry 17 }

dot11RSNAConfigNumberOfPTKSAReplayCounters OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Specifies the number of PTKSA replay counters per association:

0 –> 1 replay counter,

1 –> 2 replay counters,

2 –> 4 replay counters,

3 –> 16 replay counters"

::= { dot11RSNAConfigEntry 18 }

dot11RSNAConfigSATimeout OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

UNITS "seconds"

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The maximum time a security association shall take to set up."

DEFVAL { 60 }

::= { dot11RSNAConfigEntry 19 }

dot11RSNAAuthenticationSuiteSelected OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(4))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The selector of the last AKM suite negotiated."

::= { dot11RSNAConfigEntry 20 }

dot11RSNAPairwiseCipherSelected OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(4))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The selector of the last pairwise cipher negotiated."

::= { dot11RSNAConfigEntry 21 }

dot11RSNAGroupCipherSelected OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(4))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The selector of the last group cipher negotiated."

::= { dot11RSNAConfigEntry 22 }

dot11RSNAPMKIDUsed OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(16))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The selector of the last PMKID used in the last 4-Way Handshake."

::= { dot11RSNAConfigEntry 23 }

dot11RSNAAuthenticationSuiteRequested OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(4))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The selector of the last AKM suite requested."

::= { dot11RSNAConfigEntry 24 }

dot11RSNAPairwiseCipherRequested OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(4))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The selector of the last pairwise cipher requested."

::= { dot11RSNAConfigEntry 25 }

dot11RSNAGroupCipherRequested OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(4))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The selector of the last group cipher requested."

::= { dot11RSNAConfigEntry 26 }

dot11RSNATKIPCounterMeasuresInvoked OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Counts the number of times that a TKIP MIC failure occurred two

times within 60 s and TKIP countermeasures were invoked. This

attribute counts both local and remote MIC failure events reported

to this STA. It increments every time TKIP countermeasures are

invoked"

::= { dot11RSNAConfigEntry 27 }

dot11RSNA4WayHandshakeFailures OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Counts the number of 4-Way Handshake failures."

::= { dot11RSNAConfigEntry 28 }

dot11RSNAConfigNumberOfGTKSAReplayCounters OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Specifies the number of GTKSA replay counters per association:

0 –> 1 replay counter,

1 –> 2 replay counters,

2 –> 4 replay counters,

3 –> 16 replay counters"

::= { dot11RSNAConfigEntry 29 }

dot11RSNAConfigSTKKeysSupported OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This object indicates how many STK keys the entity supports for

RSNA."

::= { dot11RSNAConfigEntry 30 }

dot11RSNAConfigSTKCipher OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(4))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This object specifies the ciphersuite used by the STK for a DLS

link."

::= { dot11RSNAConfigEntry 31}

dot11RSNAConfigSTKRekeyTime OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

UNITS "seconds"

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The time in seconds after which an RSNA STK shall be refreshed.

The timer shall start at the moment the STK was set using the MLM-

ESETKEYS. request primitive."

DEFVAL { 86400 } -- once per day

::= { dot11RSNAConfigEntry 32 }

dot11RSNAConfigSMKUpdateCount OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The number of times Message 1 in the RSNA SMK Handshake will be

retried per SMK Handshake attempt."

DEFVAL { 3 }

::= { dot11RSNAConfigEntry 33 }

dot11RSNAConfigSTKCipherSize OBJECT-TYPE

SYNTAX Unsigned32 (0..4294967295)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This object indicates the length in bits of the STK cipher key."

::= { dot11RSNAConfigEntry 34 }

dot11RSNAConfigSMKLifetime OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

UNITS "seconds"

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The maximum lifetime of an SMK in the SMK cache."

DEFVAL { 43200 }

::= { dot11RSNAConfigEntry 35 }

dot11RSNAConfigSMKReauthThreshold OBJECT-TYPE

SYNTAX Unsigned32 (0..4294967295)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the number of seconds for which an SMK

authentication is valid. A new SMK authentication must be completed

successfully before the number of seconds indicated by this

attribute elapse, from the prior authentication, before the STAs

become unauthenticated."

::= { dot11RSNAConfigEntry 36 }

dot11RSNAConfigNumberOfSTKSAReplayCounters OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Specifies the number of STKSA replay counters per association:

0 -> 1 replay counter,

1 -> 2 replay counters,

2 -> 4 replay counters,

3 -> 16 replay counters"

::= { dot11RSNAConfigEntry 37 }

dot11RSNAPairwiseSTKSelected OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(4))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The selector of the last STK cipher negotiated."

::= { dot11RSNAConfigEntry 38 }

dot11RSNASMKHandshakeFailures OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Counts the number of SMK Handshake failures."

::= { dot11RSNAConfigEntry 39 }

-- ********************************************************************

-- * End of dot11RSNAConfig TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11RSNAConfigPairwiseCiphers TABLE

-- ********************************************************************

dot11RSNAConfigPairwiseCiphersTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11RSNAConfigPairwiseCiphersEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This table lists the pairwise ciphers supported by this entity. It

allows enabling and disabling of each pairwise cipher by network

management. The pairwise cipher suite list in the RSN element is

formed using the information in this table."

::= { dot11smt 10 }

dot11RSNAConfigPairwiseCiphersEntry OBJECT-TYPE

SYNTAX Dot11RSNAConfigPairwiseCiphersEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The table entry, indexed by the interface index (or all inter-

faces) and the pairwise cipher."

--*  dot11RSNAStatsIndex appears not to be defined or used anywhere, so remove it

--*  INDEX { dot11RSNAConfigIndex, dot11RSNAConfigPairwiseCipherIndex }

INDEX { dot11RSNAConfigPairwiseCipherIndex }

::= { dot11RSNAConfigPairwiseCiphersTable 1 }

Dot11RSNAConfigPairwiseCiphersEntry ::=

SEQUENCE {

dot11RSNAConfigPairwiseCipherIndex						Unsigned32,

dot11RSNAConfigPairwiseCipher						OCTET STRING,

dot11RSNAConfigPairwiseCipherEnabled						TruthValue,

dot11RSNAConfigPairwiseCipherSize						Unsigned32  }

dot11RSNAConfigPairwiseCipherIndex OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The auxiliary index into the dot11RSNAConfigPairwiseCiphersTable."

::= { dot11RSNAConfigPairwiseCiphersEntry 1 }

dot11RSNAConfigPairwiseCipher OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(4))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The selector of a supported pairwise cipher. It consists of an OUI

(the first 3 octets) and a cipher suite identifier (the last

octet)."

::= { dot11RSNAConfigPairwiseCiphersEntry 2 }

dot11RSNAConfigPairwiseCipherEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This object enables or disables the pairwise cipher."

::= { dot11RSNAConfigPairwiseCiphersEntry 3 }

dot11RSNAConfigPairwiseCipherSize OBJECT-TYPE

SYNTAX Unsigned32 (0..4294967295)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This object indicates the length in bits of the pairwise cipher

key. This should be 256 for TKIP and 128 for CCMP."

::= { dot11RSNAConfigPairwiseCiphersEntry 4 }

-- ********************************************************************

-- * End of dot11RSNAConfigPairwiseCiphers TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11RSNAConfigAuthenticationSuites TABLE

-- ********************************************************************

dot11RSNAConfigAuthenticationSuitesTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11RSNAConfigAuthenticationSuitesEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This table lists the AKM suites supported by this entity. Each AKM

suite can be individually enabled and disabled. The AKM suite list

in the RSN element is formed using the information in this table."

::= { dot11smt 11 }

dot11RSNAConfigAuthenticationSuitesEntry OBJECT-TYPE

SYNTAX Dot11RSNAConfigAuthenticationSuitesEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry (row) in the dot11RSNAConfigAuthenticationSuitesTable."

INDEX { dot11RSNAConfigAuthenticationSuiteIndex }

::= { dot11RSNAConfigAuthenticationSuitesTable 1 }

Dot11RSNAConfigAuthenticationSuitesEntry ::=

SEQUENCE {

dot11RSNAConfigAuthenticationSuiteIndex							Unsigned32,

dot11RSNAConfigAuthenticationSuite							OCTET STRING,

dot11RSNAConfigAuthenticationSuiteEnabled							TruthValue }

dot11RSNAConfigAuthenticationSuiteIndex OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The auxiliary variable used as an index into the

dot11RSNAConfigAuthenticationSuitesTable."

::= { dot11RSNAConfigAuthenticationSuitesEntry 1 }

dot11RSNAConfigAuthenticationSuite OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(4))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The selector of an AKM suite. It consists of an OUI (the first 3

octets) and a cipher suite identifier (the last octet)."

::= { dot11RSNAConfigAuthenticationSuitesEntry 2 }

dot11RSNAConfigAuthenticationSuiteEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This variable indicates whether the corresponding AKM suite is

enabled/disabled."

::= { dot11RSNAConfigAuthenticationSuitesEntry 3 }

-- ********************************************************************

-- * End of dot11RSNAConfigAuthenticationSuites TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11RSNAStats TABLE

-- ********************************************************************

dot11RSNAStatsTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11RSNAStatsEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This table maintains per-STA statistics in an RSN. The entry with

dot11RSNAStatsSTAAddress set to FF-FF-FF-FF-FF-FF shall contain

statistics for broadcast/multicast traffic."

::= { dot11smt 12 }

dot11RSNAStatsEntry OBJECT-TYPE

SYNTAX Dot11RSNAStatsEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11RSNAStatsTable."

--* dot11RSNAStatsIndex appears not to be defined or used anywhere, so remove it

--* INDEX { dot11RSNAConfigIndex, dot11RSNAStatsIndex }

INDEX { dot11RSNAStatsIndex }

::= { dot11RSNAStatsTable 1 }

Dot11RSNAStatsEntry ::=

SEQUENCE {

dot11RSNAStatsIndex						Unsigned32,

dot11RSNAStatsSTAAddress						MacAddress,

dot11RSNAStatsVersion						Unsigned32,

dot11RSNAStatsSelectedPairwiseCipher						OCTET STRING,

dot11RSNAStatsTKIPICVErrors						Counter32,

dot11RSNAStatsTKIPLocalMICFailures						Counter32,

dot11RSNAStatsTKIPRemoteMICFailures						Counter32,

dot11RSNAStatsCCMPReplays						Counter32,

dot11RSNAStatsCCMPDecryptErrors						Counter32,

dot11RSNAStatsTKIPReplays						Counter32 }

dot11RSNAStatsIndex OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An auxiliary index into the dot11RSNAStatsTable."

::= { dot11RSNAStatsEntry 1 }

dot11RSNAStatsSTAAddress OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The MAC address of the STA to which the statistics in this

conceptual row belong."

::= { dot11RSNAStatsEntry 2 }

dot11RSNAStatsVersion OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The RSNA version with which the STA associated."

::= { dot11RSNAStatsEntry 3 }

dot11RSNAStatsSelectedPairwiseCipher OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(4))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The pairwise cipher suite Selector (as defined in 7.3.29.1) used

during association, in transmission order."

::= { dot11RSNAStatsEntry 4 }

dot11RSNAStatsTKIPICVErrors OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Counts the number of TKIP ICV errors encountered when decrypting

packets for the STA."

::= { dot11RSNAStatsEntry 5 }

dot11RSNAStatsTKIPLocalMICFailures OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Counts the number of MIC failures encountered when checking the

integrity of packets received from the STA at this entity."

::= { dot11RSNAStatsEntry 6 }

dot11RSNAStatsTKIPRemoteMICFailures OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Counts the number of MIC failures encountered by the STA identi-

fied by dot11StatsSTAAddress and reported back to this entity."

::= { dot11RSNAStatsEntry 7 }

dot11RSNAStatsCCMPReplays OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The number of received CCMP MPDUs discarded by the replay

mechanism."

::= { dot11RSNAStatsEntry 8 }

dot11RSNAStatsCCMPDecryptErrors OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The number of received MPDUs discarded by the CCMP decryption

algorithm."

::= { dot11RSNAStatsEntry 9 }

dot11RSNAStatsTKIPReplays OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Counts the number of TKIP replay errors detected."

::= { dot11RSNAStatsEntry 10 }

-- ********************************************************************

-- * End of dot11RSNAStats TABLE

-- ********************************************************************

-- ********************************************************************

-- * Conformance information - RSN

-- ********************************************************************

-- ********************************************************************

-- * Compliance Statements - RSN

-- ********************************************************************

dot11RSNCompliance MODULE-COMPLIANCE

STATUS current

DESCRIPTION

"The compliance statement for SNMPv2 entities that implement the

IEEE 802.11 RSN MIB."

MODULE -- this module

MANDATORY-GROUPS {

dot11RSNBase }

-- OPTIONAL-GROUPS {dot11RSNPMKcachingGroup }

::= { dot11Compliances 2 }

-- ********************************************************************

-- * Groups - units of conformance - RSN

-- ********************************************************************

dot11RSNBase OBJECT-GROUP

OBJECTS {

dot11RSNAConfigVersion,

dot11RSNAConfigPairwiseKeysSupported,

dot11RSNAConfigGroupCipher,

dot11RSNAConfigGroupRekeyMethod,

dot11RSNAConfigGroupRekeyTime,

dot11RSNAConfigGroupRekeyPackets,

dot11RSNAConfigGroupRekeyStrict,

dot11RSNAConfigPSKValue,

dot11RSNAConfigPSKPassPhrase,

dot11RSNAConfigGroupUpdateCount,

dot11RSNAConfigPairwiseUpdateCount,

dot11RSNAConfigGroupCipherSize,

dot11RSNAConfigPairwiseCipher,

dot11RSNAConfigPairwiseCipherEnabled,

dot11RSNAConfigPairwiseCipherSize,

dot11RSNAConfigAuthenticationSuite,

dot11RSNAConfigAuthenticationSuiteEnabled,

dot11RSNAConfigNumberOfPTKSAReplayCounters,

dot11RSNAConfigSATimeout,

dot11RSNAConfigNumberOfGTKSAReplayCounters,

dot11RSNAAuthenticationSuiteSelected,

dot11RSNAPairwiseCipherSelected,

dot11RSNAGroupCipherSelected,

dot11RSNAPMKIDUsed,

dot11RSNAAuthenticationSuiteRequested,

dot11RSNAPairwiseCipherRequested,

dot11RSNAGroupCipherRequested,

dot11RSNAStatsSTAAddress,

dot11RSNAStatsVersion,

dot11RSNAStatsSelectedPairwiseCipher,

dot11RSNAStatsTKIPICVErrors,

dot11RSNAStatsTKIPLocalMICFailures,

dot11RSNAStatsTKIPRemoteMICFailures,

--* not defined or used anywhere

--		dot11RSNAStatsTKIPCounterMeasuresInvoked,

dot11RSNAStatsCCMPReplays,

dot11RSNAStatsCCMPDecryptErrors,

dot11RSNAStatsTKIPReplays,

--* not defined or used anywhere

--		dot11RSNAStats4WayHandshakeFailures,

dot11RSNAConfigSTKKeysSupported,

dot11RSNAConfigSTKCipher,

dot11RSNAConfigSTKRekeyTime,

--* not defined or used anywhere

--		dot11RSNAConfigSTKUpdateCount,

dot11RSNAConfigSTKCipherSize,

dot11RSNAConfigNumberOfSTKSAReplayCounters,

dot11RSNAPairwiseSTKSelected,

dot11RSNASMKHandshakeFailures }

STATUS current

DESCRIPTION

"The dot11RSNBase object class provides the necessary support for

managing RSNA functionality in the STA."

::= { dot11Groups 27 }

dot11RSNPMKcachingGroup OBJECT-GROUP

OBJECTS {

dot11RSNAConfigPMKLifetime,

dot11RSNAConfigPMKReauthThreshold

}

STATUS current

DESCRIPTION

"The dot11RSNPMKcachingGroup object class provides the necessary

support for managing PMK caching functionality in the STA"

::= { dot11Groups 28 }

dot11RSNSMKcachingGroup OBJECT-GROUP

OBJECTS {

dot11RSNAConfigSMKLifetime,

dot11RSNAConfigSMKReauthThreshold

}

STATUS current

DESCRIPTION

"The dot11RSNSMKcachingGroup object class provides the necessary

support for managing SMK caching functionality in the STA."

::= { dot11Groups 29 }

-- ********************************************************************

-- * Compliance Statements - RM

-- ********************************************************************

dot11RMCompliance MODULE-COMPLIANCE

STATUS current

DESCRIPTION

"The compliance statement for SNMPv2 entities that implement the

IEEE 802.11 MIB for Measurement Services."

MODULE -- this module

MANDATORY-GROUPS {

dot11SMTRMRequest,

dot11SMTRMReport,

dot11SMTRMConfig

}

-- OPTIONAL-GROUPS { }

::= { dot11Compliances 3 }

-- **********************************************************************

-- *   End of 802.11 MIB

-- **********************************************************************

-- Annex Q

-- (normative)

-- ASN.1 encoding of the Radio Measurements (RM) MIB

-- **********************************************************************

-- * IEEE 802.11 RM MIB

-- **************************************************************

-- * The primary interface to the Radio Measurements is meant to be

-- * real-time information obtained through the request/response mechanisms of

-- * RM.  A secondary interface to the measurements is through retention of

-- * information in the MIB.  The information, meant to be retained for later

-- * access, includes the MIB entries of Annex Q.  Non-SNMP requests for infor-

-- * mation are obtained via object IDs (OIDs) through the NDIS or “wireless”

-- * interfaces in the operating systems.  SNMP requests for information are

-- * obtained via SNMP SETs and GETs.

-- ********************************************************************

-- * Radio Measurement

-- ********************************************************************

dot11RadioResourceMeasurement OBJECT IDENTIFIER ::= { dot11smt 14 }

-- **********************************************************************

-- *  dot11RMRequest and dot11RMReport Usage

-- *

-- * The dot11RMRequest and dot11RMReport portions of the RM MIB

-- * provide access to the Radio Measurement service. By performing

-- * SET operations on the various dot11RMRequest MIB objects,

-- * radio measurements may be initiated directly on the local STA or

-- * on any peer station within the same BSS. Subsequently, by

-- * performing GET operations on the various dot11RMReport MIB

-- * objects the results of the requested measurements may be

-- * retrieved.

-- *

-- * In the diagram below, a radio measurement could be initiated

-- * for STA x by performing a MIB.set operation on the RM MIB of

-- * STA x and specifying the MAC address of STA x in

-- * dot11RMRqstTargetAdd. Additionally, it is possible to have STA x

-- * request a measurement from STA y by performing a MIB.set operation

-- * on the SME MIB of STA x and specifying the MAC address of STA y in

-- * dot11RMRqstTargetAdd. In both cases the result of the measurements

-- * can be retrieved by performing a MIB.get operation on the RM MIB

-- * of STA x upon completion of the measurement.

-- *

-- *

-- *          MIB.Set                                        MIB.Set

-- *             or                                             or

-- *          MIB.Get                                        MIB.Get

-- *    +--------|---------+                           +--------|---------+

-- *    | SME    |         |                           | SME    |         |

-- *    |       \ /        |                           |       \ /        |

-- *    |   +---------+    |                           |   +---------+    |

-- *    |   | RM MIB  |    |                           |   | RM MIB  |    |

-- *    |   |         |    |                           |   |         |    |

-- *    |   |         |    |                           |   |         |    |

-- *    |   +---------+    |                           |   +---------+    |

-- *    |                  |                           |                  |

-- *    |   / \            |                           |   / \            |

-- *    |    |   MREQUEST  |                           |    |   MREQUEST  |

-- *    +----+-------------+                           +----+-------------+

-- *    |    |   MREPORT   |                           |    |   MREPORT   |

-- *    |   \ /  MEASURE   |     Action Frames         |   \ /  MEASURE   |

-- *    |                  | <--Measurement Request--> |                  |

-- *    |                  | <--Measurement Report---> |                  |

-- *    | MLME             |                           | MLME             |

-- *    +------------------+                           +------------------+

-- *           STA x                                          STA y

-- *

-- *

-- * Each STA maintains a single dot11RMRequestTable in the SME MIB

-- * used to initiate RM Measurement Requests. Each dot11RMRequestEntry

-- * in the table represents an individual Measurement Request that

-- * makes up a complete Measurement Request Action frame.

-- * Multiple Measurement Requests may be concatenated into a single

-- * Measurement Request Action frame by setting the same

-- * dot11RMRqstToken value into multiple dot11RMRequestEntrys.

-- *

-- * Each row, dot11RMRequestEntry, of the dot11RMRequestTable

-- * provides read-create access for the initiation of a measurement

-- * request. The dot11RMRequestNextIndex object can be used to

-- * determine which is the next available row. Each row corresponding to

-- * one measurement in the sequence is created with a dot11RMRqstRowStatus

-- * set to notInService. Once the dot11RMRequestEntry(s) have been

-- * created for a desired measurement sequence the corresponding

-- * dot11RMRqstRowStatus(s) objects are set to active to indicate that

-- * the SME can trigger the appropriate MLME primitives. Upon processing

-- * the request, the SME returns the corresponding dot11RMRqstRowStatus(s)

-- * object to notInService and are now available for additional

-- * measurement requests.

-- *

-- * After a radio measurement is complete the RM populates the RMReport

-- * objects with the results of the measurement. Each STA maintains a set

-- * of RMReport tables, one corresponding to each measurement type. The

-- * results of the entire measurement sequence are spread across the tables

-- * based on the type of measurements requested. Each xxxReportEntry

-- * within a xxxReportTable contains a xxxRprtRqstToken that corresponds

-- * to the original dot11RMRqstToken in the measurement request. So the

-- * results of the measurement can be collected by searching the appropriate

-- * xxxReportTables and retrieve any reports with the matching request

-- * token.

-- **********************************************************************

-- ********************************************************************

-- * Radio Measurement Requests

-- ********************************************************************

dot11RMRequest OBJECT IDENTIFIER ::= { dot11RadioResourceMeasurement 1 }

-- ********************************************************************

-- * dot11RMRequest TABLE

-- ********************************************************************

dot11RMRequestNextIndex OBJECT-TYPE

SYNTAX Unsigned32(0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Identifies a hint for the next value of dot11RMRqstIndex to be

used in a row creation attempt for dot11RMRequestTable. If no new

rows can be created for some reason, such as memory, processing

requirements, etc., the SME shall set this attribute to 0. It

shall update this attribute to a proper value other than 0 as soon

as it is capable of receiving new measurement requests. The nex-

tIndex is not necessarily sequential nor monotonically increas-

ing."

::= { dot11RMRequest 1 }

dot11RMRequestTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11RMRequestEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This group contains the current list of requests for RM reports

to be issued and have been issued until removed. A network man-

ager adds a RM request by creating a row with createAndWait row

status and then filling in the request parameters/attributes. The

request becomes active to be issued when the row status is set to

Active. The columnar objects or attributes other than the rowSta-

tus shall not be written if the rowStatus is Active. The request

rows can be deleted, if commanded by a network manager via chang-

ing the value of dot11RMRqstRowStatus to Destroy. This may leave

orphaned rows if a manager crashes and forgets which rows are

being used by it. One recommended way to manage orphaned or fin-

ished rows is to delete rows if their dot11RMRqstRowStatus remains

other than Active for longer than a period (recommend at least 5

minutes, RFC 2579). Or another recommended way is to delete older

rows as needed based on their dot11RMRqstTimeStamp values. This

can be done by the agent as well as the manager. "

::= { dot11RMRequest 2 }

dot11RMRequestEntry OBJECT-TYPE

SYNTAX Dot11RMRequestEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11RMRequestTable Indexed by dot11RMRqstIndex."

INDEX { dot11RMRqstIndex }

::= { dot11RMRequestTable 1 }

Dot11RMRequestEntry ::=

SEQUENCE {

dot11RMRqstIndex	Unsigned32,

dot11RMRqstRowStatus	RowStatus,

dot11RMRqstToken	OCTET STRING,

dot11RMRqstRepetitions	INTEGER,

dot11RMRqstIfIndex	INTEGER,

dot11RMRqstType	INTEGER,

dot11RMRqstTargetAdd	MacAddress,

dot11RMRqstTimeStamp	TimeTicks,

dot11RMRqstChanNumber	INTEGER,

dot11RMRqstRegulatoryClass	INTEGER,

dot11RMRqstRndInterval	Unsigned32,

dot11RMRqstDuration	Unsigned32,

dot11RMRqstParallel	TruthValue,

dot11RMRqstEnable	TruthValue,

dot11RMRqstRequest	TruthValue,

dot11RMRqstReport	TruthValue,

dot11RMRqstDurationMandatory	TruthValue,

dot11RMRqstBeaconRqstMode	INTEGER,

dot11RMRqstBeaconRqstDetail	INTEGER,

dot11RMRqstFrameRqstType	INTEGER,

dot11RMRqstBssid	MacAddress,

dot11RMRqstSSID	OCTET STRING,

dot11RMRqstBeaconReportingCondition	INTEGER,

dot11RMRqstBeaconThresholdOffset	INTEGER,

dot11RMRqstSTAStatRqstGroupID	INTEGER,

dot11RMRqstLCIRqstSubject	INTEGER,

dot11RMRqstLCILatitudeResolution	INTEGER,

dot11RMRqstLCILongitudeResolution	INTEGER,

dot11RMRqstLCIAltitudeResolution	INTEGER,

dot11RMRqstLCIAzimuthType	INTEGER,

dot11RMRqstLCIAzimuthResolution	INTEGER,

dot11RMRqstPauseTime	INTEGER,

dot11RMRqstTransmitStreamPeerQSTAAddress	MacAddress,

dot11RMRqstTransmitStreamTrafficIdentifier	INTEGER,

dot11RMRqstTransmitStreamBin0Range	INTEGER,

dot11RMRqstTrigdQoSAverageCondition	TruthValue,

dot11RMRqstTrigdQoSConsecutiveCondition	TruthValue,

dot11RMRqstTrigdQoSDelayCondition	TruthValue,

dot11RMRqstTrigdQoSAverageThreshold	INTEGER,

dot11RMRqstTrigdQoSConsecutiveThreshold	INTEGER,

dot11RMRqstTrigdQoSDelayThresholdRange	INTEGER,

dot11RMRqstTrigdQoSDelayThreshold	INTEGER,

dot11RMRqstTrigdQoSMeasurementCount	INTEGER,

dot11RMRqstTrigdQoSTimeout	INTEGER,

dot11RMRqstChannelLoadReportingCondition	INTEGER,

dot11RMRqstChannelLoadReference	INTEGER,

dot11RMRqstNoiseHistogramReportingCondition	INTEGER,

dot11RMRqstAnpiReference	INTEGER,

dot11RMRqstAPChannelReport	OCTET STRING,

dot11RMRqstSTAStatPeerSTAAddress	MacAddress,

dot11RMRqstFrameTransmitterAddress	MacAddress,

dot11RMRqstVendorSpecific	OCTET STRING }

dot11RMRqstIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for RM Request elements in dot11RMRequestTable, greater

than 0."

::= { dot11RMRequestEntry 1 }

dot11RMRqstRowStatus OBJECT-TYPE

SYNTAX RowStatus

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The Row Status column of the current row, used for tracking sta-

tus of an individual request. When this attribute is set to

Active, AND a measurement request can be unambiguously created

based on the parameters in the row, then the MLME may proceed to

issue the request to its intended targets when appropriate. If

not, this attribute may be set to Not-ready immediately to indi-

cate parametric errors. However, it is the network managers

responsibility to correct the error. If the request is success-

fully issued to the target STA, then the rowStatus is set to

notInService."

REFERENCE

"Subclause 7.3.2.21 (Measurement Request element)"

::= { dot11RMRequestEntry 2 }

dot11RMRqstToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates a unique string to identify a group of

rows to be issued as parallel or sequential measurements. To guar-

antee the uniqueness of this token across multiple network manag-

ers, it is recommended that this token be prefixed with the IP

address of the network manager creating this row. This token is

not necessarily equivalent to the measurement tokens in RM request

frames. If this attribute is an empty string, then this row of

request is independent from other requests."

DEFVAL { "" }

::= { dot11RMRequestEntry 3 }

dot11RMRqstRepetitions OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the requested number of repetitions for

all the measurement request elements in this frame.  A value of

zero in the Number of Repetitions field indicates measurement

request elements are executed once without repetition."

::= { dot11RMRequestEntry 4 }

dot11RMRqstIfIndex OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The ifIndex on which this row of the RM Request is to be issued."

::= { dot11RMRequestEntry 5 }

dot11RMRqstType OBJECT-TYPE

SYNTAX INTEGER {

channelLoad(3),

noiseHistogram(4),

beacon(5),

frame(6),

staStatistics(7),

lci(8),

transmitStream(9),

pause(255)

}

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the measurement type of this RM request

row."

::= { dot11RMRequestEntry 6 }

dot11RMRqstTargetAdd OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The MAC address of STA for this row of RM Request is to be issued

to. If this attribute matches the MAC address of the

dot11RMRqstIfIndex, then measurement request is for this STA

itself to carry out."

::= { dot11RMRequestEntry 7 }

dot11RMRqstTimeStamp OBJECT-TYPE

SYNTAX TimeTicks

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the sysUpTime Value the last time when

the dot11RMRqstRowStatus is set to active or when this row is cre-

ated the first time. This attribute shall be set by this STA or AP

automatically, not by an SNMP manager."

::= { dot11RMRequestEntry 8 }

dot11RMRqstChanNumber OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The target STA channel number on which to perform the measure-

ments indicated in this request. The Channel Number is only

defined within the indicated Regulatory Class for this measure-

ment request. This attribute is ignored if dot11RMRqstType = STA

statistics Request, LCI Request, Transmit Stream/Category Measure-

ment, or Measurement Pause. However, even in that case, the man-

ager should set this attribute to the current channel for this

interface, so that the row can be set to active when ready with

all attributes indicated."

::= { dot11RMRequestEntry 9 }

dot11RMRqstRegulatoryClass OBJECT-TYPE

SYNTAX INTEGER (1..255)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the channel set for this measurement

request. Country, Regulatory Class and Channel Number together

specify the channel frequency and spacing for this measurement

request. Valid values of Regulatory Class are shown in Annex J."

REFERENCE

"Annex J"

::= { dot11RMRequestEntry 10 }

dot11RMRqstRndInterval OBJECT-TYPE

SYNTAX Unsigned32

UNITS "TUs"

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the upper bound of the random delay to

be used prior to making the measurement, expressed in units of

TUs. See 11.10.2 (Measurement start time(11k)). This attribute is

ignored if dot11RMRqstType = STA statistics Request, LCI Request,

Transmit Stream/Category Measurement or Measurement Pause."

DEFVAL { 0 }

::= { dot11RMRequestEntry 11 }

dot11RMRqstDuration OBJECT-TYPE

SYNTAX Unsigned32

UNITS "TUs"

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the preferred or mandatory measurement

duration for this Measurement Request. This attribute is ignored

if dot11RMRqstType = LCI Request or Measurement Pause."

DEFVAL { 0 }

::= { dot11RMRequestEntry 12 }

dot11RMRqstParallel OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the parallel bit for this Measurement

Request element. Default is false which means the measurement

shall be performed in sequence. This attribute, when true, indi-

cates that this measurement should start at the same time as the

measurement described by the next Measurement Request element in

the next row if the next row indicates the same value for

dot11RMRqstToken. The default value of this attribute is false."

::= { dot11RMRequestEntry 13 }

dot11RMRqstEnable OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the enable bit for this Measurement

Request element. The default value of this attribute is false."

::= { dot11RMRequestEntry 14 }

dot11RMRqstRequest OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the request bit for this Measurement

Request element. This attribute, when true, indicates that this

STA shall accept measurement requests from the target STA. The

default value of this attribute is false."

::= { dot11RMRequestEntry 15 }

dot11RMRqstReport OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the report bit for this Measurement

Request element. This attribute, when true, indicates that the

target STA may enable autonomous measurement reports to the

requesting STA. The default value of this attribute is false."

::= { dot11RMRequestEntry 16 }

dot11RMRqstDurationMandatory OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the duration mandatory bit for this Mea-

surement Request element. This attribute, when true, indicates

that the indicated Measurement Duration is a mandatory duration

for this measurement. This attribute, when false, indicates that

the indicated Measurement Duration is a maximum duration for this

measurement. The default value of this attribute is false."

::= { dot11RMRequestEntry 17 }

dot11RMRqstBeaconRqstMode OBJECT-TYPE

SYNTAX INTEGER {

passive(0),

active(1),

beaconTable(2)

}

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the Measurement Mode for this Beacon

Request element. This attribute is only valid if the

dot11RMRqstType is 5, indicating a beacon request, and is ignored

otherwise."

DEFVAL { 0 }

::= { dot11RMRequestEntry 18 }

dot11RMRqstBeaconRqstDetail OBJECT-TYPE

SYNTAX INTEGER {

noBody(0),

fixedFieldsAndRequestedElements(1),

allBody(2)

}

MAX-ACCESS read-create

STATUS current

DESCRIPTION

" dot11RMRqstBeaconRqstDetail indicates the Reporting Detail for

Beacon Request element. This attribute is only valid if the

dot11RMRqstType is 5, indicating a beacon request, and is ignored

otherwise."

DEFVAL { 2 }

::= { dot11RMRequestEntry 19 }

dot11RMRqstFrameRqstType OBJECT-TYPE

SYNTAX INTEGER {

frameCountRep(1)

}

MAX-ACCESS read-create

STATUS current

DESCRIPTION

" dot11RMRqstFrameRqstType indicates the Frame Request Type for

Frame Request element. This attribute is only valid if the

dot11RMRqstType is 6, indicating a frame request, and is ignored

otherwise."

DEFVAL { 21 }

::= { dot11RMRequestEntry 20 }

dot11RMRqstBssid OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"BSSID indicates the BSSID of the particular AP for which this

measurement

is requested. The BSSID shall be set to the wildcard BSSID when

the measurement is to be performed on any AP(s) on the indicated

channel. This attribute is only valid if the dot11RMRqstType is 5,

indicating a beacon request, and is ignored otherwise."

DEFVAL { 'FFFFFFFFFFFF'H }

::= { dot11RMRequestEntry 21 }

dot11RMRqstSSID OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..32))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the SSID for the measurement. Zero

length MIB element for SSID indicates the wildcard SSID. The SSID

shall be set to the wildcard SSID when the measurement is to be

performed on all ESSs/IBSSs on the indicated channel. This

attribute is only valid if the dot11RMRqstType is 5, indicating a

beacon request, and is ignored otherwise."

DEFVAL { ''H }

::= { dot11RMRequestEntry 22 }

dot11RMRqstBeaconReportingCondition OBJECT-TYPE

SYNTAX INTEGER {

afterEveryMeasurement(0),

rcpiAboveAbsoluteThreshold(1),

rcpiBelowAbsoluteThreshold(2),

rsniAboveAbsoluteThreshold(3),

rsniBelowAbsoluteThreshold(4),

rcpiAboveOffsetThreshold(5),

rcpiBelowOffsetThreshold(6),

rsniAboveOffsetThreshold(7),

rsniBelowOffsetThreshold(8),

rcpiInBound(9),

rsniInBound(10)

}

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates when the Beacon Measurement results are

to be reported to the requesting STA. This attribute is only valid

if the dot11RMRqstType is 5, indicating a beacon request, and is

ignored otherwise."

REFERENCE

"IEEE 802.11, Table 7-29g (Reporting Condition for Beacon Report)-

Reporting Condition values for Beacon Request element"

DEFVAL {0}

::= { dot11RMRequestEntry 23 }

dot11RMRqstBeaconThresholdOffset OBJECT-TYPE

SYNTAX INTEGER

UNITS "0.5 dB"

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"Threshold/Offset provides either the threshold value or the off-

set value to be used for conditional reporting. For indicated

Reporting Conditions 1-4, the integer range is (0..255). For indi-

cated Reporting Conditions 5-10, the integer range is

(-127..+127). This attribute is only valid if the dot11RMRqstType

is 5, indicating a beacon request, and is ignored otherwise."

DEFVAL { 0 }

::= { dot11RMRequestEntry 24 }

dot11RMRqstSTAStatRqstGroupID OBJECT-TYPE

SYNTAX INTEGER {

dot11CountersTable(0),

dot11MacStatistics(1),

dot11QosCountersTableforUP0(2),

dot11QosCountersTableforUP1(3),

dot11QosCountersTableforUP2(4),

dot11QosCountersTableforUP3(5),

dot11QosCountersTableforUP4(6),

dot11QosCountersTableforUP5(7),

dot11QosCountersTableforUP6(8),

dot11QosCountersTableforUP7(9),

bSSAverageAccessDelays(10)

}

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The attribute indicates the group identity for this Measurement

Request element. This attribute is only valid if the

dot11RMRqstType is 7, indicating a statistics request, and is

ignored otherwise."

DEFVAL { 0 }

::= { dot11RMRequestEntry 25 }

dot11RMRqstLCIRqstSubject OBJECT-TYPE

SYNTAX INTEGER {

local(0),

remote(1)

}

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The attribute indicates the subject of the LCI measurement

request. This attribute is only valid if the dot11RMRqstType is 8,

indicating an LCI request, and is ignored otherwise."

DEFVAL { 0 }

::= { dot11RMRequestEntry 26 }

dot11RMRqstLCILatitudeResolution OBJECT-TYPE

SYNTAX INTEGER (0..63)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute is 6 bits indicating the number of valid

bits in the fixed-point value of Latitude of the LCI measurement

request. This attribute is only valid if the dot11RMRqstType is 8,

indicating an LCI request, and is ignored otherwise."

::= { dot11RMRequestEntry 27 }

dot11RMRqstLCILongitudeResolution OBJECT-TYPE

SYNTAX INTEGER (0..63)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute is 6 bits indicating the number of valid

bits in the fixed-point value of Longitude of the LCI measurement

request. This attribute is only valid if the dot11RMRqstType is 8,

indicating an LCI request, and is ignored otherwise."

::= { dot11RMRequestEntry 28 }

dot11RMRqstLCIAltitudeResolution OBJECT-TYPE

SYNTAX INTEGER (0..63)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute is 6 bits indicating the number of valid

bits in the fixed-point value of Altitude of the LCI measurement

request. This attribute is only valid if the dot11RMRqstType is 8,

indicating an LCI request, and is ignored otherwise."

::= { dot11RMRequestEntry 29 }

dot11RMRqstLCIAzimuthType OBJECT-TYPE

SYNTAX INTEGER {

frontSurfaceofSta(0),

radioBeam(1)

}

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the azimuth reference for the LCI Azi-

muth measurement request. This attribute is only valid if the

dot11RMRqstType is 8, indicating an LCI request, and is ignored

otherwise."

DEFVAL{ 0 }

::= { dot11RMRequestEntry 30 }

dot11RMRqstLCIAzimuthResolution OBJECT-TYPE

SYNTAX INTEGER (0..15)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute is 4 bits indicating the number of valid

bits in the fixed-point value of Azimuth of the LCI Azimuth

measurement request. This attribute is only valid if the

dot11RMRqstType is 8, indicating an LCI request, and is ignored

otherwise."

::= { dot11RMRequestEntry 31 }

dot11RMRqstPauseTime OBJECT-TYPE

SYNTAX INTEGER (0..65535)

UNITS "10 TUs"

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute is a 16 bit unsigned integer number

representing the time period for which measurements are

suspended or paused. Measurement Pause Requests are used to

provide time delays between the execution times of measurement

request elements in a Measurement Request Frame. This attribute is

only valid if the dot11RMRqstType is 255, indicating an pause

request, and is ignored otherwise."

DEFVAL { 0 }

::= { dot11RMRequestEntry 32 }

dot11RMRqstTransmitStreamPeerQSTAAddress OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the peer STA address to be measured for

a Transmit Stream/Category Measurement measurement. This attribute

is only valid if the dot11RMRqstType is 9, indicating a transmit

stream/category request, and is ignored otherwise."

::= { dot11RMRequestEntry 33 }

dot11RMRqstTransmitStreamTrafficIdentifier OBJECT-TYPE

SYNTAX INTEGER(0..16)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the TC, or TS to be measured for a

Transmit Stream/Category Measurement measurement. This attribute

is only valid if the dot11RMRqstType is 9, indicating a transmit

stream/category request, and is ignored otherwise."

::= { dot11RMRequestEntry 34 }

dot11RMRqstTransmitStreamBin0Range OBJECT-TYPE

SYNTAX INTEGER(1..255)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the delay range for bin 0 of the trans-

mit delay histogram. This attribute is only valid if the

dot11RMRqstType is 9, indicating a transmit stream/category

request, and is ignored otherwise."

::= { dot11RMRequestEntry 35 }

dot11RMRqstTrigdQoSAverageCondition OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute, when true, indicates a request for triggered

reporting with trigger based on the number of discarded MSDUs

reaching the  dot11RMRqstTrigdQoSAverageThreshold when averaged

over dot11RMRqstTrigdQoSMEasurementCount consecutive MSDUs. This

attribute is only valid if the dot11RMRqstType is 9, indicating a

transmit stream/category request, and is ignored otherwise. The

default value of this attribute is false."

::= { dot11RMRequestEntry 36 }

dot11RMRqstTrigdQoSConsecutiveCondition OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute, when true, indicates a request for triggered

reporting with trigger based on the consecutive number of MSDUs

discarded reaching dot11RMRqstTrigdQoSConsecutiveThreshold. This

attribute is only valid if the dot11RMRqstType is 9, indicating a

transmit stream/category request, and is ignored otherwise. The

default value of this attribute is false."

::= { dot11RMRequestEntry 37 }

dot11RMRqstTrigdQoSDelayCondition OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute, when true, indicates a request for triggered

reporting with trigger based on the consecutive number of MSDUs

that experience a transmit delay greater than

dot11RMRqstTrigdQoSDelayThresholdRange reaching

dot11RMRqstTrigdQoSDelayThreshold. This attribute is only valid if

the dot11RMRqstType is 9, indicating a transmit stream/category

request, and is ignored otherwise. The default value of this

attribute is false."

::= { dot11RMRequestEntry 38 }

dot11RMRqstTrigdQoSAverageThreshold OBJECT-TYPE

SYNTAX INTEGER (1..255)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the trigger threshold for triggered

Transmit Stream/Category Measurement based on average MSDUs dis-

carded. Trigger occurs if the number of MSDUs discarded over the

moving average number of transmitted MSDUs in

dot11RMRqstTrigdQoSMeasurementCount reaches this threshold. This

attribute is only valid if the dot11RMRqstType is 9, indicating a

transmit stream/category request, and is ignored otherwise."

DEFVAL { 10 }

::= { dot11RMRequestEntry 39 }

dot11RMRqstTrigdQoSConsecutiveThreshold OBJECT-TYPE

SYNTAX INTEGER (1..255)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the trigger threshold for triggered

Transmit Stream/Category Measurement based on consecutive MSDUs

discarded. Trigger occurs if the consecutive number of MSDUs dis-

carded reaches this threshold. This attribute is only valid if the

dot11RMRqstType is 9, indicating a transmit stream/category

request, and is ignored otherwise."

DEFVAL { 5 }

::= { dot11RMRequestEntry 40 }

dot11RMRqstTrigdQoSDelayThresholdRange OBJECT-TYPE

SYNTAX INTEGER (0..3)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the minimum transmit delay for delayed

MSDU counts. Trigger occurs if the a consecutive number of MSDUs

experience a transmit delay greater than or equal to the lower

bound of the bin of the Transmit Delay Histogram given by the

value of this attribute + 2, e.g. if this attribute is 1 the lower

bound of bin 3. This attribute is only valid if the

dot11RMRqstType is 9, indicating a transmit stream/category

request, and is ignored otherwise."

DEFVAL { 1 }

::= { dot11RMRequestEntry 41 }

dot11RMRqstTrigdQoSDelayThreshold OBJECT-TYPE

SYNTAX INTEGER (1..255)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the number of consecutive delayed MSDUs

needed for trigger. Trigger occurs if the consecutive number of

MSDUs that experience a transmit delay greater than

dot11RMRqstQoSDelayThresholdRange reaches this value. This

attribute is only valid if the dot11RMRqstType is 9, indicating a

transmit stream/category request, and is ignored otherwise."

DEFVAL { 20 }

::= { dot11RMRequestEntry 42 }

dot11RMRqstTrigdQoSMeasurementCount OBJECT-TYPE

SYNTAX INTEGER (1..255)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the number of MSDUs to be used as a mov-

ing average count in the average error threshold and in determin-

ing the scope of the reported Transmit Stream/Category measurement

in a triggered measurement report. This attribute is only valid if

the dot11RMRqstType is 9, indicating a transmit stream/category

request, and is ignored otherwise."

DEFVAL { 100 }

::= { dot11RMRequestEntry 43 }

dot11RMRqstTrigdQoSTimeout OBJECT-TYPE

SYNTAX INTEGER (1..255)

UNITS "100 TUs"

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the timeout interval during which a mea-

suring STA shall not generate further triggered Transmit Stream/

Category measurement reports after a trigger condition has been

met and a report generated. This attribute is only valid if the

dot11RMRqstType is 9, indicating a transmit stream/category

request, and is ignored otherwise."

DEFVAL { 20 }

::= { dot11RMRequestEntry 44 }

dot11RMRqstChannelLoadReportingCondition OBJECT-TYPE

SYNTAX INTEGER {

afterEveryMeasurement(0),

chanLoadAboveReference(1),

chanLoadBelowReference(2),

}

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates when the Channel Load Measurement

results are to be reported to the requesting STA. This attribute

is only valid if the dot11RMRqstType is 3, indicating a channel

load request, and is ignored otherwise."

REFERENCE

"IEEE 802.11, Table 7-29b (Reporting Condition for Channel Load

Report)—Reporting Condition values for Channel Load Request ele-

ment"

DEFVAL {0}

::= { dot11RMRequestEntry 45 }

dot11RMRqstChannelLoadReference OBJECT-TYPE

SYNTAX INTEGER (0..255)

UNITS "1/255"

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the channel load reporting condition

reference value. The measured Channel Load is compared to this

reference value and a report is issued if the reporting condition

is satisfied. The reference value is in the same units as Channel

Load and represents the fractional time of the measurement dura-

tion during which the STA determined the channel to be busy. This

attribute is only valid if the dot11RMRqstType is 3, indicating a

channel load request, and is ignored otherwise."

DEFVAL { 5 }

::= { dot11RMRequestEntry 46 }

dot11RMRqstNoiseHistogramReportingCondition OBJECT-TYPE

SYNTAX INTEGER {

afterEveryMeasurement(0),

aNPIAboveReference(1),

aNPIBelowReference(2),

}

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates when the Noise Histogram Measurement

results are to be reported to the requesting STA. This attribute

is only valid if the dot11RMRqstType is 4, indicating a noise his-

togram request, and is ignored otherwise."

REFERENCE

"IEEE 802.11, Table 7-29d (Reporting Condition for Noise Histo-

gram Report)—Reporting Condition for Noise Histogram Report"

DEFVAL {0}

::= { dot11RMRequestEntry 47 }

dot11RMRqstAnpiReference OBJECT-TYPE

SYNTAX INTEGER (0..255)

UNITS "0.5 dbm"

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the noise histogram reporting condition

ANPI reference value. The measured ANPI is compared to this refer-

ence value and a report is issued if the indicated reporting con-

dition is satisfied.

ANPIval = Int[(ANPIpower in dBm + 110)*2], for ANPI in the range -

110 dBm to 0 dBm. ANPIval = 220 for ANPI > 0 dBm. ANPIval = 255

when ANPI is not available. This attribute is only valid if the

dot11RMRqstType is 4, indicating a noise histogram request, and is

ignored otherwise."

DEFVAL { 5 }

::= { dot11RMRequestEntry 48 }

dot11RMRqstAPChannelReport OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..255))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the specific channels to be used for the

requested beacon measurements. Zero length is the null default for

this attribute. Each octet indicates a different channel within

the indicated Regulatory Class. This list of channels is the Chan-

nel List in the AP Channel Report element described in 7.3.2.36

(AP Channel Report element(11k)). This attribute is only valid if

the dot11RMRqstType is 5, indicating a beacon request, and is

ignored otherwise."

DEFVAL { ''H }

::= { dot11RMRequestEntry 49 }

dot11RMRqstSTAStatPeerSTAAddress OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the peer STA address to be measured for

a statistics request. This attribute is only valid if the

dot11RMRqstType is 7, indicating a statistics request, and is

ignored otherwise."

::= { dot11RMRequestEntry 50 }

dot11RMRqstFrameTransmitterAddress OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the Transmitter Address (TA) of the

frames to be counted in this frame request. This attribute is only

valid if the dot11RMRqstType is 6, indicating a frame request, and

is ignored otherwise."

::= { dot11RMRequestEntry 51 }

dot11RMRqstVendorSpecific OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..255))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute provides an envelope for any optional vendor spe-

cific subelements which may be included in a measurement request

element. Zero length is the null default for this attribute.  This

attribute is valid for all requests."

DEFVAL { ''H }

::= { dot11RMRequestEntry 52 }

-- **********************************************************************

-- * End of dot11RMRequest TABLE

-- **********************************************************************

-- ********************************************************************

-- * Radio Measurement Reports

-- * Report tables contain measurement reports received by this STA or

-- * results of measurements performed by this STA.

-- ********************************************************************

dot11RMReport OBJECT IDENTIFIER ::= { dot11RadioResourceMeasurement 2 }

-- ********************************************************************

-- * dot11ChannelLoadReport TABLE

-- ********************************************************************

dot11ChannelLoadReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11ChannelLoadReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This table contains the current list of Channel Load reports that

have been received by the MLME. The report tables shall be main-

tained as a FIFO to preserve freshness, thus the rows in this

table can be deleted for memory constraints or other implementa-

tion constraints determined by the vendor. New rows shall have

different RprtIndex values than those deleted within the range

limitation of the index. One easy way is to monotonically increase

RprtIndex for new reports being written in the table."

::= { dot11RMReport 1 }

dot11ChannelLoadReportEntry OBJECT-TYPE

SYNTAX Dot11ChannelLoadReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11ChannelLoadReportTable Indexed by

dot11ChannelLoadRprtIndex."

INDEX { dot11ChannelLoadRprtIndex }

::= { dot11ChannelLoadReportTable 1 }

Dot11ChannelLoadReportEntry ::=

SEQUENCE {

dot11ChannelLoadRprtIndex	Unsigned32,

dot11ChannelLoadRprtRqstToken	OCTET STRING,

dot11ChannelLoadRprtIfIndex	INTEGER,

dot11ChannelLoadMeasuringSTAAddr	MacAddress,

dot11ChannelLoadRprtChanNumber	INTEGER,

dot11ChannelLoadRprtRegulatoryClass	INTEGER,

dot11ChannelLoadRprtActualStartTime	TSFType,

dot11ChannelLoadRprtMeasurementDuration	Unsigned32,

dot11ChannelLoadRprtChannelLoad	INTEGER,

dot11ChannelLoadRprtVendorSpecific	OCTET STRING,

dot11ChannelLoadRprtMeasurementMode	INTEGER }

dot11ChannelLoadRprtIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for Channel Load Report elements in

dot11ChannelLoadReportTable, greater than 0."

::= { dot11ChannelLoadReportEntry 1 }

dot11ChannelLoadRprtRqstToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the request token that was indicated in

the Measurement request that generated this measurement report.

This should be an exact match to the original dot11RMRqstToken

attribute. Note that there may be  multiple entries in the table

that match this value since a single request may generate multi-

ple measurement reports."

::= { dot11ChannelLoadReportEntry 2 }

dot11ChannelLoadRprtIfIndex OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The ifIndex of the interface over which this ChannelLoad Report

was received."

::= { dot11ChannelLoadReportEntry 3 }

dot11ChannelLoadMeasuringSTAAddr OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The MAC address of the measuring STA for this row of the Channel

Load report."

::= { dot11ChannelLoadReportEntry 4 }

dot11ChannelLoadRprtChanNumber OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the channel number used for this Chan-

nel Load Report. The Channel Number is only defined within the

indicated Regulatory Class for this measurement report."

::= { dot11ChannelLoadReportEntry 5 }

dot11ChannelLoadRprtRegulatoryClass OBJECT-TYPE

SYNTAX INTEGER(1..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the channel set for this measurement

report. Country, Regulatory Class and Channel Number together

specify the channel frequency and spacing for this measurement

report. Valid values of Regulatory Class are shown in Annex J."

REFERENCE

"Annex J"

::= { dot11ChannelLoadReportEntry 6 }

dot11ChannelLoadRprtActualStartTime OBJECT-TYPE

SYNTAX TSFType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the TSF value at the time when the

measurement started."

::= { dot11ChannelLoadReportEntry 7 }

dot11ChannelLoadRprtMeasurementDuration OBJECT-TYPE

SYNTAX Unsigned32

UNITS "TUs"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the duration over which the ChannelLoad

Report was measured."

::= { dot11ChannelLoadReportEntry 8 }

dot11ChannelLoadRprtChannelLoad OBJECT-TYPE

SYNTAX INTEGER(0..255)

UNITS "1/255"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Channel Load shall contain the fractional duration over which the

measuring STA determined the channel to be busy during the mea-

surement duration."

REFERENCE

"Subclause 7.3.2.22.4 (Channel Load Report(11k))"

::= { dot11ChannelLoadReportEntry 9 }

dot11ChannelLoadRprtVendorSpecific OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..255))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute provides an envelope for any optional vendor spe-

cific subelements which may be included in a measurement report

element. Zero length is the null default for this attribute."

DEFVAL { ''H }

::= { dot11ChannelLoadReportEntry 10 }

dot11ChannelLoadRprtMeasurementMode OBJECT-TYPE

SYNTAX INTEGER {

success(0),

incapableBit(1),

refusedBit(2),

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the outcome status for the measurement

request which generated this measurement report; status is indi-

cated using the following reason codes: 1 indicates this STA is

incapable of generating the report, 2 indicates this STA is refus-

ing to generate the report, 0 indicates the STA successfully car-

ried out the measurement request."

DEFVAL { 0 }

::= { dot11ChannelLoadReportEntry 11 }

-- ********************************************************************

-- * End of dot11ChannelLoadReport TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11NoiseHistogramReport TABLE

-- ********************************************************************

dot11NoiseHistogramReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11NoiseHistogramReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This table contains the current list of Noise Histogram reports

that have been received by the MLME. The report tables shall be

maintained as a FIFO to preserve freshness, thus the rows in this

table can be deleted for memory constraints or other implementa-

tion constraints determined by the vendor. New rows shall have

different RprtIndex values than those deleted within the range

limitation of the index. One easy way is to monotonically increase

RprtIndex for new reports being written in the table."

::= { dot11RMReport 2 }

dot11NoiseHistogramReportEntry OBJECT-TYPE

SYNTAX Dot11NoiseHistogramReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11NoiseHistogramReportTable Indexed by

dot11NoiseHistogramRprtIndex."

INDEX { dot11NoiseHistogramRprtIndex }

::= { dot11NoiseHistogramReportTable 1 }

Dot11NoiseHistogramReportEntry ::=

SEQUENCE {

dot11NoiseHistogramRprtIndex	Unsigned32,

dot11NoiseHistogramRprtRqstToken	OCTET STRING,

dot11NoiseHistogramRprtIfIndex	INTEGER,

dot11NoiseHistogramMeasuringSTAAddr	MacAddress,

dot11NoiseHistogramRprtChanNumber	INTEGER,

dot11NoiseHistogramRprtRegulatoryClass	INTEGER,

dot11NoiseHistogramRprtActualStartTime	TSFType,

dot11NoiseHistogramRprtMeasurementDuration	Unsigned32,

dot11NoiseHistogramRprtAntennaID	INTEGER,

dot11NoiseHistogramRprtANPI	INTEGER,

dot11NoiseHistogramRprtIPIDensity0	INTEGER,

dot11NoiseHistogramRprtIPIDensity1	INTEGER,

dot11NoiseHistogramRprtIPIDensity2	INTEGER,

dot11NoiseHistogramRprtIPIDensity3	INTEGER,

dot11NoiseHistogramRprtIPIDensity4	INTEGER,

dot11NoiseHistogramRprtIPIDensity5	INTEGER,

dot11NoiseHistogramRprtIPIDensity6	INTEGER,

dot11NoiseHistogramRprtIPIDensity7	INTEGER,

dot11NoiseHistogramRprtIPIDensity8	INTEGER,

dot11NoiseHistogramRprtIPIDensity9	INTEGER,

dot11NoiseHistogramRprtIPIDensity10	INTEGER,

dot11NoiseHistogramRprtVendorSpecific	OCTET STRING,

dot11NoiseHistogramRprtMeasurementMode	INTEGER}

dot11NoiseHistogramRprtIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for Noise Histogram elements in

dot11NoiseHistogramReportTable, greater than 0."

::= { dot11NoiseHistogramReportEntry 1 }

dot11NoiseHistogramRprtRqstToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the request token that was indicated in

the measurement request that generated this measurement report.

This should be an exact match to the original dot11RMRqstToken

attribute. Note that there may be multiple entries in the table

that match this value since a single request may generate multi-

ple measurement reports."

::= { dot11NoiseHistogramReportEntry 2 }

dot11NoiseHistogramRprtIfIndex OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The ifIndex of the interface over which this Noise Histogram

Report was received. "

::= { dot11NoiseHistogramReportEntry 3 }

dot11NoiseHistogramMeasuringSTAAddr OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The MAC address of the measuring STA for this row of Noise Histo-

gram report."

::= { dot11NoiseHistogramReportEntry 4 }

dot11NoiseHistogramRprtChanNumber OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the channel number used for this Noise

Histogram Report. The Channel Number is only defined within the

indicated Regulatory Class for this measurement report."

::= { dot11NoiseHistogramReportEntry 5 }

dot11NoiseHistogramRprtRegulatoryClass OBJECT-TYPE

SYNTAX INTEGER(1..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the channel set for this measurement

report. Country, Regulatory Class and Channel Number together

specify the channel frequency and spacing for this measurement

report. Valid values of Regulatory Class are shown in Annex J."

REFERENCE

"Annex J"

::= { dot11NoiseHistogramReportEntry 6 }

dot11NoiseHistogramRprtActualStartTime OBJECT-TYPE

SYNTAX TSFType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the TSF value at the time when the

measurement started."

::= { dot11NoiseHistogramReportEntry 7 }

dot11NoiseHistogramRprtMeasurementDuration OBJECT-TYPE

SYNTAX Unsigned32

UNITS "TUs"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the duration over which the Noise Histo-

gram Report was measured."

::= { dot11NoiseHistogramReportEntry 8 }

dot11NoiseHistogramRprtAntennaID OBJECT-TYPE

SYNTAX INTEGER(0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the identifying number for the antenna

used for this measurement. The value 0 indicates that the antenna

identifier is unknown. The value 255 indicates that the measure-

ment was made with multiple antennas or that the antenna ID is

unknown. The value 1 is used for a STA with only one antenna.

STAs with more than one antenna shall assign Antenna IDs to each

antenna as consecutive, ascending numbers. Each  Antenna ID num-

ber represents a unique antenna characterized by a fixed relative

position, a fixed relative direction and a peak gain for that

position and direction."

::= { dot11NoiseHistogramReportEntry 9 }

dot11NoiseHistogramRprtANPI OBJECT-TYPE

SYNTAX INTEGER(0..255)

UNITS "0.5 dBm"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the Average Noise Power Indicator (ANPI)

for this Noise Histogram measurement. The ANPI value represents

the average noise plus interference power on the measured channel

at the antenna connector during the measurement duration To calcu-

late ANPI, the STA shall measure and use IPI in the indicated

channel when NAV is equal to 0 (when virtual CS mechanism indi-

cates idle channel) except during frame transmission or recep-

tion."

::= { dot11NoiseHistogramReportEntry 10 }

dot11NoiseHistogramRprtIPIDensity0 OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the measured IPI density for non-802.11

signals with measured power satisfying the condition: Power <= -

92dBm."

::= { dot11NoiseHistogramReportEntry 11 }

dot11NoiseHistogramRprtIPIDensity1 OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the measured IPI density for non-802.11

signals with measured power satisfying the condition: -92dBm <

Power <= -89dBm."

::= { dot11NoiseHistogramReportEntry 12 }

dot11NoiseHistogramRprtIPIDensity2 OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the measured IPI density for non-802.11

signals with measured power satisfying the condition: -89dBm <

Power <= -86dBm."

::= { dot11NoiseHistogramReportEntry 13 }

dot11NoiseHistogramRprtIPIDensity3 OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the measured IPI density for non-802.11

signals with measured power satisfying the condition: -86dBm <

Power <= -83dBm."

::= { dot11NoiseHistogramReportEntry 14 }

dot11NoiseHistogramRprtIPIDensity4 OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the measured IPI density for non-802.11

signals with measured power satisfying the condition: -83dBm <

Power <= -80dBm."

::= { dot11NoiseHistogramReportEntry 15 }

dot11NoiseHistogramRprtIPIDensity5 OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the measured IPI density for non-802.11

signals with measured power satisfying the condition: -80dBm <

Power <= -75dBm."

::= { dot11NoiseHistogramReportEntry 16 }

dot11NoiseHistogramRprtIPIDensity6 OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the measured IPI density for non-802.11

signals

with measured power satisfying the condition: -75dBm < Power <= -

70dBm." ::= { dot11NoiseHistogramReportEntry 17 }

dot11NoiseHistogramRprtIPIDensity7 OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the measured IPI density for non-802.11

signals with measured power satisfying the condition: -70dBm <

Power <= -65dBm."

::= { dot11NoiseHistogramReportEntry 18 }

dot11NoiseHistogramRprtIPIDensity8 OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the measured IPI density for non-802.11

signals with measured power satisfying the condition: -65dBm <

Power <= -60dBm."

::= { dot11NoiseHistogramReportEntry 19 }

dot11NoiseHistogramRprtIPIDensity9 OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the measured IPI density for non-802.11

signals with measured power satisfying the condition: -60dBm <

Power <= -55dBm."

::= { dot11NoiseHistogramReportEntry 20 }

dot11NoiseHistogramRprtIPIDensity10 OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the measured IPI density for non-802.11

signals with measured power satisfying the condition: -55dBm <

Power."

::= { dot11NoiseHistogramReportEntry 21 }

dot11NoiseHistogramRprtVendorSpecific OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..255))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute provides an envelope for any optional vendor spe-

cific subelements which may be included in a measurement report

element. Zero length is the null default for this attribute."

DEFVAL { ''H }

::= { dot11NoiseHistogramReportEntry 22 }

dot11NoiseHistogramRprtMeasurementMode OBJECT-TYPE

SYNTAX INTEGER {

success(0),

incapableBit(1),

refusedBit(2),

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the outcome status for the measurement

request which generated this measurement report; status is indi-

cated using the following reason codes: 1 indicates this STA is

incapable of generating the report, 2 indicates this STA is refus-

ing to generate the report, 0 indicates the STA successfully car-

ried out the measurement request."

DEFVAL { 0 }

::= { dot11NoiseHistogramReportEntry 23 }

-- ********************************************************************

-- * End of dot11NoiseHistogramReport TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11BeaconReport TABLE

-- ********************************************************************

dot11BeaconReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11BeaconReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This table contains the current list of Beacon reports that have

been received by the MLME. The report tables shall be maintained

as FIFO to preserve freshness, thus the rows in this table can be

deleted for memory constraints or other implementation con-

straints determined by the vendor. New rows shall have different

RprtIndex values than those deleted within the range limitation of

the index. One easy way is to monotonically increase RprtIndex for

new reports being written in the table."

::= { dot11RMReport 3 }

dot11BeaconReportEntry OBJECT-TYPE

SYNTAX Dot11BeaconReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11BeaconReportTable Indexed by

dot11BeaconRprtIndex."

INDEX { dot11BeaconRprtIndex }

::= { dot11BeaconReportTable 1 }

Dot11BeaconReportEntry ::=

SEQUENCE {

dot11BeaconRprtIndex	Unsigned32,

dot11BeaconRprtRqstToken	OCTET STRING,

dot11BeaconRprtIfIndex	INTEGER,

dot11BeaconMeasuringSTAAddr	MacAddress,

dot11BeaconRprtChanNumber	INTEGER,

dot11BeaconRprtRegulatoryClass	INTEGER,

dot11BeaconRprtActualStartTime	TSFType,

dot11BeaconRprtMeasurementDuration	Unsigned32,

dot11BeaconRprtPhyType	INTEGER,

dot11BeaconRprtReportedFrameType	INTEGER,

dot11BeaconRprtRCPI	INTEGER,

dot11BeaconRprtRSNI	INTEGER,

dot11BeaconRprtBSSID	MacAddress,

dot11BeaconRprtAntennaID	INTEGER,

dot11BeaconRprtParentTSF	TSFType,

dot11BeaconRprtReportedFrameBody	OCTET STRING,

dot11BeaconRprtVendorSpecific	OCTET STRING,

dot11BeaconRprtMeasurementMode	INTEGER}

dot11BeaconRprtIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for Beacon Report elements in dot11BeaconReportTable,

greater than 0."

::= { dot11BeaconReportEntry 1 }

dot11BeaconRprtRqstToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the request token that was indicated in

the measurement request that generated this measurement report.

This should be an exact match to the original dot11RMRqstToken

attribute. Note that there may be multiple entries in the table

that match this value since a single request may generate multi-

ple measurement reports."

::= { dot11BeaconReportEntry 2 }

dot11BeaconRprtIfIndex OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The ifIndex of the interface over which this Beacon Report was

received."

::= { dot11BeaconReportEntry 3 }

dot11BeaconMeasuringSTAAddr OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The MAC address of the measuring STA for this row of Beacon

report."

::= { dot11BeaconReportEntry 4 }

dot11BeaconRprtChanNumber OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the channel number used for this Beacon

Report. The Channel Number is only defined within the indicated

Regulatory Class for this measurement report."

::= { dot11BeaconReportEntry 5 }

dot11BeaconRprtRegulatoryClass OBJECT-TYPE

SYNTAX INTEGER(1..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the channel set for this measurement

report. Country, Regulatory Class and Channel Number together

specify the channel frequency and spacing for this measurement

report. Valid values of Regulatory Class are shown in Annex J."

REFERENCE

"Annex J"

::= { dot11BeaconReportEntry 6 }

dot11BeaconRprtActualStartTime OBJECT-TYPE

SYNTAX TSFType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the TSF value at the time when the

measurement started."

::= { dot11BeaconReportEntry 7 }

dot11BeaconRprtMeasurementDuration OBJECT-TYPE

SYNTAX Unsigned32

UNITS "TUs"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the duration over which the Beacon

Report was measured."

::= { dot11BeaconReportEntry 8 }

dot11BeaconRprtPhyType OBJECT-TYPE

SYNTAX INTEGER {

fhss(1),

dsss(2),

irbaseband(3),

ofdm(4),

hrdsss(5),

erp(6)

}

UNITS "dot11PHYType"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the PHY Type for this row of Beacon

Report."

::= { dot11BeaconReportEntry 9 }

dot11BeaconRprtReportedFrameType OBJECT-TYPE

SYNTAX INTEGER {

beaconOrProbeResponse(0),

measurementPilot(1)

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the frame type reported in

dot11BeaconRprtReportedFrameBody"

::= { dot11BeaconReportEntry 10 }

dot11BeaconRprtRCPI OBJECT-TYPE

SYNTAX INTEGER(0..255)

UNITS "0.5 dBm"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the received channel power of the bea-

con or probe response frame in dBm, as defined in the RCPI mea-

surement subclause for the indicated PHY Type.

RCPIval = Int[(RCPIpower in dBm + 110)*2], for RCPI in the range -

110 dBm to 0 dBm. RCPIval = 220 for RCPI > 0 dBm. RCPIval = 255

when RCPI is not available."

::= { dot11BeaconReportEntry 11 }

dot11BeaconRprtRSNI OBJECT-TYPE

SYNTAX INTEGER(0..255)

UNITS "0.5 dB"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the received signal to noise ratio of

the beacon or probe response frame in dB. RSNI is the received

signal to noise plus interference ratio derived from the measured

RCPI for the received frame and from the measured ANPI for the

channel used to receive the frame. RSNI is calculated by the ratio

of  the received signal power (RCPI - ANPI) over the noise plus

interference power (ANPI) where

RSNI = [(ratio(dB) + 10) * 2], for ratios in the range -10dB to

+118dB.

"

::= { dot11BeaconReportEntry 12 }

dot11BeaconRprtBSSID OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the BSSID of the beacon for this row of

Beacon Report."

::= { dot11BeaconReportEntry 13 }

dot11BeaconRprtAntennaID OBJECT-TYPE

SYNTAX INTEGER(0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the identifying number for the antenna

used for this measurement. The value 0 indicates that the antenna

identifier is unknown.  The value 255 indicates that this measure-

ment was made with multiple antennas. The value 1 is used for a

STA with only one antenna.  STAs with more than one antenna shall

assign Antenna IDs to each antenna as consecutive, ascending num-

bers. Each Antenna ID number represents a unique antenna charac-

terized by a fixed relative position, a fixed relative direction

and a peak gain for that position and direction."

::= { dot11BeaconReportEntry 14 }

dot11BeaconRprtParentTSF OBJECT-TYPE

SYNTAX TSFType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the TSF value of the serving measuring

STA’s TSF value at the time the measuring STA received the beacon

or probe response frame."

::= { dot11BeaconReportEntry 15 }

dot11BeaconRprtReportedFrameBody OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..100))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the fixed fields and elements from the

frame body of the Beacon, Measurement Pilot or Probe Response

frame being received. All reported TIM elements are truncated to 4

octets."

::= { dot11BeaconReportEntry 16 }

dot11BeaconRprtVendorSpecific OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..255))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute provides an envelope for any optional vendor spe-

cific subelements which may be included in a measurement report

element. Zero length is the null default for this attribute."

DEFVAL { ''H }

::= { dot11BeaconReportEntry 17 }

dot11BeaconRprtMeasurementMode OBJECT-TYPE

SYNTAX INTEGER {

success(0),

incapableBit(1),

refusedBit(2),

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the outcome status for the measurement

request which generated this measurement report; status is indi-

cated using the following reason codes: 1 indicates this STA is

incapable of generating the report, 2 indicates this STA is refus-

ing to generate the report, 0 indicates the STA successfully car-

ried out the measurement request."

DEFVAL { 0 }

::= { dot11BeaconReportEntry 18 }

-- ********************************************************************

-- * End of dot11BeaconReport TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11FrameReport TABLE

-- ********************************************************************

dot11FrameReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11FrameReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This table contains the current list of Frame reports that have

been received by the MLME. The report tables shall be maintained

as a FIFO to preserve freshness, thus the rows in this table can

be deleted for memory constraints or other implementation con-

straints determined by the vendor. New rows shall have different

RprtIndex values than those deleted within the range limitation of

the index. One easy way is to monotonically increase RprtIndex for

new reports being written in the table."

::= { dot11RMReport 4 }

dot11FrameReportEntry OBJECT-TYPE

SYNTAX Dot11FrameReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11FrameReportTable Indexed by

dot11FrameRprtIndex."

INDEX { dot11FrameRprtIndex }

::= { dot11FrameReportTable 1 }

Dot11FrameReportEntry ::=

SEQUENCE {

dot11FrameRprtIndex	Unsigned32,

dot11FrameRprtIfIndex	INTEGER,

dot11FrameRprtRqstToken	Unsigned32,

dot11FrameRprtChanNumber	INTEGER,

dot11FrameRprtRegulatoryClass	INTEGER,

dot11FrameRprtActualStartTime	TSFType,

dot11FrameRprtMeasurementDuration	Unsigned32,

dot11FrameRprtTransmitSTAAddress	MacAddress,

dot11FrameRprtBSSID	MacAddress,

dot11FrameRprtPhyType	INTEGER,

dot11FrameRprtAvgRCPI	INTEGER,

dot11FrameRprtRSNI	INTEGER,

dot11FrameRprtLastRCPI	INTEGER,

dot11FrameRprtAntennaID	INTEGER,

dot11FrameRprtNumberFrames	INTEGER,

dot11FrameRprtVendorSpecific	OCTET STRING,

dot11FrameRptMeasurementMode	INTEGER}

dot11FrameRprtIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for Frame Report elements in dot11FrameReportTable, greater

than 0."

::= { dot11FrameReportEntry 1 }

dot11FrameRprtIfIndex OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The ifIndex of the interface over which this Frame Report was

received."

::= { dot11FrameReportEntry 2 }

dot11FrameRprtRqstToken OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Index for Frame Request elements in dot11FrameRequestTable that

corresponds to this row of frame report. Since a single frame

request can generate multiple rows in the frame report table, one

per BSSID, this dot11FrameRprtRqstToken indicates which request

this particular row indicates. If this row of report is received

without a particular request, this attribute should be 0"

::= { dot11FrameReportEntry 3 }

dot11FrameRprtChanNumber OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the channel number used for this Frame

Report. The Channel Number is only defined within the indicated

Regulatory Class for this measurement report."

::= { dot11FrameReportEntry 4 }

dot11FrameRprtRegulatoryClass OBJECT-TYPE

SYNTAX INTEGER(1..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the channel set for this measurement

report. Country, Regulatory Class and Channel Number together

specify the channel frequency and spacing for this measurement

report. Valid values of Regulatory Class are shown in Annex J."

REFERENCE

"Annex J"

::= { dot11FrameReportEntry 5 }

dot11FrameRprtActualStartTime OBJECT-TYPE

SYNTAX TSFType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the TSF value at the time when measure-

ment started."

::= { dot11FrameReportEntry 6 }

dot11FrameRprtMeasurementDuration OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the duration over which the Frame Report

was measured, expressed in units of TUs."

::= { dot11FrameReportEntry 7 }

dot11FrameRprtTransmitSTAAddress OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The MAC address of STA for this row of Frame report that it has

been received from."

::= { dot11FrameReportEntry 8 }

dot11FrameRprtBSSID OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the BSSID of the STA that transmitted

this frame."

::= { dot11FrameReportEntry 9 }

dot11FrameRprtPhyType OBJECT-TYPE

SYNTAX INTEGER {

fhss(1),

dsss(2),

irbaseband(3),

ofdm(4),

hrdsss(5),

erp(6)

}

UNITS "dot11PHYType"

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the PHY used for frame reception in this

row of the frame report."

::= { dot11FrameReportEntry 10 }

dot11FrameRprtAvgRCPI OBJECT-TYPE

SYNTAX INTEGER(0..255)

UNITS "0.5 dBm"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the average value for the received chan-

nel power of all the frames received and counted in this Frame

Report Entry, in dBm, as defined in the RCPI measurement sub-

clause for the indicated PHY Type.

RCPIval = Int[(RCPIpower in dBm + 110)*2], for RCPI in the range -

110 dBm to 0 dBm. RCPIval = 220 for RCPI > 0 dBm. RCPIval = 255

when RCPI is not available."

::= { dot11FrameReportEntry 11 }

dot11FrameRprtRSNI OBJECT-TYPE

SYNTAX INTEGER(0..255)

UNITS "0.5 dB"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the received signal to noise ratio of

the received frame in dBm. RSNI is the received signal to noise

plus interference ratio derived from the RCPI for the received

frame and from the most recent ANPI value measured on the channel

used to receive the frame. RSNI may be calculated by the ratio of

the received signal power (RCPI - ANPI) over the noise plus inter-

ference power (ANPI) where RSNI = [(ratio(dB) + 10) * 2],  for

ratios in the range -10dB to +118dB.  Other measurement tech-

niques are allowed."

::= { dot11FrameReportEntry 12 }

dot11FrameRprtLastRCPI OBJECT-TYPE

SYNTAX INTEGER(0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the received channel power of the most

recently measured frame in this Frame Report entry, in dBm, as

defined in the RCPI measurement subclause for the indicated PHY

Type.

RCPIval = Int[(RCPIpower in dBm + 110)*2], for RCPI in the range -

110 dBm to 0 dBm. RCPIval = 220 for RCPI > 0 dBm. RCPIval = 255

when RCPI is not available."

::= { dot11FrameReportEntry 13 }

dot11FrameRprtAntennaID OBJECT-TYPE

SYNTAX INTEGER(0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the identifying number for the antenna

used for this measurement. The value 0 indicates that the antenna

identifier is unknown.  The value 255 indicates that this measure-

ment was made with multiple antennas. The value 1 is used for a

STA with only one antenna.  STAs with more than one antenna shall

assign Antenna IDs to each antenna as consecutive, ascending num-

bers. Each  Antenna ID number represents a unique antenna charac-

terized by a fixed relative position, a fixed relative direction

and a peak gain for that position and direction."

::= { dot11FrameReportEntry 14 }

dot11FrameRprtNumberFrames  OBJECT-TYPE

SYNTAX INTEGER(0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the number of received frames in the

measurement Report Frame for this row of the Frame Report."

::= { dot11FrameReportEntry 15 }

dot11FrameRprtVendorSpecific OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..255))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute provides an envelope for any optional vendor spe-

cific subelements which may be included in a measurement report

element. Zero length is the null default for this attribute."

DEFVAL { ''H }

::= { dot11FrameReportEntry 16 }

dot11FrameRptMeasurementMode OBJECT-TYPE

SYNTAX INTEGER {

success(0),

incapableBit(1),

refusedBit(2),

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the outcome status for the measurement

request which generated this measurement report; status is indi-

cated using the following reason codes: 1 indicates this STA is

incapable of generating the report, 2 indicates this STA is refus-

ing to generate the report, 0 indicates the STA successfully car-

ried out the measurement request."

DEFVAL { 0 }

::= { dot11FrameReportEntry 17 }

-- ********************************************************************

-- * End of dot11FrameReport TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11STAStatisticsReport TABLE

-- ********************************************************************

dot11STAStatisticsReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11STAStatisticsReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This table contains the current list of STA Statistics reports

that have been received by the MLME. The report tables shall be

maintained as a FIFO to preserve freshness, thus the rows in this

table can be deleted for memory constraints or other implementa-

tion constraints determined by the vendor. New rows shall have

different RprtIndex values than those deleted within the range

limitation of the index. One easy way is to monotonically increase

RprtIndex for new reports being written in the table."

::= { dot11RMReport 5 }

dot11STAStatisticsReportEntry OBJECT-TYPE

SYNTAX Dot11STAStatisticsReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11STAStatisticsReportTable Indexed by

dot11STAStatisticsReportIndex."

INDEX { dot11STAStatisticsReportIndex }

::= { dot11STAStatisticsReportTable 1 }

Dot11STAStatisticsReportEntry ::=

SEQUENCE {

dot11STAStatisticsReportIndex	Unsigned32,

dot11STAStatisticsReportToken	OCTET STRING,

dot11STAStatisticsIfIndex	INTEGER,

dot11STAStatisticsSTAAddress	MacAddress,

dot11STAStatisticsMeasurementDuration	Unsigned32,

dot11STAStatisticsGroupID	INTEGER,

dot11STAStatisticsTransmittedFragmentCount	Counter32,

dot11STAStatisticsMulticastTransmittedFrameCount	Counter32,

dot11STAStatisticsFailedCount	Counter32,

dot11STAStatisticsRetryCount	Counter32,

dot11STAStatisticsMultipleRetryCount	Counter32,

dot11STAStatisticsFrameDuplicateCount	Counter32,

dot11STAStatisticsRTSSuccessCount	Counter32,

dot11STAStatisticsRTSFailureCount	Counter32,

dot11STAStatisticsACKFailureCount	Counter32,

dot11STAStatisticsQosTransmittedFragmentCount	Counter32,

dot11STAStatisticsQosFailedCount	Counter32,

dot11STAStatisticsQosRetryCount	Counter32,

dot11STAStatisticsQosMultipleRetryCount	Counter32,

dot11STAStatisticsQosFrameDuplicateCount	Counter32,

dot11STAStatisticsQosRTSSuccessCount	Counter32,

dot11STAStatisticsQosRTSFailureCount	Counter32,

dot11STAStatisticsQosACKFailureCount	Counter32,

dot11STAStatisticsQosReceivedFragmentCount	Counter32,

dot11STAStatisticsQosTransmittedFrameCount	Counter32,

dot11STAStatisticsQosDiscardedFrameCount	Counter32,

dot11STAStatisticsQosMPDUsReceivedCount	Counter32,

dot11STAStatisticsQosRetriesReceivedCount	Counter32,

dot11STAStatisticsReceivedFragmentCount	Counter32,

dot11STAStatisticsMulticastReceivedFrameCount	Counter32,

dot11STAStatisticsFCSErrorCount	Counter32,

dot11STAStatisticsTransmittedFrameCount	Counter32,

dot11STAStatisticsAPAverageAccessDelay	INTEGER,

dot11STAStatisticsAverageAccessDelayBestEffort	INTEGER,

dot11STAStatisticsAverageAccessDelayBackground	INTEGER,

dot11STAStatisticsAverageAccessDelayVideo	INTEGER,

dot11STAStatisticsAverageAccessDelayVoice	INTEGER,

dot11STAStatisticsStationCount	INTEGER,

dot11STAStatisticsChannelUtilization	INTEGER,

dot11STAStatisticsVendorSpecific	OCTET STRING,

dot11STAStatisticsRprtMeasurementMode	INTEGER}

dot11STAStatisticsReportIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for STA Statistics Report elements in

dot11STAStatisticsReportTable, greater than 0."

::= { dot11STAStatisticsReportEntry 1 }

dot11STAStatisticsReportToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the token that was indicated in the mea-

surement request that generated this measurement report. This

should be an exact match to the original dot11RMRqstToken

attribute. Note that there may be multiple entries in the table

that match this value since a single request may generate multi-

ple measurement reports."

::= { dot11STAStatisticsReportEntry 2 }

dot11STAStatisticsIfIndex OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Identifies the Interface that this row of STA Statistics Report

has been received on"

::= { dot11STAStatisticsReportEntry 3 }

dot11STAStatisticsSTAAddress OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The MAC address of the STA that returned this STA Statistics

Report."

::= { dot11STAStatisticsReportEntry 4 }

dot11STAStatisticsMeasurementDuration OBJECT-TYPE

SYNTAX Unsigned32

UNITS "TUs"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the duration over which the STA Statis-

tics was measured.  A zero value for this attribute indicates that

the reported statistics are a current snapshot of the statistics

variables.  A non-zero value for this attribute indicates that the

reported statistics contain the difference in the corresponding

statistics variables over the indicated duration."

::= { dot11STAStatisticsReportEntry 5 }

dot11STAStatisticsGroupID OBJECT-TYPE

SYNTAX INTEGER {

dot11CountersTable(0),

dot11MacStatistics(1),

dot11QosCountersTableforUP0(2),

dot11QosCountersTableforUP1(3),

dot11QosCountersTableforUP2(4),

dot11QosCountersTableforUP3(5),

dot11QosCountersTableforUP4(6),

dot11QosCountersTableforUP5(7),

dot11QosCountersTableforUP6(8),

dot11QosCountersTableforUP7(9),

bSSAverageAccessDelays(10)

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the value of

dot11RMRqstSTAStatRqstGroupID returned from the STA in this STA

Statistics Report."

DEFVAL { 0 }

::= { dot11STAStatisticsReportEntry 6 }

dot11STAStatisticsTransmittedFragmentCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of dot11TransmittedFragmentCount returned from

the STA in this STA Statistics Report. If

dot11STAStatisticsMeasurementDuration indicates a non-zero value,

this attribute indicates the difference in the referenced dot11

variable over the indicated duration. This attribute is only valid

if the dot11STAStatisticsGroupID is 0, and is ignored otherwise."

::= { dot11STAStatisticsReportEntry 7 }

dot11STAStatisticsMulticastTransmittedFrameCount OBJECT-TYPE SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of dot11MulticastTransmittedFrameCount

returned from the STA in this STA Statistics Report. If

dot11STAStatisticsMeasurementDuration indicates a non-zero value,

this attribute indicates the difference in the referenced dot11

variable over the indicated duration. This attribute is only valid

if the dot11STAStatisticsGroupID is 0, and is ignored otherwise."

::= { dot11STAStatisticsReportEntry 8 }

dot11STAStatisticsFailedCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of dot11FailedCount returned from the STA in

this STA Statistics Report. If

dot11STAStatisticsMeasurementDuration indicates a non-zero value,

this attribute indicates the difference in the referenced dot11

variable over the indicated duration. This attribute is only valid

if the dot11STAStatisticsGroupID is 0, and is ignored otherwise."

::= { dot11STAStatisticsReportEntry 9 }

dot11STAStatisticsRetryCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of dot11RetryCount returned from the STA in

this STA Statistics Report. If

dot11STAStatisticsMeasurementDuration indicates a non-zero value,

this attribute indicates the difference in the referenced dot11

variable over the indicated duration. This attribute is only valid

if the dot11STAStatisticsGroupID is 1, and is ignored otherwise."

::= { dot11STAStatisticsReportEntry 10 }

dot11STAStatisticsMultipleRetryCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of dot11MultipleRetryCount returned from the

STA in this STA Statistics Report. If

dot11STAStatisticsMeasurementDuration indicates a non-zero value,

this attribute indicates the difference in the referenced dot11

variable over the indicated duration. This attribute is only valid

if the dot11STAStatisticsGroupID is 1, and is ignored otherwise."

::= { dot11STAStatisticsReportEntry 11 }

dot11STAStatisticsFrameDuplicateCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of dot11FrameDuplicateCount returned from the

STA in this STA Statistics Report. If

dot11STAStatisticsMeasurementDuration indicates a non-zero value,

this attribute indicates the difference in the referenced dot11

variable over the indicated duration. This attribute is only valid

if the dot11STAStatisticsGroupID is 1, and is ignored otherwise."

::= { dot11STAStatisticsReportEntry 12 }

dot11STAStatisticsRTSSuccessCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of dot11RTSSuccessCount returned from the STA

in this STA Statistics Report. If

dot11STAStatisticsMeasurementDuration indicates a non-zero value,

this attribute indicates the difference in the referenced dot11

variable over the indicated duration. This attribute is only valid

if the dot11STAStatisticsGroupID is 1, and is ignored otherwise."

::= { dot11STAStatisticsReportEntry 13 }

dot11STAStatisticsRTSFailureCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of dot11RTSFailureCount returned from the STA

in this STA Statistics Report. If

dot11STAStatisticsMeasurementDuration indicates a non-zero value,

this attribute indicates the difference in the referenced dot11

variable over the indicated duration. This attribute is only valid

if the dot11STAStatisticsGroupID is 1, and is ignored otherwise."

::= { dot11STAStatisticsReportEntry 14 }

dot11STAStatisticsACKFailureCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of dot11ACKFailureCount returned from the STA

in this STA Statistics Report. If

dot11STAStatisticsMeasurementDuration indicates a non-zero value,

this attribute indicates the difference in the referenced dot11

variable over the indicated duration. This attribute is only valid

if the dot11STAStatisticsGroupID is 1, and is ignored otherwise."

::= { dot11STAStatisticsReportEntry 15 }

dot11STAStatisticsQosTransmittedFragmentCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of dot11QosTransmittedFragmentCount returned

from the STA in this STA Statistics Report. If

dot11STAStatisticsMeasurementDuration indicates a non-zero value,

this attribute indicates the difference in the referenced dot11

variable over the indicated duration. This attribute is only valid

if the dot11STAStatisticsGroupID is 2-9, and is ignored other-

wise."

::= { dot11STAStatisticsReportEntry 16 }

dot11STAStatisticsQosFailedCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of dot11QosFailedCount returned from the STA

in this STA Statistics Report. If

dot11STAStatisticsMeasurementDuration indicates a non-zero value,

this attribute indicates the difference in the referenced dot11

variable over the indicated duration. This attribute is only valid

if the dot11STAStatisticsGroupID is 2-9, and is ignored other-

wise."

::= { dot11STAStatisticsReportEntry 17 }

dot11STAStatisticsQosRetryCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of dot11QosRetryCount returned from the STA in

this STA Statistics Report. If

dot11STAStatisticsMeasurementDuration indicates a non-zero value,

this attribute indicates the difference in the referenced dot11

variable over the indicated duration. This attribute is only valid

if the dot11STAStatisticsGroupID is 2-9, and is ignored other-

wise."

::= { dot11STAStatisticsReportEntry 18 }

dot11STAStatisticsQosMultipleRetryCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of dot11QosMultipleRetryCount returned from

the STA in this STA Statistics Report. If

dot11STAStatisticsMeasurementDuration indicates a non-zero value,

this attribute indicates the difference in the referenced dot11

variable over the indicated duration. This attribute is only valid

if the dot11STAStatisticsGroupID is 2-9, and is ignored other-

wise."

::= { dot11STAStatisticsReportEntry 19 }

dot11STAStatisticsQosFrameDuplicateCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of dot11QosFrameDuplicateCount returned from

the STA in this STA Statistics Report. If

dot11STAStatisticsMeasurementDuration indicates a non-zero value,

this attribute indicates the difference in the referenced dot11

variable over the indicated duration. This attribute is only valid

if the dot11STAStatisticsGroupID is 2-9, and is ignored other-

wise."

::= { dot11STAStatisticsReportEntry 20 }

dot11STAStatisticsQosRTSSuccessCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of dot11QosRTSSuccessCount returned from the

STA in this STA Statistics Report. If

dot11STAStatisticsMeasurementDuration indicates a non-zero value,

this attribute indicates the difference in the referenced dot11

variable over the indicated duration. This attribute is only valid

if the dot11STAStatisticsGroupID is 2-9, and is ignored other-

wise."

::= { dot11STAStatisticsReportEntry 21 }

dot11STAStatisticsQosRTSFailureCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of dot11QosRTSFailureCount returned from the

STA in this STA Statistics Report. If

dot11STAStatisticsMeasurementDuration indicates a non-zero value,

this attribute indicates the difference in the referenced dot11

variable over the indicated duration. This attribute is only valid

if the dot11STAStatisticsGroupID is 2-9, and is ignored other-

wise."

::= { dot11STAStatisticsReportEntry 22 }

dot11STAStatisticsQosACKFailureCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of dot11QosACKFailureCount returned from the

STA in this STA Statistics Report. If

dot11STAStatisticsMeasurementDuration indicates a non-zero value,

this attribute indicates the difference in the referenced dot11

variable over the indicated duration. This attribute is only valid

if the dot11STAStatisticsGroupID is 2-9, and is ignored other-

wise."

::= { dot11STAStatisticsReportEntry 23 }

dot11STAStatisticsQosReceivedFragmentCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of dot11QosReceivedFragmentCount returned from

the STA in this STA Statistics Report. If

dot11STAStatisticsMeasurementDuration indicates a non-zero value,

this attribute indicates the difference in the referenced dot11

variable over the indicated duration. This attribute is only valid

if the dot11STAStatisticsGroupID is 2-9, and is ignored other-

wise."

::= { dot11STAStatisticsReportEntry 24 }

dot11STAStatisticsQosTransmittedFrameCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of dot11QosTransmittedFrameCount returned from

the STA in this STA Statistics Report. If

dot11STAStatisticsMeasurementDuration indicates a non-zero value,

this attribute indicates the difference in the referenced dot11

variable over the indicated duration. This attribute is only valid

if the dot11STAStatisticsGroupID is 2-9, and is ignored other-

wise."

::= { dot11STAStatisticsReportEntry 25 }

dot11STAStatisticsQosDiscardedFrameCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of dot11QosDiscardedFrameCount returned from

the STA in this STA Statistics Report. If

dot11STAStatisticsMeasurementDuration indicates a non-zero value,

this attribute indicates the difference in the referenced dot11

variable over the indicated duration. This attribute is only valid

if the dot11STAStatisticsGroupID is 2-9, and is ignored other-

wise."

::= { dot11STAStatisticsReportEntry 26 }

dot11STAStatisticsQosMPDUsReceivedCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of dot11QosMPDUsReceivedCount returned from

the STA in this STA Statistics Report. If

dot11STAStatisticsMeasurementDuration indicates a non-zero value,

this attribute indicates the difference in the referenced dot11

variable over the indicated duration. This attribute is only valid

if the dot11STAStatisticsGroupID is 2-9, and is ignored other-

wise."

::= { dot11STAStatisticsReportEntry 27 }

dot11STAStatisticsQosRetriesReceivedCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of dot11QosRetriesReceivedCount returned from

the STA in this STA Statistics Report. If

dot11STAStatisticsMeasurementDuration indicates a non-zero value,

this attribute indicates the difference in the referenced dot11

variable over the indicated duration. This attribute is only valid

if the dot11STAStatisticsGroupID is 2-9, and is ignored other-

wise."

::= { dot11STAStatisticsReportEntry 28 }

dot11STAStatisticsReceivedFragmentCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of dot11ReceivedFragmentCount returned from

the STA in this STA Statistics Report. If

dot11STAStatisticsMeasurementDuration indicates a non-zero value,

this attribute indicates the difference in the referenced dot11

variable over the indicated duration. This attribute is only valid

if the dot11STAStatisticsGroupID is 0, and is ignored otherwise."

::= { dot11STAStatisticsReportEntry 29 }

dot11STAStatisticsMulticastReceivedFrameCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of dot11MulticastReceivedFrameCount returned

from the STA in this STA Statistics Report. If

dot11STAStatisticsMeasurementDuration indicates a non-zero value,

this attribute indicates the difference in the referenced dot11

variable over the indicated duration. This attribute is only valid

if the dot11STAStatisticsGroupID is 0, and is ignored otherwise."

::= { dot11STAStatisticsReportEntry 30 }

dot11STAStatisticsFCSErrorCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of dot11FCSErrorCount returned from the STA in

this STA Statistics Report. If

dot11STAStatisticsMeasurementDuration indicates a non-zero value,

this attribute indicates the difference in the referenced dot11

variable over the indicated duration. This attribute is only valid

if the dot11STAStatisticsGroupID is 0, and is ignored otherwise."

::= { dot11STAStatisticsReportEntry 31 }

dot11STAStatisticsTransmittedFrameCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of dot11TransmittedFrameCount returned from

the STA in this STA Statistics Report. If

dot11STAStatisticsMeasurementDuration indicates a non-zero value,

this attribute indicates the difference in the referenced dot11

variable over the indicated duration. This attribute is only valid

if the dot11STAStatisticsGroupID is 0, and is ignored otherwise."

::= { dot11STAStatisticsReportEntry 32 }

dot11STAStatisticsAPAverageAccessDelay OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of the AP Average Access Delay (AAD)  returned

from the STA in this STA Statistics Report. If

dot11STAStatisticsMeasurementDuration indicates a non-zero value,

this attribute indicates the difference in the referenced access

delay value over the indicated duration. This attribute is only

valid if the dot11STAStatisticsGroupID is 10, and is ignored oth-

erwise."

REFERENCE

"IEEE 802.11 Subclause 7.3.2.39 (BSS Average Access Delay ele-

ment(11k))"

::= { dot11STAStatisticsReportEntry 33 }

dot11STAStatisticsAverageAccessDelayBestEffort OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of the Average Access Delay (AAD) for the Best

Effort Access Category returned from the STA in this STA Statis-

tics Report. If dot11STAStatisticsMeasurementDuration indicates a

non-zero value, this attribute indicates the difference in the

referenced access delay value over the indicated duration. This

attribute is only valid if the dot11STAStatisticsGroupID is 10,

and is ignored otherwise."

REFERENCE

"IEEE 802.11  Subclause 7.3.2.44 (BSS AC Access Delay ele-

ment(11k))"

::= { dot11STAStatisticsReportEntry 34 }

dot11STAStatisticsAverageAccessDelayBackground OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of the Average Access Delay (AAD) for the

Background Access Category returned from the STA in this STA Sta-

tistics Report. If dot11STAStatisticsMeasurementDuration indi-

cates a non-zero value, this attribute indicates the difference in

the referenced access delay value over the indicated duration.

This attribute is only valid if the dot11STAStatisticsGroupID is

10, and is ignored otherwise."

REFERENCE

"IEEE 802.11 Subclause 7.3.2.44 (BSS AC Access Delay ele-

ment(11k))"

::= { dot11STAStatisticsReportEntry 35 }

dot11STAStatisticsAverageAccessDelayVideo OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of the Average Access Delay (AAD) for the

Video Access Category returned from the STA in this STA Statis-

tics Report. If dot11STAStatisticsMeasurementDuration indicates a

non-zero value, this attribute indicates the difference in the

referenced access delay value over the indicated duration. This

attribute is only valid if the dot11STAStatisticsGroupID is 10,

and is ignored otherwise."

REFERENCE

"IEEE 802.11 Subclause 7.3.2.44 (BSS AC Access Delay ele-

ment(11k))"

::= { dot11STAStatisticsReportEntry 36 }

dot11STAStatisticsAverageAccessDelayVoice OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of the Average Access Delay (AAD) for the

Voice Access Category returned from the STA in this STA Statis-

tics Report. If dot11STAStatisticsMeasurementDuration indicates a

non-zero value, this attribute indicates the difference in the

referenced access delay value over the indicated duration. This

attribute is only valid if the dot11STAStatisticsGroupID is 10,

and is ignored otherwise."

REFERENCE

"IEEE 802.11 Subclause 7.3.2.44 (BSS AC Access Delay ele-

ment(11k))"

::= { dot11STAStatisticsReportEntry 37 }

dot11STAStatisticsStationCount OBJECT-TYPE

SYNTAX INTEGER (0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of dot11AssociatedStationCount returned from

the STA in this STA Statistics Report. If

dot11STAStatisticsMeasurementDuration indicates a non-zero value,

this attribute indicates the difference in the referenced dot11

variable over the indicated duration. This attribute is only valid

if the dot11STAStatisticsGroupID is 10, and is ignored otherwise."

::= { dot11STAStatisticsReportEntry 38 }

dot11STAStatisticsChannelUtilization OBJECT-TYPE

SYNTAX INTEGER (0..255)

UNITS "1/255"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"If dot11STAStatisticsMeasurementDuration is zero, this attribute

indicates the value of the Channel Utilization returned from the

STA in this STA Statistics Report. If

dot11STAStatisticsMeasurementDuration indicates a non-zero value,

this attribute indicates the difference in the Channel Utiliza-

tion value over the indicated duration. The Channel Utilization is

the time fraction during which the AP sensed the channel busy.

This attribute is only valid if the dot11STAStatisticsGroupID is

10, and is ignored otherwise."

REFERENCE

"IEEE 802.11 Subclause 7.3.2.28 (BSS Load element)"

::= { dot11STAStatisticsReportEntry 39 }

dot11STAStatisticsVendorSpecific OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..255))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute provides an envelope for any optional vendor spe-

cific subelements which may be included in a measurement report

element. Zero length is the null default for this attribute."

DEFVAL { ''H }

::= { dot11STAStatisticsReportEntry 40 }

dot11STAStatisticsRprtMeasurementMode OBJECT-TYPE

SYNTAX INTEGER {

success(0),

incapableBit(1),

refusedBit(2),

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the outcome status for the measurement

request which generated this measurement report; status is indi-

cated using the following reason codes: 1 indicates this STA is

incapable of generating the report, 2 indicates this STA is refus-

ing to generate the report, 0 indicates the STA successfully car-

ried out the measurement request."

DEFVAL { 0 }

::= { dot11STAStatisticsReportEntry 41 }

-- ********************************************************************

-- * End of dot11STAStatisticsReport TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11LCIReport TABLE

-- ********************************************************************

dot11LCIReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11LCIReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This table contains the current list of LCI reports that have

been received by the MLME. The report tables shall be maintained

as a FIFO to preserve freshness, thus the rows in this table can

be deleted for memory constraints or other implementation con-

straints determined by the vendor. New rows shall have different

RprtIndex values than those deleted within the range limitation of

the index. One easy way is to monotonically increase RprtIndex for

new reports being written in the table."

::= { dot11RMReport 6 }

dot11LCIReportEntry OBJECT-TYPE

SYNTAX Dot11LCIReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11LCIReportTable Indexed by

dot11LCIReportIndex."

INDEX { dot11LCIReportIndex }

::= { dot11LCIReportTable 1 }

Dot11LCIReportEntry ::=

SEQUENCE {

dot11LCIReportIndex	Unsigned32,

dot11LCIReportToken	OCTET STRING,

dot11LCIIfIndex	INTEGER,

dot11LCISTAAddress	MacAddress,

dot11LCILatitudeResolution	INTEGER,

dot11LCILatitudeInteger	INTEGER,

dot11LCILatitudeFraction	INTEGER,

dot11LCILongitudeResolution	INTEGER,

dot11LCILongitudeInteger	INTEGER,

dot11LCILongitudeFraction	INTEGER,

dot11LCIAltitudeType	INTEGER,

dot11LCIAltitudeResolution	INTEGER,

dot11LCIAltitudeInteger	INTEGER,

dot11LCIAltitudeFraction	INTEGER,

dot11LCIDatum	INTEGER,

dot11LCIAzimuthType	INTEGER,

dot11LCIAzimuthResolution	INTEGER,

dot11LCIAzimuth	Integer32,

dot11LCIVendorSpecific	OCTET STRING,

dot11LCIRprtMeasurementMode	INTEGER}

dot11LCIReportIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for LCI Report elements in dot11LCIReportTable, greater

than 0."

::= { dot11LCIReportEntry 1 }

dot11LCIReportToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the token that was indicated in the mea-

surement request that generated this measurement report. This

should be an exact match to the original dot11RMRqstToken

attribute. Note that there may be multiple entries in the table

that match this value since a single request may generate multi-

ple measurement reports."

::= { dot11LCIReportEntry 2 }

dot11LCIIfIndex OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Identifies the Interface that this row of LCI Report has been

received on"

::= { dot11LCIReportEntry 3 }

dot11LCISTAAddress OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The MAC address of the STA that returned this LCI Report."

::= { dot11LCIReportEntry 4 }

dot11LCILatitudeResolution OBJECT-TYPE

SYNTAX INTEGER (0..63)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the latitude resolution as 6 bits indi-

cating the number of valid bits in the fixed-point value of Lati-

tude. This field is derived from IETF RFC-3825, and is accessed

big-endian."

::= { dot11LCIReportEntry 5 }

dot11LCILatitudeInteger OBJECT-TYPE

SYNTAX INTEGER (-359..359)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the latitude as a 34 bit fixed point

value consisting of 9 bits of integer and 25 bits of fraction.

This field contains the 9 bits of integer portion of Latitude.

This field is derived from IETF RFC-3825, and is accessed big-

endian."

::= { dot11LCIReportEntry 6 }

dot11LCILatitudeFraction OBJECT-TYPE

SYNTAX INTEGER (-16777215..16777215)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the latitude as a 34 bit fixed point

value consisting of 9 bits of integer and 25 bits of fraction.

This field contains the 25 bits of fraction portion of Latitude.

This field is derived from IETF RFC-3825, and is accessed big-

endian."

::= { dot11LCIReportEntry 7 }

dot11LCILongitudeResolution OBJECT-TYPE

SYNTAX INTEGER (0..63)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the longitude resolution as 6 bits indi-

cating the number of valid bits in the fixed-point value of Longi-

tude. This field is derived from IETF RFC-3825, and is accessed

big-endian."

::= { dot11LCIReportEntry 8 }

dot11LCILongitudeInteger OBJECT-TYPE

SYNTAX INTEGER (-359..359)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the longitude as a 34 bit fixed point

value consisting of 9 bits of integer and 25 bits of fraction.

This field contains the 9 bits of integer portion of Longitude.

This field is derived from IETF RFC-3825, and is accessed big-

endian."

::= { dot11LCIReportEntry 9 }

dot11LCILongitudeFraction OBJECT-TYPE

SYNTAX INTEGER (-16777215..16777215)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the longitude as a 34 bit fixed point

value consisting of 9 bits of integer and 25 bits of fraction.

This field contains the 25 bits of fraction portion of Longitude.

This field is derived from IETF RFC-3825, and is accessed big-

endian."

::= { dot11LCIReportEntry 10 }

dot11LCIAltitudeType OBJECT-TYPE

SYNTAX INTEGER {

meters(1),

floors(2) }

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the altitude Type as four bits encoding

the type of altitude. Codes defined are:meters : in 2s-complement

fixed-point 22-bit integer part with 8-bit fraction floors : in

2s-complement fixed-point 22-bit integer part with 8-bit frac-

tion. This field is derived from IETF RFC-3825, and is accessed

big-endian."

::= { dot11LCIReportEntry 11 }

dot11LCIAltitudeResolution OBJECT-TYPE

SYNTAX INTEGER (0..63)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the altitude resolution as 6 bits indi-

cating the number of valid bits in the altitude. This field is

derived from IETF RFC-3825, and is accessed big-endian."

::= { dot11LCIReportEntry 12 }

dot11LCIAltitudeInteger OBJECT-TYPE

SYNTAX INTEGER (-2097151..2097151)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the altitude as a 30 bit value defined

by the Altitude type field. The field is encoded as a 2s-comple-

ment fixed-point 22-bit integer Part with 8-bit fraction.  This

field contains the fixed-point Part of Altitude. This field is

derived from IETF RFC-3825, and is accessed big-endian."

::= { dot11LCIReportEntry 13 }

dot11LCIAltitudeFraction OBJECT-TYPE

SYNTAX INTEGER (-127..127)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the altitude as a 30 bit value defined

by the Altitude type field. The field is encoded as a 2s-comple-

ment fixed-point 22-bit integer Part with 8-bit fraction.  This

field contains the fraction part of Altitude. This field is

derived from IETF RFC-3825, and is accessed big-endian."

::= { dot11LCIReportEntry 14 }

dot11LCIDatum OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the datum as an eight-bit value encod-

ing the horizontal and vertical references used for the coordi-

nates given in this LCI."

::= { dot11LCIReportEntry 15 }

dot11LCIAzimuthType OBJECT-TYPE

SYNTAX INTEGER {

frontSurfaceOfSTA(0),

radioBeam(1) }

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the azimuth Type as a one bit attribute

encoding the type of Azimuth. Codes defined are: front surface of

STA : in 2s-complement fixed-point 9-bit integer; and radio beam :

in 2s-complement fixed-point 9-bit integer"

::= { dot11LCIReportEntry 16 }

dot11LCIAzimuthResolution OBJECT-TYPE

SYNTAX INTEGER (0..15)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the azimuth Resolution as 4 bits indi-

cating the number of valid bits in the azimuth."

::= { dot11LCIReportEntry 17 }

dot11LCIAzimuth OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the azimuth as a 9 bit value defined by

the Azimuth Type field.The field is encoded as a 2s-complement

fixed-point 9-bit integer horizontal angle in degrees from True

North."

::= { dot11LCIReportEntry 18 }

dot11LCIVendorSpecific OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..255))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute provides an envelope for any optional vendor spe-

cific subelements which may be included in a measurement report

element. Zero length is the null default for this attribute."

DEFVAL { ''H }

::= { dot11LCIReportEntry 19 }

dot11LCIRprtMeasurementMode OBJECT-TYPE

SYNTAX INTEGER {

success(0),

incapableBit(1),

refusedBit(2),

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the outcome status for the measurement

request which generated this measurement report; status is indi-

cated using the following reason codes: 1 indicates this STA is

incapable of generating the report, 2 indicates this STA is refus-

ing to generate the report, 0 indicates the STA successfully car-

ried out the measurement request."

DEFVAL { 0 }

::= { dot11LCIReportEntry 20 }

-- ********************************************************************

-- * End of dot11LCIReport TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11TransmitStreamReport TABLE

-- ********************************************************************

dot11TransmitStreamReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11TransmitStreamReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This table contains the current list of Transmit Delay Metrics

reports that have been received by the MLME. The report tables

shall be maintained as a FIFO to preserve freshness, thus the rows

in this table can be deleted for memory constraints or other

implementation constraints determined by the vendor. New rows

shall have different RprtIndex values than those deleted within

the range limitation of the index. One easy way is to monotoni-

cally increase RprtIndex for new reports being written in the

table."

::= { dot11RMReport 7 }

dot11TransmitStreamReportEntry OBJECT-TYPE

SYNTAX Dot11TransmitStreamReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11TransmitStreamReportTable Indexed by

dot11TransmitStreamRprtIndex."

INDEX { dot11TransmitStreamRprtIndex }

::= { dot11TransmitStreamReportTable 1 }

Dot11TransmitStreamReportEntry ::=

SEQUENCE {

dot11TransmitStreamRprtIndex	Unsigned32,

dot11TransmitStreamRprtRqstToken	OCTET STRING,

dot11TransmitStreamRprtIfIndex	INTEGER,

dot11TransmitStreamMeasuringSTAAddr	MacAddress,

dot11TransmitStreamRprtActualStartTime	TSFType,

dot11TransmitStreamRprtMeasurementDuration	Unsigned32,

dot11TransmitStreamRprtPeerSTAAddress	MacAddress,

dot11TransmitStreamRprtTID	INTEGER,

dot11TransmitStreamRprtAverageQueueDelay	Integer32,

dot11TransmitStreamRprtAverageTransmitDelay	Integer32,

dot11TransmitStreamRprtTransmittedMSDUCount	Integer32,

dot11TransmitStreamRprtMSDUDiscardedCount	Integer32,

dot11TransmitStreamRprtMSDUFailedCount	Integer32,

dot11TransmitStreamRprtMultipleRetryCount	Integer32,

dot11TransmitStreamRprtCFPollsLostCount	Integer32,

dot11TransmitStreamRprtBin0Range	INTEGER,

dot11TransmitStreamRprtDelayHistogram	OCTET STRING,

dot11TransmitStreamRprtReason	INTEGER,

dot11TransmitStreamRprtVendorSpecific	OCTET STRING,

dot11TransmitStreamRprtMeasurementMode	INTEGER}

dot11TransmitStreamRprtIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for Transmit Delay Metrics Report elements in

dot11TransmitStreamReportTable, greater than 0."

::= { dot11TransmitStreamReportEntry 1 }

dot11TransmitStreamRprtRqstToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the request token that was indicated in

the measurement request that generated this measurement report.

This should be an exact match to the original dot11RMRqstToken

attribute. Note that there may be multiple entries in the table

that match this value since a single request may generate multi-

ple measurement reports."

::= { dot11TransmitStreamReportEntry 2 }

dot11TransmitStreamRprtIfIndex OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The index of the interface on which this TransmitStream

Report was received."

::= { dot11TransmitStreamReportEntry 3 }

dot11TransmitStreamMeasuringSTAAddr OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The MAC address of the measuring STA for this row of Transmit

Delay Metrics report."

::= { dot11TransmitStreamReportEntry 4 }

dot11TransmitStreamRprtActualStartTime OBJECT-TYPE

SYNTAX TSFType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the TSF value at the time when the mea-

surement started or for a triggered Transmit Stream/Category Mea-

surement report the TSF value at the reporting QoS STA when the

trigger condition was met."

::= { dot11TransmitStreamReportEntry 5 }

dot11TransmitStreamRprtMeasurementDuration OBJECT-TYPE

SYNTAX Unsigned32

UNITS "TUs"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the duration over which the Transmit

Delay Metrics Report was measured. For a triggered Transmit

Stream/Category Measurement Report, metrics are reported over a

number of transmitted MSDUs rather than a duration, hence Measure-

ment Duration shall be set to 0."

::= { dot11TransmitStreamReportEntry 6 }

dot11TransmitStreamRprtPeerSTAAddress OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The MAC address present in the Address 1 field of the measured

data frames for this row of Transmit Stream/Category Measurement

report."

::= { dot11TransmitStreamReportEntry 7 }

dot11TransmitStreamRprtTID OBJECT-TYPE

SYNTAX INTEGER(0..16)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the TC or TS for which traffic is to be

measured. Values 0 to 15 are defined. Values 16-255 are reserved."

::= { dot11TransmitStreamReportEntry 8 }

dot11TransmitStreamRprtAverageQueueDelay OBJECT-TYPE

SYNTAX Integer32

UNITS "TUs"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the average delay of the frames (MSDUs)

that are passed to the MAC during the measurement duration for the

indicated destination and the indicated Traffic Identifier. Queue

Delay shall be measured from the time the MSDU is passed to the

MAC until the transmission starts and shall be expressed in units

of TUs."

::= { dot11TransmitStreamReportEntry 9 }

dot11TransmitStreamRprtAverageTransmitDelay OBJECT-TYPE

SYNTAX Integer32

UNITS "TUs"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the average delay of the frames (MSDUs)

that are successfully transmitted during the measurement duration

for the indicated destination and the indicated Traffic Identi-

fier. Delay shall be measured from the time the MSDU is passed to

the MAC until ACK is received from the intermediate destination."

::= { dot11TransmitStreamReportEntry 10}

dot11TransmitStreamRprtTransmittedMSDUCount OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the number of MSDUs to the peer STA for

the TC, or TS given by the Traffic Identifier successfully trans-

mitted in the measurement duration"

::= {dot11TransmitStreamReportEntry 11}

dot11TransmitStreamRprtMSDUDiscardedCount OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the number of MSDUs to the peer STA for

the TC, or TS given by the Traffic Identifier discarded due either

to the number of transmit attempts exceeding dot11ShortRetryLimit

or dot11LongRetryLimit as appropriate, or due to the MSDU life-

time having been reached"

::= {dot11TransmitStreamReportEntry 12}

dot11TransmitStreamRprtMSDUFailedCount OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the number of MSDUs to the peer STA for

the TC, or TS given by the Traffic Identifier discarded during the

measurement duration due to the number of transmit attempts

exceeding dot11ShortRetryLimit or dot11LongRetryLimit as appropri-

ate."

::= {dot11TransmitStreamReportEntry 13}

dot11TransmitStreamRprtMultipleRetryCount OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the number of MSDUs for the TC, or TS

given by the Traffic Identifier that are successfully transmitted

after more than one retransmission attempt."

::= {dot11TransmitStreamReportEntry 14}

dot11TransmitStreamRprtCFPollsLostCount OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the number of QoS (+)CF-Poll frames

transmitted to the peer STA where there was no response from the

QoS STA."

::= {dot11TransmitStreamReportEntry 15}

dot11TransmitStreamRprtBin0Range OBJECT-TYPE

SYNTAX INTEGER

UNITS "TUs"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the delay range for Bin 0 of the delay

histogram."

::= { dot11TransmitStreamReportEntry 16 }

dot11TransmitStreamRprtDelayHistogram OBJECT-TYPE

SYNTAX OCTET STRING (SIZE (6))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the histogram of delay of the frames

(MSDUs) that are successfully transmitted during the measurement

duration for the indicated Traffic Identifier and the indicated

destination. Delay shall be measured from the time the MSDU is

passed to the MAC until the ACK is received from the intermediate

destination and shall be expressed in units of TUs. "

::= { dot11TransmitStreamReportEntry 17 }

dot11TransmitStreamRprtReason OBJECT-TYPE

SYNTAX INTEGER {

averageError(0),

consecutiveError(1),

delayError(2),

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the Reason field indicating the reason

that the measuring QoS STA sent the Transmit Stream/Category mea-

surement report."

DEFVAL { 0 }

::= { dot11TransmitStreamReportEntry 18 }

dot11TransmitStreamRprtVendorSpecific OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..255))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute provides an envelope for any optional vendor spe-

cific subelements which may be included in a measurement report

element. Zero length is the null default for this attribute."

DEFVAL { ''H }

::= { dot11TransmitStreamReportEntry 19 }

dot11TransmitStreamRprtMeasurementMode OBJECT-TYPE

SYNTAX INTEGER {

success(0),

incapableBit(1),

refusedBit(2),

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the outcome status for the measurement

request which generated this measurement report; status is indi-

cated using the following reason codes: 1 indicates this STA is

incapable of generating the report, 2 indicates this STA is refus-

ing to generate the report, 0 indicates the STA successfully car-

ried out the measurement request."

DEFVAL { 0 }

::= { dot11TransmitStreamReportEntry 20 }

-- ********************************************************************

-- * End of dot11TransmitStreamReport TABLE

-- ********************************************************************

-- ********************************************************************

-- * Radio Measurement Configuration Information

-- ********************************************************************

dot11RMConfig OBJECT IDENTIFIER ::= { dot11RadioResourceMeasurement 3 }

-- ********************************************************************

-- * dot11APChannelReport TABLE

-- ********************************************************************

dot11APChannelReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11APChannelReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"AP Channel Report information, in tabular form."

::= { dot11RMConfig 1 }

dot11APChannelReportEntry OBJECT-TYPE

SYNTAX Dot11APChannelReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11APChannelReportTable. Each entry in the

table is indexed by dot11APChannelReportIndex."

INDEX { dot11APChannelReportIndex }

::= { dot11APChannelReportTable 1 }

Dot11APChannelReportEntry ::=

SEQUENCE {

dot11APChannelReportIndex	Unsigned32,

dot11APChannelReportIfIndex	INTEGER,

dot11APChannelReportRegulatoryClass	INTEGER,

dot11APChannelReportChannelList	OCTET STRING}

dot11APChannelReportIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Index for AP channel report entry in dot11APChannelReportTable,

greater than 0."

::= { dot11APChannelReportEntry 1 }

dot11APChannelReportIfIndex OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The ifIndex for this row of the AP channel report."

::= { dot11APChannelReportEntry 2 }

dot11APChannelReportRegulatoryClass OBJECT-TYPE

SYNTAX INTEGER(1..255)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the channel set for this AP Channel

Report. Country, Regulatory Class and Channel Number together

specify the channel frequency and spacing for this measurement

report. Valid values of Regulatory Class are shown in Annex J."

REFERENCE

"Annex J"

::= { dot11APChannelReportEntry 3 }

dot11APChannelReportChannelList OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..255))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute lists the specific channels in this AP Channel

Report. Zero length is the null default for this attribute. Each

octet indicates a different channel within the indicated Regula-

tory Class. This list of channels is the Channel List in the AP

Channel Report element described in 7.3.2.36 (AP Channel Report

element(11k)). "

DEFVAL { ''H }

::= { dot11APChannelReportEntry 4 }

-- ********************************************************************

-- * End of dot11APChannelReportTable TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11RMNeighborReport TABLE

-- ********************************************************************

dot11RMNeighborReportNextIndex OBJECT-TYPE

SYNTAX INTEGER(0..255)

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Identifies the next available index for managing the Neighbor

Report table. If this attribute is 0, it indicates that the Neigh-

bor Report feature is not configurable via SNMP, or the table is

full and new rows cannot be accepted."

::= { dot11RMConfig 2 }

dot11RMNeighborReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11RMNeighborReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This table contains pertinent information on a collection of

BSSID’s that are candidates to which STA’s can roam. The rows are

created using createAndWait method and fill in the attributes.

When the rowStatus is set to active, the row can be included in

Neighbor Report IEs. If there is an error, the rowStatus shall be

set to notReady by SME. Since this table contains all Neighbor

Report element entries for all interfaces enabled with the Neigh-

bor Report feature, it is possible to have too many entries for

one interface, while still remaining under the MaxTableSize. In

that situation, SME shall include Neighbor Report entries only

with lower dot11RMNeighborReportIFIndex up to the maximum possi-

ble number of entries for a particular interface identified by

ifIndex. SME shall set the rowStatus to notInService for those

rows that cannot be included in the Neighbor Report element for

that interface."

::= { dot11RMConfig 3 }

dot11RMNeighborReportEntry OBJECT-TYPE

SYNTAX Dot11RMNeighborReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11RMNeighborReportTable"

INDEX { dot11RMNeighborReportIndex }

::= { dot11RMNeighborReportTable 1 }

Dot11RMNeighborReportEntry ::=

SEQUENCE {

dot11RMNeighborReportIndex	Unsigned32,

dot11RMNeighborReportIfIndex	INTEGER,

dot11RMNeighborReportBSSID	MacAddress,

dot11RMNeighborReportAPReachability	INTEGER,

dot11RMNeighborReportSecurity	TruthValue,

dot11RMNeighborReportCapSpectrumMgmt	TruthValue,

dot11RMNeighborReportCapQoS	TruthValue,

dot11RMNeighborReportCapAPSD	TruthValue,

dot11RMNeighborReportCapRM	TruthValue,

dot11RMNeighborReportCapDelayBlockAck	TruthValue,

dot11RMNeighborReportCapImmediateBlockAck	TruthValue,

dot11RMNeighborReportKeyScope	TruthValue,

dot11RMNeighborReportRegulatoryClass	INTEGER,

dot11RMNeighborReportChannelNumber	INTEGER,

dot11RMNeighborReportPhyType	INTEGER,

dot11RMNeighborReportNeighborTSFInfo	OCTET STRING,

dot11RMNeighborReportPilotInterval	Unsigned32,

dot11RMNeighborReportPilotMultipleBSSID 	OCTET STRING,

dot11RMNeighborReportRMEnabledCapabilities	OCTET STRING,

dot11RMNeighborReportVendorSpecific	OCTET STRING,

dot11RMNeighborReportRowStatus	RowStatus,

dot11RMNeighborReportMobilityDomain	TruthValue }

dot11RMNeighborReportIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for Neighbor Report configuration table in

dot11RMNeighborReportTable, greater than 0."

::= { dot11RMNeighborReportEntry 1 }

dot11RMNeighborReportIfIndex OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The ifIndex for this row of the Neighbor Report."

::= { dot11RMNeighborReportEntry 2 }

dot11RMNeighborReportBSSID OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute indicates the BSSID of the AP described by this

row of Neighbor Report."

::= { dot11RMNeighborReportEntry 3 }

dot11RMNeighborReportAPReachability OBJECT-TYPE

SYNTAX INTEGER {

notReachable(1),

unknown(2),

reachable(3)

}

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the reachability of the AP represented

by the dot11NeighborReportBSSID."

::= { dot11RMNeighborReportEntry 4 }

dot11RMNeighborReportSecurity OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute, when true, indicates that the neighbor AP identi-

fied by this BSSID supports the same security provisioning as used

by the AP which  provided this neighbor report. This attribute,

when False,  indicates either that the neighbor AP identified by

this BSSID does not support the same security provisioning or that

the security information for this neighbor AP is not available at

this time."

::= { dot11RMNeighborReportEntry 5 }

dot11RMNeighborReportCapSpectrumMgmt OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the spectrum management capability of

the AP represented by dot11NeighborReportBSSID."

::= { dot11RMNeighborReportEntry 6 }

dot11RMNeighborReportCapQoS OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute indicates the QoS capability of the AP repre-

sented by dot11NeighborReportBSSID."

::= { dot11RMNeighborReportEntry 7 }

dot11RMNeighborReportCapAPSD OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the APSD capability of the AP repre-

sented by dot11NeighborReportBSSID."

::= { dot11RMNeighborReportEntry 8 }

dot11RMNeighborReportCapRM OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the RM capability of the AP represented

by dot11NeighborReportBSSID."

::= { dot11RMNeighborReportEntry 9 }

dot11RMNeighborReportCapDelayBlockAck OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the Delayed BlockAck capability of the

AP represented by dot11NeighborReportBSSID."

::= { dot11RMNeighborReportEntry 10 }

dot11RMNeighborReportCapImmediateBlockAck OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the Immediate BlockAck capability of the

AP represented by dot11NeighborReportBSSID."

::= { dot11RMNeighborReportEntry 11 }

dot11RMNeighborReportKeyScope OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute, when true, indicates the neighbor AP identified

by this BSSID has the same authenticator as the AP which provided

this neighbor report. This attribute, when false, indicates that

the neighbor AP identified by this BSSID has a different authenti-

cator or that authenticator information is not available."

::= { dot11RMNeighborReportEntry 12 }

dot11RMNeighborReportRegulatoryClass OBJECT-TYPE

SYNTAX INTEGER(1..255)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the channel set for this Neighbor Report

entry. Country, Regulatory Class and Channel Number together spec-

ify the channel frequency and spacing for this measurement report.

Valid values of Regulatory Class are shown in Annex J."

REFERENCE

"Annex J"

::= { dot11RMNeighborReportEntry 13 }

dot11RMNeighborReportChannelNumber OBJECT-TYPE

SYNTAX INTEGER (1..255)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the current operating channel of the AP

represented by the dot11NeighborReportBSSID. The Channel Number is

only defined within the indicated Regulatory Class for this Neigh-

bor Report entry."

::= { dot11RMNeighborReportEntry 14 }

dot11RMNeighborReportPhyType OBJECT-TYPE

SYNTAX INTEGER {

fhss(1),

dsss(2),

irbaseband(3),

ofdm(4),

hrdsss(5),

erp(6)

}

UNITS "dot11PHYType"

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the PHY Type of the neighbor AP identi-

fied by this BSSID."

::= { dot11RMNeighborReportEntry 15 }

dot11RMNeighborReportNeighborTSFInfo OBJECT-TYPE

SYNTAX OCTET STRING (SIZE (6))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates TSF timing information for the neighbor

AP identified by this BSSID. The TSF timing information includes

the TSF Offset and the Beacon Interval, as defined in subclause

7.3.2.37 (Neighbor Report element(11k))."

::= { dot11RMNeighborReportEntry 16 }

dot11RMNeighborReportPilotInterval OBJECT-TYPE

SYNTAX Unsigned32

UNITS "TUs"

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates Measurement Pilot Interval for the

neighbor AP identified by this BSSID, as defined in subclause

7.3.1.18 (Measurement Pilot Interval field(11k))."

::= { dot11RMNeighborReportEntry 17 }

dot11RMNeighborReportPilotMultipleBSSID OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(1))

UNITS "BSSID LSBs"

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates n, where 2n is the maximum number of

BSSIDs in the multiple BSSID set, as described in subclause

11.10.11 (Multiple BSSID Set(11k))."

::= { dot11RMNeighborReportEntry 18 }

dot11RMNeighborReportRMEnabledCapabilities OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(7))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the detailed enabled capabilities of the

AP represented by the dot11NeighborReportBSSID, as defined in sub-

clause 7.3.2.45 ((#1704)RM Enabled Capabilities ele-

ment(11k)(#1720))."

REFERENCE

"IEEE 802.11 – Subclause 7.3.2.45 ((#1704)RM Enabled Capabilities

element(11k)(#1720))"

::= { dot11RMNeighborReportEntry 19 }

dot11RMNeighborReportVendorSpecific OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..255))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute provides an envelope for any optional vendor spe-

cific subelements which may be included in a measurement report

element. Zero length is the null default for this attribute."

DEFVAL { ''H }

::= { dot11RMNeighborReportEntry 20 }

dot11RMNeighborReportRowStatus OBJECT-TYPE

SYNTAX RowStatus

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"Contains the row status of the Neighbor Report, essentially used

for indicating whether the row has all valid attributes filled in.

Then set to active to be used in Neighbor Report elements. If any

parameter is invalid, the SME shall set this attribute back to

notReady. It is the responsibility of the manager to correct the

parameters."

::= { dot11RMNeighborReportEntry 21 }

dot11RMNeighborReportMobilityDomain OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"Indicates a common mobility domain identifier (MDID) and an iden-

tical value of the FT Capability and Policy value."

::= { dot11RMNeighborReportEntry 22 }

-- ********************************************************************

-- * End of dot11RMNeighborReport TABLE

-- ********************************************************************

dot11SMTRMRequest OBJECT-GROUP

OBJECTS { dot11RMRqstIndex,

dot11RMRqstRowStatus,

dot11RMRqstToken,

dot11RMRqstRepetitions,

dot11RMRqstIfIndex,

dot11RMRqstType,

dot11RMRqstTargetAdd,

dot11RMRqstTimeStamp,

dot11RMRqstChanNumber,

dot11RMRqstRegulatoryClass,

dot11RMRqstRndInterval,

dot11RMRqstDuration,

dot11RMRqstParallel,

dot11RMRqstEnable,

dot11RMRqstRequest,

dot11RMRqstReport,

dot11RMRqstDurationMandatory,

dot11RMRqstBeaconRqstMode,

dot11RMRqstBeaconRqstDetail,

dot11RMRqstFrameRqstType,

dot11RMRqstBssid,

dot11RMRqstSSID,

dot11RMRqstBeaconReportingCondition,

dot11RMRqstBeaconThresholdOffset,

dot11RMRqstSTAStatRqstGroupID,

dot11RMRqstLCIRqstSubject,

dot11RMRqstLCILatitudeResolution,

dot11RMRqstLCILongitudeResolution,

dot11RMRqstLCIAltitudeResolution,

dot11RMRqstLCIAzimuthType,

dot11RMRqstLCIAzimuthResolution,

dot11RMRqstPauseTime,

dot11RMRqstTransmitStreamPeerQSTAAddress,

dot11RMRqstTransmitStreamTrafficIdentifier,

dot11RMRqstTransmitStreamBin0Range,

dot11RMRqstTrigdQoSAverageCondition,

dot11RMRqstTrigdQoSConsecutiveCondition,

dot11RMRqstTrigdQoSDelayCondition,

dot11RMRqstTrigdQoSAverageThreshold,

dot11RMRqstTrigdQoSConsecutiveThreshold,

dot11RMRqstTrigdQoSDelayThresholdRange,

dot11RMRqstTrigdQoSDelayThreshold,

dot11RMRqstTrigdQoSMeasurementCount,

dot11RMRqstTrigdQoSTimeout,

dot11RMRqstChannelLoadReportingCondition,

dot11RMRqstChannelLoadReference,

dot11RMRqstNoiseHistogramReportingCondition,

dot11RMRqstAnpiReference,

dot11RMRqstAPChannelReport,

dot11RMRqstSTAStatPeerSTAAddress,

dot11RMRqstFrameTransmitterAddress,

dot11RMRqstVendorSpecific }

STATUS current

DESCRIPTION

"The SMTRMRequest package is a set of attributes that shall be

present if the STA supports the Radio Measurement service."

::= { dot11Groups 37 }

dot11SMTRMReport OBJECT-GROUP

OBJECTS {   dot11ChannelLoadRprtIndex,

dot11ChannelLoadRprtRqstToken,

dot11ChannelLoadRprtIfIndex,

dot11ChannelLoadMeasuringSTAAddr,

dot11ChannelLoadRprtChanNumber,

dot11ChannelLoadRprtRegulatoryClass,

dot11ChannelLoadRprtActualStartTime,

dot11ChannelLoadRprtMeasurementDuration,

dot11ChannelLoadRprtChannelLoad,

dot11ChannelLoadRprtVendorSpecific,

dot11ChannelLoadRprtMeasurementMode,

dot11NoiseHistogramRprtIndex,

dot11NoiseHistogramRprtRqstToken,

dot11NoiseHistogramRprtIfIndex,

dot11NoiseHistogramMeasuringSTAAddr,

dot11NoiseHistogramRprtChanNumber,

dot11NoiseHistogramRprtRegulatoryClass,

dot11NoiseHistogramRprtActualStartTime,

dot11NoiseHistogramRprtMeasurementDuration,

dot11NoiseHistogramRprtAntennaID,

dot11NoiseHistogramRprtANPI,

dot11NoiseHistogramRprtIPIDensity0,

dot11NoiseHistogramRprtIPIDensity1,

dot11NoiseHistogramRprtIPIDensity2,

dot11NoiseHistogramRprtIPIDensity3,

dot11NoiseHistogramRprtIPIDensity4,

dot11NoiseHistogramRprtIPIDensity5,

dot11NoiseHistogramRprtIPIDensity6,

dot11NoiseHistogramRprtIPIDensity7,

dot11NoiseHistogramRprtIPIDensity8,

dot11NoiseHistogramRprtIPIDensity9,

dot11NoiseHistogramRprtIPIDensity10,

dot11NoiseHistogramRprtVendorSpecific,

dot11NoiseHistogramRprtMeasurementMode,

dot11BeaconRprtIndex,

dot11BeaconRprtRqstToken,

dot11BeaconRprtIfIndex,

dot11BeaconMeasuringSTAAddr,

dot11BeaconRprtChanNumber,

dot11BeaconRprtRegulatoryClass,

dot11BeaconRprtActualStartTime,

dot11BeaconRprtMeasurementDuration,

dot11BeaconRprtPhyType,

dot11BeaconRprtReportedFrameType,

dot11BeaconRprtRCPI,

dot11BeaconRprtRSNI,

dot11BeaconRprtBSSID,

dot11BeaconRprtAntennaID,

dot11BeaconRprtParentTSF,

dot11BeaconRprtReportedFrameBody,

dot11BeaconRprtVendorSpecific,

dot11BeaconRprtMeasurementMode,

dot11FrameRprtIndex,

dot11FrameRprtIfIndex,

dot11FrameRprtRqstToken,

dot11FrameRprtChanNumber,

dot11FrameRprtRegulatoryClass,

dot11FrameRprtActualStartTime,

dot11FrameRprtMeasurementDuration,

dot11FrameRprtTransmitSTAAddress,

dot11FrameRprtBSSID,

dot11FrameRprtPhyType,

dot11FrameRprtAvgRCPI,

dot11FrameRprtRSNI,

dot11FrameRprtLastRCPI,

dot11FrameRprtAntennaID,

dot11FrameRprtNumberFrames,

dot11FrameRprtVendorSpecific,

dot11FrameRptMeasurementMode,

dot11STAStatisticsReportIndex,

dot11STAStatisticsReportToken,

dot11STAStatisticsIfIndex,

dot11STAStatisticsSTAAddress,

dot11STAStatisticsMeasurementDuration,

dot11STAStatisticsGroupID,

dot11STAStatisticsTransmittedFragmentCount,

dot11STAStatisticsMulticastTransmittedFrameCount,

dot11STAStatisticsFailedCount,

dot11STAStatisticsRetryCount,

dot11STAStatisticsMultipleRetryCount,

dot11STAStatisticsFrameDuplicateCount,

dot11STAStatisticsRTSSuccessCount,

dot11STAStatisticsRTSFailureCount,

dot11STAStatisticsACKFailureCount,

dot11STAStatisticsQosTransmittedFragmentCount,

dot11STAStatisticsQosFailedCount,

dot11STAStatisticsQosRetryCount,

dot11STAStatisticsQosMultipleRetryCount,

dot11STAStatisticsQosFrameDuplicateCount,

dot11STAStatisticsQosRTSSuccessCount,

dot11STAStatisticsQosRTSFailureCount,

dot11STAStatisticsQosACKFailureCount,

dot11STAStatisticsQosReceivedFragmentCount,

dot11STAStatisticsQosTransmittedFrameCount,

dot11STAStatisticsQosDiscardedFrameCount,

dot11STAStatisticsQosMPDUsReceivedCount,

dot11STAStatisticsQosRetriesReceivedCount,

dot11STAStatisticsReceivedFragmentCount,

dot11STAStatisticsMulticastReceivedFrameCount,

dot11STAStatisticsFCSErrorCount,

dot11STAStatisticsTransmittedFrameCount,

dot11STAStatisticsAPAverageAccessDelay,

dot11STAStatisticsAverageAccessDelayBestEffort,

dot11STAStatisticsAverageAccessDelayBackground,

dot11STAStatisticsAverageAccessDelayVideo,

dot11STAStatisticsAverageAccessDelayVoice,

dot11STAStatisticsStationCount,

dot11STAStatisticsChannelUtilization,

dot11STAStatisticsVendorSpecific,

dot11STAStatisticsRprtMeasurementMode,

dot11LCIReportIndex,

dot11LCIReportToken,

dot11LCIIfIndex,

dot11LCISTAAddress,

dot11LCILatitudeResolution,

dot11LCILatitudeInteger,

dot11LCILatitudeFraction,

dot11LCILongitudeResolution,

dot11LCILongitudeInteger,

dot11LCILongitudeFraction,

dot11LCIAltitudeType,

dot11LCIAltitudeResolution,

dot11LCIAltitudeInteger,

dot11LCIAltitudeFraction,

dot11LCIDatum,

dot11LCIAzimuthType,

dot11LCIAzimuthResolution,

dot11LCIAzimuth,

dot11LCIVendorSpecific,

dot11LCIRprtMeasurementMode,

dot11TransmitStreamRprtIndex,

dot11TransmitStreamRprtRqstToken,

dot11TransmitStreamRprtIfIndex,

dot11TransmitStreamMeasuringSTAAddr,

dot11TransmitStreamRprtActualStartTime,

dot11TransmitStreamRprtMeasurementDuration,

dot11TransmitStreamRprtPeerSTAAddress,

dot11TransmitStreamRprtTID,

dot11TransmitStreamRprtAverageQueueDelay,

dot11TransmitStreamRprtAverageTransmitDelay,

dot11TransmitStreamRprtTransmittedMSDUCount,

dot11TransmitStreamRprtMSDUDiscardedCount,

dot11TransmitStreamRprtMSDUFailedCount,

dot11TransmitStreamRprtMultipleRetryCount,

dot11TransmitStreamRprtCFPollsLostCount,

dot11TransmitStreamRprtBin0Range,

dot11TransmitStreamRprtDelayHistogram,

dot11TransmitStreamRprtReason,

dot11TransmitStreamRprtVendorSpecific,

dot11TransmitStreamRprtMeasurementMode

}

STATUS current

DESCRIPTION

"The SMTRMReport package is a set of attributes that shall be

present if the STA supports the Radio Measurement service."

::= { dot11Groups 38 }

dot11SMTRMConfig OBJECT-GROUP

OBJECTS { dot11APChannelReportIndex,

dot11APChannelReportIfIndex,

dot11APChannelReportRegulatoryClass,

dot11APChannelReportChannelList,

dot11RMNeighborReportIndex,

dot11RMNeighborReportIfIndex,

dot11RMNeighborReportBSSID,

dot11RMNeighborReportAPReachability,

dot11RMNeighborReportSecurity,

dot11RMNeighborReportCapSpectrumMgmt,

dot11RMNeighborReportCapQoS,

dot11RMNeighborReportCapAPSD,

dot11RMNeighborReportCapRM,

dot11RMNeighborReportCapDelayBlockAck,

dot11RMNeighborReportCapImmediateBlockAck,

dot11RMNeighborReportKeyScope,

dot11RMNeighborReportChannelNumber,

dot11RMNeighborReportRegulatoryClass,

dot11RMNeighborReportPhyType,

dot11RMNeighborReportNeighborTSFInfo,

dot11RMNeighborReportPilotInterval,

dot11RMNeighborReportPilotMultipleBSSID,

dot11RMNeighborReportRMEnabledCapabilities,

dot11RMNeighborReportVendorSpecific,

dot11RMNeighborReportRowStatus }

STATUS current

DESCRIPTION

"The SMTRMConfig package is a set of attributes that shall be

present if the STA supports the Radio Measurement service."

::= { dot11Groups 39 }

END