IEEE P802.11
Wireless LANs

| Categories for Mesh Action Frames |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-09-22 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Meiyuan Zhao | Intel Corporation | 2200 Mission College Blvd, Santa Clara, CA 95054 | +1 408 653 5517 | meiyuan.zhao@intel.com |
| Jesse Walker | Intel Corporation | 2111 N. 25th Ave, Hillsboro, OR 97124 | +1 503 712 1849 | jesse.walker@intel.com |

Abstract

This submission resolves comment CID 125, 352, 630, 782, 1172

Summary of the changes:

Baseline: 	802.11s Draft 3.03

IEEE 802.11 REVmb 1.0

802.11w Draft 10.0

Note to editor: Updates introduced in this document is tracked in changes:

3 Definitions

Insert the following definitions

3.114b Self Protected Action frame: Self Protected Action frames are defined as Action frames with the category value specified in 7.3.1.11 Table 7-24 and complete definition in 7.4.9b. The Self Protected Action frames are specified with “No” in the “Robust” column of Table 7-24. The protection on each Self Protected Action frame is provided by the protocol that uses the frame.

Update Table 7-24 as indicated in the following: (based on 11w draft)

| - Category valuesinput.doc |  |  |  |
| --- | --- | --- | --- |
| Value | Meaning | See subclause | Robust |
|  |  | Error: Reference source not found |  |
|  | Mesh Link Metric | Error: Reference source not found | Yes |
|  | Mesh Path Selection | Error: Reference source not found | Yes |
|  | Mesh Interworking | Error: Reference source not found | Yes |
|  | Mesh Resource Coordination | Error: Reference source not found | Yes |
|  | Mesh Proxy Forwarding | Error: Reference source not found | Yes |
| < +1> 11-125 | Reserved | — |  |
| <ANA 12> | Self Protected | 7.4.9b | No |

Insert the following note at the end of clause 7.3.1.11 (Action field)

NOTE—The action frame category “Self Protected” is suggested to be assigned with the category value 126. It is suggested to place the action frame categories that do not require Management Frame Protection protocols for robust protection to the high end value range start from 127 in descending order, i.e., a new category that does not need Management Frame Protection protocols could be assigned with category value 125, and so on. This suggested placement is used to facilitate fast frame processing by Management Frame Protection protocols. As the result, it is clear to frame processing that certain range of category values are for robust action frames, except value 4 (public action frame), and the rest of the value range are for non-robust action frames.

Insert clause 7.4.9b after 7.4.9a (based 11w draft)

7.4.9b Self Protected Action frames

7.4.9b.1 Self Protected Action details

The Self Protected Action frame is defined to allow robust STA-STA communications of the action frames that are not robust (see 7.3.1.11). The protocol that uses these action frames are responsible for deciding whether to protect these frames and supporting protection mechanisms for these frames as needed.

Self Protected Action frames have different nature compared with Public Action frames and Robust Action frames. Robust Action frames assume the existence of a completely established security association. Self Protected Action frames typically exist to manage the creation and destruction of security association, whether or not they are completely established.

Public action frames are defined as public for all STAs, including those that are not in the BSS and MBSS. Self Protected Action frames, however, are used for relationship creation and maintenance between two specific STAs. And their public nature is incidental.

The defined Self Protected Action frames are listed in Table 7-58s (Self Protected Action field values).

Table 7-s58 – Self Protected Action field values

| Action field Value | Description |
| --- | --- |
| 0 | Reserved |
| 1 | Mesh Peering Open |
| 2 | Mesh Peering Confirm |
| 3 | Mesh Peering Close |
| 4—255 | Reserved |

7.4.9b.2 Mesh Peering Open frame self protection details

The Mesh Peering Open frame format is defined in 7.4.12. The protection of the frames is provided when Authenticated Mesh Peering Exchange (AMPE) is enabled. AMPE provides integrity protection of Mesh Peering Open frames.

When the Mesh Peering Open frame is used by Mesh Peering Management protocol, integrity protection on the frame is not enabled.

7.4.9b.3 Mesh Peering Confirm frame self protection details

The Mesh Peering Confirm frame format is defined in 7.4.13. The protection of the frames is provided when Authenticated Mesh Peering Exchange (AMPE) is enabled. AMPE provides integrity protection of Mesh Peering Confirm frames.

When the Mesh Peering Confirm frame is used by Mesh Peering Management protocol, integrity protection on the frame is not enabled.

7.4.9b.2 Mesh Peering Close frame self protection details

The Mesh Peering Close frame format is defined in 7.4.14. The protection of the frames is provided when Authenticated Mesh Peering Exchange (AMPE) is enabled. AMPE provides integrity protection of Mesh Peering Close frames.

When the Mesh Peering Close frame is used by Mesh Peering Management protocol, integrity protection on the frame is not enabled.

Modify clause 7.4.12 as indicated below, re-number the subclauses and subsequent clauses after 7.4.12

- Mesh Peering Open action frame details

The Mesh Peering Open frame is used to open a mesh peering using the procedures defined in Error: Reference source not found. The Mesh Peering Open frame is also conventionally referred to as Mesh Peering Management frame, together with Mesh Peering Confirm and Mesh Peering Close action frames. The frame body of a Mesh Peering Open frame contains the information shown in Mesh Peering Open frame bodyinput.doc.

| - Mesh Peering Open frame bodyinput.doc |  |  |
| --- | --- | --- |
| Order | Information | Notes |
|  | Category |  |
|  | Action |  |
|  | Capability |  |
|  | Supported rates |  |
|  | ERP information | The ERP Information element is present if ERP mesh STA detects NonERP STAs in its vicinity, and is optionally present otherwise. |
|  | Extended Supported Rates | The Extended Supported Rates element is present if there are more than eight supported rates, and is optionally present otherwise. |
|  | Power Capability | The Power Capability element is present if dot11SpectrumManagementRequired is true. |
|  | Supported Channels | The Supported Channels element is present if dot11SpectrumManagementRequired is true and dot11ExtendedChannelSwitchEnabled is false. |
|  | RSN | The RSN information element is present only if dot11RSNAEnabled is set to TRUE. |
|  | Mesh ID |  |
|  | Mesh Configuration |  |
|  | Mesh Peering Management |  |
|  | Authenticated Mesh Peering Exchange | The Authenticated Mesh Peering Exchange element is present when dot11MeshSecurityActivated is true. |
|  | HT Capabilities | The HT Capabilities element is present when dot11HighThroughputOptionImplemented is true. |
|  | HT Information | The HT Information element is included when dot11HighThroughputOptionImplemented is true. |
|  | Extended Capabilities element | The Extended Capabilities element is present if the dot112040BSSCoexistenceManagementSupport is true and is optionally present otherwise. |
|  | 20/40 BSS Coexistence element | The 20/40 BSS Coexistence element is present when the dot112040BSSCoexistenceManagementSupport is true. |
|  | Supported MBSS Regulatory Classes and Channels | The Supported MBSS Regulatory Classes and Channels element is present if dot11ExtendedChannelSwitchEnabled is true. |
|  | Vendor Specific | Optionally present: one or more vendor-specific information elements. This information element follows all other information elements. |
| Last | MIC information element | This field is present when dot11MeshSecurityActivated is true. |

The Category field is set to the value in Category valuesinput.doc for categorytSelf Protected.

The Action field is set to the value 1 in e.

If present, MIC element (MIE) appears at the end of the Mesh Peering Open frame.

7.4.13 Mesh Peering Confirm frame format

The Mesh Peering Confirm frame is used to confirm a mesh peering using the procedures defined in Error: Reference source not found. The Mesh Peering Confirm frame is also conventionally referred to as Mesh Peering Management frame, together with Mesh Peering Open and Mesh Peering Close action frames. The frame body of a Mesh Peering Confirm frame contains the information shown in Mesh Peering Confirm frame bodyinput.doc.

| - Mesh Peering Confirm frame bodyinput.doc |  |  |
| --- | --- | --- |
| Order | Information | Notes |
|  | Category |  |
|  | Action |  |
|  | Capability |  |
|  | AID |  |
|  | Supported rates |  |
|  | Extended Supported Rates | The Extended Supported Rates element is present if there are more than eight supported rates, and is optionally present otherwise. |
|  | RSN | The RSN information element is present only when dot11RSNAEnabled is set to TRUE. |
|  | Mesh Configuration |  |
|  | Peering Management |  |
|  | Authenticated Mesh Peering Exchange | The Authenticated Mesh Peering Exchange element is present when dot11MeshSecurityActivated is true. |
|  | HT Capabilities | The HT Capabilities element is present when dot11HighThroughputOptionImplemented is true. |
|  | HT Information | The HT Information element is included when dot11HighThroughputOptionImplemented is true. |
|  | Extended Capabilities element | The Extended Capabilities element is present if the dot112040BSSCoexistenceManagementSupport is true, and is optionally present otherwise. |
|  | 20/40 BSS Coexistence element | The 20/40 BSS Coexistence element is present when the dot112040BSSCoexistenceManagementSupport is true. |
|  | Vendor Specific | Optionally present: one or more vendor-specific information elements. This information element follows all other information elements. |
| Last | MIC information element | This field is present when dot11MeshSecurityActivated is true. |

The Category field is set to the value in Category valuesinput.doc for categorytSelf Protected.

The Action field is set to the value 2 in Mesh Peering Management Action field valuesinput.doc for this action frame type.

If present, MIC element (MIE) appears at the end of the Mesh Peering Confirm frame.

7.4.14 Mesh Peering Close frame format

The Mesh Peering Close frame is used to close a mesh peering using the procedures defined in Error: Reference source not found. The Mesh Peering Close frame is also conventionally referred to as Mesh Peering Management frame, together with Mesh Peering Open and Mesh Peering Confirm action frames. The frame body of a Mesh Peering Close frame contains the information shown in Mesh Peering Close frame bodyinput.doc.

| - Mesh Peering Close frame bodyinput.doc |  |  |
| --- | --- | --- |
| Order | Information | Notes |
|  | Category |  |
|  | Action |  |
|  | Mesh ID |  |
| 4 | Peering Management |  |
| 5 | Authenticated Mesh Peering Exchange | The Authenticated Mesh Peering Exchange element is present when dot11MeshSecurityActivated is true. |
| 6 | Vendor Specific | Optionally present: one or more vendor-specific information elements. This information element follows all other information elements. |
| Last | MIC information element | This field is present when dot11MeshSecurityActivated is true. |

The Category field is set to the value in Category valuesinput.doc for categorytSelf Protected.

The Action field is set to the value 2 in Mesh Peering Management Action field valuesinput.doc for this action frame type.

If present, MIC element (MIE) appears at the end of the Mesh Peering Close frame.

Update the first paragraph in 11C.4.2.1 as indicated in the following:

Mesh peering management frames are Self Protected Action frames. When mesh peering management frames are used for MPM, the protection on these frames are not enabled (see 7.4.9b).