IEEE P802.11
Wireless LANs

| Specification Framework for TGac |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-09-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Robert Stacey | Intel | 2111 NE 25th Ave, Hillsboro OR 97124, USA | 503-724-0893 | robert.j.stacey@intel.com |
| Eldad Perahia | Intel | 2111 NE 25th Ave, Hillsboro OR 97124, USA |  | eldad.perahia@intel.com |
| Adrian Stephens | Intel |  |  | adrian.p.stephens@intel.com |
| Assaf Kasher | Intel |  |  | assaf.kasher@intel.com |
| Solomon Trainin | Intel |  |  | solomon.trainin@intel.com |
| Michelle Gong | Intel |  |  | michelle.x.gong@intel.com |
| Raja Banerjea | Marvell | 5488 Marvell Lane, Santa Clara CA, 95054 | 408.222.3713 | rajab@marvell.com |
| Hongyuan Zhang | Marvell | 5488 Marvell Lane, Santa Clara CA, 95054 | 408.222.1837 | hongyuan@marvell.com |
| Sudhir Srinivasa | Marvell | 5488 Marvell Lane, Santa Clara CA, 95054 |  | sudhirs@marvell.com |
| Yong Liu | Marvell | 5488 Marvell Lane, Santa Clara CA, 95054 |  | yongliu@marvell.com |
| Harish Ramamurthy | Marvell |  |  | harishr@marvell.com |
| Ning Zhang | Atheros | 1700 Technology Drive, San Jose, CA 95110 | 408-773-5363 | Ning.Zhang@atheros.com |
| Youhan Kim | Atheros | 1700 Technology Drive, San Jose, CA 95110 |  | Youhan,Kim@atheros,com |
| William McFarland | Atheros | 1700 Technology Drive, San Jose, CA 95110 |  | William.McFarland@atheros.com |
| Kai Shi | Atheros | 1700 Technology Drive, San Jose, CA 95110 |  | Kai.Shi@atheros.com |
| Joshua Zhao | Atheros | 1700 Technology Drive, San Jose, CA 95110 |  | Joshua.Zhao@atheros.com |
| Qifan Chen | Atheros | 1700 Technology Drive, San Jose, CA 95110 |  | Qifan.chen@atheros.com |
| James Cho | Atheros | 1700 Technology Drive, San Jose, CA 95110 |  | James.Cho@atheros.com |
| Allert Van Zelst | Qualcomm | Netherlands |  | allert@quallcomm.com |
| Richard Van Nee | Qualcomm | Netherlands |  | rvannee@qualcomm.com |
| Santosh Abraham | Qualcomm | San Diego, USA |  | sabraham@qualcomm.com |
| Hemanth Sampath | Qualcomm | San Diego, USA |  | hsampath@qualcomm.com |
| Sameer Vermani | Qualcomm |  |  | svverman@qualcomm.com |
| Rolf De Vegt | Qualcomm | Santa Clara, USA |  | rolfv@qualcomm.com |
| VK Jones | Qualcomm | Santa Clara, USA |  | vkjones@qualcomm.com |
| Simone Merlin | Qualcomm | San Diego, USA |  | smerlin@qualcomm.com |
| Lin Yang | Qualcomm | San Diego, USA |  | linyang@qualcomm.com |
| Vinko Erceg | Broadcom |  | 858 521 5885 | verceg@broadcom.com |
| Joseph Lauer | Broadcom |  |  | jlauer@broadcom.com |
| Mathew Fischer | Broadcom |  |  | mfischer@broadcom.com |
| Tushar Moorti | Broadcom |  |  | rtm@broadcom.com |
| Peiman Amini | Broadcom |  |  | pamini@broadcom.com |
| Joonsuk Kim | Broadcom |  |  | joonsuk@broadcom.com |
| Yuichi Morioka | Sony |  |  | yuichi.morioka@jp.sony.com |
| Ted Booth | Sony |  |  | ted.booth@am.sony.com |
| Yasushi Takatori | NTT |  |  | takatori.yasushi@lab.ntt.co.jp |
| Yusuke Asai | NTT |  |  | asai.yusuke@lab.ntt.co.jp |
| Ichihiko Toyoda | NTT |  |  | toyoda.ichihiko@lab.ntt.co.jp |
| Chiu Ngo | Samsung Electronics | 75 W. Plumeria Dr. San Jose, CA 95131 USA | +1-408-544-5633 | chiu.ngo@samsung.com |
| Youngsoo Kim | Samsung Electronics | Mt. 14-1 Nongseo-Ri, Giheung-Eup, Yongin-Si, Gyeonggi-Do, Korea 449-712 | +82-31-280-9614 | kimyoungsoo@samsung.com |
| Chunhui (Allan) Zhu | Samsung Electronics | 75 W. Plumeria Dr. San Jose, CA 95131 USA | +1-408-544-5667 | c.zhu@samsung.com |
| Osama Aboul-Magd | Samsung Electronics |  | 613-599-5078 | osama@samsung.com |
| Daewon Lee | LG Electronics | LG R&D Complex 533, Hogye-1dong, Dongan-Gu, Anyang-Shi, Kyungki-Do, 431-749, Korea | +82-31-450-7897 | daewon.lee@lge.com |
| Yujin Noh | LG Electronics | LG R&D Complex 533, Hogye-1dong, Dongan-Gu, Anyang-Shi, Kyungki-Do, 431-749, Korea | +82-31-450-7897 | yujin.noh@lge.com |
| Yongho Soek | LG Electronics | LG R&D Complex 533, Hogye-1dong, Dongan-Gu, Anyang-Shi, Kyungki-Do, 431-749, Korea | +82-31-450-1947 | yongho.soek@lge.com |
| Bonghoe Kim | LG Electronics | LG R&D Complex 533, Hogye-1dong, Dongan-Gu, Anyang-Shi, Kyungki-Do, 431-749, Korea | +82-31-450-4131 | bonghoe.kim@lge.com |
| Minho Cheong | ETRI | 161 Gajeong-dong, Yuseong-gu, Daejeon, Korea | +82 42 860 5635 | minho@etri.re.kr |
| Jaewoo Park | ETRI | 161 Gajeong-dong, Yuseong-gu, Daejeon, Korea | +82 42 860 5723 | parkjw@etri.re.kr |
| Jae Seung Lee | ETRI | 161 Gajeong-dong, Yuseong-gu, Daejeon, Korea | +82 42 860 1326 | jasonlee@etri.re.kr |
| Jong-Ee Oh | ETRI | 161 Gajeong-dong, Yuseong-gu, Daejeon, Korea | +82 42 860 1758 | ohjongee@etri.re.kr |
| Jeeyon Choi | ETRI | 161 Gajeong-dong, Yuseong-gu, Daejeon, Korea | +82 42 860 5247 | jychoi@etri.re.kr |
| Yun Joo Kim | ETRI | 161 Gajeong-dong, Yuseong-gu, Daejeon, Korea | +82 42 860 5480 | yunjoo@etri.re.kr |
| Sok-kyu Lee | ETRI | 161 Gajeong-dong, Yuseong-gu, Daejeon, Korea | +82 42 860 5919 | sk-lee@etri.re.kr |
| Il-Gu Lee | ETRI | 161 Gajeong-dong, Yuseong-gu, Daejeon, Korea | +82 42 860 1633 | iglee@etri.re.kr |
| James Wang | MediaTek, Inc. | 2860 Junction Avenue San Jose, CA 95134 408-526-1899 x 88363 | +1-408-526-1899 | james.wang@mediatek.com |
| Alvin Hsu | MediaTek, Inc | No. 1, Dusing Rd, 1, Hsinchu Science Park, Hsinchu, Taiwan 300, R.O.C. | +886-3-567-0766 | alvin.hsu@mediatek.com |
| Brian Hart | Cisco Systems | 170 West Tasman Drive, San Jose, CA 95134 | 408-526-3346 | brianh@cisco.com |
| Raghuram Rangarajan | Cisco Systems | 170 West Tasman Drive, San Jose, CA 95134 | 408-525-8143 | ragranga@cisco.com |
| Reza Hedayat | Cisco Systems | 2200 East President George Bush Highway, Richardson, TX 75082 | 469-255-2656 | rehedaya@cisco.com |
| Andrew Myles | Cisco Systems | 201 Pacific Highway, St Leonards, NSW, Australia | +61-2-8446-1010 | amyles@cisco.com |
| Lisa Ward | Rohde Shwarz |  |  | Lisa.Ward@rsa.rohde-schwarz |
| Peter Loc |  |  |  | peterloc@gmail.com |
| Thet Htun Khine | Radrix Co., Ltd |  |  | thet@radrix.com |
| Leonardo Lanante Jr. | Kyushu Institute of Technology |  |  |  |
| Yuhei Nagao | Kyushu Institute of Technology |  |  |  |

Abstract

This document provides the framework from which the draft TGac amendment will be developed. The document provides an outline of each of the functional blocks that will be a part of the final amendment. The document is intended to reflect the working consensus of the group on the broad outline for the draft specification. As such it is expected to begin with minimal detail reflecting agreement on specific techniques and highlighting areas on which agreement is still required. It may also begin with an incomplete feature list with additional features added as they are justified. The document will evolve over time until it includes sufficient detail on all the functional blocks and their inter-dependencies so that work can begin on the draft amendment itself.

# 0 Revision notes

R3:	Add header for Revision notes (clause 0)

Add header for MAC (clause 6) and three items to be covered by MAC adhoc.

r4:	Not adopted as a task group revision.

r5:	Added resolvable LTFs text as motioned (10/251r2 motion #1)

Added numerology from 11-10-0070r5 excluding number of MU users (10/252r2 motion #3)

Added preamble structure with TBD autodetect from 11-10-0070r5 (10/252r2 motion #4 & #5)

r6	Added Bandwidth and STBC fields to VHT-SIG-A and MCS to VHT-SIG-B (10/251r3 motion #6)

Only equal modulation on streams (10/251r3 motion #7)

r7	Deleted equal modulation requirement (motion failed). Corrected Figure 1. Task group discussion on Nss in section 3.4.

r8	All occurances of Nss changed to Nsts in section 3.4

r9	Added same modulation and coding for SU transmission (10/251r4 motion #12)

r10	Added GroupID and Nsts fields to VHT-SIG-A (10/518r2 motion #1)

Defined various L-STF, L-SIG and CSD parameters (10/518r2 motion #2)

Defined Subcarrier parameters (10/518r2 motion #3)

Added SU MCS table (10/518r2 motion #4)

Defined number of L-LTFs (10/518r2 motion #5)

Defined P Matrix for up to 4x4 and 8x8 (10/518r2 motion #6)

Defined P Matrix for 6x6 (10/518r2 motion #7)

Defined 80 MHz tone allocation (10/518r2 motion #8)

r11	Added 160 MHz requirements R3.1.1.A-C (10/518r3 motion #11)

Added primary channel selection requirement R5.C (10/518r3 motion #13)

Added smoothing bit exclusion requirement (10/518r3 motion #14)

Added text to describe use of zero for Group ID (10/518r3 motion #16)	Added R3.4.E  for same MCS across streams for MU (10/518r3 motion #17)

Added 1 bit for STBC (10/518r3 motion #18)

r12	Executed motions from 10/0714r3.

r13	Corrections to r12. Executed motions from July 2010, TGac PM2 session (report 10/0714r5)

r14	Executed motions from September 2010 Wednesday TGac session (report 10/1016r3).

r15	Corrected Max MPDU size. Executed motions from September 2010 Thursday PM1 TGac session (report 10/1016r4)

# 1 Definitions

- Multi-user, multiple input, multiple output (MU-MIMO): A technique where multiple STAs, each with potentially multiple antennas, transmit and/or receive independent data streams simultaneously.

- Downlink MU-MIMO (DL MU-MIMO): MU-MIMO with a single transmitting STA and multiple receiving STAs.

- Primary AC: the AC that wins the TXOP for channel access after both external and internal competition. There is only one primary AC at any time.

- Secondary AC: an AC that does not win a TXOP but wants to share the TXOP obtained by the primary AC for simultaneous transmissions. There could be multiple secondary ACs at any time.

- Primary Destinations: destinations targeted by the frames belonging to the primary AC. There could be one or more primary destinations at any time.

- Secondary Destinations: destinations targeted by the frames belonging to secondary ACs. There could be one or more secondary destinations at any time.

# 2 Abbreviations and acronyms

MU		Multi-user

SU		Single user

VHT		Very high throughput

# 3 VHT Physical Layer

This section describes the functional blocks in the physical layer.

## 3.1 Channelization

R3.1.A: The draft specification shall include support for 80 MHz PHY transmission.

80 MHz channels consists of two adjacent IEEE 40 MHz channels, and do not partially overlap with each other. 160 MHz channels consists of two adjacent IEEE 80 MHz channels, and do not partially overlap with each other. 80 MHz and 160 MHz channels for the US region are shown in Figure 1.

Figure 1--80 MHz and 160 MHz channels for the US region

[10/773r0][10/1064r2]

80 MHz and 160 MHz channelization for the Europe, Japan and Global operating classes tables (REVmb Annex E) are shown in Figure 2. Note that China does not have this band.

Figure 3--80 MHz and 160 MHz channelization for the Europe, Japan and Global operating class tables

[10/1064r2]

R3.1.B: The draft specification shall include support for 160 MHz PHY transmission. [10/0378r1]

R3.1.C: Tone allocation for 160 MHz operation shall consist of two 80 MHz tone allocations. [10/0378r1]

R3.1.D: The draft specification shall include support for non-contiguous 160 MHz PHY transmission whose frequency spectrum consists two segments, each transmitted using any two 11ac 80 MHz channels, possibly non-adjacent in frequency. Contiguous and non-contiguous 160 MHz devices shall be capable of transmitting and receiving frames between each other when the two segments of the non-contiguous 160 MHz device are placed in frequency to match the tone allocation of the contiguous 160 MHz device. [10/0378r1]

A VHT STA shall be capable of transmitting and receiving frames using 20 MHz, 40 MHz, and 80 MHz channel width. Contiguous and non-contiguous 160 MHz channel width transmission and reception capability is optional. [10/0827r1]

The primary and the secondary subchannels of the 80 MHz channel to be allocated within a 40 MHz channel. [10/763r0]

R3.1.E: The draft specification shall include support for an efficient channelization in China’s (5,725 ~ 5,850 MHz) spectrum.

A noncontiguous 160 MHz BSS shall be setup using any two nonadjacent 80 MHz channels on which a STA is permitted to establish an 80 MHz BSS. [10/1062r2]

3.2 VHT PLCP sublayer

### 3.2.1 Introduction

A VHT mixed format (MF) preamble shall be supported in the draft specification and device support is mandatory. The VHT mixed format preamble shall have the following characteristics:

R3.2.1.A: Robust legacy 11a deferral. The VHT MF preamble shall be designed such that a legacy 11a device will defer for the duration of the transmission to the same degree that it does for an HT MF preamble.

R3.2.1.B: Robust legacy 11n deferral. A VHT MF preamble shall be designed such that a legacy HT STA will defer for the duration of the transmission to the same degree that it does for an HT MF transmission.

R3.2.1.C: The VHT MF preamble shall permit a STA to auto detect 11a, HT MF, HT GF and VHT preambles.

R3.2.1.D: The VHT MF preamble shall include training for

- a wider channel

- 1 to 8 spatial streams (see Section 3.4)

- DL MU-MIMO

R3.2.1.E: Since the HT SIG field cannot be expanded without breaking backward compatibility, the VHT MF preamble shall include VHT SIG fields. The VHT SIG fields may include signaling for the following:

a) wider bandwidth

b) enhanced MCS (see Section 3.3)

c) more spatial streams (see Section 3.4)

### 3.2.2 VHT PPDU format

R3.2.1.F: The VHT MF PPDU format is shown in Figure 4.

Figure 4 – VHT PPDU format

The VHT MF PPDU includes a 2 symbol VHT-SIG-A field and a 1 symbol VHT-SIG-B field.

### 3.2.3 VHT preamble

The number of subcarriers and subcarrier positions of L-STF are the same as those of the 20 MHz 11n L-STF in each 20 MHz subchannel. [10/0578r1]

The number of subcarriers and pilots, including subcarrier positions, of L-LTF, L-SIG, and VHT-SIG-A are the same as those for the 20 MHz 11n L-LTF and L-SIG in each 20 MHz subchannel. [10/0578r1]

The number of subcarriers and pilots, including subcarrier positions, of VHT-LTF and VHT-DATA symbols in 20 and 40 MHz channels are the same as those for 11n HT-LTF and HT-DATA in 20 and 40 MHz channels. [10/0578r1]

The L-STF, L-LTF, L-SIG, VHT-STF and VHT-LTF portions of preamble for 160 MHz VHT transmissions shall be constructed by repeating the 80 MHz counterparts twice in frequency, once in the lower 80 MHz subchannel and one more time in the upper 80 MHz subchannel of the 160 MHz bandwidth. [10/0774r0]

In all elements of an 80 MHz VHT PPDU , i.e., the L-STF, L-LTF, L-SIG,  VHT-SIG-A, VHT-LTFs, VHT-SIG-B and the Data, subcarrier k, where -122 <= k <= 122, shall be multiplied by the following function of k:

[10/1083r0]

#### 3.2.3.1 Non-VHT portion of VHT mixed format preamble

##### 3.2.3.1.1 Cyclic shift definition

The CSD (Cyclic Shift Diversity) values for up to 4 antennas in L-STF, L-LTF, and L-SIG are the same as the CSD values for the non-HT portion of the packet defined in Table 20-8 of Std 802.11n-2009. [10/0578r1]

##### 3.2.3.1.2 L-STF definition

The 20 MHz L-STF pattern in the VHT preamble is as defined in 20.3.9.3.3 of Std 802.11n-2009. [10/0578r1]

The L-STF pattern for 160 MHz VHT transmissions shall repeat the 80 MHz L-STF pattern twice in frequency.  This corresponds to repeating the 11n 20 MHz L-STF pattern in Equation (20-8) in each of the 20 MHz subchannel, then applying the following phase rotation per 20 MHz subchannel starting from the lowest 20 MHz subchannel in frequency: [c80 c80], where c80 is the phase rotation per 20 MHz subchannel for 80 MHz transmissions. [10/0774r0]

##### 3.2.3.1.3 L-LTF definition

The 20 MHz L-LTF pattern in the VHT preamble is as defined in 20.3.9.3.4 of Std 802.11n-2009. [10/0578r1]

The L-LTF pattern for 160 MHz VHT transmissions shall repeat the 80 MHz L-LTF pattern twice in frequency.  This corresponds to repeating the 11n 20 MHz L-LTF pattern in Equation (20-11) in each of the 20 MHz subchannel, then applying the following phase rotation per 20 MHz subchannel starting from the lowest 20 MHz subchannel in frequency: [c80 c80], where c80 is the phase rotation per 20 MHz subchannel for 80 MHz transmissions. [10/0774r0]

##### 3.2.3.1.4 L-SIG definition

The L-SIG symbol is BPSK modulated.

The RATE field shall be set to indicate 6 Mbps.

The LENGTH field shall be set to indicate the duration of the packet.

L-SIG for 160 MHz VHT transmissions shall be constructed by repeating the L-SIG for 80 MHz VHT transmissions twice in frequency, once in the lower 80 MHz subchannel and one more time in the upper 80 MHz subchannel of the 160 MHz bandwidth.  The following phase rotation per 20 MHz subchannel shall be applied starting from the lowest 20 MHz subchannel in frequency: [c80 c80], where c80 is the phase rotation per 20 MHz subchannel for 80 MHz transmissions. [10/0774r0]

#### 3.2.3.2 VHT portion of VHT mixed format preamble

3.2.3.2.1 Cyclic shift definition

The CSD (Cyclic Shift Diversity) values for up to 4 antennas in VHT-SIG-A are the same as the CSD values for the non-HT portion of the packet defined in Table 20-8 of Std 802.11n-2009. [10/0578r1]

The CSD (Cyclic Shift Diversity) table for the VHT portion (starting from VHT-STF) of the SU MIMO frame shall be as follows:

[10/843r0]

##### 3.2.3.2.2 VHT-SIG-A definition

R3.2.1.G: The 1st symbol of VHT-SIG-A shall be BPSK modulated. The second symbol of VHT-SIG-A shall be 90 degree rotated BPSK (QBPSK) modulated for VHT auto-detect as shown in Figure 6.

Figure 6 - VHT-SIG-A modulation

R3.2.1.H: VHT-SIG-A includes the fields listed in Table 1.

Table 1 - VHT-SIG-A fields

| Bit Index | Field | Bit allocation | Description |
| --- | --- | --- | --- |
| VHT-SIG-A1 |  |  |  |
| 0-1 | BW | 2 | Set to 0 for 20 MHz, 1 for 40 MHz, 2 for 80 MHz, 3 for 160 MHz and 80+80 MHz mode |
| 2 | Reserved | 1 | Reserved for possible expansion of BW field. Set to 1. |
| 3 | STBC | 1 | Set to 1 if all streams use STBC, otherwise set to 0. When STBC bit is 1, an odd number of space time streams per user is not allowed. |
| 4-9 | Group ID [10/0582r1] | 6 | A value of all ones indicates [10/0382r2]: - A single user transmission - A transmission where the group membership has not yet been established - A transmission that needs to bypass a group (e.g. broadcast) |
| 10-21 | NSTS [10/0582r1] | 12 | For MU: 3 bits/user with maximum of 4 users - Set to 0 for 0 space time streams - Set to 1 for 1 space time stream - Set to 2 for 2 space time streams - Set to 3 for 3 space time streams - Set to 4 for 4 space time streams Otherwise: first 3 bits contain stream allocation, set to 0 for 1 space time stream, set to 1 for 2 space time streams, etcetera up to 8 streams. Remaining 9 bits contain partial AID: being the 9 LSB bits of AID. For Broadcast and multicast, these 9 bits are set to 0. For STA-to-AP, these 9 bits are set to a special value (TBD). |
| 22-23 | Reserved | 2 | All ones |
|  | Total | 24 |  |
| VHT-SIG-A2 |  |  |  |
| 0-1 | Short GI | 2 | Set B0 to 0 for Long GI, set to 1 for Short GI Set B1 to 1 when Short GI and Nsym%10 == 9 |
| 2-3 | Coding | 2 | Set B2 to 0 for BCC, set to 1 for LDPC B3 purpose is under discussion |
| 4-7 | MCS | 4 | For SU/Broadcast/Multicast: MCS index For MU: Reserved, set to all ones |
| 8 | SU-Beamformed | 1 | Set to 1 when packet is a SU-beamformed packet Set to 0 otherwise For MU: Reserved, set to 1 |
| 9 | Reserved | 1 | All ones |
| 10-17 | CRC | 8 | CRC calculated as in 11n Section 20.3.9.4.4 with C7 in B10 |
| 18-23 | Tail | 6 | All zeros |
|  | Total | 24 |  |

Note that the fields are transmitted in the order shown in and LSB first.

[10/1052r0]

A Smoothing bit shall not be included in either VHT-SIG-A or VHT-SIG-B. [10/0382r2]

##### 3.2.3.2.3 VHT-STF definition

The 20 MHz L-STF pattern in the VHT preamble is as defined in 20.3.9.3.3 of Std 802.11n-2009. [10/843r0]

The frequency domain sequence used to construct the VHT-STF in 20 MHz transmission is identical to the L-STF; in 40 MHz transmission, the VHT-STF is constructed from the 20 MHz version by duplicating, frequency shifting, and rotating the upper sub-carriers by 90°; in 80 MHz transmission, the VHT-STF is constructed from the 20 MHz version by replicating it in each 20 MHz band, frequency shifting, and applying appropriate phase rotations for each 20MHz sub-band. [10/843r0]

For a 160 MHz transmission, subcarriers in the VHT-STF symbol with indices -250 to -6 shall use the VHT-STF pattern for the 80 MHz VHT-STF, with the VHT-STF pattern for subcarrier index -122 mapping to subcarrier index -250 in the 160 MHz transmission.  Furthermore, subcarriers in the VHT-STF symbol in the 160 MHz transmission with indices 6 to 250 shall also use the the VHT-STF pattern for the 80 MHz VHT-STF, with the VHT-STF pattern for subcarrier index -122 mapping to subcarrier index 6 in the 160 MHz transmission.  All other subcarriers shall not be modulated. [10/843r0]

For non-contiguous transmissions using two 80 MHz frequency segments, each 80 MHz frequency segment shall use the VHT-STF pattern for the 80 MHz VHT-STF. [10/843r0]

VHT-STF sequence for 160 MHz VHT transmissions shall be constructed by repeating the VHT-STF sequence for 80 MHz VHT transmissions twice in frequency as follows

where VHTSTF-122,122 is the VHT-STF sequence for 80 MHz VHT transmissions. The following phase rotation per 20 MHz subchannel shall be applied starting from the lowest 20 MHz subchannel in frequency: [c80 c80], where c80 is the phase rotation per 20 MHz subchannel for 80 MHz transmissions. [10/774r0]

##### 3.2.3.2.4 VHT-LTF definition

The long training fields consists of one, two, four, six or eight VHT long training fields (VHT-LTFs) that are necessary for demodulation of the VHT-Data portion of the PPDU or for channel estimation during an NDP packet. [10/0566r2]

The VHT-LTF mapping matrix P for one, two or four VHT-LTFs shall be the same as defined in 802.11n standard specification (Section 20.3.9.4.6, Eq. (20-27)). [10/0566r2]

The VHT-LTF mapping matrix P for six VHT-LTFs is defined as follows:

[10/0771r0]

The VHT-LTF mapping matrix P for eight VHT-LTFs is defined as follows:

P8x8=P4x4P4x4P4x4−P4x4

where P4x4 is defined by Equation 20-27 in Std 802.11n-2009. [10/0566r2]

The VHT-LTF symbols shall have the same number of pilot subcarriers as the data symbols.  The pilot subcarrier indices of the VHT-LTF symbols shall be identical to the pilot subcarrier indices of the data symbols.  The pilot values on these subcarrier indices during VHT-LTFs shall be given by the elements at the corresponding indices of the VHT-LTF sequence.

The VHT-LTF mapping matrix P shall be applied to all subcarriers in the VHT-LTF symbols except for the pilot subcarriers.  Instead, a row-repetition matrix R shall be applied to all pilot subcarriers in the VHT-LTF symbols.  The row-repetition matrix R has the same dimensions as the matrix P (NSTS x NLTF), with all rows of the matrix R being identical to the first row of the matrix P of the corresponding dimension. This results in all space-time streams of the pilot subcarriers in VHT-LTF symbols having the same pilot values.

For each pilot subcarrier, the same per-stream CSD and spatial mapping shall be applied across VHT-LTF and data symbols. [10/0771r0]

In a 80 MHz transmission, the VHT-LTF sequence to be transmitted (on subcarriers -122 to 122) shall be:

The VHT-LTF sequence for 160 MHz VHT transmissions shall be constructed by repeating the VHT-LTF sequence for 80 MHz VHT transmissions twice in frequency as follows

where VHTLTF-122,122 is the VHT-LTF sequence for 80 MHz VHT transmissions. The following phase rotation per 20 MHz subchannel shall be applied starting from the lowest 20 MHz subchannel in frequency: [c80 c80], where c80 is the phase rotation per 20 MHz subchannel for 80 MHz transmissions. [10/0774r0]

##### 3.2.3.2.5 VHT-SIG-B definition

VHT-SIG-B shall be BPSK modulated.

VHT-SIG-B shall use long GI.

VHT-SIG-B shall consist of 26 bits for a 20 MHz PPDU, 27 bits for a 40 MHz transmission and 29 bits for a 80 MHz transmission. For the 40 MHz and 80 MHz PPDU, the VHT-SIG-B bits are repeated as shown in Figure 8.

Figure 8 - VHT-SIG-B in 20, 40 and 80 MHz transmissions

In a 160 MHz PPDU, the 80 MHz VHT-SIG-B is repeated twice in frequency.

R3.2.1.I: VHT-SIG-B includes the fields listed in Table 2

Table 2 - VHT-SIG-B fields

| Field | MU bit allocation |  |  | SU bit allocation |  |  | Description |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20 MHz | 40 MHz | 80 MHz | 20 MHz | 40 MHz | 80 MHz |  |
| Length | 16 | 17 | 19 | 17 | 19 | 21 | length of useful data in PSDU in units of 4 octets |
| MCS | 4 | 4 | 4 | - | - | - |  |
| Reserved | 0 | 0 | 0 | 3 | 2 | 2 | All ones |
| Tail | 6 | 6 | 6 | 6 | 6 | 6 | All zeros |
| Total # bits | 26 | 27 | 29 | 26 | 27 | 29 |  |

NOTE –varying the Length field size with channel width and SU/MU ensures that a consistant packet duration of approximately 5.4ms (the max packet duration from L-SIG) is maintained.

Order of transmission: Length, MCS (in case of MU) / Reserved (in case of SU), Tail. Fields are transmitted LSB first. [10/1052r0]

The number of octets implied by VHT-SIG-B length shall not be more than 3 octets longer than the number of octets implied by L-SIG LENGTH and VHT MCS.

### 3.2.4 VHT Data field

The number of OFDM symbols in the Data field shall be computed using the length field in L-SIG.

For both BCC and LDPC, all bits (including MAC and PHY pad bits) shall be encoded.

When BCC encoding is used, the Data field shall consist of the 16-bit SERVICE field, the PSDU, the PHY pad bits and the tail bits (6NES bits), in that order as shown in Figure 4. When LDPC encoding is used, the Data field shall consist of the 16-bit SERVICE field, the PSDU and the PHY pad bits.  No tails bits are included when LDPC encoding is used.

Figure 10--Data field encoding with BCC

For BCC encoding, the interleaver parameters for 20 MHz and 40 MHz 802.11ac packets will remain unchanged from 20 MHz and 40MHz 802.11n, i.e. the NCOL and NROT parameters for 20/40MHz are as in Table 20-16 from 802.11n-2009. [10/548r2]

For a SU transmission using BCC encoding, the padding flow is as follows. The MAC calculates the NSYM and NPAD using the following equations:

The MAC then pads to the last byte boundary and indicates (using the TXVECTOR) the number of PHY pad bits to the added. After receiveing the PSDU, the PHY adds the 0-7 padding bits and then NES tail bits at each encoder. [10/820r0]

For an MU transmission using BCC encoding, the padding flow is as follows. The MAC calculates NSYM for each user separately using the following equations:

For each user, based on the maximum number of symbols over all users, the MAC pads to the last byte boundary. The number of PHY padding bits added for each user is calculated as in the SU case. For each user, the encoding and stream parsing is done as in the SU case. [10/820r0]

#### 3.2.4.1 SERVICE field

The SERVICE field is as shown in Table 3.

Table 4 - SERVICE field

| Bits | Field | Description |
| --- | --- | --- |
| B0-B6 | Scrambler Initialization |  |
| B7 | Reserved |  |
| B8-B15 | CRC | CRC calculated over VHT-SIG-B (excluding tail) |

NOTE—the Reserved and CRC fields are scrambled.

The CRC calculation and insertion is illustrated in Figure 12.

Figure 12--VHT-SIG-B and SERVICE field relationship

The CRC is calculated over the VHT-SIG-B bits, excluding the tail bits using the CRC defined in 802.11n-2009 section 20.3.9.4.4. C7 of the CRC is mapped to B8 of the SERVICE field, C6 to B7, …, C0 to B15.

The resulting SERVICE field and PSDU shall be scrambled, as in 11n.

#### 3.2.4.2 Coding

The draft specification shall support the following:

- The maximum data rate per BCC encoder is 600Mbps

- The number of BCC encoders for a particular combination of MCS, Nsts and BW is determined by the short GI data rate and the same number of encoders are used for the corresponding normal GI rate

- The number of BCC encoders is not limited

#### 3.2.4.3 Data interleaver

#### 3.2.4.3.1 Stream parser

For BCC encoding, stream parsing done in the same way as 11n, i.e.,

- Consecutive blocks of  s(iss) bits are assigned to different spatial streams in a round robin fashion.

- If multiple encoders are present per user, the output of each encoder is used in a round robin cycle, i.e.,

  - At the beginning S bits from the output of first encoder are fed into all spatial streams,

  - Then S bits from the output of the next encoder are used and so on. S is a sum of s(iss) over all streams)

In case of contiguous and noncontiguous 160 MHz transmissions, even output bits of the stream parser are allocated to the lower 80 MHz and odd output bits to the upper 80 MHz for each stream.  First output bit from the stream parser in each symbol is an even bit.

#### 3.2.4.3.2 Frequency interleaver

For BCC encoding, the interleaver parameters for 20 MHz and 40 MHz 802.11ac packets will remain unchanged from 20 MHz and 40 MHz 802.11n, i.e. the NCOL and NROT parameters for 20 MHz and 40 MHz are as in Table 20-16 of 802.11n-2009. [10/548r2]

For BCC encoding, NCOL = 26 for 80 MHz. NROT = 58 for 4 or fewer streams. The cyclic shifts applied on the different streams are given by [0 2 1 3]* NROT, identical to 11n

For BCC encoding, the encoder parsing done in the same way as in 11n, i.e., the encoder parser cycles through all the encoders in a round robin fashion assigning one bit to each encoder in each cycle. Each encoder is therefore assigned an equal number of bits.

For contiguous and noncontiguous 160 MHz transmissions using BCC encoding, the lower and upper 80 MHz portions are each interleaved using the interleaver defined for 80 MHz transmissions.

#### 3.2.4.4 Constellation mapping

The mapping between bits at the output of the interleaver and complex constellation points for 256 QAM shall be as shown in Figure 14.

Figure 14--Constellation mapping for 256 QAM

The normalization factor, KMOD, for 256 QAM is  . [10/1090r0]

#### 3.2.4.5 Pilot subcarriers

The draft specification shall have 8 pilot tones, with the positions {±103, ±75, ±39, ±11}, for 80 MHz VHT data. [10/0370r1]

For 20 and 40 MHz, the pilot sequence shall be the single spatial stream pilot sequence of 11n copied to the NSTS streams before the per-stream CSDs are applied. [10/0811r1]

For 80 MHz, the pilot sequence shall be as defined in Table 6 (which is the 40 MHz pilot sequence for NSTS = 1 extended with a [1, 1] on the right, resulting in the lowest PAPR on the pilot tones after applying [1 -1 -1 -1] rotation on the 20 MHz subbands), copied to the NSTS streams before the per-stream CSDs are applied. [10/0811r1]

Table 6--80 MHz pilot sequence

#### 3.2.4.5.1 Application of pilot sequence in 20 MHz

The pilot tone mapping in 20 MHz shall be:

where  is given by the NSTS = 1 row of Table 20-18 of Std 802.11n-2009, and where n is the VHT-DATA symbol index starting at 0.

Including the pseudo random scrambling sequence, the pilot value for the kth tone, with k = {-21, -7, 7, 21}, is pn+zPnk, where z = 4 for VHT, and where pn is defined in Section 17.3.5.9 of IEEE802.11.

#### 3.2.4.5.2 Application of pilot sequence in 40 MHz

The pilot tone mapping in 40 MHz shall be:

where  is given by the NSTS = 1 row of Table 20-19 of Std 802.11n-2009, and where n is the VHT-DATA symbol index starting at 0.

Including the pseudo random scrambling sequence, the pilot value for the kth tone, with k = {-53, -25, -11, 11, 25, 53}, is pn+zPnk, where z = 4 for VHT, and where pn is defined in Section 17.3.5.9 of IEEE802.11. This does not include the rotation per 20 MHz subband yet.

#### 3.2.4.5.3 Application of pilot sequence in 80 MHz

The pilot tone mapping in 80 MHz shall be:

where  is given in Table 6, and where n is the VHT-DATA symbol index starting at 0.

Including the pseudo random scrambling sequence, the pilot value for the kth tone, with k = {-103, -75, -39, -11, 11, 39, 75, 103}, is pn+zPnk, where z = 4 for VHT, and where pn is defined in Section 17.3.5.9 of IEEE802.11. This does not include the rotation per 20 MHz subband yet.

#### 3.2.4.5.4 Application of pilot sequence in 160 MHz

The draft specification shall have 16 pilot subcarriers, with the subcarrier indices {±25, ±53, ±89, ±117, ±139, ±167, ±203}, for 160 MHz VHT transmissions.  The pilot sequence and mapping for 160 MHz VHT transmissions shall be obtained by repeating the 80 MHz pilot sequence and mapping twice in frequency.  Specifically, the pilot sequence for the nth symbol shall be as follows, where Ψn is the 80 MHz pilot pattern:

Including the pseudo random scrambling sequence, the pilot value for the kth tone, with k = {±25, ±53, ±89, ±117, ±139, ±167, ±203}, is pn+zPnk, where z = 4 for VHT, and pn is defined in Section 17.3.5.9 of IEEE802.11-2007.  Note that this does not include the phase rotation per 20 MHz subchannel yet.

[10/0774r0]

#### 3.2.4.6 OFDM modulation

The draft specification shall have 3 DC tones at (0, ±1) in the 80 MHz VHT data field. [10/0370r1]

The draft specification shall have 5 null tones at the upper tone edges (tone indices 123, 124, 125, 126, 127) and 6 null tones at the lower tone edges (tone indices -128, -127, -126, -125, -124, -123) of the 80 MHz VHT data. [10/0370r1]

For 160 MHz VHT transmissions, the same phase rotation per 20 MHz subchannel used for preamble portion of the VHT packet shall also be applied to the data symbols.  Specifically, the following phase rotation per 20 MHz subchannel shall be applied to the data symbols, starting from the lowest 20 MHz subchannel in frequency: [c80 c80], where c80 is the phase rotation per 20 MHz subchannel for 80 MHz transmissions. [10/0774r0]

### 3.2.5 VHT Sounding PPDU

R3.2.5.A: The sounding PPDU shall be enhanced from the HT sounding PPDU to support a maximum of 8 transmit antennas.

R3.2.5.B: The draft specification shall mandate a single preamble format for sounding PPDUs.

NDP shall be the only VHT sounding format. [10/1105r0]

The VHT NDP format is shown in Figure 8 and has the following properties:

- it has the same the VHT PPDU format but with no data portion

- has a VHT-SIG-A indicating a SU packet

- and has VHT-SIG-B carrying a TBD fixed bit pattern

Figure 16--VHT NDP format

## 3.3 Modulation and coding scheme (MCS)

R3.3.A: The draft specification shall include 256 QAM. Support for 256 QAM by a VHT STA is optional. [10/0827r1]

R3.3.B: The draft specification shall maintain the 11n modulation, interleaving and coding architecture.

R3.3.C: The draft specification shall minimize the number of additional MCS options.

R3.3.D: The draft specification shall include support for a different MCS for each STA in a DL MU-MIMO transmission.

R3.3.E: The draft specification shall include an expanded MCS set for the additional spatial streams supported.

R3.3.F: The SU MCSs are shown in Table 8. MCS 9 shall not be used in 20 MHz BW transmissions. [10/0568r1]

Table 8 - VHT SU MCSs

| MCS | Modulation | Coding Rate |
| --- | --- | --- |
| 0 | BPSK | ½ |
| 1 | QPSK | ½ |
| 2 | QPSK | ¾ |
| 3 | 16-QAM | ½ |
| 4 | 16-QAM | ¾ |
| 5 | 64-QAM | 2/3 |
| 6 | 64-QAM | ¾ |
| 7 | 64-QAM | 5/6 |
| 8 | 256-QAM | ¾ |
| 9 | 256-QAM | 5/6 |

For BCC encoding, some of the MCS-NSS combinations are excluded from the MCS table to avoid additional padding symbols. Allowed MCSs are selected such that the number of coded bits in each OFDM symbol contains an integer number of punctured blocks from all encoders, i.e., mathematically every allowed MCS-NSS satisfies:

[10/820r0]

## 3.4 Spatial Multiplexing

R3.4.A: The maximum number of spatial streams (NSTS) in a SU transmission shall be 8.

R3.4.B: The maximum number of streams (NSTS) summed over all users in a MU transmission shall be 8.

R3.4.C: The maximum number of streams (NSTS) for a single user in a MU transmission shall be 4.

R3.4.D: The same modulation and the same coding rate and coding type shall be used on all streams in a SU transmission.

R3.4.E: The same modulation and the same coding rate and coding type shall be used across all streams belonging to each user in a MU transmission. [10/0382r2]

## 3.5 CCA

R3.5.A: CCA shall support detection and deferral on the 20 MHz subchannels that are busy for any possible combination of channel use and signalling bandwidth with a single transmission in an otherwise idle RF bandwidth. This includes

a) a single 20 MHz transmission on any 20 MHz sub-channel

b) a single 40 MHz transmission on either 40 MHz  sub-channel

c) a single 80 MHz transmission

R3.5.B: An 11ac device shall provide a CCA per TBD1 MHz channel, for all TBD1 MHz non-overlapping channels that the device is presently capable of transmitting over. The CCA sensitivity shall be:

- TBD2 (<-62+10log10(TBD1/20)) dBm for valid 802.11 signals

- -62+10log10(TBD1/20) dBm for any signal.

[10/744r1]

## 3.6 PMD transmit specification

### 3.6.1 Transmit center frequency

Carrier (LO) and symbol clock frequencies for all transmit chains and frequency segments shall be derived from the same reference oscillator.

Phase of carrier frequency shall not be required to be correlated between the lower and upper 80 MHz frequency portions of the transmitted signal for 160 MHz PPDUs.

### 3.6.2 Transmit spectral mask

The transmit spectral mask for 20 MHz, 40 MHz, 80 MHz and 160 MHz PPDUs is shown in Table 6.

Table 10--Transmit spectral mask for various BW PPDUs

| BW (MHz) | 0 dBr | -20 dBr | -28 dBr | -40 dBr |
| --- | --- | --- | --- | --- |
| 20 | +/- 9 | +/- 11 | +/- 20 | +/- 30 |
| 40 | +/- 19 | +/- 21 | +/- 40 | +/- 60 |
| 80 | +/- 39 | +/- 41 | +/- 80 | +/- 120 |
| 160 | +/- 79 | +/- 81 | +/- 160 | +/- 240 |

[10/1109r0]

### 3.6.3 Spectral flatness

The average energy in the subcarriers of a PPDU shall deviate by no more than that shown in Table 11.

Table 11--Average energy variation on subcarriers

| BW | Average variation |  |
| --- | --- | --- |
|  | +4/-4 dB | +4/-6 dB |
| 20 | +/- 1-16 | Outer subcarriers |
| 40 | +/- 2-42 | Outer subcarriers |
| 80 | +/- 2-84 | Outer subcarriers |
| 160 | - | All subcarriers |

Note: A noncontiguous 160 MHz devices may transmit a contiguous 160 MHz waveform by placing its two 80 MHz segments adjacent to each other. It would be difficult for such transmissions to meet the +4/-4 dB requirement near the center of the contiguous 160 MHz waveform.

[10/1109r0]

# 4 DL MU-MIMO and Transmit Beamforming

R4.A: DL MU-MIMO shall be built on one type of the 11n channel sounding PPDU and transmit beamforming protocol.

R4.B: DL MU-MIMO shall provide MAC protocol extensions to support multiple acknowledgement responses from the individual STAs receiving the MU-MIMO transmission.

R4.C: The DL MU-MIMO MAC protocol extensions shall work with EDCA

A sounding responder shall have the ability to reduce the receive side feedback dimension of its MIMO channel with explicit MU-MIMO feedback. The extent of the reduction is TBD.

[10/1114r1]

Only explicit sounding and feedback shall be supported for VHT SU beamforming and DL MU-MIMO.

[10/1105r0]

## 4.1 Resolvable LTFs for DL MU-MIMO

In a DL MU-MIMO transmission, LTFs are considered “resolvable” when the AP transmits enough LTFs for an STA to estimate the channel to all spatial streams of every recipient STA. In order to enable interference cancellation at an STA during a DL MU-MIMO transmission, an AP may transmit the preamble using resolvable LTFs

# 5 Coexistence

R5.A: Channel access rules shall ensure fair access to the medium for TGac compliant devices and legacy devices operating within a BSS or in seprate overlapping BSSs.

R5.B: The draft specification shall provide a mechanism that ensures that TGac transmissions are protected from legacy channel access for the duration of the transmission.

R5.C: The Primary Channel may be designated to any 20MHz subchannel over 80MHz channel bandwidth, where Primary Channel designation is subject to co-existence (OBSS) rules yet to be defined. [10/0593r1]

R5.D: non-ht quadruplicate and non-ht octuplicate mode shall be included in the specification: A transmission format of the physical layer (PHY) that duplicates a 20 MHz non-HT transmission in four adjacent 20 MHz channels or two sets of four adjacent 20 MHz channels.

## 5.1 Channel Access

The 802.11n PIFS medium access mechanism is extended to 80MHz and 160MHz operation, as described below:

- AIFS deferral and random backoff based on the primary channel activity

- All transmissions shall occupy the primary channel and

- Secondary channels occupied by the transmission shall be sensed idle PIFS prior to the transmission

[10/1084r0]

## 5.2 Backoff procedure

Rules in section 9.9.1.5 of 802.11n-2009 can be directly applied to the MU-MIMO acknowledgment frame exchange.

If one of the frames in the MU-PPDU requires an immediate response, a missing or incorrect immediate response indicates a failed MU-PPDU and triggers the backoff procedure.

If no immediate response to the MU-PPDU is required, the backoff procedure may still be triggered by a missing or incorrect response.

A MU PPDU is successful if none of the frames require an immediate response or if the required immediate response is correctly received.

[10/1092r0]

# 6 MAC

## 6.1 Power saving

Two mechanism are provided to terminate receive processing early. The receiver may use the length indication in VHT-SIG-B and/or use the EOF indication in the MAC padding.

## 6.2 Capability negotiations

A VHT STA indicates the following capabilities:

Max A-MPDU length supported as exponent n where 0 <= n <=7 and indicates a max A-MPDU length (2^(13+n)-1)B.

Max A-MSDU length supported as 3839B, 7935B or {11454B-Max MAC Header-FCS}.

[10/1079r1]

## 6.3 Frame formats

### 6.3.1 General frame format

The maximum MPDU size is 11454 octets. [10/1079r1]

The maximum A-MSDU size is 11454-MAC Header-FCS octets. [10/1079r1]

### 6.3.2 Frame fields

#### 6.3.2.1 HT Control field

The HT Control field is modified as follows. The reserved bit 0 of the Link Adaptation Control subfield of the Link Adaptation subfield of the HT Control field is renamed to VHT_MFB. A value of 1 for the VHT_MFB bit indicates that the value in the MFB subfield of the Link Adaptation Control subfield contains 4 bits of VHT MCS and 3 bits of NSTS, and a value of 0 for the VHT_MFB bit indicates that the value in the MFB subfield of the Link Adaptation Control subfield contains HT MCS feedback. The format change is illustrated in Figure 17.

Figure 17--Modified HT Control field

[10/1095r0]

### 6.3.3 Control frames

#### 6.3.3.1 NDPA

NDPA is a new control frame that contains the following fields:

- a 1 octet sequence number

- a STA Info field including a list of STA IDs

- other TBD information

#### 6.3.3.2 Poll

Poll is a new control frame with TBD contents.

### 6.3.4 Data frames

### 6.3.5 Management frames

### 6.3.6 Action frames

#### 6.3.6.1 Sounding feedback

The sounding feedback frame is an Action No Ack frame with

- Category VHT

- a VHT MIMO Control field with 1 octet sounding sequence and other TBD fields

- a TBD Compressed Beamforming Report field

### 6.3.7 A-MPDU format

The VHT A-MPDU format is an extension of the 802.11n A-MPDU as shown in Figure 8. The extension (shaded in the figure) consists of zero or more delimiters with MPDU length zero and a possible final MAC Pad of less than 4 octets. The A-MPDU of a VHT PPDU fills the available octets in the payload. [10/0064r5 VHT Frame Padding]

Figure 18--A-MPDU format for VHT PPDU

The A-MPDU null subframes that pad through the end of the A-MPDU shall include an end-of-frame (EOF) bit. This bit indicates that there are no additional MPDUs present in the A-MPDU.

The PSDU of a VHT PPDU shall be a VHT A-MPDU.

The A-MPDU maximum length in a VHT PPDU is 1,048,576 octets. [10/1079r1]

#### 6.3.7.1 A-MPDU delimiter format

The A-MPDU delimiter is modified as shown in Figure 9.

| B0 | B1 | B2 B3 | B4 B15 | B16 B23 | B24 B31 |
| --- | --- | --- | --- | --- | --- |
| EOF | Reserved | MPDU Length Extension | MPDU Length | CRC | Delimiter Signature |

Figure 20--Modified A-MPDU delimiter format

An MPDU Length Extension field is added in B2-B3 and contains the high order bits of MPDU length.

An EOF field is added in B0.

[10/1093r1]

## 6.4 TXOP Sharing

The TXOP duration is determined by the TXOP limit of the primary AC.

At least one stream set in each DL MU-MIMO PPDU shall contain only MSDU(s) corresponding to the primary AC, where a stream set is defined as a group of spatial streams of a DL MU-MIMO PPDU that are all intended for reception by a single recipient.

[10/1123r0]

## 6.5 Single MPDU Protocol Rules

If an A-MPDU contains a single MPDU, the initiator may set the EOF field in the A-MPDU subframe containing the MPDU to 1.

A responder that receives a QoS Data MPDU with ack policy set to “normal ack/implicit block ack req” shall respond:

With ACK if the EOF field of the subframe containing the MPDU is set to 1

With BA if the EOF field of the subframe containing the MPDU is set to 0

An initiator may send a management frame requiring an ACK response in a VHT frame provided it is the only MPDU in the VHT frame and the EOF field on the subframe carrying the MPDU is set to 1

[10/1093r1]

## 6.6 Partial AID in SU VHT PPDUs

Partial AID field in VHT-SIG A shall be set to special value(s) (TBD) for STA-to-AP packets.

AP should choose AID numbers such that the probability of AID numbers overlapping between different BSSs is reduced.

Any AID values with 9 LSB bits of 0 should not be assigned as a non-AP STA AID.

[10/1065r1]

## 6.7 Sounding and Feedback Protocol

The sounding and feedback protocol is shown in Figure 21.

Figure 21--Sounding and Feedback Protocol

The sounding feedback sequence starts with AP sending an NDP Announcement (NDPA) frame followed by an NDP (after SIFS). The NDPA identifies the first responder whose response shall follow SIFS after the NDP and may identify other STAs which will be polled subsequently. The STA identified as first by the NDPA shall send Sounding Feedback frame (SND FB) SIFS time after the NDP.

When allowed by rules in 8021.11n-2009 section 9.9.1.4 (Multiple frame transmission in an EDCA TXOP), the AP should poll all STAs in the same TXOP.

Note--Section 9.9.1.4 defines the rules that allow for sending multiple frames within a TXOP with SIFS separation. It also defines the recovery procedure in case of a missing response.

Note--in the SU case, the sequence is simply NDPA-NDP-SND FB.

### 6.7.1 Recovery mechanism

Recovery in case of a missing response follows the rules for Multiple frame transmission in an EDCA TXOP (section 9.9.1.4 in 802.11n-2009), where the sequence NDPA-NDP-1stSND FB is a valid initial frame exchange.

An example of behavior following these rules is given below.

In case first STA does not send SND FB SIFS time after NDP and the NDPA frame exchange is the first frame exchange of a TXOP, then the AP shall terminate the transmissions of the current TXOP and proceed to exponential backoff according to the AC used for medium access to send the NDPA.

If NDPA frame exchange is not the first frame exchange of a TXOP and TXOP started with a frame that received a valid response, then the AP is allowed to access the medium PIFS after NDP and send a FB Poll frame (or any other frame).

In case STAi does not send SND FB SIFS time after a FB Poll and TXOP started with a frame that received a valid response then the AP is allowed to access the medium PIFS after current FB poll and send a FB poll to retrieve SND feedback from same or next STA1.

[10/1091r0]

## 6.8 Rules for using VHT PPDU

In a downlink MU PPDU, at most one A-MPDU is allowed to contain one or more MPDUs that solicit an immediate response.

[10/1092r0]

References:

IEEE Std 802.11n-2009

09/0451r5 TGac Functional Requirements and Evaluation Methodology

09/1234r1 Interference Cancelation for Downlink MU-MIMO

10/0070r5 802.11ac Preamble

10/0382r1 Bits Consideration for SIGNAL fields

10/0578r1 Preamble Parameters

10/0568r1 Single User MCS Proposal

10/0566r1 Sounding and P Matrix Proposal

10/0370r1 80 MHz Tone Allocation

10/0593r1 Channel Selection and Management for 11ac

10/0378r1 160 MHz PHY Transmission

10/0382r2 Bits Consideration for SIGNAL fields

10/0771r0 Phase Tracking During VHT-LTF

10/0811r1 Pilot Sequence for VHT-Data

10/0876r0 802.11ac Preamble

10/0064r5 VHT Frame Padding

10/0548r2 11ac 80MHz Transmission Flow

10/0774r0 160 MHz Transmissions

10/0744r1 Improved CCA for 80 and 160 MHz BSSs

10/0773r0 80 MHz Channelization

10/0763r0 Regulatory Classes for 80 MHz Channel

10/0843r0 VHT-STF for 11AC

10/0890r0 Phase Rotations for VHT 80 MHz

10/0820r0 MCS Selection and Padding Equations

10/1123r0 TXOP Sharing for DL MU-MIMO Support

10/1093r1 A-MPDU delimiter changes

10/1065r1 AID Selection

10/1095r0 Link Adaptation Subfield for VHT

10/1079r1 Max Frame Sizes

10/1084r0 Medium Access for Wider Bandwidth

10/1052r0 VHT-SIG-A and VHT-SIG-B Field Structure

10/1109r0 Spectral Mask and Flatness

10/1105r0 11ac Explicit Sounding and Feedback

10/1063r1 160 MHz Transmission Flow

10/1090r0 256 QAM Scaling

10/1091r0 Protocol for SU and MU Sounding Feedback

10/1092r0 ACK Protocol and Backoff Procedure for MU-MIMO

10/1064r2 Channelization for 11ac

10/1114r1 Reducing Channel Dimension in MU-MIMO Explicit  Feedback Operation