IEEE P802.11
Wireless LANs

| Re-Organization of Mesh Peering Text |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-09-20 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Meiyuan Zhao | Intel Corporation | 2200 Mission College Blvd, Santa Clara, CA 95054 | +1 408 653 5517 | meiyuan.zhao@intel.com |

Abstract

This submission resolves comment CID 373, 410, 415, 423, 526, 694, 695, 764, 901, 1091

Issues identified by these comments:

- Lack of overall description to explain how the protocols interact with each other to establish and manage mesh peering and security association

- Ordering of clauses for each protocol is not the best way for readers to understand

Summary of changes:

- Re-organize the existing clauses and update the text accordingly to adjust their inter-dependency

- Modify the protocol interaction clause to clarify the protocol interaction. Added a overall state machine for full specification of protocol interaction

Baseline: 	802.11s Draft 3.03

IEEE 802.11 REVmb 1.0

802.11w Draft 10.0

Note to editor: Updates introduced in this document is tracked in changes:

Re-number the sub-clauses in clause 11C as summarized in the following:

11C.1 Mesh discovery (unchanged)

11C.2 MBSS Peering management framework

11C.2.1 Overview (new subclause)

(incorporating 11C.5 protocol interactions)

11C.2.2 Mesh Peering Instance Controller

11C.3 Mesh Peering Management protocol for non-secure peering

11C.4 Authenticated Mesh Peering Exchange handshake

11C.5 Mesh Group Key Handshake

Delete clause 11C.2 and incorporate some text in 11C.2.1 into new 11C.2.1 (details below)

Re-number 11C.3 to 11C.2.2 (Mesh Peering Instance Controller), including all text in the clause

Re-number 11C.4 to 11C.3 (Mesh peering management)

Re-number 11C.2.2 to 11C.4 (Authenticated Mesh Peering Exchange), including all text

Re-number 11C.2.3 to 11C.5 (Mesh Group Key Handshake)

Delete clause 11C.5 (protocol interactions) and move some of the text into clause 11C.2.1

Rest of the docments: re-organized and updated text for clause 11C.2, 11C.3, 11C.4, and 11C.5

Create a new clause 11C.2: MBSS peering management framework, incorporate some text from 11C.2.1 and 11C.5 (from Draft D3.03), and update the text as indicated in the following:

Note: updates on moved text are tracked. And new text are tracked changes as “insertion”.

11C.2 MBSS peering management framework

11C.2.1 Overview

MBSS Peering management framework supports all functions to establish, management, and tear down peerings between mesh STAs. When security is enabled in an MBSS, all mesh STAs shall manage the peering and the mesh TKSA, and their tight binding for each peer mesh STA.

MBSS peering management functions shall be invoked after a candidate peering mesh STA is discovered via Candidate peer mesh STA discovery procedure in 11C.1.4.

One of the following protocols shall be invoked to establish the peering with the candidate peer mesh STA:

- Mesh Peering Management protocol (MPM) that establishes and manages the peering between peer mesh STAs. When security is not enabled for MBSS, the mesh STAs shall initiate MPM protocol to manage their peerings with peer mesh STAs. See 11C.3 for MPM specification.

- Authenticated Mesh Peering Exchange protocol (AMPE) that establishes and manages the peering and mesh TKSA between peer mesh STAs. When security is enabled for MBSS, the mesh STAs shall execute AMPE protocol to manage their peerings and the bounded mesh TKSAs with peer mesh STAs.  See 11C.4 for AMPE specification

Mesh STA shall use a Mesh Peering Instance Controller (11C.2.2) to manage all peering instances established or in the process of establishment or teardown with its peer mesh STAs and candidate peer mesh STAs.

AMPE protocol initiation requires existence of a shared mesh PMK security association (Mesh PMKSA) established between the two candidate mesh peer STAs. If via discovery procedure, the mesh STA identifies an existing valid Mesh PMKSA it shares with the candidate peer mesh STA, the mesh STA shall initiate AMPE protocol directly to establish peering and mesh TKSA with the candidate peer mesh STA. If AMPE execution fails, the mesh STA shall use the mesh peering instance controller to handle failure operations. The mesh STA shall continue to discover more candidate peer mesh STAs.

If the shared MPMKSA is not identified and a commonly supported authentication protocol is negotiated via peer discovery procedure, the mesh STA shall execute the supported authentication protocol to establish a shared MPMKSA with the candidate peer mesh STA. Upon successful completion of the authentication protocol, the mesh STA shall initiate AMPE protocol to establish peering and mesh TKSA with the candidate peer mesh STA. Failure of authentication protocol execution shall terminates the peering establishment procedure with current candidate peer mesh STA, and the mesh STA shall continue to discover more candidate peer mesh STAs.

When authentication is required, the mesh STA shall execute SAE authentication protocol (see 8.2A Authentication Using a Pre-Shared Secret) to mutually authenticate with the candidate peer mesh STA using a pre-shared secret. Successful completion of SAE protocol establishes shared Mesh PMKSA for the two mesh STAs. Failure of SAE execution shall terminate the current peering establishment procedure with the candidate peer mesh STA. The mesh STA shall continue to discover more dandidate peer mesh STAs.

Figure s53 demonstrates the logical flow of protocol interactions in the peering management framework.

Note1: To support vendor specific authentication protocol, other than SAE, it is suggested a procedure of negotiating authentication protocol be inserted as indicated in Figure s53. This authentication protocol neogitiation procedure is used by two mesh STAs to determine the authentication protocol they can support for mutual authentication. If a vendor specific authentication protocol is negotiated, the execution of this authentication protocol replaces SAE procedure as shown in Figure s53. The details of the authentication protocol negotiation are out of the scope of this specification and are dependent on the vendor specific authentication protocol that it supports.

Note2: SAE protocol assumes no existing peering between two mesh STAs for authentication completion. If the active authentication protocol requires existence of peering for protocol information exchange, it is suggested to execute mesh peering management protocol (MPM) to establish a peering to support authentication. For instance, EAP authentication uses data frames to transport EAP messages. Hence, a peering can be established to support EAP authentication. Once authentication completes, successfully or failed, the peering instance mesh  is used to tear down the peering established for authentication purpose only.

e Logical flowchart of protocol interaction.

- Logical flowchart of protocol interaction in Mesh Peering Management Framework

In addition to managing peering and mesh TKSA, the mesh STA shall also manage Mesh GTKSA of the mesh STA and the Mesh GTKSA of every peer mesh STA.

AMPE protocol establishes the Mesh GTKSAs for the two peer mesh STAs in addition to the shared mesh TKSA. The mesh STA may update its GTK for broadcast/multicast data confidentiality protection. In this case, the separate mesh group key handshake protocol (see 11C.5) shall be initiate to update the GTK of the mesh STA for all its peer mesh STAs.

11C.2.2 Meshing Peering Instance Controller

11C.2.2.1 functions

The mesh STA shall maintain a Mesh Peering Instance Controller to manage peering instances by Mesh Peering Management protocol and Authenticated Mesh Peering Exchange Protocol.

The Mesh Peering Instance Controller shall support the following functions:

- Create and destroy MPM finite state machines and Authenticated Mesh Peering Exchange finite state machines

- Manage identifier and mesh TKSA states for each mesh peering instance

- Pre-process the mesh peering instance identifier of the incoming mesh peering management frames and pass the frames to the corresponding protocol finite state machine with matching instance identifier

- Pass internal command to corresponding protocol finite state machine which has matching instance identifier

11C.2.2.2 Creating mesh peering instance and Mesh TKSA for a peer mesh STA

The mesh peering instance controller may actively generate a new protocol finite state machine after successful peer discovery. The ACT_OPN event shall be passed to the new finite state machine. If a mesh PMKSA is established with the candidate peer mesh STA, the mesh peering instance controller shall generate an Authenticated Mesh Peering Exchange finite state machine.

Multiple mesh peering instances with the same candidate peer mesh STA may be initiated at any time. Once a mesh peering is established successfully, all other mesh peering instances with the same peer mesh STA shall be closed properly.

A new mesh peering instance may be started when the mesh STA already maintains a valid mesh peering with the same peer mesh STA, due to the change of some mesh peering parameter. Once the new mesh peering is established successfully, the previous valid mesh peering shall be closed properly.

Note: This may also happen when a peering is established temporarily to enable authentication that utilizes the peering. Upon successful authentication, the AMPE protocol is executed to establish the peering and mesh TKSA with the same peer mesh STA. Once successful, the peering for temporary authentication purose should be closed propertly.

The mesh peering and corresponding Mesh TKSA is established by AMPE. Once a mesh PMKSA is established with a candidate peer mesh STA or a peer mesh STA, only the Authenticated Mesh Peering Exchange finite state machine shall be created to establish the mesh peering and the mesh TKSA with the candidate peer mesh STA. In this case, a Mesh Peering Open frame from the candidate peer mesh STA that requests to establish a new mesh peering using Mesh Peering Management protocol shall be rejected.

11C.2.2.3 Deleting mesh peering instances

To actively close a mesh peering instance, the Mesh Peering Instance Controller shall invoke a CNCL event in the AMPE finite state machine. The CNCL event will trigger closing the mesh peering instance as well as the Mesh TKSA that is bound to the mesh peering.

The mesh peering instance closure may be triggered by receipt of a Mesh Peering Close frame from the peer mesh STA or candidate peer mesh STA. The Mesh Peering Close frame shall be passed to the corresponding AMPE state machine to further processing.

When the finite state machine of the mesh peering instance transitions back to IDLE state, the tearing down of this mesh peering instance completes and the mesh peering instance controller shall destroy the corresponding finite state machine.

11C.2.2.4 Pre-processing Mesh Peering Management Frames

Each mesh peering instance shall be identified by the mesh peering instance identifier. The MPM FSMs are identified by a set of data including localLinkID, peerLinkID, localMAC, and peerMAC. The Authenticated Mesh Peering Exchange FSMs are identified by localNonce, peerNonce, and PMKName, in addition to the data set for mesh peering instance identifier.

The mesh STA shall pre-process the incoming mesh peering management frame. As the result, the mesh peering instance controller shall either discard the frame or pass it to the corresponding active mesh peering instance finite state machine for further processing.

If the Mesh Peering Protocol Identifier is set to “Mesh Peering Management Protocol”, Mesh Peering Management information element shall be pre-processed to identify the mesh peering instance. In addition, the Authenticated Mesh Peering information element and MIC, if present, shall be ignored.

If the Mesh Peering Protocol Identifier is set to “Authenticated Mesh Peering Exchange”, the Mesh Peering Management information element, RSN information element, and Authenticated Mesh Peering Exchange information element shall be pre-processed to identify the mesh peering instance. The frame shall be silently discarded if the Authenticated Mesh Peering Exchange or MIC information element doesn’t exist in the frame.

If the frame contains a group address in TA or RA, it shall be silently discarded.

The instance identifier in the frame shall be processed next. The incoming mesh peering management frame belongs to an active mesh peering instance, if the mesh peering identifier in the incoming frame matches an existing active mesh peering instance. To match a mesh peering instance,

- If a mesh peering instance is identified by MAC addresses and Link IDs by both mesh STAs

- The sender’s MAC address shall be the same as the peerMAC of the mesh peering instance

- The receiver’s MAC address shall be the same as the localMAC of the mesh peering instance

- The value of Local Link ID field shall be the same as the peerLinkID of the mesh peering instance

- The value of Peer Link ID field (if exists) shall be the same as the localLinkID of the mesh peering instance

- If the matching fails, and there exists a mesh peering instance that is identified by only the localMAC and localLinkID, the incoming Mesh Peering Open frame or Mesh Peering Close frame with no value set to Peer Link ID field shall match this mesh peering instance and the peerMAC and peerLinkID of the mesh peering instance are set accordingly.

If the incoming mesh peering management frame is for Authenticated Mesh Peering Exchange (as specified in Mesh Peering Protocol Identifier field in the Mesh Peering Management information element), the identifier shall further match the identifier for Authenticated Mesh Peering Exchange. If the chosen PMK from the frame is different than the PMKName that identifies the mesh PMKSA that the mesh STA establishes with the candidate peer mesh STA, the incoming frame is a mismatch.

If the received chosen PMK is a match, the mesh peering instance controller shall further examine the nonces in the frame.

- If the matched mesh peering instance by MAC addresses and Link IDs has also peerNonce, the incoming frame is a match if

- The value of Local Nonce field shall be the same as the peerNonce of the mesh peering instance, and

- The value of Peer Nonce field (if exists) shall be the same as the localNonce of the mesh peering instance

- If the matched mesh peering instance by MAC addresses and Link IDs does not have peerNonce, the incoming Mesh Peering Open frame or the Mesh Peering Close frame with no value set to Peer Nonce field shall match this mesh peering instance. The peerNonce of the mesh peering instance is set accordingly.

The mismatched Mesh Peering Confirm frame or Mesh Peering Close frame shall be silently discarded.

The mesh peering instance controller treats the mismatched incoming Mesh Peering Open frame as a request to establish a new mesh peering, or a new mesh peering and a mesh TKSA if the frame is for Authenticated Mesh Peering Exchange protocol.

When the mesh STA has established a mesh PMKSA with the candidate peer mesh STA, the mesh peering instance controller shall silently discard the Mesh Peering Open frame in the following two conditions:

- The Mesh Peering Open frame supports Mesh Peering Management protocol, or

- The Mesh Peering Open frame supports Authenticated Mesh Peering Exchange but the mesh STA does not support the mesh PMKSA as identified by PMKName in the Chosen PMK field in Authenticated Mesh Peering Exchange information element.

If the Mesh Peering Open frame is not discarded the mesh peering instance controller may generate a new protocol finite state machine and actively reject or accept the mesh peering open request. A new local link ID shall be generated for the mesh peering instance. If the mesh peering instance is to be established by Authenticated Mesh Peering Exchange, a random local nonce shall be generated for identifying the mesh peering instance as well.

The mesh peering open request may be rejected due to an internal reason. If the mesh peering open request is rejected, the REQ_RJCT event shall be passed to the newly generated protocol finite state machine.

NOTE— Example internal reasons to reject new mesh peering request could be the mesh STA has reached its capacity to set up more mesh peering, the mesh STA is configured to reject mesh peering request from another specific peer mesh STA.

When the mesh peering open request is accepted, the received Mesh Peering Open frame shall be passed to the newly generated protocol finite state machine.

11C.3 Mesh Peering Management protocol (MPM)

11C.3.1 Overview

The Mesh Peering Management protocol is used to establish, maintain, and close mesh peerings between mesh STAs when security is not required. gThis clause summarizes the protocol operations. Clause 11C.3.3 specifies the protocol details.

Mesh STAs shall not transmit frames other than the ones used for discovery, mesh peering management, and SAE to a neighboring mesh STA until a mesh peering has been established with the mesh STA.

After discovering a candidate peer mesh STA, the mesh STA may start the Mesh Peering Management protocol to establish a mesh peering with the candidate peer mesh STA. The SME controlling the mesh STA uses the Mesh Peering Instance Controller to manage mesh peering instances. A mesh peering instance is a logical entity that the mesh STA uses to handle a mesh peering or an attempt of establishing a mesh peering. Its behavior is governed by a mesh peering management finite state machine defined in 11C.3.3 Mesh Peering Management Finite State Machine. The behavior of the Mesh Peering Instance Controller is defined in Error: Reference source not found.

Each mesh peering instance is identified by a mesh peering instance identifier. The mesh peering management finite state machine identifier is defined as <localMAC, peerMAC, localLinkID, peerLinkID>. localMAC is the MAC address of the mesh STA that is being used with this mesh peering instance. peerMAC is the MAC address of the peer mesh STA or the candidate peer mesh STA. localLinkID is an integer generated by the mesh STA. peerLinkID is an integer generated by the peer mesh STA or the candidate peer mesh STA. The localLinkID shall be unique among all existing link identifiers used by the mesh STA for its current mesh peering management finite state machines. The mesh STA selects the localLinkID to provide high assurance that the same number has not been used to identify a recent mesh peering management finite state machine. The peerLinkID shall be supplied by the peer mesh STA or candidate peer mesh STA in Mesh Peering Open and Confirm frames. The mesh peering management finite state machine identifiers are transmitted via mesh peering management frames.

A mesh STA shall assign an AID to every peer mesh STA during the mesh peering establishment procedure. Mesh STAs shall assign AID values uniquely to each of the peer mesh STAs. AID 0 (zero) is reserved to indicate the presence of buffered groupcast MSDUs and MMPDUs.

The mesh STA shall keep information of mesh peering management finite state machine identifier and the respective policy as the states of the mesh peering management finite state machine. The actual method of handling the states is out of the scope of this specification.

The mesh STA shall start the mesh peering management protocol in either of the following two cases. In case one, the mesh STA passively listen to incoming Mesh Peering Open requests from a candidate peer mesh STA. In case two, the mesh STA actively creates a mesh peering instance to establish a mesh peering with a candidate peer mesh STA.

A mesh peering instance ends when the mesh peering is closed. The mesh peering close can be caused by the mesh STA due to events outside the scope of this specification. The response to these events is beyond the scope of this standard.

The mesh STA uses the Mesh Peering Management frames to manage a mesh peering instance.

A Mesh Peering Open frame requests that a mesh peering instance be established between the Mesh Peering Open sender and the receiver. Mesh Peering Open frames propose mesh parameters for the mesh peering instance. The mesh STA processes the received parameters. If it agrees with the parameters, the mesh STA sends a Mesh Peering Confirm frame in response to the Mesh Peering Open frame. If the mesh STA disagrees on the parameters or there is a failure due to other reasons, the mesh STA closes the mesh peering by sending a Mesh Peering Close frame. The protocol succeeds in establishing a mesh peering when the following requirements are satisfied: 1) both mesh STAs have sent and received (and correctly processed) a Mesh Peering Open frame regarding this mesh peering; 2) both mesh STAs have sent and received (and correctly processed) a corresponding Mesh Peering Confirm frame regarding this mesh peering.

The mesh peering management protocol uses timers to control the protocol behavior. For each mesh peering instance, there are three timers: retryTimer, confirmTimer, and holdingTimer. The retryTimer controls the maximum time the mesh STA waits for a Mesh Peering Confirm frame responding to any Mesh Peering Open frame the mesh STA has sent for that mesh peering instance. The confirmTimer bounds the time that the mesh STA waits for a Mesh Peering Open frame after receiving a Mesh Peering Confirm frame from the candidate peer mesh STA. The holdingTimer is used to provide a graceful closing period for a mesh peering instance.

The detailed protocol behavior is specified in mesh peering management finite state machine as specified in 11C.3.3 Mesh Peering Management Finite State Machine.

11C.3.2 Processing Mesh Peering Management Frames

11C.3.2.1 Overview

Mesh Peering management frames are Public Action frames. The protection of the mesh peering management frames shall be provided by the mesh peering management protocol, independent of the robust management protection mechanism.

The construction of mesh peering management frames is specified in 11C.3.2.2.1 Mesh Peering Open frame contents, 11C.3.2.3.1 Mesh Peering Confirm frame contents, and 11C.3.2.4.1 Mesh Peering Close frame contents.

The mesh STA shall process mesh peering instance identifier in the incoming mesh peering management frame using the Mesh Peering Instance Controller. Mesh Peering Management frames accepted by the Mesh Peering Instance Controller shall be passed to the corresponding mesh peering management state machine for further processing.

The mesh peering management finite state machine shall classify the incoming mesh peering management frames to decide either to accept, reject, or silently discard the frame. If the frame contains a group address in TA or RA, it shall be silently discarded. Processing of mesh peering management frames is specified in 11C.3.2.1 Overview, 11C.3.2.2.2 Processing Mesh Peering Open frames, 11C.3.2.3.2 Processing Mesh Peering Confirm frames, and 11C.3.2.4.2 Processing Mesh Peering Close frames. If the incoming frame is accepted or rejected, further actions shall be taken according to the mesh peering management finite state machine as specified in 11C.3.3 Mesh Peering Management Finite State Machine.

The mesh STA shall process the configuration parameters conveyed in the Mesh Peering Open and Mesh Peering Confirm frames in a mesh peering instance. Mesh STA shall verify that the parameters reported by the candidate peer mesh STA in the Mesh Peering Confirm frame match the parameter values the mesh STA has agreed to use for this mesh peering instance. In particular, the mesh STA shall verify the following fields or subfields. This verification is needed to satisfy the consistency property, i.e., to guarantee that mesh STAs agree on the configuration before establishing a mesh peering.

- Mesh ID element

- Fields in Mesh Configuration element

- Active Path Selection Protocol Identifier field

- Active Path Selection Metric Identifier field

- Congestion Control Mode Identifier field

- Synchronization Protocol Identifier field

- Authentication Protocol Identifier field

- Mesh Capability field, including the following subfields

- Accepting Mesh Peerings

- MCCA Enabled

- Supported rates element, Extended Supported Rates element, and HT Operation element if present

Mesh STAs shall verify that the same Mesh ID is used.

Mesh STAs shall verify that the same Path Selection Protocol and the same Path Selection Metric are used.

The mesh STA shall verify that it supports MCCA services when the candidate peer mesh STA sets the “MCCA Enabled” field to 1.

Mesh STAs shall verify that the same Authentication Protocol is used. In addition, if the frame indicates the Mesh Peering Management protocol, the RSN information element, Authenticated Mesh Peering Exchange information element, and MIC field shall be ignored.

Mesh STAs shall verify that the candidate peer mesh STA uses the same BSSBasicRateSet parameter.

HT mesh STAs shall verify that the candidate peer mesh STA uses the same BSSBasicMCSSet parameter when the candidate mesh STA is a HT STA.

If any of the verifications fails, the received Mesh Peering Open or Mesh Peering Confirm frame shall be rejected and mesh peering establishment attempt shall be terminated. Further actions of closing a mesh peering instance is specified in 11C.3.3 Mesh Peering Management Finite State Machine.

When two HT mesh STA establish a mesh peering, if they support a common HT capability, the HT feature may be used for the mesh peering communication.

11C.3.2.2 Mesh Peering Open frames

11C.3.2.2.1 Mesh Peering Open frame contents

A mesh STA initiates the establishment of a mesh peering with a candidate peer mesh STA by sending a Mesh Peering Open frame to the candidate peer mesh STA. The Mesh Peering Open frame shall contain:

- In the Mesh Peering Management information element, the Mesh Peering Protocol Identifier shall be set to 00-0F-AC:0 “Mesh Peering Management Protocol”.

- Mesh ID, which shall be set to the Mesh ID supported by the mesh STA’s mesh profile.

- Mesh Configuration

- Active Path Selection Protocol ID field shall be set to the value supported by the mesh STA’s mesh profile

- Active Path Selection Metric ID field shall be set to the value supported by the mesh STA’s mesh profile

- Congestion Control Mode ID shall be set to the value supported by the mesh STA's mesh profile.

- Synchronization Protocol ID shall be set to the value supported by the mesh STA's mesh profile.

- Authentication Protocol ID shall be set to the value supported by the mesh STA's mesh profile.

- Mesh Capability field

- Accept Mesh Peerings field is set to 1

- MCCA Enabled field is set according to the mesh STA’s configuration

- Mesh Peering Management element shall contain the Local Link ID field. The Local Link ID field shall be set to the identifier generated to identify the current mesh peering instance.

- Other information elements as defined in Error: Reference source not found may be present and set to the value according to mesh STA’s parameter configuration.

11C.3.2.2.2 Processing Mesh Peering Open frames

The mesh STA shall first process the Mesh ID element and Mesh Configuration element as specified in 11C.3.2.1 Overview.

The mesh STA shall reject the Mesh Peering Open frame if it contains an unacceptable or erroneous configuration parameter (see 11C.3.2.1 Overview) or the value of a configuration parameter (see 11C.3.2.1 Overview) is not the same as the value in either a Mesh Peering Open frame or a Mesh Peering Confirm frame received earlier for this mesh peering instance.

In other cases, the mesh STA shall accept the Mesh Peering Open frame. The mesh peering state shall be updated to include the mesh peering instance identifier and other information from Mesh Configuration element. The mesh STA may also update the mesh peering state based on other parameters in the Mesh Peering Open frame.

11C.3.2.3 Mesh Peering Confirm frames

11C.3.2.3.1 Mesh Peering Confirm frame contents

A mesh STA sends a Mesh Peering Confirm frame to the candidate peer mesh STA to respond the Mesh Peering Open frame sent by candidate peer mesh STA. The Mesh Peering Confirm frame shall contain:

- In the Mesh Peering Management information element, the Mesh Peering Protocol Identifier shall be set to 00-0F-AC:0 “Mesh Peering Management Protocol”.

- Mesh ID element and Mesh Configuration element shall be set to the same value as in the received Mesh Peering Open frame.

- Mesh Peering Management element shall contain the Local Link ID field and Peer link ID field. The Local Link ID field shall be set according to the local state of localLinkID. The Peer link ID field shall be set to the same value as received in the Mesh Peering Open frame by the candidate peer mesh STA.

- Other information elements as defined in Error: Reference source not found may be present and set to the value according to mesh STA’s parameter configuration.

11C.3.2.3.2 Processing Mesh Peering Confirm frames

The mesh STA shall silently discard the Mesh Peering Confirm frame if it contains a mismatched instance identifier.

The mesh STA shall reject the Mesh Peering Confirm frame if it contains an unacceptable or erroneous configuration parameter (see 11C.3.2.1 Overview) or the value of a configuration parameter (see 11C.3.2.1 Overview) is not the same as the value from a frame (either a Mesh Peering Open frame or a Mesh Peering Confirm frame) received earlier during the mesh peering instance establishment attempt.

In other cases, the mesh STA shall accept the Mesh Peering Confirm frame. The mesh peering state shall be updated to include the mesh peering instance identifier and other information from Mesh Configuration element. The mesh STA may also update the mesh peering state based on other parameters in the Mesh Peering Confirm frame.

11C.3.2.4 Mesh Peering Close frames

11C.3.2.4.1 Mesh Peering Close frame contents

A mesh STA sends a Mesh Peering Close frame to the peer mesh STA or candidate peer mesh STA to close a mesh peering instance or to terminate an attempt to establish a mesh peering. The Mesh Peering Close frame shall contain:

- In the Mesh Peering Management information element, the Mesh Peering Protocol Identifier shall be set to 00-0F-AC:0 “Mesh Peering Management Protocol”.

- Mesh ID shall be set to the same value as in the mesh STA’s mesh profile.

- Mesh Peering Management element shall contain the Local Link ID field, which shall be set according to the local state of localLinkID. If the mesh STA has local state of peerLinkID, the mesh STA shall include Peer link ID field in the Mesh Peering Management element and the value shall be set according to the local state of peerLinkID.

- Reason code shall be set to the value that specifies the reason to close the mesh peering instance. See Error: Reference source not found.

11C.3.2.4.2 Processing Mesh Peering Close frames

The mesh STA shall reject the Mesh Peering Close frame if the value in the Mesh ID element is not the same as the mesh STA’s mesh profile. Otherwise, the mesh STA shall accept the Mesh Peering Close frame.

11C.3.3 Mesh Peering Management Finite State Machine

11C.3.3.1 General

Each mesh peering instance, including its states and resource, shall be managed by a mesh peering management finite state machine (MPM FSM). The MPM FSM uses MLME primitives to control the mesh STA to send and receive Mesh Peering Management frames.

11C.3.3.2 States

The mesh peering management finite state machine uses the following six states:

- IDLE —IDLE state is a terminal state. In the IDLE state, the MPM FSM is ready to start a new mesh peering instance by either passively listening for an incoming Mesh Peering Open frame or actively initiating a mesh peering instance.

- OPN_SNT — In the OPN_SNT state, the finite state machine has actively sent a Mesh Peering Open frame and is waiting for the incoming Mesh Peering Open and Mesh Peering Confirm frames from the candidate peer mesh STA.

- CNF_RCVD — In the CNF_RCVD state, the finite state machine has received a Mesh Peering Confirm frame, but has not received a Mesh Peering Open frame; therefore the mesh STA has not sent the corresponding Mesh Peering Confirm frame.

- OPN_RCVD —In the OPN_RCVD state, the finite state machine has received only the Mesh Peering Open frame but not the Mesh Peering Confirm. The mesh STA has also sent a Mesh Peering Confirm frame upon receiving a Mesh Peering Open frame.

- ESTAB — In the ESTAB state, the finite state machine has received both the Mesh Peering Open and Mesh Peering Confirm frames. The mesh STA has also sent both the Mesh Peering Open frame and Mesh Peering Confirm frame. The mesh peering is established and configured for exchanging frames with the peer mesh STA in the ESTAB state.

- HOLDING — In the HOLDING state, the finite state machine is closing the mesh peering instance with the peer mesh STA or the candidate peer mesh STA.

11C.3.3.3 Events and Actions

The finite state machine uses three types of events: events for state machine controlling external events generated by frame processing, and events associated with internal timers.

The state machine controlling events are:

- CNCL(localLinkID, peerMAC, ReasonCode) — Used to instruct the mesh peering instance to cancel the mesh peering with the peer mesh STA. localLinkID identifies the MPM FSM for the corresponding mesh peering instance. peerMAC is the MAC address of the peer mesh entity. ReasonCode is used to inform the reason to cancel the mesh peering instance.

- ACTOPN(peerMAC, localLinkID) — The SME uses this event to create a new mesh peering instance to actively initiate the mesh peering establishment with the candidate peer mesh STA whose MAC address is peerMAC. localLinkID identifies the MPM FSM.

The events generated by frame processing are

- OPN_ACPT — PeeringOpen_Accept(peerMAC, peerLinkID) event indicates that a Mesh Peering Open frame meeting the correctness criteria of 11C.3.2.2 Mesh Peering Open frames has been received from peerMAC for the mesh peering instance identified by peerLinkID.

- OPN_RJCT — PeeringOpen_Reject(peerMAC, peerLinkID, Configuration, ReasonCode) event indicates that a Mesh Peering Open frame from peerMAC for the mesh peering instance identified by peerLinkID is rejected due to incomplete or errorenous configuration or other internal reasons. The ReasonCode can be set to MESH-CONFIGURATION-POLICY-VIOLATION.

- CNF_ACPT — PeeringConfirm_Accept(peerMAC, localLinkID, peerLinkID) event indicates that a Mesh Peering Confirm frame meeting the correctness criteria of 11C.3.2.3 Mesh Peering Confirm frames has been received from peerMAC for the mesh peering instance identified by localLinkID and peerLinkID.

- CNF_RJCT — PeeringConfirm_Reject(peerMAC, localLinkID, peerLinkID, ReasonCode) event indicates that a Mesh Peering Confirm frame from peerMAC for the mesh peering instance identified by localLinkID and peerLinkID is rejected due to incomplete or errorenous configuration. The ReasonCode is set to MESH-CONFIGURATION-POLICY-VIOLATION.

- CLS_ACPT — PeeringClose_Accept(peerMAC, localLinkID, peerLinkID, reasonCode) event indicates that a Mesh Peering Close frame meeting the correctness criteria of 11C.3.2.4 Mesh Peering Close frames has been received from peerMAC for the mesh peering instance identified by localLinkID and peerLinkID. The reasonCode specifies the reason that caused the generation of the Mesh Peering Close frame.

- REQ_RJCT – PeeringRequest_Reject(peerMAC, peerLinkID, reasonCode) event indicates a special incidence that the mesh STA rejects the incoming Mesh Peering Open frame requesting to set up a new mesh peering due to internal reason. The incoming request is identified by the peerMAC and peerLinkID as received from the Mesh Peering Open frame. The reasonCode is set to PEER_LINK_CLOSED.

The internal events are as follows. The term Timeout(localLinkID, item) represents a timeout identified locally by item, for the mesh peering instance identified by localLinkID.

Three types of timers are used by the finite state machine.

The retryTimer triggers a re-send of the Mesh Peering Open frame when a Mesh Peering Confirm frame was not received as a response.

The confirmTimer signals that a link establishment attempt should be aborted because a Mesh Peering Confirm frame responding to a Mesh Peering Open frame was never received. The holdingTimer signals that its mesh peering instance may be completely closed and facilitates graceful shutdown.

- TOR1— This event refers to Timeout(localLinkID, retryTimer) and the dot11MeshMaxRetries has not been reached. The Mesh Peering Open frame shall be resent.

- TOR2—This event refers to Timeout(localLinkID, retryTimer) and the dot11MeshMaxRetries has been reached. The mesh peering instance shall be closed when TOR2 occurs.

- TOC— The Timeout(localLinkID, confirmTimer) event. The confirmTimer aborts a mesh peering establishment attempt if a Mesh Peering Open frame never arrives after receiving the Mesh Peering Confirm frame. TOC event occurs, the mesh peering instance shall be closed.

- TOH event— The Timeout(localLinkID, holdingTimer) event. The holdingTimer allows a grace period for closing the mesh peering instance; it is necessary to avoid deadlocks and livelocks that arise due to interactions between mesh peering establishment and termination. When TOH occurs, the mesh peering instance shall be closed completely and the finite state machine shall transition to IDLE state.

The finite state machine may take an action triggered by an event. It uses two types of actions: sending a mesh peering management frame and handling a timer.

Actions related to sending a mesh peering management frame:

- sndOPN — The sendOpen(peerMAC, localLinkID, Configuration) is the action that the mesh STA takes to send a Mesh Peering Open frame to the candidate peer mesh STA, whose MAC address is peerMAC. The frame shall carry localLinkID and the supported Mesh Configuration, as specified in Configuration.

- The MLME-MeshPeeringManagement.request primitive shall be invoked to send the frame to the peer mesh entity.

- sndCNF — The sendConfirm(peerMAC, localLinkID, peerLinkID, Configuration) is the action that the mesh STA takes to send a Mesh Peering Confirm frame to the candidate peer mesh STA, whose MAC address is peerMAC. The frame shall carry localLinkID, peerLinkID, and the supported Mesh Configuration, as specified in Configuration.

- The MLME-MeshPeeringManagement.request primitive shall be invoked to send the frame to the peer mesh entity.

- sndCLS — The sendClose(peerMAC, localLinkID, peerLinkID, reasonCode) is the action that the mesh STA takes to send a Mesh Peering Close frame to the peer mesh STA or candidate peer mesh STA, whose MAC address is peerMAC. The MLME-MeshPeeringManagement.request primitive shall be invoked to send the frame to the peer mesh entity.

The actions on handling timers are setTimer(localLinkID, item, value) and clearTimer(localLinkID, item).

- The setTimer(localLinkID, item, timeout) action sets the timeout value specified by timeout to the timer specified by item. This action only sets the timer for one time for the mesh peering instance identified by localLinkID. When the timeout time has passed, the timer expires and the event Timeout(localLinkID, item) is triggered, after which the timer is no longer in effect.

The corresponding actions are denoted as setR, setC, setH, for timer retryTimer, confirmTimer, holdingTimer respectively.

Before setting the retryTimer, the finite state machine shall apply the default mesh peering open request backoff algorithm to compute the updated timeout value as the following:

timeout = return timeout + (getRandom mod timeout),

where getRandom routine generates a random value. The initial value of timeout shall be set to dot11MeshRetryTimeout. This function statistically increases the length of time for each Mesh Peering Open retry by 50%. The backoff was inserted into the design to recover from a “gold rush”, which could happen if several already-linked mesh STAs simultaneously detected a new mesh STA trying to enter the MBSS.

- The clearTimer(localLinkID, item) action clears the timer item for the mesh peering instance identified by localLinkID. The corresponding actions are denoted as clR, clC, clH, for timer retryTimer, confirmTimer, holdingTimer respectively.

11C.3.3.4 State transitions

Mesh Peering Management Finite State Machine and Finite State Machine of Mesh Peering Management Protocol summarize the state transitions for the mesh peering management protocol.

In Mesh Peering Management Finite State Machine, each row represents state transitions from the state to all other states. The blank entry means impossible transition.

| - Mesh Peering Management Finite State Machine |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | To State |  |  |  |  |  |
|  |  | IDLE | OPN_SNT | CNF_RCVD | OPN_RCVD | ESTAB | HOLDING |
| From State | IDLE | REQ_RJCT / sndCLS | ACTOPN/ (sndOPN, setR) |  | OPN_ACPT/ (sndOPN, sndCNF, setR) |  |  |
|  | OPN_SNT |  | TOR1/ (sndOPN, setR) | CNF_ACPT/ (clR, setC) | OPN_ACPT/ (sndCNF) |  | CLS_ACPT, OPN_RJCT, CNF_RJCT, TOR2, CNCL/ (sndCLS, clR, setH) |
|  | CNF_RCVD |  |  |  |  | OPN_ACPT / (clC, sndCNF) | CLS_ACPT, OPN_RJCT, CNF_RJCT, CNCL/ (sndCLS, clC, setH) TOC / (sndCLS, setH) |
|  | OPN_RCVD |  |  |  | OPN_ACPT/ sndCNF TOR1 / (sndOPN, setR) | CNF_ACPT / clR | CLS_ACPT, OPN_RJCT, CNF_RJCT,TOR2, CNCL/ (sndCLS, clR, setH) |
|  | ESTAB |  |  |  |  | OPN_ACPT / sndCNF | CLS_ACPT, OPN_RJCT, CNF_RJCT, CNCL/ (sndCLS, setH) |
|  | HOLDING | TOH/—, CLS_ACPT /clH |  |  |  |  | OPN_ACPT, CNF_ACPT, OPN_RJCT, CNF_RJCT/ sndCLS |

In Finite State Machine of Mesh Peering Management Protocol, each arrow represents a state transition. All other events not shown in Finite State Machine of Mesh Peering Management Protocol indicates that no action causes the state transition.

- Finite State Machine of Mesh Peering Management Protocol

The event/action representation is defined as the following. “E/A” string represents that the action A is taken given that the event E occurs. “E1, E2/A” string represents that the action A is taken given that the event E1 or event E2 occurs. “E/(A1, A2)” string represents that the action A1 and A2 are taken at a time when event E occurs.

Note that Mesh Peering Management Finite State Machine and Finite State Machine of Mesh Peering Management Protocol are used for illustration purpose. The protocol behavior is in the following subclauses.

11C.3.3.5 IDLE state

In the IDLE state the finite state machine shall be ready to establish a new mesh peering.

When ACTOPN event occurs, the mesh STA shall initiate the retryCounter to zero, and send a Mesh Peering Open frame to the candidate peer mesh STA whose address is peerMAC. The retryTimer shall be set according to retryTimeout. The finite state machine shall transition to OPN_SNT state.

When an OPN_ACPT event occurs, the mesh STA shall send the corresponding Mesh Peering Confirm frame to respond to the Mesh Peering Open frame. It shall also send a Mesh Peering Open frame to request a Mesh Peering Confirm frame from the candidate peer mesh STA. The retryTimer is set according to dot11MeshRetryTimeout value. The finite state machine shall transition to OPN_RCVD state.

When an REQ_RJCT event occurs, a Mesh Peering Close frame shall be sent to reject the mesh peering open request. The reason code the Mesh Peering Close frame shall be set to the same as the reason code in REQ_RJCT event. The finite state machine shall stay in the IDLE state.

All other events shall be ignored in this state.

11C.3.3.6 OPN_SNT state

In the OPN_SNT state, the mesh STA waits for a Mesh Peering Confirm frame. In this state, the retryTimer is set.

When a CNCL event occurs, the mesh STA shall clear the retryTimer, send a Mesh Peering Close frame with reason code MESH-PEERING-CANCELLED, and set the holdingTimer according to the value of dot11MeshHoldingTimeout. The finite state machine shall transition to HOLDING state.

When a CLS_ACPT event occurs, the mesh STA shall clear the retryTimer, send a Mesh Peering Close frame with reason code MESH-CLOSE-RCVD, and set the holdingTimer according to the value of dot11MeshHoldingTimeout. The finite state machine shall transition to HOLDING state.

When an OPN_ACPT event occurs, the mesh STA shall send the corresponding Mesh Peering Confirm frame to respond to the incoming Mesh Peering Open frame. The finite state machine shall transition to OPN_RCVD state.

NOTE—The retryTimer is still in effect after the state transition.

When an OPN_RJCT event occurs, the mesh STA shall clear the retryTimer, send a Mesh Peering Close frame with reason code specified by the OPN_RJCT event, and set the holdingTimer according to the value of dot11MeshHoldingTimeout. The finite state machine shall transition to HOLDING state.

When a CNF_ACPT event occurs, the mesh STA shall clear the retryTimer and shall set the confirmTimer according to the value of dot11MeshConfirmTimeout and the finite state machine shall transition to CNF_RCVD state.

When a CNF_RJCT event occurs, the mesh STA shall clear the retryTimer, send a Mesh Peering Close frame with reason code specified by the CNF_RJCT event, and set the holdingTimer according to the value of dot11MeshHoldingTimeout. The finite state machine shall transition to HOLDING state.

When a TOR1 event occurs, the Mesh Peering Open frame shall be resent and the retryCounter shall be incremented. The retryTimer shall be set according to the updated retryTimeout computed by the backoff algorithm. The finite state machine shall stay in the OPN_SNT state.

When a TOR2 event occurs, the mesh STA shall send a Mesh Peering Close frame with reason code MESH-MAX-RETRIES. The holdingTimer shall be set according to the value of dot11MeshHoldingTimeout, and the finite state machine shall transition to HOLDING state.

All other events shall be ignored in this state.

11C.3.3.7 CNF_RCVD state

In the CNF_RCVD state, the mesh STA has received a Mesh Peering Confirm frame and is waiting for a Mesh Peering Open frame.

When a CNCL event occurs, the mesh STA shall clear the confirmTimer, send a Mesh Peering Close frame with the reason code MESH-PEERING-CANCELLED, and set the holdingTimer according to the value of dot11MeshHoldingTimeout. The finite state machine shall transition to HOLDING state.

When a CLS_ACPT event occurs, the mesh STA shall clear the confirmTimer, send a Mesh Peering Close frame with reason code MESH-CLOSE-RCVD, and set the holdingTimer according to the value of dot11MeshHoldingTimeout. The finite state machine shall transition to HOLDING state.

When an OPN_ACPT event occurs, the mesh STA shall clear the confirmTimer and shall send the corresponding Mesh Peering Confirm frame to respond to the incoming Mesh Peering Open frame. The finite state machine shall transition to ESTAB state.

When an OPN_RJCT event occurs, the mesh STA shall clear the confirmTimer, send a Mesh Peering Close frame with reason code as specified by the OPN_RJCT event, and set the holdingTimer according to the value of dot11MeshHoldingTimeout. The finite state machine shall transition to HOLDING state.

When a CNF_RJCT event occurs, the mesh STA shall clear the confirmTimer, send a Mesh Peering Close frame with reason code as specified by the CNF_RJCT event, and set the holdingTimer according to the value of dot11MeshHoldingTimeout. The finite state machine shall transition to HOLDING state.

When TOC event occurs, the mesh STA shall send a Mesh Peering Close frame with reason code MESH-CONFIRM-TIMEOUT and set the holdingTimer according to the value of dot11MeshHoldingTimeout. The finite state machine shall transition to HOLDING state.

All other events shall be ignored in this state.

11C.3.3.8 OPN_RCVD state

In the OPN_RCVD state, the mesh STA has received a Mesh Peering Open frame and sent a Mesh Peering Open frame and the corresponding Mesh Peering Confirm frame. An incoming Mesh Peering Confirm is expected.

When a CNCL event occurs, the mesh STA shall clear the retryTimer, send a Mesh Peering Close frame with reason code MESH-PEERING-CANCELLED, and set the holdingTimer according to the value of dot11MeshHoldingTimeout. The finite state machine shall transition to HOLDING state.

When a CLS_ACPT event occurs, the mesh STA shall clear the retryTimer, send a Mesh Peering Close frame, and set the holdingTimer according to the value of dot11MeshHoldingTimeout. The finite state machine shall transition to HOLDING state.

When an OPN_ACPT event occurs, the mesh STA shall resend the corresponding Mesh Peering Confirm frame. The finite state machine shall stay in the OPN_RCVD state.

When an OPN_RJCT event occurs, the mesh STA shall clear the retryTimer, send a Mesh Peering Close frame with reason code as specified by the OPN_RJCT event, and set the holdingTimer according to the value of dot11MeshHoldingTimeout. The finite state machine shall transition to HOLDING state.

When a CNF_ACPT event occurs, the retryTimer shall be cleared. The finite state machine shall transition to ESTAB state.

When a CNF_RJCT event occurs, the mesh STA shall clear the retryTimer, send a Mesh Peering Close frame with reason code as specified by the CNF_RJCT event, and set the holdingTimer according to the value of dot11MeshHoldingTimeout. The finite state machine shall transition to HOLDING state.

When a TOR1 event occurs, the Mesh Peering Open frame shall be resent and the retryCounter shall be incremented. The retryTimer shall be set according to the updated retryTimeout computed by the backoff algorithm. The finite state machine shall stay in the OPN_RCVD state.

When a TOR2 event occurs, the mesh STA shall send a Mesh Peering Close frame with reason code MESH-MAX-RETRIES. The holdingTimer shall be set according to the value of dot11MeshHoldingTimeout, and the finite state machine shall transition to HOLDING state.

All other events shall be ignored in this state.

11C.3.3.9 ESTAB state

In the ESTAB state, mesh peering has been successfully established with the peer mesh STA.

When a CNCL event occurs, the mesh STA shall send a Mesh Peering Close frame with reason code MESH-PEERING-CANCELLED, and set the holdingTimer according to the value of dot11MeshHoldingTimeout. The finite state machine shall transition to HOLDING state.

When a CLS_ACPT event occurs, the mesh STA shall send a Mesh Peering Close frame with reason code MESH-CLOSE-RCVD, and set the holdingTimer according to the value of dot11MeshHoldingTimeout. The finite state machine shall transition to HOLDING state.

When an OPN_ACPT event occurs, the mesh STA shall respond by resending the corresponding Mesh Peering Confirm frame. The finite state machine shall stay in the ESTAB state.

All other events shall be ignored in this state.

11C.3.3.10 HOLDING state

In HOLDING state, the mesh STA is closing the mesh peering. The holdingTimer has been set according to the value of dot11MeshHoldingTimeOut.

When a CLS_ACPT event occurs, the holdingTimer shall be cleared. The finite state machine shall transition to IDLE state.

When any of the following four events occurs— OPN ACPT, CNF_ACPT, OPN_RJCT, CNF_RJCT, the mesh STA shall send the Mesh Peering Close frame. If the mesh TKSAs bound with the mesh peering instance, the Mesh Peering Close frame shall be MIC protected. See 11C.3.3.3 Events and Actions for composing a Mesh Peering Close frame. The finite state machine shall stay in the HOLDING state.

When a TOH event occurs, the finite state machine shall transition to IDLE state.

All other events are ignored in this state.

11C.4 Authenticated Mesh Peering Exchange (AMPE)

11C.4.1 Overview

The Authenticated Mesh Peering Exchange establishes an authenticated mesh peering between the mesh STAs, under the assumption that Mesh PMKSA has already been established before the initiation of the protocol via the active authentication protocol. An authenticated mesh peering includes a mesh peering, corresponding mesh TKSA, and the two mesh STAs mesh GTKSAs.

The Authenticated Mesh Peering Exchange uses Mesh Peering Management frames. Parameters are exchanged via RSN information elements and Authenticated Mesh Peering Exchange elements.

The major supported Authenticated Mesh Peering Exchange functions are Security Capability Selection and Key Management.

- The Security Capability Selection function (specified in 11C.4.2 Security Capabilities Selection) achieves the agreement on the security parameters used for the protocol instance, including the Mesh PMKSA, pairwise cipher suite, and group cipher suite.

- The Key Management function (specified in Error: Reference source not found) derives key encryption, key confirmation, and temporal keys for the authenticated mesh peering and distributes both mesh STAs’ GTKs and IGTKs to each other.

During the Authenticated Mesh Peering Exchange, the mesh STAs generate nonces and transmit them via mesh peering management action frames. The secure mesh peering instance is identified as

Identifier = <min(localMAC, peerMAC),

max(localMAC, peerMAC),

min(localNonce || localLinkID, peerNonce || peerLinkID),

max(localNonce || localLinkID, peerNonce || peerLinkID)>.

The mesh STA shall generate a random value for its localNonce, as specified in 8.5.7. The candidate peer mesh STA is expected to generate a random value for the peerNonce, which the mesh STA receives from the candidate peer mesh STA in Confirm and Close action frames.

11C.4.2 Security Capabilities Selection

11C.4.2.1 Instance Pairwise Cipher Suite Selection

Pairwise cipher suite selectors WEP-40, WEP-104, and TKIP shall not be used as the pairwise cipher suite when dot11MeshSecurityActivated is enabled.

If the pairwise cipher suite has not been selected, mesh STAs shall attempt to reach the agreement on the pairwise cipher suite using the following procedure in four phases:

- The mesh STA shall announce the list of pairwise cipher suites it supports using an ordered list in the RSN information element in the Mesh Peering Open frame. The first value in the list is the most preferred cipher suite by the mesh STA, and last value the least preferred.

- If the mesh STA receives a Mesh Peering Open frame from the candidate peer mesh STA, the mesh STA shall independently make decision on the selected pairwise cipher suite based on intersection of its own ordered list and the received ordered list.

- If the intersection is empty, the pairwise cipher suite selection fails and failure reason code MESH-INVALID-SECURITY-CAPABILITY shall be generated and corresponding actions shall be taken according to 11C.4.6 Authenticated Mesh Peering Exchange Finite State Machine

- If the intersection is not empty and contains more than one value, the selected cipher suite shall be the entry in the intersection list most preferred by the mesh STA with the largest MAC address in the lexicographic ordering.

- If the mesh STA receives a Mesh Peering Confirm frame from the candidate peer mesh STA before receiving a Mesh Peering Open frame, the mesh STA shall verify that the mesh STA supports the chosen pairwise cipher suite by the candidate peer mesh STA. Otherwise, the selection fails and the failure reason code MESH-INVALID-SECURITY-CAPABILITY shall be generated. 
Furthermore, once receiving a Mesh Peering Open frame, the mesh STA shall verify that the accepted selected pairwise cipher suite matches the chosen pairwise cipher suites as the result of phase 2). If they do not match, the selection fails and the failure reason code MESH-INVALID-SECURITY-CAPABILITY shall be generated. Otherwise, the pairwise cipher suite selection succeeds, and the mesh STA shall proceed to phase 4).

- Upon the successful pairwise cipher suite selection, if generating the Mesh Peering Confirm frame, the mesh STA shall set the Selected Pairwise Cipher Suite to the cipher suite selector of the selected pairwise cipher suite.

11C.4.2.2 Group Cipher Suite Selection

Group cipher suite selectors WEP-40, WEP-104, and TKIP shall not be used as the group cipher suite when dot11MeshSecurityActivated is true.

The mesh STAs shall announce the group cipher suite used for its own broadcast protection in the Mesh Peering Open action frame. The mesh STA shall verify whether it supports the group cipher suite announced by the candidate peer mesh STA in the Mesh Peering Open frame. If the cipher suite is supported, the selection succeeds. Furthermore the group cipher suite received in the Mesh Peering Confirm frame shall be supported by the mesh STA as well. If the group cipher suite selection fails, the reason code MESH-INVALID-SECURITY-CAPABILITY shall be generated.

11C.4.3 GTK Distribution

The mesh STA shall distribute the GTK to the peer mesh STA using the Mesh Peering Open frame during the Authenticated Mesh Peering Exchange. Upon successful completion of AMPE, each mesh STA shall establish states for the peer mesh STA’s mesh GTKSA. The GTKData subfield in Authenticated Mesh Peering Exchange element shall contain the encrypted key data. The deterministic authenticated encryption mode of AES-SIV, defined in IETF RFC 5297, shall be used to protect the GTK field using the AKEK derived from the chosen PMK. The data to be protected shall be the bit string: {GTK, peerMAC, Key RSC, GTKExpirationTime}.The GTK shall be passed to SIV as the plaintext and the peerMAC, KeyRSC, and GTKExpiration Time shall be separate, distinct, components of associated data (AD).

When unwrapping the GTK, the mesh STA shall use algorithm as defined in IETF RFC 3394 with the default initial value and the derived AKEK from the selected PMK. The mesh STA shall verify that the second element in the resulting string matches the receiver's MAC address, peerMAC, as sent in the Mesh Peering Open frame. The mesh STA shall extract the GTK value, the Key RSC, and GTK life time by removing the bits of the peerMAC from the resulting bit string of the key unwrapping operation.

If the key unwrapping operation fails or the concatenated values do not match, the GTK distribution fails, the mesh STA shall terminate the protocol instance and shall report the reason code “MESH-INVALID-GTK”.

11C.4.4 Constructing and verifying MIC

AKCK derived from the chosen PMK for the authenticated mesh peering exchange instance shall be used to compute and verify the MIC value. AES-128 in CMAC Mode shall be used for MIC computation. NIST SP 800-38B defines the CMAC algorithm. Authenticated Mesh Peering Exchange uses AES with a 128-bit integrity key and a 128-bit block size, and a CMAC TLen value of 128 (16 octets). The CMAC output is truncated to 64 bits:

MIC = L(CMAC Output, 0, 64)								 (1)

Where L is defined in 8.5.1.

When constructing the MIC information element (MIE) for the Mesh Peering Management frame, the mesh STA shall

- Construct the MIE (see Error: Reference source not found) with the MIC field masked to zero.

- Compute AES-128-CMAC over the concatenation of (localMAC || peerMAC || Mesh Peering Management frame body including MIE), and insert the 64-bit output into the MIE MIC field.

When verifying the MIC on the received Mesh Peering Management frame, the mesh STA shall verify the received PMKID from the Chosen PMK element matches the chosen PMK for the authenticated mesh peering exchange instance by the mesh STA. The mesh STA then shall

- Extract and save the received MIC value.

- Compute the AES-128-CMAC over the concatenation of (peerMAC || localMAC || Mesh Peering Management frame body including MIC) with the MIC field masked to zero in the MIE.

- If the result does not match the received value, the MIC verification fails.

11C.4.5 Mesh Peering Management frames for Authenticated Mesh Peering Exchange

11C.4.5.1 General

The Mesh Peering Management frames shall be generated with additional information using RSN information element and Authenticated Mesh Peering Exchange information element to support Authenticated Mesh Peering Exchange. Upon receiving a Mesh Peering Management frame when Authenticated Mesh Peering Exchange is enabled, the mesh STA shall perform Authenticated Mesh Peering Exchange specific processing operations followed by the basic Mesh Peering Management protocol frame processing as specified in 11C.3.3 Mesh Peering Management Finite State Machine.

11C.4.5.2 Mesh Peering Open frame contents for Authenticated Mesh Peering Exchange

A mesh STA initiates the establishment of a mesh peering and Mesh TKSA with a candidate peer mesh STA by sending a Mesh Peering Open frame to the candidate peer mesh STA. In addition to contents for establishing a mesh peering as specified in 11C.3.2.2.1 Mesh Peering Open frame contents, the Mesh Peering Open frame shall contain the following:

- In the Mesh Peering Management information element, the Mesh Peering Protocol Identifier shall be set to 00-0F-AC:1 “Authenticated Mesh Peering Exchange Protocol”.

- In the RSN information element

- The Group Cipher Suite field shall be set to the cipher suite selector that the mesh STA uses for protecting its group addressed traffic.

- The Pairwise Cipher Suite Count field shall be set to the number of cipher suite selectors in the Pairwise Cipher Suite list field.

- The Pairwise Cipher Suite List field shall contain a series of cipher suite selectors that indicate the pairwise cipher suites that the mesh STA could support for protecting individually addressed traffic. The list is ordered by the priority decided by the mesh STA (see 11C.4.2 Security Capabilities Selection).

- In the Authenticated Mesh Peering Exchange element

- The Selected Pairwise Cipher Suite field shall be set to the first cipher suite selector in the Pairwise Cipher Suite List field in RSN information element.

- The Chosen PMK field shall be set to PMKName that identifies the Mesh PMKSA the mesh STA established with the candidate peer mesh STA.

- The Local Nonce field shall be set to the localNonce value generated by the mesh STA for identifying the current mesh peering instance.

- The Peer Nonce field shall be set to 0.

- The GTKdata field shall be present and shall contain the KDE data for the mesh STA’s GTK. The GTK wrapping is specified in 11C.4.3 GTK Distribution.

- The MIC information element shall be constructed and set according to 11C.4.4 Constructing and verifying MIC.

11C.4.5.3 Processing Mesh Peering Open frames for Authenticated Mesh Peering Exchange

On receiving a Mesh Peering Open frame, the mesh STA shall verify the MIC on the received frame according to 11C.4.4 Constructing and verifying MIC. The received frame shall be discarded if the MIC verification fails.

The received frame shall be rejected if the security capability selection fails (see 11C.4.2 Security Capabilities Selection). The OPN_RJCT event shall be invoked to the corresponding Authenticated Mesh Peering Exchange finite state machine.

If none of the failures occur and the candidate peer mesh STA’s GTK has not been unwrapped, the mesh STA may proceed to perform key unwrapping operation to extract the peer mesh STA’s GTK value, as specified in 11C.4.3 GTK Distribution. If this operation fails, the mesh peering instance shall be aborted. The OPN_RJCT event shall be invoked to the corresponding Authenticated Mesh Peering Exchange finite state machine and the reason code “MESH-INVALID-GTK” is generated.

If all operations succeeds, the mesh STA shall proceed to process the Mesh Peering Open frame on basic parameters as specified in 11C.3.2.1 Overview and 11C.3.2.2 Mesh Peering Open frames.

11C.4.5.4 Mesh Peering Confirm frame content for Authenticated Mesh Peering Exchange

When sending a Mesh Peering Confirm frame in response to a Mesh Peering Open frame, the constructed Mesh Peering Confirm frame shall contain the following in addition to contents for establishing a mesh peering as specified in 11C.3.2.3.1 Mesh Peering Confirm frame contents.

- In the Mesh Peering Management information element, the Mesh Peering Protocol Identifier shall be set to 00-0F-AC:1 “Authenticated Mesh Peering Exchange Protocol”.

- The RSN information element shall be set to the same values as sent in the Mesh Peering Open frame.

- In the Authenticated Mesh Peering Exchange element

- The Selected Pairwise Cipher Suite field shall be set to the cipher suite selector that indicates the successfully selected pairwise cipher suite (specified in 11C.4.2.1 Instance Pairwise Cipher Suite Selection).

- The Peer Nonce field shall be set to the nonce value chosen by the peer mesh STA as received in the Local Nonce field in the Mesh Peering Open frame from the candidate peer mesh STA.

- The GTKdata field shall not be present.

- The rest of fields are set to the same values sent in the Mesh Peering Open frame.

- The MIC information element shall be constructed and set according to 11C.4.4 Constructing and verifying MIC.

11C.4.5.5 Processing Mesh Peering Confirm action frames

On receiving a Mesh Peering Confirm frame, the mesh STA shall verify the MIC on the received frame according to 11C.4.4 Constructing and verifying MIC. The received frame shall be discarded if the MIC verification fails.

If the MIC verification succeeds, the following operations shall be performed in order:

- The Chosen Pairwise Cipher Suite is checked. If the security capability selection has been done and the received value from Chosen Pairwise Cipher Suite field is not the same as the agreed pairwise cipher suite, the mesh STA shall reject the received frame and the CNF_RJCT event is invoked to the corresponding Authenticated Mesh Peering Exchange finite state machine with the failure reason code MESH-INVALID-SECURITY-CAPABILITY.

- The Group Cipher Suite is checked. If the received group cipher suite is not supported by the mesh STA, the mesh STA shall reject the received Mesh Peering Confirm frame and the CNF_RJCT event is invoked to the corresponding Authenticated Mesh Peering Exchange finite state machine with the failure reason code MESH-INVALID-SECURITY-CAPABILITY.

- If none of them is true and the candidate peer mesh STA’s GTK has not been unwrapped, the mesh STA may proceed to perform key unwrapping operation to extract the peer mesh STA’s GTK value, as specified in 11C.4.3 GTK Distribution. If this operation fails, the mesh peering instance shall be aborted. The OPN_RJCT event shall be invoked to the corresponding Authenticated Mesh Peering Exchange finite state machine and the reason code MESH-INVALID-GTK is generated.

If none of the cases is true, the mesh STA shall proceed to process the Mesh Peering Confirm action frame on basic parameters as specified in 11C.3.2.1 Overview and 11C.3.2.3 Mesh Peering Confirm frames.

11C.4.5.6 Mesh Peering Close frame content for Authenticated Mesh Peering Exchange

When sending a Mesh Peering Close frame, the mesh STA shall generate additional information for Authenticated Mesh Peering Exchange, specified as following:

- In the Mesh Peering Management information element, the Mesh Peering Protocol Identifier shall be set to 00-0F-AC:1 “Authenticated Mesh Peering Exchange Protocol”.

- In the Authenticated Mesh Peering Exchange element

- The Selected Pairwise Cipher Suite field shall be set to the same value as sent in the Mesh Peering Open frame.

NOTE—If the reason for sending the Mesh Peering Close is the pairwise cipher suite selection failure, the information in this field is used to inform the candidate peer mesh STA what was announced by the mesh STA for the mesh peering instance.

- The Chosen PMK field shall be set to the same value as sent in the Mesh Peering Open frame.

- The Local Nonce field shall be set to the same value as sent in the Mesh Peering Open frame.

- The Peer Nonce field shall be set to the same value as received in the Local Nonce field of the Authenticated Mesh Peering Exchange of the incoming Mesh Peering Management frame from the candidate peer mesh STA.

- The MIC information element shall be constructed and set according to 11C.4.4 Constructing and verifying MIC.

11C.4.5.7 Processing Mesh Peering Close action frames

On receiving a Mesh Peering Close frame, the mesh STA shall verify the MIC on the received frame according to 11C.4.4 Constructing and verifying MIC. The received frame shall be discarded if the MIC verification fails.

If the MIC verification succeeds, the mesh STA shall proceed to process the Mesh Peering Close frame on basic parameters as specified in 11C.3.2.1 Overview and 11C.3.2.4 Mesh Peering Close frames.

11C.4.6 Authenticated Mesh Peering Exchange Finite State Machine

11C.4.6.1 Overview

The finite state machine for Authenticated Mesh Peering Exchange supports all the states, events, and actions defined for the finite state machine for the Mesh Peering Management protocol. New events, actions, and state transitions are added to specify the security functions for Authenticated Mesh Peering Exchange.

When a finite state machine is generated and activated for an Authenticated Mesh Peering Exchange instance, the localNonce shall be generated and used together with a new localLinkID to identify the instance.

11C.4.6.2 Additional Events and Actions to Mesh Peering Management FSM

All events for rejecting or ignoring received action frames shall report the corresponding reason code related to Authenticated Mesh Peering Exchange functions as described in 11C.4.5 Mesh Peering.

In addition, there is one new event.

TOR3 – This event refers to Timeout(localLinkID, retryTimer), the dot11MeshMaxRetries has been reached, with the Authenticated Mesh Peering Exchange enabled, but the mesh STA fails to confirm the selection of the shared mesh PMKSA. When this event triggers, the protocol instance shall be closed, but no Mesh Peering Close frame shall be sent.

The actions of sending mesh peering management frames are updated as the following.

sndOPN — Generate a Mesh Peering Open frame for the current Authenticated Mesh Peering Exchange protocol instance (as specified in 11C.4.5.2 Mesh Peering Open frame contents for Authenticated Mesh Peering Exchange) and send it to the candidate peer mesh STA.

sndCNF — Generate a Mesh Peering Confirm frame for the current Authenticated Mesh Peering Exchange protocol instance (as specified in 11C.4.5.4 Mesh Peering Confirm frame content for Authenticated Mesh Peering Exchange) and send it to the candidate peer mesh STA.

sndClose — Generate a Mesh Peering Close frame for the current Authenticated Mesh Peering Exchange protocol instance (as specified in 11C.4.5.6 Mesh Peering Close frame content for Authenticated Mesh Peering Exchange) and send it to the candidate peer mesh STA.

11C.4.6.3 State transitions

All state transitions specified in Mesh Peering Management finite state machine shall be used for Authenticated Mesh Peering Exchange finite state machine.

In OPN_SNT state, the following are additional state transitions and actions

When TOR3 event occurs, the retryTimer shall be cleared and the holdingTimer shall be set. The finite state machine shall transition to HOLDING state.

In OPN_RCVD state, the following are the additional actions

When CNF_ACPT event occurs, in addition to the actions for Mesh Peering Management protocol, the mesh STA shall signal the completion of key management by utilizing the MLME-SETKEYS.request primitive to configure the agreed-upon mesh temporal pairwise key into the IEEE 802.11 MAC and by calling the MLME-SETPROTECTION.request primitive to enable its use.

In CNF_RCVD state, the following are the additional actions

When OPN_ACPT event occurs, in addition to the actions for Mesh Peering Management protocol, the mesh STA shall signal the completion of key management by utilizing the MLME-SETKEYS.request primitive to configure the agreed-upon mesh temporal pairwise key into the IEEE 802.11 MAC and by calling the MLME-SETPROTECTION.request primitive to enable its use.

Authenticated Mesh Peering Exchange Finite State Machine and  specify the state transitions of the finite state machine for Authenticated Mesh Peering Exchange.

| - Authenticated Mesh Peering Exchange Finite State Machine |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | To State |  |  |  |  |  |
|  |  | IDLE | OPN_SNT | CNF_RCVD | OPN_RCVD | ESTAB | HOLDING |
| From State | IDLE | REQ_RJCT / sndCLS | ACTOPN/ (sndOPN, setR) |  | OPN_ACPT/ (sndOPN, sndCNF, setR) |  |  |
|  | OPN_SNT |  | TOR1/ (sndOPN, setR) | CNF_ACPT/ (clR, setC) | OPN_ACPT/ (sndCNF) |  | CLS_ACPT, OPN_RJCT, CNF_RJCT, TOR2, CNCL/ (sndCLS, clR, setH) TOR3 / (clR, setH) |
|  | CNF_RCVD |  |  |  |  | OPN_ACPT / (clC, sndCNF) | CLS_ACPT, OPN_RJCT, CNF_RJCT, CNCL/ (sndCLS, clC, setH) TOC / (sndCLS, setH) |
|  | OPN_RCVD |  |  |  | TOR1 / (sndOPN, setR) OPN_ACPT / sndCNF | CNF_ACPT / clR | CLS_ACPT, OPN_RJCT, CNF_RJCT,TOR2, CNCL/ (sndCLS, clR, setH) |
|  | ESTAB |  |  |  |  | OPN_ACPT / sndCNF | CLS_ACPT, OPN_RJCT, CNF_RJCT,CNCL/ (sndCLS, setH) |
|  | HOLDING | TOH/—, CLS_ACPT / clH- |  |  |  |  | OPN_ACPT, CNF_ACPT, OPN_RJCT, CNF_RJCT / sndCLS |

- Finite State Machine of Authenticated Mesh Peering Exchange Protocol

11C.5 Mesh Group Key Handshake

The Mesh Group Key Handshake may be used by either mesh STA, after a secure mesh peering has been established, to update the GTK that it uses to protect group addressed MPDUs that it transmits. The Mesh Group Key Handshake is similar to the Group Key Handshake defined in 8.5.4, but with minor updates required for use within a mesh.

The EAPOL-Key frame notation is defined in 8.5.2.1. The “GTK Source” is the mesh STA that is sending the GTK to a peer mesh STA using this protocol. A “GTK Recipient” is a mesh STA receiving the GTK being sent by the GTK Source.

Message 1: GTK SourceGTK Recipient: EAPOL-Key(1,1,1,0,G,0,Key RSC,0,MIC,DataKD_GM1) where DataKD_GM1 = (Mesh GTK Delivery KDE, GTK KDE).

Message 2: GTK RecipientGTK Source: EAPOL-Key(1,1,0,0,G,0,0,0,MIC,DataKD_GM2) where DataKD_GM2 = Mesh GTK Delivery KDE.

The contents of each message shall be as described in 8.5.4, except as follows:

- Message 1: The Key Data field shall contain a Mesh GTK Delivery KDE (see 11C.3.3 Mesh Peering Management Finite State Machine), where the Sender mesh STA Address field is set to the MAC address of the GTK Source, and the Destination mesh STA Address field is set to the MAC address of the GTK Recipient. The Key Data field shall also contain the GTK KDE containing the GTK to be sent. The entire Key Data field shall be encrypted. The Key Data Length field shall indicate the length of the Key Data field after encryption, including any padding.

- Message 2: The Key Data field shall contain a Mesh GTK Delivery KDE (see 11C.3.3 Mesh Peering Management Finite State Machine), where the Sender mesh STA Address field is set to the MAC address of the GTK Recipient, and the Destination mesh STA address field is set to the MAC address of the GTK Source. The Key Data field shall not be encrypted. The Key Data Length field shall indicate the length of the Mesh GTK Delivery KDE.

The processing, upon reception, of Message 1 of the Mesh Group Key Handshake shall be as described in 8.5.4.1. Additionally, after verifying the MIC, the recipient of Message 1 shall verify that the Sender and Destination mesh STA address fields in the Mesh GTK Delivery KDE contain the MAC address of the mesh STA sending the GTK and the local mesh STA's MAC address, respectively. If these addresses are not correctly included, the message shall be silently discarded.

The processing, upon reception, of Message 2 of the Mesh Group Key Handshake shall be as described in 8.5.4.2. Additionally, after verifying the MIC, the recipient of Message 2 shall verify that the Sender and Destination mesh STA address fields in the Mesh GTK Delivery KDE contain the MAC address of the GTK Recipient and the local mesh STA's MAC address, respectively. If these addresses are not correctly included, the message shall be silently discarded.