IEEE P802.11
Wireless LANs

| TGp SB Speculative Resolutions for Editorial Comments |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-11-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Francois Simon | US DOT | ARINC, Inc. 2551 Riva Road Annapolis, MD 21401 | 410-266-4531 | fsimon@arinc.com |

Abstract

This document addresses comments received for TGp Letter Ballot 154. These resolutions are provided for consideration for comments that may be received in TGp’s initial Sponsor Ballot.  The comment numbers were from LB 154; however, these numbers no longer apply.

- COMMENT:  [From Spreadsheet]  INSERT Original Comment Here:

| ID | Commenter | Clause | Pg | Ln | Type | Comment | Suggested Remedy | Recommended Resolution |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | Roebuck, Randy | 5.3.1 | 3 | 32 | E | Data confidentiality (not used when dot11OCBEnabled is true) is not discussed (unlike a or b) in the requirements section or listed as PIC. | Add "Data Confidentiality" explaination in the appropiate section. | Accepted in principle – “Data Confidentiality” provides the security of a WLAN up to the level of security implicit in wired LAN. In the P802.11p amendment, security services are outside the scope of 802.11 and are performed in layers above the MAC sublayer. See proposed text change below. |
| 3 | Malarky, Alastair | 7.1.3.3.3 | 4 | 32 | E | The order of the sentences in this last paragraph of 7.1.3.3.3 is a bit awkward. It seems incorrect to specify a specific use of wildcard BSSID and then make the general statement using the word "elsewhere" which excludes the previous sentence. | recommend changing last paragraph to read "The value of all 1s is used to indicate the wildcard BSSID. The wildcard value shall not be used in the BSSID field except where explicitly permitted in this standard. When dot11OCBEnabled is true, the wildcard value shall be used in the BSSID field." | Accepted per Suggested Remedy. |
| 4 | Roebuck, Randy | 7.1.3.5.1 | 4 | 41 | E | PIC needs to be added for "No tracffic streams when dot11OCBEnabled is true" | Change from "..., traffic streams are not used …" to "…., traffic streams shall not be used …". Add PIC QP8. | Denied – It has been understood that there should not be any “shall” in Clause 7. The 802.11 WG is currently working on removing them from the current text. There seems to be no requirement to change the text and add a PICS. |
| 5 | Malarky, Alastair | 7.2.2 | 5 | 10 | E | The Address field contains, not carries, the value (see previous sentence in paragraph). | Replace "or that it carries" with "or that it contains" | Accepted per Suggested Remedy. |
| 7 | Roebuck, Randy | 7.2.3.14 | 5 | 48 | E | Time Advertisement element in table 7-18a is different order than 10.3.51.1.2 service primitive. | Frame format needs to have consisent ordering with primitives | Counter – Table 7-18a identifies the IEs that may be included in the Timing Advertisement frame and their proper order. That frame is to be transmitted on the “air interface”. On the other hand, 10.3.51.1.2 specifies the parameter passed between the SME and the LME. This abstract operation is internal to the STA and it is implementation dependent. The parameters are only loosly associated with the frame’s IEs and ordering is not required. It is the responsibility of the implementation to “translate” the parameters to IE content. It is proposed to make no change to the text. |
| 8 | Roebuck, Randy | 7.2.3.14 | 5 | 49 | E | Table 7-18a does not include "RCPI & Source MAC Address" in its format as shown in the receiving indication. | Add "RCPI & Source MAC Address" line between Power constraint and Extended Capabilities. | Denied – First, RCPI and Source MAC Address are NOT IEs; they are parameters and cannot be included in the Timing Advertisement frame. Second, these parameters are related to the receiver; not transmitter. Parameters are forwarded From the receiver MLME to SME via MLME-TIMING_ ADVERTISEMENT. Indication (see 10.3.51.3.2) when a Timing Advertisement frame is RECEIVED. |
| 9 | Malarky, Alastair | 7.3.2 | 6 | 40 | E | "Ids" is incorrect in Table 7-26 title. It should be "IDs". | As per comment | Accepted per Suggested Remedy. |
| 12 | Ecclesine, Peter | 7.3.2.26 | 7 | 4 | ER | After making the change, the text has an extra space befor "j": "The length of the vendor specific content is n- j octets" | Fix text | Accepted per Suggested Remedy. |
| 15 | Chaplin, Clint | 9.9.1.2 | 10 | 25 | E | "Insert the following text at the end the second paragraph of 9.9.1.2 after NOTE 3:" missing "of" | "Insert the following text at the end of the second paragraph of 9.9.1.2 after NOTE 3:" | Accepted per Suggested Remedy. |
| 16 | Roebuck, Randy | 10.3.51.1.2 | 14 | 44 | E | Time Advertisement element comes in different order than shown in Table 7-18a. | Primitive needs to be consisent with the frame format. | Counter – This comment is related to CID# 7. The same Explanation applies for CID# 16. |
| 17 | Roebuck, Randy | 10.3.51.3.2 | 16 | 37 | E | Time Advertisement element comes in different order than shown in Table 7-18a. | Primitive needs to be consisent with the frame format. | Counter – This comment is related to CID# 7. The same Explanation applies for CID# 17. |
| 22 | Chaplin, Clint | 11.20 | 18 | 18 | E | "subtype Action and Timing Advertisement" | "subtypes Action and Timing Advertisement" | Accepted per Suggested Remedy. |
| 28 | Roebuck, Randy | A.4.4.1 | 21 | 34 | E | Add "No association" to PIC PC1.1a and modify references that do not exist any more. | Change title to "No Authenication or Association State". Delete 5.7.6, 5.7.7 & 8.1 references and replace with 5.2.10, 5.3.1 & 11.3. | Accepted as per Suggested Remedy |
| 31 | Roebuck, Randy | A.4.4.1 | 21 | 44 | T | Add PIC PC11.13 for "No Synchronization required for Common Clock". | Add "No Synchronization required for Common Clock", Reference 11.1, Status CF18:O as PIC PC11.13 | Accepted as per Suggested Remedy |
| 32 | Roebuck, Randy | A4.4.4 | 22 | 20 | E | Add section 7.2 to references for Wildcard BSSID AD4 PIC. | Add "7.2" to the references. | Accepted as per Suggested Remedy |
| 34 | Roebuck, Randy | A4.15 | 23 | 34 | E | Add section 9.9.1.2 to references for PIC QD5 (default EDCA). | Add "9.9.1.2" to the references. | Accepted as per Suggested Remedy |

- Background

This submission proposes the resolutions for Editorial Comments”

- Recommended Resolution of the Comments:

Proposed change of text in green and deletion in yellow

CID# 2

5.2.10 STA transmission of data frames outside the context of a BSS

Change the text in the second paragraph of 5.2.10 as follows:

When dot11OCBEnabled is true a data frame can be sent to either an individual or a group destination MAC address. This type of communication is only possible between STAs that are able to communicate directly over the wireless medium. It allows immediate communication, avoiding the latency associated with establishing a BSS. When dot11OCBEnabled is true a STA is not a member of a BSS and it does not utilize the 802.11 authentication, or association, or data confidentiality services. This capability is These capabilities are particularly well-suited for use in rapidly varying communication environments such as those involving mobile STAs where the interval over which the communication exchanges take place may be of very short-duration (e.g., on the order of tens or hundreds of milliseconds). Since 802.11 MAC sublayer authentication services are not used when dot11OCBEnabled is true, any required authentication services would be provided by the station management entity (SME) or by applications outside of the MAC sublayer. STAs that do not have the MIB variable dot11OCBEnabled defined operate as if dot11OCBEnabled is false.

CID# 3

7.1.3.3.3 BSSID field

Change the last paragraph of 7.1.3.3.3 to:

The value of all 1s is used to indicate the wildcard BSSID. A wildcard BSSID The wildcard value shall not be used in the BSSID field except where explicitely permitted in this standard. When dot11OCBEnabled is true, the wildcard value shall be used in the BSSID field. The wildcard value shall not be used in the BSSID field except for management frames of subtype probe request and of subtype Action with Category Public. where explicitly permitted elsewhere in this standard.

CID# 5

7.2.2 Data frames

Change the statements immediately following Table 7-7 as shown:

A STA uses the contents of the Address 1 field to perform address matching for receive decisions. In cases where the Address 1 field contains a group address, the BSSID also is validated to ensure either that the broadcast or multicast originated from a STA in the BSS of which the receiving STA is a member, or that it carries or that it contains the wildcard BSSID value, indicating a data frame sent outside the context of a BSS (dot11OCBEnabled is true in the transmitting STA).

CID# 9

7.3.2 Information elements

Change Table 7-26 caption as follows:

Table 7-26—Element Ids IDs

CID# 12

7.3.2.26 Vendor Specific information element

Change 7.3.2.26 as follows:

The Vendor Specific information element is used to carry information not defined in this standard within a single defined format, so that reserved information element IDs are not usurped for nonstandard purposes and so that interoperability is more easily achieved in the presence of nonstandard information. The information element is in the format shown in Figure 7-75 and requires that the first 3 or more octets of the information field contain the OUI of identify the entity that has defined the content of the particular Vendor Specific information element. The length of the information field (n) is constrained by 3 length Organization Identifier) < n < 255. The OUI field shall be a public OUI assigned by the IEEE. It is 3 octets in length.  The length of the vendor specific content is n-3 j n-3j octets, where j is the length of the Organization Identifier field as described in 7.3.1.31.

CID# 15

9.9.1.2 EDCA TXOPs

Insert the following text at the end of the second paragraph of 9.9.1.2 after NOTE 3:

CID# 22

11.20 STAs communicating data frames outside the context of a BSS

Change bullet a) as follows:

a)  Synchronization, authentication, and association as defined in Clause 11.1 and Clause 11.3 are not used. The STA may send management frames of subtypes Action and Timing Advertisement.

CID# 28, CID#31

A.4.4.1 MAC protocol capabilities

Insert these entries in A.4.4.1:

| Item | Protocol capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| PC1.1a | No Authentication state or Association | 5.7.6, 5.7.7, 8.1 5.2.10, 5.3.1, 11.3 | CF2.1:M | Yes []No[] N/A [] |
| PC11.7.1 | No Passive scanning | 11.20 | CF2.1:M | Yes []No[] N/A [] |
| PC11.8.1 | No Active scanning | 11.20 | CF2.1:M | Yes []No[] N/A [] |
| PC11.11 | Time Advertisement generation | 11.21 | CF2.1:M | Yes []No[] N/A [] |
| PC11.12 | Incrementing and maintaining of TSF timer | 11.21 | CF18:M | Yes []No[] N/A [] |
| PC11.13 | No Synchronization required for Common Clock | 11.1 | CF18:O | Yes []No[] N/A [] |
| PC37 | Independent station operating outside the context of a BSS | 11.20 | O | Yes []No[] |
| PC37.1 | No STA communication outside BSS if dot11OCBEnabled is false | 11.20 | CF18:M | Yes []No[] N/A [] |
| PC37.2 | Only one EDCA parameter set used on current channel outside of BSS | 11.20 | CF18:M | Yes []No[] N/A [] |
| PC37.3 | Dot11OCBEnabled is false when STA is a BSS member | 11.20 | CF18:M | Yes []No[] N/A [] |

CID# 32

A.4.4.4 MAC addressing function

Insert a new row at the end of table A.4.4.4:

| Item | MAC addressing function | References | Status | Support |
| --- | --- | --- | --- | --- |
| AD4 | Wildcard BSSID | 7.1.3.3.3, 7.2 | CF18:M | Yes []No[] N/A [] |
| AD5 | MAC & PHY operation resumes with appropriate MIB attributes in less than 2 TU | 11.20 | CF18:M | Yes []No[] N/A [] |

CID# 34

A.4.15 QoS enhanced distributed channel access (EDCA)

Insert the following at the end of A.4.15:

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| QD8 | Default EDCA parameters for communications outside context of BBS | 7.3.2.29, 9.9.1.2 | CF18:M | Yes []No[] N/A [] |

- Motion (if technical and/or significant):

(And instructions to the editor.)

Move to accept the Recommended Resolutions to these comments and the recommended changes to P802.11p  noted above and instruct the editor to make these changes to P802.11p.

- Motion by: ___Francois Y. Simon _________________Date: ___________

- Second:  ______________________

| Approve: | Disapprove: | Abstain: |
| --- | --- | --- |

- References: