IEEE P802.11
Wireless LANs

| TDLS Peer Discovery |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-11-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Michael Montemurro | Research in Motion | 4701 Tahoe Blvd, Mississauga, ON. L4W 0B4 | +1-905-629-4746 | mmontemurro@rim.com |
| Daniel R. Borges | Apple, Inc. | 1 Infinite Loop MS 306-2HN, Cupertino, CA 95014 | +1 (415) 425-7347 | drborges@apple.com |
| Leo Estevez | Texas Instruments, Inc. | 12500 TI Blvd. MS 8679 Dallas, TX 75243 | 214-567-2622 | leonardo@ti.com |
| Menzo Wentink | Qualcomm | Breukelen, the Netherlands | +31-65-183-6231 | mwentink@qualcomm.com |

### Overview

This submission proposes a mechanism for TDLS peer discovery that is capable of leveraging layer 2 or layer 3 mechanisms.

7. Frame formats

7.3.2 Information Elements

In Table 7-26, insert two new information elements, and renumber the reserved values accordingly:

| Table 7-26—Element IDs |  |  |  |
| --- | --- | --- | --- |
| Information Element | Element ID | Length (in octets) | Extensible |
| BSSID (see 7.3.2.65a) | <ANA> | 14 | Yes |
| TDLS Discovery (see 7.3.2.65b) | <ANA> | 3 to 256 | Yes |

Insert the following new subclause after 7.3.2.65

#### 7.3.2.65a BSSID

The BSSID element contains information that identifies the BSSID of the STA sending the BSSID element. The element information format is defined in Figure 7-95o34.

|  | Element ID | Length | BSSID | STA MAC Address |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 6 | 6 |

Figure 7-95o34—BSSID element format

The Length field is set to 12.

The BSSID field is set to the BSSID of the BSS to which the STA is currently associated.

The STA MAC Address field is set to the STA’s MAC address.

Insert the following new subclause after 7.3.2.65.

7.3.2.65b TDLS Discovery

The TDLS Discovery element contains information that may be used by STAs to discover TDLS capabilities of a peer STA. This element would be used in a native GAS query (see Clause 11.23.2).

|  | Element ID | Length | OUI | TDLS Discovery Information |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 3 | 0-250 |

Figure 7-95o33- TDLS Discovery

The Length field is set to 12.

The OUI field indicates the type of TDLS discovery information.

The TDLS Discovery Information contains capability information that a STA may use to establish a TDLS link. An example of the peer discovery information is described in Annex U.1a.

Insert new TDLS frames in 7.4.11 as follows

### 7.4.11 TDLS Action frame details

Several Action frame formats are defined to support tunneled direct link setup (TDLS). The Action field values associated with each frame format within the TDLS category are defined in Table 7-57v1.

Table 7-57v1—TDLS Action field values

| Action field value | Meaning |
| --- | --- |
| 0 | TDLS Setup Request |
| 1 | TDLS Setup Response |
| 2 | TDLS Setup Confirm |
| 3 | TDLS Teardown |
| 4 | TDLS Peer Traffic Indication |
| 5 | TDLS Channel Switch Request |
| 6 | TDLS Channel Switch Response |
| 7 | TDLS Peer PSM Request |
| 8 | TDLS Peer PSM Response |
| 9 | TDLS Peer Traffic Response |
| 10 | TDLS Discovery Request |
| 11 | TDLS Discovery Response |
| 10 – 255 | Reserved |

Insert the following subclauses following 7.4.11.10

### 7.4.11.11 TDLS Discovery Request frame format

The TDLS Discovery Request frame contains the information shown in Table 7-57v12.

Table 7-57v12—Information for TDLS Discovery Request frame

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Category | The Category field is set to the value for TDLS, as defined in Table 7-24. |
| 2 | Action | The Action field is set to 10, representing TDLS Discovery Request. |
| 3 | Dialog Token | The Dialog Token can be used to match TDLS Disocvery Response frames to the corresponding TDLS Discovery Request frame. The Dialog Token is specified in 7.3.1.12. |
| 4 | BSSID | The BSSID element is specified in 7.3.2.65a. |
| 5 | TDLS Discovery | Optional. The TDLS Discovery element is specified in 7.3.2.65b. |

The TDLS Discovery Request frame is encapsulated in a Data frame and transmitted to a TDLS peer STA or to the Broadcast address through the AP, to request TDLS capable STAs in the same BSS to respond with a TDLS Discovery Response frame. See 11.19.

### 7.4.11.12 TDLS Discovery Response frame format

The TDLS Discovery Response frame contains the information shown in Table 7-57v13.

Table 7-57v13—Information for TDLS Discovery Response frame

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Category | The Category field is set to the value for TDLS, as defined in Table 7-24. |
| 2 | Action | The Action field is set to 11, representing TDLS Discovery Response. |
| 3 | Dialog Token | The Dialog Token is copied from the corresponding TDLS Discovery Request frame. The Dialog Token is specified in 7.3.1.12. |
| 4 | BSSID | The BSSID element is specified in 7.3.2.65a. |
| 5 | TDLS Discovery | Optional. The TDLS Discovery element is specified in 7.3.2.65b. |

The TDLS Discovery Response frame is encapsulated in a Data frame and transmitted directly (i.e., not via the AP) to the TDLS peer STA that sent the corresponding TDLS Discovery Request frame. See 11.19.

11. Frame formats

11.19 Tunneled Direct Link Setup

Modify the text of this clause as follows:

Tunneled Direct Link Setup (TDLS) is characterized by encapsulating setup frames in Data frames, which allows them to be transmitted through an AP transparently. Therefore, the AP does not need to be direct link capable, nor does it have to support the same set of capabilities that will be used on the direct link between the two TDLS peer STAs. TDLS also includes power saving, in the form of TDLS Peer PSM (scheduled) and Peer U-APSD (unscheduled). STAs that set up a TDLS direct link remain associated with their BSS, but have the option of transmitting frames directly to the other TDLS peer STA.

To set up and maintain a direct link, both TDLS peer STAs need to be associated with the same infrastructure BSS.

To discover TDLS capable STAs in the same BSS, a STA sends a TDLS Discovery frame to the Broadcast address, through the AP. A TDLS capable STA that receives a TDLS Discovery frame with a matching BSSID in the BSSID element shall send a TDLS Discovery Response frame to the requesting STA, via the direct path.

An example higher layer discovery mechanism is given in Annex U.1a.

Add the following text to 11u:

Alternatively, a TDLS capable STA may use Native GAS to discover another TDLS capable STA, using the TDLS discovery element. The procedure for TDLS discovery using native GAS is given in Annex U.1a.1.

# Annex U (normative) Usage of Ethertype 89-0d

# U.1 Ethertype 89-0d frame body

Add the following subclause at the end of this Annex:

U.1a TDLS Discovery

U.1a.1 TDLS discovery information

TDLS discovery information allows TDLS peer STA’s to discover their capabilities. An example of TDLS discovery information could be expressed in an extensible xml format as follows:

<TDLS discovery>

<mode> tdls </mode>

<wireless info>

<mac address> 00:01:02:03:04:05 </mac address>

<bssid> 00:0a:0b:0c:0d:0e </bssid>

</wireless info>

< network info>

< DHCP> No</DHCP>

< IP address>192.168.2.1 </IP address>

< Subnet> 255.255.255.0 </subnet>

<Hostname> myphone </Hostname>

</network info>

</TDLS discovery>

The minimum required information for TDLS discovery is contained in “wireless info”.  The “mac address” field contains the MAC address of the transmitting STA and the “bssid” identifies the BSS in which the STA is a member. The “network info” could optionally be used to aid the peer devices in establishing a TDLS link.

U.1a.2 TDLS discovery by Higher layers

The discovery of TDLS capable STAs can be accomplished by leveraging well-defined high layer protocols.  Since TDLS peer STAs are members of the same BSS and most likely on the same IP network, there are two IP based multicast protocols that are well suited for this purpose, namely:

- Bonjour (http://www.apple.com/support/bonjour/)

- UPnP (http://www.upnp.org)

For the Bonjour multicast advertisement protocol, a STA could advertise a Bonjour record with a Service Type (http://www.dns-sd.org/ServiceTypes.html) that identifies TDLS capabilities. The TDLS Bonjour record, at a mininimum, should include whether the STA is TDLS capable and the BSSID of the AP to which the STA is associated.  The TDLS Bonjour record may contain other information that is useful and relevant for TDLS discovery and setup.

For the UPnP protocol, a STA could advertise its presence to other STAs on the network by using the Simple Service Discovery Protocol (SSDP). A new STA could use SSDP to discover other STAs that supports UPnP on the network. To support TDLS discovery, the information that is exchanged between STAs, at a minimum, should contain whether the STA is TDLS capable and the BSSID of the AP to which the STA is associated, in addition to basic information about the devices and their services and a description URL, which can be used to gather additional information about a STA.

Add the following subclause at the end of this Annex in TGu

U.1a.3 TDLS discovery by Native GAS (informative)

The TDLS Discovery Element can be used in a native GAS query to discover TDLS capabilities using public action frames. An example of TDLS discovery using GAS is given in Figure U-2. The discovery could work as follows:

- STA1 determines the MAC address of STA2.

- STA1 sends a Native GAS request to STA2. The GAS request includes a TDLS Discovery element and transmits the frame as a public action frame.

- STA2 receives the GAS request frame and includes the TDLS Discovery element in the GAS response frame to STA1.

Figure U-2—Example TDLS capability discovery using GAS