IEEE P802.11
Wireless LANs

| TDLS Peer Discovery |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-01-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Michael Montemurro | Research in Motion | 4701 Tahoe Blvd, Mississauga, ON. L4W 0B4 | +1-905-629-4746 | mmontemurro@rim.com |
| Daniel R. Borges | Apple, Inc. | 1 Infinite Loop MS 306-2HN, Cupertino, CA 95014 | +1 (415) 425-7347 | drborges@apple.com |
| Leo Estevez | Texas Instruments, Inc. | 12500 TI Blvd. MS 8679 Dallas, TX 75243 | 214-567-2622 | leonardo@ti.com |
| Menzo Wentink | Qualcomm | Breukelen, the Netherlands | +31-65-183-6231 | mwentink@qualcomm.com |
| Ashish Shukla | Marvell | 5488 Marvell Lane, Santa Clara, CA |  | ashishs@marvell.com |

### Overview

This submission proposes a mechanism for TDLS peer discovery that is capable of leveraging layer 2 or layer 3 mechanisms.

# 3. Definitions

Modify definitions 3.z2 and 3.z10 as shown. Move definition 3.z10 to 3.z3 and renumber the other definitions accordingly.

3.z2 TDLS initiator STA: STA that transmits a TDLS Setup Request frame or a TDLS Discovery Request frame.

3.z3 TDLS responder STA: STA that receives or is the intended recipient of a TDLS Setup Request frame or TDLS Discovery Request frame.

Figure 7-

Insert new TDLS frames in 7.4.11 as follows

### 7.4.11 TDLS Action frame details

Several Action frame formats are defined to support tunneled direct link setup (TDLS). The Action field values associated with each frame format within the TDLS category are defined in Table 7-57v1.

Table 7-57v1—TDLS Action field values

| Action field value | Meaning |
| --- | --- |
| 0 | TDLS Setup Request |
| 1 | TDLS Setup Response |
| 2 | TDLS Setup Confirm |
| 3 | TDLS Teardown |
| 4 | TDLS Peer Traffic Indication |
| 5 | TDLS Channel Switch Request |
| 6 | TDLS Channel Switch Response |
| 7 | TDLS Peer PSM Request |
| 8 | TDLS Peer PSM Response |
| 9 | TDLS Peer Traffic Response |
| 10 | TDLS Discovery Request |
| 11 | TDLS Discovery Response |
| 10 – 255 | Reserved |

Insert the following subclauses following 7.4.11.10:

### 7.4.11.11 TDLS Discovery Request frame format

The TDLS Discovery Request frame contains the information shown in Table 7-57v12.

Table 7-57v12—Information for TDLS Discovery Request frame

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Category | The Category field is set to the value for TDLS, as defined in Table 7-24. |
| 2 | Action | The Action field is set to 10, representing TDLS Discovery Request. |
| 3 | Dialog Token | The Dialog Token can be used to match TDLS Disocvery Response frames to the corresponding TDLS Discovery Request frame. The Dialog Token is specified in 7.3.1.12. |
| 4 | Link Identifier | The Link Identifier element is specified in 7.3.2.65a. |
| 5 | Capability | The Capability field indicates the capabilities of the STA. The Capability element is defined in 7.3.1.4. |
| 6 | Supported rates | The Supported Rates element indicates the rates which are supported by the STA. The Supported Rates element is defined in 7.3.2.2. |
| 7 | Extended supported rates | The Extended Supported Rates element is present whenever there are more than eight supported rates, and it is optional otherwise. The Extended Supported Rates element is defined in 7.3.2.14. |
| 8 | HT Capabilities | The HT Capabilities element is present when the dot11HighThroughputOptionImplemented attribute is true. The HT Capabilities element is defined in 7.3.2.56. |
| 9 | 20/40 BSS Coexistence | The 20/40 BSS Coexistence element is present when the dot112040BSSCoexistenceManagementSupport is true. |
| 10 | Extended Capabilities | The Extended Capabilities element may be present if any of the fields in this element are nonzero. |
| 11 | RSNIE | The RSNIE may be included if security is required on the direct link. The RSNIE is defined in 7.3.2.25. |
| 12 | FTIE | The FTIE may be included if security is required on the direct link. The FTIE is defined in 7.3.2.48. |
| 13 | Timeout Interval | The Timeout Interval element contains the TPK Key Lifetime (optional). Included if security is required on the direct link. The Timeout Interval element is defined in 7.3.2.49. |
| 14 | Supported Channels | The Supported Channels element is defined in 7.3.2.19 (optional). Included if the TDLS channel switching capability field is set to one. |
| 15 | Supported Regulatory Classes | The Supported Regulatory Classes element is defined in 7.3.2.51 (optional). Included if the TDLS channel switching capability field is set to one. |
| 16 | TDLS Discovery | Optional. The TDLS Discovery element is specified in 7.3.2.65b. |

The TDLS Discovery Request frame is encapsulated in a Data frame and transmitted to a TDLS peer STA or to the Broadcast address through the AP, to request TDLS capable STAs in the same BSS to respond with a TDLS Discovery Response frame. See 11.19.

### 7.4.11.12 TDLS Discovery Response frame format

The TDLS Discovery Response frame contains the information shown in Table 7-57v13.

Table 7-57v13—Information for TDLS Discovery Response frame

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Category | The Category field is set to the value for TDLS, as defined in Table 7-24. |
| 2 | Action | The Action field is set to 11, representing TDLS Discovery Response. |
| 3 | Dialog Token | The Dialog Token is copied from the corresponding TDLS Discovery Request frame. The Dialog Token is specified in 7.3.1.12. |
| 4 | Link Identifier | The Link Identifier element is specified in 7.3.2.65a. |
| 5 | Capability | The Capability field indicates the capabilities of the STA. The Capability element is defined in 7.3.1.4. |
| 6 | Supported rates | The Supported Rates element indicates the rates which are supported by the STA. The Supported Rates element is defined in 7.3.2.2. |
| 7 | Extended supported rates | The Extended Supported Rates element is present whenever there are more than eight supported rates, and it is optional otherwise. The Extended Supported Rates element is defined in 7.3.2.14. |
| 8 | HT Capabilities | The HT Capabilities element is present when the dot11HighThroughputOptionImplemented attribute is true. The HT Capabilities element is defined in 7.3.2.56. |
| 9 | 20/40 BSS Coexistence | The 20/40 BSS Coexistence element is present when the dot112040BSSCoexistenceManagementSupport is true. |
| 10 | Extended Capabilities | The Extended Capabilities element may be present if any of the fields in this element are nonzero. |
| 11 | RSNIE | The RSNIE may be included if security is required on the direct link. The RSNIE is defined in 7.3.2.25. |
| 12 | FTIE | The FTIE may be included if security is required on the direct link. The FTIE is defined in 7.3.2.48. |
| 13 | Timeout Interval | The Timeout Interval element contains the TPK Key Lifetime (optional). Included if security is required on the direct link. The Timeout Interval element is defined in 7.3.2.49. |
| 14 | Supported Channels | The Supported Channels element is defined in 7.3.2.19 (optional). Included if the TDLS channel switching capability field is set to one. |
| 15 | Supported Regulatory Classes | The Supported Regulatory Classes element is defined in 7.3.2.51 (optional). Included if the TDLS channel switching capability field is set to one. |
| 16 | TDLS Discovery | Optional. The TDLS Discovery element is specified in 7.3.2.65b. |

The TDLS Discovery Response frame is encapsulated in a Data frame and transmitted directly (i.e., not via the AP) to the TDLS peer STA that sent the corresponding TDLS Discovery Request frame. See 11.19.

Insert a new subclause 11.19.1 as follows:

### 11.19.1 TDLS Discovery

To discover TDLS capable STAs in the same BSS, a TDLS initiator STA sends a TDLS Discovery Request frame to a unicast DA or to the Broadcast DA, through the AP. When the TDLS Discovery Request frame is sent to the Broadcast address, the TDLS responder STA Address field contained in the Link Identifier element of the TDLS Discovery Request frame shall be set to the Broadcast address. A TDLS capable STA that receives a TDLS Discovery Request frame with a matching BSSID in the BSSID element (TDLS responder STA) shall send a TDLS Discovery Response frame to the requesting STA, via the direct path. The TDLS responder STA Address field contained in the Link Identifier element of the TDLS Discovery Response frame shall be set to the MAC address of the STA sending the TDLS Discovery Response frame. The TDLS Discovery Response frame shall be sent using AC_BE when the TDLS responder STA Address field contained in the Link Identifier element of the TDLS Discovery Request frame indicates the Broadcast address.

A TDLS STA may also send TDLS Setup Request frame to a STA in the same BSS to discover whether the peer STA is TDLS capable or not. A TDLS Setup Response frame in response to TDLS Setup Request frame indicates peer STA sending TDLS Setup Response TDLS capable.

Delete 11.19.5 Example of TDLS Peer Discovery.

Modify 11.19.2 as follows:

### 11.19.2 TDLS Direct Link Establishment

To establish a TDLS direct link, the TDLS initiator STA shall send a TDLS Setup Request frame to the intended TDLS responder STA.

Editorial Note: The following changes are suggested for the IEEE 802.11u document. As TGu is scheduled to finish its work after TGz, references to GAS have to be made retrospectively. The change adds TDLS Discovery to Native GAS, thus creating a pre-association discovery mechanism.

Modify IEEE 802.11u as follows:

# Annex U (normative) Usage of Ethertype 89-0d

# U.1 Ethertype 89-0d frame body

Add the following subclause at the end of this Annex:

U.1a TDLS Discovery

U.1a.1 TDLS discovery information

TDLS discovery information allows TDLS peer STA’s to discover their capabilities. An example of TDLS discovery information could be expressed in an extensible xml format as follows:

<TDLS discovery>

<mode> tdls </mode>

<wireless info>

<mac address> 00:01:02:03:04:05 </mac address>

<bssid> 00:0a:0b:0c:0d:0e </bssid>

</wireless info>

< network info>

< DHCP> No</DHCP>

< IP address>192.168.2.1 </IP address>

< Subnet> 255.255.255.0 </subnet>

<Hostname> myphone </Hostname>

</network info>

</TDLS discovery>

The minimum required information for TDLS discovery is contained in “wireless info”.  The “mac address” field contains the MAC address of the transmitting STA and the “bssid” identifies the BSS in which the STA is a member. The “network info” could optionally be used to aid the peer devices in establishing a TDLS link.

U.1a.2 TDLS discovery by Higher layers

The discovery of TDLS capable STAs can be accomplished by leveraging well-defined high layer protocols.  Since TDLS peer STAs are members of the same BSS and most likely on the same IP network, there are two IP based multicast protocols that are well suited for this purpose, namely:

- Bonjour (http://www.apple.com/support/bonjour/)

- UPnP (http://www.upnp.org)

For the Bonjour multicast advertisement protocol, a STA could advertise a Bonjour record with a Service Type (http://www.dns-sd.org/ServiceTypes.html) that identifies TDLS capabilities. The TDLS Bonjour record, at a mininimum, should include whether the STA is TDLS capable and the BSSID of the AP to which the STA is associated.  The TDLS Bonjour record may contain other information that is useful and relevant for TDLS discovery and setup.

For the UPnP protocol, a STA could advertise its presence to other STAs on the network by using the Simple Service Discovery Protocol (SSDP). A new STA could use SSDP to discover other STAs that supports UPnP on the network. To support TDLS discovery, the information that is exchanged between STAs, at a minimum, should contain whether the STA is TDLS capable and the BSSID of the AP to which the STA is associated, in addition to basic information about the devices and their services and a description URL, which can be used to gather additional information about a STA.

7. Frame formats

7.3.2 Information Elements

In Table 7-26, insert a  new information element, and renumber the reserved values accordingly:

| Table 7-26—Element IDs |  |  |  |
| --- | --- | --- | --- |
| Information Element | Element ID | Length (in octets) | Extensible |
| TDLS Discovery (see 7.3.2.65b) | <ANA> | 3 to 255 | Yes |

Insert the following new subclause after 7.3.2.65a

7.3.2.65b TDLS Discovery

The TDLS Discovery element contains information that may be used by STAs to discover TDLS capabilities of a peer STA. This element would be used in a native GAS query (see Clause 11.23.2).

|  | Element ID | Length | OUI | TDLS Discovery Information |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 3 | 0-252 |

Figure 7-95o33- TDLS Discovery

The Length field is set to the number of octets in the TDLS Discovery element following the Length field.

The OUI field indicates the type of TDLS discovery information.

The TDLS Discovery Information contains capability information that a STA may use to establish a TDLS link. An example of the peer discovery information is described in Annex U.1a.

7.3.4.1 Capability List

The Capability List provides a list of information/capabilities that has been configured on a STA. The Native Query Protocol elements that may be configured are shown in Table 7-43bg. The Capability List element is returned in response to a Native GAS Query Request.

| Table 7-43bg—Native Query Protocol info ID definitions |  |  |
| --- | --- | --- |
| Info Name | Info ID | Native Info Element (clause) |
| Reserved | 0-255 | n/a |
| Capability List | 256 | 7.3.4.1 |
| Venue Name information | 257 | 7.3.4.2 |
| Emergency Call Number information | 258 | 7.3.4.3 |
| Network Authentication Type information | 259 | 7.3.4.4 |
| Roaming Consortium List | 260 | 7.3.4.5 |
| IP Address Type Availability information | 261 | 7.3.4.7 |
| NAI Realm List | 262 | 7.3.4.8 |
| 3GPP Cellular Network information | 263 | 7.3.4.9 |
| AP Geospatial Location | 264 | 7.3.4.10 |
| AP Civic Location | 265 | 7.3.4.11 |
| Domain Name List | 266 | 7.3.4.12 |
| Emergency Alert URI | 267 | 7.3.4.13 |
| NQ TDLS Discovery | 268 | 7.3.4.14 |
| Reserved | 269– 56796 | n/a |
| Native Query Protocol vendor-specific list | 56797 | 7.3.4.6 |
| Reserved | 56798 – 65535 | n/a |

Add the following clauses to 11u

7.3.4.14 NQ TDLS Discovery

The Native Query (NQ) TDLS Discovery element contains information that may be used by STAs to discover TDLS capabilities of a peer STA.

|  | Info ID | Length | OUI | TDLS Discovery Information |
| --- | --- | --- | --- | --- |
| Octets: | 2 | 2 | 3 | 0-252 |

Figure 7-95o33- NQ TDLS Discovery

The Length field is set to 12.

The OUI field indicates the type of TDLS discovery information.

The TDLS Discovery Information contains capability information that a STA may use to establish a TDLS link. An example of the peer discovery information is described in 11.23.8.

11.19 Tunneled Direct Link Setup

Add the following sentence to the end of 11.19:

An example higher layer discovery mechanism is given in Annex U.1a.

Add the following subclause:

11.23.8 Support for NQ TDLS Discovery

The NQ TDLS Discovery element (7.3.4.14) can be used in a native GAS query to discover TDLS capabilities (e.g. another TDLS capable STA) using public action frames. An example of TDLS discovery using GAS is given in Figure 11-xxxx. The discovery could work as follows:

- STA1 determines the MAC address of STA2.

- STA1 sends a Native GAS request to STA2. The GAS request includes a TDLS Discovery element and transmits the frame as a public action frame.

- STA2 receives the GAS request frame and includes the TDLS Discovery element in the GAS response frame to STA1.

Figure 11-xxxx - Example TDLS capability discovery using GAS