IEEE P802.11
Wireless LANs

| RFI Tüddelkram Resolutions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-11-19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Michael Bahr | Siemens AG, Corporate Technology | Otto-Hahn-Ring 6 80200 München, Germany |  | bahr et siemens dod com |

Abstract

Resolutions to several Tüddelkram comments in the RFI category.

All text changes based on IEEE 802.11s D3.04.

CID 884: Counter

Instruction to Editor: Change text in clauses 7.4.14 7.4.14 Mesh Path Selection action frame details,  as indicated by the tracked changes.

- Mesh Path Selection action frame details

- General

The Action frame format for management of Mesh Path Selection is defined in this subclause. Detailed addressing information for this action frame is provided in Error: Reference source not found Addressing of Mesh Path Selection action frame.

The Action field values associated with each frame format are defined in Mesh Path Selection Action field valuesinput.doc.

| - Mesh Path Selection Action field valuesinput.doc |  |
| --- | --- |
| Action field value | Description |
|  | HWMP Mesh Path Selection |
| 1-255 | Reserved |

- HWMP Mesh Path Selection frame format

The HWMP Mesh Path Selection frame is transmitted by a mesh STA for the purpose of establishing paths to other mesh STAs using the HWMP protocol defined in Error: Reference source not found. The HWMP Mesh Path Selection frame can contain any number but at least one of the information elements defined in HWMP Mesh Path Selection frame bodyinput.doc.

| - HWMP Mesh Path Selection frame bodyinput.doc |  |  |
| --- | --- | --- |
| Order | Information | Notes |
|  | Category |  |
|  | Action |  |
| 3 | Root Announcement element (optional) |  |
| 4 | Path Request element (optional) |  |
| 5 | Path Reply element (optional) |  |
| 6 | Path Error element (optional) |  |
| Last | Vendor Specific | Optionally present: one or more vendor-specific information elements. This information element follows all other information elements. |

The Category field is set to the value in Error: Reference source not found for category Mesh Path Selection.

The Action field is set to the value in Mesh Path Selection Action field valuesinput.doc for this action frame type.

The Root Announcement element is set as described in RANN element.

The Path Request element is set as described in Error: Reference source not found.

The Path Reply element is set as described in Error: Reference source not found.

The Path Error element is set as described in Error: Reference source not found.

CIDs 156, 157, 645: Counter

Instruction to Editor: In Table 7-26 Element IDs, change the length value for the Portal Announcement (PANN) from 15 to 17 and the length value for the Root Announcement (RANN) from 19 to 23.

Instruction to Editor: Change text in clause 7.3.2.98 PANN element as indicated by the tracked changes.

- PANN element

The Portal Announcement (PANN) element is used for announcing in the MBSS the presence of a mesh STA collocated with a portal.

The PANN element is transmitted in a beacon or a Mesh Interworking action frame (see Error: Reference source not found).

The format of the PANN element is shown in PANN  element.

| Element ID | Length | Flags | Hopcount | Time to Live | Mesh Portal Address | PANN Sequence Number | Interval |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: 1 | 1 | 1 | 1 | 1 | 6 | 4 | 2 |
| - PANN element |  |  |  |  |  |  |  |

The Element ID is set to the value given in Error: Reference source not found for this information element. The length is set to 13.

The Flags field is reserved for future use.

The Hop Count field is coded as an unsigned integer. When the Mesh Portal sends the PANN primitive, it sets the hop count to 0. The Hop Count field indicates the number of hops the PANN primitive has traversed.

The Time to Live field is coded as an unsigned integer and indicates the maximum number of hops allowed for this element.

The Mesh Portal Address is represented as a 48-bit MAC address and is set to the MAC address of the mesh STA that is collocated with the portal.

The PANN Sequence Number field is coded as an unsigned integer and is set to a PANN Sequence Number specific for the originating mesh portal.

The Interval field is coded as an unsigned integer and is set to the number of seconds between the periodic transmissions of Portal Announcements by the mesh STA collocated with the portal.

Detailed usage of the PANN element is described in Error: Reference source not found.

Instruction to Editor: Change text in clause 7.3.2.99 RANN element as indicated by the tracked changes.

- RANN element

The Root Announcement (RANN) element is used for announcing the presence of a mesh STA configured as root mesh STA with dot11MeshHWMProotMode set to rann (4). RANN elements are sent out periodically by such a root mesh STA.

The RANN element is transmitted in a beacon or a Mesh Path Selection action frame (see CID 884: Counter).

The format of the RANN element is shown in RANN  element.

| Element ID | Length | Flags | Hopcount | Time to Live | Root mesh STA Address | HWMP Sequence Number | Interval | Metric |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: 1 | 1 | 1 | 1 | 1 | 6 | 4 | 4 | 4 |
| - RANN element |  |  |  |  |  |  |  |  |

The Element ID is set to the value given in Error: Reference source not found for this information element. The length is set to 17.

The format of the Flags field is shown in RANN Flags field format.

| B0 | B1 B7 |
| --- | --- |
| Portal Role | Reserved |
| Bits: 1 | 7 |
| - RANN Flags field format |  |

The Flags field is set as follows.

Bit 0: Portal Role (0 = non-portal, 1 = portal).

Bit 1-7: Reserved.

The Hop Count field is coded as an unsigned integer and indicates the number of hops from the originating root mesh STA to the mesh STA transmitting this element.

The Time to Live field is coded as an unsigned integer and indicates the remaining number of times the RANN will be propagated.

The Root mesh STA Address field is represented as a 48-bit MAC address and is set to the Root mesh STA MAC address.

The HWMP Sequence Number is coded as an unsigned integer and is set to the HWMP sequence number specific to the root mesh STA.

The Interval field is coded as an unsigned integer and is set to the number of TUs between the periodic transmissions of Root Announcements.

The Metric field is set to the cumulative metric from the originating root mesh STA to the mesh STA transmitting the announcement.

Detailed usage of the RANN element is described in Error: Reference source not found.

Instruction to Editor: Change text in clause 11C.8.2.3 Conditions for generating and sending a PANN as indicated by the tracked changes.

- Conditions for generating and sending a PANN

A mesh STA shall propagate a PANN frame in the following cases:

Case A: Original transmission

All of the following applies:

- The mesh STA is collocated with portal

- dot11MeshPortalAnnouncementProtocol is set to TRUE

- At every dot11MeshPortalAnnouncementInterval

The content of a PANN element in Case A shall be as shown in Content of a PANN element in Case A.

| - Content of a PANN element in Case A |  |
| --- | --- |
| Field | Value |
| ID | Value given in Error: Reference source not found for the PANN element |
| Length | 13 |
| Flags | Not used |
| Hop Count | 0 |
| Time to Live | Maximum number of hops allowed for the Portal Announcement |
| Mesh Portal Address | Mesh STA MAC address |
| PANN Sequence Number | A sequence number specific for the portal |
| Interval | dot11MeshPortalAnnouncementInterval |

Case B:	Propagation

All of the following applies:

- The mesh STA has received a Portal Announcement

- The decremented time to live of the Portal Announcement is equal to or greater than 1

- dot11MeshForwarding is set to TRUE

The content of a PANN element in Case B shall be as shown in Content of a PANN element in Case B.

| - Content of a PANN element in Case B |  |
| --- | --- |
| Field | Value |
| ID | Value given in Error: Reference source not found for the PANN element |
| Length | 13 |
| Flags | As received |
| Hop Count | As received + 1 |
| Time to Live | As received – 1 |
| Mesh Portal Address | As received |
| PANN Sequence Number | As received |
| Interval | As received |

Instruction to Editor: Change text in clause 11C.10.9.3 Conditions for generating and sending a RANN as indicated by the tracked changes.

- Conditions for generating and sending a RANN

A mesh STA sends out a RANN element in a Mesh Path Selection frame, as defined in HWMP Mesh Path Selection frame format, in the following cases:

Case A: Original transmission

All of the following conditions apply:

- The mesh STA is configured as a root mesh STA using the proactive RANN mechanism (dot11MeshHWMProotMode = 4).

- The root mesh STA sent its previous RANN dot11MeshHWMPrannInterval TUs ago

The contents of a RANN element in Case A shall be as shown in Contents of a RANN element in Case Ainput.doc.

| - Contents of a RANN element in Case Ainput.doc |  |
| --- | --- |
| Field | Value |
| ID | Value given in Error: Reference source not found for the RANN element |
| Length | 17 |
| Flags | Bit 0: Portal Role (0 = non-portal, 1 = portal) Bit 1 – 7: Reserved |
| Hop Count | 0 |
| HWMP TTL | Maximum number of hops allowed for this element |
| Root mesh STA Address | MAC address of the root mesh STA |
| HWMP Sequence Number | Last used HWMP sequence number of the root mesh STA + 1 |
| Interval | dot11MeshHWMPrannInterval |
| Metric | Initial value of active path selection metric |

Case B:	Propagation

All of the following conditions apply:

- The mesh STA has valid forwarding information to a root mesh STA using the proactive RANN mechanism (dot11MeshHWMProotMode = 4)

- The mesh STA sent its previous RANN dot11MeshHWMPrannInterval TUs ago

- dot11MeshForwarding is set to TRUE

- The Time To Live field is greater than 1 —See Error: Reference source not found

The contents of a RANN element in Case B shall be as shown in Contents of a RANN element in Case Binput.doc.

| - Contents of a RANN element in Case Binput.doc |  |  |  |
| --- | --- | --- | --- |
| Field | Value |  |  |
| ID | Value given in Error: Reference source not found for the RANN element |  |  |
| Length | As received |  |  |
| Flags | As received |  |  |
| Hop Count | As received + 1 |  |  |
| HWMP TTL | As received – 1 |  |  |
| Root mesh STA Address | As received |  |  |
| HWMP Sequence Number | As received |  |  |
| Interval |  | dot11MeshHWMPrannInterval |  |
| Metric | As receivedown link metric toward the transmitting mesh STA |  |  |

References: