IEEE P802.11
Wireless LANs

| PREP Addressing Correction |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-11-19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Guenael Strutt | Motorola | 1064 Greenwood Blvd, Ste 400 Lake Mary, FL 32746 -- USA | +1-407-562-4050 | Guenael.Strutt@motorola.com |

Abstract

Resolution of CIDs 502, 503 and 926

| 502 | case A? case B? case C? case D? That's the first mention of these cases! | describe the addressing of the frames with respect to the mesh STAs in figure s60. Explain how the "path originator" addresses a frame, then how "intermediate" mesh STAs address the frame, then how the frame given to the "path target" looks and what, if anything, a frame emitted by the "path target" (e.g. possibly to a proxied mesh STA) would look like. | Counter | The comment and the proposed resolution are different. The counter is for the proposed resolution. The comment is valid and is addressed by CID 926. The reason why we cannot always describe the addressing of frames with respect to the mesh STAs in figure s60 is the same reason clause 11C.10.1.4 Collocated STAs exists. HWMP keeps track of STA MAC addresses, but what the actual physical address used by for the purpose of propagating Path Selection elements may vary slightly. |
| --- | --- | --- | --- | --- |

| 503 | aren't there 6 addresses in a mesh frame? This only talks about 3 addresses. | explain what the other 3 addresses are or why they are not there and maybe mention what the TODS and FROMDS bits are set to. | Reject | These action frames are plain vanilla action frames. That's why there are only 3 addresses. ToDS and FromDS are not used in management frames. |
| --- | --- | --- | --- | --- |

| 926 | the addressing for PREP cases B, C, and D is missing | add addressing for missing cases. | Counter | Added cases B, C and D |
| --- | --- | --- | --- | --- |

Instruction to Editor: In clause 11C.10.4 Addressing of Mesh Path Selection action frame, change the following paragraph as indicated by change tracking.

- Addressing of Mesh Path Selection action frame

All HWMP information elements are sent in a Mesh Path Selection action frame (see Error: Reference source not found). The RANN element may also be sent in a Beacon frame.  “Cases” refer to the different conditions that trigger the transmission of a Mesh Path Selection action frame.  PREQ cases are specified in 11C.10.6.3 (Conditions for generating and sending a PREQ).  PREP conditions are specified in 11C.10.7.3 (Conditions for generating and sending a PREP).  PERR cases are specified in 11C.10.8.3 (Conditions for generating and sending a PERR). RANN cases are specified in 11C.10.9.3 (Conditions for generating and sending a RANN).

- PREQ Case A, Case B (if previous PREQ received was group addressed), Case D (if previous PREQ received was group addressed), Case E:

- Address 1: Group address

- Address 2: Address of the mesh STA sending the PREQ

- Address 3: Same as Address 2

- PREQ Case B (if previous PREQ received was individually addressed), Case D (if previous PREQ received was individually addressed):

- Address 1: Next hop MAC address to the Mesh STA identified as the Target MAC address in the PREQ element

- Address 2: Address of the mesh STA sending the PREQ

- Address 3: Same as Address 2

- PREQ Case C:

- Address 1: Address 2 of the frame containing the RANN element that triggered the PREQ

- Address 2: Address of the mesh STA sending the PREQ

- Address 3: Same as Address 2

NOTE—For all individually addressed PREQ cases, Address 1 can only be identical to the “Target Address” in the PREQ element, if Address 2 is the last hop to the Target.

- PREP Case A, Case C, Case D:

- Address 1: Address of the next hop to the Originator mesh STA Address of the PREQ that triggered the PREP

- Address 2: Address of the mesh STA sending the PREP

- Address 3: Same as Address 2

- PREP Case B:

- Address 1: Address of the next hop to the Originator mesh STA Address of the PREP that triggered the PREP

- Address 2: Address of the mesh STA sending the PREP

- Address 3: Same as Address 2

NOTE—Address 1 can only be identical to the “Originator mesh STA Address” in the PREP element, if Address 2 is the last hop to the Originator.

- PERR (individually addressed):

- Address 1: Address of the precursor that the PERR is intended for

- Address 2: Address of the mesh STA sending the PERR

- Address 3: Same as Address 2

- PERR (group addressed):

- Address 1: group address

- Address 2: Address of the mesh STA sending the PERR

- Address 3: Same as Address 2

- RANN:

- Address 1: group address

- Address 2: Address of the mesh STA sending the RANN

- Address 3: Same as Address 2

Multiple HWMP information elements may be sent in the same Mesh Path Selection action frame if they share the same intended Address 1.

Instruction to Editor: change table s56 as indicated by change tracking.

| - Contents of a PREP element in Case Binput.doc |  |
| --- | --- |
| Field | Value |
| ID | Value given in Error: Reference source not found for the PREP element |
| Length | As received |
| Flags | As received |
| Hop Count | As received + 1 |
| Time to Live | As received – 1 |
| Target Address | As received |
| Target HWMP Sequence Number | As received |
| Proxied Address | As received |
| Lifetime | As received |
| Metric | As receivedown metric toward the transmitting mesh STA |
| Originator mesh STA Address | As received |
| Originator HWMP Sequence Number | As received |

References: