IEEE P802.11
Wireless LANs

| Adding Dialog Token to IEEE 802.11u MLME Primitives |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-12-2 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Dave Stephenson | Cisco Systems, Inc. | 170 W. Tasman Dr. San Jose, CA 95134 | +1 408 527 7991 | daves@cisco.com |

Abstract

This submission addresses the following comments from the P802.11u Initial Sponsor Ballot: 7389500023, 7389600023, 7389700023 and 7389800023.  Note that these comments did not address the protected dual of the GAS public action frames; nonetheless, the similar edits were made for those primitives.

In addition, there was an erroneous cross reference fixed in the table describing parameters for the MLME-PDGAS-response primitive (Table 7-15 corrected to Table 11-4).

In addition, several of the .response and .confirm primitives had incorrect valid ranges for result code (not all had PARTIAL_QUERY_RESPONSE_CONFIG, PARTIAL_QUERY_RESPONSE_SIZE correct) which were fixed.

In addition, there were two Dialog Token parameters in the MLME-ADDTS.indication primitive—one was deleted.

This document is based on IEEE 802.11u-d8.0.

10	Layer management

10.3	MLME SAP interface

10.3.24.3 MLME- ADDTS.indication

10.3.24.3.2 Semantics of the service primitive

Delete the following text as shown below:

MLME-ADDTS.indication(

DialogToken,

Non-APSTAAddress

TSPEC,

TCLAS,

TCLASProcessing,

EBR,

VendorSpecificInfo

)

10.3.62 Network Discovery and Selection Support

10.3.62.1 MLME-GAS.request

10.3.62.1.2 Semantics of the service primitive

Insert the following text as shown below:

The primitive parameters are as follows:

MLME-GAS.request (
	PeerSTAAddress,

DialogToken
	AdvertisementProtocolID,
	Query,
	QueryFailureTimeout
	)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MacAddress | Any valid individual MacAddress | Specifies the address of the peer MAC entity to which query is transmitted. |
| DialogToken | Integer | 0 – 255 | The dialog token to identify the GAS transaction. |
| AdvertisementProtocolID | Integer or Sequence of Integers | As defined in Table 7-43bw | Identifies which protocol is used to format Query. This is either an 802.11 assigned Advertisement Protocol ID or a vendor-specific information element containing a vendor-specific advertisement protocol ID. |
| Query | String | N/A | Query string formatted using protocol identified in AdvertisementProtocolID. E.g., if the AdvertisementProtocolID value is 1, then Query is formatted as defined in IEEE Std 802.21-2008. |
| QueryFailureTimeout | Integer | > 1 | The time limit, in units of Beacon intervals, after which the GAS query procedure will be terminated. |

10.3.62.2 MLME-GAS.confirm

10.3.62.2.2 Semantics of the service primitive

Insert the following text as shown below:

The primitive parameters are as follows:

MLME-GAS.confirm (
	PeerSTAAddress,

DialogToken
	ResultCode,
	ResponseInfo 
	)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MacAddress | Any valid individual MacAddress | Specifies the address of the peer MAC entity to which query is transmitted. |
| DialogToken | Integer | 0 – 255 | The dialog token to identify the GAS transaction. |
| ResultCode | Enumeration | SUCCESS, TIMEOUT, UNSPECIFIED_FAILURE, ADVERTISEMENT_PROTOCOL_NOT_SUPPORTED, QUERY_RESPONSE_TOO_LARGE, PARTIAL_QUERY_RESPONSE_CONFIG, PARTIAL_QUERY_RESPONSE_SIZE, SERVER_UNREACHABLE, REQUEST_INFO_NOT_CONFIGURED, TRANSMISSION_FAILURE | Indicates the result response to the GAS request from the peer MAC entity. |
| ResponseInfo | String | N/A | Query response string formatted using protocol identified in AdvertisementProtocolID. E.g., if the AdvertisementProtocolID value is 1, then Query is formatted as defined in IEEE Std 802.21-2008. |

10.3.62.3 MLME-GAS.indication

10.3.62.3.2 Semantics of the service primitive

Insert the following text as shown below:

The primitive parameters are as follows:

MLME-GAS.indication (

PeerSTAAddress,

DialogToken

AdvertisementProtocolID,

Query

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MacAddress | Any valid individual MAC address | Specifies the address of the peer MAC entity from which the query message was received. |
| DialogToken | Integer | 0 – 255 | The dialog token to identify the GAS transaction. |
| AdvertisementProtocolID | Integer or Sequence of Integers | As defined in Table 7-43ew | Identifies which protocol is used to format Query. This is either an 802.11 assigned Advertisement Protocol ID or a vendor-specific information element containing a vendor-specific advertisement protocol ID. |
| Query | String | N/A | Query string formatted using protocol identified in AdvertisementProtocolID. E.g., if the AdvertisementProtocolID value is 1, then Query is formatted as defined in IEEE Std 802.21-2008. |

10.3.62.4 MLME-GAS.response

10.3.62.4.2 Semantics of the service primitive

Insert the following text as shown below:

The primitive parameters are as follows:

MLME-GAS.response (
	PeerSTAAddress,

DialogToken
	ResultCode,
	ResponseInfo
	)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MacAddress | Any valid individual MAC address | Specifies the address of the peer MAC entity to which query response information is transmitted. |
| DialogToken | Integer | 0 – 255 | The dialog token to identify the GAS transaction. |
| ResultCode | Enumeration | SUCCESS, NO_REQUEST_OUTSTANDING, ADVERTISEMENT_PROTOCOL_NOT_SUPPORTED, QUERY_RESPONSE_OUTSTANDING, QUERY_RESPONSE_TOO_LARGE, PARTIAL_QUERY_RESPONSE_CONFIG, PARTIAL_QUERY_RESPONSE_SIZE, SERVER_UNREACHABLE, REQUEST_INFO_NOT_CONFIGURED, TIMEOUT | Indicates the result response to the GAS-request from the peer MAC entity. See Table 11-4. |
| ResponseInfo | String | N/A | Query response string formatted using protocol identified in AdvertisementProtocolID. E.g., if the AdvertisementProtocolID value is 1, then Query is formatted as defined in IEEE Std 802.21-2008. |

10.3.63 Protected Dual of Network Discovery and Selection Support

10.3.63.1 MLME-PDGAS.request

10.3.63.1.2 Semantics of the service primitive

Insert the following text as shown below:

The primitive parameters are as follows:

MLME-PDGAS.request (
	PeerSTAAddress,

DialogToken
	AdvertisementProtocolID,
	Query,
	QueryFailureTimeout
	)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MacAddress | Any valid individual MacAddress | Specifies the address of the peer MAC entity to which query is transmitted. |
| DialogToken | Integer | 0 – 255 | The dialog token to identify the GAS transaction. |
| AdvertisementProtocolID | Integer or Sequence of Integers | As defined in Table 7-43be | Identifies which protocol is used to format Query. This is either an 802.11 assigned Advertisement Protocol ID or a vendor-specific information element containing a vendor-specific advertisement protocol ID. |
| Query | String | N/A | Query string formatted using protocol identified in AdvertisementProtocolID. E.g., if the AdvertisementProtocolID value is 1, then Query is formatted as defined in IEEE Std 802.21-2008. |
| QueryFailureTimeout | Integer | > 1 | The time limit, in units of Beacon intervals, after which the GAS query procedure will be terminated. |

10.3.63.2 MLME-PDGAS.confirm

10.3.63.2.2 Semantics of the service primitive

Insert the following text as shown below:

The primitive parameters are as follows:

MLME-PDGAS.confirm (
	PeerSTAAddress,

DialogToken
	ResultCode,
	ResponseInfo 
	)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MacAddress | Any valid individual MacAddress | Specifies the address of the peer MAC entity to which query is transmitted. |
| DialogToken | Integer | 0 – 255 | The dialog token to identify the GAS transaction. |
| ResultCode | Enumeration | SUCCESS, TIMEOUT, UNSPECIFIED_FAILURE, ADVERTISEMENT_PROTOCOL_NOT_SUPPORTED, QUERY_RESPONSE_TOO_LARGE, PARTIAL_QUERY_RESPONSE_CONFIG, PARTIAL_QUERY_RESPONSE_SIZE, SERVER_UNREACHABLE, REQUEST_INFO_NOT_CONFIGURED, TRANSMISSION_FAILURE | Indicates the result response to the GAS request from the peer MAC entity. |
| ResponseInfo | String | N/A | Query response string formatted using protocol identified in AdvertisementProtocolID. E.g., if the AdvertisementProtocolID value is 1, then Query is formatted as defined in IEEE Std 802.21-2008. |

10.3.63.3 MLME-PDGAS.indication

10.3.63.3.2 Semantics of the service primitive

Insert the following text as shown below:

The primitive parameters are as follows:

MLME-PDGAS.indication (
PeerSTAAddress,

DialogToken
AdvertisementProtocolID,
Query

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MacAddress | Any valid individual MAC address | Specifies the address of the peer MAC entity from which the query message was received. |
| DialogToken | Integer | 0 – 255 | The dialog token to identify the GAS transaction. |
| AdvertisementProtocolID | Integer or Sequence of Integers | As defined in Table 7-43be | Identifies which protocol is used to format Query. This is either an 802.11 assigned Advertisement Protocol ID or a vendor-specific information element containing a vendor-specific advertisement protocol ID. |
| Query | String | N/A | Query string formatted using protocol identified in AdvertisementProtocolID. E.g., if the AdvertisementProtocolID value is 1, then Query is formatted as defined in IEEE Std 802.21-2008. |

10.3.63.4 MLME-PDGAS.response

10.3.63.4.2 Semantics of the service primitive

Insert the following text as shown below:

The primitive parameters are as follows:

MLME-PDGAS.response (
	PeerSTAAddress,

DialogToken
	ResultCode,
	ResponseInfo
	)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MacAddress | Any valid individual MAC address | Specifies the address of the peer MAC entity to which query response information is transmitted. |
| DialogToken | Integer | 0 – 255 | The dialog token to identify the GAS transaction. |
| ResultCode | Enumeration | SUCCESS, NO_REQUEST_OUTSTANDING, ADVERTISEMENT_PROTOCOL_NOT_SUPPORTED, QUERY_RESPONSE_OUTSTANDING, QUERY_RESPONSE_TOO_LARGE, PARTIAL_QUERY_RESPONSE_CONFIG, PARTIAL_QUERY_RESPONSE_SIZE, SERVER_UNREACHABLE, REQUEST_INFO_NOT_CONFIGURED, TIMEOUT | Indicates the result response to the GAS-request from the peer MAC entity. See Table 11-4. |
| ResponseInfo | String | N/A | Query response string formatted using protocol identified in AdvertisementProtocolID. E.g., if the AdvertisementProtocolID value is 1, then Query is formatted as defined in IEEE Std 802.21-2008. |