IEEE P802.11
Wireless LANs

| P802.11p-D9_SB_Editorial Comment Resolutions–V3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-01-07 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Francois Simon | US DOT | ARINC, Inc. 2551 Riva Road Annapolis, MD 21401 | 410-266-4531 | fsimon@arinc.com |

Abstract

This paper addresses the comments: FS-1, 1002, FS-2, FS-3, 1020, 1122, 1128, 1131, 1035, 1103, 1115, 1104, 1036, 1105, 1022, 1023, 1038, 1095, FS-4, 1096, 1097, 1024, FS-5, 1025, 1098, 1106, 1099, 1003, 1101, 1102, 1147, 1107, 1108, 1009, 1039, FS-6, 1040, FS-7, 1041, 1062, 1013, 1109, FS-8, 1017, 1043, 1044, 1045, 1046, 1048, 1050, 1051, 1033, 1092, 1004, 1021, and FS-9.

FS-x – Additional Editorial comment not addressed elsewhere.

|  | “Sponsor Ballot” Comment Resolution |
| --- | --- |

- COMMENT:  [From Spreadsheet]  INSERT Original Comment Here:

| ID | Commenter | Clause | Pg | Ln | Type | Comment | Suggested Remedy | Recommended Resolution |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FS-1 | Francois Simon | Cover page | 0 | 42 | E | While the remaining of the document is “justify” format, this paragraph is not. Note that this papgraph will not be included in IEEE Std 802.11 | “Justify” the paragraph | Make change as per Suggested Remedy |
| 1002 | Turner, Michelle | 0 | 0 | 0 | ER | Is ITU-R TF.460.6 in the Base or introduced in a previous amendment? If not, please cite the document in text in this amendment if it is needed for the implementation of the document. |  | Declined – ITU-R TF.460.6 is mentioned in subclause 7.3.2.65, Table 7-35b of P802.11p/D9.0. The appropriate normative reference is added in clause 2 of P802.11p/D9.0. The current IEEE Std 802.11 does not include this reference. |
| FS-2 | Francois Simon | Intro. | iii | 6 | E | If the text in the box is intended to reflect the title of the P802.11p amendement (front page), then 2 items needs to be changed: 1) “7” after “Amendment” needs to added; 2) the word “(WAVE”) needs to be removed. | Make the corrections as per comment. | Make change as per Suggested Remedy |
| FS-3 | Francois Simon | Copyrights | iii | 49 | E | It is not believed that “Selfregulation” is an English word (American). | Replace “selfregulation” with “self regulation” | Suggesting making the change. |
| 1020 | Malarky, Alastair |  | 1 | 39 | E | The amendment base cited is outdated. For example IEEE P802.11w is now IEEE Std 802.11w 2009. | Update amendment base to current status of amendments applicable to 802.11p. Reflect changes where applicable in body | Accepted - TBD – All the latest documents need to be identified. Replace “P802.11w” with “P802.11w-2009”. Replace “P802.11z-D5.0” with “P802.11z-D6.0. |
| 1122 | Chaplin, Clint | 2 | 2 | 16 | ER | Here the ITU-T specification is called TF.460.6, while in the body of the standard it's called TF.460-6. Which is correct? | Correct here to TF460-6, or correct elsewhere to TF.460.6 | Accepted: The identification of ITU Recommendation specified in the comment is: “ITU-R TF.460-6”. It is proposed to correct the reference in clause 2. It is correct in subclause 7.3.2.65 |
| 1128 | Roy, Richard | 5.2.6 | 2 | 36 | ER T (J.K.) | Text reads: "A QoS STA or QoS AP must implement those core QoS facilities necessary for its QoS functions to interoperate with other STAs in the BSS or to interoperate with other STAs when exchanging QoS data frames outside the context of a BSS (see 5.2.10 and 11.20)." where "or to interoperate with other STAs when exchanging QoS data frames outside the context of a BSS (see 5.2.10 and 11.20)." has been inserted. | Instead of adding the clause, delete "in the BSS" and add a "QoS" modifier so the text reads: "A QoS STA or QoS AP must implement those core QoS facilities necessary for its QoS functions to interoperate with other QoS STAs." This is after all the intent, and it is much clearer. | TBD – For information, responding to the comment question, this was added in P802.11p/D6.0 – March 2009. |
| 1131 | Roy, Richard | 5.2.10 | 2 | 43 | E | Text reads: "5.2.10 STA transmission of data frames outide the context of a BSS" could be clearer. | Suggested replacement: "5.2.10 STA transmission of data frames over links outside the context of a BSS" | Counter – The expression “over links” seems to be too specific and can be subjected to various interpretations. It would not seem appropriate when including broadcast transmission over the air. It is proposed to retain the current clause title. |
| 1035 | Roebuck, Randal | 5.2.10 | 3 | 11 | E | Add "to" between defined & operate. | Read as "dot11OCBEnabled defined to operate as if" | Declined – The sentence is correct as is. One interpretation of the sentence could be: “STA’s that do not have the MIB variable defined operate as if it is false”. It is proposed to make no change to the text. |
| 1103 | Mcnew, Justin | 5.2.10 | 3 | 18 | E | Regarding "(see clause 7.2.3.14)", the word "clause" is not necessary, unless referring to a major clause heading | Remove the word "clause" so it reads, "see 7.2.3.14) | Accepted – as per Suggested Remedy. The proposed change text reads: “…(see 72.3.14)…” |
| 1115 | Mcnew, Justin | 5.2.10 | 3 | 20 | E | The paragraph specifically calls out the TA, but in fact the vendor specific action frame is more likely to be used | Add the sentence " The Vendor Specific Action frame is another example of a management frame used OCB | Accepted – as per Suggested Remedy. It is proposed to add the following sentence: “The Vendor Specific Action frame is another example of a management frame used outside the context of a BSS” placed before the last sentence of the last paragraph. |
| 1104 | Mcnew, Justin | 5.2.10 | 3 | 21 | E | Regarding "When dot11OCBEnabled is true, a sending STA sets the BSSID field to thewildcard BSSID value", it's confusing because "wildcard BSSID value" hasn't been defined yet. | Add "(see 7.1.3.3.3)" to the end of the cited sentence | Accepted – as per Suggested Remedy. The proposed text change reads: “….field to the wildcard BSSID value (see 7.1.3.3.3).” |
| 1036 | Roebuck, Randal | 5.3.1 | 3 | 32 | E | Data confidentiality (not used when dot11OCBEnabled is true) is not discussed (unlike a or b) in the requirements section or listed as PIC. | Add "Data Confidentiality" explaination in the appropiate section. | Accepted in principle – “Data Confidentiality” provides the security of a WLAN up to the level of security implicit in wired LAN. In the P802.11p amendment, security services are outside the scope of 802.11 and are performed in layers above the MAC sublayer. It is proposed to change the text in subclause 5.2.10, 2nd paragraph, at the end of the 3rd sentence. Replace “….does not utilize the 802.11 authentication or association services.” with “…does not utilize the 802.11 authentication, or association, or data confidentiality services”. |
| 1105 | Mcnew, Justin | 7.1.3.1.2 | 3 | 54 | E | Since the editing instructions specify "insert", there's no need to underscore the entries row 2 of the table | Remove the underlining from the entries for "00", "Management", "0110" and "Timing Advertisement" in row 2 | Accepted in principle – According to the instructions on page 1 – “Insert” - , the Suggested Remedy appears to be correct. Proposing to remove the underlines. |
| 1022 | Malarky, Alastair | 7.1.3.3.3 | 4 | 32 | E | The order of the sentences in this last paragraph of 7.1.3.3.3 is a bit awkward. It seems incorrect to specify a specific use of wildcard BSSID and then make the general statement using the word "elsewhere" which excludes the previous sentence. | recommend changing last paragraph to read "The value of all 1s is used to indicate the wildcard BSSID. The wildcard value shall not be used in the BSSID field except where explicitly permitted in this standard. When dot11OCBEnabled is true, the wildcard value shall be used in the BSSID field." | Accepted per Suggested Remedy. It is propose to change the paragraph as follows: “The value of all 1s is used to indicate the wildcard BSSID. The wildcard value shall not be used in the BSSID field except where explicitly permitted in this standard. When dot11OCBEnabled is true, the wildcard value shall be used in the BSSID field." |
| 1023 | Malarky, Alastair | 7.2.2 | 5 | 10 | E | The Address field contains, not carries, the value (see previous sentence in paragraph). | Replace "or that it carries" with "or that it contains" | Accepted per Suggested Remedy. Propose to change sentence as follows: “or that it contains" |
| 1038 | Roebuck, Randal | 7.2.3.14 | 5 | 48 | ER | Time Advertisement element in table 7-18a is different order than 10.3.51.1.2 service primitive. | Frame format is the mandatory ordering where primitives should be consisent and not cause confusion. | Counter – Table 7-18a identifies the IEs that may be included in the Timing Advertisement frame and their proper order. That frame is to be transmitted on the “air interface”. On the other hand, 10.3.51.1.2 specifies the parameter passed between the SME and the LME. This abstract operation is internal to the STA and it is implementation dependent. The parameters are only loosly associated with the frame’s IEs and ordering is not required. It is the responsibility of the implementation to “translate” the parameters to IE content. While not required, the order in the primitives will be reordered to track those in the frame format. It is propose to change the order of the parameters in 10.3.51.1.2 as follows: - Capability information - Country - Power constraint - Time advertisement - Extended Capabilities - VendorSpecificInfo |
| 1095 | Kenney, John | 7.3.1.31 | 6 | 20 | E | It is unwise for a standard to use the phrase "currently assigns" unless necessary, since it is apt to become incorrect in the future. | Change "currently assigns" to "has assigned" | Accepted per Suggested Remedy. The proposed text reads: “The IEEE has assigned public…..” |
| FS-4 | Francois Simon | 7.3.1.31 | 6 | 21 | E | Misspelling of “e.g.,” | Replace “e.g.” with “e.g,” | Change as per suggested remedy. |
| 1096 | Kenney, John | 7.3.1.31 | 6 | 21 | E | The clause construction is grammatically incorrect. The words "both of" should not be followed by "and of some". | Omit the word "some". Put a period after "longer length". After the inserted period, change "where" to "In the latter case" before "specific OUI values" | Accepted – The current text in the third sentence: “…(OUI) and some longer length where specific OUI…” is proposed to be changed to: “…(OUI) and longer length, in the latter case specific OUI…” |
| 1097 | Kenney, John | 7.3.1.31 | 6 | 32 | E | the word "This" in the final sentence of the note is unclear. | Change "This" to "The value of y" | Accepted per Suggested Remedy. The proposed change reads: “The value of “ y” is specified…” |
| 1024 | Malarky, Alastair | 7.3.2 | 6 | 40 | E | "Ids" is incorrect in Table 7-26 title. It should be "IDs". | As per comment | Accepted in principal – It is proposed to correct Table 7-26 title. However, the title is not part of the P802.11p amendment as this table exists and titled correctly in the base standard. The proposed correction in D9.0 is as follows: “Table 7-26-Element IDs” |
| FS-5 | Francois Simon | 7.3.2 | 6 | 45 | E | All entries in the table should be “”centered” | Center all entries in the table in their respective cells to be consistent with the base standard. | Change as per suggested remedy. |
| 1025 | Malarky, Alastair | 7.3.2.26 | 7 | 4 | E | After making the change, the text has an extra space before "j": "The length of the vendor specific content is n- j octets" | Fix text | Accepted per Suggested Remedy. Propose change: “n-j”. |
| 1098 | Kenney, John | 7.3.2.26 | 7 | 4 | E | There is an extra space between the minus sign and the inserted character "j" | remove the extra space so it reads "n-j" | Accepted per Suggested Remedy. Propose change: “n-j”. |
| 1106 | Mcnew, Justin | 7.3.2.26 | 7 |  | E | In 7.3.2.26, there are several instances of mathematical variables "j" and "n". It would be more clear if these variables were in italics. There are many examples of this in the base document. | Put variables "j" and "n" in italics. Also, remove parentheses around variables,e.g. "(n)", where they occur in the text. | Accepted in principal - As per Suggested remedy. However, this rule will also affect subclause 7.3.1.31. The propose changes will affect: 7.3.1.31 – Page 6 - Line 22 7.3.2.26 – Page 7 – Line 2 - Line 3 - Line 4 - Line 10 |
| 1099 | Kenney, John | 7.3.2.29 | 7 | 30 | E | The edited sentece omits the explicit mention of EDCA parameters found in the baseline sentence | Insert "EDCA parameters" between "the default" and "values" | Accepted per Suggested Remedy. The proposed change reads: “…the default EDCA parameters values used….” |
| 1003 | Hsu, Chun-Yen | 7.3.2.29 | 7 | 34 | E | '...if dot11OCBEnabled is false' should be '...if dot11OCBEnabled is FALSE' | Change the title of Table 7-37 as 'Table 7-37 - Default EDCA Parameter Set element parameter values if dot11OCBEnabled is FALSE' | Decline – From the 802.11 WG Editor’s Meeting (March 2009); doc. IEEE 802.11-09/0322r1, states that “TRUE” and “FALSE” sould be capatalized when preceeded by “set to”; otherwise, “true” and “false” should be lower case. TGp has adopted this rule. Thus, the title of Table 7-37a is correct as is. |
| 1101 | Kenney, John | 7.3.2.65 | 8 | 36 | E | Second sentence is missing the word "field" | Insert "field" after "Timing Capabilities" and "is 1" | Accepted per Suggested Remedy. The proposed text change reads: “When the value of the Timing Capabilities field is 1 the Time Value…” |
| 1102 | Kenney, John | 7.3.2.65 | 8 | 37 | E | hyphenation appears at wrong point in Timestamp (between s and t) | Change so either Timestamp is not hyphenated or hyphen appears between e and s | Accepted in principal – This depends on the editor being used. MSWord, even justified, would not hyphen the word. Perhaps this is a characteristic of FrameMaker? |
| 1147 | Roy, Richard | 7.3.2.65 | 8 |  | E T (Fygs) | Throughout the clause, "Time Value" is used to denote an estimate of a time OFFSET. This is misleading and not amenable to adding further optional terms in the polynomial expansion of the clock difference function (i.e., frequency differences and differential oscillaotr drifts). | Replace "Time Value" with "TSF timer time offset estimate (TTTOE)" to make it perfectly clear what the field contains. Make a similar change to the "Time Error" field. (See 11-08-1165-07-000p-timing-information-element.doc) for details. | TBD |
| 1107 | Mcnew, Justin | 7.4.5 | 9 | 7 | E | It would be more clear if the mathematical variable "j" were in italics. There are many examples of this in the base document. | In Figure 7-101, put "j" (the length of the Organization Identifier field) in italics, or use "Variable" | Accepted per Suggested Remedy. The proposed change is “j”. |
| 1108 | Mcnew, Justin | 7.4.5 | 9 | 17 | E | The sentence "The Vendor Specific Content contains vendor specific fields." doesn't match the base document, yet there are no strikethroughs or underscores. | If the intention is to change the text, use underscores and/or strikethroughs. Otherwise, make the text match the base document. | Counter – It is believed that no change is required for the last paragraph in 7.4.5 in D9.0. It is proposed to retain the text from the base standard and is as follows: ” The Vendor Specific Content contains vendor-specific field(s). The length of the Vendor Specific Content in a Vendor Specific Action frame is limited by the maximum allowed MMPDU size” |
| 1009 | Armstrong, Lee | 9.9.1.2 | 10 | 25 | E | "Insert the following text at the end the second paragraph of 9.9.1.2 after NOTE 3:" missing "of" | "Insert the following text at the end of the second paragraph of 9.9.1.2 after NOTE 3:" | Counter – The “instruction” text in the Suggested Remedy appears to be the same text as specified in P802.11p/D9.0. No proposed change for this comment. |
| 1039 | Roebuck, Randal | 9.9.1.3 | 10 | 33 | ER | Second sentence is not complete. | Delete "In an" and read as "Infrastructure BSS AIFSN[AC] is advertised" | Agree in principal – However, the proposed text is as follows: “ In an infrastructure BSS, AIFSN[AC] is…” |
| FS-6 | Francois Simon | 10.3.50.1.1 | 13 | 14 | E | Spelling error in “MLME return” | Correct to with “ MLME returns” | Propose to correct with “ …MLME retuns….” |
| 1040 | Roebuck, Randal | 10.3.51.1.2 | 15 | 3 | ER | Time Advertisement element comes in different order than shown in Table 7-18a. | Primitive needs to be consisent with the mandatory frame format to avoid any confusion. | Accept in principal – This comment as been addressed in Comment #1038 - 10.3.51.1.2 specifies the parameters passed between the SME and the LME. This abstract operation is internal to the STA and it is implementation dependent. The parameters are only loosly associated with the frame’s IEs and ordering is not required. It is the responsibility of the Implementation to “translate” the parameters to IE content. While not required the order in the primitives will be reordered to track those in the frame format. It is propose to change the order of the parameters in 10.3.51.1.2 as follows: 1) Capability information 2) Country 3) Power constraint 4) Time advertisement 5) Extended Capabilities 6) VendorSpecificInfo |
| FS-7 | F. Simon | 10.3.51.1.3 | 15 | 31 | E | Spelling error in “…MLME generate….” | Replace “…MLME generate…” with “….MLME generates…” | Proposed to correct per Suggested Remedy |
| 1041 | Roebuck, Randal | 10.3.51.3.2 | 16 | 37 | ER | Time Advertisement element comes in different order than shown in Table 7-18a. | Primitive needs to be consisent with the frame format. | Accept in principal – This comment resolution is similar of CID# 1038, except it addresses the primitive associated with a received Timing Advertisement frame. Here again, the order of the parameters to be passed from the MLME to the SME is not required to follow the order of the Timing Advertisement’s fields received. In addition, some of the parameters included in the MLME-TIMING_ADVERTISEMENT.indication primitive are related to the receiver itself and were NOT transmitted over the air interface. The following order of parameters for this primitive is proposed: - Timestamp - Capability Information - Local Time - Country - Power Constraint - Timing Advertisement - Extended Capabilities - RCPI - Source MAC Address - VendorSpecificInfo |
| 1062 | Kenney, John | 11.1 | 17 | 47 | E | The "and" clause of the inserted sentence follows from the initial clause. This would be clarified if the word "therefore" was inserted. | Insert "therefore" after "and" and before "is not" | Accepted per Suggested Remedy. The propose change reads: “…of a BSS, and therefore is not required…” |
| 1013 | Armstrong, Lee | 11.20 (Fygs) | 18 | 18 | E | "subtype Action and Timing Advertisement" | "subtypes Action and Timing Advertisement" | Note: The CID reference should be 11.20; not 11.2 Accepted per Suggested Remedy. The proposed changed text reads: “…subtypes Action and Timing Advertisement…” |
| 1109 | Mcnew, Justin | 17.4.1 | 20 | 35 | E | Since the editing instructions specify "insert", there's no need to underscore the entries in the table | Remove the underlining from the entries in the row to be inserted | Accepted per Suggested Remedy. The proposed text change reads as follow: “dot11ACRType – Implementation dependent – Dynamic” |
| FS-8 | F. Simon | A | 21 |  | E | The entries in the tables of Annex A, in D9.0 does not match the “justification” rules of the base standard | In the Item column, when the format of the PICS name is “MNx”, the name is left justified. When the PICS name name is “MNx.x”, the name is offset to the right. For Feature column, the same rule than for “item” applies. For the References and Status columns, the content is always left justified. For the support column, the content is always “centered”. | Suggest to apply the base standard “justification” rules to P802.11p, Annex A amendment. |
| 1017 | Armstrong, Lee | A.4.3 | 21 | 18 | E | CF2.1 and PC37 are the same. Why? I know that you are not following any authoritative standard on the construction of PICS proformas, so it is hard to guess why you are duplicating items. | Change in A.4.4.1, PC37 status column from "O" to "CF18:O" | Counter – PC37 PICS is proposed to be deleted as per document 09-1242r1 |
| 1043 | Roebuck, Randal | A.4.4.1 | 21 | 34 | ER | Add "No association" to PIC PC1.1a and modify references that do not exist any more. | Change title to "No Authenication or Association State". Delete 5.7.6, 5.7.7 & 8.1 references and replace with 5.2.10, 5.3.1 & 11.3. | Accepted per Suggested Remedy. Propose to change “PC1.1a as follows: “No Authentication or Association state” |
| 1044 | Roebuck, Randal | A.4.4.1 | 21 | 36 | ER | Did not find any supporting text for "No Passive Scanning" PIC PC11.7.1 in section 11.20 or in entire document | Add supporting text for No passive scanning for station operating outside the context of BSS in section 11.20 | TBD |
| 1045 | Roebuck, Randal | A.4.4.1 | 21 | 38 | ER | Did not find any supporting text for "No Active Scanning" PIC PC11.8.1 in section 11.20 or in entire document | Add supporting text for No active scanning for station operating outside the context of BSS in section 11.20 | TBD |
| 1046 | Roebuck, Randal | A.4.4.1 | 21 | 44 | ER | Add PIC PC11.13 for "No Synchronization required for Common Clock". | Add "No Synchronization required for Common Clock", Reference 11.1, Status CF18:O as PIC PC11.13 | Accepted in Principal - Remedy. It is proposed to replace all references to “CF 18” with “CF2.1” as per 09/1242r1. The following PICS are affected: FT7.1, FR7.1, AD4, AD5, OF3.2.8 and QD8 |
| 1048 | Roebuck, Randal | A4.4.4 A.4.4.4 (Fygs) | 22 | 20 | ER | Add section 7.2 to references for Wildcard BSSID AD4 PIC. | Add "7.2" to the references. | Note: the reference should be A.4.4.4 Accepted per Suggested Remedy. Proposed to add “7.2” to the References |
| 1050 | Roebuck, Randal | A4.15 A.4.15 (Fygs) | 23 | 34 | ER | Add section 9.9.1.2 to references for PIC QD8 (default EDCA). | Add "9.9.1.2" to the references. | Accepted per Suggested Remedy. Proposed to add “9.9.1.2” to the References Add section 9.9.1.2 to references for PIC QD8 (default EDCA). Note: the reference should be A.4.15 |
| 1051 | Roebuck, Randal | A4.16 | 23 | 37 | ER | Add PIC QP8 for "No tracffic streams when dot11OCBEnabled is true" | Add "No traffic streams when dot11OCBEnabled is true", References 7.1.3.5.1, Status CF18:M | TBD |
| 1033 | Vlantis, George | D | 24 | 31 | E | "TruthValue," following "dot11ImmediateBlockAckOptionImplemented" is misaligned with the other text in the column. | Delete the extra space before "TruthValue". | Accepted per Suggested Remedy. |
| 1092 | Ecclesine, Peter | Annex J | 32 | 33 | E | IEEE Std. 802.11y-2008 changed the title of Table J.2 to "Regulatory classes in Europe", while text here is from 802.11-2007. | Make the title change per 802.11y. | Accepted per Suggested Remedy. |
| 1004 | Goodall, David |  |  |  | ER | The acronym OCB still appears within the MIB variable dotOCBEnabled and is not defined anywhere. | Either define OCB or remove it from the MIB variable name. | Declined – The acronym “OCB” is not used in IEEE 802.11p/D9.0. Therefore, it is not required to be defined. The acronym “OCB” is an integral part of the MIB attribute name, “dot11OCBEnabled”, and is defined in Annex D. There is no change proposed to 802.11p/D9.0 based on the comment. |
| 1021 | Malarky, Alastair |  |  |  | E | GPS and UTC are defined in 10.3.50.2.4 but no entry is provided for these in section 4 and these are not in the amendment base. | Provide entries for GPS and UTC in section 4. | Accepted per Suggested Remedy. It is proposed to add Clause 4 to P802.11p with 2 acronyms defined and presented as follows: “ 4. Abbreviations and acronyms Change clause 4 by adding the following acronyms in the appropriate alphabetical order: GPS – Global Positioning System UTC – Coordinated Universal Time “ |
| FS-9 | F. Simon | Annex J | 32 | 6 | E | Clause ID “J.1” is missing. This Annex does have “J.2” but not “J.1” | Correct as per comment | It is proposed to introduce “J.1” as follows: “J.1 Country information element and regulatory classes” |

- Motion (if technical and/or significant):

(And instructions to the editor.)

Move to accept the Recommended Resolutions to these comments and the recommended changes to P802.11p D9.0 noted above and instruct the editor to make these changes to P802.11p D9.01.

- Motion by: ___Francois Y. Simon _________________Date: ___________

- Second:  ______________________

| Approve: | Disapprove: | Abstain: |
| --- | --- | --- |

- References: