IEEE P802.11
Wireless LANs

| Firmware Update Notification Text Proposal |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-01-21 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Samir Hodoj | T-Mobile (USA) | 12920 SE 38th Street Bellevue, WA 98006 | +1 (425) 383-3301 | Samir.Hodroj@T-Mobile.com |
| Omar Hassan | T-Mobile USA | 12920 SE 38th Street Bellevue, WA 98006 | +1(425) 383-4000 | Omar.hassan@T-Mobile.com |
| Matthew Gast | Trapeze Networks | 5753 W. Las Positas Blvd Pleasanton, CA 94588 | +1 (925) 474-2273 | msg@trapezenetworks.com |
| Dorothy Stanley | Aruba Networks | 1344 Crossman Ave. Sunnyvale, CA 94089 | +1 (630)-363-1389 | DStanley@arubanetworks.com |

Abstract

This document contains the text changes to resolve TGv initial Sponsor Ballot CIDs 99, 229.

A new Firmware Update Notification diagnostic is introduced, to improve manageability of AP devices in consumer applications.

Insert the changes indicated into the TGv draft:

7.3.2.68 Diagnostic Request element

7.3.2.68.1 Diagnostic Request definition

The Diagnostic Request element contains a request that the receiving STA undertake the specified diagnostic action. The format of the Diagnostic Request element is shown in Figure 7-95o52.

|  | Element ID | Length | Diagnostic Token | Diagnostic Request Type | Diagnostic Timeout | Diagnostic Information Subelements (optional) |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 2 | variable |
| Figure 7-95o52—Diagnostic Request element format |  |  |  |  |  |  |

The Element ID field is equal to the Diagnostic Request value in Table 7-26.

The value of the Length field is 4 plus the length of the Diagnostic Information Subelements field. The minimum value of the Length field is 4 (based on a minimum length for the Diagnostic Information Subelements field of 0 octets).

The Diagnostic Token field is set to a number that is unique among the Diagnostic Request elements sent to each destination MAC address for which a corresponding Diagnostic Report element has not been received.

The Diagnostic Request Type field is set to a number that identifies a type of diagnostic request. The defined Diagnostic Request Types are shown in Table 7-43y.

| Table 7-43y—Diagnostic Type definitions |  |
| --- | --- |
| Name | Diagnostic Type Values |
| Cancel Diagnostic Request | 0 |
| Manufacturer Information STA Report | 1 |
| Configuration Profile | 2 |
| Association Diagnostic | 3 |
| IEEE 802.1X Authentication Diagnostic | 4 |
| Firmware Update Notification | 5 |
| Reserved | 6 – 220 |
| Vendor Specific | 221 |
| Reserved | 222 – 255 |

The Diagnostic Timeout field contains the time, in seconds, after which no response is returned.

The Diagnostic Information Subelements field contains zero or more diagnostic information subelements depending on the specific Diagnostic Request Type.

The Manufacturer Information STA Report and Configuration Profile Diagnostic Request elements carry no Diagnostic Information subelements.

The Diagnostic Request element is included in a Diagnostic Request frame as described in 7.4.12.4. The use of Diagnostic Request element and frames is described in 11.22.3.

7.3.2.68.2 Association Diagnostic request

The Diagnostic Information Subelements field corresponding to an Association Diagnostic Request Type is shown in Table 7-43z. The corresponding Diagnostic Information Subelements are defined in 7.3.2.68.6.

| Table 7-43z—Association Diagnostic request contents |  |
| --- | --- |
| Order | Information Subelement |
| 1 | AP Descriptor |
| 2 | Profile ID |

7.3.2.68.3 IEEE 802.1X Authentication Diagnostic request

The Diagnostic Information Subelements field corresponding to an IEEE 802.1X Authentication Diagnostic Request Type is shown in Table 7-43aa. The corresponding Diagnostic Information Subelements are defined in 7.3.2.68.6.

| Table 7-43aa—IEEE 802.1X Authentication Diagnostic request contents |  |
| --- | --- |
| Order | Information Subelement |
| 1 | AP Descriptor |
| 2 | EAP Method |
| 3 | Credential Type |
| 4 | Profile ID |

7.3.2.68.4 Firmware Update Notification

The Diagnostic Information Subelements field corresponding to a Firmware Update Notification Diagnostic Request Type is shown in Table 7-43xx. The corresponding Diagnostic Information Subelements are defined in 7.3.2.68.6.

| Table 7-43xx—Firmware Update Notification Diagnostic request contents |  |
| --- | --- |
| Order | Information Subelement |
| 1 | AP Descriptor |
| 2 | Firmware Version (current) |
| 3 | Firmware Version (new) |

7.3.2.68.5 Vendor Specific Diagnostic request

The Diagnostic Information Subelements field corresponding to a Diagnostic Report element of type Vendor Specific Diagnostic Report contains zero or more Vendor specific subelements. The Vendor Specific subelements have the same format as their corresponding elements (see 7.3.2.26).

7.3.2.68.6 Diagnostic Information Subelement descriptions

The following text describes the various subelements that may be included in Diagnostic Information Subelements field of a Diagnostic Request element or a Diagnostic Report element. The format of a Diagnostic information subelement is shown in Figure 7-95o53.

|  | Diagnostic Information Subelement ID | Length | Diagnostic Information Subelement Contents |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | variable |
| Figure 7-95o53—Diagnostic Information Subelement format |  |  |  |

The Diagnostic Information Subelement ID field indicates the Diagnostic Information Subelement type and is set to any allocated value in Table 7-43ab.

| Table 7-43ab—Diagnostic Information Subelement ID values |  |  |
| --- | --- | --- |
| Identifier | Subelement Name | Length (in octets) |
| 0 | Credential Type | 3 |
| 1 | AKM Suite | 6 |
| 2 | AP Descriptor | 10 |
| 3 | Antenna Gain | 3 |
| 4 | Antenna Type | 3 to 251 |
| 5 | Cipher Suite | 6 |
| 6 | Collocated Radio Type | 3 |
| 7 | Device Type | 3 |
| 8 | EAP Method | 3 to 10 |
| 9 | Firmware Version | 3 to 251 |
| 10 | MAC Address | 8 |
| 11 | Manufacturer ID String | 3 to 251 |
| 12 | Manufacturer Model String | 3 to 251 |
| 13 | Manufacturer OI | 5 or 7 |
| 14 | Manufacturer Serial Number String | 3 to 251 |
| 15 | Power Save Mode | 6 |
| 16 | Profile ID | 3 |
| 17 | Supported Regulatory Classes | 3 to 251 |
| 18 | Status Code | 4 |
| 19 | SSID | 4 to 36 |
| 20 | Tx Power Capability | 3 to 251 |
| 21 | WFA Certificate ID | 3 to 251 |
| 22-220 | Reserved |  |
| 221 | Vendor Specific | 3 to 251 |
| 221-255 | Reserved |  |

The Length field is set to the length in octets of the Diagnostic Information Subelement Contents field.

The Diagnostic Information Subelement Contents values are described in Table 7-43ab.

The format for the Credential Type subelement is shown in Figure 7-95o54.

|  | Subelement ID | Length | Credential Values |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | variable |
| Figure 7-95o54—Credential Type Subelement format |  |  |  |

The Credentials field indicates one or more types of credential. Each value is chosen from those shown in Table 7-43ac.

| Table 7-43ac— Credentials Values |  |
| --- | --- |
| Value | Description |
| 0 | None |
| 1 | Pre-shared key |
| 2 | Username and password |
| 3 | X.509 certificate |
| 4 | Other certificate |
| 5 | One time password |
| 6 | Token |
| 7–255 | Reserved |

The format for the AKM Suite subelement is shown in Figure 7-95o55.

|  | Subelement ID | Length | OUI | AKM Suite |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 3 | 1 |
| Figure 7-95o55—AKM Suite Subelement format |  |  |  |  |

The OUI and AKM Suite fields identify the authentication method and is set to one of the values in Table 7-34 in 7.3.2.25.2.

The format of the AP descriptor subelement is described in Figure 7-95o56.

|  | Subelement ID | Length | BSSID | Regulatory Class | Channel Number |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 6 | 1 | 1 |
| Figure 7-95o56—AP Descriptor Subelement format |  |  |  |  |  |

The BSSID field is a 6-octet field as described in 7.1.3.3.3 that identifies the BSS indicated in the AP Descriptor subelement.

The Regulatory Class field contains an enumerated value from Annex J specifying the frequency band in which the Channel Number is valid.

The Channel Number field indicates the current operating channel of the AP identified by the BSSID in the AP Descriptor.

The format for the Antenna Gain subelement is shown in Figure 7-95o57.

|  | Subelement ID | Length | Antenna Gain |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 |
| Figure 7-95o57—Antenna Gain Subelement format |  |  |  |

The Antenna Gain field contains the peak gain in dBi of the antenna.

The format for the Antenna Type subelement is shown in Figure 7-95o58.

|  | Subelement ID | Length | Antenna Count | Antenna Type |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | variable |
| Figure 7-95o58—Antenna Type Subelement format |  |  |  |  |

The Antenna Count field contains a one octet field indicating the number of antennas of the indicated Antenna Type.

The Antenna Type field contains an ASCII string (truncated to 251 octets if required) indicating the type of antenna connected to the wireless network adapter. This string is not null terminated.

The format for the Cipher Suite subelement is shown in Figure 7-95o59.

|  | Subelement ID | Length | OUI | Suite Type |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 3 | 1 |
| Figure 7-95o59—Cipher Suite Subelement format |  |  |  |  |

The OUI and Suite Type fields identify the cipher suite and is set to one of the values in Table 7-32.

The format for the Collocated Radio Type subelement is shown in Figure 7-95o60.

|  | Subelement ID | Length | Collocated Radio Type |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 |
| Figure 7-95o60—Collocated Radio Type Subelement format |  |  |  |

The Collocated Radio Type subelement contains a one octet field indicating the type of collocated radio, and is set to one of the values in Table 7-43ad. The method that a STA uses to obtain the information on the Collocated Radio is out of the scope of this standard.

| Table 7-43ad—Collocated Radio Type |  |
| --- | --- |
| Collocated Radio Type | Value |
| Reserved | 0 |
| Cellular | 1 |
| Cordless | 2 |
| GPS | 3 |
| IEEE 802.11 | 4 |
| IEEE 802.15 | 5 |
| IEEE 802.16 | 6 |
| IEEE 802.20 | 7 |
| IEEE 802.22 | 8 |
| Digital Audio Broadcasting | 9 |
| Digital Video Broadcasting | 10 |
| Reserved | 11–255 |

The Device Type subelement reports the type of device in which the 802.11 STA resides. The format of the Device Type subelement is shown in Figure 7-95o61.

|  | Subelement ID | Length | Device Type |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 |
| Figure 7-95o61—Device Type Subelement format |  |  |  |

The Device Type field is a one octet field indicating the category of device based on the Wi-Fi Alliance  category definitions. The numerical assignment to each device type category is defined in Table 7-43ae.

| Table 7-43ae—Device Type definitions |  |
| --- | --- |
| Device Type | Value |
| Reserved | 0 |
| Reference Design | 1 |
| Access Point or Wireless Router for Home or Small Office | 2 |
| Enterprise Access Point | 3 |
| Cable, DSL or Other Broadband Gateway | 4 |
| Digital Still Camera | 5 |
| Portable Video Camera | 6 |
| Networked Web Camera | 7 |
| Digital Audio - Stationary | 8 |
| Digital Audio - Portable | 9 |
| Set-Top Box, Media Extender, Media Server (includes players & recorders) | 10 |
| Display Device (television, monitor, picture frame) | 11 |
| Game Console or Game Console Adapter | 12 |
| Gaming Device - Portable | 13 |
| Media Server or Media Adapter | 14 |
| Network Storage Device | 15 |
| External Wi-Fi Card | 16 |
| Internal Wi-Fi Card | 17 |
| Ultra-Mobile PC | 18 |
| Notebook Computer | 19 |
| PDA (Personal Digital Assistant) | 20 |
| Printer or Print Server (includes scanner and/or fax capability) | 21 |
| Phone - Dual-Mode | 22 |
| Phone - Single-Mode | 23 |
| Smartphone - Dual-Mode | 24 |
| Smartphone - Single-Mode | 25 |
| Reserved | 26-220 |
| Other devices | 221 |
| Reserved | 222–255 |

The format for the EAP method subelement is shown in Figure 7-95o62.

|  | Subelement ID | Length | EAP Type | EAP Vendor ID (optional) | EAP Vendor Type (optional) |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 0 or 3 | 0 or 4 |
| Figure 7-95o62—EAP Method Subelement format |  |  |  |  |  |

The EAP Type field contains a value that identifies a single EAP method and is set to any valid IANA assigned EAP type.

The EAP Vendor ID field contains a value that identifies the EAP Vendor. The EAP Vendor ID field is included when EAP Type field is set to 254, and is excluded otherwise.

The EAP Vendor Type field contains a value that identifies the EAP Type as defined by the vendor. The EAP Vendor Type field is included when EAP Type field is set to 254, and is excluded otherwise.

The format for the Firmware Version subelement is shown in Figure 7-95o63.

|  | Subelement ID | Length | Firmware Version |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | variable |
| Figure 7-95o63—Firmware Version Subelement format |  |  |  |

This Firmware Version field contains an ASCII string identifying the version of firmware currently installed on the wireless network adaptor. This string is not null terminated.

The format for the MAC Address subelement is shown in Figure 7-95o64.

|  | Subelement ID | Length | MAC Address |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 6 |
| Figure 7-95o64—MAC Address Subelement format |  |  |  |

This MAC Address field contains the 6-octet IEEE 802 MAC address of the STA.

The format for the Manufacturer ID String subelement is shown in Figure 7-95o65.

|  | Subelement ID | Length | ID |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | variable |
| Figure 7-95o65—Manufacturer ID String Subelement format |  |  |  |

The ID field contains an ASCII string indicating the manufacturer identifier of the wireless network adaptor. This string is not null terminated.

The format for the Manufacturer Model String subelement is shown in Figure 7-95o66.

|  | Subelement ID | Length | Model |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | variable |
| Figure 7-95o66—Manufacturer Model String Subelement format |  |  |  |

The Model field contains an ASCII string indicating the model of the wireless network adaptor. This string is not null terminated.

The format for the Manufacturer OI subelement is shown in Figure 7-95o67.

|  | Subelement ID | Length | OI |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 3 or 5 |
| Figure 7-95o67—Manufacturer OI Subelement format |  |  |  |

The OI field contains an Organizational Identifier, as defined in 7.3.1.31.

The format for the Manufacturer Serial Number String subelement is shown in Figure 7-95o68.

|  | Subelement ID | Length | Serial Number |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | variable |
| Figure 7-95o68—Manufacturer Serial Number String Subelement format |  |  |  |

The Serial Number field contains an ASCII string indicating the serial number of the wireless network adaptor. This string is not null terminated.

The format for the Power Save Mode subelement is shown in Figure 7-95o69.

|  | Subelement ID | Length | Power Save Mode |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 4 |
| Figure 7-95o69—Power Save Mode Subelement format |  |  |  |

The Power Save Mode field is a bitmap field that indicates the power save mode(s) in use by the STA, as defined in Table 7-43af.

| Table 7-43af—Power Save Mode definition |  |
| --- | --- |
| Power Save Mode | Bit |
| Unknown | 0 |
| None | 1 |
| PS mode (ReceiveDTIMs=1, see 11.2.1) | 2 |
| PS mode (ReceiveDTIMs=0, see 11.2.1) | 3 |
| U-APSD (see 11.2.1.4) | 4 |
| S-APSD (see 11.2.1.4) | 5 |
| U-PSMP (see 9.16) | 6 |
| S-PSMP (see 9.16) | 7 |
| SM Power Save (see 11.2.3) | 8 |
| WNM-Sleep Mode (see 11.2.1.16) | 9 |
| FMS (see 11.2.1.4a) | 10 |
| TIM Broadcast (see 11.2.1.15) | 11 |
| TFS (see 11.22.11) | 12 |
| TDLS Peer U-APSD (see 11.2.1.14) | 13 |
| TDLS Peer PSM (see 11.2.1.13) | 14 |
| Reserved | 15–31 |

The format for the Profile ID subelement is shown in Figure 7-95o70.

|  | Subelement ID | Length | Profile ID |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 |
| Figure 7-95o70— Profile ID Subelement format |  |  |  |

The Profile ID field contains a unique identifier for referencing a configuration profile available on a device. The value of the identifier can be any arbitrary value, as long as it is uniquely associated to a single configuration profile on the device sending the identifier.

The format for the Supported Regulatory Classes subelement is shown in Figure 7-95o71.

|  | Subelement ID | Length | Supported Regulatory Classes Element |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | variable |
| Figure 7-95o71—Supported Regulatory Classes Subelement format |  |  |  |

The Supported Regulatory Classes Element field contains the Supported Regulatory Classes element, as defined in 7.3.2.54.

EDITORIAL NOTE—7.3.2.54 is defined in IEEE 802.11y-2008.

The format for the Status Code subelement is shown in Figure 7-95o72.

|  | Subelement ID | Length | Status Code |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 2 |
| Figure 7-95o72—Status Code Subelement format |  |  |  |

The Status Code field contains the final IEEE 802.11 Status code, as defined in Table 7-23 in 7.3.1.9, received at the end of the applicable operation.

The format for the SSID subelement is shown in Figure 7-95o73.

|  | Subelement ID | Length | SSID |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 0 to 32 |
| Figure 7-95o73—SSID Subelement format |  |  |  |

The SSID field contains a Service Set Identifier with a maximum length of 32 octets, as defined in 7.3.2.1.

The format for the Tx Power Capability subelement is shown in Figure 7-95o74.

|  | Subelement ID | Length | Tx Power Mode | Tx Power |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | variable |
| Figure 7-95o74—Tx Power Capability Subelement format |  |  |  |  |

The Tx Power Mode field identifies the transmit power mode of the non-AP STA and is set to one of the values in Table 7-43ag.

| Table 7-43ag—Tx Power Modes |  |
| --- | --- |
| Tx Power Mode | Value |
| Discrete | 0 |
| Range | 1 |
| Reserved | 2-255 |

The Tx Power field indicates the target transmit power level(s) at the antenna(s), where the actual power is within +/-5dB to the target. Each transmit power level is encoded in a single octet as a 2's complement value in dBm, rounded to the nearest integer. If the Tx Power Mode field is set to 0 then the Tx Power field contains one or more transmit power levels in increasing numerical order. If the Tx Power Mode field is set to 1, the Tx Power field contains the STA's minimum and non-zero maximum transmit power levels, in that order.

The format for the WFA Certificate ID subelement is shown in Figure 7-95o75.

|  | Subelement ID | Length | WFA Certificate ID |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | variable |
| Figure 7-95o75—WFA Certificate ID Subelement format |  |  |  |

The WFA Certificate ID field contains an UTF-8 string indicating the Wi-Fi Alliance assigned identifier for the STA. This string is not null terminated. The WFA Certificate ID typically takes the form of "WFA3991" and can be used by a receiving STA to look up the WFA certificate assigned to that ID using a web lookup url such as "http://certifications.wi-fi.org/pdf_certificate.php?cid= WFA3991”.

The Vendor Specific subelements have the same format as their corresponding elements (see 7.3.2.26).

7.3.2.69 Diagnostic Report element

7.3.2.69.1 Diagnostic Report definition

The Diagnostic Report element contains a Diagnostic report. The format of the Diagnostic Report element is shown in Figure 7-95o76.

|  | Element ID | Length | Diagnostic Token | Diagnostic Report Type | Diagnostic Status | Diagnostic Information Subelements |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 1 | variable |
| Figure 7-95o76—Diagnostic Report element format |  |  |  |  |  |  |

The Element ID field is equal to the Diagnostic Report value in Table 7-26.

The value of the Length field is 3 plus the length of the Diagnostic Information Subelements field. The minimum value of the Length field is 3.

The Diagnostic Token field is set to the Diagnostic Token in the corresponding Diagnostic Request element.

The Diagnostic Report Type field is set to a number that identifies the Diagnostic report. The Diagnostic Report Types are defined in Table 7-43y.

The Diagnostic Status field is set to a value in Table 7-43v (see 7.3.2.67), indicating the STA’s response to the Diagnostic Request indicated by the Diagnostic Token.

The Diagnostic Information Subelements field contains the results of the diagnostic request.

The Diagnostic Information Subelements field contains the Diagnostic Subelement values, defined in 7.3.2.68.6  corresponding to the Diagnostic Report Type field value.

The Diagnostic Report element is included in a Diagnostic Report frame as described in 7.4.12.5. The use of Diagnostic Report element and frames is described in 11.22.3.

7.3.2.69.2 Manufacturer Information STA Report

The contents of the Diagnostic Information subelements field of a Diagnostic Report element of type Manufacturer Information STA Report is shown in Table 7-43ah. The corresponding Diagnostic Information Subelements are defined in 7.3.2.68.6.

| Table 7-43ah— Manufacturer Information STA Report contents |  |
| --- | --- |
| Order | Information Subelement |
| 1 | Manufacturer OI |
| 2 | Manufacturer ID string |
| 3 | Manufacturer model string |
| 4 | Manufacturer serial number string |
| 6 | Firmware Version |
| 7 | Antenna Type |
| 8 | Antenna Gain |
| 9 | Collocated Radio Type |
| 10 | Device Type |
| 11 | WFA Certificate ID |

7.3.2.69.3 Configuration Profile Report

The contents of the Diagnostic Information subelements field of a Diagnostic Report element of type Configuration Profile Report is shown in Table 7-43ai. The corresponding Diagnostic Information Subelements are defined in 7.3.2.68.6.

| Table 7-43ai—Configuration Profile Report contents |  |
| --- | --- |
| Order | Information Subelement |
| 1 | Profile ID |
| 2 | Supported Regulatory Classes |
| 3 | Tx Power |
| 4 | Cipher Suite |
| 5 | AKM Suite |
| 6 | EAP Method |
| 7 | Credential Type |
| 8 | SSID |
| 9 | Power Save Mode |

7.3.2.69.4 Association Diagnostic Report

The contents of the Diagnostic Information subelements field of a Diagnostic Report element of type Association Diagnostic Report is shown in Table 7-43aj. The corresponding Diagnostic Information Subelements are defined in 7.3.2.68.6.

| Table 7-43aj—Association Diagnostic Report contents |  |
| --- | --- |
| Order | Information Subelement |
| 1 | AP Descriptor |
| 2 | Status Code |

7.3.2.69.5 IEEE 802.1X Authentication Diagnostic Report

The contents of the Diagnostic Information subelements field of a Diagnostic Report element of type IEEE 802.1X Authentication Diagnostic Report is shown in Table 7-43ak. The corresponding Diagnostic Information Subelements are defined in 7.3.2.68.6.

| Table 7-43ak—IEEE 802.1X Authentication Diagnostic Report contents |  |
| --- | --- |
| Order | Information Subelement |
| 1 | AP Descriptor |
| 2 | EAP Method |
| 3 | Credential Type |
| 4 | Status Code |

7.3.2.69.6 Firmware Update Notification Report

The contents of the Diagnostic Information subelements field of a Diagnostic Report element of type Firmware Update Notification is shown in Table 7-43xx. The corresponding Diagnostic Information Subelements are defined in 7.3.2.68.6.

| Table 7-43xx—Firmware Update Notification Report contents |  |
| --- | --- |
| Order | Information Subelement |
| 1 | AP Descriptor |
| 2 | Status Code |

7.3.2.69.7 Vendor Specific Diagnostic report

The contents of the Diagnostic Information subelements field of a Diagnostic Report element of type Vendor Specific Diagnostic Report contains zero or more Vendor Specific subelements as defined in 7.3.2.26.

11.22.3 Diagnostic request and report procedures

11.22.3.1 Diagnostic request and diagnostic report

The Diagnostic Request and Diagnostic Report protocol provides a tool to diagnose and debug complex network issues. A STA that has a value of true for the MIB attribute dot11MgmtOptionDiagnosticsEnabled is defined as a STA that supports diagnostics reporting. A STA for which the MIB attribute dot11MgmtOptionDiagnosticEnabled is set to true shall set the Diagnostics field of the Extended Capabilities information element to 1.

A STA that supports diagnostics reporting shall only send a Diagnostics Request or Diagnostics Report frame to a STA within the same infrastructure BSS or the same IBSS whose last received Extended Capabilities information element contained a value of 1 for the Diagnostics bit in the Capabilities field.

The Diagnostic Request frame contains a unique Dialog Token. A Dialog Token value of zero is a reserved value and shall not be used. The source and destination of a Diagnostic Request frame shall both be members of the same BSS. The permitted source and destination STAs for a Diagnostic Request frame are shown in Table 11-14. A STA may include one or more Diagnostic Request elements in a Diagnostic Request frame. Each Diagnostic Request element contains a unique Diagnostic Token that identifies the element within the Diagnostic Request frame.

If a STA that supports diagnostic reporting receives a Diagnostic Request frame without error, the STA shall respond with a Diagnostic Report frame that includes the Dialog Token that matches the one in the Diagnostic Request frame. Each Diagnostic Report element that corresponds to the Diagnostic Request element shall contain the same Diagnostic Token that was included in the original request.

Only a single Diagnostic Request frame from a STA is outstanding at a given STA at any time. If a STA receives a subsequent Diagnostic Request frame with a different Dialog Token before completing the Diagnostic Report for the previous request from the requesting STA, the STA shall only respond to the most recent Request frame. The STA need not send a Diagnostic Report frame for the previous Diagnostic Request frame.

All outstanding diagnostic requests, as indicated by received MLME-DIAGREQUEST.ind primitives, are cancelled upon a BSS transition, except when the BSS transition occurs as a result of responding to or initiating a diagnostic request. All outstanding diagnostic requests, as indicated by received MLME-DIAGREQUEST.indication primitives, are cancelled after the time indicated in the Diagnostic Timeout field, in the Diagnostic Request frame. When a STA that supports diagnostic reporting receives a Diagnostic Request frame with a Diagnostic Request Type of Cancel Diagnostic Request, the STA cancels all outstanding diagnostic requests, and discards all pending diagnostic reports, as indicated by received MLME-DIAGREQUEST.indication primitives.

All Diagnostic Report elements shall include a Status field that indicates the overall result of the transaction. If the STA is able to complete the diagnostic request made in the Diagnostic Request element, then a value of “Successful” shall be returned. If the STA is unable to process the request at that time a value of “Fail” shall be returned. If the STA is incapable of generating a report of the type specified, it shall return a value of “Incapable”. If the STA cannot support the request for any other reason, the value of Refused shall be returned.

A STA shall only transmit both the Diagnostic Request frame and the Diagnostic Report frame with an individually addressed destination address. If a STA that supports diagnostic reporting receives a Diagnostic Request or Report frame with a broadcast or multicast destination address, it shall discard the frame.

If the Diagnostic Report elements do not fit into a single MMPDU, the reporting STA shall send the remaining elements in additional frames until all Diagnostic Report elements have been returned to the requesting STA. A STA shall include the same Dialog Token and Diagnostic Token that was transmitted in the corresponding Diagnostic Request frame in each subsequent Diagnostic Report frame and Diagnostic Report elements. When multiple MMPDUs are required, the STA shall include complete Diagnostic Report elements and shall not fragment an element across multiple MMPDUs.

A STA that supports diagnostic reporting may cancel a previously sent Diagnostic Request frame for which it has not yet received a Diagnostic Report frame by sending a Diagnostic Request frame with the Diagnostic Request Type field value of 0, indicating “Cancelled”, to the STA to which it previously sent the Diagnostic Request frame. A STA that supports diagnostic reporting shall inform a STA from which it has previously received a Diagnostic Request frame that the request has been locally cancelled by sending a Diagnostic Report frame with the Diagnostic Status field set to a value of 4, indicating "Cancelled", to the requesting STA. In a Diagnostic Request frame with the Diagnostic Request Type field value of 0, and a Diagnostic Report frame with the Diagnostic Status field set to a value of 4, no Diagnostic Information subelements are included in the Diagnostic Request or Response element.

11.22.3.2 Configuration profile report

The Configuration Profile report enables an AP to discover the current profile in use for an associated device, and additional profiles for the current ESS. A non-AP STA that supports diagnostic reporting and receives a Configuration Profile report request type shall respond with a Diagnostic Report frame that includes all available Diagnostic Information elements allowed for the type.

Devices that support multiple configuration profiles for an ESS may include multiple Diagnostic Report elements in a single Diagnostic Report frame (or multiple frames if required). Each Diagnostic Report element shall contain a Profile ID element that uniquely identifies the configuration profile(s) for the current ESS that are available on the device.

11.22.3.3 Manufacturer information STA report

The Manufacturer Information STA Report enables an AP to discover static manufacturer specific data about an associated STA device. A non-AP STA that supports diagnostic reporting and receives a Manufacturer Information STA Report request type shall respond with a Diagnostic Report frame that includes all available Diagnostic Information elements allowed for the type.

When more than one Antenna Type, Collocated Radio Type, or Device Type is supported, multiple Antenna Type subelements, Collocated Radio Type subelements, or Device Type subelements shall be included in the Manufacturer Information STA Report Diagnostic Report element. If the existence or the type of collocated radio is unknown, no Collocated Radio Type subelements shall be included.

11.22.3.4 Association diagnostic

The purpose of the association diagnostic is to determine that a STA is able to associate with a designated BSS. This test directs an association to be completed with a specific AP.

To initiate the test, an AP that supports diagnostic reporting shall send a Diagnostic Request frame containing a Diagnostic Request Type field set to 3 (i.e., Association Diagnostic) to a STA that supports diagnostic reporting. The AP shall not send an association diagnostic request with a designated BSS that is not part of the ESS and the STA receiving an association diagnostic request shall reject requests to perform diagnostics on a BSS that is not part of the ESS. All parameters required to complete the association are included in the Diagnostic Request element.

Upon receipt of the Diagnostic Request frame containing a Diagnostic Request element specifying an Association request, the STA determines whether or not to accept the request. If the STA declines the request, it shall send a Diagnostic Report frame with the Status field of a Diagnostic Report element set to Refused. If the STA accepts the request, it shall cause an authentication to occur to the AP indicated in the Diagnostic Request element and the STA's SME shall issue an MLME-DIAGREPORT.request primitive, indicating the results of the diagnostic.

One means to cause an authentication to occur is for the STA's SME to issue an MLME-DEAUTHENTICATE.request primitive to deauthenticate from the current AP, and an MLME-AUTHENTICATE.request primitive to authenticate to the AP indicated in the Diagnostic Request element. If successful, the STA shall issue an MLME-(RE)ASSOCIATE.request to associate to the AP indicated in the Diagnostic Request element. If successful, the STA's SME shall then issue an MLME-DEAUTHENTICATE.request to deauthenticate from the AP indicated in the Diagnostic Request element, reassociate to the AP from which it received the Diagnostic Request, and issue an MLME-DIAGREPORT.request primitive, indicating the results of the diagnostic

11.22.3.5 IEEE 802.1X authentication diagnostic

The purpose of the IEEE 802.1X authentication diagnostic is to determine that the STA is able to complete an IEEE 802.1X authentication with a designated BSS. This test directs an association and IEEE 802.1X authentication to be completed with a specific AP. If a STA that supports diagnostic reporting also supports RSN, the STA shall support the IEEE 802.1X authentication diagnostic.

To initiate the test, an AP that supports diagnostic reporting shall send a Diagnostic Request frame containing a Diagnostic Request Type field set to 4 (i.e. IEEE 802.1X Authentication Diagnostic) to a STA that supports diagnostic reporting. A STA that supports diagnostic reporting in an IBSS or an AP that supports diagnostic reporting shall not send an IEEE 802.1X authentication diagnostic request with a designated BSS that is not part of the ESS, or IBSS and a STA that supports diagnostic reporting which receives a diagnostic request shall reject requests to perform diagnostics on other networks. The AP, EAP method and credential type values included in the AP Descriptor, EAP Method and Credential Type subelements in the Diagnostic Request element shall be used to complete the association and IEEE 802.1X authentication.

Upon receipt of the Diagnostic Request frame containing a Diagnostic Request element specifying an IEEE 802.1X Authentication Diagnostic request, the STA determines whether or not to accept the request. If the STA declines the request, it shall send a Diagnostic Report frame with the Status field of a Diagnostic Report element set to Refused. If the STA accepts the request, it shall cause an IEEE 802.1X authentication to occur to the AP indicated in the Diagnostic Request element and the STA's SME shall issue an MLME-DIAGREPORT.request primitive indicating the results of the diagnostic.

One means to cause an authentication to occur is for the STA's SME to issue an MLME-DEAUTHENTICATE.request to deauthenticate from the current AP, and an MLME-AUTHENTICATE.request to authenticate to the AP indicated in the Diagnostic Request element. If successful, the STA shall issue an MLME-(RE)ASSOCIATE.request to the AP indicated in the Diagnostic Request element. If (re)association succeeds, the STA shall try to complete IEEE 802.1X authentication using parameters specified in the Diagnostic Request element. The STA shall then issue an MLME-DEAUTHENTICATE.request to deauthenticate from the AP indicated in the Diagnostic Request element, would reassociate to the AP from which it received a diagnostic request, and issue an MLME-DIAGREPORT.request primitive, indicating the results of the diagnostic.

11.22.3.6 Firmware Update Notification diagnostic

The Firmware Update Notification diagnostic enables an AP to indicate information about its firmware to an associated STA. Use of the information provided is outside the scope of this standard. A non-AP STA that supports diagnostic reporting and receives a Firmware Update notification request type shall respond with a Diagnostic Report frame that includes all available Diagnostic Information elements allowed for the type.

A value of “Successful” shall be returned to acknowledge that the non-AP STA has received the Firmware Update Notification diagnostic request..

A.4.21 WNM extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| Item | Protocol Capability | References | Status | Support |
| WNM1 | Extended Capabilities information element | 7.3.2.27 | CFv:M | Yes, No, N/A |
| WNM2 | STA Statistics (Triggered) and Multicast Diagnostics | 11.10.7 | CFv:M | Yes, No, N/A |
| WNM2.1 | Protocol for Triggered Measurements | 11.10.7 | CFv:M | Yes, No, N/A |
| WNM2.2 | Triggered STA Statistics | 7.3.2.21.8, 7.3.2.22.8, 7.4.6.1,7.4.6.2, 11.10.8.5 | CFv:O | Yes, No, N/A |
| WNM2.3 | Multicast Diagnostics | 7.3.2.21.12, 7.3.2.22.11, 7.4.6.1,7.4.6.2, 11.20.2 | CFv:M | Yes, No, N/A |
| WNM3 | Event | 11.22.2 | CFv:M | Yes, No, N/A |
| WNM3.1 | Event Request frame | 7.3.2.66, 7.4.12.2 | (CFv AND CF1):M | Yes, No, N/A |
| WNM3.2 | Event Report frame | 7.3.2.67, 7.4.12.3 | (CFv AND CF2):M | Yes, No, N/A |
| WNM4 | Diagnostic | 11.22.3 | CFv:M | Yes, No, N/A |
| WNM4.1 | Diagnostic Request frame | 7.3.2.68, 7.4.12.4 | (CFv AND CF1):M | Yes, No, N/A |
| WNM4.2 | Diagnostic Report frame | 7.3.2.69, 7.4.12.5 | (CFv AND CF2):M | Yes, No, N/A |
| WNM4.3 | Configuration Profile Diagnostic Type | 7.3.2.69.3, 11.22.3.2 | CFv:M | Yes, No, N/A |
| WNM4.4 | Manufacturer Information STA Report Diagnostic Type | 7.3.2.69.2, 11.22.3.3 | CFv:M | Yes, No, N/A |
| WNM4.5 | Association Diagnostic Type | 7.3.2.68.2, 7.3.2.69.4, 11.22.3.4 | CFv:M | Yes, No, N/A |
| WNM4.6 | 802.1X Authentication Diagnostic Type | 7.3.2.68.3, 7.3.2.69.5, 11.22.3.5 | (CFv AND PC34):M | Yes, No, N/A |
| WNM 4.7 | Firmware Update Notification Diagnostic Type | 7.3.2.68.4, 7.3.2.69.6, 11.22.3.6 | CFv:O | Yes, No, N/A |
| WNM5 | Location | 11.22.4, 7.3.2.70 | CFv:M | Yes, No, N/A |
| WNM5.1 | Location Civic Request/Report | 11.10.8.9 | CFv:M | Yes, No, N/A |
| WNM5.2 | Location Identifier Request/Report | 11.10.8.10 | CFv:M | Yes, No, N/A |
| WNM5.3 WNM5.3.1 WNM5.3.2 | Location Track Notification Time Of Departure Notifications Motion Detection Notifications | 11.22.4,7.4.7.10 11.22.4,7.3.2.70 11.22.4,7.3.2.70 | CFv:O CFv:O CFv:O | Yes, No, N/A |
| WNM5.4 WNM5.4.1 WNM5.4.2 | Location Configuration Request frame Normal Indication Motion Indication | 7.4.12.6,7.3.2.70 7.4.12.6,7.3.2.70 7.4.12.6,7.3.2.70 | CFv:M CFv:O CFv:O | Yes, No, N/A Yes, No, N/A Yes, No, N/A |
| WNM5.5 | Location Configuration Response frame | 7.4.12.7,7.3.2.70 | CFv:M | Yes, No, N/A |
| *WNM6 | Multiple BSSID Support | 11.1.2.3a, 11.1.3, 11.10.11 | CFv:O | Yes, No, N/A |
| WNM6.1 | Multiple BSSID element | 7.3.2.46 | WNM6:M | Yes, No, N/A |
| WNM6.2 | Multiple BSSID-index element | 7.3.2.73 | WNM6:M | Yes, No, N/A |
| WNM7 | BSS Transition Management | 11.22.6 | CFv:M | Yes, No, N/A |
| WNM7.1 | Neighbor Report element | 7.3.2.37 | (CFv AND CF1):M | Yes, No, N/A |
| WNM7.2 | BSS Transition Management Query frame | 7.4.12.8 | (CFv AND CF1):M | Yes, No, N/A |
| WNM7.3 | BSS Transition Management Request frame | 7.4.12.9 | (CFv AND CF2):M | Yes, No, N/A |
| WNM7.4 | BSS Transition Management Response frame | 7.4.12.10 | (CFv AND CF2):M | Yes, No, N/A |
| *WNM8 | FMS | 11.2.1.4a | CFv:O | Yes, No, N/A |
| WNM8.1 | FMS Request frame | 7.4.12.11 | (CF2&WNM8):M | Yes, No, N/A |
| WNM8.2 | FMS Response frame | 7.4.12.12 | (CF1&WNM8):M | Yes, No, N/A |
| WNM9 | Proxy ARP | 11.22.13 | CFv:O | Yes, No, N/A |
| *WNM10 | Collocated Interference Reporting | 11.22.9 | CFv:O | Yes, No, N/A |
| WNM10.1 | Collocated Interference Request frame | 7.4.12.13 | WNM10:M | Yes, No, N/A |
| WNM10.2 | Collocated Interference Response frame | 7.4.12.12 | WNM10:M | Yes, No, N/A |
| *WNM11 | BSS Max idle period | 11.22.12 | CFv:M | Yes, No, N/A |
| WNM11.1 | BSS Max Idle Period element | 7.3.2.78 | WNM11:M | Yes, No, N/A |
| *WNM12 | TFS | 11.22.11 | CFv:O | Yes, No, N/A |
| WNM12.1 | TFS Request frame | 7.3.2.79, 7.4.12.15 | WNM12:M | Yes, No, N/A |
| WNM12.2 | TFS Response frame | 7.3.2.80, 7.4.12.16 | WNM12:M | Yes, No, N/A |
| WNM12.3 | TFS Notify frame | 7.4.12.17 | (CF1&WNM12):M, (CF2&WNM12):O | Yes, No, N/A |
| *WNM13 | WNM-Sleep Mode | 11.2.1.16 | WNM12:O | Yes, No, N/A |
| WNM13.1 | WNM-Sleep Mode Request frame | 7.3.2.81, 7.4.12.18 | WNM13:M | Yes, No, N/A |
| WNM13.2 | WNM-Sleep Mode Response frame | 7.3.2.81, 7.4.12.19 | WNM13:M | Yes, No, N/A |
| *WNM14 | TIM Broadcast | 11.2.1.13 | CFv:O | Yes, No. N/A |
| WNM14.1 | TIM Broadcast Request frame | 7.3.2.82, 7.4.12.20, | WNM14:M | Yes, No, N/A |
| WNM14.2 | TIM Broadcast Response frame | 7.3.2.83, 7.4.12.21 | WNM14:M | Yes, No, N/A |
| WNM14.3 | TIM Broadcast frame | 7.4.13.2 | WNM14:M | Yes, No, N/A |
| *WNM15 | QoS Traffic Capability | 11.22.9 | (CFv AND CF2):O | Yes, No, N/A |
| WNM15.1 | QoS Traffic Capability element | 7.3.2.77, | WNM15:M | Yes, No, N/A |
| WNM15.2 | QoS Traffic Capability update frame | 7.4.12.22 | WNM15:M | Yes, No, N/A |
| WNM16 | AC Station Count | 11.22.10 | (CFv AND CF2):O | Yes, No, N/A |
| WNM17 | Timing Measurement | 11.22.5 | CFv:O | Yes, No, N/A |
| *WNM18 | Channel Usage | 11.22.14 | CFv:O | Yes, No, N/A |
| WNM18.1 | Channel Usage Request frame | 7.3.2.85, 7.4.12.23 | WNM18:M | Yes, No, N/A |
| WNM18.2 | Channel Usage Response frame | 7.3.2.85, 7.4.12.24 | WNM18:M | Yes, No, N/A |
| *WNM19 | DMS | 11.22.15 | (CFv AND CF16):O | Yes, No, N/A |
| WNM19.1 | DMS Request frame | 7.3.2.87, 7.4.12.25 | WNM19:M | Yes, No, N/A |
| WNM19.2 | DMS Response frame | 7.3.2.88, 7.4.12.26 | WNM19:M | Yes, No, N/A |
| WNM20 | UTC TSF Offset | 11.21.3, 7.3.2.65, 7.3.2.86 | CFv:O | Yes, No, N/A |

References:

The Wi-Fi Alliance Category definitions may be found at http://www.wi-fi.org/knowledge_center/insist-on-wifi-certified.