IEEE P802.11
Wireless LANs

| Firmware Update Notification Text Proposal |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-02-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Samir Hodoj | T-Mobile (USA) | 12920 SE 38th Street Bellevue, WA 98006 | +1 (425) 383-3301 | Samir.Hodroj@T-Mobile.com |
| Omar Hassan | T-Mobile USA | 12920 SE 38th Street Bellevue, WA 98006 | +1(425) 383-4000 | Omar.hassan@T-Mobile.com |
| Matthew Gast | Trapeze Networks | 5753 W. Las Positas Blvd Pleasanton, CA 94588 | +1 (925) 474-2273 | msg@trapezenetworks.com |
| Dorothy Stanley | Aruba Networks | 1344 Crossman Ave. Sunnyvale, CA 94089 | +1 (630)-363-1389 | DStanley@arubanetworks.com |

Abstract

This document contains the text changes to resolve TGv initial Sponsor Ballot CIDs 99, 229, and recirculation 1 Sponsor Ballot CID 16.

A new Management Notification frame exchange is introduced, to improve manageability of AP devices, initially with a Firmware Update Notification, for use in consumer applications.

Insert the changes indicated into the TGv draft:

1. Overview

1.2 Purpose

Insert the following dash list item at the end of the list of items in 1.2:

—	Defines mechanisms for Wireless Network Management of STAs, including BSS Max idle period management, BSS transition management, channel usage, collocated interference reporting, diagnostic, multicast diagnostic, event reporting, a flexible multicast service, a multiple BSSID advertisement capability, proxy ARP service advertisement, location services, timing measurement service, TIM broadcast service, directed multicast service, WNM-Sleep mode and traffic filtering service, and a WNM notification capability.

5.2.11.1 Overview

Wireless Network Management (WNM) enables STAs to exchange information for the purpose of improving the overall performance of the wireless network. STAs use WNM protocols to exchange operational data so that each STA is aware of the network conditions, allowing STAs to be more cognizant of the topology and state of the network. WNM protocols provide a means for STAs to be aware of the presence of collocated interference, and enable STAs to manage RF parameters based on network conditions.

In addition to providing information on network conditions, WNM also provides a means to exchange location information, provide support for the multiple BSSID advertisement capability on the same wireless infrastructure, support efficient delivery of group addressed frames, and enable a WNM-Sleep mode in which a STA can sleep for long periods of time without receiving frames from the AP.

The WNM service includes:

—	BSS Max idle period management

—	BSS transition management

ﾑ	Channel usage

—	Collocated interference reporting

—	Diagnostic reporting

—	Directed Multicast Service (DMS)

—	Event reporting

—	Flexible Multicast Service (FMS)

—	Location services

—	Multicast diagnostic reporting

—	Multiple BSSID advertisement capability

—	Proxy ARP

—	QoS traffic capability

—	SSID list

—	Triggered STA statistics

—	TIM broadcast

—	Timing measurement

—	Traffic filtering service

—	WNM-Sleep mode

—	WNM Notification

5.2.11.20 WNM Notification

WNM Notification provides a mechanism for a STA to notify a peer STA of a management event. One event is defined, firmware update notification.

7.3.2.27 Extended Capabilities information element

Insert the following Bits and change the Reserved row in Table 7-35a as follows (note that the entire table is not shown here):

| Table 7-35a—Capabilities field |  |  |
| --- | --- | --- |
| Bit(s) | Information | Notes |
| 7 | Event | The STA sets the Event field to 1 when the MIB attribute dot11MgmtOptionEventsEnabled is true, and sets it to 0 otherwise. See 11.22.2. |
| 8 | Diagnostics | The STA sets the Diagnostics field to 1 when the MIB attribute dot11MgmtOptionDiagnosticsEnabled is true, and sets it to 0 otherwise. See 11.22.3. |
| 9 | Multicast Diagnostics | The STA sets the Multicast Diagnostics field to 1 when the MIB attribute dot11MgmtOptionMulticastDiagnosticsEnabled is true, and sets it to 0 otherwise. See 11.22.2. |
| 10 | Location Tracking | The STA sets the Location Tracking field to 1 when the MIB attribute dot11MgmtOptionLocationEnabled is true, and sets it to 0 otherwise. See 11.22.4. |
| 11 | FMS | The STA sets the FMS field to 1 when the MIB attribute dot11MgmtOptionFMSEnabled is true, and sets it to 0 otherwise. See 11.2.1.4a and 11.22.7. |
| 12 | Proxy ARP Service | The AP sets the Proxy ARP Service field to 1 when the MIB attribute dot11MgmtOptionProxyARPEnabled is true, and sets it to 0 otherwise. See 11.22.13. A non-AP STA sets the Proxy ARP Service field to 0. |
| 13 | Collocated Interference Reporting | The STA sets the Collocated Interference Reporting field to 1 when the MIB attribute dot11MgmtOptionCoLocIntfReportingEnabled is true, and sets it to 0 otherwise. See 11.22.9. |
| 14 | CIVIC Location | The STA sets the CIVIC Location field to 1 when the MIB attribute dot11RRMCivicMeasurementEnabled is true, and sets it to 0 otherwise. See 11.10.8.9. |
| 15 | Geo Location | The STA sets the Geo Location field to 1 when the MIB attribute dot11RRMLCIMeasurementEnabled is true, and sets it to 0 otherwise. See 11.10.8.6. |
| 16 | TFS | The STA sets the TFS field to 1 when the MIB attribute dot11MgmtOptionTFSEnabled is true, and sets it to 0 otherwise. See 11.22.11. |
| 17 | WNM-Sleep Mode | The STA sets the WNM-Sleep Mode field to 1 when the MIB attribute dot11MgmtOptionWNMSleepModeEnabled is true, and sets it to 0 otherwise. See 11.2.1.16. |
| 18 | TIM Broadcast | The STA sets the TIM Broadcast field to 1 when the MIB attribute dot11MgmtOptionTIMBroadcastEnabled is true, and sets it to 0 otherwise. See 11.2.1.15. |
| 19 | BSS Transition | The STA sets the BSS Transition field to 1 when the MIB attribute dot11MgmtOptionBSSTransitionEnabled is true, and sets it to 0 otherwise. See 11.22.6. |
| 20 | QoS Traffic Capability | The STA sets the QoS Traffic Capability field to 1 when the MIB attribute dot11MgmtOptionQoSTrafficCapabilityEnabled is true, and sets it to 0 otherwise. See 11.22.9. |
| 21 | AC Station Count | The STA sets the AC Station Count field to 1 when the MIB attribute dot11MgmtOptionACStationCountEnabled is true, and sets it to 0 otherwise. See 11.22.10. |
| 22 | Multiple BSSID | The STA sets the Multiple BSSID field to 1 when the MIB attribute dot11MgmtOptionMultiBSSEDEnabled is true, and sets it to 0 otherwise. See 11.10.11 and 11.1.2.3a. |
| 23 | Timing Measurement | The STA sets the Timing Measurement field to 1 when the MIB attribute dot11MgmtOptionTimingMsmtEnabled is true, and sets it to 0 otherwise. See 11.22.5. |
| 24 | Channel Usage | The STA sets the Channel Usage field to 1 when the MIB attribute dot11MgmtOptionChannelUsageEnabled is true and sets it to 0 otherwise. See 11.22.14. |
| 25 | SSID List | The STA sets the SSID List field to 1 when the MIB attributedot11MgmtOptionSSIDListEnabled is true, and sets it to 0 otherwise. See 11.1.3. |
| 26 | DMS | The STA sets the DMS field to 1 when the MIB attribute dot11MgmtOptionDMSEnabled is true and sets it to 0 otherwise. See 11.22.15. |
| 27 | UTC TSF Offset | The STA sets the UTC TSF Offset field to 1 when the MIB attribute dot11MgmtOptionUTCTSFOffsetEnabled is true and sets it to 0 otherwise. See 11.21.3. |
| 28 | WNM Notification | The STA sets the WNM Notification field to 1 when the MIB attribute dot11MgmtOptionWNMNotificationEnabled is true and sets it to 0 otherwise. See 11.22.xx. |
| 29 — n | Reserved | All other bits are reserved, and are set to 0 on transmission and ignored on reception. |

7.4.12.1 WNM Action fields

Several Action frame formats are defined for Wireless Network Management (WNM) purposes. An Action

field, in the octet field immediately after the Category field, differentiates the formats. The Action field values associated with each frame format are defined in Table 7-57ac.

Table 7-57ac—WNM Action field values

| Action field value | Description |  |
| --- | --- | --- |
| 0 | Event Request |  |
| 1 | Event Report |  |
| 2 | Diagnostic Request |  |
| 3 | Diagnostic Report |  |
| 4 | Location Configuration Request |  |
| 5 | Location Configuration Response |  |
| 6 | BSS Transition Management Query |  |
| 7 | BSS Transition Management Request |  |
| 8 | BSS Transition Management Response |  |
| 9 | FMS Request |  |
| 10 | FMS Response |  |
| 11 |  | Collocated Interference Request |
| 12 |  | Collocated Interference Response |
| 13 |  | TFS Request |
| 14 |  | TFS Response |
| 15 |  | TFS Notify |
| 16 |  | WNM-Sleep Mode Request |
| 17 |  | WNM-Sleep Mode Response |
| 18 |  | TIM Broadcast Request |
| 19 |  | TIM Broadcast Response |
| 20 |  | QoS Traffic Capability Update |
| 21 |  | Channel Usage Request |
| 22 |  | Channel Usage Response |
| 23 |  | DMS Request |
| 24 |  | DMS Response |
| 25 |  | WNM Notification Request |
| 26 |  | WNM Notification Response |
| 27-255 |  | Reserved |

- Add the following subsection into the draft:

- 7.4.12.xx WNM Notification Request Frame format

| - Category | - Action | - Dialog Token | - Type | - Optional Subelements |
| --- | --- | --- | --- | --- |

octets:			1		1   	 1	      1	         variable

Figure 7-101xyx

The Dialog Token field is set to a nonzero value chosen by the STA sending the WNM-Notification request to identify the request/response transaction.

- The Category field is set to the value indicating the WNM category, as specified in Table 7-24 in 7.3.1.11.

- The Action field is set to the value indicating WNM Notification Request frame, as specified in Table 7-57ac in 7.4.12.1.

The Type field indicates the type of WNM Notification, as defined in  Table 7-43xx.

| Table 7-43xx—WNM Notification Type |  |
| --- | --- |
| Value | Description |
| 0 | Firmware Update Notification |
| 1-255 | Reserved |

The Optional Subelements field format contains zero or more subelements, each consisting of a one octet Subelement ID field, a one octet Length field and a variable length Data field, as shown in Figure 7-95p. The optional subelements are ordered by non-decreasing Subelement ID.

The Subelement ID field values for the defined optional subelements are shown in Table 7-wxyz. A Yes in the Extensible column of a subelement listed in Table 7-wxyz indicates that the length of the subelement might be extended in future revisions or amendments of this standard. When the Extensible column of an element is Subelement, then the subelement might be extended in future revisions or amendments of this standard by defining additional subelements within the subelement. See 9.14.2.

| Table 7-wxyz – WNM Notification Optional Subelements |  |  |  |  |
| --- | --- | --- | --- | --- |
| Subelement ID | Name | Length field (octets) | Extensible |  |
| 0 | AP Descriptor | 10 |  |  |
| 1 | Firmware Version - Current | 3-251 |  |  |
| 2 | Firmware Version - New | 3-251 |  |  |
| 3-220 | Reserved |  |  |  |
| 221 | Vendor Specific | 1 to 239 |  |  |
| 222-255 | Reserved |  |  |  |

When the Type field is 0, the AP Descriptor, Firmware Version-Current and Firmware Version –New optional subelements are included in the WNM Notification Request frame. The AP descriptor field format is as defined in Table 7-43ab. The Firmware Version-Current subelement contains the current firmware version, in the Firmware Version subelement format defined in Table 7-43ab. The Firmware Version-New subelement contains the new firmware version, in the Firmware Version subelement format defined in Table 7-43ab.

The Vendor Specific subelements have the same format as their corresponding elements (see 7.3.2.26). Multiple Vendor Specific subelements can be included in the list of Optional Subelements.

- 7.4.12.xy WNM Notification Response Frame format

| - Category | - Action | - Dialog Token | - Response Status | - Optional Subelements |
| --- | --- | --- | --- | --- |

octets:			1		1	   1		1	       variable

Figure 7-101xyx

- The Category field is set to the value indicating the WNM category, as specified in Table 7-24 in 7.3.1.11.

- The Action field is set to the value indicating WNM Notification Response frame, as specified in Table 7-57ac in 7.4.12.1.

The Dialog Token field is set to the nonzero value of the corresponding WNM Notification Request frame. If the WNM Notification Response frame is being transmitted other than in response to a WNM Notification Request frame, then the Dialog token is set to zero.

The Response Status field indicates the Response Status value, as defined in  Table 7-43xz.

| Table 7-43xz—WNM Notification Response Status |  |
| --- | --- |
| Value | Description |
| 0 | Notification Acknowledged |
| 1-255 | Reserved |

The Optional Subelements field format contains zero or more subelements, each consisting of a one octet Subelement ID field, a one octet Length field and a variable length Data field, as shown in Figure 7-95p. The optional subelements are ordered by non-decreasing Subelement ID.

The Subelement ID field values for the defined optional subelements are shown in Table 7-xyz. A Yes in the Extensible column of a subelement listed in Table 7-xyz indicates that the length of the subelement might be extended in future revisions or amendments of this standard. When the Extensible column of an element is Subelement, then the subelement might be extended in future revisions or amendments of this standard by defining additional subelements within the subelement. See 9.14.2.

| Table 7-xyz—Optional Subelements for WNM Notification Response |  |  |  |  |
| --- | --- | --- | --- | --- |
| Subelement ID |  | Name | Length field (octets) | Extensible |
| 0-220 |  | Reserved |  |  |
| 221 |  | Vendor Specific | 1 to 239 |  |
| 222-255 |  | Reserved |  |  |

The Vendor Specific subelements have the same format as their corresponding elements (see 7.3.2.26). Multiple Vendor Specific subelements can be included in the list of Optional Subelements.

10.3.xx WNM Notification request

This set of primitives supports the exchange of WNM Notification Request and Response frames between peer SMEs.

10.3.xx.1 MLME-WNMNOTIFICATIONREQUEST.request

10.3.xx.1.1 Function

This primitive requests the transmission of a WNM Notification Request frame to a peer entity.

10.3.xx.1.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-WNMNOTIFICATIONREQUEST.request	(
Peer MAC Address,
Dialog Token

Type,
Optional Subelements)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual MAC Address | The address of the peer MAC entity to which the WNM Notification Request frame is sent. |
| Dialog Token | Integer | 1-255 | The dialog token to identify the WNM notification transaction. |
| Type | Integer | 1 – 255 | The type of WNM notification. |
| Optional Subelements | Set subelements | Set of sub- elements | A set of sub-elements describing the notification. |

10.3.xx.1.3 When generated

This primitive is generated by the SME to request that a WNM Notification Request frame be sent to a peer entity to initiate a WNM notification.

10.3.xx.1.4 Effect of receipt

On receipt of this primitive, the MLME constructs a WNM Notification Request frame containing the elements specified. This frame is then scheduled for transmission.

10.3.xx.2 MLME- WNMNOTIFICATIONREQUEST.confirm

10.3.xx.2.1 Function

This primitive reports the result of the WNM Notification request.

10.3.xx.2.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME- WNMNOTIFICATIONREQUEST.confirm	(
ResultCode)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, TRANSMISSION FAILURE, INVALID PARAMETERS, or UNSPECIFIED FAILURE | Reports the outcome of a request to send a WNM Notification Request frame. |

10.3.xx.2.3 When generated

This primitive is generated by the MLME when transmission of the WNM Notification Request frame is acknowledged, (re) transmission of the WNM Notification Request frame fails, the WNM Notification Request frame contains invalid parameters, or for unspecified failure reasons.

10.3.xx.2.4 Effect of receipt

On receipt of this primitive, the SME evaluates the result code.

10.3.xx.3 MLME- WNMNOTIFICATIONREQUEST indication

10.3.xx.3.1 Function

This primitive indicates that a WNM Notification Request frame has been received.

10.3.xx.3.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME- WNMNOTIFICATIONREQUEST.indication	(
Peer MAC Address,
Dialog Token

Type,
Optional Subelements)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual MAC Address | The address of the peer MAC entity from which the WNM Notification request was received. |
| Dialog Token | Integer | 1 – 255 | The dialog token to identify the WNM notification transaction. |
| Type | Integer | 1 – 255 | The type of WNM Notification Request |
| Optional Subelements | Set ofsub- elements | Set of sub- elements | A set of sub-elements comprising the notification. |

10.3.xx.3.3 When generated

This primitive is generated by the MLME when a valid WNM Notification Request frame is received.

10.3.xx.3.4 Effect of receipt

On receipt of this primitive, the SME replies to the request.

10.3.xy WNM Notification response

10.3.xy.1 MLME-WNMRESPONSE.request

10.3.xy.1.1 Function

This primitive supports the signaling of the WNM Notification Response frame between peer SMEs.

10.3.xy.1.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME- WNMNOTIFICATIONRESPONSE.request	(
Peer MAC Address,
Dialog Token

Response Status,
Optional Subelements)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MAC Address | Any valid individual MAC Address | The address of the peer MAC entity to which the WNM Notification Reponse frame is sent. |
| Dialog Token | Integer | 1 – 255 | The dialog token to identify the WNM notification transaction. |
| Response Status | Integer | 1 – 255 | The response status of the WNM notification. |
| Optional Subelements | Set of subements. | Set of subelements | A set of subelements describing the results of the WNM notification. |

10.3.xy.1.3 When generated

This primitive is generated by the SME to request that a WNM Notification Response frame be sent to a peer entity to report the results of the WNM notification.

10.3.xy.1.4 Effect of receipt

On receipt of this primitive, the MLME constructs a WNM Notification Response frame containing the indicated fields. This frame is then scheduled for transmission.

10.3.xy.2 MLME- WNMRESPONSE.confirm

10.3.xy.2.1 Function

This primitive reports the result of the requested transmission of the WNM Notification Response frame frame.

10.3.xy.2.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME- WNMNOTIFICATIONRESPONSE.confirm	(,
ResultCode)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID PARAMETERS, or UNSPECIFIED FAILURE | Reports the outcome of a request to send a WNM Notification Response frame. |

10.3.xy.2.3 When generated

This primitive is generated by the MLME when the request to transmit a WNM Notification Response frame completes.

10.3.xy.2.4 Effect of receipt

On receipt of this primitive, the SME evaluates the result code.

10.3.xy.3 MLME-WNMNOTIFICATIONRESPONSE.indication

10.3.xy.3.1 Function

This primitive indicates that a WNM Notification Response frame has been received from a peer entity.

10.3.xy.3.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME- WNMNOTIFICATIONRESPONSE.indication	(
Peer MAC Address,
Dialog Token,
Reponse Status

Optional Subelements)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual MAC Address | The address of the peer MAC entity from which the WNM-Notification Response frame was received. |
| Dialog Token | Integer | 1 – 255 | The dialog token to identify the WNM notification transaction. |
| Response Status | Integer | 1 – 255 | The response status of the WNM notification. |
| Optional Subelements | Set of subements. | Set of subelements | A set of subelements describing the results of the WNM notification. |

10.3.xy.3.3 When generated

This primitive is generated by the MLME when a valid WNM Notification Response frame is received.

10.3.xy.3.4 Effect of receipt

On receipt of this primitive, the WNM notification can be made available for SME processes.

11.22.xx WNM Notification

Implementation of the WNM Notification capability is optional for a WNM STA. A STA that implements the WNM Notification capability has the MIB attribute dot11MgmtOptionWNMNotificationImplemented set to true. When dot11MgmtOptionWNMImplemented is true, dot11WirelessManagementImplemented shall be true. A STA that has a value of true for the MIB attribute dot11MgmtOptionWNMNotificationEnabled is defined as a STA that supports WNM Notification. A STA for which the MIB attribute dot11MgmtOptionWNMNotificationEnabled is set to true shall set the WNM Enabled field of the Extended Capabilities information element to 1.

A STA that supports WNM Notification reporting shall only send a WNM Notification Request or Response frame to a STA within the same infrastructure BSS or the same IBSS whose last received Extended Capabilities information element contained a value of 1 for the WNM Notification bit in the Capabilities field.

A STA shall only transmit both the WNM Notification Request frame and the WNM Notification Response frame with an individually addressed destination address.

The WNM Notification capability enables a STA to indicate management information, including information about its firmware to a peer STA. Use of the information provided is outside the scope of this standard. A non-AP STA that supports WNM Notification and receives a WNM Notification request frame with the Type field set to 0 shall respond with a WNM Notification Response frame with the Response Status field set to 0.

A.4.21 WNM extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| Item | Protocol Capability | References | Status | Support |
| WNM1 | Extended Capabilities information element | 7.3.2.27 | CFv:M | Yes, No, N/A |
| WNM2 | STA Statistics (Triggered) and Multicast Diagnostics | 11.10.7 | CFv:M | Yes, No, N/A |
| WNM2.1 | Protocol for Triggered Measurements | 11.10.7 | CFv:M | Yes, No, N/A |
| WNM2.2 | Triggered STA Statistics | 7.3.2.21.8, 7.3.2.22.8, 7.4.6.1,7.4.6.2, 11.10.8.5 | CFv:O | Yes, No, N/A |
| WNM2.3 | Multicast Diagnostics | 7.3.2.21.12, 7.3.2.22.11, 7.4.6.1,7.4.6.2, 11.20.2 | CFv:M | Yes, No, N/A |
| WNM3 | Event | 11.22.2 | CFv:M | Yes, No, N/A |
| WNM3.1 | Event Request frame | 7.3.2.66, 7.4.12.2 | (CFv AND CF1):M | Yes, No, N/A |
| WNM3.2 | Event Report frame | 7.3.2.67, 7.4.12.3 | (CFv AND CF2):M | Yes, No, N/A |
| WNM4 | Diagnostic | 11.22.3 | CFv:M | Yes, No, N/A |
| WNM4.1 | Diagnostic Request frame | 7.3.2.68, 7.4.12.4 | (CFv AND CF1):M | Yes, No, N/A |
| WNM4.2 | Diagnostic Report frame | 7.3.2.69, 7.4.12.5 | (CFv AND CF2):M | Yes, No, N/A |
| WNM4.3 | Configuration Profile Diagnostic Type | 7.3.2.69.3, 11.22.3.2 | CFv:M | Yes, No, N/A |
| WNM4.4 | Manufacturer Information STA Report Diagnostic Type | 7.3.2.69.2, 11.22.3.3 | CFv:M | Yes, No, N/A |
| WNM4.5 | Association Diagnostic Type | 7.3.2.68.2, 7.3.2.69.4, 11.22.3.4 | CFv:M | Yes, No, N/A |
| WNM4.6 | 802.1X Authentication Diagnostic Type | 7.3.2.68.3, 7.3.2.69.5, 11.22.3.5 | (CFv AND PC34):M | Yes, No, N/A |
| WNM5 | Location | 11.22.4, 7.3.2.70 | CFv:M | Yes, No, N/A |
| WNM5.1 | Location Civic Request/Report | 11.10.8.9 | CFv:M | Yes, No, N/A |
| WNM5.2 | Location Identifier Request/Report | 11.10.8.10 | CFv:M | Yes, No, N/A |
| WNM5.3 WNM5.3.1 WNM5.3.2 | Location Track Notification Time Of Departure Notifications Motion Detection Notifications | 11.22.4,7.4.7.10 11.22.4,7.3.2.70 11.22.4,7.3.2.70 | CFv:O CFv:O CFv:O | Yes, No, N/A |
| WNM5.4 WNM5.4.1 WNM5.4.2 | Location Configuration Request frame Normal Indication Motion Indication | 7.4.12.6,7.3.2.70 7.4.12.6,7.3.2.70 7.4.12.6,7.3.2.70 | CFv:M CFv:O CFv:O | Yes, No, N/A Yes, No, N/A Yes, No, N/A |
| WNM5.5 | Location Configuration Response frame | 7.4.12.7,7.3.2.70 | CFv:M | Yes, No, N/A |
| *WNM6 | Multiple BSSID Support | 11.1.2.3a, 11.1.3, 11.10.11 | CFv:O | Yes, No, N/A |
| WNM6.1 | Multiple BSSID element | 7.3.2.46 | WNM6:M | Yes, No, N/A |
| WNM6.2 | Multiple BSSID-index element | 7.3.2.73 | WNM6:M | Yes, No, N/A |
| WNM7 | BSS Transition Management | 11.22.6 | CFv:M | Yes, No, N/A |
| WNM7.1 | Neighbor Report element | 7.3.2.37 | (CFv AND CF1):M | Yes, No, N/A |
| WNM7.2 | BSS Transition Management Query frame | 7.4.12.8 | (CFv AND CF1):M | Yes, No, N/A |
| WNM7.3 | BSS Transition Management Request frame | 7.4.12.9 | (CFv AND CF2):M | Yes, No, N/A |
| WNM7.4 | BSS Transition Management Response frame | 7.4.12.10 | (CFv AND CF2):M | Yes, No, N/A |
| *WNM8 | FMS | 11.2.1.4a | CFv:O | Yes, No, N/A |
| WNM8.1 | FMS Request frame | 7.4.12.11 | (CF2&WNM8):M | Yes, No, N/A |
| WNM8.2 | FMS Response frame | 7.4.12.12 | (CF1&WNM8):M | Yes, No, N/A |
| WNM9 | Proxy ARP | 11.22.13 | CFv:O | Yes, No, N/A |
| *WNM10 | Collocated Interference Reporting | 11.22.9 | CFv:O | Yes, No, N/A |
| WNM10.1 | Collocated Interference Request frame | 7.4.12.13 | WNM10:M | Yes, No, N/A |
| WNM10.2 | Collocated Interference Response frame | 7.4.12.12 | WNM10:M | Yes, No, N/A |
| *WNM11 | BSS Max idle period | 11.22.12 | CFv:M | Yes, No, N/A |
| WNM11.1 | BSS Max Idle Period element | 7.3.2.78 | WNM11:M | Yes, No, N/A |
| *WNM12 | TFS | 11.22.11 | CFv:O | Yes, No, N/A |
| WNM12.1 | TFS Request frame | 7.3.2.79, 7.4.12.15 | WNM12:M | Yes, No, N/A |
| WNM12.2 | TFS Response frame | 7.3.2.80, 7.4.12.16 | WNM12:M | Yes, No, N/A |
| WNM12.3 | TFS Notify frame | 7.4.12.17 | (CF1&WNM12):M, (CF2&WNM12):O | Yes, No, N/A |
| *WNM13 | WNM-Sleep Mode | 11.2.1.16 | WNM12:O | Yes, No, N/A |
| WNM13.1 | WNM-Sleep Mode Request frame | 7.3.2.81, 7.4.12.18 | WNM13:M | Yes, No, N/A |
| WNM13.2 | WNM-Sleep Mode Response frame | 7.3.2.81, 7.4.12.19 | WNM13:M | Yes, No, N/A |
| *WNM14 | TIM Broadcast | 11.2.1.13 | CFv:O | Yes, No. N/A |
| WNM14.1 | TIM Broadcast Request frame | 7.3.2.82, 7.4.12.20, | WNM14:M | Yes, No, N/A |
| WNM14.2 | TIM Broadcast Response frame | 7.3.2.83, 7.4.12.21 | WNM14:M | Yes, No, N/A |
| WNM14.3 | TIM Broadcast frame | 7.4.13.2 | WNM14:M | Yes, No, N/A |
| *WNM15 | QoS Traffic Capability | 11.22.9 | (CFv AND CF2):O | Yes, No, N/A |
| WNM15.1 | QoS Traffic Capability element | 7.3.2.77, | WNM15:M | Yes, No, N/A |
| WNM15.2 | QoS Traffic Capability update frame | 7.4.12.22 | WNM15:M | Yes, No, N/A |
| WNM16 | AC Station Count | 11.22.10 | (CFv AND CF2):O | Yes, No, N/A |
| WNM17 | Timing Measurement | 11.22.5 | CFv:O | Yes, No, N/A |
| *WNM18 | Channel Usage | 11.22.14 | CFv:O | Yes, No, N/A |
| WNM18.1 | Channel Usage Request frame | 7.3.2.85, 7.4.12.23 | WNM18:M | Yes, No, N/A |
| WNM18.2 | Channel Usage Response frame | 7.3.2.85, 7.4.12.24 | WNM18:M | Yes, No, N/A |
| *WNM19 | DMS | 11.22.15 | (CFv AND CF16):O | Yes, No, N/A |
| WNM19.1 | DMS Request frame | 7.3.2.87, 7.4.12.25 | WNM19:M | Yes, No, N/A |
| WNM19.2 | DMS Response frame | 7.3.2.88, 7.4.12.26 | WNM19:M | Yes, No, N/A |
| WNM20 | UTC TSF Offset | 11.21.3, 7.3.2.65, 7.3.2.86 | CFv:O | Yes, No, N/A |
| WNM21 | WNM Notification | 11.22.xx | (CFv AND CF16):O | Yes, No, N/A |
| WNM21.1 | WNM Management Notification Request frame | 7.4.12.xx | WNM19:M | Yes, No, N/A |
| WNM21.2 | WNM Management Notification Response frame | 7.4.12.xx | WNM19:M | Yes, No, N/A |

Insert the following new MIB entries, also inserting entries into the Dot11WirelessMgmtOptionsEntry table.

dot11MgmtOptionWNMNotificationImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the station implementation is capable of supporting WNM Notification when the dot11WirelessManagementImplemented is set to TRUE."

DEFVAL { false}

::= { dot11WirelessMgmtOptionsEntry x }

dot11MgmtOptionWNMNotificationEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the capability of the station to provide WNM Notification is enabled. The capability is disabled, otherwise"

DEFVAL { false}

::= { dot11WirelessMgmtOptionsEntry y }

Additional suggested changes to the base spec:

1. P221, L38, change from

“A STA shall transmit both the Event Request frame and the Event Report frame only with an individually

addressed destination address. If a non-AP STA that supports event reporting receives an Event Request

frame with a multicast destination address, it shall discard the frame. “

to

“A STA shall transmit both the Event Request frame and the Event Report frame only with an individually

addressed destination address.

2. P225, L19, Change from

“A STA shall only transmit both the Diagnostic Request frame and the Diagnostic Report frame with an individually addressed destination address. If a STA that supports diagnostic reporting receives a Diagnostic Request or Report frame with a broadcast or multicast destination address, it shall discard the frame. “

To

“A STA shall only transmit both the Diagnostic Request frame and the Diagnostic Report frame with an individually addressed destination address.

3. Throughout clause 10, the .indication primitives, MAC address entry descriptions, change as below:

P145, L26, P147,L21, change from “The address of the peer MAC entity to which the

event request shall be sent.”

To “The address of the peer MAC entity from which the

event request was received.”

P147, L21, change from 

The address of the peer MAC entity to which the

event report shall be sent. 
to

“The address of the peer MAC entity from which the

event report was received. “

Similarly for P151, L23, P152, L19, P156, L20, P158, L48, P175, L54, and P177, L51

4. In Clause 10, P149, L63 and similar, change from

“to which the

diagnostic request shall be sent.” To

“to which the

diagnostic request is sent.”

References: