IEEE P802.11
Wireless LANs

| TGv Draft 7.04 Mib compilation results |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-01-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Paul Gray | Aruba Networks | 1322 Crossman Ave Sunnycale, CA 94089 |  | PGray@arubanetworks.com |

Abstract

This document contains the MIB compilation results for TGv Draft 7.04.

Annex A

(normative)

Protocol Implementation Conformance Statement (PICS) proforma

A.2 Abbreviations and special symbols

A.2.2 General abbreviations for Item and Support columns

Insert one new list item at the end of A.2.2 as indicated below:

WNM	wireless network management

A.4 PICS proforma–IEEE Std. 802.11-2007

A.4.3 IUT configuration

Insert this entry to the end of the IUT configuration table:

| Item | IUT configuration | References | Status | Support |
| --- | --- | --- | --- | --- |
| *CFv | Is WNM supported? |  | (CF8 & CF11 & CF13 & CF15 & DSE5 & DSE6 & DSE7 & DSE8 & DSE9 ):O | Yes, No, N/A |

Insert this new clause after A.4.20:

EDITORIAL NOTE—last clause in IEEE 802.11k-2008 is A.4.17.

A.4.21 WNM extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| Item | Protocol Capability | References | Status | Support |
| WNM1 | Extended Capabilities information element | 7.3.2.27 | CFv:M | Yes, No, N/A |
| WNM2 | STA Statistics (Triggered) and Multicast Diagnostics | 11.10.7 | CFv:M | Yes, No, N/A |
| WNM2.1 | Protocol for Triggered Measurements | 11.10.7 | CFv:M | Yes, No, N/A |
| WNM2.2 | Triggered STA Statistics | 7.3.2.21.8, 7.3.2.22.8, 7.4.6.1,7.4.6.2, 11.10.8.5 | CFv:O | Yes, No, N/A |
| WNM2.3 | Multicast Diagnostics | 7.3.2.21.12, 7.3.2.22.11, 7.4.6.1,7.4.6.2, 11.20.2 | CFv:M | Yes, No, N/A |
| WNM3 | Event | 11.22.2 | CFv:M | Yes, No, N/A |
| WNM3.1 | Event Request frame | 7.3.2.66, 7.4.12.2 | (CFv AND CF1):M | Yes, No, N/A |
| WNM3.2 | Event Report frame | 7.3.2.67, 7.4.12.3 | (CFv AND CF2):M | Yes, No, N/A |
| WNM4 | Diagnostic | 11.22.3 | CFv:M | Yes, No, N/A |
| WNM4.1 | Diagnostic Request frame | 7.3.2.68, 7.4.12.4 | (CFv AND CF1):M | Yes, No, N/A |
| WNM4.2 | Diagnostic Report frame | 7.3.2.69, 7.4.12.5 | (CFv AND CF2):M | Yes, No, N/A |
| WNM4.3 | Configuration Profile Diagnostic Type | 7.3.2.69.3, 11.22.3.2 | CFv:M | Yes, No, N/A |
| WNM4.4 | Manufacturer Information STA Report Diagnostic Type | 7.3.2.69.2, 11.22.3.3 | CFv:M | Yes, No, N/A |
| WNM4.5 | Association Diagnostic Type | 7.3.2.68.2, 7.3.2.69.4, 11.22.3.4 | CFv:M | Yes, No, N/A |
| WNM4.6 | 802.1X Authentication Diagnostic Type | 7.3.2.68.3, 7.3.2.69.5, 11.22.3.5 | (CFv AND PC34):M | Yes, No, N/A |
| WNM5 | Location | 11.22.4, 7.3.2.70 | CFv:M | Yes, No, N/A |
| WNM5.1 | Location Civic Request/Report | 11.10.8.9 | CFv:M | Yes, No, N/A |
| WNM5.2 | Location Identifier Request/Report | 11.10.8.10 | CFv:M | Yes, No, N/A |
| WNM5.3 WNM5.3.1 WNM5.3.2 | Location Track Notification Time Of Departure Notifications Motion Detection Notifications | 11.22.4,7.4.7.10 11.22.4,7.3.2.70 11.22.4,7.3.2.70 | CFv:O CFv:O CFv:O | Yes, No, N/A |
| WNM5.4 WNM5.4.1 WNM5.4.2 | Location Configuration Request frame Normal Indication Motion Indication | 7.4.12.6,7.3.2.70 7.4.12.6,7.3.2.70 7.4.12.6,7.3.2.70 | CFv:M CFv:O CFv:O | Yes, No, N/A Yes, No, N/A Yes, No, N/A |
| WNM5.5 | Location Configuration Response frame | 7.4.12.7,7.3.2.70 | CFv:M | Yes, No, N/A |
| *WNM6 | Multiple BSSID Support | 11.1.2.3a, 11.1.3, 11.10.11 | CFv:O | Yes, No, N/A |
| WNM6.1 | Multiple BSSID element | 7.3.2.46 | WNM6:M | Yes, No, N/A |
| WNM6.2 | Multiple BSSID-index element | 7.3.2.73 | WNM6:M | Yes, No, N/A |
| WNM7 | BSS Transition Management | 11.22.6 | CFv:M | Yes, No, N/A |
| WNM7.1 | Neighbor Report element | 7.3.2.37 | (CFv AND CF1):M | Yes, No, N/A |
| WNM7.2 | BSS Transition Management Query frame | 7.4.12.8 | (CFv AND CF1):M | Yes, No, N/A |
| WNM7.3 | BSS Transition Management Request frame | 7.4.12.9 | (CFv AND CF2):M | Yes, No, N/A |
| WNM7.4 | BSS Transition Management Response frame | 7.4.12.10 | (CFv AND CF2):M | Yes, No, N/A |
| *WNM8 | FMS | 11.2.1.4a | CFv:O | Yes, No, N/A |
| WNM8.1 | FMS Request frame | 7.4.12.11 | (CF2&WNM8):M | Yes, No, N/A |
| WNM8.2 | FMS Response frame | 7.4.12.12 | (CF1&WNM8):M | Yes, No, N/A |
| WNM9 | Proxy ARP | 11.22.13 | CFv:O | Yes, No, N/A |
| *WNM10 | Collocated Interference Reporting | 11.22.9 | CFv:O | Yes, No, N/A |
| WNM10.1 | Collocated Interference Request frame | 7.4.12.13 | WNM10:M | Yes, No, N/A |
| WNM10.2 | Collocated Interference Response frame | 7.4.12.12 | WNM10:M | Yes, No, N/A |
| *WNM11 | BSS Max idle period | 11.22.12 | CFv:M | Yes, No, N/A |
| WNM11.1 | BSS Max Idle Period element | 7.3.2.78 | WNM11:M | Yes, No, N/A |
| *WNM12 | TFS | 11.22.11 | CFv:O | Yes, No, N/A |
| WNM12.1 | TFS Request frame | 7.3.2.79, 7.4.12.15 | WNM12:M | Yes, No, N/A |
| WNM12.2 | TFS Response frame | 7.3.2.80, 7.4.12.16 | WNM12:M | Yes, No, N/A |
| WNM12.3 | TFS Notify frame | 7.4.12.17 | (CF1&WNM12):M, (CF2&WNM12):O | Yes, No, N/A |
| *WNM13 | WNM-Sleep Mode | 11.2.1.16 | WNM12:O | Yes, No, N/A |
| WNM13.1 | WNM-Sleep Mode Request frame | 7.3.2.81, 7.4.12.18 | WNM13:M | Yes, No, N/A |
| WNM13.2 | WNM-Sleep Mode Response frame | 7.3.2.81, 7.4.12.19 | WNM13:M | Yes, No, N/A |
| *WNM14 | TIM Broadcast | 11.2.1.13 | CFv:O | Yes, No. N/A |
| WNM14.1 | TIM Broadcast Request frame | 7.3.2.82, 7.4.12.20, | WNM14:M | Yes, No, N/A |
| WNM14.2 | TIM Broadcast Response frame | 7.3.2.83, 7.4.12.21 | WNM14:M | Yes, No, N/A |
| WNM14.3 | TIM Broadcast frame | 7.4.13.2 | WNM14:M | Yes, No, N/A |
| *WNM15 | QoS Traffic Capability | 11.22.9 | (CFv AND CF2):O | Yes, No, N/A |
| WNM15.1 | QoS Traffic Capability element | 7.3.2.77, | WNM15:M | Yes, No, N/A |
| WNM15.2 | QoS Traffic Capability update frame | 7.4.12.22 | WNM15:M | Yes, No, N/A |
| WNM16 | AC Station Count | 11.22.10 | (CFv AND CF2):O | Yes, No, N/A |
| WNM17 | Timing Measurement | 11.22.5 | CFv:O | Yes, No, N/A |
| *WNM18 | Channel Usage | 11.22.14 | CFv:O | Yes, No, N/A |
| WNM18.1 | Channel Usage Request frame | 7.3.2.85, 7.4.12.23 | WNM18:M | Yes, No, N/A |
| WNM18.2 | Channel Usage Response frame | 7.3.2.85, 7.4.12.24 | WNM18:M | Yes, No, N/A |
| *WNM19 | DMS | 11.22.15 | (CFv AND CF16):O | Yes, No, N/A |
| WNM19.1 | DMS Request frame | 7.3.2.87, 7.4.12.25 | WNM19:M | Yes, No, N/A |
| WNM19.2 | DMS Response frame | 7.3.2.88, 7.4.12.26 | WNM19:M | Yes, No, N/A |
| WNM20 | UTC TSF Offset | 11.21.3, 7.3.2.65, 7.3.2.86 | CFv:O | Yes, No, N/A |

Annex D

(normative)

ASN.1 encoding of the MAC and PHY MIB

Change the “Dot11StationConfigEntry” of the “dotStationConfig TABLE” as follows:

-- *********************************************************************

-- * dotStationConfig TABLE

-- *********************************************************************

Dot11StationConfigEntry ::=
SEQUENCE {
	dot11StationID 					MacAddress,
	dot11MediumOccupancyLimit 					INTEGER,
	dot11CFPollable 					TruthValue,
	dot11CFPeriod 					INTEGER,
	dot11CFPMaxDuration 					INTEGER,
	dot11AuthenticationResponseTimeOut 					Unsigned32,
	dot11PrivacyOptionImplemented 					TruthValue,
	dot11PowerManagementMode 					INTEGER,
	dot11DesiredSSID 					OCTET STRING,
	dot11DesiredBSSType 					INTEGER,
	dot11OperationalRateSet 					OCTET STRING,
	dot11BeaconPeriod 					INTEGER,
	dot11DTIMPeriod 					INTEGER,
	dot11AssociationResponseTimeOut 					Unsigned32,
	dot11DisassociateReason 					INTEGER,
	dot11DisassociateStation 					MacAddress,
	dot11DeauthenticateReason 					INTEGER,
	dot11DeauthenticateStation 					MacAddress,
	dot11AuthenticateFailStatus 					INTEGER,
	dot11AuthenticateFailStation 					MacAddress,
	dot11MultiDomainCapabilityImplemented 					TruthValue,
	dot11MultiDomainCapabilityEnabled 					TruthValue,
	dot11CountryString 					OCTET STRING,
	dot11SpectrumManagementImplemented 					TruthValue,
	dot11SpectrumManagementRequired 					TruthValue,
	dot11RSNAOptionImplemented 					TruthValue,
	dot11RSNAPreauthenticationImplemented 					TruthValue,
	dot11RegulatoryClassesImplemented 					TruthValue,
	dot11RegulatoryClassesRequired 					TruthValue,
	dot11QosOptionImplemented 					TruthValue,
	dot11ImmediateBlockAckOptionImplemented 					TruthValue,
	dot11DelayedBlockAckOptionImplemented 					TruthValue,
	dot11DirectOptionImplemented 					TruthValue,
	dot11APSDOptionImplemented 					TruthValue,
	dot11QAckOptionImplemented 					TruthValue,
	dot11QBSSLoadOptionImplemented 					TruthValue,
	dot11QueueRequestOptionImplemented 					TruthValue,
	dot11TXOPRequestOptionImplemented 					TruthValue,
	dot11MoreDataAckOptionImplemented 					TruthValue,
	dot11AssociatedinNQBSS 					TruthValue,
	dot11DLSAllowdInQBSS 					TruthValue,
	dot11DLSAllowed 					TruthValue,
	dot11AssociateStation 					MacAddress,
	dot11AssociateID 					INTEGER,
	dot11AssociateFailStation 					MacAddress,
	dot11AssociateFailStatus 					INTEGER,
	dot11ReassociateStation 					MacAddress,
	dot11ReassociateID 					INTEGER,
	dot11ReassociateFailStation 					MacAddress,
	dot11ReassociateFailStatus 					INTEGER,
	dot11RadioMeasurementCapable 					TruthValue,
	dot11RadioMeasurementEnabled 					TruthValue,
	dot11RadioMeasurementProbeDelay 					INTEGER,
	dot11MeasurementPilotReceptionEnabled TruthValue,
	dot11MeasurementPilotTransmissionEnabled TruthValue,
	dot11MeasurementPilotTransmissionVirtualApSetEnabled TruthValue,
	dot11MeasurementPilotPeriod 					INTEGER,
	dot11LinkMeasurementEnabled 					TruthValue,
	dot11NeighborReportEnabled 					TruthValue,
	dot11ParallelMeasurementsEnabled 					TruthValue,
	dot11TriggeredMeasurementsEnabled 					TruthValue,
	dot11RepeatedMeasurementsEnabled 					TruthValue,
	dot11MeasurementPauseEnabled 					TruthValue,
	dot11QuietIntervalEnabled 					TruthValue,
	dot11PassiveBeaconMeasurementEnabled 					TruthValue,
	dot11ActiveBeaconMeasurementEnabled 					TruthValue,
	dot11TableBeaconMeasurementEnabled 					TruthValue,
	dot11ReportingConditionsEnabled 					TruthValue,
	dot11FrameMeasurementEnabled 					TruthValue,
	dot11ChannelLoadEnabled 					TruthValue,
	dot11NoiseHistogramEnabled 					TruthValue,
	dot11StatisticsReportEnabled 					TruthValue,
	dot11LCIReportEnabled 					TruthValue,
	dot11TransmitStreamMeasurementEnabled 					TruthValue,
	dot11APChannelReportEnabled 					TruthValue,
	dot11AnnexQMIBSupportEnabled 					TruthValue,
	dot11NonOperatingChannelMeasurementsEnabled TruthValue,
	dot11MaximumMeasurementDuration 					Unsigned32,
	dot11MeasurementPilotSupport 					Unsigned32,
	dot11FastBSSTransitionImplemented 					TruthValue,
	dot11LCIDSEImplemented 					TruthValue, 
	dot11LCIDSERequired 					TruthValue, 
	dot11DSERequired 					TruthValue, 
	dot11ExtendedChannelSwitchEnabled 					TruthValue, 
	dot11HighThroughputOptionImplemented 					TruthValue,	dot11TunneledDirectLinkSetupImplemented 						TruthValue,
dot11TDLSPeerUAPSDImplemented 						TruthValue,
dot11TDLSPeerPSMImplemented 						TruthValue,
dot11TDLSPeerUAPSDIndicationWindow 						INTEGER,
dot11TDLSChannelSwitchingImplemented 						TruthValue,
dot11TDLSPeerSTAMissingAckRetryLimit 						INTEGER,
dot11TDLSResponseTimeout 						INTEGER,
dot11TDLSProbeDelay 						INTEGER,
dot11OCBEnabled 						TruthValue,	dot11WirelessManagementImplemented 						TruthValue, 
	dot11MaxIdlePeriod 					INTEGER,
	dot11TIMBroadcastInterval     					INTEGER,
	dot11TIMBroadcastOffset       					INTEGER,
	dot11MinTriggerTimeout     					INTEGER,
	dot11RRMCivicMeasurementEnabled 					TruthValue,
	dot11RRMIdentifierMeasurementEnabled 					TruthValue,
dot11DMSMaxSTAS       						INTEGER,
dot11DMSMaxChannelLoadForNewService 						INTEGER,
dot11DMSMaxChannelLoad 						INTEGER,
dot11TimeAdvertisementDTIMIntervals 						INTEGER,
dot11UTCTSFOffsetTimeError 						OCTET STRING ,
dot11UTCTSFOffsetTimeValue 						OCTET STRING}

Insert the following elements at the end of the dot11StationConfigTable element definitions:

EDITORIAL NOTE—last entry in 802.11-2007 has order 42. 11k adds 42-56. 11r adds 57. 11y adds none. 11n adds 1 entry (58). 11s adds none.

dot11WirelessManagementImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This attribute, when TRUE, indicates that the station 		implementation is capable of supporting one or more Wireless Network 		Management services.”
		DEFVAL { false }

::= { dot11StationConfigEntry 103}

dot11MaxIdlePeriod OBJECT-TYPE

SYNTAX INTEGER (1..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute indicates that the number of 1000 TUs that pass before an AP disassociates an inactive non-AP STA. This value is transmitted in the Association Response and Reassociation frames.”

::= { dot11StationConfigEntry 104}

dot11TIMBroadcastInterval OBJECT-TYPE

SYNTAX INTEGER (0...255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute indicates the smallest active number of beacon periods between TIM frame transmissions.”

DEFVAL { 0 }

::= { dot11StationConfigEntry 105}

dot11TIMBroadcastOffset OBJECT-TYPE

SYNTAX INTEGER (-2..648)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute indicates the offset in microseconds relative to the TBTT for which a TIM frame is scheduled for transmission.”

DEFVAL { 0 }

::= { dot11StationConfigEntry 106}

dot11MinTriggerTimeout OBJECT-TYPE

SYNTAX INTEGER (10..7200)

UNITS "seconds"

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute indicates the minimum allowable value for Triggered Timeout. A Triggered STA Statistics report is generated by the STA after the timeout if none of the trigger conditions are satisfied."

DEFVAL { 10 }

::= { dot11StationConfigEntry 107 }

dot11RRMCivicMeasurementEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that dot11RadioMeasurementEnabled is TRUE and that the station capability for Location Civic Measurement is enabled. FALSE indicates the station has no Location Civic Measurement capability or that the capability is present but is disabled."

DEFVAL { false }

::= { dot11StationConfigEntry 108 }

dot11RRMIdentifierMeasurementEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that dot11RadioMeasurementEnabled is TRUE and that the station capability for Location Identifier Measurement is enabled. FALSE indicates the station has no Location Identifier Measurement capability or that the capability is present but is disabled."

DEFVAL { false }

::= { dot11StationConfigEntry 109 }

dot11DMSMaxSTAS OBJECT-TYPE

SYNTAX INTEGER (1..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute indicates the maximum number of STAs which can be granted for Directed Multicast Service (DMS)."

DEFVAL { 255 }

::= { dot11StationConfigEntry 110 }

dot11DMSMaxChannelLoadForNewService OBJECT-TYPE

SYNTAX INTEGER (1..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute indicates the maximum allowed channel load in which a new Directed Multicast Service (DMS) may be granted. If the current channel load is larger than or equal to this attribute, the AP shall shall reject any new DMS service. The channel load measurement is defined in 11.10.8.3 (IEEE 802.11k-2008)."

DEFVAL { 255 }

::= { dot11StationConfigEntry 111 }

dot11DMSMaxChannelLoad OBJECT-TYPE

SYNTAX INTEGER (1..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute indicates the maximum allowed channel load in which AP can support the existing DMS services. If the current channel load is larger than or equal to this attribute, the AP may terminate some of existing Directed Multicast Services (DMS) to free up the bandwidth that is consumed by the DMS. The value of dot11DMSMaxChannelLoad shall be larger than the value of dot11DMSMaxChannelLoadForNewService. The channel load measurement is defined in 11.10.8.3 (IEEE 802.11k-2008)."

DEFVAL { 255 }

::= { dot11StationConfigEntry 112 }

dot11TimeAdvertisementDTIMIntervals OBJECT-TYPE

SYNTAX INTEGER (1..255)

UNITS "dtims"

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute indicates the interval in number of DTIMS when the UTC TSF Offset element is included in beacon frames."

DEFVAL { 1 }

::= { dot11StationConfigEntry 113 }

dot11UTCTSFOffsetTimeError OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(5))

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute indicates the UTC TSF Offset Time Error value as defined in the Time Advertisement IE Time Error field when the Time Capabilities field is set to 2. This field is included in the Time Advertisement element in Beacon and Probe Response frames."

DEFVAL { 0 }

::= { dot11StationConfigEntry 114 }

dot11UTCTSFOffsetTimeValue OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(10))

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute indicates the UTC TSF Offset Time Value as defined in the Time Advertisement IE Time Value field when the Time Capabilities field is set to 2. The format is defined in Table 7-37c and is included in the Time Advertisement element in Beacon and Probe Response frames."

::= { dot11StationConfigEntry 115 }

--***********************************************************

--*		dotCounters TABLE

--************************************************************

Change the “dot11CountersEntry” as follows:

Dot11CountersEntry ::=

SEQUENCE {

dot11TransmittedFragmentCount 				Counter32,

dot11MulticastTransmittedFrameCount 				Counter32,

dot11FailedCount 				Counter32,

dot11RetryCount 				Counter32,

dot11MultipleRetryCount 				Counter32,

dot11FrameDuplicateCount 				Counter32,

dot11RTSSuccessCount 				Counter32,

dot11RTSFailureCount 				Counter32,

dot11ACKFailureCount 				Counter32,

dot11ReceivedFragmentCount 				Counter32,

dot11MulticastReceivedFrameCount 				Counter32

dot11FCSErrorCount 				Counter32,

dot11TransmittedFrameCount 				Counter32,

dot11WEPUndecryptableCount 				Counter32,

dot11QoSDiscardedFragmentCount 				Counter32,

dot11AssociatedStationCount 				Counter32,

dot11QoSCFPollsReceivedCount 				Counter32,

dot11QoSCFPollsUnusedCount 				Counter32,

dot11QoSCFPollsUnusableCount 				Counter32,

dot11QoSCFPollsLostCount 				Counter32,

dot11DeniedAssociationCounterDueToBSSLoad       Counter32}

Insert the following elements at the end of the dot11Counters table definitions:

dot11DeniedAssociationCounterDueToBSSLoad OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter, available at a WNM AP, shall increment  when an association or re-association request is denied because the AP has insufficient bandwidth to handle the additional STA."

DEFVAL {0}

::= { dot11CountersEntry 21}

EDITORIAL NOTE—11k uses 14. 11r uses 15, 11n uses 16.

Insert the following element at the end of the list of child objects for dot11smt object identifier:

--***********************************************************

--*Wireless Network Management

--************************************************************

-- dot11WirelessMgmtOptionsTable	::= { dot11smt 17}

Insert the following text at the end of the Station management MIB:

-- **********************************************************************

-- * dot11WirelessMgmtOptions TABLE

-- **********************************************************************

dot11WirelessMgmtOptionsTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11WirelessMgmtOptionsEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

“Wireless Management attributes. In tabular form to allow for multiple instances on an agent. This table only applies to the interface if dot11WirelessManagementImplemented is set to TRUE in the dot11StationConfigTable. Otherwise this table should be ignored.”

::= { dot11smt 17 }

dot11WirelessMgmtOptionsEntry OBJECT-TYPE

SYNTAX Dot11WirelessMgmtOptionsEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11WirelessMgmtOptionsTable. For all Wireless Management features, an enabled MIB variable is used to enable or disable the corresponding feature.  An Implemented MIB variable is used for an optional feature to indicate whether the feature is implemented. A mandatory feature does not have a corresponding Implemented MIB variable. It is possible for there to be multiple IEEE 802.11 interfaces on one agent, each with its unique MAC address. The

relationship between an IEEE 802.11 interface and an interface in the context of the Internet-standard MIB is one-to-one. As such, the value of an ifIndex object

instance can be directly used to identify corresponding instances of the objects defined herein.

ifIndex - Each IEEE 802.11 interface is represented by an ifEntry. Interface tables in this MIB module are indexed by ifIndex."

INDEX { ifIndex }

::= { dot11WirelessMgmtOptionsTable 1 }

Dot11WirelessMgmtOptionsEntry ::=

SEQUENCE {

dot11MgmtOptionLocationEnabled				TruthValue,

dot11MgmtOptionFMSImplemented				TruthValue,

dot11MgmtOptionFMSEnabled				TruthValue,

dot11MgmtOptionEventsEnabled				TruthValue,

dot11MgmtOptionDiagnosticsEnabled				TruthValue,

dot11MgmtOptionMultiBSSIDImplemented				TruthValue,

dot11MgmtOptionMultiBSSIDEnabled				TruthValue,

dot11MgmtOptionTFSImplemented 				TruthValue,

dot11MgmtOptionTFSEnabled				TruthValue,

dot11MgmtOptionWNMSleepModeImplemented				TruthValue, 																			 			dot11MgmtOptionWNMSleepModeEnabled				TruthValue,

dot11MgmtOptionTIMBroadcastImplemented				TruthValue,

dot11MgmtOptionTIMBroadcastEnabled				TruthValue,

dot11MgmtOptionProxyARPEnabled				TruthValue,
		dot11MgmtOptionProxyARPImplemented 				TruthValue,

dot11MgmtOptionBSSTransitionEnabled				TruthValue,

dot11MgmtOptionQoSTrafficCapabilityImplemented					TruthValue,

dot11MgmtOptionQoSTrafficCapabilityEnabled					TruthValue,

dot11MgmtOptionACStationCountImplemented					TruthValue,

dot11MgmtOptionACStationCountEnabled				TruthValue,

dot11MgmtOptionMulticastDiagnosticsEnabled					TruthValue,
		dot11MgmtOptionCoLocIntfReportingEnabled					TruthValue,

dot11MgmtOptionCoLocIntReportingImplemented					TruthValue,

dot11MgmtOptionLocationTrackNotificationImplemented						TruthValue,

dot11MgmtOptionLocationTrackNotificationEnabled					TruthValue,

dot11MgmtOptionMotionDetectionImplemented					TruthValue, 
		dot11MgmtOptionMotionDetectionEnabled				TruthValue,
		dot11MgmtOptionTODImplemented 				TruthValue,

dot11MgmtOptionTODEnabled				TruthValue,

dot11MgmtOptionTimingMsmtImplemented 				TruthValue,

dot11MgmtOptionTimingMsmtEnabled				TruthValue,
		dot11MgmtOptionChannelUsageImplemented				TruthValue,
		dot11MgmtOptionChannelUsageEnabled				TruthValue,

dot11MgmtOptionTriggerSTAStatisticsEnabled					TruthValue,

dot11MgmtOptionSSIDListImplemented 				TruthValue,

dot11MgmtOptionSSIDListEnabled 				TruthValue,

dot11MgmtOptionDMSImplemented				TruthValue,

dot11MgmtOptionDMSEnabled				TruthValue,

dot11MgmtOptionUTCTSFOffsetImplemented				TruthValue,

dot11MgmtOptionUTCTSFOffsetEnabled				TruthValue}

dot11MgmtOptionLocationEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the capability of the station to provide location is enabled. The capability is disabled, otherwise."

DEFVAL { false}

::= { dot11WirelessMgmtOptionsEntry 1 }

dot11MgmtOptionFMSImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the station implementation is capable of supporting FMS when the dot11WirelessManagementImplemented is set to TRUE."

DEFVAL { false}

::= { dot11WirelessMgmtOptionsEntry 2 }

dot11MgmtOptionFMSEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the capability of the station to provide FMS is enabled. The capability is disabled, otherwise"

DEFVAL { false}

::= { dot11WirelessMgmtOptionsEntry 3 }

dot11MgmtOptionEventsEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the capability of the station to provide Event Reporting is enabled. The capability is disabled, otherwise"

DEFVAL { false}

::= { dot11WirelessMgmtOptionsEntry 4 }

dot11MgmtOptionDiagnosticsEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the capability of the station to provide Diagnostic Reporting is enabled. The capability is disabled, otherwise."

DEFVAL { false}

::= { dot11WirelessMgmtOptionsEntry 5 }

dot11MgmtOptionMultiBSSIDImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the station

implementation is capable of supporting Multiple BSSID when the dot11WirelessManagementImplemented is set to TRUE."

DEFVAL { false}

::= { dot11WirelessMgmtOptionsEntry 6 }

dot11MgmtOptionMultiBSSIDEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the

capability of the station to provide Multi BSSID is

enabled. The capability is disabled, otherwise."

DEFVAL { false}

::= { dot11WirelessMgmtOptionsEntry 7 }

dot11MgmtOptionTFSImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the station implementation is capable of supporting TFS when the dot11WirelessManagementImplemented is set to TRUE."
DEFVAL { false}

::= { dot11WirelessMgmtOptionsEntry 8 }

dot11MgmtOptionTFSEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that TFS is enabled. TFS is disabled otherwise."

DEFVAL { false}

::= { dot11WirelessMgmtOptionsEntry 9 }

dot11MgmtOptionWNMSleepModeImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the station implementation is capable of supporting WNMSleep Mode when the dot11WirelessManagementImplemented is set to TRUE."

DEFVAL { false}

::= { dot11WirelessMgmtOptionsEntry 10 }

dot11MgmtOptionWNMSleepModeEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that WNMSleep Mode is enabled. WNMSleep Mode is disabled otherwise."

DEFVAL { false}

::= { dot11WirelessMgmtOptionsEntry 11 }

dot11MgmtOptionTIMBroadcastImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the station implementation is capable of supporting TIM Broadcast when the dot11WirelessManagementImplemented is set to TRUE."

DEFVAL { false}

::= { dot11WirelessMgmtOptionsEntry 12}

dot11MgmtOptionTIMBroadcastEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that TIM broadcast is enabled. TIM broadcast is disabled otherwise."

DEFVAL { false}

::= { dot11WirelessMgmtOptionsEntry 13}

dot11MgmtOptionProxyARPEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the capability of the AP to provide the Proxy ARP service is enabled. The capability is disabled, otherwise."

DEFVAL { false}

::= { dot11WirelessMgmtOptionsEntry 14 }

dot11MgmtOptionProxyARPImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the station implementation is capable of supporting the Proxy ARP service, when the dot11WirelessManagementImplemented is set to TRUE.”

DEFVAL { false}

::= { dot11WirelessMgmtOptionsEntry 15 }

dot11MgmtOptionBSSTransitionEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the capability of the station to provide BSS Transition is enabled. The capability is disabled, otherwise. "

DEFVAL { false}

::= { dot11WirelessMgmtOptionsEntry 16 }

dot11MgmtOptionQoSTrafficCapabilityImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the station implementation is capable of supporting QoS Traffic Capability when the dot11WirelessManagementImplemented is set to TRUE."

DEFVAL { false}

::= { dot11WirelessMgmtOptionsEntry 17 }

dot11MgmtOptionQoSTrafficCapabilityEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the capability of the station to provide QoS Traffic Capability is enabled. QoS Traffic Capability is disabled otherwise."

DEFVAL { false}

::= { dot11WirelessMgmtOptionsEntry 18 }

dot11MgmtOptionACStationCountImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the station implementation is capable of supporting AC Station Count when the dot11WirelessManagementImplemented is set to TRUE."

DEFVAL { false}

::= { dot11WirelessMgmtOptionsEntry 19 }

dot11MgmtOptionACStationCountEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the capability of the station to provide AC Station Count is enabled. AC Station Count is disabled otherwise."

DEFVAL { false}

::= { dot11WirelessMgmtOptionsEntry 20 }

dot11MgmtOptionCoLocIntfReportingEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the capability of the station to support supports Colocated Interference Reporting is enabled. The capability is disabled, otherwise."

DEFVAL { false}

::= { dot11WirelessMgmtOptionsEntry 21 }

dot11MgmtOptionCoLocIntReportingImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the station implementation is capable of supporting Colocated Interference Reporting. The capability is disabled, otherwise."

DEFVAL { false}

::= { dot11WirelessMgmtOptionsEntry 22 }

dot11MgmtOptionMotionDetectionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the station implementation is capable of supporting motion detection when the dot11WirelessManagementImplemented is set to TRUE. "

DEFVAL { false}

::= { dot11WirelessMgmtOptionsEntry 23 }

dot11MgmtOptionMotionDetectionEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the capability to support motion detection is enabled."

DEFVAL { false}

::= { dot11WirelessMgmtOptionsEntry 24 }

dot11MgmtOptionTODImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the station implementation is capable of supporting Time Of Departure for transmitted Clause 15, 17, 18, 19 and 20 frames when the dot11WirelessManagementImplemented is set to TRUE."

DEFVAL { false}

::= { dot11WirelessMgmtOptionsEntry 25 }

dot11MgmtOptionTODEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the capability to support Time Of Departure frames for transmitted Clause 15, 17, 18, 19 and 20 frames is enabled."

DEFVAL { false}

::= { dot11WirelessMgmtOptionsEntry 26 }

dot11MgmtOptionTimingMsmtImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the station implementation is capable of supporting Timing Measurement capability when the dot11WirelessManagementImplemented is set to TRUE."

DEFVAL { false}

::= { dot11WirelessMgmtOptionsEntry 27 }

dot11MgmtOptionTimingMsmtEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the station capability for Timing Measurement is enabled. FALSE indicates the station has no Timing Measurement capability or that the capability is present but is disabled."

DEFVAL { false}

::= { dot11WirelessMgmtOptionsEntry 28 }

dot11MgmtOptionChannelUsageImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the station implementation is capable of supporting Channel Usage when the dot11WirelessManagementImplemented is set to TRUE.”
	DEFVAL { false}
::= { dot11WirelessMgmtOptionsEntry 29 }

dot11MgmtOptionChannelUsageEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that Channel Usage is enabled. Channel Usage is disabled otherwise.”
		DEFVAL { false}
		::= { dot11WirelessMgmtOptionsEntry 30 }

dot11MgmtOptionTriggerSTAStatisticsEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute, when TRUE, indicates that the capability of the station to provide triggered STA statistics is enabled. The capability is disabled otherwise"

DEFVAL { false}

::= { dot11WirelessMgmtOptionsEntry 31 }

dot11MgmtOptionSSIDListImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the station implementation is capable of supporting the SSID List capability when the dot11WirelessManagementImplemented is TRUE."

DEFVAL { false}

::= { dot11WirelessMgmtOptionsEntry 32 }

dot11MgmtOptionSSIDListEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the capability of the station to support the SSID List capability is enabled. The capability is disabled, otherwise"

DEFVAL { false}

::= { dot11WirelessMgmtOptionsEntry 33 }

dot11MgmtOptionMulticastDiagnosticsEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the

capability of the station to provide Multicast Diagnostic Reporting is

enabled. The capability is disabled, otherwise."

DEFVAL { false}

::= { dot11WirelessMgmtOptionsEntry 34 }

dot11MgmtOptionLocationTrackNotificationImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the station

implementation is capable of supporting Location Track Notification when the dot11WirelessManagementImplemented is TRUE."

DEFVAL { false}

::= { dot11WirelessMgmtOptionsEntry 35 }

dot11MgmtOptionLocationTrackNotificationEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the

capability of the station to provide Location Track Notification is

enabled. The capability is disabled otherwise."

DEFVAL { false}

::= { dot11WirelessMgmtOptionsEntry 36 }

dot11MgmtOptionDMSImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the station implementation is capable of supporting DMS when the dot11WirelessManagementImplemented is TRUE."

DEFVAL { false}

::= { dot11WirelessMgmtOptionsEntry 37 }

dot11MgmtOptionDMSEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that DMS is enabled. DMS is disabled otherwise."

DEFVAL { false}

::= { dot11WirelessMgmtOptionsEntry 38 }

dot11MgmtOptionUTCTSFOffsetImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the

Station implementation is capable of supporting UTC TSF Offset advertisement when the dot11WirelessManagementImplemented is set to TRUE."

DEFVAL { false}

::= { dot11WirelessMgmtOptionsEntry 39 }

dot11MgmtOptionUTCTSFOffsetEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that UTC TSF Offset advertisement is enabled at the station. The capability is disabled, otherwise."

DEFVAL { false}

::= { dot11WirelessMgmtOptionsEntry 40}

-- ********************************************************************

-- * dot11LocationServices TABLE

-- ********************************************************************

dot11LocationServicesTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11LocationServicesEntry

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Group contains conceptual table of attributes for

WNM LocationServices."

::= { dot11smt 19 }

dot11LocationServicesEntry OBJECT-TYPE

SYNTAX Dot11LocationServicesEntry

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"An entry in the dot11LocationServicesTable

Indexed by dot11LocationServicesIndex."

INDEX { dot11LocationServicesIndex }

::= { dot11LocationServicesTable 1 }

Dot11LocationServicesEntry ::=

SEQUENCE {

dot11LocationServicesIndex Unsigned32,
		dot11LocationServicesMACAddress MacAddress,

dot11LLocationServicesLocationIndicationParametersReportIntervalUnits INTEGER,

dot11LocationServicesLocationIndicationParametersNormalReportInterval INTEGER,

dot11LocationServicesLocationIndicationParametersNormalFramesperChannel INTEGER,

dot11LocationServicesLocationIndicationParametersInMotionReportInterval INTEGER,

dot11LocationServicesLocationIndicationParametersInMotionFramesperChannel INTEGER,

dot11LocationServicesLocationIndicationParametersBurstInterframeInterval INTEGER,

dot11LocationServicesLocationIndicationChannelsChannel OCTET STRING,

dot11LocationServicesLocationStatus INTEGER}

dot11LocationConfigIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute is the Index for LocationServices Report elements in dot11LocationServicesTable,

greater than 0."

::= { dot11LocationServicesEntry 1 }

dot11LocationServicesMACAddress OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute is the MAC address of the STA reporting location information."

::= { dot11LocationServicesEntry 2 }

dot11LocationServicesLocationIndicationParametersReportIntervalUnits OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute contains the Location Indication Parameters Report Interval Units value."

::= { dot11LocationServicesEntry 3 }

dot11LocationServicesLocationIndicationParametersNormalReportInterval OBJECT-TYPE

SYNTAX INTEGER (0..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute contains the Location Indication Parameters Normal Report Interval value."

::= { dot11LocationServicesEntry 4 }

dot11LocationServicesLocationIndicationParametersNormalFramesperChannel OBJECT-TYPE
SYNTAX INTEGER (0..255)
MAX-ACCESS read-write
STATUS current
DESCRIPTION
		"This attribute contains the Location Indication Parameters Normal Frames per Channel value."
::= { dot11LocationServicesEntry 5 }

dot11LocationServicesLocationIndicationParametersInMotionReportInterval OBJECT-TYPE
SYNTAX INTEGER (0..65535)
MAX-ACCESS read-write
STATUS current
DESCRIPTION
		"This attribute contains the Location Indication Parameters in-motion Report Interval value."
::= { dot11LocationServicesEntry 6 }

dot11LocationServicesLocationIndicationParametersInMotionFramesperChannel OBJECT-TYPE
	SYNTAX INTEGER (0..255)
	MAX-ACCESS read-write
STATUS current

DESCRIPTION

"This attribute contains the Location Indication Parameters in-motion Frames per Channel value."

::= { dot11LocationServicesEntry 7 }

dot11LocationServicesLocationIndicationParametersBurstInterframeInterval OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute contains the Location Indication Parameters inter-frame Interval value."

::= { dot11LocationServicesEntry 8 }

dot11LocationServicesLocationIndicationChannels OBJECT-TYPE

SYNTAX OCTET STRING (SIZE (2..254))

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute contains the Location Indication Channels Channel and Regulatory Class octet fields."

::= { dot11LocationServicesEntry 9 }

dot11LocationServicesLocationStatus OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the Location Status value."

::= { dot11LocationServicesEntry 10 }

-- ********************************************************************

-- * End of dot11LocationServices TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11WirelessMGTEvent TABLE

-- ********************************************************************

dot11WirelessMGTEventTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11WirelessMGTEventEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains the current list of WIRELESS Management reports that have been

received by the MLME. The report tables shall be maintained as FIFO to

preserve freshness, thus the rows in this table can be deleted for memory

constraints or other implementation constraints determined by the vendor.

New rows shall have different RprtIndex values than those deleted within the

range limitation of the index. One easy way is to monotonically increase

the EventIndex for new reports being written in the table."

::= { dot11smt 20 }

dot11WirelessMGTEventEntry OBJECT-TYPE

SYNTAX dot11WirelessMGTEventEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11WirelessMGTEventTable

Indexed by dot11WirelessMGTEventIndex."

INDEX { dot11WirelessMGTEventIndex }

::= { dot11WirelessMGTEventTable 1 }

Dot11WirelessMGTEventEntry ::=

SEQUENCE {

dot11WirelessMGTEventIndex 							Unsigned32,

dot11WirelessMGTEventMACAddress 							MacAddress,

dot11WirelessMGTEventType 							INTEGER,

dot11WirelessMGTEventStatus 							INTEGER,

dot11WirelessMGTEventTimestamp 							OCTET STRING,

dot11WirelessMGTEventTransitionSourceBSSID 							MacAddress,

dot11WirelessMGTEventTransitionTargetBSSID 							MacAddress,

dot11WirelessMGTEventTransitionTime 							INTEGER,

dot11WirelessMGTEventTransitionReason 							INTEGER,

dot11WirelessMGTEventTransitionResult 							INTEGER,

dot11WirelessMGTEventTransitionSourceRCPI 							INTEGER,

dot11WirelessMGTEventTransitionSourceRSNI 							INTEGER,

dot11WirelessMGTEventTransitionTargetRCPI 							INTEGER,

dot11WirelessMGTEventTransitionTargetRSNI 							INTEGER,

dot11WirelessMGTEventRSNATargetBSSID 							MacAddress,

dot11WirelessMGTEventRSNAAuthenticationType 							OCTET STRING,

dot11WirelessMGTEventRSNAEAPMethod 							OCTET STRING,

dot11WirelessMGTEventRSNAResult 							INTEGER,

dot11WirelessMGTEventRSNARSNElement 							OCTET STRING,

dot11WirelessMGTEventPeerSTAAddress 							MacAddress,

dot11WirelessMGTEventPeerRegulatoryClass 							INTEGER,

dot11WirelessMGTEventPeerChannelNumber 							INTEGER,

dot11WirelessMGTEventPeerSTATxPower 							INTEGER,

dot11WirelessMGTEventPeerConnectionTime 							INTEGER,

dot11WirelessMGTEventWNMLog 							DisplayString}

dot11WirelessMGTEventIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the Index for EVENT Report elements in dot11WirelessMGTEventTable,greater than 0."

::= { dot11WirelessMGTEventEntry 1 }

dot11WirelessMGTEventMACAddress OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute is the MAC address of the STA providing the Event Report."

::= { dot11WirelessMGTEventEntry 2 }

dot11WirelessMGTEventType OBJECT-TYPE

SYNTAX INTEGER (0...255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the Type value of the Event Report element."

::= { dot11WirelessMGTEventEntry 3 }

dot11WirelessMGTEventStatus OBJECT-TYPE

SYNTAX INTEGER (0...255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the status value included in the Event Report."

::= { dot11WirelessMGTEventEntry 4 }

dot11WirelessMGTEventTimestamp OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(8))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the value of the Event timestamp field."

::= { dot11WirelessMGTEventEntry 5 }

dot11WirelessMGTEventTransitionSourceBSSID OBJECT-TYPE

SYNTAX MacAddress (SIZE (6))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the value of the Source BSSID field in a Transition event report."

::= { dot11WirelessMGTEventEntry 6 }

dot11WirelessMGTEventTransitionTargetBSSID OBJECT-TYPE

SYNTAX MacAddress (SIZE (6))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the value of the Target BSSID field in a Transition event report."

::= { dot11WirelessMGTEventEntry 7 }

dot11WirelessMGTEventTransitionTime OBJECT-TYPE

SYNTAX INTEGER (0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the value of the Transition Time in the Transition event report."

::= { dot11WirelessMGTEventEntry 8 }

dot11WirelessMGTEventTransitionReason OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the value of the Transition Reason field in the Transition event report."

::= { dot11WirelessMGTEventEntry 9 }

dot11WirelessMGTEventTransitionResult OBJECT-TYPE

SYNTAX INTEGER (0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the value of the Transition Result field in the Transition event report."

::= { dot11WirelessMGTEventEntry 10 }

dot11WirelessMGTEventTransitionSourceRCPI OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the value of the Source RCPI field in the Transition event report."

::= { dot11WirelessMGTEventEntry 11 }

dot11WirelessMGTEventTransitionSourceRSNI OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the value of the Source RSNI field in the Transition event report."

::= { dot11WirelessMGTEventEntry 12 }

dot11WirelessMGTEventTransitionTargetRCPI OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the value of the Target RCPI field in the Transition event report.”

::= { dot11WirelessMGTEventEntry 13 }

dot11WirelessMGTEventTransitionTargetRSNI OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the value of the Target RSNI field in the Transition event report."

::= { dot11WirelessMGTEventEntry 14 }

dot11WirelessMGTEventRSNATargetBSSID OBJECT-TYPE

SYNTAX MacAddress (SIZE (6))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the value of the Target BSSID field in an RSNA event report."

::= { dot11WirelessMGTEventEntry 15 }

dot11WirelessMGTEventRSNAAuthenticationType OBJECT-TYPE

SYNTAX OCTET STRING (SIZE (4))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the value of the Authentication Type field in an RSNA event report."

::= { dot11WirelessMGTEventEntry 16 }

dot11WirelessMGTEventRSNAEAPMethod OBJECT-TYPE

SYNTAX OCTET STRING(SIZE (8))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the value of the EAP Method field in an RSNA event report."

::= { dot11WirelessMGTEventEntry 17 }

dot11WirelessMGTEventRSNAResult OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the value of the RSNA Result field in an RSNA event report."

::= { dot11WirelessMGTEventEntry 18 }

dot11WirelessMGTEventRSNARSNElement OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(36..256))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the value of the RSN Element field in an RSNA event report."

::= { dot11WirelessMGTEventEntry 19 }

dot11WirelessMGTEventPeerSTAAddress OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the value of the Peer STA Address field in a Peer-to-Peer Link event report."

::= { dot11WirelessMGTEventEntry 20 }

dot11WirelessMGTEventPeerRegulatoryClass OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the value of the Regulatory Class field in a Peer-to-Peer Link event report."

::= { dot11WirelessMGTEventEntry 21 }

dot11WirelessMGTEventPeerChannelNumber OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the value of the Channel Number field in a Peer-to-Peer Link event report."

::= { dot11WirelessMGTEventEntry 22 }

dot11WirelessMGTEventPeerSTATxPower OBJECT-TYPE

SYNTAX INTEGER (-128..127)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the value of the STA Tx Power field in a Peer-to-Peer Link event report. This is a signed integer, one octet in length, in dBm. A value of -128 indicates that the value is unknown."

::= { dot11WirelessMGTEventEntry 23 }

dot11WirelessMGTEventPeerConnectionTime OBJECT-TYPE

SYNTAX INTEGER (0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the value of the Connection Time field in a Peer-to-Peer Link event report."

::= { dot11WirelessMGTEventEntry 24 }

dot11WirelessMGTEventWNMLog OBJECT-TYPE

SYNTAX DisplayString

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the value of the WNM Log Msg field in a WNM Log event report."

::= { dot11WirelessMGTEventEntry 25 }

-- ********************************************************************

-- * End of dot11WirelessMGTEvent TABLE

-- ********************************************************************

Change the dot11Compliance MODULE-COMPLIANCE of the Compliance Statements as follows:

dot11Compliance MODULE-COMPLIANCE

STATUS current

DESCRIPTION

"The compliance statement for SNMPv2 entities that implement The IEEE 802.11 MIB."

MODULE -- this module

MANDATORY-GROUPS {

dot11SMTbase1011, dot11MACbase2, dot11CountersGroup2,

dot11SmtAuthenticationAlgorithms, dot11ResourceTypeID,

dot11PhyOperationComplianceGroup }

Change the "OPTIONAL-GROUPS" of the "Compliance Statements" as follows:

-- OPTIONAL-GROUPS { dot11SMTprivacy, dot11MACStatistics,

-- dot11PhyAntennaComplianceGroup, dot11PhyTxPowerComplianceGroup,

-- dot11PhyRegDomainsSupportGroup,

-- dot11PhyAntennasListGroup, dot11PhyRateGroup,

-- dot11SMTbase3, dot11MultiDomainCapabilityGroup,

-- dot11PhyFHSSComplianceGroup2, dot11RSNAadditions,

-- dot11RegulatoryClassesGroup, dot11Qosadditions,

-- dot11RRMCompliance,dot11FTComplianceGroup,

-- dot11PhyAntennaComplianceGroup2,

-- dot11HTMACadditions,

-- dot11PhyMCSGroup,

-- dot11TransmitBeamformingGroup,

-- dot11WNMCompliance}

--

::= { dot11Compliances 1 }

Change the “dot11SMTbase10” of the “Groups - units of conformance” as follows:

dot11SMTbase10 OBJECT-GROUP

OBJECTS { dot11MediumOccupancyLimit,

dot11CFPollable,

dot11CFPPeriod,

dot11CFPMaxDuration,

dot11AuthenticationResponseTimeOut,

dot11PrivacyOptionImplemented,

dot11PowerManagementMode,

dot11DesiredSSID, dot11DesiredBSSType,

dot11OperationalRateSet,

dot11BeaconPeriod, dot11DTIMPeriod,

dot11AssociationResponseTimeOut,

dot11DisassociateReason,

dot11DisassociateStation,

dot11DeauthenticateReason,

dot11DeauthenticateStation,

dot11AuthenticateFailStatus,

dot11AuthenticateFailStation,

dot11MultiDomainCapabilityImplemented,

dot11MultiDomainCapabilityEnabled,

dot11CountryString,

dot11SpectrumManagementImplemented,

dot11SpectrumManagementRequired,

dot11RSNAOptionImplemented,

dot11RegulatoryClassesImplemented,

dot11RegulatoryClassesRequired,

dot11QosOptionImplemented,

dot11ImmediateBlockAckOptionImplemented,

dot11DelayedBlockAckOptionImplemented,

dot11DirectOptionImplemented,

dot11APSDOptionImplemented,

dot11QAckOptionImplemented,

dot11QBSSLoadOptionImplemented,

dot11QueueRequestOptionImplemented,

dot11TXOPRequestOptionImplemented,

dot11MoreDataAckOptionImplemented,

dot11AssociateinNQBSS,

dot11DLSAllowedinQBSS,

dot11DLSAllowed,

dot11SMTRRMRequest,

dot11SMTRRMReport,

dot11SMTRRMConfig,

dot11AssociateStation,
			dot11AssociateID,
			dot11AssociateFailStation,
			dot11AssociateFailStatus,
			dot11ReassociateStation,
			dot11ReassociateID,
			dot11ReassociateFailStation,
			dot11ReassociateFailStatus,
			dot11RadioMeasurementCapable,
			dot11RadioMeasurementEnabled,
			dot11RRMMeasurementProbeDelay,
			dot11RRMMeasurementPilotPeriod,
			dot11RRMLinkMeasurementEnabled,
			dot11RRMNeighborReportEnabled,
			dot11RRMParallelMeasurementsEnabled,
			dot11RRMRepeatedMeasurementsEnabled,
			dot11RRMBeaconPassiveMeasurementEnabled,
			dot11RRMBeaconActiveMeasurementEnabled,
			dot11RRMBeaconTableMeasurementEnabled,
			dot11RRMBeaconMeasurementReportingConditionsEnabled,
			dot11RRMFrameMeasurementEnabled,
			dot11RRMChannelLoadMeasurementEnabled,
			dot11RRMNoiseHistogramMeasurementEnabled,
			dot11RRMStatisticsMeasaurementEnabled,
			dot11RRMLCIMeasurementEnabled,
			dot11RRMLCIAzimuthEnabled,
			dot11RRMTransmitStreamCategoryMeasurementEnabled,
			dot11RRMTriggeredTransmitStreamCategoryMeasurementEnabled,
			dot11RRMAPChannelReportEnabled,
			dot11RRMMIBEnabled,
			dot11RRMMaxMeasurementDuration,
			dot11RRMNonOperatingChannelMaxMeasurementDuration,
			dot11RRMMeasurementPilotTransmissionInformationEnabled,
			dot11RRMMeasurementPilotCapability,
			dot11RRMNeighborReportTSFOffsetEnabled,
			dot11RRMRCPIMeasurementEnabled,
			dot11RRMRSNIMeasurementEnabled,
			dot11RRMBSSAverageAccessDelayEnabled,
			dot11RRMBSSAvailableAdmissionCapacityEnabled,
			dot11RRMAntennaInformationEnabled,

dot11FastBSSTransitionImplemented,

dot11LCIDSEImplemented,

dot11LCIDSERequired,

dot11DSERequired,

dot11ExtendedChannelSwitchEnabled,

dot11HighThroughputOptionImplemented }

STATUS currentdeprecated

DESCRIPTION

"The SMTbase8 object class provides the necessary support at the STA to manage the processes in the STA so that the STA may work cooperatively as a part of an IEEE 802.11 network, when the STA is capable of multidomain operation. This object group should be implemented when the multidomain capability option is implemented."

::= { dot11Groups 51 }

Insert at the end of the "Groups - units of conformance" as follows:

-- ********************************************************************

-- * Compliance Statements - WNM

-- ********************************************************************

dot11WNMCompliance MODULE-COMPLIANCE
	STATUS current

DESCRIPTION

" This object class provides the objects from the IEEE 802.11

MIB required to manage Wireless Network Management

functionality. Note that additional objects for managing this

functionality are located in the IEEE 802.11 WNM MIB."

OBJECTS

{

dot11WirelessNetworkManagementImplemented

}

::= { dot11Groups 52}

dot11SMTbase11 OBJECT-GROUP

OBJECTS { dot11MediumOccupancyLimit,

dot11CFPollable,

dot11CFPPeriod,

dot11CFPMaxDuration,

dot11AuthenticationResponseTimeOut,

dot11PrivacyOptionImplemented,

dot11PowerManagementMode,

dot11DesiredSSID, dot11DesiredBSSType,

dot11OperationalRateSet,

dot11BeaconPeriod, dot11DTIMPeriod,

dot11AssociationResponseTimeOut,

dot11DisassociateReason,

dot11DisassociateStation,

dot11DeauthenticateReason,

dot11DeauthenticateStation,

dot11AuthenticateFailStatus,

dot11AuthenticateFailStation,

dot11MultiDomainCapabilityImplemented,

dot11MultiDomainCapabilityEnabled,

dot11CountryString,

dot11SpectrumManagementImplemented,

dot11SpectrumManagementRequired,

dot11RSNAOptionImplemented,

dot11RegulatoryClassesImplemented,

dot11RegulatoryClassesRequired,

dot11QosOptionImplemented,

dot11ImmediateBlockAckOptionImplemented,

dot11DelayedBlockAckOptionImplemented,

dot11DirectOptionImplemented,

dot11APSDOptionImplemented,

dot11QAckOptionImplemented,

dot11QBSSLoadOptionImplemented,

dot11QueueRequestOptionImplemented,

dot11TXOPRequestOptionImplemented,

dot11MoreDataAckOptionImplemented,

dot11AssociateinNQBSS,

dot11DLSAllowedinQBSS,

dot11DLSAllowed,

dot11SMTRRMRequest,

dot11SMTRRMReport,

dot11SMTRRMConfig,

dot11AssociateStation,
			dot11AssociateID,
			dot11AssociateFailStation,
			dot11AssociateFailStatus,
			dot11ReassociateStation,
			dot11ReassociateID,
			dot11ReassociateFailStation,
			dot11ReassociateFailStatus,
			dot11RadioMeasurementCapable,
			dot11RadioMeasurementEnabled,
			dot11RRMMeasurementProbeDelay,
			dot11RRMMeasurementPilotPeriod,
			dot11RRMLinkMeasurementEnabled,
			dot11RRMNeighborReportEnabled,
			dot11RRMParallelMeasurementsEnabled,
			dot11RRMRepeatedMeasurementsEnabled,
			dot11RRMBeaconPassiveMeasurementEnabled,
			dot11RRMBeaconActiveMeasurementEnabled,
			dot11RRMBeaconTableMeasurementEnabled,
			dot11RRMBeaconMeasurementReportingConditionsEnabled,
			dot11RRMFrameMeasurementEnabled,
			dot11RRMChannelLoadMeasurementEnabled,
			dot11RRMNoiseHistogramMeasurementEnabled,
			dot11RRMStatisticsMeasaurementEnabled,
			dot11RRMLCIMeasurementEnabled,
			dot11RRMLCIAzimuthEnabled,
			dot11RRMTransmitStreamCategoryMeasurementEnabled,
			dot11RRMTriggeredTransmitStreamCategoryMeasurementEnabled,
			dot11RRMAPChannelReportEnabled,
			dot11RRMMIBEnabled,
			dot11RRMMaxMeasurementDuration,
			dot11RRMNonOperatingChannelMaxMeasurementDuration,
			dot11RRMMeasurementPilotTransmissionInformationEnabled,
			dot11RRMMeasurementPilotCapability,
			dot11RRMNeighborReportTSFOffsetEnabled,
			dot11RRMRCPIMeasurementEnabled,
			dot11RRMRSNIMeasurementEnabled,
			dot11RRMBSSAverageAccessDelayEnabled,
			dot11RRMBSSAvailableAdmissionCapacityEnabled,

dot11FastBSSTransitionImplemented,

dot11LCIDSEImplemented,

dot11LCIDSERequired,

dot11DSERequired,

dot11ExtendedChannelSwitchEnabled,

dot11HighThroughputOptionImplemented,

dot11WirelessNetworkManagementImplemented}

STATUS current

DESCRIPTION

"The SMTbase11 object class provides the necessary support at the STA to manage the processes in the STA so that the STA may work cooperatively as a part of an IEEE 802.11 network, when the STA is capable of multidomain operation. This object group should be implemented when the multidomain capability option is implemented."

::= { dot11Groups 53 }

Annex L

(informative)

An example of encoding a TIM virtual bit map

L.2 Examples

Change the first sentence in L.2 as follows:

The following examples help clarify the use of TIM values, both with and without the Multiple BSSID capability.

Insert the following text and figures at the end of L.2:

The three examples listed above describe the construction of the TIM Virtual Bitmap when the Multiple-BSSID capability is not supported. The following three examples demonstrate how to construct the TIM Virtual Bitmap, when Multiple-BSSID is supported.

The first example with Multiple BSSID is one in which there are eight BSSIDs and the lowest possible AID that can be assigned to any STA in this example is 8. There are no group addressed frames buffered in the AP for any of the eight BSSIDs. However, STAs with AID 9 and AID 11 have unicast frame buffered in the AP. Figure L.4 shows the values of the Bitmap Control and Partial Virtual Bitmap fields that would be part of the TIM information element for this example when either Method A or Method B is used. It is noted that Method B reduces to Method A in this example.

Figure L.4—		Virtual Bitmap Example #4, Method A and Method B

In the next example, there are eight BSSIDs and the lowest possible AID that can be assigned to any STA in this example is 8. There are group addressed frames buffered at the AP for the transmitted BSSID, and the DTIM Count field in the TIM IE of the transmitted BSSID is zero. The non-transmitted BSSID with BSSID Index 3 also has the DTIM Count field set to zero and has buffered group addressed frames. All other non-transmitted BSSIDs have no buffered group addressed frames. In addition, STAs with AID 12, AID 17, AID 22 and AID 24 have data buffered at the AP. Figure L.5 shows the values of the Bitmap Control and Partial Virtual Bitmap fields that would be part of the TIM information element for this example when either Method A or Method B is used. It is noted that Method B reduces to Method A in this example.

Figure L.5—		Virtual Bitmap Example #5, Method A or Method B

In the third example, there are sixteen BSSIDs and the lowest possible AID that can be assigned to any STA is 16. There are no group addressed frames buffered at the AP for the transmitted BSSID, and the DTIM Count field in the TIM IE of the transmitted BSSID is zero. The non-transmitted BSSID Index 3 also has the DTIM Count field set to zero and has group addressed frames buffered at the AP. All other non-transmitted BSSIDs have no buffered group addressed frames. In addition, STA with AID 39 has unicast frames buffered at the AP. Figure L.6 and Figure L.7 show the values of the Bitmap Control and Partial Virtual Bitmap fields that would be part of the TIM information element for this example when Method A and Method B are used, respectively.

Figure L.6—Virtual Bitmap Example #5, Method A

Figure L.7—Virtual Example #5, Method B

L.3 Sample C code

Replace the text in L.3 with the following:

The following C source code illustrates how to construct the TIM Virtual Bitmap.Because this is an illustration, no efficiency or appropriateness for actual implementation is implied.

#include <stdio.h>

#include <limits.h>

#define ADD_TIM_BIT     0

#define REMOVE_TIM_BIT  1

#define TIM_ELEMENT_ID  5

#define TIM_BASE_SIZE   3       /* size of TIM fixed fields */

#define AID_SIZE        2008    /* valid AIDs are numBssids thru 2007 */

#define VBM_SIZE        251     /* size of VBM array = 2008/8 = 251 */

#define MAX_BSSIDS  128 /* maximum possible number of BSSIDs per AP, is a power of 2 */

typedef unsigned char UINT8;

typedef unsigned short int UINT16;

struct  _tim

{

UINT8   Element_id;

UINT8   IELength;

UINT8   DtimCount;

UINT8   DtimPeriod;

UINT8   BitMapControl;

UINT8   PartialVirtualBitMap [VBM_SIZE];

};

UINT8   virtualBitMap [VBM_SIZE];

UINT8   mcast_pending[MAX_BSSIDS] = {0};

UINT8   dtimCount[MAX_BSSIDS] = {0};

UINT8   dtimPeriod[MAX_BSSIDS] = {5};

int     numBssids = 1;

void

Build_TIM (struct _tim * Tim, char  TIM_method, int numBssids)

{

UINT8 octetIndex = 0;

UINT8 octetIndex0 = 0;

UINT8 octetIndex1 = 0;

UINT8 offset = 0;

UINT8 lengthOfPartialVirtualBitMap = 0;

UINT8 bcast_octet = 0;

UINT8 bcast_bit = 0;

UINT16 bssidIndex = 0;

int max_bcast_octetIndex;

int N1;

int N2;

printf("numBssids = %d.\n", numBssids);

/* Compute the largest octet_index for bcast_indication */

max_bcast_octetIndex = (numBssids-1)/8;

/* Initialize PartialVirtualBitMap */

for(octetIndex=0; octetIndex < VBM_SIZE; octetIndex++)

Tim->PartialVirtualBitMap[octetIndex] = 0;

octetIndex = 0;

if (numBssids == 1)

{

/* Find first nonzero octet in the virtual bit map */

for (octetIndex = 0; ((virtualBitMap [octetIndex] == 0) && (octetIndex < VBM_SIZE)); octetIndex++)

/* empty */;

if (octetIndex < VBM_SIZE)

offset = octetIndex & 0xFE;

/* Find last nonzero octet in the virtual bit map */

for (octetIndex = (VBM_SIZE-1); ((virtualBitMap [octetIndex] == 0) && (octetIndex > 0)); octetIndex--)

/* empty */;

lengthOfPartialVirtualBitMap = octetIndex - offset + 1;

Tim->IELength = lengthOfPartialVirtualBitMap + TIM_BASE_SIZE;

Tim->BitMapControl = offset;

/* Copy the virtual bit map octets that are nonzero */

/* Note: A NULL virtualBitMap will still add a single octet of zero */

for (octetIndex = 0; octetIndex < lengthOfPartialVirtualBitMap; octetIndex++)

Tim->PartialVirtualBitMap [octetIndex] = virtualBitMap [offset + octetIndex];

}

if (numBssids > 1)

{

/* Update the broadcast/multicast bits, when numBssids > 1*/

for(bssidIndex = 1; bssidIndex < numBssids; bssidIndex++)

{

bcast_octet = (UINT8) (bssidIndex >> 3);

bcast_bit = (UINT8) (0x01 << (bssidIndex &0x07));

if(mcast_pending[bssidIndex]) {

virtualBitMap[bcast_octet] |= bcast_bit;

}

else{

virtualBitMap[bcast_octet] &= ~bcast_bit;

}

}

if (TIM_method =='A')

{

offset = 0;

octetIndex = 0;

for (octetIndex = (VBM_SIZE-1); (virtualBitMap [octetIndex] == 0) &&

(octetIndex > max_bcast_octetIndex); octetIndex--)

/* empty */;

lengthOfPartialVirtualBitMap = octetIndex - offset + 1;

N2 = octetIndex;

}

if (TIM_method =='B')

{

for (octetIndex1 = (max_bcast_octetIndex + 1); (virtualBitMap [octetIndex1] == 0

&& octetIndex1 < VBM_SIZE); octetIndex1++)

/* empty */;

if ((octetIndex1 < VBM_SIZE) && (octetIndex1 > (max_bcast_octetIndex + 1)) )

{

if ((max_bcast_octetIndex + 1 + octetIndex1) & 0x01 == 0x01 && (octetIndex1 > 1))

octetIndex1 = octetIndex1 & 0xFE;

offset = octetIndex1 - max_bcast_octetIndex - 1;

}

N1 = octetIndex1;

/* find N2, i.e., find the last non-zero octet in the virtual bit map */

for (octetIndex = (VBM_SIZE-1); ((virtualBitMap [octetIndex] == 0) &&

(octetIndex > max_bcast_octetIndex)); octetIndex--)

/* empty */;

N2 = octetIndex;

if (N1 < VBM_SIZE)

lengthOfPartialVirtualBitMap = (max_bcast_octetIndex + 1) + (N2 - N1 +1);

else

lengthOfPartialVirtualBitMap = 1;

}

/* PVB only contains bcast inidation, no buffered ucast traffic */

/* PVB is a single all-0 byte */

for (octetIndex0 = 0; ( octetIndex0 < VBM_SIZE) && (virtualBitMap[octetIndex0] == 0); octetIndex0++)

/* empty */;

if (octetIndex0 == VBM_SIZE)

{

lengthOfPartialVirtualBitMap = 1;

offset = 0;

Tim->IELength = lengthOfPartialVirtualBitMap + TIM_BASE_SIZE;

Tim->BitMapControl = offset;

Tim->PartialVirtualBitMap [0] = virtualBitMap [0];

}

/* PVB only contains bcast inidation, no buffered ucast traffic */

for (octetIndex1 = (max_bcast_octetIndex + 1); (( octetIndex1 < VBM_SIZE) && (virtualBitMap[octetIndex1] == 0)); octetIndex1++)

/* empty */;

if ((octetIndex1 == VBM_SIZE) && (octetIndex0 < VBM_SIZE))

{

lengthOfPartialVirtualBitMap = max_bcast_octetIndex + 1;

offset = 0;

Tim->IELength = lengthOfPartialVirtualBitMap + TIM_BASE_SIZE;

Tim->BitMapControl = offset;

for (octetIndex = 0; octetIndex < (max_bcast_octetIndex + 1); octetIndex++)

Tim->PartialVirtualBitMap [octetIndex] = virtualBitMap [octetIndex];

}

/* PVB contains ucast indication with or without buffered bcast traffic */

if (octetIndex1 < VBM_SIZE)

{

for (octetIndex = 0; octetIndex < (max_bcast_octetIndex + 1); octetIndex++)

Tim->PartialVirtualBitMap [octetIndex] = virtualBitMap [octetIndex];

if (TIM_method == 'A')

{

for (octetIndex = (max_bcast_octetIndex + 1);   (octetIndex <= N2); octetIndex++)

Tim->PartialVirtualBitMap [octetIndex] = virtualBitMap [octetIndex];

Tim->IELength = lengthOfPartialVirtualBitMap + TIM_BASE_SIZE;

Tim->BitMapControl = offset;

}

if (TIM_method == 'B')

{

for (octetIndex = (max_bcast_octetIndex + 1); (octetIndex <= (lengthOfPartialVirtualBitMap - 1)); octetIndex++)

Tim->PartialVirtualBitMap [octetIndex] = virtualBitMap [offset + octetIndex];

Tim->IELength = lengthOfPartialVirtualBitMap + TIM_BASE_SIZE;

Tim->BitMapControl = offset;

}

}

}

Tim->Element_id = TIM_ELEMENT_ID;

Tim->DtimCount = dtimCount[0];

Tim->DtimPeriod = dtimPeriod[0];

/* Update broadcast/ multicast indication bit for transmitted BSSID if necessary */

if ((Tim->DtimCount == 0) && mcast_pending[0])

Tim->BitMapControl |= 0x01;

}

void

Update_VirtualBitMap (UINT16 station_id, UINT8 Action)

{

UINT16  aid = station_id;

UINT8   aid_octet;

UINT8   aid_bit;

UINT16  aid_min= numBssids -1;

if ((aid > 0) && (aid < AID_SIZE))

{

/* Get aid position in Virtual Bit Map. */

aid_octet = (UINT8)(aid >> 3);

aid_bit   = (UINT8)(0x01 << (aid & 0x07));

if (Action == REMOVE_TIM_BIT)

virtualBitMap [aid_octet] &= ~aid_bit;

else

virtualBitMap [aid_octet] |= aid_bit;

}

}

main (void)

{

struct _tim     Tim;

UINT8           ExampleCase;

UINT16          count =0;

char            TIM_method;

ExampleCase = 1;

/* TIM_method='A';   */

TIM_method = 'B';

switch (ExampleCase)

{

/* Nine examples with numBssids = 1, TIM_method =  Method B */

case 1:

mcast_pending[0] = 0;

Update_VirtualBitMap (2, ADD_TIM_BIT);

Update_VirtualBitMap (7, ADD_TIM_BIT);

break;

case 2:

mcast_pending[0]= 1;

Update_VirtualBitMap (2, ADD_TIM_BIT);

Update_VirtualBitMap (7, ADD_TIM_BIT);

Update_VirtualBitMap (22, ADD_TIM_BIT);

Update_VirtualBitMap (24, ADD_TIM_BIT);

break;

case 3:

mcast_pending[0] = 1;

Update_VirtualBitMap (24, ADD_TIM_BIT);

break;

case 4:

mcast_pending[0] = 0;

Update_VirtualBitMap (3, ADD_TIM_BIT);

Update_VirtualBitMap (37, ADD_TIM_BIT);

Update_VirtualBitMap (43, ADD_TIM_BIT);

break;

case 5:

mcast_pending[0] = 0;

Update_VirtualBitMap (35,ADD_TIM_BIT);

break;

case 6:

mcast_pending[0] = 0;

Update_VirtualBitMap (43, ADD_TIM_BIT);

break;

case 7:

mcast_pending[0] = 1;

Update_VirtualBitMap (35, ADD_TIM_BIT);

Update_VirtualBitMap (35, REMOVE_TIM_BIT);

break;

case 8:

mcast_pending[0] = 1;

Update_VirtualBitMap (13, ADD_TIM_BIT);

Update_VirtualBitMap (43, ADD_TIM_BIT);

Update_VirtualBitMap (63, ADD_TIM_BIT);

Update_VirtualBitMap (73, ADD_TIM_BIT);

break;

case 9:

mcast_pending[0] = 1;

Update_VirtualBitMap (2007, ADD_TIM_BIT);

break;

/* Nine examples with numBssids > 1, TIM_method = (Method B) or (Method A)  */

case 10:

numBssids = 4;

Update_VirtualBitMap (numBssids-1+1, ADD_TIM_BIT);

Update_VirtualBitMap (numBssids-1+3, ADD_TIM_BIT);

break;

case 11:

numBssids = 8;

mcast_pending[0] = 1;

mcast_pending[3] = 1;

Update_VirtualBitMap (numBssids-1+4, ADD_TIM_BIT);

Update_VirtualBitMap (numBssids-1+9, ADD_TIM_BIT);

Update_VirtualBitMap (numBssids-1+14, ADD_TIM_BIT);

Update_VirtualBitMap (numBssids-1+16, ADD_TIM_BIT);

break;

case 12:

numBssids = 8;

mcast_pending[5] = 1;

mcast_pending[7] = 1;

Update_VirtualBitMap (numBssids-1+16, ADD_TIM_BIT);

break;

case 13:

numBssids = 8;

mcast_pending[2] = 1;

mcast_pending[7] = 1;

Update_VirtualBitMap (numBssids-1+3, ADD_TIM_BIT);

break;

case 14:

numBssids = 16;

mcast_pending[3] = 1;

Update_VirtualBitMap (numBssids-1+24, ADD_TIM_BIT);

break;

case 15:

numBssids = 16;

break;

case 16:

numBssids = 16;

mcast_pending[5] = 1;

mcast_pending[7] = 1;

Update_VirtualBitMap (numBssids-1+1992, ADD_TIM_BIT);

break;

case 17:

numBssids = 16;

mcast_pending[5] = 1;

mcast_pending[7] = 1;

Update_VirtualBitMap (numBssids-1+1982, ADD_TIM_BIT);

Update_VirtualBitMap (numBssids-1+1984, ADD_TIM_BIT);

break;

case 18:

numBssids = 32;

for(count=0;count<numBssids;count+=2)

mcast_pending[count]=1;

Update_VirtualBitMap (numBssids-1+1, ADD_TIM_BIT);

Update_VirtualBitMap (numBssids-1+2, ADD_TIM_BIT);

Update_VirtualBitMap (numBssids-1+8, ADD_TIM_BIT);

break;

default:

break;

}

Build_TIM (&Tim, TIM_method, numBssids);

printf ("Element_id = %d.\n", Tim.Element_id);

printf ("IELength = %d.\n", Tim.IELength);

printf ("DtimCount = %d.\n", Tim.DtimCount);

printf ("DtimPeriod = %d.\n", Tim.DtimPeriod);

printf ("BitMapControl = 0x%02X\n", Tim.BitMapControl);

if (Tim.IELength - TIM_BASE_SIZE > 0)

{

int octetIndex;

for (octetIndex = 0;  octetIndex < Tim.IELength - TIM_BASE_SIZE;  octetIndex++)

printf ("PartialVirtualBitMap [%d] = 0x%02X\n", octetIndex, Tim.PartialVirtualBitMap [octetIndex]);

}

}

/* The End. */

Annex Q

(normative)

Modify Annex Q as shown:

ASN.1 encoding of the RRM and WNM MIB

-- **********************************************************************

-- * IEEE 802.11 RRM and WNM MIB

-- **************************************************************

-- *The primary interface to the Radio Resource Measurements is meant to be

-- * real-time information obtained through the request/response mechanisms of

-- * RRM. A secondary interface to the measurements is through retention of

-- * information in the MIB. The information, meant to be retained for later

-- * access, includes the MIB entries of Annex Q. Non-SNMP requests for infor-

-- * mation are obtained via object IDs (OIDs) through the NDIS or “wireless”

-- * interfaces in the operating systems. SNMP requests for information are

-- * obtained via SNMP SETs and GETs.

-- * The primary interface to the Radio Resource Measurements and Wireless

-- * Network Management functions is meant to be real-time information

-- * obtained through the request/response mechanisms of RRM and WNM.

-- * A secondary interface to the measurements is through retention of

-- * information in this MIB.  The information, meant to be retained for

-- * later access, includes the MIB entries of Annex Q.  Non-SNMP requests

-- * for information are obtained via object IDs (OIDs) through the NDIS

-- * or “wireless” interfaces in the operating systems.  SNMP requests for

-- *  information are obtained via SNMP SETs and GETs.

-- ********************************************************************
-- * Radio Resource Measurement
-- ********************************************************************

dot11RadioResourceMeasurement OBJECT IDENTIFIER ::= { dot11smt 14 }

-- **********************************************************************

-- *  dot11RRMRequest and dot11RRMReport Usage

-- *

-- * The dot11RRMRequest and dot11RRMReport portions of the RRM MIB

-- * provide access to the Radio Measurement service. By performing

-- * SET operations on the various dot11RRMRequest MIB objects,

-- * radio measurements may be initiated directly on the local STA or
-- * on any peer station within the same BSS. Subsequently, by

-- * performing GET operations on the various dot11RRMReport MIB

-- * objects the results of the requested measurements may be

-- * retrieved.

-- *

-- * In the diagram below, a radio measurement could be initiated

-- * for STA x by performing a MIB.set operation on the RRM MIB of

-- * STA x and specifying the MAC address of STA x in

-- * dot11RRMRqstTargetAdd. Additionally, it is possible to have STA x

-- * request a measurement from STA y by performing a MIB.set operation

-- * on the SME MIB of STA x and specifying the MAC address of STA y in

-- * dot11RRMRqstTargetAdd. In both cases the result of the measurements

-- * can be retrieved by performing a MIB.get operation on the RRM MIB

-- * of STA x upon completion of the measurement.

-- *

-- *

-- *          MIB.Set                                       MIB.Set

-- *            or                                             or

-- *          MIB.Get                                       MIB.Get

-- *    +--------|---------+                           +--------|---------+

-- *    | SME    |         |                           | SME    |         |

-- *    |       \ /        |                           |       \ /        |

-- *    |   +---------+    |                           |   +---------+    |

-- *    |   | RRM and |    |                           |   | RRM and |    |

-- *    |   | WNM MIB |    |                           |   | WNM MIB |    |

-- *    |   |         |    |                           |   |         |    |

-- *    |   +---------+    |                           |   +---------+    |

-- *    |                  |                           |                  |

-- *    |   / \            |                           |   / \            |

-- *    |    |   MREQUEST  |                           |    |   MREQUEST  |

-- *    +----+-------------+                           +----+-------------+

-- *    |    |   MREPORT   |                           |    |   MREPORT   |

-- *    |   \ /  MEASURE   |     Action Frames         |   \ /  MEASURE   |

-- *    |                  | <--Measurement Request--> |                  |

-- *    |                  | <--Measurement Report---> |                  |

-- *    | MLME             |                           | MLME             |

-- *    +------------------+                           +------------------+

-- *        STA x                                 STA y

-- *

-- *

-- * Each STA maintains a single dot11RRMRequestTable in the SME MIB

-- * used to initiate RM Measurement Requests. Each dot11RRMRequestEntry

-- * in the table represents an individual Measurement Request that

-- * makes up a complete Measurement Request Action frame.

-- * Multiple Measurement Requests may be concatenated into a single

-- * Measurement Request Action frame by setting the same

-- * dot11RRMRqstToken value into multiple dot11RRMRequestEntrys.

-- *

-- * Each row, dot11RRMRequestEntry, of the dot11RRMRequestTable

-- * provides read-create access for the initiation of a measurement

-- * request. The dot11RRMRequestNextIndex object can be used to

-- * determine which is the next row available. Each row corresponding to

-- * one measurement in the sequence is created with a dot11RRMRqstRowStatus

-- * set to notInService. Once the dot11RRMRequestEntry(s) have been

-- * created for a desired measurement sequence the corresponding

-- * dot11RRMRqstRowStatus(s) objects are set to active to indicate that

-- * the SME can trigger the appropriate MLME primitives. Upon processing

-- * the request, the SME returns the corresponding dot11RRMRqstRowStatus(s)

-- * object to notInService and are now available for additional

-- * measurement requests.

-- *

-- * After a radio measurement is complete the RRM populates the RRMReport

-- * objects with the results of the measurement. Each STA maintains a set

-- * of RRMReport tables, one for each corresponding measurement type. The

-- * results of the entire measurement sequence are spread across the tables

-- * based on what types of measurements were requested. Each xxxReportEntry

-- * within a xxxReportTable contains a xxxRprtRqstToken that corresponds

-- * to the original dot11RRMRqstToken in the measurement request. So the

-- * results of the measurement can be collected by searching the appropriate

-- * xxxReportTables and retrieve any reports with the matching request

-- * token.

-- *

-- *

-- * Similar structures and mechanisms are used for WNM

-- * Request and Reports.  The WNM MIB defintions follow the RRM MIB definitions
-- * in this Annex.

Insert the follow text as the end of Annex Q:

-- ********************************************************************
-- * Wireless Network Management (WNM)
-- ********************************************************************

dot11WirelessNetworkManagement OBJECT IDENTIFIER ::= { dot11smt 15 }

-- ********************************************************************

-- * Wireless Network Management Requests

-- ********************************************************************

dot11WNMRequest OBJECT IDENTIFIER ::= { dot11WirelessNetworkManagement 1 }

-- ********************************************************************

-- * dot11WNMRequest TABLE

-- ********************************************************************

dot11WNMRequestNextIndex OBJECT-TYPE

SYNTAX Unsigned32(0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Identifies a hint for the next value of dot11WNMRqstIndex to be used in a row creation attempt for dot11WNMRequestTable. If no new rows can be created for some reason, such as memory, processing requirements, etc, the SME shall set this attribute to 0. It shall update this attribute to a proper value other than 0 as soon as it is capable of receiving new measurement requests. The nextIndex is not necessarily sequential nor monotonically increasing."

::= { dot11WNMRequest 1 }

dot11WNMRequestTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11WNMRequestEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This group contains the current list of requests for WNM reports to be issued and have been issued until removed. A network manager adds a WNM request by creating a row with createAndWait row status and then filling in the request parameters/attributes. The request becomes active to be issued when the row status is set to Active. The columnar objects or attributes other than the rowStatus shall not be written if the rowStatus is Active. The request rows can be deleted, if commanded by a network manager via changing the value of dot11WNMRqstRowStatus to Destroy. This may leave orphaned rows if a manager crashes and forgets which rows are being used by it. One recommended way to manage orphaned or finished rows is to delete rows if their dot11WNMRqstRowStatus remains other than Active for longer than a period (recommend at least 5 minutes, RFC 2579). Or another recommended way is to delete older rows as needed based on their dot11WNMRqstTimeStamp values. This can be done by the agent as well as the manager. "

::= { dot11WNMRequest 2 }

dot11WNMRequestEntry OBJECT-TYPE

SYNTAX Dot11WNMRequestEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11WNMRequestTable Indexed by dot11WNMRqstIndex."

INDEX { dot11WNMRqstIndex }

::= { dot11WNMRequestTable 1 }

Dot11WNMRequestEntry ::=

SEQUENCE {

dot11WNMRqstIndex	Unsigned32,

dot11WNMRqstRowStatus	RowStatus,

dot11WNMRqstToken	OCTET STRING,

dot11WNMRqstIfIndex	InterfaceIndex,

dot11WNMRqstType	INTEGER,

dot11WNMRqstTargetAdd	MacAddress,

dot11WNMRqstTimeStamp	TimeTicks,

dot11WNMRqstRndInterval	Unsigned32,

dot11WNMRqstDuration	Unsigned32,

dot11WNMRqstMcstGroup	MacAddress,

dot11WNMRqstMcstTrigCon	OCTET STRING,

dot11WNMRqstMcstRprtTimeout	INTEGER,

dot11WNMRqstMcstTrigTimeout	INTEGER,

dot11WNMRqstLCRRqstSubject	INTEGER,

dot11WNMRqstLCRIntervalUnits	INTEGER,

dot11WNMRqstLCRServiceInterval	INTEGER,

dot11WNMRqstLIRRqstSubject	INTEGER,

dot11WNMRqstLIRIntervalUnits	INTEGER,

dot11WNMRqstLIRServiceInterval	INTEGER,

dot11WNMRqstEventToken	INTEGER,

dot11WNMRqstEventType	INTEGER,

dot11WNMRqstEventResponseLimit	INTEGER,

dot11WNMRqstEventTargetBssid	MacAddress,

dot11WNMRqstEventSourceBssid	MacAddress,

dot11WNMRqstEventTransitTimeThresh	INTEGER,

dot11WNMRqstEventTransitMatchValue	OCTET STRING,

dot11WNMRqstEventFreqTransitCountThresh	INTEGER,

dot11WNMRqstEventFreqTransitInterval	INTEGER,

dot11WNMRqstEventRsnaAuthType	OCTET STRING,

dot11WNMRqstEapType	INTEGER,

dot11WNMRqstEapVendorId	OCTET STRING,

dot11WNMRqstEapVendorType	OCTET STRING,

dot11WNMRqstEventRsnaMatchValue	OCTET STRING,

dot11WNMRqstEventPeerMacAddress	MacAddress,

dot11WNMRqstChanNumber	INTEGER,

dot11WNMRqstRegulatoryClass	INTEGER,

dot11WNMRqstChanNumber	INTEGER,

dot11WNMRqstDiagToken	INTEGER,

dot11WNMRqstDiagType	INTEGER,

dot11WNMRqstDiagTimeout	INTEGER,

dot11WNMRqstDiagBssid	MacAddress,

dot11WNMRqstDiagProfileId	INTEGER,

dot11WNMRqstDiag8021xCredentials	INTEGER,

dot11WNMRqstLCILocIndParams	OCTET STRING,

dot11WNMRqstLCIChanList	OCTET STRING,

dot11WNMRqstLCIBcastRate	INTEGER,

dot11WNMRqstBssTransitQueryReason	INTEGER,

dot11WNMRqstBssTransitReqMode	OCTET STRING,

dot11WNMRqstBssTransitDisocTimer	INTEGER,

dot11WNMRqstBssTransitValidInterval	INTEGER,

dot11WNMRqstBssTransitCandidateList	OCTET STRING,

dot11WNMRqstColocInterfAutoEnable	TruthValue,

dot11WNMRqstColocInterfRptTimeout	INTEGER,

dot11WNMRqstVendorSpecific	OCTET STRING }

dot11WNMRqstIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for WNM Request elements in dot11WNMRequestTable, greater than 0."

::= { dot11WNMRequestEntry 1 }

dot11WNMRqstRowStatus OBJECT-TYPE

SYNTAX RowStatus

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The Row Status column of the current row, used for tracking status of an individual request. When this attribute is set to Active, AND a measurement request can be unambiguously created based on the parameters in the row, then the MLME may proceed to issue the request to its intended targets when appropriate. If not, this attribute may be set to Not-ready immediately to indicate parametric errors. However, it is the network managers

responsibility to correct the error. If the request is successfully issued to the target STA, then the rowStatus is set to notInService."

REFERENCE

"Clause 7.3.2.21"

::= { dot11WNMRequestEntry 2 }

dot11WNMRqstToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates a unique string to identify this request. To guarantee the uniqueness of this token across multiple network managers, it is recommended that this token be prefixed with the IP address of the network manager creating this row. This token is not necessarily equivalent to the measurement tokens in WNM request frames."

::= { dot11WNMRequestEntry 3 }

dot11WNMRqstIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The ifIndex for this row of WNM Request to be issued on."

::= { dot11WNMRequestEntry 4 }

dot11WNMRqstType OBJECT-TYPE

SYNTAX INTEGER {

mcastDiagnostics(0),

locationCivic(1),

locationIdentifier(2),

event(3),

dignostic(4)

locationConfiguration(5)

bssTransitionQuery(6)

bssTransitionRqst(7)

fms(8)

colocInterference(9)

}

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the request type of this WNM request row."

::= { dot11WNMRequestEntry 5 }

dot11WNMRqstTargetAdd OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The MAC address of STA for this row of RRM Request is to be issued to. If

this attribute matches the MAC address of the dot11RRMRqstIfIndex, then measurement request is for this STA itself to carry out."

::= { dot11WNMRequestEntry 6 }

dot11WNMRqstTimeStamp OBJECT-TYPE

SYNTAX TimeTicks

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the SysUpTime Value the last time when the dot11RRMRqstRowStatus is set to active or when this row is created the first time. This attribute shall be set by this STA or AP automatically, not by an SNMP manager."

::= { dot11WNMRequestEntry 7 }

dot11WNMRqstRndInterval OBJECT-TYPE

SYNTAX Unsigned32

UNITS "TUs"

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the upper bound of the random delay to be used prior to making the measurement, expressed in units of TUs. See 11.10.2."

DEFVAL { 0 }

::= { dot11WNMRequestEntry 8 }

dot11WNMRqstDuration OBJECT-TYPE

SYNTAX Unsigned32

UNITS "TUs"

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the preferred or mandatory measurement duration for this Measurement Request. This attribute is ignored if dot11WNMRqstType = LCI Request or Measurement Pause."

DEFVAL { 0 }

::= { dot11WNMRequestEntry 9 }

dot11WNMRqstMcstGroup OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"Multicast Group address indicates the MAC address of the multicast group for which diagnostics are requested. The BSSID shall be set to the wildcard BSSID when the measurement is to be performed on any muliticast group on the operating channel. This attribute is only valid if the dot11WNMRqstType is 10, indicating a maulticast diagnostic request, and is ignored otherwise."

DEFVAL { 'FFFFFFFFFFFF'H }

::= { dot11WNMRequestEntry 10 }

dot11WNMRqstMcstTrigCon OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(1))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the trigger condition for the Multicast Diagnositic request."

::= { dot11WNMRequestEntry 11 }

dot11WNMRqstMcstRprtTimeout OBJECT-TYPE

SYNTAX INTEGER (1..255)

UNITS "100 TUs"

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates a time interval value in units of 100 TU to be used as the threshold value for the Report Timeout trigger condition."

::= { dot11WNMRequestEntry 12 }

dot11WNMRqstMcstTrigTimeout OBJECT-TYPE

SYNTAX INTEGER (1..255)

UNITS "100 TUs"

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the time interval value in units of 100 TU during which a measuring STA does not generate further Multicast Triggered Reports after a trigger condition has been met."

::= { dot11WNMRequestEntry 13 }

dot11WNMRqstLCRRqstSubject OBJECT-TYPE

SYNTAX INTEGER {

local(0),

remote(1)

}

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The attribute indicates the subject of the LCR request."

DEFVAL { 0 }

::= { dot11WNMRequestEntry 14 }

dot11WNMRqstLCRIntervalUnits OBJECT-TYPE

SYNTAX INTEGER {

seconds(0),

minutes(1),

hours(2),

}

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the units used in the LCR Service Interval."

::= { dot11WNMRequestEntry 15 }

dot11WNMRqstLCRServiceInterval OBJECT-TYPE

SYNTAX INTEGER (0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the periodic interval requested for periodic reporting of Location Civic Reports."

::= { dot11WNMRequestEntry 16 }

dot11WNMRqstLIRRqstSubject OBJECT-TYPE

SYNTAX INTEGER {

local(0),

remote(1)

}

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The attribute indicates the subject of the LIR request."

DEFVAL { 0 }

::= { dot11WNMRequestEntry 17 }

dot11WNMRqstLIRIntervalUnits OBJECT-TYPE

SYNTAX INTEGER {

seconds(0),

minutes(1),

hours(2),

}

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the units used in the LIR Service Interval."

::= { dot11WNMRequestEntry 18 }

dot11WNMRqstLIRServiceInterval OBJECT-TYPE

SYNTAX INTEGER (0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the periodic interval requested for periodic reporting of Location Identifier Reports."

::= { dot11WNMRequestEntry 19 }

dot11WNMRqstEventToken OBJECT-TYPE

SYNTAX INTEGER (1..255)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates a unique string to identify this request."

::= { dot11WNMRequestEntry 20 }

dot11WNMRqstEventType OBJECT-TYPE

SYNTAX INTEGER {

transition(0),

rsna(1),

peerToPeer(2),

wnmLog(3),

vendorSpecific(221)

}

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the request type of this WNM Event request."

::= { dot11WNMRequestEntry 21 }

dot11WNMRqstEventResponseLimit OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the maximum number of requested Event Reports to be included in the Event Report Element."

::= { dot11WNMRequestEntry 22 }

dot11WNMRqstEventTargetBssid OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute is used to request that a Transition or RSNA Event Report includes the event entry when the target BSSID is equal to the indicated BSSID.  A transition event is a STA movement or attempted movement from one BSS (the source BSS) in one ESS to another BSS (the target BSS) within the same ESS. The BSSID shall be set to the wildcard BSSID when the transitions to any BSSID is requested."

DEFVAL { 'FFFFFFFFFFFF'H }

::= { dot11WNMRequestEntry 23 }

dot11WNMRqstEventSourceBssid OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute is used to request that a Transition Event Report includes the transition event entry when the source BSSID is equal to the indicated BSSID.  A transition event is a STA movement or attempted movement from one BSS (the source BSS) in one ESS to another BSS (the target BSS) within the same ESS. The BSSID shall be set to the wildcard BSSID when the transitions from any BSSID is requested."

DEFVAL { 'FFFFFFFFFFFF'H }

::= { dot11WNMRequestEntry 24 }

dot11WNMRqstEventTransitTimeThresh OBJECT-TYPE

SYNTAX INTEGER (0..65535)

UNITS "TUs"

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates a value representing the transition time to be used as the threshold value for the Transition Time condition in TUs. The Transition Time is defined in 11.20.3.2"

::= { dot11WNMRequestEntry 25 }

dot11WNMRqstEventTransitMatchValue OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(1))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates a request for the specified transition results that match the bit descriptions of this field.  b0 indicates match when transition is successful.  b1 indicates match when transition fails."

::= { dot11WNMRequestEntry 26 }

dot11WNMRqstEventFreqTransitCountThresh OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the minimum number of matching transitions detected in the measurement duration to generate a Transition Event Report."

::= { dot11WNMRequestEntry 27 }

dot11WNMRqstEventFreqTransitInterval OBJECT-TYPE

SYNTAX INTEGER (0..65535)

UNITS "TUs"

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the sliding window time interval, in TUs, during which the STA detects mathing transitions to determine if the Frequent Transition Count Threshold is exceeded in order to generate a Transition Event Report. "

::= { dot11WNMRequestEntry 28 }

dot11WNMRqstEventRsnaAuthType OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(4))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates one of the AKM suite selectors defined in Table 7-34 in 7.3.2.25.2."

::= { dot11WNMRequestEntry 29 }

dot11WNMRqstEapType OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates a value that identifies a single EAP method and is set to any valid IANA assigned EAP type as defined at http://www.iana.org/assignments/eap-numbers."

::= { dot11WNMRequestEntry 30 }

dot11WNMRqstEapVendorId OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..3))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates a value that identifies the EAP Vendor. The EAP Vendor ID field is included when EAP Type field is set to 254, and is excluded otherwise."

::= { dot11WNMRequestEntry 31 }

dot11WNMRqstEapVendorType OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..4))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates value that identifies the EAP Type as defined by the vendor. The EAP Vendor Type field is included when EAP Type field is set to 254, and is excluded otherwise."

::= { dot11WNMRequestEntry 32 }

dot11WNMRqstEventRsnaMatchValue OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(1))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates a request for the specified transition results that match the bit descriptions of this field.   b0 indicates match when RSNA is successful.  b1 indicates match when RSNA fails."

::= { dot11WNMRequestEntry 33 }

dot11WNMRqstEventPeerMacAddress OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute is used to request that a Peer-to-Peer Event Report includes the transition event entry when the MAC address of the peer STA or IBSS BSSID is equal to the indicated MAC address.   The MAC address shall be set to the wildcard BSSID when the transitions from any peer STA or IBSS BSSID is requested."

DEFVAL { 'FFFFFFFFFFFF'H }

::= { dot11WNMRequestEntry 34 }

dot11WNMRqstRegulatoryClass OBJECT-TYPE

SYNTAX INTEGER(1..255)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the channel set for this WNM request. Country, Regulatory Class and Channel Number together specify the channel frequency and spacing for this measurement request. Valid values of Regulatory Class are shown in Annex J."

REFERENCE

"Annex J"

::= { dot11WNMRequestEntry 35 }

dot11WNMRqstChanNumber OBJECT-TYPE

SYNTAX INTEGER (1..255)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the current operating channel for this WNM request. The Channel Number is only defined within the indicated Regulatory Class for this WNM request."

::= { dot11WNMRequestEntry 36 }

dot11WNMRqstDiagToken OBJECT-TYPE

SYNTAX INTEGER (1..255)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates a unique string to identify this request."

::= { dot11WNMRequestEntry 37 }

dot11WNMRqstDiagType OBJECT-TYPE

SYNTAX INTEGER {

cancelRequest(0),

manufacturerInfoStaRep(1),

configurationProfile(2),

associationDiag(3),

ieee8021xAuthDiag(4),

vendorSpecific(221)

}

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the request type of this WNM Diagnostic request."

::= { dot11WNMRequestEntry 38 }

dot11WNMRqstDiagTimeout OBJECT-TYPE

SYNTAX INTEGER (0..65535)

UNITS "seconds"

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates a value representing the time interval after a Diagnostic Report is generated during which no additional Diagnostic Reports shall be sent."

::= { dot11WNMRequestEntry 39 }

dot11WNMRqstDiagBssid OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates a request for a Diagnostic Report from the indicated BSSID. The BSSID shall be set to the wildcard BSSID when diagnostics from any BSSID is requested."

DEFVAL { 'FFFFFFFFFFFF'H }

::= { dot11WNMRequestEntry 40 }

dot11WNMRqstDiagProfileId OBJECT-TYPE

SYNTAX INTEGER (1..255)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates a unique identifier for referencing a configuration profile available on a device. The value of the identifier can be any arbitrary value, as long as it is uniquely associated to a single configuration profile on the device sending the identifier."

::= { dot11WNMRequestEntry 41 }

dot11WNMRqstDiag8021xCredentials OBJECT-TYPE

SYNTAX INTEGER {

none(0),

pre-sharedKey(1),

configurationProfile(2),

usernamePassword(3),

otherCertificate(4),

oneTimePassword(5),

token(6),

certificateUsernamePasswword(7),

certificateToken(8),

}

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the type of credential used for the 8021x authentication."

::= { dot11WNMRequestEntry 42 }

dot11WNMRqstLCILocIndParams OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(17))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates STA Location reporting characteristics. The format of these Location Indication Parameters are detailed in 7.3.2.66.2"

::= { dot11WNMRequestEntry 43 }

dot11WNMRqstLCIChanList OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..255))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute lists location reporting channel information for this LCI request. Zero length is the null default for this attribute. Each pair of octets indicates a different regulatory class and channel nu,mber for this request. The detailed format for this list of channels ls described in 7.3.2.66.3"

DEFVAL { ''H }

::= { dot11WNMRequestEntry 44 }

dot11WNMRqstLCIBcastRate OBJECT-TYPE

SYNTAX INTEGER (0..65535)

UNITS "0.5Mbps"

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the data rate, in 0.5Mb/s units, at which the STA broadcasts Location Track Notification frames."

::= { dot11WNMRequestEntry 45 }

dot11WNMRqstBssTransitQueryReason OBJECT-TYPE

SYNTAX INTEGER {

unspecified(0),

excessiveFrameLossRatesPoorConditions(1),

excessiveDelayForCurrentTrafficStreams(2),

insufficientQosCapacityForCurrentTrafficStreams(3),

firstAssociationToEss(4),

loadBalancing(5),

betteApFound(6),

deauthenticatedDisassociatedFromPreviousAp(7),

certificateToken(8),

apFailedIeee8021XEapAuthentication(9),

apFailed4wayHandshake(10),

excessiveDataMICFailures(11),

exceededFrameTransmissionRetryLimit(12),

ecessiveBroadcastDisassociations(13),

excessiveBroadcastDeauthentications(14),

previousTransitionFailed(15)

}

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the reason for the BSS Transition Query. The  format for this list of reasons is further detailed in 7.3.2.63.2."

::= { dot11WNMRequestEntry 46 }

dot11WNMRqstBssTransitReqMode OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(1))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the type of BSS request transition.  b0 indicates the Prferred Candidate list is included in this frame.  b1 indicates sn sbridged format for all BSSIDs not listed in this frame. b2 indicates that the STA will be dissassociated for the current A. The format for this field is detailed in 7.4.11.8."

::= { dot11WNMRequestEntry 47 }

dot11WNMRqstBssTransitDisocTimer OBJECT-TYPE

SYNTAX INTEGER (1..255)

UNITS "TBTTs"

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates  the number of beacon transmission times (TBTTs) until the serving AP sends a Disassociation frame to this STA. If the Disassociation Imminent bit of the Request Mode field is set to 0, this field is ignored."

::= { dot11WNMRequestEntry 48 }

dot11WNMRqstBssTransitValidInterval OBJECT-TYPE

SYNTAX INTEGER (1..255)

UNITS "TBTTs"

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates  the number of beacon transmission times (TBTTs) until this recommendation of this BSS transition candidate is no longer valid."

::= { dot11WNMRequestEntry 49 }

dot11WNMRqstBssTransitCandidateList OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..2304))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute lists one or more Neighbor Report elements described in 7.3.2.37. If the STA has no Transition Candidate information in response to the BSS Transition Management Query frame, the candidate list size is set to 0. "

::= { dot11WNMRequestEntry 50 }

dot11WNMRqstColocInterfAutoEnable OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the requesting STA requests the receiving STA to send the Collocated Interference Response frames automatically with the Report Period interval, as defined in 7.4.11.13, or when the STA detects a change in the collocated interference."

::= { dot11WNMRequestEntry 51 }

dot11WNMRqstColocInterfRptTimeout OBJECT-TYPE

SYNTAX INTEGER (0..127)

UNITS "100 TUs"

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the minimum duration between two consecutive Collocated Interference Response frames from the reporting STA."

::= { dot11WNMRequestEntry 52 }

dot11WNMRqstVendorSpecific OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..255))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute provides an envelope for any optional vendor specific subelements which may be included in a WNM equest element. Zero length is the null default for this attribute."

DEFVAL { ''H }

::= { dot11WNMRequestEntry 53}

-- **********************************************************************

-- * End of dot11WNMRequest TABLE

-- **********************************************************************

-- ********************************************************************

-- * Wireless Network Management Reports:

-- * Report tables contain WNM reports received by this STA or

-- * results of WNM requests performed by this STA.

-- ********************************************************************

dot11WNMReport OBJECT IDENTIFIER ::= { dot11WirelessNetworkManagement 2 }

-- ********************************************************************

-- * dot11WNMVendorSpecificReport TABLE

-- ********************************************************************

dot11WNMVendorSpecificReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11WNMVendorSpecificReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains the current list of Vendor Specific reports that have been received by the MLME. The report tables shall be maintained as FIFO to preserve freshness, thus the rows in this table can be deleted for memory constraints or other implementation constraints determined by the vendor. New rows shall have different RprtIndex values than those deleted within the range limitation of the index. One easy way is to monotonically increase RprtIndex for new reports being written in the table."

::= { dot11WNMReport 1 }

dot11WNMVendorSpecificReportEntry OBJECT-TYPE

SYNTAX Dot11WNMVendorSpecificReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11WNMVendorSpecificReportTable Indexed by dot11WNMVendorSpecificRprtIndex."

INDEX { dot11WNMVendorSpecificRprtIndex }

::= { dot11WNMVendorSpecificReportTable 1 }

Dot11WNMVendorSpecificReportEntry ::=

SEQUENCE {

dot11WNMVendorSpecificRprtIndex	Unsigned32,

dot11WNMVendorSpecificRprtRqstToken	OCTET STRING,

dot11WNMVendorSpecificRprtIfIndex	InterfaceIndex,

dot11WNMVendorSpecificRprtContent	OCTET STRING }

dot11WNMVendorSpecificRprtIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for Vendor Specific Report elements in dot11WNMVendorSpecificReportTable, greater than 0."

::= { dot11WNMVendorSpecificReportEntry 1 }

dot11WNMVendorSpecificRprtRqstToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the request token that was indicated in the WNM request that generated this measurement report. This should be an exact match to the original dot11WNMRqstToken attribute. Note that there may be  multiple entries in the table that match this value since a single request may generate multiple WNM reports."

::= { dot11WNMVendorSpecificReportEntry 2 }

dot11WNMVendorSpecificRprtIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The ifIndex for this row of WNMVendorSpecific Report has been received on."

::= { dot11WNMVendorSpecificReportEntry 3 }

dot11WNMVendorSpecificRprtContent OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..255))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute provides an envelope for all the vendor specific subelements which may be included in a WNM Vendor Specific request element. Zero length is the null default for this attribute."

DEFVAL { ''H }

::= { dot11WNMVendorSpecificReportEntry 4 }

-- ********************************************************************

-- * End of dot11WNMVendorSpecificReport TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11WNMMulticastDiagnosticReport TABLE

-- ********************************************************************

dot11WNMMulticastDiagnosticReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11WNMMulticastDiagnosticReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains the current list of Multicast Diagnostic reports that have been received by the MLME. The report tables shall be maintained as FIFO to preserve freshness, thus the rows in this table can be deleted for memory constraints or other implementation constraints determined by the vendor. New rows shall have different RprtIndex values than those deleted within the range limitation of the index. One easy way is to monotonically increase RprtIndex for new reports being written in the table."

::= { dot11WNMReport 2 }

dot11WNMMulticastDiagnosticReportEntry OBJECT-TYPE

SYNTAX Dot11WNMMulticastDiagnosticReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11WNMMulticastDiagnosticReportTable Indexed by dot11WNMMulticastDiagnosticRprtIndex."

INDEX { dot11WNMMulticastDiagnosticRprtIndex }

::= { dot11WNMMulticastDiagnosticReportTable 1 }

Dot11WNMMulticastDiagnosticReportEntry ::=

SEQUENCE {

dot11WNMMulticastDiagnosticRprtIndex	Unsigned32,

dot11WNMMulticastDiagnosticRprtRqstToken	OCTET STRING,

dot11WNMMulticastDiagnosticRprtIfIndex	InterfaceIndex,

dot11WNMMulticastDiagnosticRprtMeasurementTime	TSFType,

dot11WNMMulticastDiagnosticRprtDuration	Unsigned32,

dot11WNMMulticastDiagnosticRprtMcstGroup	MacAddress,

dot11WNMMulticastDiagnosticRprtReason	OCTET STRING,

dot11WNMMulticastDiagnosticRprtRcvdMsduCount	Unsigned32,

dot11WNMMulticastDiagnosticRprtFirstSeqNumber	INTEGER,

dot11WNMMulticastDiagnosticRprtLastSeqNumber	INTEGER,

dot11WNMMulticastDiagnosticRprtMcstRate	INTEGER }

dot11WNMMulticastDiagnosticRprtIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for Multicast Diagnostic Report elements in dot11WNMMulticastDiagnosticReportTable, greater than 0."

::= { dot11WNMMulticastDiagnosticReportEntry 1 }

dot11WNMMulticastDiagnosticRprtRqstToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the request token that was indicated in the WNM request that generated this measurement report. This should be an exact match to the original dot11WNMRqstToken attribute. Note that there may be  multiple entries in the table that match this value since a single request may generate multiple WNM reports."

::= { dot11WNMMulticastDiagnosticReportEntry 2 }

dot11WNMMulticastDiagnosticRprtIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The ifIndex for this row of WNMMulticastDiagnostic Report has been received on."

::= { dot11WNMMulticastDiagnosticReportEntry 3 }

dot11WNMMulticastDiagnosticRprtMeasurementTime OBJECT-TYPE

SYNTAX TSFType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the TSF value at the time when the

measurement started."

::= { dot11WNMMulticastDiagnosticReportEntry 4 }

dot11WNMMulticastDiagnosticRprtDuration OBJECT-TYPE

SYNTAX Unsigned32

UNITS "TUs"

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the actual duration used for this Measurement Request. This attribute is ignored if dot11WNMRqstType = LCI Request or Measurement Pause."

DEFVAL { 0 }

::= { dot11WNMMulticastDiagnosticReportEntry 5 }

dot11WNMMulticastDiagnosticRprtMcstGroup OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"Multicast Group address indicates the MAC address of the multicast group for for this report element."

DEFVAL { 'FFFFFFFFFFFF'H }

::= { dot11WNMMulticastDiagnosticReportEntry 6 }

dot11WNMMulticastDiagnosticRprtReason OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(1))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the reason why the measuring STA sent the Multicast Diagnostics report.   b0 indicates Report Timeout Trigger.  b1 indicates the Performance Measurement.  These are defined further in 7.3.22.10a."

::= { dot11WNMMulticastDiagnosticReportEntry 7 }

dot11WNMMulticastDiagnosticRprtRcvdMsduCount OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the total number of multicast MSDUs with the indicated Multicast MAC Address that were received during the Measurement Duration. For a triggered multicast diagnostics measurement this is the total number of frames received with the indicated Multicast MAC Address."

DEFVAL { 0 }

::= { dot11WNMMulticastDiagnosticReportEntry 8 }

dot11WNMMulticastDiagnosticRprtFirstSeqNumber OBJECT-TYPE

SYNTAX INTEGER (0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the twelve least significant bits of the First Sequence Number field contain the IEEE 802.11 sequence number of the first frame received with destination address equal to the value in the Multicast MAC address field during the measurement period. The four most significant bits of the First Sequence Number field are set to zero.This field is used only if the multicast reporting reason is performance measurement; otherwise, it is set to 0."

DEFVAL { 0 }

::= { dot11WNMMulticastDiagnosticReportEntry 9 }

dot11WNMMulticastDiagnosticRprtLastSeqNumber OBJECT-TYPE

SYNTAX INTEGER (0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the twelve least significant bits of the Last Sequence Number field is the IEEE 802.11 sequence number of the last frame received with destination address equal to the value in the Multicast MAC address field during the measurement period. The four most significant bits of the Last Sequence Number field are set to zero. This field is used only if the multicast reporting reason is performance measurement; otherwise, it is set to 0."

DEFVAL { 0 }

::= { dot11WNMMulticastDiagnosticReportEntry 10 }

dot11WNMMulticastDiagnosticRprtMcstRate OBJECT-TYPE

SYNTAX INTEGER (0..65535)

UNITS "0.5Mbps"

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates  the data rate, in 0.5 Mb/s units, at which the STA requests to receive group addressed frames. The Multicast Rate field is encoded with the MSB set to 1 to indicate that the data rate is in the basic rate set, and set to 0 to indicate that the data rate is not in the basic rate set. The remaining 15 bit value is multiplied by 0.5 Mb/s to indicate the data rate. The Multicast Rate field is set to 0 by the STA to indicate that it has not received a group addressed frame with a valid FCS during the measurement period."

::= { dot11WNMMulticastDiagnosticReportEntry 11 }

-- ********************************************************************

-- * End of dot11WNMMulticastDiagnosticReport TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11WNMLocationCivicReport TABLE

-- ********************************************************************

dot11WNMLocationCivicReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11WNMLocationCivicReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains the current list of Location Civic reports that have been received by the MLME. The report tables shall be maintained as FIFO to preserve freshness, thus the rows in this table can be deleted for memory constraints or other implementation constraints determined by the vendor. New rows shall have different RprtIndex values than those deleted within the range limitation of the index. One easy way is to monotonically increase RprtIndex for new reports being written in the table."

::= { dot11WNMReport 3 }

dot11WNMLocationCivicReportEntry OBJECT-TYPE

SYNTAX Dot11WNMLocationCivicReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11WNMLocationCivicReportTable Indexed by dot11WNMLocationCivicRprtIndex."

INDEX { dot11WNMLocationCivicRprtIndex }

::= { dot11WNMLocationCivicReportTable 1 }

Dot11WNMLocationCivicReportEntry ::=

SEQUENCE {

dot11WNMLocationCivicRprtIndex	Unsigned32,

dot11WNMLocationCivicRprtRqstToken	OCTET STRING,

dot11WNMLocationCivicRprtIfIndex	InterfaceIndex,

dot11WNMLocationCivicRprtContent	OCTET STRING,

dot11WNMLocationCivicRprtLocXAccuracy	INTEGER,

dot11WNMLocationCivicRprtLocYAccuracy	INTEGER,

dot11WNMLocationCivicRprtLocZAccuracy	INTEGER,

dot11WNMLocationCivicRprtCivicLocation	OCTET STRING }

dot11WNMLocationCivicRprtIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for Location Civic Report elements in dot11WNMLocationCivicReportTable, greater than 0."

::= { dot11WNMLocationCivicReportEntry 1 }

dot11WNMLocationCivicRprtRqstToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the request token that was indicated in the WNM request that generated this measurement report. This should be an exact match to the original dot11WNMRqstToken attribute. Note that there may be  multiple entries in the table that match this value since a single request may generate multiple WNM reports."

::= { dot11WNMLocationCivicReportEntry 2 }

dot11WNMLocationCivicRprtIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The ifIndex for this row of WNMLocationCivic Report has been received on."

::= { dot11WNMLocationCivicReportEntry 3 }

dot11WNMLocationCivicRprtLocXAccuracy OBJECT-TYPE

SYNTAX INTEGER (0..65535)

UNITS "0.1M"

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates an estimated accuracy in the X-dimension in 0.1 meter increments, defined by a little endian 16 bit unsigned integer. For example, an accuracy estimate of +/- 5 meters is represented by the number X'32'. If the location accuracy estimate in the X-dimension is unknown the field is set to 65535."

DEFVAL { 65535 }

::= { dot11WNMLocationCivicReportEntry 4 }

dot11WNMLocationCivicRprtLocYAccuracy OBJECT-TYPE

SYNTAX INTEGER (0..65535)

UNITS "0.1M"

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates an estimated accuracy in the Y-dimension in 0.1 meter increments, defined by a little endian 16 bit unsigned integer. For example, an accuracy estimate of +/- 5 meters is represented by the number X'32'. If the location accuracy estimate in the Y-dimension is unknown the field is set to 65535."

DEFVAL { 65535 }

::= { dot11WNMLocationCivicReportEntry 5 }

dot11WNMLocationCivicRprtLocZAccuracy OBJECT-TYPE

SYNTAX INTEGER (0..65535)

UNITS "0.1M"

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates an estimated accuracy in the Z-dimension in 0.1 meter increments, defined by a little endian 16 bit unsigned integer. For example, an accuracy estimate of +/- 5 meters is represented by the number X'32'. If the location accuracy estimate in the Z-dimension is unknown the field is set to 65535."

DEFVAL { 65535 }

::= { dot11WNMLocationCivicReportEntry 6 }

dot11WNMLocationCivicRprtCivicLocation OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates a variable octet field in wich the format of the data is defined in RFC 4776."

::= { dot11WNMLocationCivicReportEntry 7 }

-- ********************************************************************

-- * End of dot11WNMLocationCivicReport TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11WNMLocationIdentifierReport TABLE

-- ********************************************************************

dot11WNMLocationIdentifierReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11WNMLocationIdentifierReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains the current list of Location Identifier reports that have been received by the MLME. The report tables shall be maintained as FIFO to preserve freshness, thus the rows in this table can be deleted for memory constraints or other implementation constraints determined by the vendor. New rows shall have different RprtIndex values than those deleted within the range limitation of the index. One easy way is to monotonically increase RprtIndex for new reports being written in the table."

::= { dot11WNMReport 4 }

dot11WNMLocationIdentifierReportEntry OBJECT-TYPE

SYNTAX Dot11WNMLocationIdentifierReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11WNMLocationIdentifierReportTable Indexed by dot11WNMLocationIdentifierRprtIndex."

INDEX { dot11WNMLocationIdentifierRprtIndex }

::= { dot11WNMLocationIdentifierReportTable 1 }

Dot11WNMLocationIdentifierReportEntry ::=

SEQUENCE {

dot11WNMLocationIdentifierRprtIndex	Unsigned32,

dot11WNMLocationIdentifierRprtRqstToken	OCTET STRING,

dot11WNMLocationIdentifierRprtIfIndex	InterfaceIndex,

dot11WNMLocationIdentifierRprtPublicIdUri	OCTET STRING }

dot11WNMLocationIdentifierRprtIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for Location Identifier Report elements in dot11WNMLocationIdentifierReportTable, greater than 0."

::= { dot11WNMLocationIdentifierReportEntry 1 }

dot11WNMLocationIdentifierRprtRqstToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the request token that was indicated in the WNM request that generated this measurement report. This should be an exact match to the original dot11WNMRqstToken attribute. Note that there may be  multiple entries in the table that match this value since a single request may generate multiple WNM reports."

::= { dot11WNMLocationIdentifierReportEntry 2 }

dot11WNMLocationIdentifierRprtIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The ifIndex for this row of WNMLocationIdentifier Report has been received on."

::= { dot11WNMLocationIdentifierReportEntry 3 }

dot11WNMLocationIdentifierRprtPublicIdUri OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates a variable octet field in URI (aka URL) format that contains the location by reference for the requesting STA."

::= { dot11WNMLocationIdentifierReportEntry 4 }

-- ********************************************************************

-- * End of dot11WNMLocationIdentifierReport TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11WNMEventTransitReport TABLE

-- ********************************************************************

dot11WNMEventTransitReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11WNMEventTransitReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains the current list of Transition Event reports that have been received by the MLME. The report tables shall be maintained as FIFO to preserve freshness, thus the rows in this table can be deleted for memory constraints or other implementation constraints determined by the vendor. New rows shall have different RprtIndex values than those deleted within the range limitation of the index. One easy way is to monotonically increase RprtIndex for new reports being written in the table."

::= { dot11WNMReport 5 }

dot11WNMEventTransitReportEntry OBJECT-TYPE

SYNTAX Dot11WNMEventTransitReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11WNMEventTransitReportTable Indexed by dot11WNMEventTransitRprtIndex."

INDEX { dot11WNMEventTransitRprtIndex }

::= { dot11WNMEventTransitReportTable 1 }

Dot11WNMEventTransitReportEntry ::=

SEQUENCE {

dot11WNMEventTransitRprtIndex	Unsigned32,

dot11WNMEventTransitRprtRqstToken	OCTET STRING,

dot11WNMEventTransitRprtIfIndex	InterfaceIndex,

dot11WNMEventTransitRprtSourceBssid	MacAddress,

dot11WNMEventTransitRprtTargetBssid	MacAddress,

dot11WNMEventTransitRprtTransitTime	INTEGER,

dot11WNMEventTransitRprtTransitReason	INTEGER,

dot11WNMEventTransitRprtTransitResult	INTEGER,

dot11WNMEventTransitRprtSourceRCPI	INTEGER,

dot11WNMEventTransitRprtSourceRSNI	INTEGER,

dot11WNMEventTransitRprtTargetRCPI	INTEGER,

dot11WNMEventTransitRprtTargetRSNI	INTEGER }

dot11WNMEventTransitRprtIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for Transition Event Report elements in dot11WNMEventTransitReportTable, greater than 0."

::= { dot11WNMEventTransitReportEntry 1 }

dot11WNMEventTransitRprtRqstToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the request token that was indicated in the WNM request that generated this measurement report. This should be an exact match to the original dot11WNMRqstToken attribute. Note that there may be  multiple entries in the table that match this value since a single request may generate multiple WNM reports."

::= { dot11WNMEventTransitReportEntry 2 }

dot11WNMEventTransitRprtIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The ifIndex for this row of WNMEventTransit Report has been received on."

::= { dot11WNMEventTransitReportEntry 3 }

dot11WNMEventTransitRprtSourceBssid OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the source BSSID for the reported transition event."

::= { dot11WNMEventTransitReportEntry 4 }

dot11WNMEventTransitRprtTargetBssid OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the target BSSID for the reported transition event."

::= { dot11WNMEventTransitReportEntry 5 }

dot11WNMEventTransitRprtTransitTime OBJECT-TYPE

SYNTAX INTEGER (0..65535)

UNITS "TUs"

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the transition time for the reported transition event in TUs. The Transition time is defined as the time difference between the starting time and the ending time of a transition between APs, even if the transition results in remaining on the same AP. Start and end times for a transition event are defined in 11.20.3.2"

::= { dot11WNMEventTransitReportEntry 6 }

dot11WNMEventTransitRprtTransitReason OBJECT-TYPE

SYNTAX INTEGER {

unspecified(0),

excessiveFrameLossRatesPoorConditions(1),

excessiveDelayForCurrentTrafficStreams(2),

insufficientQosCapacityForCurrentTrafficStreams(3),

firstAssociationToEss(4),

loadBalancing(5),

betteApFound(6),

deauthenticatedDisassociatedFromPreviousAp(7),

certificateToken(8),

apFailedIeee8021XEapAuthentication(9),

apFailed4wayHandshake(10),

excessiveDataMICFailures(11),

exceededFrameTransmissionRetryLimit(12),

ecessiveBroadcastDisassociations(13),

excessiveBroadcastDeauthentications(14),

previousTransitionFailed(15)

}

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the reason for the reported BSS Transition event. The  format for this list of reasons is further detailed in 7.3.2.63.2."

::= { dot11WNMEventTransitReportEntry 7 }

dot11WNMEventTransitRprtTransitResult OBJECT-TYPE

SYNTAX INTEGER (0..65535)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the result of the attempted transition and is set to one of the Status Codes specified in Table7-23 in 7.3.1.9."

::= { dot11WNMEventTransitReportEntry 8 }

dot11WNMEventTransitRprtSourceRCPI OBJECT-TYPE

SYNTAX INTEGER(0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the received channel power of the most recently measured frame from the Source BSSID before the STA reassociates to the Target BSSID. The Source RCPI is reported in dBm, as defined in the RCPI measurement clause for the PHY Type."

::= { dot11WNMEventTransitReportEntry 9 }

dot11WNMEventTransitRprtSourceRSNI OBJECT-TYPE

SYNTAX INTEGER(0..255)

UNITS "0.5 dB"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the received signal to noise indication of the most recently measured frame from the Source BSSID before the STA reassociates to the Target BSSID. The Source RSNI is reported in dB, as defined in 7.3.2.41."

::= { dot11WNMEventTransitReportEntry 10 }

dot11WNMEventTransitRprtTargetRCPI OBJECT-TYPE

SYNTAX INTEGER(0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the received channel power of the first measured frame just after STA reassociates to the Target BSSID. If association with target BSSID failed, the Target RCPI field indicates the received channel power of the most recently measured frame from the Target BSSID. The Target RCPI is reported in dBm, as defined in the RCPI measurement clause for the PHY Type."

::= { dot11WNMEventTransitReportEntry 11 }

dot11WNMEventTransitRprtTargetRSNI OBJECT-TYPE

SYNTAX INTEGER(0..255)

UNITS "0.5 dB"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the received signal to noise indication of the first measured frame just after STA reassociates to the Target BSSID. If association with target BSSID failed, the Target RCPI field indicates the received signal to noise indication of the most recently measured frame from the Target BSSID. The Target RSNI is reported in dB, as defined in 7.3.2.41."

::= { dot11WNMEventTransitReportEntry 12 }

-- ********************************************************************

-- * End of dot11WNMEventTransitReport TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11WNMEventRsnaReport TABLE

-- ********************************************************************

dot11WNMEventRsnaReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11WNMEventRsnaReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains the current list of RSNA Event reports that have been received by the MLME. The report tables shall be maintained as FIFO to preserve freshness, thus the rows in this table can be deleted for memory constraints or other implementation constraints determined by the vendor. New rows shall have different RprtIndex values than those deleted within the range limitation of the index. One easy way is to monotonically increase RprtIndex for new reports being written in the table."

::= { dot11WNMReport 6 }

dot11WNMEventRsnaReportEntry OBJECT-TYPE

SYNTAX Dot11WNMEventRsnaReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11WNMEventRsnaReportTable Indexed by dot11WNMEventRsnaRprtIndex."

INDEX { dot11WNMEventRsnaRprtIndex }

::= { dot11WNMEventRsnaReportTable 1 }

Dot11WNMEventRsnaReportEntry ::=

SEQUENCE {

dot11WNMEventRsnaRprtIndex	Unsigned32,

dot11WNMEventRsnaRprtRqstToken	OCTET STRING,

dot11WNMEventRsnaRprtIfIndex	InterfaceIndex,

dot11WNMEventRsnaRprtTargetBssid	MacAddress,

dot11WNMEventRsnaRprtAuthType	OCTET STRING,

dot11WNMEventRsnaRprtEapMethod	OCTET STRING,

dot11WNMEventRsnaRprtResult	INTEGER,

dot11WNMEventRsnaRprtRsnElement	OCTET STRING }

dot11WNMEventRsnaRprtIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for RSNA Event Report elements in dot11WNMEventRsnaReportTable, greater than 0."

::= { dot11WNMEventRsnaReportEntry 1 }

dot11WNMEventRsnaRprtRqstToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the request token that was indicated in the WNM request that generated this measurement report. This should be an exact match to the original dot11WNMRqstToken attribute. Note that there may be  multiple entries in the table that match this value since a single request may generate multiple WNM reports."

::= { dot11WNMEventRsnaReportEntry 2 }

dot11WNMEventRsnaRprtIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The ifIndex for this row of WNMEventRsna Report has been received on."

::= { dot11WNMEventRsnaReportEntry 3 }

dot11WNMEventRsnaRprtTargetBssid OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the target BSSID for the reported RSNA event."

::= { dot11WNMEventRsnaReportEntry 4 }

dot11WNMEventRsnaRprtAuthType OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(4))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates one of the AKM suite selectors defined in Table 7-34 in 7.3.2.25.2."

::= { dot11WNMEventRsnaReportEntry 5 }

dot11WNMEventRsnaRprtEapMethod OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(1..8))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates a value that identifies the EAP Method. When the Authentication Type field is set to the value of either 00-0F-AC:1 (Authentication negotiated over IEEE 802.1X or using PMKSA caching as defined in 8.4.6.2) or 00-0F-AC:3 (AKM suite selector for Fast BSS Transition as defined in 8.4.3), the EAP Method field contains the IANA assigned EAP type defined at http://www.iana.org/assignments/eap-numbers. The EAP type contains either the legacy type (1 octet) or the expanded type (1 octet type = 254, 3-octet Vendor ID, 4-octet Vendor-Type). The EAP Method field is set to 0 otherwise."

::= { dot11WNMEventRsnaReportEntry 6 }

dot11WNMEventRsnaRprtResult OBJECT-TYPE

SYNTAX INTEGER (0..65535)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the result of the RSNA event and is set to one of the Status Codes specified in Table7-23 in 7.3.1.9."

::= { dot11WNMEventRsnaReportEntry 7 }

dot11WNMEventRsnaRprtRsnElement OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..257))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute contains the entire contents of the negotiated RSN information element at the time of the authentication attempt.  The format of the RSN information element is defined in 7.3.2.25."

DEFVAL { ''H }

::= { dot11WNMEventRsnaReportEntry 8 }

-- ********************************************************************

-- * End of dot11WNMEventRsnaReport TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11WNMEventPeerReport TABLE

-- ********************************************************************

dot11WNMEventPeerReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11WNMEventPeerReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains the current list of Peer-to-Peer Event reports that have been received by the MLME. The report tables shall be maintained as FIFO to preserve freshness, thus the rows in this table can be deleted for memory constraints or other implementation constraints determined by the vendor. New rows shall have different RprtIndex values than those deleted within the range limitation of the index. One easy way is to monotonically increase RprtIndex for new reports being written in the table."

::= { dot11WNMReport 7 }

dot11WNMEventPeerReportEntry OBJECT-TYPE

SYNTAX Dot11WNMEventPeerReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11WNMEventPeerReportTable Indexed by dot11WNMEventPeerRprtIndex."

INDEX { dot11WNMEventPeerRprtIndex }

::= { dot11WNMEventPeerReportTable 1 }

Dot11WNMEventPeerReportEntry ::=

SEQUENCE {

dot11WNMEventPeerRprtIndex	Unsigned32,

dot11WNMEventPeerRprtRqstToken	OCTET STRING,

dot11WNMEventPeerRprtIfIndex	InterfaceIndex,

dot11WNMEventPeerRprtPeerMacAddress	MacAddress,

dot11WNMEventPeerRprtRegulatoryClass	INTEGER,

dot11WNMEventPeerRprtChanNumber	INTEGER,

dot11WNMEventPeerRprtStaTxPower	Integer32,

dot11WNMEventPeerRprtConnTime	INTEGER,

dot11WNMEventPeerRprtPeerStatus	INTEGER }

dot11WNMEventPeerRprtIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for Peer-to-Peer Event Report elements in dot11WNMEventPeerReportTable, greater than 0."

::= { dot11WNMEventPeerReportEntry 1 }

dot11WNMEventPeerRprtRqstToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the request token that was indicated in the WNM request that generated this measurement report. This should be an exact match to the original dot11WNMRqstToken attribute. Note that there may be  multiple entries in the table that match this value since a single request may generate multiple WNM reports."

::= { dot11WNMEventPeerReportEntry 2 }

dot11WNMEventPeerRprtIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The ifIndex for this row of WNMEventPeer Report has been received on."

::= { dot11WNMEventPeerReportEntry 3 }

dot11WNMEventPeerRprtPeerMacAddress OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the MAC address of the peer STA or IBSS BSSID is equal to the indicated MAC address. If this event is for a Peer-to-Peer Link in an infrastructure BSS, this field contains the MAC address of the peer STA. If this event is for a Peer-to-Peer Link in an IBSS, this field contains the BSSID of the IBSS."

::= { dot11WNMEventPeerReportEntry 4 }

dot11WNMEventPeerRprtRegulatoryClass OBJECT-TYPE

SYNTAX INTEGER(1..255)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the channel set for this Peer-to-Peer Event report. Country, Regulatory Class and Channel Number together specify the channel frequency and spacing for this measurement request. Valid values of Regulatory Class are shown in Annex J."

REFERENCE

"Annex J"

::= { dot11WNMEventPeerReportEntry 5 }

dot11WNMEventPeerRprtChanNumber OBJECT-TYPE

SYNTAX INTEGER (1..255)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the current operating channel for this Peer-to-Peer Event report. The Channel Number is only defined within the indicated Regulatory Class for this WNM request."

::= { dot11WNMEventPeerReportEntry 6 }

dot11WNMEventPeerRprtStaTxPower OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute indicates the STA transmit power used for the Peer-to-Peer link.The STA Tx Power field indicates the target transmit power at the antenna in dBm with a tolerance of +/-5dB of the lowest basic rate of the reporting STA."

::= { dot11WNMEventPeerReportEntry 7 }

dot11WNMEventPeerRprtConnTime OBJECT-TYPE

SYNTAX INTEGER (0..16777215)

UNITS "seconds"

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates a value representing the connectioin time for the reported Peer-to-Peer event. If the Peer Status is 0, this field indicates the duration of the Direct Link. If the Peer Status is 1, this field indicates the time difference from the time the Direct Link was established to the current time. If the Peer Status is 2, this field indicates the duration of the IBSS membership. If the Peer Status is 3, this field indicates the time difference from the time the STA joined the IBSS to the current time."

::= { dot11WNMEventPeerReportEntry 8 }

dot11WNMEventPeerRprtPeerStatus OBJECT-TYPE

SYNTAX INTEGER {

directLinkTerminated(0),

directLinkActive(1),

ibssMembershipTerminated(2),

ibssMembershipActive(3)

}

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the peer link connection status."

::= { dot11WNMEventPeerReportEntry 9 }

-- ********************************************************************

-- * End of dot11WNMEventPeerReport TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11WNMEventWNMLogReport TABLE

-- ********************************************************************

dot11WNMEventWNMLogReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11WNMEventWNMLogReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains the current list of WNMLog Event reports that have been received by the MLME. The report tables shall be maintained as FIFO to preserve freshness, thus the rows in this table can be deleted for memory constraints or other implementation constraints determined by the vendor. New rows shall have different RprtIndex values than those deleted within the range limitation of the index. One easy way is to monotonically increase RprtIndex for new reports being written in the table."

::= { dot11WNMReport 8 }

dot11WNMEventWNMLogReportEntry OBJECT-TYPE

SYNTAX Dot11WNMEventWNMLogReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11WNMEventWNMLogReportTable Indexed by dot11WNMEventWNMLogRprtIndex."

INDEX { dot11WNMEventWNMLogRprtIndex }

::= { dot11WNMEventWNMLogReportTable 1 }

Dot11WNMEventWNMLogReportEntry ::=

SEQUENCE {

dot11WNMEventWNMLogRprtIndex	Unsigned32,

dot11WNMEventWNMLogRprtRqstToken	OCTET STRING,

dot11WNMEventWNMLogRprtIfIndex	InterfaceIndex,

dot11WNMEventWNMLogRprtContent	OCTET STRING }

dot11WNMEventWNMLogRprtIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for WNMLog Event Report elements in dot11WNMEventWNMLogReportTable, greater than 0."

::= { dot11WNMEventWNMLogReportEntry 1 }

dot11WNMEventWNMLogRprtRqstToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the request token that was indicated in the WNM request that generated this measurement report. This should be an exact match to the original dot11WNMRqstToken attribute. Note that there may be  multiple entries in the table that match this value since a single request may generate multiple WNM reports."

::= { dot11WNMEventWNMLogReportEntry 2 }

dot11WNMEventWNMLogRprtIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The ifIndex for this row of WNMEventWNMLog Report has been received on."

::= { dot11WNMEventWNMLogReportEntry 3 }

dot11WNMEventWNMLogRprtContent OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..2284))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute provides an envelope for the WNMLog Event subelements required by the WNM request element. Zero length is the null default for this attribute."

DEFVAL { ''H }

::= { dot11WNMEventWNMLogReportEntry 4 }

-- ********************************************************************

-- * End of dot11WNMEventWNMLogReport TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11WNMDiagMfrInfoReport TABLE

-- ********************************************************************

dot11WNMDiagMfrInfoReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11WNMDiagMfrInfoReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains the current list of Manufacturer Information STA reports that have been received by the MLME. The report tables shall be maintained as FIFO to preserve freshness, thus the rows in this table can be deleted for memory constraints or other implementation constraints determined by the vendor. New rows shall have different RprtIndex values than those deleted within the range limitation of the index. One easy way is to monotonically increase RprtIndex for new reports being written in the table."

::= { dot11WNMReport 9 }

dot11WNMDiagMfrInfoReportEntry OBJECT-TYPE

SYNTAX Dot11WNMDiagMfrInfoReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11WNMDiagMfrInfoReportTable Indexed by dot11WNMDiagMfrInfoRprtIndex."

INDEX { dot11WNMDiagMfrInfoRprtIndex }

::= { dot11WNMDiagMfrInfoReportTable 1 }

Dot11WNMDiagMfrInfoReportEntry ::=

SEQUENCE {

dot11WNMDiagMfrInfoRprtIndex	Unsigned32,

dot11WNMDiagMfrInfoRprtRqstToken	OCTET STRING,

dot11WNMDiagMfrInfoRprtIfIndex	InterfaceIndex,

dot11WNMDiagMfrInfoRprtMfrOui	OCTET STRING,

dot11WNMDiagMfrInfoRprtMfrIdString	OCTET STRING,

dot11WNMDiagMfrInfoRprtMfrModelString	OCTET STRING,

dot11WNMDiagMfrInfoRprtMfrSerialNumberString	OCTET STRING,

dot11WNMDiagMfrInfoRprtMfrFirmwareVersion	OCTET STRING,

dot11WNMDiagMfrInfoRprtMfrAntennaType	OCTET STRING,

dot11WNMDiagMfrInfoRprtMfrAntennaGain	OCTET STRING }

dot11WNMDiagMfrInfoRprtIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for Manufacturer Information STA Report elements in dot11WNMDiagMfrInfoReportTable, greater than 0."

::= { dot11WNMDiagMfrInfoReportEntry 1 }

dot11WNMDiagMfrInfoRprtRqstToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the request token that was indicated in the WNM request that generated this measurement report. This should be an exact match to the original dot11WNMRqstToken attribute. Note that there may be  multiple entries in the table that match this value since a single request may generate multiple WNM reports."

::= { dot11WNMDiagMfrInfoReportEntry 2 }

dot11WNMDiagMfrInfoRprtIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The ifIndex for this row of WNMDiagMfrInfo Report has been received on."

::= { dot11WNMDiagMfrInfoReportEntry 3 }

dot11WNMDiagMfrInfoRprtMfrOui OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..5))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the Manufacturer OI for the reported Manufacturer Information STA Diagnostic. The OUI attribute contains an Organizational Unique Identification, the first 24-bits of the network connected device that indicate the specific vendor for that device."

DEFVAL { ''H }

::= { dot11WNMDiagMfrInfoReportEntry 4 }

dot11WNMDiagMfrInfoRprtMfrIdString OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..255))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the Manufacturer ID string for the reported Manufacturer Information STA Diagnostic. The ID attribute contains an ASCII string indicating the manufacturer identifier of the wireless network adaptor. This string is not null terminated."

DEFVAL { ''H }

::= { dot11WNMDiagMfrInfoReportEntry 5 }

dot11WNMDiagMfrInfoRprtMfrModelString OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..255))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the Manufacturer model string for the reported Manufacturer Information STA Diagnostic. The model attribute contains an ASCII string indicating the model of the wireless network adaptor. This string is not null terminated."

DEFVAL { ''H }

::= { dot11WNMDiagMfrInfoReportEntry 6 }

dot11WNMDiagMfrInfoRprtMfrSerialNumberString OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..255))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the Manufacturer serial number string for the reported Manufacturer Information STA Diagnostic. The serial number attribute contains an ASCII string indicating the serial number of the wireless network adaptor. This string is not null terminated."

DEFVAL { ''H }

::= { dot11WNMDiagMfrInfoReportEntry 7 }

dot11WNMDiagMfrInfoRprtMfrFirmwareVersion OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..255))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the Manufacturer firmware version string for the reported Manufacturer Information STA Diagnostic. The firmware version attribute contains an ASCII string identifying the version of firmware currently installed on the wireless network adaptor. This string is not null terminated."

DEFVAL { ''H }

::= { dot11WNMDiagMfrInfoReportEntry 8 }

dot11WNMDiagMfrInfoRprtMfrAntennaType OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..255))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the Manufacturer antenna type string for the reported Manufacturer Information STA Diagnostic. The antenna type attribute contains an ASCII string indicating the type of antenna connected to the wireless network adaptor."

DEFVAL { ''H }

::= { dot11WNMDiagMfrInfoReportEntry 9 }

dot11WNMDiagMfrInfoRprtMfrAntennaGain OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(1))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the Manufacturer antenna gain string for the reported Manufacturer Information STA Diagnostic. The antenna gain attribute contains the peak gain in dBi of the antenna connected to the wireless network adaptor."

DEFVAL { ''H }

::= { dot11WNMDiagMfrInfoReportEntry 10 }

-- ********************************************************************

-- * End of dot11WNMDiagMfrInfoReport TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11WNMDiagConfigProfReport TABLE

-- ********************************************************************

dot11WNMDiagConfigProfReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11WNMDiagConfigProfReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains the current list of Configuration Profile reports that have been received by the MLME. The report tables shall be maintained as FIFO to preserve freshness, thus the rows in this table can be deleted for memory constraints or other implementation constraints determined by the vendor. New rows shall have different RprtIndex values than those deleted within the range limitation of the index. One easy way is to monotonically increase RprtIndex for new reports being written in the table."

::= { dot11WNMReport 10 }

dot11WNMDiagConfigProfReportEntry OBJECT-TYPE

SYNTAX Dot11WNMDiagConfigProfReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11WNMDiagConfigProfReportTable Indexed by dot11WNMDiagConfigProfRprtIndex."

INDEX { dot11WNMDiagConfigProfRprtIndex }

::= { dot11WNMDiagConfigProfReportTable 1 }

Dot11WNMDiagConfigProfReportEntry ::=

SEQUENCE {

dot11WNMDiagConfigProfRprtIndex	Unsigned32,

dot11WNMDiagConfigProfRprtRqstToken	OCTET STRING,

dot11WNMDiagConfigProfRprtIfIndex	InterfaceIndex,

dot11WNMDiagConfigProfRprtProfileId	INTEGER,

dot11WNMDiagConfigProfRprtSupportedRegClasses	OCTET STRING,

dot11WNMDiagConfigProfRprtTxPowerMode 	INTEGER,

dot11WNMDiagConfigProfRprtTxPowerLevels 	OCTET STRING,

dot11WNMDiagConfigProfRprtCipherSuite 	OCTET STRING,

dot11WNMDiagConfigProfRprtAkmSuite 	OCTET STRING,

dot11WNMDiagConfigProfRprtEapMethod 	OCTET STRING,

dot11WNMDiagConfigProfRprtSSID 	OCTET STRING,

dot11WNMDiagConfigProfRprtPowerSaveMode 	INTEGER }

dot11WNMDiagConfigProfRprtIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for Configuration Profile Report elements in dot11WNMDiagConfigProfReportTable, greater than 0."

::= { dot11WNMDiagConfigProfReportEntry 1 }

dot11WNMDiagConfigProfRprtRqstToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the request token that was indicated in the WNM request that generated this measurement report. This should be an exact match to the original dot11WNMRqstToken attribute. Note that there may be  multiple entries in the table that match this value since a single request may generate multiple WNM reports."

::= { dot11WNMDiagConfigProfReportEntry 2 }

dot11WNMDiagConfigProfRprtIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The ifIndex for this row of WNMDiagConfigProf Report has been received on."

::= { dot11WNMDiagConfigProfReportEntry 3 }

dot11WNMDiagConfigProfRprtProfileId OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates a unique identifier for referencing a configuration profile available on a device. The value of the identifier can be any arbitrary value, as long as it is uniquely associated to a single configuration profile on the device sending the identifier."

::= { dot11WNMDiagConfigProfReportEntry 4 }

dot11WNMDiagConfigProfRprtSupportedRegClasses OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..255))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the current Regulatory Class followed by a list of each Supported Regulatory Class, as defined in 7.3.2.51. Each octet contains an integer representing a regulatory class. Zero length is the null default for this attribute."

DEFVAL { ''H }

::= { dot11WNMDiagConfigProfReportEntry 5 }

dot11WNMDiagConfigProfRprtTxPowerMode OBJECT-TYPE

SYNTAX INTEGER {

fixedPowerMode(0),

automaticPowerMode(1)

}

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the power mode of the STA."

::= { dot11WNMDiagConfigProfReportEntry 6 }

dot11WNMDiagConfigProfRprtTxPowerLevels OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(2..255))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute lists the power levels for the STA. Each octet contains an integer representing a power level encoded as a 2's complement value in dBm, rounded to the nearest integer. If the Power Mode is automatic, the list contains only the minimum and the maximum power levels for the STA. If the Power Mode is fixed, the list contains all the fixed power level settings available at this STA."

::= { dot11WNMDiagConfigProfReportEntry 7 }

dot11WNMDiagConfigProfRprtCipherSuite OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(4))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the cipher suite, as defined in Table 7-32. The first three octets indicate the OUI. The last octet indicates the suite type."

::= { dot11WNMDiagConfigProfReportEntry 8 }

dot11WNMDiagConfigProfRprtAkmSuite OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(4))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the AKM suite, as defined in Table 7-34. The first three octets indicate the OUI. The last octet indicates the suite type."

::= { dot11WNMDiagConfigProfReportEntry 9 }

dot11WNMDiagConfigProfRprtEapMethod OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(1..8))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the IANA assigned EAP type as defined at http://www.iana.org/assignments/eap-numbers. The EAP type contains either the legacy type (1 octet) or the expanded type (1 octet type = 254, 3-octet Vendor ID, 4-octet Vendor-Type)."

::= { dot11WNMDiagConfigProfReportEntry 10 }

dot11WNMDiagConfigProfRprtSSID OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(1..32))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the SSID for the diagnostic report, as defined in 7.3.2.1."

::= { dot11WNMDiagConfigProfReportEntry 11 }

dot11WNMDiagConfigProfRprtPowerSaveMode OBJECT-TYPE

SYNTAX INTEGER {

unknownMode(0),

none(1),

psDtims1Mode(2),

psDtims0Mode(3),

uapsdMode(4),

sapsdMode(5),

upsmpMode(6),

spsmpMode(7),

wnmSleepMode(8),

fmsMode(9),

timBroadcastMode(10),

tfsMode(11)

}

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the power save mode in use by the STA, as defined in Table 7-v14."

::= { dot11WNMDiagConfigProfReportEntry 12 }

-- ********************************************************************

-- * End of dot11WNMDiagConfigProfReport TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11WNMDiagAssocReport TABLE

-- ********************************************************************

dot11WNMDiagAssocReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11WNMDiagAssocReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains the current list of Association Diagnostic reports that have been received by the MLME. The report tables shall be maintained as FIFO to preserve freshness, thus the rows in this table can be deleted for memory constraints or other implementation constraints determined by the vendor. New rows shall have different RprtIndex values than those deleted within the range limitation of the index. One easy way is to monotonically increase RprtIndex for new reports being written in the table."

::= { dot11WNMReport 11 }

dot11WNMDiagAssocReportEntry OBJECT-TYPE

SYNTAX Dot11WNMDiagAssocReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11WNMDiagAssocReportTable Indexed by dot11WNMDiagAssocRprtIndex."

INDEX { dot11WNMDiagAssocRprtIndex }

::= { dot11WNMDiagAssocReportTable 1 }

Dot11WNMDiagAssocReportEntry ::=

SEQUENCE {

dot11WNMDiagAssocRprtIndex	Unsigned32,

dot11WNMDiagAssocRprtRqstToken	OCTET STRING,

dot11WNMDiagAssocRprtIfIndex	InterfaceIndex,

dot11WNMDiagAssocRprtBssid	MacAddress,

dot11WNMDiagAssocRprtRegulatoryClass	INTEGER,

dot11WNMDiagAssocRprtChannelNumber	INTEGER,

dot11WNMDiagAssocRprtStatusCode	INTEGER }

dot11WNMDiagAssocRprtIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for Association Diagnostic Report elements in dot11WNMDiagAssocReportTable, greater than 0."

::= { dot11WNMDiagAssocReportEntry 1 }

dot11WNMDiagAssocRprtRqstToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the request token that was indicated in the WNM request that generated this measurement report. This should be an exact match to the original dot11WNMRqstToken attribute. Note that there may be  multiple entries in the table that match this value since a single request may generate multiple WNM reports."

::= { dot11WNMDiagAssocReportEntry 2 }

dot11WNMDiagAssocRprtIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The ifIndex for this row of WNMDiagAssoc Report has been received on."

::= { dot11WNMDiagAssocReportEntry 3 }

dot11WNMDiagAssocRprtBssid OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the BSSID for the target AP for this Assoication Diagnostic Report."

::= { dot11WNMDiagAssocReportEntry 4 }

dot11WNMDiagAssocRprtRegulatoryClass OBJECT-TYPE

SYNTAX INTEGER(1..255)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the regulatory class of the target AP for this Association Diagnostic Report. Valid values of Regulatory Class are shown in Annex J."

REFERENCE

"Annex J"

::= { dot11WNMDiagAssocReportEntry 5 }

dot11WNMDiagAssocRprtChannelNumber OBJECT-TYPE

SYNTAX INTEGER (1..255)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the operating channel of the target AP for this Association Diagnostic Report. The Channel Number is only defined within the indicated Regulatory Class for this WNM request."

::= { dot11WNMDiagAssocReportEntry 6 }

dot11WNMDiagAssocRprtStatusCode OBJECT-TYPE

SYNTAX INTEGER (0..65535)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the result of the association diagnostic and is set to one of the Status Codes specified in Table7-23 in 7.3.1.9."

::= { dot11WNMDiagAssocReportEntry 7 }

-- ********************************************************************

-- * End of dot11WNMDiagAssocReport TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11WNMDiag8021xAuthReport TABLE

-- ********************************************************************

dot11WNMDiag8021xAuthReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11WNMDiag8021xAuthReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains the current list of 802.1X Authentication Diagnostic reports that have been received by the MLME. The report tables shall be maintained as FIFO to preserve freshness, thus the rows in this table can be deleted for memory constraints or other implementation constraints determined by the vendor. New rows shall have different RprtIndex values than those deleted within the range limitation of the index. One easy way is to monotonically increase RprtIndex for new reports being written in the table."

::= { dot11WNMReport 12 }

dot11WNMDiag8021xAuthReportEntry OBJECT-TYPE

SYNTAX Dot11WNMDiag8021xAuthReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11WNMDiag8021xAuthReportTable Indexed by dot11WNMDiag8021xAuthRprtIndex."

INDEX { dot11WNMDiag8021xAuthRprtIndex }

::= { dot11WNMDiag8021xAuthReportTable 1 }

Dot11WNMDiag8021xAuthReportEntry ::=

SEQUENCE {

dot11WNMDiag8021xAuthRprtIndex	Unsigned32,

dot11WNMDiag8021xAuthRprtRqstToken	OCTET STRING,

dot11WNMDiag8021xAuthRprtIfIndex	InterfaceIndex,

dot11WNMDiag8021xAuthRprtBssid	MacAddress,

dot11WNMDiag8021xAuthRprtRegulatoryClass 	INTEGER,

dot11WNMDiag8021xAuthRprtChannelNumber	INTEGER,

dot11WNMDiag8021xAuthRprtEapMethod	OCTET STRING,

dot11WNMDiag8021xAuthRprt8021xCredentials	INTEGER,

dot11WNMDiag8021xAuthRprtStatusCode	INTEGER }

dot11WNMDiag8021xAuthRprtIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for 802.1X Authentication Diagnostic Report elements in dot11WNMDiag8021xAuthReportTable, greater than 0."

::= { dot11WNMDiag8021xAuthReportEntry 1 }

dot11WNMDiag8021xAuthRprtRqstToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the request token that was indicated in the WNM request that generated this measurement report. This should be an exact match to the original dot11WNMRqstToken attribute. Note that there may be  multiple entries in the table that match this value since a single request may generate multiple WNM reports."

::= { dot11WNMDiag8021xAuthReportEntry 2 }

dot11WNMDiag8021xAuthRprtIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The ifIndex for this row of WNMDiag8021xAuth Report has been received on."

::= { dot11WNMDiag8021xAuthReportEntry 3 }

dot11WNMDiag8021xAuthRprtBssid OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the BSSID for the target AP for this Authentication Diagnostic Report."

::= { dot11WNMDiag8021xAuthReportEntry 4 }

dot11WNMDiag8021xAuthRprtRegulatoryClass OBJECT-TYPE

SYNTAX INTEGER(1..255)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the regulatory class of the target AP for this Authentication Diagnostic Report. Valid values of Regulatory Class are shown in Annex J."

REFERENCE

"Annex J"

::= { dot11WNMDiag8021xAuthReportEntry 5 }

dot11WNMDiag8021xAuthRprtChannelNumber OBJECT-TYPE

SYNTAX INTEGER (1..255)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the operating channel of the target AP for this Authentication Diagnostic Report. The Channel Number is only defined within the indicated Regulatory Class for this WNM request."

::= { dot11WNMDiag8021xAuthReportEntry 6 }

dot11WNMDiag8021xAuthRprtEapMethod OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(1..8))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the IANA assigned EAP type as defined at http://www.iana.org/assignments/eap-numbers. The EAP type contains either the legacy type (1 octet) or the expanded type (1 octet type = 254, 3-octet Vendor ID, 4-octet Vendor-Type)."

::= { dot11WNMDiag8021xAuthReportEntry 7 }

dot11WNMDiag8021xAuthRprt8021xCredentials OBJECT-TYPE

SYNTAX INTEGER {

none(0),

preSharedKey(1),

userNamePassword(2),

x509Certificate(3),

otherCertificate(4),

oneTimePassword(5),

token(6),

certificateUserNamePassword(7),

cerrtificateToken(8)

}

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the type of 802.1X credentials used by the STA for this authentication diagnostic."

::= { dot11WNMDiag8021xAuthReportEntry 8 }

dot11WNMDiag8021xAuthRprtStatusCode OBJECT-TYPE

SYNTAX INTEGER (0..65535)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the result of the authentication diagnostic and is set to one of the Status Codes specified in Table7-23 in 7.3.1.9."

::= { dot11WNMDiag8021xAuthReportEntry 9 }

-- ********************************************************************

-- * End of dot11WNMDiag8021xAuthReport TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11WNMLCIReport TABLE

-- ********************************************************************

dot11WNMLCIReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11WNMLCIReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains the current list of Location Configuration reports that have been received by the MLME. The report tables shall be maintained as FIFO to preserve freshness, thus the rows in this table can be deleted for memory constraints or other implementation constraints determined by the vendor. New rows shall have different RprtIndex values than those deleted within the range limitation of the index. One easy way is to monotonically increase RprtIndex for new reports being written in the table."

::= { dot11WNMReport 13 }

dot11WNMLCIReportEntry OBJECT-TYPE

SYNTAX Dot11WNMLCIReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11WNMLCIReportTable Indexed by dot11WNMLCIRprtIndex."

INDEX { dot11WNMLCIRprtIndex }

::= { dot11WNMLCIReportTable 1 }

Dot11WNMLCIReportEntry ::=

SEQUENCE {

dot11WNMLCIRprtIndex	Unsigned32,

dot11WNMLCIRprtRqstToken	OCTET STRING,

dot11WNMLCIRprtIfIndex	InterfaceIndex,

dot11WNMLCIRprtLocIndParams	OCTET STRING,

dot11WNMLCIRprtLCIChanList	OCTET STRING,

dot11WNMLCIRprtLCISubElementID	INTEGER,

dot11WNMLCIRprtLCIStatus	INTEGER,

dot11WNMLCIRprtTxPower	INTEGER,

dot11WNMLCIRprtAntennaID	INTEGER,

dot11WNMLCIRprtAntennaGain	INTEGER,

dot11WNMLCIRprtRCPI	INTEGER,

dot11WNMLCIRprtRSNI	INTEGER,

dot11WNMLCIRprtMotionIndicator	INTEGER,

dot11WNMLCIRprtBearing	INTEGER,

dot11WNMLCIRprtSpeedUnits	INTEGER,

dot11WNMLCIRprtHorizontalSpeed	INTEGER,

dot11WNMLCIRprtVerticalSpeed	INTEGER,

dot11WNMLCIRprtLCIBcastRate	INTEGER,

dot11WNMLCIRprtTODTimestamp	OCTET STRING,

dot11WNMLCIRprtTODTolerance	OCTET STRING,

dot11WNMLCIRprtTODClockRate	INTEGER,

dot11WNMLCIRprtVendorSpecificRprtContent	OCTET STRING }

dot11WNMLCIRprtIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for Location Configuration Report elements in dot11WNMLCIReportTable, greater than 0."

::= { dot11WNMLCIReportEntry 1 }

dot11WNMLCIRprtRqstToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the request token that was indicated in the WNM request that generated this measurement report. This should be an exact match to the original dot11WNMRqstToken attribute. Note that there may be  multiple entries in the table that match this value since a single request may generate multiple WNM reports."

::= { dot11WNMLCIReportEntry 2 }

dot11WNMLCIRprtIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The ifIndex for this row of WNMLCI Report has been received on."

::= { dot11WNMLCIReportEntry 3 }

dot11WNMLCIRprtLocIndParams OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(17))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates STA Location reporting characteristics. The format of these Location Indication Parameters are detailed in 7.3.2.66.2"

::= { dot11WNMLCIReportEntry 4 }

dot11WNMLCIRprtLCIChanList OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..255))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute lists location reporting channel information for this LCI request. Zero length is the null default for this attribute. Each pair of octets indicates a different regulatory class and channel nu,mber for this request. The detailed format for this list of channels ls described in 7.3.2.66.3"

DEFVAL { ''H }

::= { dot11WNMLCIReportEntry 5 }

dot11WNMLCIRprtLCISubElementID OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the specific Location Parameters subelement ID transmitted in the LCI Request frame for this Report. Additional detail and exceptions are described in 7.3.2.66.4"

::= { dot11WNMLCIReportEntry 6 }

dot11WNMLCIRprtLCIStatus OBJECT-TYPE

SYNTAX INTEGER {

successful(0),

failed(1),

refused(2),

incapable(3),

cancelled(4),

}

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the status of this LCI report."

::= { dot11WNMLCIReportEntry 7 }

dot11WNMLCIRprtTxPower OBJECT-TYPE

SYNTAX INTEGER(0..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute indicates the STA transmit power used to transmit the LCI Report. The Tx Power field indicates the target transmit power at the antenna in in a two’s complement integer in dBm with a tolerance of +/-5dB."

::= { dot11WNMLCIReportEntry 8 }

dot11WNMLCIRprtAntennaID OBJECT-TYPE

SYNTAX INTEGER(0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the identifying number for the antenna used to transmit the LCI Report. The value 0 indicates that the antenna identifier is unknown.  'The value 255 indicates that the measurement was made with multiple antennas or that the antenna ID is unknown. that the antenna identifier is unknown. The value 255 indicates that this measurement was made with multiple antennas. The value 1 is used for a STA with only one antenna.  STAs with more than one antenna shall assign Antenna IDs to each antenna as consecutive, ascending numbers. Each  Antenna ID number represents a unique antenna characterized by a fixed relative position,  a fixed relative direction and a peak gain for that position and direction."

::= { dot11WNMLCIReportEntry 9 }

dot11WNMLCIRprtAntennaGain OBJECT-TYPE

SYNTAX INTEGER(0..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute indicates the antenna gain of the antenna used to transmit the LCI Report. The Antenna Gain indicates antenna gain in in a two’s complement integer in dB.  The value -128 indicates the antenna gain is unknown."

::= { dot11WNMLCIReportEntry 10 }

dot11WNMLCIRprtRCPI OBJECT-TYPE

SYNTAX INTEGER(0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the received channel power of the most recently measured Location Configuration Request frame.  The RCPI is reported in dBm, as defined in the RCPI measurement clause for the PHY Type."

::= { dot11WNMLCIReportEntry 11 }

dot11WNMLCIRprtRSNI OBJECT-TYPE

SYNTAX INTEGER(0..255)

UNITS "0.5 dB"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the received signal to noise indication of the most recently measured Location Configuration Request frame.  The RSNI is reported in dB, as defined in 7.3.2.41."

::= { dot11WNMLCIReportEntry 12 }

dot11WNMLCIRprtMotionIndicator OBJECT-TYPE

SYNTAX INTEGER {

stationary(0),

startOfMotion(1),

inMotion(2),

endOfMotion(3),

unknown(4)

}

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the state of the motion indicator."

::= { dot11WNMLCIReportEntry 13 }

dot11WNMLCIRprtBearing OBJECT-TYPE

SYNTAX INTEGER (0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the direction of travel for the STA motion.  Bearing is reported in azimuth degrees from true north, from 0 to 359. The value 65535 indicates that the bearing is unknown."

::= { dot11WNMLCIReportEntry 14 }

dot11WNMLCIRprtSpeedUnits OBJECT-TYPE

SYNTAX INTEGER {

centimetersPerSecond(0),

metersPerSecond(1)

}

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the speed units used to report the vertical and horizontal speed."

::= { dot11WNMLCIReportEntry 15 }

dot11WNMLCIRprtHorizontalSpeed OBJECT-TYPE

SYNTAX INTEGER(0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the horizontal speed of the STA. The value 65535 indicates that the speed is unknown."

::= { dot11WNMLCIReportEntry 16 }

dot11WNMLCIRprtVerticalSpeed OBJECT-TYPE

SYNTAX INTEGER(0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the vertical speed of the STA. The value 65535 indicates that the speed is unknown."

::= { dot11WNMLCIReportEntry 17 }

dot11WNMLCIRprtLCIBcastRate OBJECT-TYPE

SYNTAX INTEGER (0..65535)

UNITS "0.5Mbps"

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the data rate, in 0.5Mb/s units, at which the STA broadcasts Location Track Notification frames."

::= { dot11WNMLCIReportEntry 18 }

dot11WNMLCIRprtTODTimestamp OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(4))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the value of the TOD timestamp counter at a constant interval before the frame carrying the TOD timestamp is transmitted in units specified by the TOD Units field."

::= { dot11WNMLCIReportEntry 19 }

dot11WNMLCIRprtTODTolerance OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(2))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the 95% tolerance of the TOD Timestamp field value"

::= { dot11WNMLCIReportEntry 20 }

dot11WNMLCIRprtTODClockRate OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the clock rate used to generate the values included in the TOD timestamp field.  Details are provided in 7.3.2.66.8."

::= { dot11WNMLCIReportEntry 21 }

dot11WNMLCIRprtVendorSpecificRprtContent OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..255))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute provides an envelope for all the vendor specific subelements which may be included in LCI report element. Zero length is the null default for this attribute."

DEFVAL { ''H }

::= { dot11WNMLCIReportEntry 22 }

-- ********************************************************************

-- * End of dot11WNMLCIReport TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11WNMBssTransitReport TABLE

-- ********************************************************************

dot11WNMBssTransitReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11WNMBssTransitReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains the current list of BSS Transition Management reports that have been received by the MLME. The report tables shall be maintained as FIFO to preserve freshness, thus the rows in this table can be deleted for memory constraints or other implementation constraints determined by the vendor. New rows shall have different RprtIndex values than those deleted within the range limitation of the index. One easy way is to monotonically increase RprtIndex for new reports being written in the table."

::= { dot11WNMReport 14 }

dot11WNMBssTransitReportEntry OBJECT-TYPE

SYNTAX Dot11WNMBssTransitReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11WNMBssTransitReportTable Indexed by dot11WNMBssTransitRprtIndex."

INDEX { dot11WNMBssTransitRprtIndex }

::= { dot11WNMBssTransitReportTable 1 }

Dot11WNMBssTransitReportEntry ::=

SEQUENCE {

dot11WNMBssTransitRprtIndex	Unsigned32,

dot11WNMBssTransitRprtRqstToken	OCTET STRING,

dot11WNMBssTransitRprtIfIndex	InterfaceIndex,

dot11WNMBssTransitRprtStatus	INTEGER,

dot11WNMBssTransitRprtTargetBssid	MacAddress }

dot11WNMBssTransitRprtIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for BSS Transition Management Report elements in dot11WNMBssTransitReportTable, greater than 0."

::= { dot11WNMBssTransitReportEntry 1 }

dot11WNMBssTransitRprtRqstToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the request token that was indicated in the WNM request that generated this measurement report. This should be an exact match to the original dot11WNMRqstToken attribute. Note that there may be  multiple entries in the table that match this value since a single request may generate multiple WNM reports."

::= { dot11WNMBssTransitReportEntry 2 }

dot11WNMBssTransitRprtIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The ifIndex for this row of WNMBssTransit Report has been received on."

::= { dot11WNMBssTransitReportEntry 3 }

dot11WNMBssTransitRprtStatus OBJECT-TYPE

SYNTAX INTEGER {

accept(0),

rejectUnspecified(1),

rejectInsufficientBeacons(2),

rejectInsufficientCapacity(3)

}

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the status of this BSS Transition report."

::= { dot11WNMBssTransitReportEntry 4 }

dot11WNMBssTransitRprtTargetBssid OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the target BSSID for this BSS Transition Report."

::= { dot11WNMBssTransitReportEntry 5 }

-- ********************************************************************

-- * End of dot11WNMBssTransitReport TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11WNMColocInterfReport TABLE

-- ********************************************************************

dot11WNMColocInterfReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11WNMColocInterfReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains the current list of Collocated Interference reports that have been received by the MLME. The report tables shall be maintained as FIFO to preserve freshness, thus the rows in this table can be deleted for memory constraints or other implementation constraints determined by the vendor. New rows shall have different RprtIndex values than those deleted within the range limitation of the index. One easy way is to monotonically increase RprtIndex for new reports being written in the table."

::= { dot11WNMReport 16 }

dot11WNMColocInterfReportEntry OBJECT-TYPE

SYNTAX Dot11WNMColocInterfReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11WNMColocInterfReportTable Indexed by dot11WNMColocInterfRprtIndex."

INDEX { dot11WNMColocInterfRprtIndex }

::= { dot11WNMColocInterfReportTable 1 }

Dot11WNMColocInterfReportEntry ::=

SEQUENCE {

dot11WNMColocInterfRprtIndex	Unsigned32,

dot11WNMColocInterfRprtRqstToken	OCTET STRING,

dot11WNMColocInterfRprtIfIndex	InterfaceIndex,

dot11WNMColocInterfRprtPeriod	INTEGER,

dot11WNMColocInterfRprtInterfLevel	INTEGER,

dot11WNMColocInterfRprtInterfAccuracy	INTEGER,

dot11WNMColocInterfRprtInterfIndex	INTEGER,

dot11WNMColocInterfRprtInterfInterval	Integer32,

dot11WNMColocInterfRprtInterfBurstLength	Integer32,

dot11WNMColocInterfRprtInterfStartTime	Integer32,

dot11WNMColocInterfRprtInterfCenterFreq	INTEGER,

dot11WNMColocInterfRprtInterfBandwidth	INTEGER }

dot11WNMColocInterfRprtIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for Collocated Interference Report elements in dot11WNMColocInterfReportTable, greater than 0."

::= { dot11WNMColocInterfReportEntry 1 }

dot11WNMColocInterfRprtRqstToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the request token that was indicated in the WNM request that generated this measurement report. This should be an exact match to the original dot11WNMRqstToken attribute. Note that there may be  multiple entries in the table that match this value since a single request may generate multiple WNM reports."

::= { dot11WNMColocInterfReportEntry 2 }

dot11WNMColocInterfRprtIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The ifIndex for this row of WNMColocInterf Report has been received on."

::= { dot11WNMColocInterfReportEntry 3 }

dot11WNMColocInterfRprtPeriod OBJECT-TYPE

SYNTAX INTEGER(0..255)

UNITS "100 TU"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates how often the STA automatically reports the collocated interference. The field is in units of 100 TUs. If the Report Period field is set to 0, then the reporting is not periodic, and a report is generated when the STA detects a change in the collocated interference. See 11.20.11 for further details."

::= { dot11WNMColocInterfReportEntry 4 }

dot11WNMColocInterfRprtInterfLevel OBJECT-TYPE

SYNTAX INTEGER(0..255)

UNITS "dBm"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the level of the collocated interference in a 2's complement signed integer in units of dBm averaged over a 4 s period during an interference period. When the interference level is unknown, the field is set to +127dBm. When the interference level is equal or greater than 126dBm, the field is set to +126dBm. If no collocated interference is present the field is set to -128 dBm. The interference level is referenced to the antenna connector used for reception."

::= { dot11WNMColocInterfReportEntry 5 }

dot11WNMColocInterfRprtInterfAccuracy OBJECT-TYPE

SYNTAX INTEGER(0..15)

UNITS "dB"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates an unsigned integer indicating the expected accuracy of the estimate of interference in dB with 95% confidence interval. If the Interference Level field is X (dBm) and the expected accuracy field is Y (dB), the actual interference level is in the range of [X - Y, X +Y] with the probability of 95%. If the accuracy is unknown then the Expected Accuracy field is set to 15."

::= { dot11WNMColocInterfReportEntry 6 }

dot11WNMColocInterfRprtInterfIndex OBJECT-TYPE

SYNTAX INTEGER(0..15)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the interference index that is unique for each type of interference source. The field set to 0 indicates that no collocated interference is present. See 11.20.11 for further details."

::= { dot11WNMColocInterfReportEntry 7 }

dot11WNMColocInterfRprtInterfInterval OBJECT-TYPE

SYNTAX Integer32

UNITS "microseconds"

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute indicates the interval between two successive periods of interference in microseconds. When the interval between two successive periods of interference is variable the field is set to 2E32-1. When the interval between two successive periods of interference is equal or greater than 232-2 the field is set to 2E32-2. If no collocated interference is present the field is set to 0."

::= { dot11WNMColocInterfReportEntry 8 }

dot11WNMColocInterfRprtInterfBurstLength OBJECT-TYPE

SYNTAX Integer32

UNITS "microseconds"

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute indicates the duration of each period of interference in microseconds. When the duration of each period of interference is variable the field is set to 2E32-1)\. When the duration of each period of interference is equal or greater than 2E32-2 the field is set to 2E32-2. If no collocated interference is present the field is set to 0."

::= { dot11WNMColocInterfReportEntry 9 }

dot11WNMColocInterfRprtInterfStartTime OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute contains the least significant 4 octets (i.e. B0-B31) of the TSF timer at the start of the interference burst. When either the Interference Interval or the Interference Burst Length fields are set to 2E32-1, this field indicates the average duty cycle. The average duty cycle value is defined as Round-to-Integer ((2E32-2)[average interference burst length (microsecond)]/[average interference interval (microsecond)]). When the interference is non-periodic the Interference Start Time field is set to 0. If no collocated interference is present the field is set to 0."

::= { dot11WNMColocInterfReportEntry 10 }

dot11WNMColocInterfRprtInterfCenterFreq OBJECT-TYPE

SYNTAX INTEGER (0..65535)

UNITS "2 MHz"

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the center frequency of interference in 2 MHz. When center frequency is unknown or interference covers the whole channel, the center frequency of the channel is reported. If no collocated interference is present the field is set to 0."

::= { dot11WNMColocInterfReportEntry 11 }

dot11WNMColocInterfRprtInterfBandwidth OBJECT-TYPE

SYNTAX INTEGER (0..65535)

UNITS "5 kHz"

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates  the bandwidth at the -3dB roll-off point of the interference signal in 5 kHz. When bandwidth of the interference signal is unknown, the field is set to 65535. When bandwidth of the interference signal is equal or greater than 65534 the field is set to 65534. If no collocated interference is present the field is set to 0."

::= { dot11WNMColocInterfReportEntry 12 }

-- ********************************************************************

-- * End of dot11WNMColocInterfReport TABLE

-- ********************************************************************

dot11SMTWNMRequest OBJECT-GROUP

OBJECTS { dot11WNMRqstIndex,

dot11WNMRqstRowStatus,

dot11WNMRqstToken,

dot11WNMRqstIfIndex,

dot11WNMRqstType,

dot11WNMRqstTargetAdd,

dot11RRMRqstTimeStamp,

dot11WNMRqstRndInterval,

dot11WNMRqstDuration,

dot11WNMRqstMcstGroup,

dot11WNMRqstMcstTrigCon,

dot11WNMRqstMcstRprtTimeout,

dot11WNMRqstMcstTrigTimeout,

dot11WNMRqstLCRRqstSubject,

dot11WNMRqstLCRIntervalUnits,

dot11WNMRqstLCRServiceInterval,

dot11WNMRqstLIRRqstSubject,

dot11WNMRqstLIRIntervalUnits,

dot11WNMRqstLIRServiceInterval,

dot11WNMRqstEventToken,

dot11WNMRqstEventType,

dot11WNMRqstEventResponseLimit,

dot11WNMRqstEventTargetBssid,

dot11WNMRqstEventSourceBssid,

dot11WNMRqstEventTransitTimeThresh,

dot11WNMRqstEventTransitMatchValue,

dot11WNMRqstEventFreqTransitCountThresh,

dot11WNMRqstEventFreqTransitInterval,

dot11WNMRqstEventRsnaAuthType,

dot11WNMRqstEapType,

dot11WNMRqstEapVendorId,

dot11WNMRqstEapVendorType,

dot11WNMRqstEventRsnaMatchValue,

dot11WNMRqstEventPeerMacAddress,

dot11WNMRqstChanNumber,

dot11WNMRqstRegulatoryClass,

dot11WNMRqstChanNumber,

dot11WNMRqstDiagToken,

dot11WNMRqstDiagType,

dot11WNMRqstDiagTimeout,

dot11WNMRqstDiagBssid,

dot11WNMRqstDiagProfileId,

dot11WNMRqstDiag8021xCredentials,

dot11WNMRqstLCILocIndParams,

dot11WNMRqstLCIChanList,

dot11WNMRqstLCIBcastRate,

dot11WNMRqstBssTransitQueryReason,

dot11WNMRqstBssTransitReqMode,

dot11WNMRqstBssTransitDisocTimer,

dot11WNMRqstBssTransitValidIterval,

dot11WNMRqstBssTransitCandidateLis,

dot11WNMRqstColocInterfAutoEnable,

dot11WNMRqstColocInterfRptTimeout,

dot11WNMRqstVendorSpecific,

dot11RRMRqstVendorSpecific }

STATUS current

DESCRIPTION

"The SMTWNMRequest package is a set of attributes that shall be present if the STA supports the WNM service."

::= { dot11Groups 54 }

EDITORIAL NOTE—dot11Groups number assignment needs to be coordinated.

dot11SMTWNMReport OBJECT-GROUP

OBJECTS { dot11WNMVendorSpecificRprtIndex,

dot11WNMVendorSpecificRprtRqstToken,

dot11WNMVendorSpecificRprtIfIndex,

dot11WNMVendorSpecificRprtContent,

dot11WNMMulticastDiagnosticRprtIndex,

dot11WNMMulticastDiagnosticRprtRqstToken,

dot11WNMMulticastDiagnosticRprtIfIndex,

dot11WNMMulticastDiagnosticRprtMeasurementTime,

dot11WNMMulticastDiagnosticRprtDuration,

dot11WNMMulticastDiagnosticRprtMcstGroup,

dot11WNMMulticastDiagnosticRprtReason,

dot11WNMMulticastDiagnosticRprtRcvdMsduCount,

dot11WNMMulticastDiagnosticRprtFirstSeqNumber,

dot11WNMMulticastDiagnosticRprtLastSeqNumber,

dot11WNMMulticastDiagnosticRprtMcstRate,

dot11WNMLocationCivicRprtIndex,

dot11WNMLocationCivicRprtRqstToken,

dot11WNMLocationCivicRprtIfIndex,

dot11WNMLocationCivicRprtContent,

dot11WNMLocationCivicRprtLocXAccuracy,

dot11WNMLocationCivicRprtLocYAccuracy,

dot11WNMLocationCivicRprtLocZAccuracy,

dot11WNMLocationCivicRprtCivicLocation,

dot11WNMLocationIdentifierRprtIndex,

dot11WNMLocationIdentifierRprtRqstToken,

dot11WNMLocationIdentifierRprtIfIndex,

dot11WNMLocationIdentifierRprtPublicIdUri,

dot11WNMEventTransitRprtIndex,

dot11WNMEventTransitRprtRqstToken,

dot11WNMEventTransitRprtIfIndex,

dot11WNMEventTransitRprtSourceBssid,

dot11WNMEventTransitRprtTargetBssid,

dot11WNMEventTransitRprtTransitTime,

dot11WNMEventTransitRprtTransitReason,

dot11WNMEventTransitRprtTransitResult,

dot11WNMEventTransitRprtSourceRCPI,

dot11WNMEventTransitRprtSourceRSNI,

dot11WNMEventTransitRprtTargetRCPI,

dot11WNMEventTransitRprtTargetRSNI,

dot11WNMEventRsnaRprtIndex,

dot11WNMEventRsnaRprtRqstToken,

dot11WNMEventRsnaRprtIfIndex,

dot11WNMEventRsnaRprtTargetBssid,

dot11WNMEventRsnaRprtAuthType,

dot11WNMEventRsnaRprtEapMethod,

dot11WNMEventRsnaRprtResult,

dot11WNMEventRsnaRprtRsnElement,

dot11WNMEventPeerRprtIndex,

dot11WNMEventPeerRprtRqstToken,

dot11WNMEventPeerRprtIfIndex,

dot11WNMEventPeerRprtPeerMacAddress,

dot11WNMEventPeerRprtRegulatoryClass,

dot11WNMEventPeerRprtChanNumber,

dot11WNMEventPeerRprtStaTxPower,

dot11WNMEventPeerRprtConnTime,

dot11WNMEventPeerRprtPeerStatus,

dot11WNMEventWNMLogRprtIndex,

dot11WNMEventWNMLogRprtRqstToken,

dot11WNMEventWNMLogRprtIfIndex,

dot11WNMEventWNMLogRprtContent,

dot11WNMDiagMfrInfoRprtIndex,

dot11WNMDiagMfrInfoRprtRqstToken,

dot11WNMDiagMfrInfoRprtIfIndex,

dot11WNMDiagMfrInfoRprtMfrOui,

dot11WNMDiagMfrInfoRprtMfrIdString,

dot11WNMDiagMfrInfoRprtMfrModelString,

dot11WNMDiagMfrInfoRprtMfrSerialNumberString,

dot11WNMDiagMfrInfoRprtMfrFirmwareVersion,

dot11WNMDiagMfrInfoRprtMfrAntennaType,

dot11WNMDiagMfrInfoRprtMfrAntennaGain,

dot11WNMDiagConfigProfRprtIndex,

dot11WNMDiagConfigProfRprtRqstToken,

dot11WNMDiagConfigProfRprtIfIndex,

dot11WNMDiagConfigProfRprtProfileId,

dot11WNMDiagConfigProfRprtSupportedRegClasses,

dot11WNMDiagConfigProfRprtTxPowerMode,

dot11WNMDiagConfigProfRprtTxPowerLevels,

dot11WNMDiagConfigProfRprtCipherSuite,

dot11WNMDiagConfigProfRprtAkmSuite,

dot11WNMDiagConfigProfRprtEapMethod,

dot11WNMDiagConfigProfRprtSSID,

dot11WNMDiagConfigProfRprtPowerSaveMode,

dot11WNMDiagAssocRprtIndex,

dot11WNMDiagAssocRprtRqstToken,

dot11WNMDiagAssocRprtIfIndex,

dot11WNMDiagAssocRprtBssid,

dot11WNMDiagAssocRprtRegulatoryClass,

dot11WNMDiagAssocRprtChannelNumber,

dot11WNMDiagAssocRprtStatusCode,

dot11WNMDiag8021xAuthRprtIndex,

dot11WNMDiag8021xAuthRprtRqstToken,

dot11WNMDiag8021xAuthRprtIfIndex,

dot11WNMDiag8021xAuthRprtBssid,

dot11WNMDiag8021xAuthRprtRegulatoryClass,

dot11WNMDiag8021xAuthRprtChannelNumber,

dot11WNMDiag8021xAuthRprtEapMethod,

dot11WNMDiag8021xAuthRprt8021xCredentials,

dot11WNMDiag8021xAuthRprtStatusCode,

dot11WNMLCIRprtIndex,

dot11WNMLCIRprtRqstToken,

dot11WNMLCIRprtIfIndex,

dot11WNMLCIRprtLocIndParams,

dot11WNMLCIRprtLCIChanList,

dot11WNMLCIRprtLCISubElementID,

dot11WNMLCIRprtLCIStatus,

dot11WNMLCIRprtTxPower,

dot11WNMLCIRprtAntennaID,

dot11WNMLCIRprtAntennaGain,

dot11WNMLCIRprtRCPI,

dot11WNMLCIRprtRSNI,

dot11WNMLCIRprtMotionIndicator,

dot11WNMLCIRprtBearing,

dot11WNMLCIRprtSpeedUnits,

dot11WNMLCIRprtHorizontalSpeed,

dot11WNMLCIRprtVerticalSpeed,

dot11WNMLCIRprtLCIBcastRate,

dot11WNMLCIRprtTODTimestamp,

dot11WNMLCIRprtTODTolerance,

dot11WNMLCIRprtTODClockRate,

dot11WNMLCIRprtVendorSpecificRprtContent,

dot11WNMBssTransitRprtIndex,

dot11WNMBssTransitRprtRqstToken,

dot11WNMBssTransitRprtIfIndex,

dot11WNMBssTransitRprtStatus,

dot11WNMBssTransitRprtTargetBssid,

dot11WNMColocInterfRprtIndex,

dot11WNMColocInterfRprtRqstToken,

dot11WNMColocInterfRprtIfIndex,

dot11WNMColocInterfRprtPeriod,

dot11WNMColocInterfRprtInterfLevel,

dot11WNMColocInterfRprtInterfAccuracy,

dot11WNMColocInterfRprtInterfIndex,

dot11WNMColocInterfRprtInterfInterval,

dot11WNMColocInterfRprtInterfBurstLength,

dot11WNMColocInterfRprtInterfStartTime,

dot11WNMColocInterfRprtInterfCenterFreq,

dot11WNMColocInterfRprtInterfBandwidth }

STATUS current

DESCRIPTION

"The SMTWNMReport package is a set of attributes that shall be present if the STA supports the WNM service."

::= { dot11Groups 55 }

EDITORIAL NOTE—dot11Groups number assignment needs to be coordinated.

Annex V

(informative)

Location and Time Difference accuracy test

V.1 Location via Time Difference of arrival

The location of a device may be determined in multiple methods, including

—	its signal strength at different sensors

—	the time of flight between the device and different sensors

—	the time difference of arrival between the device and pairs of sensors

A typical implementation of the time difference of arrival method requires that the sensors are co-channel, have synchronized clocks, and receive the same transmission from the device. The sensor's time of arrival measurements are shared, and the device location is determined via multilateration: i.e. each pair of sensor measurements provides a time difference of arrival measurement that represents a hyperbola in 2D space of most likely candidate locations, and the overlap of the multiple hyperbola from multiple pairs of sensors leads to the location estimate. The single time of departure is not relevant in this typical multilateration implementation as it is canceled out when time differences of arrival are computed.

When the sensors are on different channels, such as APs in a typical multi-channel deployment, the device transmits on each channel and thus with multiple times of departure. In this environment it is necessary for the device to advertise each time of departure so that it can be subtracted from the times of arrivals measured by sensors on different channels. Furthermore, the device's clock frequency typically does not match the clock frequency of the synchronized sensors, and so the device should transmit on the same channel multiple widely spaced times in order for the sensors to estimate the device's clock frequency relative to themselves and to be able to suitably scale the device's advertised times of departure.

An example transmission sequence comprises frames transmitted on channels 1, 6, 11, 1 at 2.4 GHz. From this, the device's clock frequency can be determined relative to the synchronized APs on channel 1, and all APs on channels 1, 6 and 11 can measure a time of arrival. These four transmissions enable a single location calculation. The Time of Departure Accuracy test in V.2 is designed to measure errors within such a transmission sequence that would degrade a multi-channel time difference of arrival location scheme.

V.2 Time Difference of departure accuracy test

The Time Difference of Departure accuracy test is an informative description of how time difference of departure accuracy can be measured for any parameterizable PHY waveform. This accuracy test does not apply when the Time of Departure timestamps are exclusively used for timing measurement (See 11.20.5).

The Time Difference of Departure accuracy test is parameterized by the following test parameters:

—	TIME_OF_DEPARTURE(j,i), 1 <= j <= 500, 1 <= i <= I (scalar entries)

—	MULTICHANNEL_SAMPLING_RATE (scalar)

—	FIRST_TRANSITION_FIELD(j,i), 1 <= j <= 500, 1 <= i <= I (waveform entries)

—	SECOND_TRANSITION_FIELD(j,i), 1 <= j <= 500, 1 <= i <= I (waveform entries)

—	TRAINING_FIELD(j,i), 1 <= j <= 500, 1 <= i <= I (waveform entries)

—	TIME_OF_DEPARTURE_ACCURACY_TEST_THRESH (scalar)

TIME_OF_DEPARTURE(j,i) is exposed externally through the TOD Timestamp field in the Time of Departure subelement in Location Track Notification frames.

The Time Difference of Departure accuracy test is performed as follows or in an equivalent or more accurate manner.

The Time Difference of Departure accuracy test is performed by instrumentation capable of converting signals transmitted on one or more channels into a stream of complex samples at fs sample/s or more, with sufficient accuracy in terms of I/Q arm amplitude and phase balance, dc offsets, phase noise, etc, and at a fixed delay from the transmitter. The minimum sampling rate is MULTICHANNEL_SAMPLING_RATE sample/s respectively. A possible embodiment of such a setup is converting the signal to a low IF frequency with a cabled microwave synthesizer, sampling the signal with a digital oscilloscope and decomposing it digitally into quadrature components. The sampled signal is processed in a manner similar to an actual time of arrival processor, according to the following steps:

a)		Repeat steps b) to j) indexed by j = 1,   , 500

b)	Repeat steps c) to i) indexed by i = 1,   , I

c)	Start of frame is detected.

d)	Channel number, coarse and fine frequency offsets are estimated.

e)	The packet is derotated according to estimated frequency offsets.

f)	The transition from FIRST_TRANSITION_FIELD(j,i) to SECOND_TRANSITION_FIELD(j,i) is detected; and fine timing (with one sample resolution) is established.

g)		The TRAINING_FIELD(j,i) of the derotated signal is up-sampled to meet the TIME_OF_DEPARTURE_ACCURACY_TEST_THRESH requirement. For example, a TIME_OF_DEPARTURE_ACCURACY_TEST_THRESH of 1ns requires up-sampling of at least 1 GHz.

h)	The up-sampled signal is cross-correlated with a reference waveform of the TRAINING_FIELD(j,i)

i)	The measured time of departure xj,i is determined from the time of the peak of the magnitude of the cross-correlation.

NOTE — the time of the peak of the magnitude of the cross-correlation is actually a time of arrival measurement that equals the time of departure up to a fixed delay. Since the fixed delay is removed within step j), the fixed need not be known or explicitly compensated for.

j)	Having repeated steps c) to i) I times, the (j,i)th time of departure error ej,i is calculated as TIME_OF_DEPARTURE(j,i) minus the synchronized time of departure. Defining xj = [xj,1,..,xj,I]T as the I measured times of departure, y = [TIME_OF_DEPARTURE(j,1),   TIME_OF_DEPARTURE(j,I)]T, ej = [ej,1,  ej,I]T are the I time of departure errors and Xj = [1Ix1, xj], where 1Ix1 is an Ix1 matrix of ones, then the relative clock intercept, rcij, and slope, rcsj, between device and instrumentation are determined as the linear least squares line of best fit: i.e. [rcij, rcsj]T = (XjTXj)-1XjTyj. With these definitions and calculations, the synchronized time of departure sj = [sj,1,..,sj,I]T equals rcsj xj + rcij 1Ix1, and so the I time of departure errors equal ej = yj  - sj.

k)	Having repeated steps b) to j) 500 times, there are 500 I values of the time of departure errors e = [e1,1,  e500,I]

l)	The Time of Departure accuracy test is passed if (1) the RMS value of e is less than aTxPmdTxStartRMS, and (2) aTxPmdTxStartRMS is less than TIME_OF_DEPARTURE_ACCURACY_TEST_THRESH, where the units of e, aTxPmdTxStartRMS, and TIME_OF_DEPARTURE_ACCURACY_TEST_THRESH are properly accounted for.

NOTE 1— One possible implementation of a time of departure measurement system is a free-running oscillator clocking (a) the digital-to-analog converter(s) used to transmit the packet, (b) a 32-bit continuously-counting counter and (c) a hardware finite state machine such that PMD_TXSTART.request causes a transition within the FSM which in turn causes frame transmission at the DACs a fixed number of cycles later; where the time of departure is recorded as the value of the counter at that transition minus aTxPmdTxStartRFDelay (using TIME_OF_DEPARTURE_ClockRate), where aTxPmdTxStartRFDelay can vary by channel. In this implementation, the principal source of time of departure error is short term oscillator imperfection (e.g. phase noise) and RF group delay variation across channels uncompensated by aTxPmdTxStartRFDelay.

NOTE 2 — 1 ns of time of departure error corresponds to approximately 0.3 m of distance error, so high location accuracy depends upon a tight time of departure standard deviation.

References:

Copyright release for PICS proforma: Users of this standard may freely reproduce the PICS proforma in this annex so that it can be

used for its intended purpose and may further publish the completed PICS.