IEEE P802.11
Wireless LANs

| Initial Sponsor Ballot – Venue Info Update |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-1-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Dave Stephenson | Cisco Systems, Inc. | 170 W. Tasman Dr. San Jose, CA 95134 | +1 408 527 7991 | daves@cisco.com |

Abstract

This submission addresses the following comment from the P802.11u Initial Sponsor Ballot: 7443700023 (row # 278).

This document is based on IEEE 802.11u-d8.01.

7	Frame formats

7.3	Management frame body components

7.3.1	Fields that are not information elements

Insert the new sub-clause shown below, re-numbering subsequent sub-clauses as appropriate:

7.3.1.34	Venue Info field

The Venue Info field is a 2-octet field that provides information describing the venue in which an IEEE 802.11 access network is deployed. It contains Venue Group and Venue Type subfields. The format of Venue Info field is shown in Figure 7-95o115.

|  | Venue Group | Venue Type |
| --- | --- | --- |
| Octets: | 1 | 1 |
| Figure 7-95o115—Venue Info format |  |  |

The Venue Group and Venue Type subfields are both one octet values selected from Table 7-43bc and Table 7-43bd respectively. The entries in Table 7-43bc and Table 7-43bd are drawn from the International Building Code’s Use and Occupancy Classifications [B52].

| Table 7-43bc—Venue Group codes and descriptions |  |
| --- | --- |
| Venue Group Code | Venue Group Description |
| 0 | Unspecified |
| 1 | Assembly |
| 2 | Business |
| 3 | Educational |
| 4 | Factory and Industrial |
| 5 | Institutional |
| 6 | Mercantile |
| 7 | Residential |
| 8 | Storage |
| 9 | Utility and Miscellaneous |
| 10 | Vehicular |
| 11 | Outdoor |
| 12 | Personal Network |
| 13 – 255 | Reserved |

| Table 7-43bd—Venue Type assignments |  |  |
| --- | --- | --- |
| Venue Group | Venue Type Code | Venue Description |
| 0 | 0 | Unspecified |
| 0 | 1 - 255 | Reserved |
| 1 | 0 | Unspecified Assembly |
| 1 | 1 | Arena |
| 1 | 2 | Stadium |
| 1 | 3 | Passenger Terminal (e.g., airport, bus, ferry, train station) |
| 1 | 4 | Amphitheater |
| 1 | 5 | Amusement Park |
| 1 | 6 | Place of Worship |
| 1 | 7 | Convention Center |
| 1 | 8 | Library |
| 1 | 9 | Museum |
| 1 | 10 | Restaurant |
| 1 | 11 | Theater |
| 1 | 12 | Bar |
| 1 | 13 | Coffee Shop |
| 1 | 14 | Zoo or Aquarium |
| 1 | 15 | Emergency Coordination Center |
| 1 | 16 - 255 | Reserved |
| 2 | 0 | Unspecified Business |
| 2 | 1 | Doctor or Dentist office |
| 2 | 2 | Bank |
| 2 | 3 | Fire Station |
| 2 | 4 | Police Station |
| 2 | 6 | Post Office |
| 2 | 7 | Professional Office |
| 2 | 8 | Research and Development Facility |
| 2 | 9 | Attorney Office |
| 2 | 10 – 255 | Reserved |
| 3 | 0 | Unspecified Educational |
| 3 | 1 | School, Primary |
| 3 | 2 | School, Secondary |
| 3 | 3 | University or College |
| 3 | 4-255 | Reserved |
| 4 | 0 | Unspecified Factory and Industrial |
| 4 | 1 | Factory |
| 4 | 2 – 255 | Reserved |
| 5 | 0 | Unspecified Institutional |
| 5 | 1 | Hospital |
| 5 | 2 | Long-Term Care Facility (e.g., Nursing home, Hospice, etc.) |
| 5 | 3 | Alcohol and Drug Re-habilitation Center |
| 5 | 4 | Group Home |
| 5 | 5 | Prison or Jail |
| 5 | 6 – 255 | Reserved |
| 6 | 0 | Unspecified Mercantile |
| 6 | 1 | Retail Store |
| 6 | 2 | Grocery Market |
| 6 | 3 | Automotive Service Station |
| 6 | 4 | Shopping Mall |
| 6 | 5 | Gas Station |
| 6 | 6 – 255 | Reserved |
| 7 | 0 | Unspecified Residential |
| 7 | 1 | Hotel or Motel |
| 7 | 2 | Dormitory |
| 7 | 3 | Boarding House |
| 7 | 4 – 255 | Reserved |
| 8 | 0 – 255 | Reserved |
| 9 | 0 – 255 | Reserved |
| 10 | 0 | Unspecified Vehicular |
| 10 | 1 | Automobile or Truck |
| 10 | 2 | Airplane |
| 10 | 3 | Bus |
| 10 | 4 | Ferry |
| 10 | 5 | Ship or Boat |
| 10 | 6 | Train |
| 10 | 7 | Motor Bike |
| 10 | 8 – 255 | Reserved |
| 11 | 0 | Unspecified Outdoor |
| 11 | 1 | Muni-mesh Network |
| 11 | 2 | City Park |
| 11 | 3 | Rest Area |
| 11 | 4 | Traffic Control |
| 11 | 5– 255 | Reserved |
| 12 | 0 | Reserved |

7.3.2	Information elements

7.3.2.89	Interworking information element

Change the text starting at the 9th paragraph as shown below:

The Venue Info field is defined in 7.3.1.34.

The Venue Group and Venue Type subfields are both one octet values selected from Table 7-43bc and Table 7-43bd respectively. The entries in Table 7-43bc and Table 7-43bd are drawn from the International Building Code’s Use and Occupancy Classifications [B52].

| Table 7-43bc—Venue Group codes and descriptions |  |
| --- | --- |
| Venue Group Code | Venue Group Description |
| 0 | Unspecified |
| 1 | Assembly |
| 2 | Business |
| 3 | Educational |
| 4 | Factory and Industrial |
| 5 | Institutional |
| 6 | Mercantile |
| 7 | Residential |
| 8 | Storage |
| 9 | Utility and Miscellaneous |
| 10 | Vehicular |
| 11 | Outdoor |
| 12 | Personal Network |
| 13 – 255 | Reserved |

| Table 7-43bd—Venue Type assignments |  |  |
| --- | --- | --- |
| Venue Group | Venue Type Code | Venue Description |
| 0 | 0 | Unspecified |
| 0 | 1 - 255 | Reserved |
| 1 | 0 | Unspecified Assembly |
| 1 | 1 | Arena |
| 1 | 2 | Stadium |
| 1 | 3 | Passenger Terminal (e.g., airport, bus, ferry, train station) |
| 1 | 4 | Amphitheater |
| 1 | 5 | Amusement Park |
| 1 | 6 | Place of Worship |
| 1 | 7 | Convention Center |
| 1 | 8 | Library |
| 1 | 9 | Museum |
| 1 | 10 | Restaurant |
| 1 | 11 | Theater |
| 1 | 12 | Bar |
| 1 | 13 | Coffee Shop |
| 1 | 14 | Zoo or Aquarium |
| 1 | 15 | Emergency Coordination Center |
| 1 | 16 - 255 | Reserved |
| 2 | 0 | Unspecified Business |
| 2 | 1 | Doctor or Dentist office |
| 2 | 2 | Bank |
| 2 | 3 | Fire Station |
| 2 | 4 | Police Station |
| 2 | 6 | Post Office |
| 2 | 7 | Professional Office |
| 2 | 8 | Research and Development Facility |
| 2 | 9 | Attorney Office |
| 2 | 10 – 255 | Reserved |
| 3 | 0 | Unspecified Educational |
| 3 | 1 | School, Primary |
| 3 | 2 | School, Secondary |
| 3 | 3 | University or College |
| 3 | 4-255 | Reserved |
| 4 | 0 | Unspecified Factory and Industrial |
| 4 | 1 | Factory |
| 4 | 2 – 255 | Reserved |
| 5 | 0 | Unspecified Institutional |
| 5 | 1 | Hospital |
| 5 | 2 | Long-Term Care Facility (e.g., Nursing home, Hospice, etc.) |
| 5 | 3 | Alcohol and Drug Re-habilitation Center |
| 5 | 4 | Group Home |
| 5 | 5 | Prison or Jail |
| 5 | 6 – 255 | Reserved |
| 6 | 0 | Unspecified Mercantile |
| 6 | 1 | Retail Store |
| 6 | 2 | Grocery Market |
| 6 | 3 | Automotive Service Station |
| 6 | 4 | Shopping Mall |
| 6 | 5 | Gas Station |
| 6 | 6 – 255 | Reserved |
| 7 | 0 | Unspecified Residential |
| 7 | 1 | Hotel or Motel |
| 7 | 2 | Dormitory |
| 7 | 3 | Boarding House |
| 7 | 4 – 255 | Reserved |
| 8 | 0 – 255 | Reserved |
| 9 | 0 – 255 | Reserved |
| 10 | 0 | Unspecified Vehicular |
| 10 | 1 | Automobile or Truck |
| 10 | 2 | Airplane |
| 10 | 3 | Bus |
| 10 | 4 | Ferry |
| 10 | 5 | Ship or Boat |
| 10 | 6 | Train |
| 10 | 7 | Motor Bike |
| 10 | 8 – 255 | Reserved |
| 11 | 0 | Unspecified Outdoor |
| 11 | 1 | Muni-mesh Network |
| 11 | 2 | City Park |
| 11 | 3 | Rest Area |
| 11 | 4 | Traffic Control |
| 11 | 5– 255 | Reserved |
| 12 | 0 | Reserved |

7.3.4	Native Query Protocol elements

7.3.4.2	Venue Name information

Change figure 7-95o127 as shown below:

|  | Info ID | Length | Venue Info | Venue Name Duple #1 | Venue Name Duple #2 (optional) |  |  | Venue Name Duple #N (optional) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 2 | 2 | 2 | variable | variable |  |  | variable |
| Figure 7-95o127—Venue Name information format |  |  |  |  |  |  |  |  |

Change the text as shown below:

The Length is a 2-octet field whose value is equal to two plus the number of octets in Venue Name Duple fields.

The Venue Info field is defined in 7.3.1.34.