IEEE P802.11
Wireless LANs

| TDLS Capability Bits |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-01-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Menzo Wentink | Qualcomm | Breukelen, the Netherlands | +31-65-183-6231 | mwentink@qualcomm.com |

### Overview

This submission proposes the addition of three capability bits related to TDLS, as approved during the November 2009 meeting in Atlanta.

#### 7.3.2.27 Extended Capabilities information element

Modify Table 7-35a as shown:

Table 7-35a—Capabilities field

| Bit | Information | Notes |
| --- | --- | --- |
| 28 | TDLS Support | The TDLS Support subfield indicates support for TDLS, as defined in 11.19. When dot11TunneledDirectLinkSetupImplemented is true, this field is set to one to indicate support for TDLS. The field is set to zero otherwise, to indicate that TDLS is not supported, |
| 29 | Peer U-APSD Buffer STA Support | The Peer U-APSD Buffer STA Support subfield indicates support for the Peer U-APSD Buffer STA function, as defined in 11.2.1.14. When dot11TDLSPeerUAPSDBufferSTAEnabled is true, and to indicate support for Peer U-APSD on this link, the Peer U-APSD Buffer STA Support subfield is set to one. Otherwise, the Peer U-APSD Buffer STA Support subfield is set to zero to indicate that this capability is not supported on this link. |
| 30 | TDLS Peer PSM Support | The TDLS Peer PSM Support subfield indicates support for TDLS Peer PSM, as defined in 11.2.1.13. When dot11TDLSPeerPSMEnabled is true, and to indicate support for TDLS Peer PSM on this link, the TDLS Peer PSM Support subfield is set to one. Otherwise, the TDLS Peer PSM Support subfield is set to zero to indicate that this capability is not supported on this link. |
| 31 | TDLS channel switching | When dot11TDLSChannelSwitchingEnabled is true, and to indicate that the STA supports Tunneled Direct Link Setup (TDLS) with TDLS Channel Switching on this link as described in 11.19, the TDLS Channel Switching capability subfield is set to one. Otherwise, the TDLS Channel Switching subfield is set to zero to indicate that this capability is not supported on this link. |
| 32 | TDLS Prohibited | The TDLS Prohibited subfield indicates whether the use of TDLS is prohibited. The field is set to on to indicate that TDLS is prohibited, and to to zero to indicate that TDLS is allowed. |
| 33 | TDLS Channel Switching Prohibited | The TDLS Channel Switching Prohibited subfield indicates whether the use of TDLS Channel Switching is prohibited. The field is set to on to indicate that TDLS Channel Switching is prohibited, and to to zero to indicate that TDLS Channel Switching is allowed. |

Add the following text at the end of 11.19:

TDLS shall not be used when the TDLS Prohibited subfield included in the Extended Capability element of the Association Response frame or Reassociation Response frame that led to the current association is set to one.

Add the following text at the end of 11.19.4:

TDLS Channel Switching shall not be used when the TDLS Channel Switching Prohibited subfield included in the Extended Capability element of the Association Response frame or Reassociation Response frame that led to the current association is set to one.