IEEE P802.11
Wireless LANs

| 3rd Party Location Request/Response |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-01-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Gabor Bajko | Nokia | 313 Fairchild dr. Mountain View, CA, 94043 | +1 858 717 xxxx | gabor.bajko@nokia.com |
| Leonardo Estevez | TI | 12500 TI Blvd. Dallas, TX 75243 | 214-616-5193 | leonardo@ti.com |

Introduction

This proposal addresses Comment ID 7383800023

This proposal is based on Draft P802.11v_D7.04

The purpose of the submission is to extend the Measurement Request and Measurement Report to allow a STA to indicate the privacy preferences of its own location. Ie, the location of a STA indicated as private is not to be distributed to a third party, while a location indicated as public can be distributed to a third party requestor.

- Make the following changes to  Table 7-29l:

- Table 7-29l – Location Subject Definition

| - Value | - Location Subject |
| --- | --- |
| - 0 | - Location Subject Local |
| - 1 | - Location Subject Remote |
| - 2 | - Location Subject 3rd Party |
| - 3-255 | - Reserved |

- Make the following changes to  Table 7-29m:

- Table 7-29m -- Optional Subelement IDs for LCI Request

| - Subelement ID | - Name | - Length Field (octets) | - Extensible |
| --- | --- | --- | --- |
| - 0 | - Reserved |  |  |
| - 1 | - Azimuth Request | - 1 | - yes |
| - 2 | - Location Privacy | - 1 | - yes |
| - 3 | - MAC Address | - 6 | - no |
| - 4-220 | - Reserved |  |  |
| - 221 | - Vendor Specific | - 1 to 246 |  |
| - 222-255 | - Reserved |  |  |

- The Location Privacy sub-element is shown in Figure 7-62zz:

| - Sub-element ID | - Length | - Location Privacy |
| --- | --- | --- |

- Octets:		1				1				1

- A value of zero for Location Privacy indicates that the location of the subject is to be kept private.

- A value of 1 for Location Privacy indicates that the location of the subject can be reported to a third party STA. A Location Privacy subelement may only be present when the Location Subject Value is set to zero.

- The MAC Address subelement field is only present when the Location Subject Value is set to 3 and indicates the MAC address of the STA whose location is requested.

- Make the following changes to  Table 7-31h:

- Table 7-31h -- Optional Subelement IDs for Location Configuration Information Report

| - Subelement ID | - Name | - Length Field (octets) | - Extensible |
| --- | --- | --- | --- |
| - 0 | - Reserved |  |  |
| - 1 | - Azimuth Report | - 2 | - yes |
| - 2 | - Location Privacy | - 1 | - yes |
| - 3-220 | - Reserved |  |  |
| - 221 | - Vendor Specific | - 1 to 232 |  |
| - 222-255 | - Reserved |  |  |

- The Location Privacy sub-element is shown in Figure 7-62yy:

| - Sub-element ID | - Length | - Location Privacy |
| --- | --- | --- |

- Octets:		1				1					1

- A value of zero for Location Privacy indicates that the location of the subject is to be kept private.

- A value of 1 for Location Privacy indicates that the location of the subject can be reported to a third party STA. A Location Privacy subelement may only be present when the Location Subject Value is set to 1 or 2.

- Insert the following text to section11.10.8.6, after the paragraph starting with “An LCI request shall indicate …” and section 11.10.8.9, after the paragraph starting with “A Location Civic Request shall indicate …”:

- When the Location Subject field is set to the value 2, the requesting STA requests the location of the STA with the MAC address specified in the sub-element ID 3. The responder STA will only provide the location of the STA with the specified MAC address to the requestor, if the responder STA received previously an explicit indication from the STA with the specified MAC address, that its location can be shared with other STAs.

- Make the following changes to  Table 7-62r4:

- Figure 7-62r4—Location Civic Request field format

| - Location Subject | - Location Service Interval Units | - Location Service Interval | - Optional Sub-element |
| --- | --- | --- | --- |

- The Optional Sub-Element sub-field is defined in Table 7-29m.

- Make the following changes to  Table 7-68q3:

| - Civic Location | - Optional Sub-elements |
| --- | --- |

- Figure 7-68q3—Location Civic Report field format

- The Optional Sub-Element sub-field is defined in Table 7-31h.