IEEE P802.11
Wireless LANs

| 3rd Party Location Request/Response |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-04-01 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Gabor Bajko | Nokia | 313 Fairchild dr. Mountain View, CA, 94043 | +1 858 717 xxxx | gabor.bajko@nokia.com |
| Leonardo Estevez | TI | 12500 TI Blvd. Dallas, TX 75243 | 214-616-5193 | leonardo@ti.com |

Introduction

This proposal addresses Comment ID 7383800023

This proposal is based on Draft P802.11v_D7.04

The purpose of the submission is to extend the Measurement Request and Measurement Report to allow a STA to indicate the privacy preferences of its own location. Ie, the location of a STA indicated as private is not to be distributed to a third party, while a location indicated as public can be distributed to a third party requestor.

- Add the following text into section 3:

- 3. Definitions

3.266 Location Subject 3rd party: The term used when the location request is for the location of a STA other than the requesting STA or the reporting STA, i.e., when the requesting STA asks “Where is he/she?”.

- Make the following changes to  Table 7-29l (from the base spec):

- Table 7-29l – Location Subject Definition

| - Value | - Location Subject |
| --- | --- |
| - 0 | - LCI Subject Local |
| - 1 | - LCI Subject Remote |
| - 2 | - Location Subject 3rd Party |
| - 3-255 | - Reserved |

- Make the following changes to  Table 7-29m (from the base spec):

- Table 7-29m -- Optional Subelement IDs for LCI Request

| - Subelement ID | - Name | - Length Field (octets) | - Extensible |
| --- | --- | --- | --- |
| - 0 | - Reserved |  |  |
| - 1 | - Azimuth Request | - 1 | - yes |
| - 2 | - Requestor MAC address | - 6 | - no |
| - 3 | - Target MAC address | - 6 | - no |
| - 4-220 | - Reserved |  |  |
| - 221 | - Vendor Specific | - 1 to 246 |  |
| - 222-255 | - Reserved |  |  |

The Requestor MAC Address is the MAC address of the STA requesting for the Location Information and it is present whenever the location subject definition field is set to 2. The format of the Requestor MAC address field is shown in Figure 7-62xx.

| Subelement ID | Length | Requestor MAC address |
| --- | --- | --- |

Octets:			1		1			6

Figure 7-62xx: Requestor MAC address subelement format

The Target MAC address is the MAC address of the STA whose Location Information is requested and it is present whenever the location subject definition field is set to 2. The format of the Target MAC address field is shown in Figure 7-62yy.

| Subelement ID | Length | Target MAC address |
| --- | --- | --- |

Octets:			1		1			6

Figure 7-62yy: Target MAC address subelement format

- Make the following changes to  Table 7-31h (from the base spec):

- Table 7-31h -- Optional Subelement IDs for Location Configuration Information Report

| - Subelement ID | - Name | - Length Field (octets) | - Extensible |
| --- | --- | --- | --- |
| - 0 | - Reserved |  |  |
| - 1 | - Azimuth Report | - 2 | - yes |
| - 2 | - Requestor MAC address | - 6 | - no |
| - 3 | - Responder MAC address | - 6 | - no |
| - 2-220 | - Reserved |  |  |
| - 221 | - Vendor Specific | - 1 to 232 |  |
| - 222-255 | - Reserved |  |  |

The Requestor MAC Address is the MAC address of the STA which requested the Location Information and it is present whenever the location subject definition field in the corresponding Measurement Request was set to 2. The format of the Requestor MAC address field is shown in Figure 7-62xx.

| Subelement ID | Length | Requestor MAC address |
| --- | --- | --- |

Octets:			1		1			6

Figure 7-68xx: Requestor MAC address subelement format

The Responder MAC address is the MAC address of the STA whose Location Information was requested and it is present whenever the location subject definition field in the corresponding Measurement Request was set to 2. The format of the Target MAC address field is shown in Figure 7-62yy.

| Subelement ID | Length | Responder MAC address |
| --- | --- | --- |

Octets:			1		1			6

Figure 7-68yy: Responder MAC address subelement format

- Make the following changes to  Table 7-29p3:

| Table 7-29p3—Optional Subelement IDs for Location Civic Request |  |  |  |
| --- | --- | --- | --- |
| Subelement ID | Name | Length field (octets) | Extensible |
| 0 | Reserved |  |  |
| 1 | Requestor MAC address | 6 | no |
| 2 | Target MAC address | 6 | no |
| 3-220 | Reserved |  |  |
| 221 | Vendor Specific | 1 to 237 |  |
| 222-255 | Reserved |  |  |

The Requestor MAC Address is the MAC address of the STA requesting for the Location Information and it is present whenever the location subject definition field is set to 2. The format of the Requestor MAC address field is shown in Figure 7-62xx.

| Subelement ID | Length | Requestor MAC address |
| --- | --- | --- |

Octets:			1		1			6

Figure 7-62xx: Requestor MAC address subelement format

The Target MAC address is the MAC address of the STA whose Location Information is requested and it is present whenever the location subject definition field is set to 2. The format of the Target MAC address field is shown in Figure 7-62yy.

| Subelement ID | Length | Target MAC address |
| --- | --- | --- |

Octets:			1		1			6

Figure 7-62yy: Target MAC address subelement format

- Make the following changes to  Table 7-31l:

| Table 7-31l—Optional Subelement IDs for Location Civic Report |  |  |  |
| --- | --- | --- | --- |
| Subelement ID | Name | Length field (octets) | Extensible |
| 0 | Reserved |  |  |
| 1 | Requestor MAC address | 6 | no |
| 2 | Responder MAC address | 6 | no |
| 3-220 | Reserved |  |  |
| 221 | Vendor Specific | 1 to 237 |  |
| 222-255 | Reserved |  |  |

The Requestor MAC Address is the MAC address of the STA which requested the Location Information and it is present whenever the location subject definition field in the corresponding Measurement Request was set to 2. The format of the Requestor MAC address field is shown in Figure 7-62xx.

| Subelement ID | Length | Requestor MAC address |
| --- | --- | --- |

Octets:			1		1			6

Figure 7-68xx: Requestor MAC address subelement format

The Responder MAC address is the MAC address of the STA whose Location Information was requested and it is present whenever the location subject definition field in the corresponding Measurement Request was set to 2. The format of the Target MAC address field is shown in Figure 7-62yy.

| Subelement ID | Length | Responder MAC address |
| --- | --- | --- |

Octets:			1		1			6

Figure 7-68yy: Responder MAC address subelement format

- Add the following table to section 7.3.2.21.14:

| Table 7-31zz—Optional Subelement IDs for Location Identifier Request |  |  |  |
| --- | --- | --- | --- |
| Subelement ID | Name | Length field (octets) | Extensible |
| 0 | Reserved |  |  |
| 1 | Requestor MAC address | 6 | no |
| 2 | Target MAC address | 6 | no |
| 3-220 | Reserved |  |  |
| 221 | Vendor Specific | 1 to 237 |  |
| 222-255 | Reserved |  |  |

The Requestor MAC Address is the MAC address of the STA requesting for the Location Identifier and it is present whenever the location subject definition field is set to 2. The format of the Requestor MAC address field is shown in Figure 7-62xx.

| Subelement ID | Length | Requestor MAC address |
| --- | --- | --- |

Octets:			1		1			6

Figure 7-62xx: Requestor MAC address subelement format

The Target MAC address is the MAC address of the STA whose Location Identifier is requested and it is present whenever the location subject definition field is set to 2. The format of the Target MAC address field is shown in Figure 7-62yy.

| Subelement ID | Length | Target MAC address |
| --- | --- | --- |

Octets:			1		1			6

Figure 7-62yy: Target MAC address subelement format

- Add the following table to section 7.3.2.22.13:

| Table 7-31l—Optional Subelement IDs for Location Identifier Report |  |  |  |
| --- | --- | --- | --- |
| Subelement ID | Name | Length field (octets) | Extensible |
| 0 | Reserved |  |  |
| 1 | Requestor MAC address | 6 | no |
| 2 | Responder MAC address | 6 | no |
| 3-220 | Reserved |  |  |
| 221 | Vendor Specific | 1 to 237 |  |
| 222-255 | Reserved |  |  |

The Requestor MAC Address is the MAC address of the STA which requested the Location Information and it is present whenever the location subject definition field in the corresponding Measurement Request was set to 2. The format of the Requestor MAC address field is shown in Figure 7-62xx.

| Subelement ID | Length | Requestor MAC address |
| --- | --- | --- |

Octets:			1		1			6

Figure 7-68xx: Requestor MAC address subelement format

The Responder MAC address is the MAC address of the STA whose Location Information was requested and it is present whenever the location subject definition field in the corresponding Measurement Request was set to 2. The format of the Target MAC address field is shown in Figure 7-62yy.

| Subelement ID | Length | Responder MAC address |
| --- | --- | --- |

Octets:			1		1			6

Figure 7-68yy: Responder MAC address subelement format

- Change paragraph 11.10.8.6 as follows:

11.10.8.6 Location Configuration Information Report

If dot11RRMLCIMeasurementEnabled is true, a STA shall reject any LCI Request for location information

that is not available and shall respond with a Radio Measurement Report frame including a Radio

Measurement Report element with the incapable bit set to 1. If dot11RRMLCIMeasurementEnabled is true

and a STA accepts an LCI Request that does not include an Azimuth Request, it shall respond with a Radio

Measurement Report frame including one LCI element (LCI Report). If both

dot11RRMLCIMeasurementEnabled and dot11RRMLCIAzimuthEnabled are true, and the STA accepts an

- LCI request that includes an Azimuth Request, it shall respond with a Radio Measurement Report frame

including one LCI element (LCI Report) that includes the requested Azimuth Report, if available. If

dot11RRMLCIAzimuthEnabled is false, a STA shall reject any LCI Request that includes an Azimuth

Request and shall respond with a Radio Measurement Report frame including an Radio Measurement

Report element with the incapable bit set to 1.

NOTE—IETF RFC 3825, 2.1, defines formats and information fields for reporting physical location to sub-centimeter

resolution. The fixed-point values have integer and fractional parts, which together represent Latitude, Longitude and

Altitude to a maximum resolution of 34-, 34-, and 30-bits respectively. A Latitude report with 24-bit resolution would be

reporting with a precision of about 3.18 m in Latitude at the equator. The physical location and azimuth MIB information

of the STA may be set by administrative means.

The Datum value shall be 1 (World Geodetic System 1984), unless another datum is required for operation

in the regulatory domain.

If the Altitude Type is 2 (Floors of Altitude), the value reported shall be as required for operation in the

regulatory domain.

An LCI request shall indicate a location request for the requesting STA,, the reporting STA, or a 3rd STA with the MAC address specified in the Target MAC address sub-option field, by setting the

LCI request Location Subject field to indicate a Local, a Remote or a 3rd party request, respectively. Local LCI

Measurement Request is used by the requesting STA to obtain its own location by asking “Where am I?”.

Remote LCI Measurement Request is used by requesting STA to obtain location of reporting STA by asking

“Where are you?”. 3rd party Location request is used by requesting STA to obtain location of a STA with the MAC address specified in the Target MAC address sub-option field.

If the STA receiving an LCI request lacks the means to report the requested location to the requested

resolution, then the LCI Report shall have that corresponding Latitude, Longitude, Altitude, or Azimuth

resolution set to the known value; otherwise Latitude, Longitude, Altitude, and Azimuth fields shall be

reported to their requested resolutions, with the remaining less significant bits set to zero.

If the STA receiving an LCI request has no location information about the requested LCI Subject physical

location or requested Azimuth, it shall set the Incapable bit in the Measurement Report Mode field. The

method by which the physical location and azimuth information in the LCI Report is generated is outside the

scope of this standard.

When the requesting STA asks for the location of a STA with the MAC address specified in the Target MAC address sub-option field, it shall also include its own MAC address to the Requestor MAC address sub-option field.

When an AP STA receives an LCI Request with the Location Subject field value set to 2, it shall send the request to the non-AP STA with the MAC address specified in the Target MAC address subfield.

When the non-AP STA receives an LCI Request with the Location Subject field value set to 2, it may only generate an LCI Report if the MAC address in the Target MAC address sub-option field is its own MAC address. When an LCI Report is generated, the reporting STA shall include its MAC address into the Responder MAC address sub-option field and the MAC address of the STA which generated the corresponding LCI Request into the Requestor MAC address sub-option field.

When an AP STA receives an LCI Report with the Location Subject field value set to 2, it shall send the report to the non-AP STA with the MAC address specified in the Requestor MAC address sub-option field.

NOTE—A STA that requested a “Local” LCI and received an LCI Report with the Incapable bit set may alternatively

make a “Remote” LCI request to obtain the reporting STA’s physical location. A STA that requested an LCI including

an Azimuth Request, and received an LCI Report with the Incapable bit set may alternatively request the LCI with no

Azimuth requested.

If dot11RRMLCIMeasurementEnabled is false, a station shall reject the received LCI Measurement Request

- and shall respond with a LCI Report where the Incapable bit is set in the Measurement Report Mode field.

- Change paragraph 11.10.8.9 as follows:

11.10.8.9 Location Civic report

If dot11RRMCivicMeasurementEnabled is set to true and civic location information is not available, the STA shall reject a Location Civic Request and shall respond with a Measurement Report frame including a Measurement Report element with the incapable bit set to 1. If dot11RRMCivicMeasurementEnabled is set to true and civic location information is available, the STA shall respond with a Measurement Report frame including one Location Civic Report element.

A Location Civic Request shall indicate a location request for the requesting STA, the reporting STA, or a 3rd STA with the MAC address specified in the Target MAC address sub-option field, by setting the Location Civic request Location Subject field to indicate a Local, a Remote or a 3rd party request respectively. Local Location Civic Measurement Request is used by requesting STA to obtain its own location by asking “Where am I?”. Remote Location Civic Measurement Request is used by requesting STA to obtain location of the reporting STA by asking “Where are you?”. 3rd party Location request is used by requesting STA to obtain location of a STA with the MAC address specified in the Target MAC address sub-option field.

When the requesting STA asks for the location of a STA with the MAC address specified in the Target MAC address sub-option field, it shall also include its own MAC address to the Requestor MAC address sub-option field.

When an AP STA receives a Location Civic Request with the Location Subject field value set to 2, it shall send the request to the non-AP STA with the MAC address specified in the Target MAC address subfield.

When the non-AP STA receives a Location Civic Request with the Location Subject field value set to 2, it may only generate a Location Civic Report if the MAC address in the Target MAC address sub-option field is its own MAC address. When a Location Civic Report is generated, the reporting STA shall include its MAC address into the Responder MAC address sub-option field and the MAC address of the STA which generated the corresponding Location Civic Request into the Requestor MAC address sub-option field.

When an AP STA receives a Location Civic Report with the Location Subject field value set to 2, it shall send the report to the non-AP STA with the MAC address specified in the Requestor MAC address sub-option field.

If dot11RRMCivicMeasurementEnabled is set to false, a STA shall reject the received Location Civic Measurement Request and shall respond with a Location Civic Report where the Incapable bit is set in the Measurement Report Mode field.

If dot11RRMCivicMeasurementEnabled is set to true and a STA has its own location configured in Civic format, it shall set the Civic Location field to 1 in the Extended Capabilities information element.

If dot11RRMCivicMeasurementEnabled is set to true and a STA has its own location available in one or more location formats, as defined in Table 7-29p1, and includes Civic Location Type Value 0, indicating "RFC4776-2006", it shall set the Civic Location field to 1 in the Extended Capabilities information element.

- Change paragraph 11.10.8.10 as follows:

11.10.8.10 Location identifier report

The Location Identifier Report provides the ability for a STA to receive an indirect URI reference and forward that reference to an external agent for the purposes of that agent gathering the STA’s location value. The protocol used to query for a location report based on the Public Identifier URI provided in the Location Identifier Report is beyond the scope of this standard.

If dot11RRMIdentifierMeasurementEnabled is set to true and location information is not available, the STA shall reject any Location Identifier Request and shall respond with a Measurement Report frame including a Measurement Report element with the incapable bit set to 1. If dot11RRMIdentifierMeasurementEnabled is set to true and location information is available, the STA shall respond with a Measurement Report frame including one Location Identifier Report element.

A Location Identifier Request shall indicate a location request for the requesting STA, the reporting STA, or a 3rd party STA with the MAC address specified in the Target MAC address sub-option field, by setting the Location Identifier request Location Subject field to indicate a Local, a Remote or a 3rd party request respectively. Local Location Identifier Request is used by requesting STA to obtain its own location by asking “Where am I?”. Remote Location Civic Measurement Request is used by requesting STA to obtain location of the reporting STA by asking “Where are you?”. 3rd party Location request is used by requesting STA to obtain location of a STA with the MAC address specified in the Target MAC address sub-option field.

When the requesting STA asks for the location of a STA with the MAC address specified in the Target MAC address sub-option field, it shall also include its own MAC address to the Requestor MAC address sub-option field.

When an AP STA receives a Location Identifier Request with the Location Subject field value set to 2, it shall send the request to the non-AP STA with the MAC address specified in the Target MAC address subfield.

When the non-AP STA receives a Location Identifier Request with the Location Subject field value set to 2, it may only generate a Location Identifier Report if the MAC address in the Target MAC address sub-option field is its own MAC address. When a Location Identifier Report is generated, the reporting STA shall include its MAC address into the Responder MAC address sub-option field and the MAC address of the STA which generated the corresponding Location Identifier Request into the Requestor MAC address sub-option field.

When an AP STA receives a Location Identifier Report with the Location Subject field value set to 2, it shall send the report to the non-AP STA with the MAC address specified in the Requestor MAC address sub-option field.

If dot11RRMIdentifierMeasurementEnabled is set to false, a STA shall reject the received Location Identifier Request and shall respond with a Location Identifier Report where the Incapable bit is set in the Measurement Report Mode field.

- If dot11RRMIdentifierMeasurementEnabled is set to true and a STA has its own location available in one or more location formats, as defined in Table v7-29p1, and includes Civic Location Type Value 0, indicating "RFC4776-2006" or Geospatial format that can be referenced by a location identifier, it shall set the Identifier Location field to 1 in the Extended Capabilities information element.