IEEE P802.11
Wireless LANs

| TGae Requirements and Use Cases |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-01-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Michael Montemurro | Research in Motion | 4701 Tahoe Blvd, Mississauga, ON. L4W 0B4 | +1-905-629-4746 | mmontemurro@rim.com |

### Overview

This summarizes the Use Cases and Requirements for prioritization of management frames.

# Requirements

The following table summarizes the requirements for Prioritization of Management Frames.

|  | Requirement | Use Case Description |
| --- | --- | --- |
| 1 | AP advertises Management Frame Prioritization capability. | 1,2 |
| 2 | AP is configured and advertises Management Frame Prioritization Policy. | 1,2 |
| 3 | Management Frame Prioritization capable STA indicates management frame prioritization capability when it associates. | 1 |
| 4 | Management frame priortization covers RRM and WNM requests and reports. | 1 |
| 5 | STA and AP adhere to Management Frame prioritization policy to transmit RRM and WNM managmeent frames. | 1,2,3 |
| 6 | Management frame prioritization should be considered for the pre-association state. | 2 |
| 7 | Management frame prioritization should be considered for VSA and Timing Advertisement action frames. | 3 |
| 8 | Management frame prioritization will be applied to Location tracking managment frames. | 4 |
| 9 | Management frame prioritization policy for location tracking will be communicated from the AP to the STA | 4 |
| 10 | Mesh peering, path selection and Link metrics should be always kept at the highest priority? | 5 |
| 11 | Frames of sub-type Action, Timing Advertisement, and Multihop Action frames should be considered for prioritization. |  |

### Issues:

- Does TGae need to address any other management frames other than WNM and RRM?

- Is the Management Frame Prioritization policy dynamic or static?

- Which types of networks do we address? BSS, IBSS, not-BSS (11p), Mesh.

- Can the AP advertise different management frame prioritization policies for use, pre and post association?

- What is a policy? How complex is it? What are the criteria for assigning a policy?

- Does TGae resolve prioritization of group addressed management frames?

- Should TGae consider both delay (channel access) and packet loss as QoS aspects for management frame prioritization?

- Should the group consider addressing management traffic that is tunnelled inside data frames? (e.g. FT frames could be considered as higher priority while TDLS set-up frames could be considered at a lower priority).

### Assumptions

- TGae only addresses QoS-enabled networks.

- The AP controls the underlying management frame prioritization policy for the BSS.

Use cases

### 1. Use of Radio Resource Management(RRM) or Wireless Network Management(WNM) Frames when there are stations using QoS.

- QoS-capable STA associates to an AP that supports RRM or WNM.

- STA and AP exchange Management Frame Prioritzation capabilities

  - Frame Prioritization is configured in the AP infrastructure and advertised by the AP.

  - WNM and RRM frames are included as supported for Frame Prioritization.

- The STA initiates a QoS stream on the WLAN network (call can be incoming or outgoing)

  - For example, voice frames are transmitted at AC-VO

  - AP initiates RRM or WNM report requests (the RRM or WNM report request is transmitted at the configured AC). The STA on the QoS session or other STA’s on the network respond to the RRM or WNM at the configured/negotiated AC.

- STA’s do not support Management Frame Prioritization use legacy behaviour for medium access. AP uses legacy behavior for medium access to communicate with a legacy STA.

### 2. Use of management frames (e.g. native GAS) in pre-associated state

- A STA discovers an AP using either active or passive scanning procedures.

- The AP advertises management frame prioritization policy for pre-association frames (only Native GAS?)

- The STA issues a Native GAS request to the AP according to the advertised policy.

- The AP issues a Native GAS response to the STA according to the advertised policy.

- Could the STA use a different AC for Native GAS after it has been associated?

### 3. Use of prioritized management frames for Vehicle Safety.

- See document https://mentor.ieee.org/802.11/dcn/10/11-10-0095-01-00ae-case-study-for-reduced-priority-management-frames-vehicular-safety-communication.ppt. The only management frames are vendor-specific and timing advertisement action frames.

- There could be some RRM and WNM frames used for these applications.

### 4. Using WLAN for location tracking in a warehouse or hospital environment

- The WNM location tracking feature is used to track the location of objects in an multi-AP environment.

- The AP instructs the STA to use a specific set of EDCA parameters for location tracking frames.

- The STA’s with location tracking enabled use these parameters to transmit location updates.

- The STA continues to use the specific set of EDCA parameters after it has lost connectivity of the with the requesting AP, until location tracking is terminated.

### 5. Mesh path selection and link metrics.

- In a mesh environment, mesh path selection, peering, and link metrics are kept at the highest priority.

- Would it make sense to drop mesh management frames to AC_VI while maintaining voice at AC_VO?