IEEE P802.11
Wireless LANs

| MAC beaconing and sync comment resolution |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-01-20 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Kazuyuki Sakoda | Sony Corporation | 5-1-12 Kita-Shinagawa, Shinagawa-ku, Tokyo, 141-0001 Japan | +81-3-5448-4018 | KazuyukiA.Sakoda(at)jp.sony.com |

Abstract

This document provides suggested resolutions to CID2001, 2002, 2003, 2005, 2008, 2015, 2017, 2018, 2020, 2023, 2024, 2027, 2028, 2031, 2034, 2035, 2038, 2040, 2041, 2042, 2045, 2046, 2047, 2470, 2522, 2609, 2714, 2715, 2716, 2717, 2733, 2741, 2742,

2004, 2006, 2036, 2719, 2789, 2026, 2029, 2037, 2043, 2044, 2720, 2019, 2010, 2011, 2505, 2519, 2025, 2033, 2051, 2192, 2722

## Summary of the intention of this document

- Polish the text on beaconing and sync related text.

- Define Beacon Timing Information field in the Beacon Timing element.

- Clean up and consolidate the description on MBCA.

## Suggested changes to the draft spec

- Fix typo or editorial errors as indicated by the received comments.

- Modify 7.3.2.102 to compy with the received comments. Adjust the references to this description.

- Merge clause X.3 to 11C.12.4 and consolidate the description on MBCA.

Change “dot11MeshTbttSelectionActivated” to “dot11MeshTBTTSelectionActivated” globally in D4.0. There are 8 occurrences of  “dot11MeshTbttSelectionActivated” in D4.0.

Apply the following changes.
Corresponding changes to D4.0 are indicated in the following text with “Track Changes” on, to clarify the direction to the editor. Please update the part indicated by the “Track Changes” only.

- Status Code field

Insert the following rows into Status codesinput.doc and change the last row (Reserved) as shown.

| - Status codesinput.doc |  |
| --- | --- |
| Status code | Meaning |
|  | Authentication is rejected because an anti-clogging token is required |
|  | The request has not been successful as the STA could not find the appropriate TBTT. |
| +1 42-65535 | Reserved |

- Beacon Timing element

The Beacon Timing element is used to advertise the beacon timing information of neighboring STAs (mesh STAs, APs, or STAs in IBSS).

The format of the Beacon Timing element is shown in Beacon Timing element.

| Element ID | Length | Beacon Timing Information #1 | ... | Beacon Timing Information #N |
| --- | --- | --- | --- | --- |
| Octets: 1 | 1 | 5 | ... | 5 |
| - Beacon Timing element |  |  |  |  |

The Element ID is set to the value given in Error: Reference source not found for this element.

The Length field indicates the number of octets in the information field (fields following the Element ID and Length fields).

The Beacon Timing Information field contains a neighbor STA’s beacon timing information. When the mesh STA reports multiple beacon timing information, multiple Beacon Timing Information field are included in this element. The structure of the Beacon Timing Information field is defined in Figure sXX.

| Neighbor STA ID | Neighbor Last Beacon Time | Neighbor Beacon Interval |  |
| --- | --- | --- | --- |
| 1 | 2 | 2 |  |
| Figure sXX--Beacon Timing Information field |  |  |  |

The Neighbor STA ID field is an unsigned integer that indicates the identification of the neighbor STA corresponding to this beacon timing information. When a mesh peering is established with this neighbor STA, the MSB of this field is set to 0, and the rest of this field is set to the last 7 digits (7 LSBs) of the AID value assigned to this neighbor mesh STA. When a mesh peering is not established with this neighbor STA, the MSB of this field is set to 1, and the rest of this field is set to the last 7 digits (7 LSBs, taking the I/G bit as the MSB) of the 48-bit MAC address of this neighbor STA.

NOTE—Since the Neighbor STA ID field is provided in abbreviated form, it is possible that the same Neighbor STA ID value appears in multiple Beacon Timing Information fields.

The Neighbor Last Beacon Time field is an unsigned integer that indicates the latest reception time of the Beacon frame (or the TBTT of the received Beacon frame, see note in ) from the corresponding mesh STA, measured in the reporting mesh STA's local TSF timer. The 2nd and 3rd least significant octets of the TSF value are contained in this field, in order to represent the time in units of 256 µs.

The Neighbor Last Beacon Time field is an unsigned integer that indicates a TBTT of the corresponding neighbor STA, measured in the mesh STA's local TSF timer. The TBTT is calculated from the latest reception time of the Beacon frame and values in the Timestamp field and the Beacon interval field of the Beacon frame, as described in 11C.12.4.2. The 2nd and 3rd least significant octets of the measured TSF value are contained in this field, in order to represent the time in units of 256 µs.

The Neighbor Beacon Interval field is an unsigned integer that indicates the beacon interval being used by the corresponding neighbor STA.

Detailed usage of the Beacon Timing element is described in Mesh Beacon Collision Avoidance (MBCA) .

- Action frame format details

- Mesh Resource Coordination Action frame details

- General

Action frame formats for management of Mesh Resource Coordination are defined in this subclause.

An Action field, in the octet field immediately after the Category field, differentiates the frame format. The Action field values associated with each frame format are defined in Mesh Resource Coordination Action field valuesinput.doc.

| - Mesh Resource Coordination Action field valuesinput.doc |  |
| --- | --- |
| Action field value | Description |
|  | Congestion Control Notification |
|  | MCCA Setup Request |
|  | MCCA Setup Reply |
|  | MCCAOP Advertisement Request |
|  | MCCAOP Advertisements |
|  | MCCAOP Reservation Teardown |
|  | TBTT Adjustment Request |
|  | TBTT Adjustment Response |
| 8 | Mesh Channel Switch Announcement |
| 9-255 | Reserved |

- TBTT Adjustment Request frame format

The TBTT Adjustment Request frame is used to request a particular neighbor peer mesh STA to adjust its TBTT. This frame is transmitted using individual addresses. The frame body of a TBTT Adjustment Request frame contains the information shown in TBTT Adjustment Request frame bodyinput.doc.

| - TBTT Adjustment Request frame bodyinput.doc |  |  |
| --- | --- | --- |
| Order | Information | Notes |
|  | Category |  |
|  | Action |  |
|  | Beacon Timing element |  |
| Last | Vendor Specific | One or more vendor-specific elements are optionally present. These elements follow all other elements. |

The Category field is set to the value in Error: Reference source not found for category Mesh Resource Coordination.

The Action field is set to the value in Mesh Resource Coordination Action field valuesinput.doc for this Action frame type.

The Beacon Timing element is set as described in Error: Reference source not found.

- TBTT Adjustment Response frame format

The TBTT Adjustment Response frame is used to respond to a TBTT adjustment request. This frame is transmitted using individual addresses. The frame body of a TBTT Adjustment Response frame contains the information shown in TBTT Adjustment Request frame bodyinput.doc.

| - TBTT Adjustment Request frame bodyinput.doc |  |  |
| --- | --- | --- |
| Order | Information | Notes |
|  | Category |  |
|  | Action |  |
|  | Status Code |  |
| - 4 | Beacon Timing element (optional) |  |
| Last | Vendor Specific | One or more vendor-specific elements are optionally present. These elements follow all other elements. |

The Category field is set to the value in Error: Reference source not found for category Mesh Resource Coordination.

The Action field is set to the value in Mesh Resource Coordination Action field valuesinput.doc for this Action frame type.

The Status Code field is defined in 7.3.1.9.

The Beacon Timing element, defined in Error: Reference source not found is present only if the Status Code is set to <ANA16> (i.e., when the request is not successful due to the neighbor constraint).

- MLME SAP interface

- Scan

- MLME-SCAN.request

- Semantics of the service primitive

Change the primitive parameters in 10.3.2.1.2 as follows:

- following text is based on the amendment by TGk, TGv, and TGu

The primitive parameters are as follows:

MLME-SCAN.request(

BSSType,

BSSID,

SSID,

ScanType,

ProbeDelay,

ChannelList,

MinChannelTime,

MaxChannelTime,

RequestInformation,

SSIDList,

ChannelUsage,

NetworkType,

HESSID,

MeshID,

VendorSpecificInfo

)

Change the BSSType row of the untitled table defining the primitive parameters in 10.3.2.1.2 as shown:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| BSSType | Enumeration | NFRASTRUCTURE, INDEPENDENT, MESH, ANY_BSS | Determines whether infrastructure BSS, IBSS, MBSS, or both all are included in the scan. |

Insert the following new row before the VendorSpecificInfo row of the untitled table defining the primitive parameters in 10.3.2.1.2:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Mesh ID | Octet string | 0 – 32 octets | Only present if BSSType = MESH or BSSType = ANY_BSS. Specifies the desired Mesh ID or wildcard Mesh ID. |

- MLME-SCAN.confirm

- Semantics of the service primitive

Change the BSSID, BSSType, Beacon Period, DTIM Period, Timestamp, Local Time rows in the BSSDescription parameter table in 10.3.2.2.2 as shown:

- following table is based on the amendment by TGn

| Name | Type | Valid range | Description | IBSS adoption |
| --- | --- | --- | --- | --- |
| BSSID | MACAddress | N/A | The BSSID of the found BSS or the MAC address of the found mesh STA. | Adopt |
| BSSType | Enumeration | INFRASTRUCTURE, INDEPENDENT, MESH | The type of found BSS. | Adopt |
| Beacon Period | Integer | N/A | The Beacon period of the found BSS, or the found mesh STA if the BSSType = MESH (in TU). | Adopt |
| DTIM Period | Integer | As defined in frame format | The DTIM period of the BSS, or the found mesh STA if the BSSType = MESH (in beacon periods). | Adopt |
| Timestamp | Integer | N/A | The timestamp of the received frame (probe response/beacon) from the found BSS, or the found mesh STA if the BSSType = MESH. | Adopt |
| Local Time | Integer | N/A | The value of the STA’s TSF timer at the start of reception of the first octet of the timestamp field of the received frame (probe response or beacon) from the found BSS, or from the found mesh STA if the BSSType = MESH. | Do not adopt |

Insert the following new rows to the BSSDescription parameter table in 10.3.2.2.2:

| Name | Type | Valid range | Description | IBSS adoption |
| --- | --- | --- | --- | --- |
| MeshID | As defined in frame format | As defined in frame format | The value of MeshID element if such element was present in the probe response or Beacon frame, else null. | Do not adopt |
| MeshConfiguration | As defined in frame format | As defined in frame format | The values from the Mesh Configuration element if such an element was present in the probe response or Beacon frame, else null. | Do not adopt |
| Neighbor Beacon Timing | As defined in frame format | As defined in frame format | The values from the Beacon Timing element if such an element was present in the probe response or Beacon frame, else null. Present only if if dot11MeshTBTTSelectionActivated is true. | Do not adopt |

- Synchronization

- MLME-JOIN.request

- Function

Change 10.3.3.1.1 as follows:

This primitive requests synchronization with a BSS which type is infrastructure or independent.

- Start

Change 10.3.10 as follows:

This mechanism supports the process of creating a new BSS which type is infrastructure or independent.

- MLME-START.request

- Function

Change 10.3.10.1.1 as follows:

This primitive requests that the MAC entity start a new BSS which type is infrastructure or independent.

- MLME-Mesh Neighbor Offset Synchronization

This mechanism supports the Mesh Neighbor Offset synchronization status with a targeted neighbor mesh STA.

- MLME-MeshNeighborOffsetMeasure.request

- Function

This primitive requests a measurement of TSF timer offset value for a specific neighbor mesh STA.

- Semantics of the service primitive

The primitive parameters are as follows:

MLME-MeshNeighborOffsetMeasure.request(

peerMAC

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| peerMAC | MAC Address | Any valid individual MAC address | Specifies the address of the peer MAC entity with which to measure the TSF offset value. |

- When generated

This primitive is generated by the SME to order the measurement of TSF timer offset value with the specified neighbor mesh STA.

- Effect of receipt

On receipt of this primitive, the MLME commences the measurement and the calculation of the TSF timer offset value. The MLME subsequently issues an MLME-MeshNeighborOffsetMeasure.confirm that reflects the results of this request.

- MLME-MeshNeighborOffsetMeasure.confirm

- Function

This primitive reports the results of a mesh neighbor offset measurement request.

- Semantics of the service primitive

The primitive parameters are as follows:

MLME-MeshNeighborOffsetMeasure.confirm(

ResultCode

,

TSFOffsetValue

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID_PARAMETERS, or NOT_SUPPORTED | Indicates the result of the mesh neighbor offset measurement request. |
| TSFOffsetValue | Integer | 0- (264-1) | Indicates the TSF offset value with the specified neighbor mesh STA, expressed in 2’s complement in µs. |

- When generated

This primitive is generated by the MLME as a result of an MLME-MeshNeighborOfsetMeasure.request to report a measurement result.

- Effect of receipt

The SME is notified of the results of the mesh neighbor offset measurement request.

- MLME-Mesh TBTT Adjustment

The following primitives describe how a mesh STA requests TBTT adjustment to its neighboring peer mesh STA.

- MLME-MeshTBTTAdjustment.request

- Function

This primitive requests that a TBTT Adjustment Request frame be sent to the specified peer mesh STA.

- Semantics of the service primitive

The primitive parameters are as follows:

MLME-MeshTBTTAdjustment.request(

peerMAC,

BeaconTiming,

VendorSpecificInfo

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| peerMAC | MAC Address | Any valid individual MAC address | Specifies the address of the peer MAC entity to which the TBTT Adjustment Request is sent. |
| BeaconTiming | Beacon Timing element | As defined in Error: Reference source not found | A set of beacon timing information of the mesh STA. |
| VendorSpecificInfo | A set of elements | As defined in 7.3.2.26 | Zero or more elements |

- When generated

This primitive is generated by the SME to request that a TBTT Adjustment Request frame to be sent to a peer entity to request the adjustment of peer entity’s TBTT.

- Effect of receipt

On receipt of this primitive, the MLME constructs a TBTT Adjustment Request frame containing the Beacon Timing element. This frame is then scheduled for transmission. The MLME subsequently issues an MLME- MeshTBTTAdjustment.confirm that reflects the results of this request.

- MLME-MeshTBTTAdjustment.confirm

- Function

This primitive reports the results of a mesh TBTT adjustment request.

- Semantics of the service primitive

The primitive parameters are as follows:

MLME-MeshTBTTAdjustment.confirm(

peerMAC,

Status Code,

BeaconTiming,

VendorSpecificInfo

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| peerMAC | MAC Address | Any valid individual MAC address | Specifies the address of the peer MAC entity to which the TBTT Adjustment Request is sent. |
| Status Code | As defined in frame format. | As defined in frame format. | Indicates the result of the TBTT adjustment request. |
| BeaconTiming | Beacon Timing element | As defined in Error: Reference source not found | A set of beacon timing information of the requesting mesh STA. Present only when such an element was present in the TBTT Adjustment Response response. |
| VendorSpecificInfo | A set of elements | As defined in 7.3.2.26 | Zero or more elements |

- When generated

This primitive is generated by the MLME as a result of  an MLME-MeshTBTTAdjustment.request primitive to indicate  the request of that request.

- Effect of receipt

The SME is notified of the results of the mesh TBTT adjustment request.

- MLME-MeshTBTTAdjustment.indication

- Function

This primitive indicates that a specific peer MAC entity is requesting to adjust TBTT.

- Semantics of the service primitive

The primitive parameters are as follows:

MLME-MeshTBTTAdjustment.indication(

peerMAC,

BeaconTiming,

VendorSpecificInfo

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| peerMAC | MAC Address | Any valid individual MAC address | Specifies the address of the peer MAC entity from which the TBTT Adjustment request was received. |
| BeaconTiming | Beacon Timing element | As defined in Error: Reference source not found | A set of beacon timing information of the requesting mesh STA. |
| VendorSpecificInfo | A set of elements | As defined in 7.3.2.26 | Zero or more elements |

- When generated

This primitive is generated by the MLME as a result of the receipt of a TBTT Adjustment Request frame from a specific peer MAC entity.

- Effect of receipt

The SME is notified of the results of the receipt of the TBTT adjustment request by the specified peer MAC entity. The mesh STA received this primitive subsequently processes the TBTT adjustment described in 11C.12.4.3.

- MLME-MeshTBTTAdjustment.response

- Function

This primitive is used to send a response to a specific peer MAC entity that requested a TBTT adjustment to the STA that issued this primitive.

- Semantics of the service primitive

The primitive parameters are as follows:

MLME-MeshTBTTAdjustment.indication(

peerMAC,

Status Code,

BeaconTiming,

VendorSpecificInfo

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| peerMAC | MAC Address | Any valid individual MAC address | Specifies the address of the peer MAC entity from which the TBTT Adjustment request was received. |
| Status Code | As defined in frame format. | As defined in frame format. | Indicates the result response to the TBTT adjustment request from the peer mesh STA. |
| BeaconTiming | Beacon Timing element | As defined in Error: Reference source not found | A set of beacon timing information of the requesting mesh STA. Only present when Status Code is set to non-zero value. |
| VendorSpecificInfo | A set of elements | As defined in 7.3.2.26 | Zero or more elements |

- When generated

This primitive is generated by the SME of a STA as a resonse to an MLME-MeshTBTTAdjustment.indication primitive.

- Effect of receipt

This primitive initiates transmission of a response to the specific peer MAC entity that requested the TBTT adjustment.

On receipt of this primitive, the MLME constructs a TBTT Adjustment Response frame. When the Status Code is set to non-zero value, the frame contains the Beacon Timing element. This frame is then scheduled for transmission.

.

- MLME-Start Beaconing

- MLME-STARTBEACONING.request

- Function

This primitive requests that the MAC entity start a new MBSS or becomes a member to an existing MBSS.

- Semantics of the service primitive

The primitive parameters are as follows:

MLME-STARTBEACON.request(

MeshID,

MeshConfiguration,

BeaconPeriod,

DTIMPeriod,

PHY parameter set,

ProbeDelay.

CapabilityInformation,

BSSBasicRateSet,

OperationalRateSet,

Country,

VendorSpecificInfo

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Mesh ID | Octet string | 0 – 32 octets | The MeshID of the started or found MBSS. |
| Mesh Configuration | As defined in frame format | As defined in frame format | The values from the Mesh Configuration element that is started or to which the mesh STA becomes a member. |
| Beacon Period | Integer | ≥1 | The Beacon period of the MBSS (in TU). |
| DTIM Period | Integer | As defined in 7.3.2.6 | The DTIM period of the MBSS (in beacon periods). |
| PHY Parameter Set | As defined in frame format | As defined in 7.3.2.3 or 7.3.2.4. | The parameter sets relevant to the PHY. |
| ProbeDelay | Integer | N/A | Delay (in µs) to be used prior to transmitting a Probe frame during active scanning. |
| Capability Information | As defined in frame format | As defined in 7.3.1.4 | The capabilities to be advertised for the BSS. |
| BSSBasic RateSet | Set of integers | 1–127 inclusive (for each integer in the set) | The set of data rates that must be supported by all STAs that desire to join this BSS. The STAs must be able to receive and transmit at each of the data rates listed in the set. |
| Operational RateSet | Set of integers | 1–127 inclusive (for each integer in the set) | The set of data rates that the STA desires to use for communication within the BSS. The STA must be able to receive at each of the data rates listed in the set. This set is a superset of the rates contained in the BSSBasicRateSet parameter. |
| Country element | As defined in the Country element. | As defined in the Country element. | The information required to identify the regulatory domain in which the STA is located and to configure its PHY for operation in that regulatory domain. Present only when TPC functionality is required, as specified in 11.8 or when dot11MultiDomainCapabilityEnabled is true. |
| RSN | RSN element | As defined in frame format | A description of the cipher suites and AKM suites supported in the BSS. |
| VendorSpecificInfo | A set of elements | As defined in 7.3.2.26 | Zero or more elements |

- When generated

This primitive is generated by the SME to start an MBSS (with the MAC entity acting as the first STA in the MBSS) or become a member to an existing MBSS.

The MLME-STARTBEACONING.request primitive must be generated after an MLME-RESET.request primitive has been used to reset the MAC entity and before an MLME-MeshNeighborOffsetSync.request primitive and MLME-MeshPeeringManagement.request has been used to successfully establish mesh peering in the MBSS.

The MLME-STARTBEACONING.request primitive must not be used after successful use of the MLME-STARTBEACONING.request primitive without generating an intervening MLMERESET.request primitive.

- Effect of receipt

This primitive initiates the MBSS initialization procedure once the current frame exchange sequence is complete. The MLME subsequently issues an MLME-STARTBEACONING.confirm that reflects the results of the creation procedure.

- MLME- STARTBEACONING.confirm

- Function

This primitive reports the results of a MBSS creation procedure.

- Semantics of the service primitive

The primitive parameters are as follows:

MLME-STARTBEACONING.confirm(

ResultCode

,

VendorSpecificInfo

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID_PARAMETERS, RESET_REQUIRED_BEFORE_START, NOT_SUPPORTED | Indicates the result of the MLME-STARTBEACONING.request. |
| VendorSpecificInfo | A set of elements | As defined in 7.3.2.26 | Zero or more elements |

- When generated

This primitive is generated by the MLME as a result of an MLME-STARTBEACONING.request to create a new MBSS.

- Effect of receipt

The SME is notified of the results of the MBSS creation procedure.

- Synchronization

Change the first paragraph in 11.1 as follows:

- following text is based on the amendment by TGp

In infrastructure BSSs or in IBSSs, Aall STAs within a single BSS shall be synchronized to a common clock using the mechanisms defined herein. A STA for which dot11OCBEnabled is true is not a member of a BSS, and is not required to synchronize to a common clock or use these mechanisms.

In mesh BSSs, STAs use a synchronization protocol as defined as a part of the extensible synchronization framework. The synchronization in MBSS is described in Synchronization and beaconing in MBSSs.

- Acquiring synchronization

- Active scanning

- Sending a probe response

Change the first paragraph in 11.1.3.2.1 as follows:

- The following text is based on the amendment by TGv and TGu

STAs, subject to criteria below, receiving Probe Request frames not containing an Interworking field in the Extended capabilities element set to 1, shall respond with a probe response only if

- The SSID in the probe request is the wildcard SSID, the SSID in the probe request is the specific SSID of the STA, or the specific SSID of the STA is included in the SSID List element,

- The BSSID field in the probe request is the wildcard BSSID or the BSSID of the STA, and

- The DA field in the probe request is the broadcast address or the specific MAC address of the STA.

- The Address 1 field in the probe request is the broadcast address or the specific MAC address of the STA, and either b or c below.

- The STA is a mesh STA and the Mesh ID in the probe request is the wildcard Mesh ID or the specific Mesh ID of the STA.

- The STA is not a mesh STA and

- The SSID in the probe request is the wildcard SSID, the SSID in the probe request is the specific SSID of the STA, or the specific SSID of the STA is included in the SSID List element, and

- The Address 3 field in the probe request is the wildcard BSSID or the BSSID of the STA.

- Synchronization and beaconing in MBSSs

- TSF for MBSSs

The mesh STA shall initialize its TSF timer depending on its active synchronization protocol and the given attribute of dot11MeshTbttSelectionActivated. The mesh STA shall periodically transmit Beacon frames that contain a copy of its TSF timer to announce its local time reference to its neighbor mesh STAs. Mesh STAs receiving a Beacon frame may accept the timing information depending on their active synchronization protocol and the given attribute of dot11MeshTSFAdjustmentActivated.

Each mesh STA shall maintain a TSF timer as described in 11.1.2, and conform to the TSF timer accuracy as described in 11.1.2.4.

- Extensible synchronization framework

- General

This standard introduces an extensible framework to enable the implementation of multiple synchronization protocols for mesh STAs. Within the extensible synchronization framework, the neighbor offset synchronization protocol is defined as a default mandatory protocol in order to enable minimal synchronization capabilities and interoperability between mesh STAs that use MCCA, MBCA, or operate in power save mode. However, to accommodate various application needs, the framework allows flexibility to integrate future synchronization protocols for MBSSs. A vendor can implement any synchronization protocol using this framework to meet special application needs.

Mesh STAs shall announce the active synchronization protocol using the Synchronization Protocol Identifier field in the Mesh Configuration element in their Beacon and Probe Response frames. Although a mesh STA may include multiple implementations of the synchronisation protocol, all the mesh STAs in an MBSS shall use the same synchronisation protocol.

- Neighbor Offset Protocol

- Basic operation

The Neighbor Offset Protocol allows a mesh STA to keep track of the time base of its neighbor mesh STAs. The attributes of the Neighbor Offset Protocol are given by dot11MeshNbrOffsetActivated and dot11MeshNbrOffsetMaxNeighbor. When dot11MeshNbrOffsetActivated is true, the mesh STA shall maintain a timing offset value between its own TSF timer and the TSF timer of each neighbor mesh STA with which it synchronizes. A mesh STA shall maintain synchronization with all of its neighbor peer mesh STAs that are in the same MBSS. Additionally, mesh STA should maintain synchronization with neighbor STAs that are outside of the MBSS, as long as the number of neighbors does not exceed dot11MeshNbrOffsetMaxNeighbor.

A mesh STA may start its TSF timer independently of other mesh STAs. A mesh STA may update the value of its TSF timer based on time stamps received in Beacon or Probe Response frames from neighbor STAs when dot11MeshTSFAdjustmentActivated or dot11MeshTbttSelectionActivated are true.

A mesh STA that utilizes the neighbor offset protocol shall update the timing offset value with respect to the neighbor mesh STA based on time stamps from the received Beacon and Probe Response frames as follows:

OffsetForNeighborSTA = received time stamp from neighbor mesh STA - own TSF timer value

The offset value is represented as 2’s complement. The unit of the offset value is µs.

A mesh STA may translate its own TSF timer value into another mesh STA's timer base by adding the offset as follows:

Translated neighbor mesh STA's TSF timer value = own TSF timer value+OffsetForNeighborSTA

Upon receipt of an MLME-MeshNeighborOffsetMeasure.request, the MLME shall report the measured OffsetForNeighborSTA value to SME by responding with MLME-MeshNeighborOffsetMeasure.confirm. The OffsetForNeighborSTA values are used to provide the timing reference of neighboring mesh STAs. These values are needed in order to provide such services as MCCA, MBCA, or power management.

- TSF Adjustment

A mesh STA may adjust its TSF timer to compensate the clock drift among STAs with which it maintains synchronization. When dot11MeshTSFAdjustmentActivated is true, the mesh STA shall keep on monitoring the reception time of the Beacon frames from neighboring STAs with which it maintains synchronization, and adjust its TSF timer to compensate the relative timing error among neighbor mesh STAs caused by the clock drift. A mesh STA that activates this TSF adjustment function shall set TSF Adjustment Enabled subfield in the Mesh Capability field of the Mesh Configuration element to 1.

When dot11MeshTSFAdjustmentActivated is true, the mesh STA shall operate the following procedures.

- When the mesh STA receives a Beacon frame or a Probe Response frame from one of the neighboring STAs with which it maintains synchronization, the mesh STA shall calculate the clock drift amount by comparing the OffsetForNeighborSTA value obtained previously for this neighbor and the OffsetForNeighborSTA value obtained from the frame reception.

- If the received Beacon frame or Probe Response frame contains the Mesh Configuration element and the TBTT Adjusting subfield in the Mesh Configuration field is 1, the mesh STA shall not perform the clock drift compensation based on the received frame.

- When the mesh STA sees one of its neighbor’s TSF is delaying due to the clock drift, it shall suspend its TSF timer for the delay amount, so that its TSF counting frequency will be identical to the most delaying neighbor’s. The mesh STA shall suspend its TSF timer frequently enough so that each delay amount is smaller than 1/8 of the Group Delivery Idle Time (defined in Error: Reference source not found).

NOTE— By operating this way, clock drift is compensated among neighbors when synchronization is maintained mutually, and neighbor’s TBTTs do not drift away. Since the TSF is adjusted in the slower direction, neighboring mesh STAs do not miss the adjusted TBTT.

- Beaconing

- Beacon generation in MBSSs

A mesh STA transmits Beacon frames that are specific to an MBSS. Beacon frames for MBSS and infrastructure BSS or IBSS are differentiated by the Capability Information field in the Beacon frame as specified in Error: Reference source not found. A mesh STA that collocates with an AP generates Beacon frames for the MBSS independently of the AP.

The mesh STA shall define a series of TBTTs exactly dot11BeaconPeriod TUs apart. Time zero is defined to be a TBTT with the Beacon frame being a DTIM. At each TBTT, the mesh STA shall schedule a Beacon frame as the next frame for transmission according to the medium access rules specified in Clause 9. The beacon period is included in Beacon and Probe Response frames.

- Beacon reception for mesh STA

A mesh STA may receive the Beacon frames from neighboring STAs with which it synchronize, without regard for the BSSID, SSID, or Mesh ID.

- Mesh Beacon Collision Avoidance (MBCA)

- Overview

Mesh STAs use the Mesh Beacon Collision Avoidance (MBCA) to detect and mitigate collisions among Beacon frames transmitted by other STAs (including mesh STAs, APs, and STAs in an IBSS) on the same channel within the range of 2 hops. MBCA is intended to resolve hidden node problems with respect to Beacon frames.

NOTE—Beacon frames are essential especially for passive scan, power management, and MCCA. However, Beacon frames are transmitted without acknowledgement and might collide with other frames. In a mesh BSS, multiple STAs transmit Beacon frames periodically, and mesh STAs might be located out of range of each other. This implies that Beacon frames might suffer from so-called hidden node problem and can not be received by neighbor STAs. Once Beacon frames from hidden nodes start to collide each other, Beacon frames will keep on colliding if these hidden nodes transmit Beacon frames at the same beacon interval which is a typical operation. MBCA provides a set of rules to mitigate this problem.

- Beacon timing advertisement

11C.12.4.2.1 Transmitter’s procedure

When dot11MeshBeaconTimingReportInterval is set to non-zero value, the mesh STA shall report the TBTT and beacon interval of its neighbor STAs through the Beacon Timing element as described in this subclause. When dot11MeshBeaconTimingReportInterval is set to non-zero value, a mesh STA shall set Beacon Timing Report Enabled subfield in the Mesh Capability field of the Mesh Configuration element to 1.

The Beacon Timing element reports on MBSS, IBSS and infrastructure BSS Beacon frames that are received from the neighbor STAs with which it maintains synchronization on the operating channel. The Beacon Timing element shall be contained in Probe Response frames and in TBTT Adjustment Request frames. The Beacon Timing element shall be contained in Beacon frames as specified by the attributes of the beacon timing report procedure given by dot11MeshBeaconTimingReportInterval and dot11MeshBeaconTimingReportMaxNum.

When a Beacon frame is received from one of its neighbor STA with which a mesh STA maintains synchronization, the mesh STA shall calculate the TBTT of the received Beacon frame as follows:

TTBTT = Tr – (VT modulo (VB* 1024))

Where TTBTT is the calculated TBTT, Tr is the Beacon frame reception time measured in the mesh STA’s TSF timer, VT is the value in the Timestamp field in the received Beacon frame, and VB is the value in the Beacon interval field in the received Beacon frame. The TBTT calculated from the latest Beacon frame reception shall be contained in the Beacon Timing element, unless the elapsed time since the latest Beacon frame reception time is greater than 16 s.

Depending on the number of neighbors, a mesh STA may not include all the beacon reception timing from all of its neighbors within a single Beacon frame.

11C.12.4.2.2 Receiver’s procedure

A mesh STA that receives a Beacon Timing element can obtain the beacon reception timing of its neighbor mesh STA. This information is used for its TBTT selection (see TBTT selection) and its TBTT adjustment (see 11C.12.4.4). When dot11MeshTbttSelectionActivated is true, a mesh STA shall operate TBTT selection as described in TBTT selection) and TBTT adjustment as described in 11C.12.4.4. A mesh STA shall set TBTT Adjustment Enabled subfield in the Mesh Capability field of the Mesh Configuration element to 1, when dot11MeshTbttSelectionActivated is true.

A mesh STA can also check if its neighbor mesh STAs received its Beacon frame successfully, by checking whether neighbor mesh STAs' Beacon Timing elements contain Beacon Timing Information of which the Neighbor STA ID field matches either the 7 LSBs of the assigned AID or the 7 LSBs of its MAC address (taking the I/G bit as the MSB). When the Beacon Timing element is received from one of the peer mesh STAs, a mesh STA checks if the MSB of the Neighbor STA ID field is set to 0 and the rest of the field matches with the 7 LSBs of the AID value assigned by this peer mesh STA through the mesh peering establishment. When the Beacon Timing element is received from non-peer mesh STA, a mesh STA checks if the MSB of the Neighbor STA ID field is set to 1 and the rest of the field matches with the 7 LSBs of its own MAC address. If the matching is verified, the corresponding beacon timing information represents the mesh STA’s beacon reception, which means the neighbor mesh STA receives that Beacon frame correctly.

- TBTT selection

When dot11MeshTbttSelectionActivated is true, a mesh STA shall select its TBTT and its beacon interval so that its Beacon frames do not collide with Beacon frames transmitted by other STAs in its 2 hop range, using the information obtained from the Beacon Timing element from its neighbor mesh STAs.

Before start beaconing, a mesh STA shall obtain the TBTT and beacon interval of its neighbor STAs through discovery process (see 11C.1). The mesh STA shall also collect the beacon timing information contained in the Beacon Timing element from its neighbor STAs to find out the TBTT and beacon interval of STAs in 2 hop range. After collecting these information, the mesh STA shall look for the appropriate timing of its beacon transmission so that its Beacon frames do not collide with Beacon frames transmitted by other STAs in its 2 hop range. The mesh STA shall update its TSF timer and select its beacon interval to set its TBTTs to the appropriate timing, and then it shall start beaconing.

11C.12.4.4 TBTT adjustment

11C.12.4.4.1 Proactive adjustment

When dot11MeshTbttSelectionActivated is true, a mesh STA shall keep on monitoring the Beacon Timing element contained in the received Beacon frame from neighbor mesh STAs. The mesh STA shall verify that other STAs within its 2 hop range do not transmit Beacon frames at the same time by checking the conent of Beacon Timing element. If the Beacon frame is received from neighbor mesh STA that is either in active mode or in light sleep mode, the mesh STA shall also verify if the neighbor mesh STA received its Beacon frame as described in 11C.12.4.2.2.

When the mesh STA discovers that its Beacon frame may repeatedly collide with the Beacon frame of a neighbor or a neighbor’s neighbor, it shall start TBTT adjustment procedure described in 11C.12.4.4..3.

11C.12.4.4.2 Reactive adjustment

When a mesh STA discovers that Beacon frames from 2 or more neighbor mesh STAs are colliding repeatedly or series of TBTTs are close enough to trigger the consecutive beacon collision, it may transmit a TBTT Adjustment Request frame to the mesh STA whose TBTT comes last at a particular collision timing, in order to request the mesh STA to adjust its TBTT.

When dot11MeshTbttSelectionActivated is true, a mesh STA that receives a TBTT Adjustment Request frame shall determine if it can find out the appropriate timing for its TBTT. After the determination, the mesh STA that receives a TBTT Adjustment Request frame shall respond with a TBTT Adjustment Response frame containing the result of the determination in the Status Code field. If it agrees with this request, the Status Code field is set to 0, and shall start the TBTT adjustment procedure described in 11C.12.4.4..3.

11C.12.4.4.3 Adjustment procedure

TBTT Adjustment is achieved by suspending the TSF for a period of time. Specifically, the mesh STA shall operate the following procedure when it adjust its TBTT.

- Prior to and during the TBTT adjustment procedure, the mesh STA shall set the TBTT Adjusting field in the Mesh Configuration element to 1, in order to announce that it is adjusting its TBTT to avoid beacon collision.

- The mesh STA shall suspend its TSF timer for a period of time, no longer than half of the Group Delivery Idle Time (defined in Error: Reference source not found) within a single beacon period, to slow its TSs.

- The mesh STA may repeat suspending its TSF timer over multiple beacon periods until the beacon collision is avoided.

e)	Upon completion of the TBTT adjustment, the mesh STA shall change the TBTT Adjusting field in the Mesh Configuration element to 0.

11C.12.4.5 Frame transmission acrossing reported TBTT

When dot11MeshTbttSelectionActivated is true, a mesh STA should not extend its transmission other than Beacon frames across its neighbors reported beacon reception timing, in order to assist the proper Beacon frame reception at its neighbor mesh STAs. This operation helps in mitigating the Beacon collision at its neighbor mesh STAs. When both dot11MeshTbttSelectionActivated and dot11MCCAActivated is true, a mesh STA shall not extend its transmission other than Beacon frames across its neighbors reported beacon reception timing. After silencing for dot11MeshAverageBeaconFrameDuration µs from the reported TBTT, the mesh STA may start transmitting frames again.

11C.12.4.6 Delayed beacon transmission

A mesh STA may occasionally delay its Beacon frame transmission from its TBTT for a pseudo-random time. This attribute is specified by dot11MeshDelayedBeaconTxInterval, dot11MeshDelayedBeaconTxMinDelay, and dot11MeshDelayedBeaconTxMaxDelay. When dot11MeshDelayedBeaconTxInterval is set to non-zero value, a mesh STA shall delay its Beacon frame transmission from TBTT, once in dot11MeshDelayedBeaconTxInterval. Each delay time shall be randomly selected between dot11MeshDelayedBeaconTxMinDelay and dot11MeshDelayedBeaconTxMaxDelay µs

NOTE—Delayed beacon transmission allows mesh STAs to discover Beacon frames transmitted from multiple mesh STAs of which TBTTs are set nearly at the same time. It is recommended to set dot11MeshDelayedBeaconTxMaxDelay to a longer time than the typical duration of Beacon frames.

- (normative) ASN.1 encoding of the MAC and PHY MIB

dot11MeshNbrOffsetActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This object shall specify whether or not the station is neighbor offset protocol activated. The default value for this attribute is true. "

dot11MeshNbrOffsetMaxNeighbor OBJECT-TYPE

SYNTAX INTEGER  (1..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This object shall specify the maximum number of neighbor STAs with which the mesh STA maintains synchronization using the neighbor offset protocol. The default value for this attribute is 16. "

dot11MeshBeaconTimingReportInterval OBJECT-TYPE

SYNTAX INTEGER  (0..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This object shall specify when the Beacon Timing element is present in Beacon frames. When set to 0, Beacon Timing element is not present in either Beacon frames or Probe Response frames. In other cases, Beacon Timing element is present when the DTIM Count value in the Beacon frame is zero or equal to integer multiply of set value. The default value for this attribute is 0. "

dot11MeshBeaconTimingReportMaxNum OBJECT-TYPE

SYNTAX INTEGER  (0..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This object shall specify the maximum number of Beacon Timing Information field  contained in a Beacon Timing element in the transmitting Beacon frames. The default value for this attribute is 16. "

dot11MeshTSFAdjustmentActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This object shall specify whether or not the station activates TSF adjustment functions. When set to true, the mesh STA adjusts its local TSF timer according to the TSF offset value among neighbor STAs with which the STA maintains synchronization. The default value for this attribute is false. "

dot11MeshTbttSelectionActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This object shall specify whether or not the station activates Mesh TBTT selection functions, which is utilized to avoid beacon collisions. When set to true, the mesh STA selects its TBTT not to collide with neighbor’s and neighbor’s neighbor’s TBTT. The default value for this attribute is false. "

dot11MeshDelayedBeaconTxInterval OBJECT-TYPE

SYNTAX INTEGER  (0..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This attribute shall specify the interval of the delayed beacon transmission, for MBCA purpose. The value 0 indicates that the delayed beacon transmission is disabled. The default value for this attribute is 0."

dot11MeshDelayedBeaconTxMaxDelay OBJECT-TYPE

SYNTAX INTEGER  (0..4023)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This attribute shall specify the maximum delay time from a TBTT for the delayed beacon transmission, for MBCA purpose. The value is expressed in units of microseconds. The default value for this attribute is 0."

dot11MeshDelayedBeaconTxMinDelay OBJECT-TYPE

SYNTAX INTEGER  (0..4023)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This attribute shall specify the minimum delay time from a TBTT for the delayed beacon transmission, for MBCA purpose. The value is expressed in units of microseconds. The default value for this attribute is 0."

dot11MeshAverageBeaconFrameDuration OBJECT-TYPE

SYNTAX INTEGER  (0..4023)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This attribute shall specify the expected averaged duration of last 16 Beacon frames received by the neighboring mesh STAs. The value is expressed in units of microseconds."

References:

[1] Draft Amendment: Mesh Networking.  doc.: IEEE P802.11s/D4.0, December 2009.

[2] MAC Beaconing and sync comment resolution xls, 11-10/0099r0, January 2010.

[3] MAC Beaconing and sync comment resolution overview, 11-10/0100r0, January 2010.