IEEE P802.11
Wireless LANs

| MAC beaconing and sync comment resolution |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-01-20 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Kazuyuki Sakoda | Sony Corporation | 5-1-12 Kita-Shinagawa, Shinagawa-ku, Tokyo, 141-0001 Japan | +81-3-5448-4018 | KazuyukiA.Sakoda(at)jp.sony.com |

Abstract

This document provides suggested resolutions to CID 2004, 2006, 2010, 2011, 2016, 2021, 2022, 2025, 2026, 2029, 2030, 2033, 2037, 2038, 2039, 2043, 2044, 2048, 2051, 2192, 2505, 2720, 2722, 2798, 2799, 2001, 2002, 2003, 2005, 2008, 2015, 2017, 2018, 2019, 2020, 2023, 2024, 2027, 2028, 2031, 2034, 2035, 2036, 2040, 2041, 2042, 2045, 2046, 2047, 2470, 2609, 2714, 2716, 2717, 2719, 2733, 2742, and 2789.

## Summary of the intention of this document

- Polish the text on beaconing and sync related text.

- Define Beacon Timing Information field in the Beacon Timing element.

- Clean up and consolidate the description on MBCA, with clause restructuring.

- Change the TBTTAdjustment primitive to operate request/response handshake.

- Clarify the definition and the usage of Neighbor Last Beacon Time field.

- Clarify interaction between synchronization and power save.

## Suggested changes to the draft spec

- Fix typo or editorial errors as indicated by the received comments.

- Modify 7.3.2.102 to compy with the received comments. Adjust the references to this description.

- Merge clause X.3 to 11C.12.4 and consolidate the description on MBCA with clause restructuring.

- Define TTBT Adjustment Response frame, change clause 10.3.77 (MLME-TBTTA djustment primitive), change 11C.12.4 to use request/response handshake.

- Change the text to read Neighbor Last Beacon Time field contains the calculated TBTT rather than beacon reception time in a precise way.

- Add text to clarify how TSF adjustment and TBTT adjustment impact power save mesh STA.

Change “dot11MeshTbttSelectionActivated” to “dot11MeshTBTTSelectionActivated” globally in D4.0. There are 8 occurrences of  “dot11MeshTbttSelectionActivated” in D4.0.

Apply the following changes.
Corresponding changes to D4.0 are indicated in the following text with “Track Changes” on, to clarify the direction to the editor. Please update the part indicated by the “Track Changes” only.

- Status Code field

Insert the following rows into Status codesinput.doc and change the last row (Reserved) as shown.

| - Status codesinput.doc |  |
| --- | --- |
| Status code | Meaning |
|  | Authentication is rejected because an anti-clogging token is required |
|  | The request has not been successful as the STA could not find the appropriate TBTT. |
| +1 42-65535 | Reserved |

- Mesh Configuration element

- Mesh Capability

The Mesh Capability field comprises a set of values indicating whether a mesh STA is a possible candidate for mesh peering establishment. The details of the Mesh Capability field are shown in .

| B0 | B1 | B2 | B3 | B4 | B5 | B6 |  | B8 | B9 B15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Accepting Mesh Peerings | MCCA Supported | MCCA Enabled | Forwarding | Beacon Timing Report Enabled | TBTT Adjustment Enabled | TBTT Adjusting |  | Mesh Power Save Level | Reserved |
| Bits: 1 | 1 | 1 | 1 | 1 | 1 | 1 |  | 1 | 7 |
| - Mesh Capability field |  |  |  |  |  |  |  |  |  |

The Accepting Mesh Peerings subfield is set to 1 if the mesh STA is willing to establish mesh peerings with other mesh STAs and set to 0 otherwise.

The MCCA Supported subfield is set to 1 if the mesh STA implements MCCA services and set to 0 otherwise.

The MCCA Enabled subfield is set to 1 if the mesh STA is using MCCA services and set to 0 otherwise.

The Forwarding subfield is set to the value of the MIB variable dot11MeshForwarding. (#305)

The Beacon Timing Report Enabled subfield is set to 1 if MBCA beacon timing advertisement  is enableA and is set to 0 otherwise. (see Beacon timing advertisement)

The TBTT Adjustment Enabled subfield is set to 1 if the TBTT selection and TBTT adjustment is enabled, and is set to 0 otherwise. (See TBTT selection and 11C.12.4.4)

The TBTT Adjusting subfield is set to 1 while the TBTT adjusting procedure is on going to notify that the mesh STA’s TBTT is shifting intentionally, and is set to 0 otherwise. (See 11C.12.4.4.3TBTT selection)

When the Power Management subfield is 1, the value 0 in the Mesh Power Save Level field indicates that the mesh STA is operating in light sleep mode, and the value 1 in the Mesh Power Save Level field indicates that mesh STA is operating in deep sleep mode. When the Power Management subfield is 0, the Mesh Power Save Level field is reserved.

- Beacon Timing element

The Beacon Timing element is used to advertise the beacon timing information of neighboring STAs (mesh STAs, APs, or STAs in IBSS).

The format of the Beacon Timing element is shown in Beacon Timing element.

| Element ID | Length | Beacon Timing Information #1 | ... | Beacon Timing Information #N |
| --- | --- | --- | --- | --- |
| Octets: 1 | 1 | 5 | ... | 5 |
| - Beacon Timing element |  |  |  |  |

The Element ID is set to the value given in Error: Reference source not found for this element.

The Length field indicates the number of octets in the information field (fields following the Element ID and Length fields).

The Beacon Timing Information field contains a neighbor STA’s beacon timing information. When the mesh STA reports multiple beacon timing information, multiple Beacon Timing Information field are included in this element. The structure of the Beacon Timing Information field is defined in Figure sXX.

| Neighbor STA ID | Neighbor Last Beacon Time | Neighbor Beacon Interval |  |
| --- | --- | --- | --- |
| 1 | 2 | 2 |  |
| Figure sXX--Beacon Timing Information field |  |  |  |

The Neighbor STA ID field is an unsigned integer that indicates the identification of the neighbor STA corresponding to this beacon timing information. When a mesh peering is established with this neighbor STA, the MSB of this field is set to 0, and the rest of this field is set to the last 7 digits (7 LSBs) of the AID value assigned to this neighbor mesh STA. When a mesh peering is not established with this neighbor STA, the MSB of this field is set to 1, and the rest of this field is set to the last 7 digits (7 LSBs, taking the I/G bit as the MSB) of the 48-bit MAC address of this neighbor STA.

NOTE—Since the Neighbor STA ID field is provided in abbreviated form, it is possible that the same Neighbor STA ID value appears in multiple Beacon Timing Information fields.

The Neighbor Last Beacon Time field is an unsigned integer that indicates a TBTT  of the corresponding neighbor STA, measured in the mesh STA's local TSF timer. The TBTT is calculated from the latest reception time of the Beacon frame and values in the Timestamp field and the Beacon interval field of the Beacon frame, as described in 11C.12.4.2. The 2nd and 3rd least significant octets of the measured TSF value are contained in this field, in order to represent the time in units of 256 µs.

The Neighbor Beacon Interval field is an unsigned integer that indicates the beacon interval being used by the corresponding neighbor STA.

Detailed usage of the Beacon Timing element is described in Mesh Beacon Collision Avoidance (MBCA) .

- Action frame format details

- Mesh Resource Coordination Action frame details

- General

Action frame formats for management of Mesh Resource Coordination are defined in this subclause.

An Action field, in the octet field immediately after the Category field, differentiates the frame format. The Action field values associated with each frame format are defined in Mesh Resource Coordination Action field valuesinput.doc.

| - Mesh Resource Coordination Action field valuesinput.doc |  |
| --- | --- |
| Action field value | Description |
|  | Congestion Control Notification |
|  | MCCA Setup Request |
|  | MCCA Setup Reply |
|  | MCCAOP Advertisement Request |
|  | MCCAOP Advertisements |
|  | MCCAOP Reservation Teardown |
|  | TBTT Adjustment Request |
|  | TBTT Adjustment Response |
| 8 | Mesh Channel Switch Announcement |
| 9-255 | Reserved |

- TBTT Adjustment Request frame format

The TBTT Adjustment Request frame is used to request a particular neighbor peer mesh STA to adjust its TBTT. This frame is transmitted using individual addresses. The frame body of a TBTT Adjustment Request frame contains the information shown in TBTT Adjustment Request frame bodyinput.doc.

| - TBTT Adjustment Request frame bodyinput.doc |  |  |
| --- | --- | --- |
| Order | Information | Notes |
|  | Category |  |
|  | Action |  |
|  | Beacon Timing element |  |
| Last | Vendor Specific | One or more vendor-specific elements are optionally present. These elements follow all other elements. |

The Category field is set to the value in Error: Reference source not found for category Mesh Resource Coordination.

The Action field is set to the value in Mesh Resource Coordination Action field valuesinput.doc for this Action frame type.

The Beacon Timing element is set as described in Error: Reference source not found.

- TBTT Adjustment Response frame format

The TBTT Adjustment Response frame is used to respond to a TBTT adjustment request. This frame is transmitted using individual addresses. The frame body of a TBTT Adjustment Response frame contains the information shown in TBTT Adjustment Request frame bodyinput.doc.

| - TBTT Adjustment Request frame bodyinput.doc |  |  |
| --- | --- | --- |
| Order | Information | Notes |
|  | Category |  |
|  | Action |  |
|  | Status Code |  |
| - 4 | Beacon Timing element (optional) |  |
| Last | Vendor Specific | One or more vendor-specific elements are optionally present. These elements follow all other elements. |

The Category field is set to the value in Error: Reference source not found for category Mesh Resource Coordination.

The Action field is set to the value in Mesh Resource Coordination Action field valuesinput.doc for this Action frame type.

The Status Code field is defined in 7.3.1.9.

The Beacon Timing element, defined in Error: Reference source not found is present only if the Status Code is set to <ANA16> (i.e., when the request is not successful due to the neighbor constraint).

- MLME SAP interface

- MLME-Mesh TBTT Adjustment

The following primitives describe how a mesh STA requests TBTT adjustment to its neighboring peer mesh STA.

- MLME-MeshTBTTAdjustment.request

- Function

This primitive requests that a TBTT Adjustment Request frame be sent to the specified peer mesh STA.

- Semantics of the service primitive

The primitive parameters are as follows:

MLME-MeshTBTTAdjustment.request(

peerMAC,

BeaconTiming,

VendorSpecificInfo

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| peerMAC | MAC Address | Any valid individual MAC address | Specifies the address of the peer MAC entity to which the TBTT Adjustment Request is sent. |
| BeaconTiming | Beacon Timing element | As defined in Error: Reference source not found | A set of beacon timing information of the mesh STA. |
| VendorSpecificInfo | A set of elements | As defined in 7.3.2.26 | Zero or more elements |

- When generated

This primitive is generated by the SME to request that a TBTT Adjustment Request frame to be sent to a peer entity to request the adjustment of peer entity’s TBTT.

- Effect of receipt

On receipt of this primitive, the MLME constructs a TBTT Adjustment Request frame containing the Beacon Timing element. This frame is then scheduled for transmission. The MLME subsequently issues an MLME- MeshTBTTAdjustment.confirm that reflects the results of this request.

- MLME-MeshTBTTAdjustment.confirm

- Function

This primitive reports the results of a mesh TBTT adjustment request.

- Semantics of the service primitive

The primitive parameters are as follows:

MLME-MeshTBTTAdjustment.confirm(

peerMAC,

Status Code,

BeaconTiming,

VendorSpecificInfo

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| peerMAC | MAC Address | Any valid individual MAC address | Specifies the address of the peer MAC entity to which the TBTT Adjustment Request is sent. |
| Status Code | As defined in frame format. | As defined in frame format. | Indicates the result of the TBTT adjustment request. |
| BeaconTiming | Beacon Timing element | As defined in Error: Reference source not found | A set of beacon timing information of the requesting mesh STA. Present only when such an element was present in the TBTT Adjustment Response response. |
| VendorSpecificInfo | A set of elements | As defined in 7.3.2.26 | Zero or more elements |

- When generated

This primitive is generated by the MLME as a result of  an MLME-MeshTBTTAdjustment.request primitive to indicate  the request of that request.

- Effect of receipt

The SME is notified of the results of the mesh TBTT adjustment request.

- MLME-MeshTBTTAdjustment.indication

- Function

This primitive indicates that a specific peer MAC entity is requesting to adjust TBTT.

- Semantics of the service primitive

The primitive parameters are as follows:

MLME-MeshTBTTAdjustment.indication(

peerMAC,

BeaconTiming,

VendorSpecificInfo

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| peerMAC | MAC Address | Any valid individual MAC address | Specifies the address of the peer MAC entity from which the TBTT Adjustment request was received. |
| BeaconTiming | Beacon Timing element | As defined in Error: Reference source not found | A set of beacon timing information of the requesting mesh STA. |
| VendorSpecificInfo | A set of elements | As defined in 7.3.2.26 | Zero or more elements |

- When generated

This primitive is generated by the MLME as a result of the receipt of a TBTT Adjustment Request frame from a specific peer MAC entity.

- Effect of receipt

The SME is notified of the results of the receipt of the TBTT adjustment request by the specified peer MAC entity. The mesh STA received this primitive subsequently processes the TBTT adjustment described in 11C.12.4.4.3.

- MLME-MeshTBTTAdjustment.response

- Function

This primitive is used to send a response to a specific peer MAC entity that requested a TBTT adjustment to the STA that issued this primitive.

- Semantics of the service primitive

The primitive parameters are as follows:

MLME-MeshTBTTAdjustment.indication(

peerMAC,

Status Code,

BeaconTiming,

VendorSpecificInfo

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| peerMAC | MAC Address | Any valid individual MAC address | Specifies the address of the peer MAC entity from which the TBTT Adjustment request was received. |
| Status Code | As defined in frame format. | As defined in frame format. | Indicates the result response to the TBTT adjustment request from the peer mesh STA. |
| BeaconTiming | Beacon Timing element | As defined in Error: Reference source not found | A set of beacon timing information of the requesting mesh STA. Only present when Status Code is set to non-zero value. |
| VendorSpecificInfo | A set of elements | As defined in 7.3.2.26 | Zero or more elements |

- When generated

This primitive is generated by the SME of a STA as a resonse to an MLME-MeshTBTTAdjustment.indication primitive.

- Effect of receipt

This primitive initiates transmission of a response to the specific peer MAC entity that requested the TBTT adjustment.

On receipt of this primitive, the MLME constructs a TBTT Adjustment Response frame. When the Status Code is set to non-zero value, the frame contains the Beacon Timing element. This frame is then scheduled for transmission.

- Synchronization and beaconing in MBSSs

- TSF for MBSSs

The mesh STA shall initialize its TSF timer depending on its active synchronization protocol and the given attribute of dot11MeshTbttSelectionActivated. The mesh STA shall periodically transmit Beacon frames that contain a copy of its TSF timer to announce its local time reference to its neighbor mesh STAs. Mesh STAs receiving a Beacon frame may accept the timing information depending on their active synchronization protocol.

Each mesh STA shall maintain a TSF timer as described in 11.1.2, and conform to the TSF timer accuracy as described in 11.1.2.4.

- Extensible synchronization framework

- General

This standard introduces an extensible framework to enable the implementation of multiple synchronization protocols for mesh STAs. Within the extensible synchronization framework, the neighbor offset synchronization protocol is defined as a default mandatory protocol in order to enable minimal synchronization capabilities and interoperability between mesh STAs that use MCCA, MBCA, or operate in power save mode. However, to accommodate various application needs, the framework allows flexibility to integrate future synchronization protocols for MBSSs. A vendor can implement any synchronization protocol using this framework to meet special application needs.

Mesh STAs shall announce the active synchronization protocol using the Synchronization Protocol Identifier field in the Mesh Configuration element in their Beacon and Probe Response frames. Although a mesh STA may include multiple implementations of the synchronisation protocol, all the mesh STAs in an MBSS shall use the same synchronisation protocol.

- Neighbor Offset Protocol

- Basic operation

The Neighbor Offset Protocol allows a mesh STA to keep track of the time base of its neighbor mesh STAs. The attributes of the Neighbor Offset Protocol are given by dot11MeshNbrOffsetActivated and dot11MeshNbrOffsetMaxNeighbor. When dot11MeshNbrOffsetActivated is true, the mesh STA shall maintain a timing offset value between its own TSF timer and the TSF timer of each neighbor mesh STA with which it synchronizes. A mesh STA shall maintain synchronization with all of its neighbor peer mesh STAs. A mesh STA should maintain synchronization with neighbor mesh STAs that are in the same the MBSS, as long as the number of neighbor mesh STAs in the same mesh BSS does not exceed dot11MeshNbrOffsetMaxNeighbor. Additionally, mesh STA should maintain synchronization with neighbor STAs that are outside of the MBSS, as long as the number of neighbor STAs does not exceed dot11MeshNbrOffsetMaxNeighbor.

A mesh STA may start its TSF timer independently of other mesh STAs. A mesh STA shall update the value of its TSF timer based on time stamps received in Beacon or Probe Response frames from neighbor STAs as described in 11C.12.2.2.2.

A mesh STA that utilizes the neighbor offset protocol shall update the timing offset value with respect to the neighbor mesh STA based on time stamps from the received Beacon and Probe Response frames as follows:

OffsetForNeighborSTA = received time stamp from neighbor mesh STA - own TSF timer value

The offset value is represented as 2’s complement. The unit of the offset value is µs.

A mesh STA may translate its own TSF timer value into another mesh STA's timer base by adding the offset as follows:

Translated neighbor mesh STA's TSF timer value = own TSF timer value+OffsetForNeighborSTA

Upon receipt of an MLME-MeshNeighborOffsetMeasure.request, the MLME shall report the measured OffsetForNeighborSTA value to SME by responding with MLME-MeshNeighborOffsetMeasure.confirm. The OffsetForNeighborSTA values are used to provide the timing reference of neighboring mesh STAs. These values are needed in order to provide such services as MCCA, MBCA, or power management.

- TSF Adjustment

A mesh STA shall adjust its TSF timer to compensate the clock drift among STAs with which it maintains synchronization. A mesh STA shall examine the reception time of the Beacon frames from neighboring STAs with which it maintains synchronization and adjust its TSF timer to compensate the relative timing error among neighbor mesh STAs caused by the clock drift.

A mesh STA shall operate the following clock drift compensation procedures.

- When the mesh STA receives a Beacon frame or a Probe Response frame from one of the neighboring STAs with which it maintains synchronization, the mesh STA shall calculate the clock drift amount by comparing the OffsetForNeighborSTA value obtained previously for this neighbor STA and the OffsetForNeighborSTA value obtained from the frame reception.

- If the received Beacon frame or Probe Response frame contains the Mesh Configuration element and the TBTT Adjusting subfield in the Mesh Configuration field is 1, the mesh STA shall not perform the clock drift compensation based on the received frame.

- When the mesh STA sees one of its neighbor STA’s TSF is delaying due to the clock drift, it shall suspend its TSF timer for the delay amount, so that its TSF counting frequency will be identical to the most delaying neighbor STA’s. The mesh STA shall suspend its TSF timer frequently enough so that the delay amount within a single beacon period does not exceed 0.08 % of its beacon interval.).

NOTE— This clock drift compensation procedure do not intend to maintain a strict synchronization, It aims to stop TBTT drifting away among neighbor mesh STAs, allowing affordable jitter of TSF timer. Since the TSF is adjusted in the slower direction, TSF adjustment does not cause a lost of Beacon frame at neighbor mesh STAs that alternate Active state and Doze state.

A mesh STA in deep sleep mode may not listen to neighbor mesh STA’s Beacon frame. However, it shall conform to the clock drift compensation procedures and TSF jitter allowance described in this subclause. See X.4.6 for some more guideline.

- Beaconing

- Beacon generation in MBSSs

A mesh STA transmits Beacon frames that are specific to an MBSS. Beacon frames for MBSS and infrastructure BSS or IBSS are differentiated by the Capability Information field in the Beacon frame as specified in Error: Reference source not found. A mesh STA that collocates with an AP generates Beacon frames for the MBSS independently of the AP.

The mesh STA shall define a series of TBTTs exactly dot11BeaconPeriod TUs apart. Time zero is defined to be a TBTT with the Beacon frame being a DTIM. At each TBTT, the mesh STA shall schedule a Beacon frame as the next frame for transmission according to the medium access rules specified in Clause 9. The beacon period is included in Beacon and Probe Response frames.

- Beacon reception for mesh STA

A mesh STA may receive the Beacon frames from neighboring STAs with which it synchronize, without regard for the BSSID, SSID, or Mesh ID.

- Mesh Beacon Collision Avoidance (MBCA)

- Overview

Mesh STAs use the Mesh Beacon Collision Avoidance (MBCA) to detect and mitigate collisions among Beacon frames transmitted by other STAs (including mesh STAs, APs, and STAs in an IBSS) on the same channel within the range of 2 hops. MBCA is intended to resolve hidden node problems with respect to Beacon frames.

NOTE—Beacon frames are essential for passive scan, power management, and MCCA. However, Beacon frames are transmitted without acknowledgement and might collide with other frames. In a mesh BSS, multiple STAs transmit Beacon frames periodically, and mesh STAs might be located out of range of each other. This implies that Beacon frames might suffer from so-called hidden node problem and can not be received by neighbor STAs. Once Beacon frames from hidden nodes start to collide each other, Beacon frames will keep on colliding if these hidden nodes transmit Beacon frames at the same beacon interval which is a typical operation. MBCA provides a set of rules to mitigate this problem.

- Beacon timing advertisement

11C.12.4.2.1 Transmitter’s procedure

When dot11MeshBeaconTimingReportInterval is set to non-zero value, the mesh STA shall report the TBTT and beacon interval of its neighbor STAs through the Beacon Timing element as described in this subclause. When dot11MeshBeaconTimingReportInterval is set to non-zero value, a mesh STA shall set Beacon Timing Report Enabled subfield in the Mesh Capability field of the Mesh Configuration element to 1.

The Beacon Timing element reports on MBSS, IBSS and infrastructure BSS Beacon frames that are received from the neighbor STAs with which it maintains synchronization on the operating channel. The Beacon Timing element shall be contained in Probe Response frames and in TBTT Adjustment Request frames. The Beacon Timing element shall be contained in Beacon frames as specified by the attributes of the beacon timing report procedure given by dot11MeshBeaconTimingReportInterval and dot11MeshBeaconTimingReportMaxNum.

When a Beacon frame is received from one of its neighbor STA with which a mesh STA maintains synchronization, the mesh STA shall calculate the TBTT of the received Beacon frame as follows:

TTBTT = Tr – (VT modulo (VB* 1024))

Where TTBTT is the calculated TBTT, Tr is the Beacon frame reception time measured in the mesh STA’s TSF timer, VT is the value in the Timestamp field in the received Beacon frame, and VB is the value in the Beacon interval field in the received Beacon frame. The TBTT calculated from the latest Beacon frame reception shall be contained in the Beacon Timing element, unless the elapsed time since the latest Beacon frame reception time is greater than 16 s.

Depending on the number of neighbors, a mesh STA may not include all the beacon reception timing from all of its neighbors within a single Beacon frame.

11C.12.4.2.2 Receiver’s procedure

A mesh STA that receives a Beacon Timing element can obtain the beacon reception timing of its neighbor mesh STA. This information is used for its TBTT selection (see TBTT selection) and its TBTT adjustment (see 11C.12.4.4). When dot11MeshTbttSelectionActivated is true, a mesh STA shall operate TBTT selection as described in TBTT selection) and TBTT adjustment as described in 11C.12.4.4. A mesh STA shall set TBTT Adjustment Enabled subfield in the Mesh Capability field of the Mesh Configuration element to 1, when dot11MeshTbttSelectionActivated is true.

A mesh STA can also check if its neighbor mesh STAs received its Beacon frame successfully, by checking whether neighbor mesh STAs' Beacon Timing elements contain Beacon Timing Information of which the Neighbor STA ID field matches either the 7 LSBs of the assigned AID or the 7 LSBs of its MAC address (taking the I/G bit as the MSB). When the Beacon Timing element is received from one of the peer mesh STAs, a mesh STA checks if the MSB of the Neighbor STA ID field is set to 0 and the rest of the field matches with the 7 LSBs of the AID value assigned by this peer mesh STA through the mesh peering establishment. When the Beacon Timing element is received from non-peer mesh STA, a mesh STA checks if the MSB of the Neighbor STA ID field is set to 1 and the rest of the field matches with the 7 LSBs of its own MAC address. If the matching is verified, the corresponding beacon timing information represents the mesh STA’s beacon reception, which means the neighbor mesh STA receives that Beacon frame correctly.

- TBTT selection

When dot11MeshTbttSelectionActivated is true, a mesh STA shall select its TBTT and its beacon interval so that its Beacon frames do not collide with Beacon frames transmitted by other STAs in its 2 hop range, using the information obtained from the Beacon Timing element from its neighbor mesh STAs.

Before start beaconing, a mesh STA shall obtain the TBTT and beacon interval of its neighbor STAs through discovery process (see 11C.1). The mesh STA shall also collect the beacon timing information contained in the Beacon Timing element from its neighbor STAs to find out the TBTT and beacon interval of STAs in 2 hop range. After collecting these information, the mesh STA shall look for the appropriate timing of its beacon transmission so that its Beacon frames do not collide with Beacon frames transmitted by other STAs in its 2 hop range. The mesh STA shall update its TSF timer and select its beacon interval to set its TBTTs to the appropriate timing, and then it shall start beaconing.

11C.12.4.4 TBTT adjustment

11C.12.4.4.1 Proactive adjustment

When dot11MeshTbttSelectionActivated is true, a mesh STA shall keep on monitoring the Beacon Timing element contained in the received Beacon frame from neighbor mesh STAs. The mesh STA shall verify that other STAs within its 2 hop range do not transmit Beacon frames at the same time by checking the conent of Beacon Timing element. If the Beacon frame is received from neighbor mesh STA that is either in active mode or in light sleep mode, the mesh STA shall also verify if the neighbor mesh STA received its Beacon frame as described in 11C.12.4.2.2.

When the mesh STA discovers that its Beacon frame may repeatedly collide with the Beacon frame of a neighbor or a neighbor’s neighbor and it discovers that its TBTT appears later than that of colliding STA, it shall start TBTT adjustment procedure described in 11C.12.4.4..3.

11C.12.4.4.2 Reactive adjustment

When a mesh STA discovers that Beacon frames from 2 or more neighbor mesh STAs are colliding repeatedly or series of TBTTs are close enough to trigger the consecutive beacon collision, it may transmit a TBTT Adjustment Request frame to the mesh STA whose TBTT appears last at a particular collision timing, in order to request the mesh STA to adjust its TBTT.

When dot11MeshTbttSelectionActivated is true, a mesh STA that receives a TBTT Adjustment Request frame shall determine if it can find out the appropriate timing for its TBTT. After the determination, the mesh STA that receives a TBTT Adjustment Request frame shall respond with a TBTT Adjustment Response frame containing the result of the determination in the Status Code field. If it agrees with this request, the Status Code field is set to 0, and shall start the TBTT adjustment procedure described in 11C.12.4.4.3. If it does not agree with the request, it shall indicate the reason in the Status Code field in the TBTT Adjustment Response frame.

11C.12.4.4.3 Adjustment procedure

TBTT Adjustment is achieved by suspending the TSF for a period of time. Specifically, a mesh STA shall operate the following procedure when it adjust its TBTT.

- Prior to and during the TBTT adjustment procedure, the mesh STA shall set the TBTT Adjusting field in the Mesh Configuration element to 1, in order to announce that it is adjusting its TBTT to avoid beacon collision.

- The mesh STA shall look for the alternative TBTT that does not cause beacon collision. When the mesh STA is in light sleep mode or in deep sleep mode, it shall stay awake state at least for its beacon period and assure that the alternative TBTT does not cause beacon collision.

- The mesh STA shall suspend its TSF timer for a period of time, no longer than half of the Group Delivery Idle Time (defined in Error: Reference source not found) within a single beacon period, to slow its TSs.

- The mesh STA may repeat suspending its TSF timer over multiple beacon periods until its TBTT is set to the timing that does not cause beacon collision.

e)	Upon completion of the TBTT adjustment, the mesh STA shall change the TBTT Adjusting field in the Mesh Configuration element to 0.

NOTE—Since the TBTT is adjusted in the slower direction, TBTT adjustment does not cause a lost of Beacon frame at neighbor mesh STAs that alternate Active state and Doze state.

NOTE—A mesh STA in deep sleep mode may interpret its neighbor mesh STA’s TBTT adjustment as a large TSF jitter. When a mesh STA in deep sleep mode observes this, it should not take this jitter as clock drift.

11C.12.4.5 Frame transmission acrossing reported TBTT

When dot11MeshTbttSelectionActivated is true, a mesh STA should not extend its transmission other than Beacon frames across its neighbors reported beacon reception timing, in order to assist the proper Beacon frame reception at its neighbor mesh STAs. This operation helps in mitigating the Beacon collision at its neighbor mesh STAs. When both dot11MeshTbttSelectionActivated and dot11MCCAActivated is true, a mesh STA shall not extend its transmission other than Beacon frames across its neighbors reported beacon reception timing. After silencing for dot11MeshAverageBeaconFrameDuration µs from the reported TBTT, the mesh STA may start transmitting frames again.

11C.12.4.6 Delayed beacon transmission

A mesh STA may occasionally delay its Beacon frame transmission from its TBTT for a pseudo-random time. This attribute is specified by dot11MeshDelayedBeaconTxInterval, dot11MeshDelayedBeaconTxMinDelay, and dot11MeshDelayedBeaconTxMaxDelay. When dot11MeshDelayedBeaconTxInterval is set to non-zero value, a mesh STA shall delay its Beacon frame transmission from TBTT, once in dot11MeshDelayedBeaconTxInterval. Each delay time shall be randomly selected between dot11MeshDelayedBeaconTxMinDelay and dot11MeshDelayedBeaconTxMaxDelay µs

NOTE—Delayed beacon transmission allows mesh STAs to discover Beacon frames transmitted from multiple mesh STAs of which TBTTs are set nearly at the same time. It is recommended to set dot11MeshDelayedBeaconTxMaxDelay to a longer time than the typical duration of Beacon frames.

- (normative) ASN.1 encoding of the MAC and PHY MIB

Dot11MeshStationConfigEntry ::=

SEQUENCE {

dot11MeshID								OCTET STRING,

dot11MeshSecurityActivated						TruthValue,

dot11MeshConfigGroupUpdateCount					INTEGER,

dot11MeshActivePathSelectionProtocol				INTEGER,

dot11MeshForwarding							TruthValue,

dot11MeshMaxRetries							INTEGER,

dot11MeshRetryTimeout						INTEGER,

dot11MeshTTL								INTEGER,

dot11MeshPortalAnnouncementProtocol 				TruthValue,

dot11MeshPortalAnnouncementInterval				INTEGER,

dot11MeshConfirmTimeout						INTEGER,

dot11MeshHoldingTimeout						INTEGER,

dot11MeshNbrOffsetActivated					TruthValue,

dot11MeshNbrOffsetMaxNeighbor					INTEGER,

dot11MeshBeaconTimingReportInterval				INTEGER,

dot11MeshBeaconTimingReportMaxNum					INTEGER,

dot11MeshTbttSelectionActivated					TruthValue,

dot11MeshDelayedBeaconTxInterval					INTEGER,

dot11MeshDelayedBeaconTxMaxDelay					INTEGER,

dot11MeshDelayedBeaconTxMinDelay					INTEGER,

dot11MeshAverageBeaconFrameDuration				INTEGER,

dot11MeshSTAMissingAckRetryLimit					INTEGER,

dot11MCCAImplemented 						TruthValue,

dot11MCCAActivated 							TruthValue,

dot11MAFlimit 							INTEGER,

dot11MCCAScanDuration						INTEGER,

dot11MCCAdvertPeriodMax						INTEGER,

dot11MCCATrackStates OBJECT-TYPE					INTEGER,

dot11MCCAOPtimeout							INTEGER,

dot11MCCACWmin							INTEGER,

dot11MCCAAIFSN							INTEGER

}

dot11MeshNbrOffsetActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This object shall specify whether or not the station is neighbor offset protocol activated. The default value for this attribute is true. "

dot11MeshNbrOffsetMaxNeighbor OBJECT-TYPE

SYNTAX INTEGER  (1..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This object shall specify the maximum number of neighbor STAs with which the mesh STA maintains synchronization using the neighbor offset protocol. The default value for this attribute is 16. "

dot11MeshBeaconTimingReportInterval OBJECT-TYPE

SYNTAX INTEGER  (0..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This object shall specify when the Beacon Timing element is present in Beacon frames. When set to 0, Beacon Timing element is not present in either Beacon frames or Probe Response frames. In other cases, Beacon Timing element is present when the DTIM Count value in the Beacon frame is zero or equal to integer multiply of set value. The default value for this attribute is 0. "

dot11MeshBeaconTimingReportMaxNum OBJECT-TYPE

SYNTAX INTEGER  (0..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This object shall specify the maximum number of Beacon Timing Information field  contained in a Beacon Timing element in the transmitting Beacon frames. The default value for this attribute is 16. "

dot11MeshTbttSelectionActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This object shall specify whether or not the station activates Mesh TBTT selection functions, which is utilized to avoid beacon collisions. When set to true, the mesh STA selects its TBTT not to collide with neighbor’s and neighbor’s neighbor’s TBTT. The default value for this attribute is false. "

dot11MeshDelayedBeaconTxInterval OBJECT-TYPE

SYNTAX INTEGER  (0..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This attribute shall specify the interval of the delayed beacon transmission, for MBCA purpose. The value 0 indicates that the delayed beacon transmission is disabled. The default value for this attribute is 0."

dot11MeshDelayedBeaconTxMaxDelay OBJECT-TYPE

SYNTAX INTEGER  (0..4023)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This attribute shall specify the maximum delay time from a TBTT for the delayed beacon transmission, for MBCA purpose. The value is expressed in units of microseconds. The default value for this attribute is 0."

dot11MeshDelayedBeaconTxMinDelay OBJECT-TYPE

SYNTAX INTEGER  (0..4023)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This attribute shall specify the minimum delay time from a TBTT for the delayed beacon transmission, for MBCA purpose. The value is expressed in units of microseconds. The default value for this attribute is 0."

dot11MeshAverageBeaconFrameDuration OBJECT-TYPE

SYNTAX INTEGER  (0..4023)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This attribute shall specify the expected averaged duration of last 16 Beacon frames received by the neighboring mesh STAs. The value is expressed in units of microseconds."

References:

[1] Draft Amendment: Mesh Networking.  doc.: IEEE P802.11s/D4.0, December 2009.

[2] MAC Beaconing and sync comment resolution xls, 11-10/0099r2, January 2010.

[3] MAC Beaconing and sync comment resolution overview, 11-10/0100r2, January 2010.