IEEE P802.11
Wireless LANs

| Initial Sponsor Ballot – GAS MIB Update |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-1-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Dave Stephenson | Cisco Systems, Inc. | 170 W. Tasman Dr. San Jose, CA 95134 | +1 408 527 7991 | daves@cisco.com |
| Stephen McCann | Research in Motion | 200 Bath Road, Slough, Berks, SL1 3XE, UK | +44 1753 667099 | smccann@rim.com |

Abstract

This submission addresses the following comments from the P802.11u Initial Sponsor Ballot: rows # 297, 298, 299 and 300.

This document is based on IEEE 802.11u-d8.01.

Annex D

(normative) 

ASN.1 encoding of the MAC and PHY MIB

Change the dot11GASAdvertisement Table as shown below:

-- **********************************************************************

-- * dot11GASAdvertisement TABLE

-- **********************************************************************

dot11GASAdvertisementTable OBJECT-TYPE

SYNTAX         SEQUENCE OF Dot11GASAdvertisementEntry

MAX-ACCESS     not-accessible

STATUS         current

DESCRIPTION

“This object is a table of GAS counters that allows for multiple instantiations of those counters on an STA.”

::= { dot11imt 7}

dot11GASAdvertisementEntry OBJECT-TYPE

SYNTAX 		Dot11GASAdvertisementEntry

MAX-ACCESS 		not-accessible

STATUS     	current

DESCRIPTION

“This object provides the attributes identifying a GAS counter within an STA.”

INDEX { dot11GASAdvertisementId }

::= { dot11GASAdvertisementTable 1 }

Dot11GASAdvertisementEntry ::=

SEQUENCE 	{

dot11GASAdvertisementId		INTEGER,

dot11GASResponseTimeout	INTEGER,

dot11GASComebackDelay		INTEGER,

dot11GASQueryResponseLengthLimit		INTEGER,

dot11GASQueries		Counter32,

dot11GASQueryRate		Gauge,

dot11GASResponses		Counter32,

dot11GASResponseRate		Gauge,

dot11GASTransmittedFragmentCount		Counter32,

dot11GASReceivedFragmentCount		Counter32,

dot11GASNoRequestOutstanding	Counter32,

dot11GASResponsesDiscarded	Counter32,

dot11GASFailedResponses	Counter32

}

dot11GASAdvertisementId OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

“The one octet identification number for the GAS Advertisement protocol, as defined in Table 7-43be, for which statistics are stored the logical row of the GASAdvertisement table.”

::= { dot11GASAdvertisementEntry 1}

dot11GASResponseTimeout OBJECT-TYPE

SYNTAX INTEGER (1000..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This parameter shall indicate the GAS response timeout value in TUs for the protocol identified by dot11GASAdvertisementId.”

DEFVAL {5000)

::= { dot11GASAdvertisementEntry 2 }

dot11GASComebackDelay OBJECT-TYPE

SYNTAX INTEGER (0..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This object identifies the GAS Comeback Delay (in TUs) to be used for the protocol identified by dot11GASAdvertisementId.”

DEFVAL {1000}

::= { dot11GASAdvertisementEntry 3 }

dot11GASQueryResponseLengthLimit OBJECT-TYPE

SYNTAX INTEGER (1..127)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This object indicates the maximum number of octets an AP will transmit in one or more Query Response fields contained within GAS Comeback Response frame(s) for the protocol identified by dot11GASAdvertisementId. A value of 127 means the maximum limit enforced is contained by the maximum allowable number of fragments in the GAS Query Fragment Response ID.”

::= { dot11GASAdvertisementEntry 4}

dot11GASQueries OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“The number of GAS queries sent or received for the protocol identified by dot11GASAdvertisementId.”

::= { dot11GASAdvertisementEntry 5 }

dot11GASQueryRate OBJECT-TYPE

SYNTAX Gauge

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“The number of GAS queries per minute received for the protocol identified by dot11GASAdvertisementId, averaged over the previous ten minutes.”

::= { dot11GASAdvertisementEntry 6}

dot11GASResponses OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“The number of GAS responses sent or received for the protocol identified by dot11GASAdvertisementId.”

::= { dot11GASAdvertisementEntry 7}

dot11GASResponseRate OBJECT-TYPE

SYNTAX Gauge

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“The number of responses to GAS queries per minute received for the protocol identified by dot11GASAdvertisementId, averaged over the previous ten minutes.”

::= { dot11GASAdvertisementEntry 8}

dot11GASTransmittedFragmentCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This counter shall be incremented for an acknowledged GAS MMPDU for the protocol identified by dot11GASAdvertisementId.”

::= { dot11GASAdvertisementEntry 9}

dot11GASReceivedFragmentCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This counter shall be incremented for each successfully received GAS MMPDU for the protocol identified by dot11GASAdvertisementId.”

::= { dot11GASAdvertisementEntry 10 }

dot11GASNoRequestOutstanding OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This counter shall be incremented each time a STA returns a status code of no request outstanding in a GAS Initial Response or GAS Comeback Response frame for the protocol identified by dot11GASAdvertisementId.”

::= { dot11GASAdvertisementEntry 11 }

dot11GASResponsesDiscarded OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This counter shall be incremented each a STA discards a GAS response due to the expiry of the dot11GASResponseBufferingTime timer for the protocol identified by dot11GASAdvertisementId.”

::= { dot11GASAdvertisementEntry 12 }

dot11GASFailedResponses OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This counter shall be incremented each time a STA commences transmitting a GAS response but fails to successfully complete the transmission of all GAS fragments in that response due to the expiry of the dot11GASResponseTimeout timer for the protocol identified by dot11GASAdvertisementId.”

::= { dot11GASAdvertisementEntry 13 }

-- **********************************************************************

-- * End of dot11GASAdvertisement TABLE

-- **********************************************************************