IEEE P802.11
Wireless LANs

| Initial Sponsor Ballot – Location by Reference |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-01-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Dave Stephenson | Cisco Systems, Inc. | 170 W. Tasman Dr. San Jose, CA 95134 | +1 408 527 7991 | daves@cisco.com |

Abstract

This submission addresses the following comments from the P802.11u Initial Sponsor Ballot: row # 256.

This document is based on IEEE 802.11u-d8.01.

7	Frame formats

7.3	Management frame body components

7.3.4	Native query protocol elements

7.3.4.1	Capability List

Insert a new row in Table 7-43bg as shown below:

| Table 7-43bg—Native Query Protocol info ID definitions |  |  |
| --- | --- | --- |
| Info Name | Info ID | Native Info Element (clause) |
| Reserved | 0-255 | n/a |
| Capability List | 256 | 7.3.4.1 |
| Venue Name information | 257 | 7.3.4.2 |
| Emergency Call Number information | 258 | 7.3.4.3 |
| Network Authentication Type information | 259 | 7.3.4.4 |
| Roaming Consortium List | 260 | 7.3.4.5 |
| IP Address Type Availability information | 261 | 7.3.4.7 |
| NAI Realm List | 262 | 7.3.4.8 |
| 3GPP Cellular Network information | 263 | 7.3.4.9 |
| AP Geospatial Location | 264 | 7.3.4.10 |
| AP Civic Location | 265 | 7.3.4.11 |
| AP Location Public Identifier URI | 266 | 7.3.4.12 |
| Domain Name List | 267 | 7.3.4.13 |
| Emergency Alert URI | 268 | 7.3.4.14 |
| Reserved | 269– 56796 | n/a |
| Native Query Protocol vendor-specific list | 56797 | 7.3.4.6 |
| Reserved | 56798 – 65535 | n/a |

Insert the following new sub-clause, re-number subsequent sub-clauses as needed:

7.3.4.12	AP Location Public Identifier URI

The AP Location Public Identifier URI element provides an indirect reference to where the location information for the AP can be retrieved. This list element may be returned in response to a Native GAS Query Request. The format of the AP Location Public Identifier URI element is provided in Figure 7-95o145a.

|  | Info ID | Length | Public Identifier URI |
| --- | --- | --- | --- |
| Octets: | 2 | 2 | variable |

Figure 7-95o145a—AP Location Public Identifier URI format

The Info ID field is equal to the value in Table 7-43by corresponding to the AP Location Public Identifier URI.

The Length field is a 2-octet field whose value is the length of the AP Location Public Identifier URI field.

The Public Identifier URI field is defined in 7.3.2.22.13.