IEEE P802.11
Wireless LANs

| Proposed Specification Framework edits for preamble structure and A-MPDU |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-01-21 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Robert Stacey | Intel | 2111 NE 25th Ave, Hillsboro OR 97124, USA | 503-724-0893 | robert.j.stacey@intel.com |
| Eldad Perahia | Intel | 2111 NE 25th Ave, Hillsboro OR 97124, USA |  | eldad.perahia@intel.com |
| Adrian Stephens | Intel |  |  | adrian.p.stephens@intel.com |
| Assaf Kasher | Intel |  |  | assaf.kasher@intel.com |
| Solomon Trainin | Intel |  |  | solomon.trainin@intel.com |
| Michelle Gong | Intel |  |  | michelle.x.gong@intel.com |
| Raja Banerjea | Marvell | 5488 Marvell Lane, Santa Clara CA, 95054 | 408.222.3713 | rajab@marvell.com |
| Hongyuan Zhang | Marvell | 5488 Marvell Lane, Santa Clara CA, 95054 | 408.222.1837 | hongyuan@marvell.com |
| Sudhir Srinivasa | Marvell | 5488 Marvell Lane, Santa Clara CA, 95054 |  | sudhirs@marvell.com |
| Yong Liu | Marvell | 5488 Marvell Lane, Santa Clara CA, 95054 |  | yongliu@marvell.com |
| Ning Zhang | Atheros | 5480 Great America Parkway, Santa Clara, CA 95054, USA | 408-773-5363 | Ning.Zhang@atheros.com |
| William McFarland | Atheros | 5480 Great America Parkway, Santa Clara, CA 95054, USA |  | William.McFarland@atheros.com |
| Kai Shi | Atheros | 5480 Great America Parkway, Santa Clara, CA 95054, USA |  | Kai.Shi@atheros.com |
| Joshua Zhao | Atheros | 5480 Great America Parkway, Santa Clara, CA 95054, USA |  | Joshua.Zhao@atheros.com |
| Qifan Chen | Atheros | 5480 Great America Parkway, Santa Clara, CA 95054, USA |  | Qifan.chen@atheros.com |
| James Cho | Atheros | 5480 Great America Parkway, Santa Clara, CA 95054, USA |  | James.Cho@atheros.com |
| Allert Van Zelst | Qualcomm | Netherlands |  | allert@quallcomm.com |
| Richard Van Nee | Qualcomm | Netherlands |  | rvannee@qualcomm.com |
| Santosh Abraham | Qualcomm | San Diego, USA |  | sabraham@qualcomm.com |
| Hemanth Sampath | Qualcomm | San Diego, USA |  | hsampath@qualcomm.com |
| Rolf De Vegt | Qualcomm | Santa Clara, USA |  | rolfv@qualcomm.com |
| VK Jones | Qualcomm | Santa Clara, USA |  | vkjones@qualcomm.com |
| Vinko Erceg | Broadcom |  | 858 521 5885 | verceg@broadcom.com |
| Joseph Lauer | Broadcom |  |  | jlauer@broadcom.com |
| Mathew Fischer | Broadcom |  |  | mfischer@broadcom.com |
| Tushar Moorti | Broadcom |  |  | rtm@broadcom.com |
| Peiman Amini | Broadcom |  |  | pamini@broadcom.com |
| Joonsuk Kim | Broadcom |  |  | joonsuk@broadcom.com |
| Yuichi Morioka | Sony |  |  | yuichi.morioka@jp.sony.com |
| Ted Booth | Sony |  |  | ted.booth@am.sony.com |
| Yasushi Takatori | NTT |  |  | takatori.yasushi@lab.ntt.co.jp |
| Yusuke Asai | NTT |  |  | asai.yusuke@lab.ntt.co.jp |
| Ichihiko Toyoda | NTT |  |  | toyoda.ichihiko@lab.ntt.co.jp |
| Chiu Ngo | Samsung Electronics | 75 W. Plumeria Dr. San Jose, CA 95131 USA | +1-408-544-5633 | chiu.ngo@samsung.com |
| Youngsoo Kim | Samsung Electronics | Mt. 14-1 Nongseo-Ri, Giheung-Eup, Yongin-Si, Gyeonggi-Do, Korea 449-712 | +82-31-280-9614 | kimyoungsoo@samsung.com |
| Chunhui (Allan) Zhu | Samsung Electronics | 75 W. Plumeria Dr. San Jose, CA 95131 USA | +1-408-544-5667 | c.zhu@samsung.com |
| Osama Aboul-Magd | Samsung Electronics |  | 613-599-5078 | Osama53@rogers.com |

Abstract

This document provides the framework from which the draft TGac amendment will be developed. The document provides an outline of each the functional blocks that will be a part of the final amendment. The document is intended to reflect the working consensus of the group on the broad outline for the draft specification. As such it is expected to begin with minimal detail reflecting agreement on specific techniques and highlighting areas on which agreement is still required. It may also begin with an incomplete feature list with additional features added as they are justified. The document will evolve over time until it includes sufficient detail on all the functional blocks and their inter-dependencies so that work can begin on the draft amendment itself.

# 0 Revision notes

R3:	Add header for Revision notes (clause 0)

Add header for MAC (clause 6) and three items to be covered by MAC adhoc.

r4:	Define max number of STAs supported in MU-MIMO transmission, max number of spatial streams in SU transmission, number of spatial streams per user in MU transmission

Added VHT preamble detail. Added A-MPDU extensions.

# 1 Definitions

- Multi-user, multiple input, multiple output (MU-MIMO): A technique where multiple STAs, each with potentially multiple antennas, transmit and/or receive independent data streams simultaneously.

- Downlink MU-MIMO (DL MU-MIMO): MU-MIMO with a single transmitting STA and multiple receiving STAs.

# 2 Abbreviations and acronyms

VHT		Very high throughput

# 3 VHT Physical Layer

This section describes the functional blocks in the physical layer.

## 3.1 Channelization

R3.1.A: The draft specification shall include support for 80 MHz PHY transmission.

For 80 MHz operation, the channelization shown in Figure xxx shall be used.

Include figure or table showing 80 MHz channelization in major regulatory domains

## 3.2 Preamble

### 3.2.1 VHT mixed format PPDU

A VHT mixed format (MF) preamble shall be supported in the draft specification and device support is mandatory. The VHT mixed format preamble shall have the following characteristics:

R3.2.1.A: Robust legacy 11a deferral. The VHT MF preamble shall be designed such that a legacy 11a device will defer for the duration of the transmission to the same degree that it does for an HT MF preamble.

R3.2.1.B: Robust legacy 11n deferral. A VHT MF preamble shall be designed such that a legacy HT STA will defer for the duration of the transmission to the same degree that it does for an HT MF transmission.

R3.2.1.C: The VHT MF preamble shall permit a STA to auto detect 11a, HT MF, HT GF and VHT preambles.

R3.2.1.D: The VHT MF preamble shall include training for

- a wider channel

- 1 to 8 spatial streams (see Section 3.4)

- DL MU-MIMO

R3.2.1.E: Since the HT SIG field cannot be expanded without breaking backward compatibility, the VHT MF preamble shall include a VHT SIG field. The VHT SIG field may include singaling for the following:

a) wider bandwidth

b) enhanced MCS (see Section 3.3)

c) more spatial streams (see Section 3.4)

The VHT MF PPDU format is shown in Figure 1. A single format supports both SU and MU transmissions.

Figure 1 – VHT PPDU format

The L-SIG and VHT-SIG-A fields are modulated as illustrated in Figure 2. The constellation rotation on the second symbol of VHT-SIG-A permits a VHT receiver to distinguish the VHT format PPDU from both legacy and HT format PPDUs.

Figure 2 - L-SIG and VHT-SIG-A modulation in the VHT format PPDU

### 3.2.1.1 VHT-SIG-A and VHT-SIG-B definition

The VHT-SIG-A and VHT-SIG-B fields carry information used to interpret the VHT packet format. In the case of an MU transmission, VHT-SIG-A carries the common bits intended for all clients while VHT-SIG-B carries signalling intended for a specific user. The VHT-SIG-A and VHT-SIG-B field contents for a SU transmission are described in Table 1 although assignment of the fields to the specific SIGs is TBD.

Table 1 - VHT-SIG-A/B fields for VHT SU PPDU

| Field | Number of bits | Description |
| --- | --- | --- |
| Duration | 12 | The number of symbols carrying the DATA portion of the PPDU |
| Bandwidth |  | Indicates the transmission bandwidth |
| Short GI |  | Indicates guard interval duration |
| SU/MU | 1 | Set to ‘SU’ and indicates SU packet. The interpretation of some of the fields in VHT-SIG-A and VHT-SIG-B is dependent on the setting of this field. |
| MCS |  | Modulation and coding scheme used for the DATA portion of the PPDU |
| STBC |  | Indicates use of space-time block coding |
| Coding Type | 1 | Indicates coding type as either BCC or LDPC |
| Smoothing | 1 |  |
| Sounding | 1 |  |
| CRC |  | CRC for prior bits in this SIG field. Present in both VHT-SIG-A and VHT-SIG-B. |
| Tail | 6 | Set to 0. Used to terminate the trellis of the convolutional coder. Present in both VHT-SIG-A and VHT-SIG-B. |

The VHT-SIG-A and VHT-SIG-B field contents for a MU transmission are described in Table 2 although the assignment of the fields to the specific SIGs is TBD.

Table 2 - VHT-SIG-A/B fields for VHT MU PPDU

| Field | Number of bits | Description |
| --- | --- | --- |
| Duration | 12 | The number of symbols carrying the DATA portion of the PPDU |
| Bandwidth |  | Indicates the transmission bandwidth |
| Short GI |  | Indicates guard interval duration |
| SU/MU | 1 | Set to ‘MU’ and indicates MU packet. The interpretation of some of the fields in VHT-SIG-A and VHT-SIG-B is dependent on the setting of this field. |
| Group ID |  | A preassigned identifier by which a user determines which stream(s) to demodulate |
| MCS per user |  | Per-user modulation and coding scheme used for the DATA portion of the PPDU |
| Number of space-time streams per user |  |  |
| STBC |  |  |
| Coding Type | 1 | Indicates coding type as either BCC or LDPC |
| CRC |  | CRC for prior bits in this SIG field. Present in both VHT-SIG-A and VHT-SIG-B. |
| Tail | 6 | Set to 0. Used to terminate the trellis of the convolutional coder. Present in both VHT-SIG-A and VHT-SIG-B. |

### 3.2.2 VHT Sounding PPDU

R3.2.2.A: The sounding PPDU shall be enhanced from the HT sounding PPDU to support up to 8 transmit antennas.

R3.2.2.B: The draft specification shall mandate a single preamble format for sounding PPDUs.

## 3.3 Enhanced MCS

R3.3.A: The draft specification shall include 256 QAM. Device support for 256 QAM is TBD [mandatory or optional].

R3.3.B: The draft specification shall maintain the 11n modulation, interleaving and coding architecture.

R3.3.C: The draft specification shall minimize the number of additional MCS options.

R3.3.D: The draft specification shall include support for a different MCS for each STA in a DL MU-MIMO transmission.

R3.3.E: The draft specification shall include an expanded MCS set for the additional spatial streams supported.

## 3.4 Spatial Multiplexing

R3.4.A: The number of spatial streams in a SU transmission shall be less than or equal to 8.

R3.4.B: The number of per STA spatial streams in a MU transmission shall be less than or equal to 4.

R3.4.C: The total number of streams in a MU transmission shall be less than or equal to 8.

## 3.5 CCA

R3.5.A: CCA shall support detection and deferral on the 20 MHz subchannels that are busy for any possible combination of channel use and signalling bandwidth with a single transmission in an otherwise idle RF bandwidth. This includes

a) a single 20 MHz transmission on any 20 MHz sub-channel

b) a single 40 MHz transmission on either 40 MHz  sub-channel

c) a single 80 MHz transmission

# 4 DL MU-MIMO

R4.A: DL MU-MIMO shall be built on one type of the 11n channel sounding PPDU and transmit beamforming protocol.

R4.B: DL MU-MIMO shall provide MAC protocol extensions to support multiple acknowledgement responses from the individual STAs receiving the MU-MIMO transmission.

R4.C: The DL MU-MIMO MAC protocol extensions shall work with EDCA

R4.D: A DL MU-MIMO transmission shall support a maximum of 4 STAs

# 5 Coexistence

R5.A: Channel access rules shall ensure fair access to the medium for TGac compliant devices and legacy devices operating within a BSS or in seprate overlapping BSSs.

R5.B: The draft specification shall provide a mechanism that ensures that TGac transmissions are protected from legacy channel access for the duration of the transmission.

# 6 MAC

R6.A: Power saving

R6.B: Capability negotiations

## 6.3 Frame formats

### 6.3.1 A-MPDU format

The VHT format PPDU always carries an A-MPDU payload. The A-MPDU format used is an extension of the 802.11n A-MPDU as shown in Figure 3. The extension (shaded in the figure) consists of zero of more A-MPDU null subframes (subframes with zero length MPDU) and a possible MAC Pad of less than 4 octets. The A-MPDU of a VHT PPDU fills the available octets in the payload.

Figure 3 - A-MPDU format for VHT PPDU

The A-MPDU subframe format is extended to include an EOF (end of frame) field in the MPDU Delimiter as shown in Figure 4. The EOF field is set to 1 in all A-MPDU null subframes following the last non-null A-MPDU subframe and set to 0 otherwise.

Figure 4 - A-MPDU subframe format

References:

09/0451r5 TGac Functional Requirements and Evaluation Methodology

10/0070r0 11ac Preamble Discussions

10/0073r0 Group ID Concept for Downlink MU-MIMO Transmission

10/0064r0 VHT Frame Padding