IEEE P802.11
Wireless LANs

| Sponsor Ballot – P802.11p/D9.0 Comment Resolution Submission for Subclause J.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-01-19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Alastair Malarky | Mark IV Industries | 6030 Ambler Drive, Mississaugua, Ontario, Canada L4W 2P1 | 905-624-3020 | amalarky@ivhs.com |
| George Vlantis | STMicroelectronics | 1060 East Brokaw Road, San Jose, CA 95131 | 408-451-8109 (work) 408-893-9357 (cell) | George.Vlantis@st.com |

Abstract

This submission addresses the initial Sponsor Ballot comments within Subclause J.2 of Annex J in the 802.11p amendment Draft 9.0.

Comment resolutions to CIDs 1119, 1120, 1121, and 1156, plus changes relative to Draft 9.0 (if applicable) are provided for consideration by the Comment Resolution Committee.

|  | SB D9.0 Comment Resolution |
| --- | --- |

- COMMENTS:  [From 11-09-1200-00-000p-sb0-tgp-comment-resolution-master.xls]  INSERT Original Comment Here:

| CID | Commenter | Clause | Pg | Ln | Type | Comment | Suggested Remedy | Resolution |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1119 | McNew, Justin | J.2.2 | 33 | 22 | T | MIB attributes required to be set to TRUE may imply functional requirements not intended for OCB (e.g. DFS and TPS) - lines 22 - 25 | Verify the intended requirements and correct accordingly | Principle. It is agreed in principle that to mandate these MIB variables to be TRUE will have consequences which are not intended for all channels in the band. Furthermore, the only MIB attribute that supports the conformance to ASTM2213-03, is the dot11OCBEnabled. The other attributes listed in subclause J.1 apply only to Europe. Therefore: (1) On page 33,line 21: Change “STAs shall have the following elements set to TRUE” to “STAs shall have the capability of setting the following MIB attributes to TRUE” (2) On page 33, lines 22-24: Delete the bullets “dot11MultiDomainCapabilityEnabled”, “dot11RegulatoryClassesRequired”, and “dot11SpectrumManagementRequired” (Editor: Leave dot11OCBEnabled) |
| 1120 | McNew, Justin | J.2.3 | 33 | 33 |  | MIB attributes required to be set to TRUE may imply functional requirements not intended for OCB (e.g. DFS and TPS). Also is the band requirement for the 5.47 - 5.725 band in Europe correct? - lines 33 - 36 | Verify the intended requirements and correct accordingly | Principle. This band is an ISM band and the 802.11-2007 baseline does not but any restrictions on the setting of the MIB attributes “dot11MultiDomainCapabilityEnabled”, “dot11RegulatoryClassesRequired”, and “dot11SpectrumManagementRequired”. Furthermore, the re are no requirements in ETSI EN 302571 bearing on the state of dot11OCBEnabled. Therefore: (1) On page 33,line 28-37: Delete the entirety of subclause J.2.3. (2) On page 33, line 38, Renumber subclause J.2.4 to J.2.3. |
| 1121 | McNew, Justin | J.2.4 | 33 | 43 |  | MIB attributes required to be set to TRUE may imply functional requirements not intended for OCB (e.g. DFS and TPS) - lines 43 - 47 | Same concern as comment for US frequency band | Principle. When dot11OCBEnabled is true, the channel management is performed by higher (than 802.11) layers. While TPC is a required functionality in this band [EN 301 893], the 802.11 mechanisms are not applicable when dot11OCBEnabled is true. Note DFS is not applicable in a licensed band, but Listen-Before-Talk for 5.855 GHz to 5.875 GHz (not DFS) is required. Therefore: (1) On page 33,line 21: Change “STAs shall have the following elements set to TRUE” to “STAs shall have the capability of setting the following MIB attributes to TRUE” (2) On page 33, lines 22-24: Delete the bullets “dot11MultiDomainCapabilityEnabled”, “dot11RegulatoryClassesRequired”, and “dot11SpectrumManagementRequired” (Editor: Leave dot11OCBEnabled) |
| 1156 | Roy, Richard | J.2.2 | 33 | 25 | T | The text unnecessarily constrains STAs operating in 5.85 band to having dot11OCBEnabled set to TRUE. While this is certainly going to be the generallyaccepted mode of operation, there is no reason to require it in this standard. | Remove the restriction that dot11OCBEnabled must be TRUE in all three places in J.2 | Principle. Superceded by events. |

2.   Recommended Resolution of the Comments:

See the right column of the table above for the resolutions of the individual comments.

3.   Discussion

Not Applicable.