IEEE P802.11
Wireless LANs

| Revised Cellular Network Information |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-01-20 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| George Bumiller | Research in Motion | 39 Lakeview Terrace Ramsey, NJ 07446 USA | +1 201 327 1136 | gbumiller@rim.com |

Abstract

This submission addresses the following comment from the P802.11u Initial Sponsor Ballot: 7415000023

The submission changes the 3GPP Cellular Network Information element, to be generic for different external organisational network types.

This document is based on IEEE 802.11u-d8.01

Modify the following clause

7.3.4 Native Query Protocol elements

Native Query Protocol elements are defined to have a common format consisting of a 2-octet Info ID field, a 2-octet length field, and a variable-length element-specific information field. Each element is assigned a unique Info ID as defined in this standard. The Native Query Protocol query element format is shown in Figure 7-95o125. See Annex W.1 for informative text on NQP usage.

|  | Info ID | Length | Information |
| --- | --- | --- | --- |
| Octets: | 2 | 2 | variable |
| Figure 7-95o125—Native Query Protocol query element format |  |  |  |

Each Native Query Protocol element in 7.3.4 is assigned a unique 2-octet Info ID. The set of valid Info IDs are defined in Table 7-43bg. The 2-octet Info ID is encoded following the conventions given in 7.1.1.

The Length field specifies the number of octets in the Information field and is encoded following the conventions given in 7.1.1.

The Native Query Protocol elements that may be configured are shown in Table 7-43bg.

| Table 7-43bg—Native Query Protocol info ID definitions |  |  |
| --- | --- | --- |
| Info Name | Info ID | Native Info Element (clause) |
| Reserved | 0-255 | n/a |
| Interworking Capability List | 256 | 7.3.4.1 |
| Venue Name information | 257 | 7.3.4.2 |
| Emergency Call Number information | 258 | 7.3.4.3 |
| Network Authentication Type information | 259 | 7.3.4.4 |
| Roaming Consortium List | 260 | 7.3.4.5 |
| IP Address Type Availability information | 261 | 7.3.4.7 |
| NAI Realm List | 262 | 7.3.4.8 |
| Cellular Network information | 263 | 7.3.4.9 |
| AP Geo Location | 264 | 7.3.4.10 |
| AP Civic Location | 265 | 7.3.4.11 |
| Domain Name List | 266 | 7.3.4.12 |
| Emergency Alert URI | 267 | 7.3.4.13 |
| Reserved | 268– 56796 | n/a |
| Native Query Protocol vendor-specific list | 56797 | 7.3.4.6 |
| Reserved | 56798 – 65535 | n/a |

Modify the following clause

7.3.4.9 Cellular Network information

The Cellular Network information contains cellular information such as network advertisement information e.g. network codes and country codes to assist a dual mode (e.g. IEEE 802.11 and Cellular) non-AP STA in selecting an AP to access cellular networks. The format of the Cellular Network information is shown in Figure 7-95o143.

|  | Info ID | Length | Organisation ID | Payload |
| --- | --- | --- | --- | --- |
| Octets: | 2 | 2 | 1 | variable |
| Figure 7-95o143—Cellular Network information format |  |  |  |  |

The Info ID field is equal to the value in Table 7-43bg corresponding to the Cellular Network information.

The Length field is a 2-octet field and is equal to the length of the Payload field.

The Organisation ID field is a 1-octet field which identifies the specific organisation representing the cellular network. Each organisation is assigned a unique Organisation  ID as shown in Table xx.

| Table xx—Organisation ID codes and descriptions |  |
| --- | --- |
| Organisation ID | Organisation ID Description |
| 0 | Unspecified |
| 1 | 3rd Generation Partnership Program (3GPP) |
| 2 | 3rd Generation Partnership Program 2 (3GPP2) |
| 3 | Worldwide Interoperability for Microwave Access (WiMAX) |
| 4 | Vendor Specific |
| 5 – 255 | Reserved |

The Payload field is a generic container whose content is defined by each organisation.

Note: For 3GPP the payload is defined in Annex A of 3GPP TS 24.234 v8.1.0.