IEEE P802.11
Wireless LANs

| Suggested improvements for Clauses 9, 11, and A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-1-20 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| John Kenney | VSC3 and Toyota InfoTechnology Center |  | 574-272-1403 | johnkenney@alumni.nd.edu |

Abstract

This document suggests three improvements to changes TGp has made during this meeting.  The improvements deal with clauses 9.6.0a, 11.20, and A.4.4.2

# Clause 9.6.0a

TGp inserted the following sentence in 9.6.0a to resolve CID 1005:

Only the data transfer rates of the mandatory rate set of the PHY are guaranteed to be supported by any STA when transmitting management and data frames outside the context of a BSS.

The substance of this is good, but it would be useful to reword the sentence to refer to dot11OCBEnabled rather than to “transmitting … frames outside the context of a BSS”.  This clarification would be particularly helpful in avoiding confusion with regard to management frames of type Action, which could be interpreted as being transmitted outside of a BSS even by a STA that has dot11OCBEnabled equal to false.

Two options for modified language are presented below for the task group’s consideration.

Option 1:

Only the data transfer rates of the mandatory rate set of the PHY are guaranteed to be supported when a STA for which dot11OCBEnabled is true transmits a management or data frame.

Option 2:

When a STA for which dot11OCBEnabled is true transmits a management or data frame it is not required to support any data transfer rate that is not in the mandatory rate set of the PHY.

Motion: In clause 9.0.6a of the P802.11p draft amendment, change the first sentence to:

Only the data transfer rates of the mandatory rate set of the PHY are guaranteed to be supported when a STA for which dot11OCBEnabled is true transmits a management or data frame.

Motion by: ___John Kenney________________Date: __Jan. 20, 2010__  Second:  ____Justin McNew_____________

| Approve:9 | Disapprove:0 | Abstain:1 |
| --- | --- | --- |

# Clause 11.20

TGp inserted the following sentence in 11.20 to resolve CID 1064:

STAs shall use information from the CF Parameter Set element of all received Beacon frames, without regard for the BSSID, to update their NAV as specified in 9.3.2.2.

This implies that a STA must have the capability to receive Beacon frames.  However, if a STA only operates in a band for which dot11OCBEnabled is required to be true, it will never see a Beacon frame.  Such a STA therefore does not require the capability to receive Beacon frames.  The PICS Status for FR7 (capability to receive Beacon frames) is “not CF17:M”, which means the capability is mandatory for a STA not operating in the 5.9 GHz band.  It would be useful to qualify the sentence above to reflect that exception.

Suggested change:

When operating in a band for which dot11OCBEnabled is permitted to be false, a STA for which dot11OCBEnabled is true shall use information from the CF Parameter Set element of all received Beacon frames, without regard for the BSSID, to update its NAV as specified in 9.3.2.2.

Motion: In clause 11.20 of the P802.11p draft amendment, change the sentence that begins

“STAs shall use information …”

to:

When operating in a band for which dot11OCBEnabled is permitted to be false, a STA for which dot11OCBEnabled is true shall use information from the CF Parameter Set element of all received Beacon frames, without regard for the BSSID, to update its NAV as specified in 9.3.2.2.

- Motion by: ___John Kenney________________Date: ____  Jan. 20, 2010 _______

- Second:  ______         Carl Kain  ________________

| Approve:7 | Disapprove:0 | Abstain:1 |
| --- | --- | --- |

# Clause A.4.4.2

PICs entries FT7.1 and FR7.1 cover the capability to transmit and receive Timing Advertisement frames, respectively.  They are subordinate to FT7 and FR7, which deal with transmission and reception of Beacon frames.  They should not be subordinate (this is a legacy of an earlier version of the amendment when a variation on the beacon was used instead of the Timing Advertisement frame.

Motion: In clause A.4.4.2 change FT7.1 to FTx, where x is the next number available in the FT list.  Change FR7.1 to FRy, where y is the next number available in the FR list.

- Motion by: ___John Kenney________________Date: ____Jan. 20, 2010   _______

- Second:  ______    George Vlantis       ________________

| Approve:8 | Disapprove:0 | Abstain:0 |
| --- | --- | --- |