IEEE P802.11
Wireless LANs

| MCCA text for LB 159 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-01-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Dee Denteneer | Philips | HTC 37; Eindhoven; The Netherlands | +31 40 27 49743 | Dee.denteneer@philips.com |

Abstract

This document provides text changes that intend to resolve the MCCA related CIDs for LB 159 identified in the accompanying spreadsheet 10/158r0

- Definitions specific to IEEE 802.11

Insert the following new definitions:

- The subclause numbering of definition is of the form “3A.s<x>” where <x> is an increasing number. These numbers need to be assigned before the sponsor ballot.

- DTIM interval: The value, in Time Units, that is equivalent to the product of the Beacon Interval and the DTIM Period subfields specified within the TIM element in Beacon frames.

- Mesh Awake Window: A period of time where the mesh STA operates in awake state after its Beacon or Probe Response frame transmission that contained the Mesh Awake Window element.

- MCF coordinated channel access (MCCA): A coordination function for the MBSS.

- MCF coordinated channel access opportunity (MCCAOP): A period of time that is set up between a transmitter and a receiver.

- mesh coordination function (MCF): A coordination function that combines aspects of the contention-based and reduced contention access methods. The MCF includes the functionality provided by both enhanced distributed channel access (EDCA) and MCF coordinated channel access (MCCA).

- Abbreviations and acronyms

Insert the following new acronym in alphabetical order:

AODV	Ad hoc On-demand Distance Vector

AMPE	Authenticated Mesh Peering Exchange

FSM	Finite State Machine

HWMP	Hybrid Wireless Mesh Protocol

MBCA	Mesh Beacon Collision Avoidance

MBSS	Mesh Basic Service Set

MCCA	MCF Coordinated Channel Access

MCCAOP	MCF Coordinated Channel Access Opportunity

MCF	Mesh Coordination Function

MICE	MIC element

MPM	Mesh Peering Management

PANN	Portal Announcement

PERR	Path Error

PREP	Path Reply

PREQ	Path Request

PXU	Proxy Update

PXUC	Proxy Update Confirm

RANN	Route Announcement

RSPI 	Receiver Service Period Initiation

SAE	Simultaneous Authentication of Equals

TTL	Time to Live

- Frame formats

- MAC frame formats

- MCCAOP Reservation field

The MCCAOP Reservation field is a 4 octet field specifying a schedule for one or more frame transmissions (called MCCAOPs) relative to the DTIM interval of the reporting mesh STA. The MCCAOP Reservation field consists of 3 subfields and its format is illustrated in MCCAOP Reservation field.

| MCCAOP Duration | MCCAOP Periodicity | MCCAOP Offset |
| --- | --- | --- |
| Octets: 1 | 1 | 2 |
| - MCCAOP Reservation field |  |  |

The MCCAOP Duration field is one octet in length and contains an unsigned integer. It specifies the duration the MCCAOP in multiples of 32 µs.

The MCCAOP Periodicity field is one octet in length and contains an unsigned integer. A non-zero Periodicity field specifies a series of MCCAOPs. These MCCAOPs are repeated in all DTIM intervals until the MCCAOP Reservation is torn down. The starting points of the MCCAOPs of this series are separated by a time interval with a duration equal to the duration of the DTIM interval divided by the MCCAOP Periodicity. A value of zero indicates a non-repeated MCCAOP in the DTIM interval following the setup.

The MCCAOP Offset field is two octets in length and contains an unsigned integer. It specifies the beginning of the first MCCAOP of the one or more MCCAOPs specified in the reservation relative to the beginning of the DTIM interval. The value is specified in multiples of 32 µs.

An example of periodicity, duration, and offset values for an MCCAOP Reservation field is shown in Example of MCCAOP Reservation with Periodicity equal to 4. In this particular example, the MCCAOP Periodicity field equals four, so that there are four MCCAOPs within the DTIM interval. As further illustrated in the figure, the value of the MCCAOP offset field indicates the beginning of first MCCAOP relative to the beginning of the DTIM interval..

- Example of MCCAOP Reservation with Periodicity equal to 4

- Information elements

Insert the following rows (ignoring the header row and footer note) in Element IDsinput.doc—Element IDs in the correct position to preserve ordering by the “Element ID” column and update the “Reserved” range of codes appropriately.

| - Element IDsinput.doc |  |  |  |
| --- | --- | --- | --- |
| Information element | Element ID | Total length of element in octets including the Type and Length octets | Extensible |
| Mesh Configuration Error: Reference source not found |  | 10 |  |
| Mesh ID Error: Reference source not found |  | 2 to 34 |  |
| Mesh Link Metric Report Error: Reference source not found |  | 3 to 257 |  |
| Congestion Notification Error: Reference source not found |  | 10 |  |
| Mesh Peering Management Error: Reference source not found |  | 8, 10, or 12 |  |
| Mesh Channel Switch Announcement Error: Reference source not found |  | 9 |  |
| Mesh Awake Window Error: Reference source not found |  | 4 |  |
| Beacon Timing Error: Reference source not found |  | 7 to 257 |  |
| MCCAOP Setup Request Error: Reference source not found |  | 7 |  |
| MCCAOP Setup Reply Error: Reference source not found |  | 4 or 8 |  |
| MCCAOP Advertisements Error: Reference source not found |  | 6 to 257 |  |
| MCCAOP Reservation Teardown Error: Reference source not found |  | 3 or 9 |  |
| Portal Announcement (PANN) Error: Reference source not found |  | 17 |  |
| Root Announcement (RANN) Error: Reference source not found |  | 23 |  |
| Path Request (PREQ) Error: Reference source not found |  | 39 to 257 |  |
| Path Reply (PREP) Error: Reference source not found |  | 33 or 39 |  |
| Path Error (PERR) Error: Reference source not found |  | 14 to 254 |  |
| Proxy Update (PXU) Error: Reference source not found |  | 17 to 257 |  |
| Proxy Update Confirmation (PXUC) Error: Reference source not found |  | 10 |  |
| Authenticated Mesh Peering Exchange Error: Reference source not found |  | 86 to 257 |  |
| MIC Error: Reference source not found |  | 18 |  |

- Element ID for these elements needs to be allocated before sponsor ballot by ANA.

- Mesh Capability

The Mesh Capability field comprises a set of values indicating whether a mesh STA is a possible candidate for mesh peering establishment. The details of the Mesh Capability field are shown in .

| B0 | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 B15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Accepting Mesh Peerings | MCCA Supported | MCCA Enabled | Forwarding | Beacon Timing Report Enabled | TBTT Adjustment Enabled | TBTT Adjusting | TSF Adjustment Enabled | Mesh Power Save Level | Reserved |
| Bits: 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 7 |
| - Mesh Capability field |  |  |  |  |  |  |  |  |  |

The Accepting Mesh Peerings subfield is set to 1 if the mesh STA is willing to establish mesh peerings with other mesh STAs and set to 0 otherwise.

The MCCA Supported subfield is set to 1 if the mesh STA implements the MCCA CF and set to 0 otherwise.

The MCCA Enabled subfield is set to 1 if the mesh STA is using the MCCA CF and set to 0 otherwise.

The Forwarding subfield is set to the value of the MIB variable dot11MeshForwarding. (#305)

The Beacon Timing Report Enabled subfield is set to 1 if MBCA beacon timing report function (see Error: Reference source not found) is enabled on the mesh STA and is set to 0 otherwise.

The TBTT Adjustment Enabled subfield is set to 1 if the TBTT adjusting function is enabled on the mesh STA, and is set to 0 otherwise. (See Error: Reference source not found)

The TBTT Adjusting subfield is set to 1 while the TBTT adjusting procedure is on going to notify that the mesh STA’s TBTT is shifting intentionally, and is set to 0 otherwise. (See Error: Reference source not found)

The TSF Adjustment Enabled subfield is set to 1 if the TSF drift compensation function (see Error: Reference source not found) is enabled on the mesh STA, and is set to 0 otherwise.

When the Power Management subfield is 1, the value 0 in the Mesh Power Save Level field indicates that the mesh STA is operating in light sleep mode, and the value 1 in the Mesh Power Save Level field indicates that mesh STA is operating in deep sleep mode. When the Power Management subfield is 0, the Mesh Power Save Level field is reserved.

.

- MCCAOP Setup Request element

The MCCAOP Setup Request element is used to setup one or more MCCAOPs. This element is transmitted in individually addressed MCCA Setup Request frames. The mesh STA transmitting the MCCA Setup Request element is the MCCAOP owner of the MCCAOPs defined in this setup. The receivers of the MCCAOP Setup Request are the MCCAOP responders. The format of the element is as shown in MCCAOP Setup Request element.

| Element ID | Length | MCCAOP Reservation ID | MCCAOP Reservation |
| --- | --- | --- | --- |
| Octets: 1 | 1 | 1 | 4 |
| - MCCAOP Setup Request element |  |  |  |

The Element ID is set to the value given in Element IDsinput.doc for this element.

The Length field is set to 5 octets.

The MCCAOP Reservation ID field is an eight bit unsigned integer that represents the ID for the MCCAOP reservation. It is determined by the MCCAOP owner. When used in combination with the MAC address of the MCCAOP owner, the MCCAOP Reservation ID uniquely identifies the MCCAOP Reservation.

If this MCCAOP Setup Request is for an individually addressed transmission, the MCCAOP Reservation ID is between 0 and 127 and the Setup Request is transmitted in an individually addressed frame to the intended responder. If this MCCAOP Setup Request is for a group addressed transmission, the MCCAOP Reservation ID is between 128 and 254 and the Setup Request is transmitted in individually addressed frames to each of the intended responders. The value 255 is reserved and is not used to identify a single MCCAOP reservation.

The MCCAOP Reservation field specifies the starts and duration of a series of MCCAOPs, as described in MCCAOP Reservation field.

- MCCAOP Setup Reply element

The MCCAOP Setup Reply element is used to reply to an MCCAOP Setup Request. This element is transmitted in individually addressed MCCA Setup Reply frames. The mesh STA transmitting the MCCA Setup Reply element is the MCCAOP responder of the series of MCCAOPs defined in this reply. The receiver of the MCCAOP Setup Reply is the MCCAOP owner. The format of the element is as shown in Error: Reference source not found.

| Element ID | Length | MCCAOP Reservation ID | MCCA Reply Code | MCCAOP Reservation |
| --- | --- | --- | --- | --- |
| Octets: 1 | 1 | 1 | 1 | 4 |
| - MCCAOP Setup Reply element |  |  |  |  |

The Element ID is set to the value given in Element IDsinput.doc for this element.

The Length field is set to 2 or 6 octets.

The MCCAOP Reservation ID field is an eight bit unsigned integer that represents the ID for the requested series of MCCAOPs. It is determined by the MCCAOP owner and copied from the MCCAOP Setup Request element. When used in combination with the MAC address of the MCCAOP owner, the MCCAOP Reservation ID uniquely identifies one or more MCCAOPs. If this MCCAOP Setup Request is for an individually addressed transmission, the MCCAOP Reservation ID is between 0 and 127. If this MCCAOP Setup Request is for a group addressed transmission, the MCCAOP Reservation ID is between 128 to 254. The value 255 is reserved and is not used to identify a single MCCAOP reservation.

The MCCA Reply Code field is a one octet field that contains the reply code used in an MCCAOP Setup Reply element. The reply codes are defined in MCCA Reply codesinput.doc

| - MCCA Reply codesinput.doc |  |
| --- | --- |
| MCCA Reply code | Meaning |
| 0 | Accept |
| 1 | Reject: MCCAOP Reservation conflict |
| 2 | Reject: MAF limit exceeded |
| Other | Reserved |

The MCCAOP Reservation field includes an alternative to the MCCAOP Reservation specified in the MCCAOP Setup Request message. Its format is described in MCCAOP Reservation field. This field is an optional field and it is optionally included when the MCCA Reply Code is 1 or 2.

- MCCAOP Advertisements element

The MCCAOP Advertisements element is used by a mesh STA to advertise its MCCA state to its neighbors. This element can be carried in selected Beacon frames with a chosen frequency. This element is also transmitted in an MCCAOP Advertisements frame. The format of the element is as shown in MCCAOP Advertisements element.

| Element ID | Length | MCCA Information | TX-RX Times Report | Broadcast Times Report | Interfering Times Report |
| --- | --- | --- | --- | --- | --- |
| Octets: 1 | 1 | 4 | variable | variable | variable |
| - MCCAOP Advertisements element |  |  |  |  |  |

The Element ID is set to the value given in Element IDsinput.doc for this element. The Length is set to 4 to 255 octets.

The MCCA Information field is a field of 4 octets. It is described in MCCA Information field.

The TX-RX Times Report field is a variable length field that contains an MCCAOP Reservation Report field as described in Error: Reference source not found. This field is only present when the TX-RX Report Present subfield of the MCCAOP Information field is equal to 1. Through the TX-RX Times Report field, a mesh STA advertises the MCCAOPs in which it is involved as an MCCAOP owner or an MCCAOP responder and in which individually addressed frames are transmitted.

The Broadcast Times Report field is a variable length field that contains an MCCAOP Reservation Report field as described in Error: Reference source not found. This field is only present when the Broadcast Times Report Present subfield of the MCCAOP Information field is equal to 1. Through the Broadcast Times Report field, a mesh STA advertises the MCCAOPs in which it is involved as an MCCAOP owner or an MCCAOP responder and in which group addressed frames are transmitted. These reports optionally include known and otherwise un-advertised transmission and reception times of the mesh STA besides MCCAOPs, for example, its HCCA times.

The Interfering Times Report field is a variable length field that contains an MCCAOP Reservation Report field as described in Error: Reference source not found. This field is only present when the Interfering Times Report Present subfield of the MCCAOP Information field is equal to 1. Through the Interfering Times Report field, a mesh STA reports the TX-RX and Broadcast Times of its neighbors, excluding its own TX-RX and Broadcast times.

Note to the editor: swap Clauses 7.3.2.105.2 and 7.3.2.105.3 and rename 7.3.2.105.3 to 7.3.2.105.1.

- MCCAOP Reservation Report field

The MCCAOP Reservation Report field is a variable length field that is used to report a number of MCCAOP Reservations. The field consists of a variable number of subfields and its format is illustrated in MCCAOP Reservation Report field

| Number of Reported MCCAOP Reservations | MCCAOP Reservation 1 | ... | MCCAOP Reservation n |
| --- | --- | --- | --- |
| Octets: 1 | 4 |  | 4 |
| - MCCAOP Reservation Report field |  |  |  |

The Number of Reported MCCAOP Reservations is a field of one octet with an unsigned integer that specifies the number, n, of MCCAOP Reservations reported in this field.

The MCCAOP Reservation 1 through MCCAOP Reservation n fields specify the MCCAOP Reservations reported. Each field is of length 4 Octets and contains an MCCAOP Reservation field, whose format is shown in MCCAOP Reservation field in MCCAOP Reservation field.

- MCCA Information field

The MCCA Information field is a field of length 4 octets that is used to provide information on the MCCAOP Reservations. The field consists of seven subfields and its format is illustrated in .

| B0 B7 | B8 B15 | B16 B23 | B24 | B25 | B26 | B27 | B28 B31 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MCCA Access Fraction | MCCA Access Fraction Limit | Max Track | TX-RX Report Present | Broadcast Report Present | Interfering Report Present | Partial Report | Reserved |
| Bits: 8 | 8 | 8 | 1 | 1 | 1 | 1 | 4 |
| - MCCA Information field |  |  |  |  |  |  |  |

The MCCA Access Fraction is an eight bit unsigned integer. The MCCA Access Fraction field is set to the current value of MCCA Access Fraction at the mesh STA rounded down (floor) to the nearest multiple of (1/255).

The MCCA Access Fraction Limit field is an eight bit unsigned integer. The MCCA Access Fraction Limit field is set to the maximum MCCA access fraction allowed at the mesh STA rounded down (floor) to the nearest multiple of (1/255).

The MaxTrack field is an eight bit unsigned integer that expresses the maximum number of MCCAOP Reservations that a mesh STA can track.

The TX-RX Report Present is a one bit field. It is set to 1 if a TX-RX Report field is present in the MCCAOP Advertisement and set to 0 if no TX-RX Report field is present.

The Broadcast Report Present is a one bit field. It is set to 1 if a Broadcast TX-RX Report field is present in the MCCAOP Advertisement and set to 0 if no Broadcast TX-RX Report field is present.

The Interfering Report Present is a one bit field. It is set to 1 if an Interfering Times Report field is present in the MCCAOP Advertisement and set to 0 if no Interfering Times Report field is present.

The Partial Report field is a one bit field. It is set to 0 if the Advertisement element includes all known reservations and it is set to 1 if the Advertisement element does not include all known reservations.

- MCCAOP Reservation Teardown element

The MCCAOP Reservation Teardown element is used to announce the teardown of an MCCAOP Reservation. The MCCAOP Reservation Teardown element is transmitted in Beacon frames or group addressed MCCAOP Reservation Teardown frames. Its format is shown in MCCAOP Reservation Teardown element.

| Element ID | Length | MCCAOP Reservation ID | MCCAOP Owner |
| --- | --- | --- | --- |
| Octets: 1 | 1 | 1 | 6 |
| - MCCAOP Reservation Teardown element |  |  |  |

The Element ID is set to the value given in Element IDsinput.doc for this element.

The Length is variable and set to 1 or 7 octets.

An MCCAOP Reservation Teardown element is transmitted by either the MCCAOP owner or the MCCAOP responder of a MCCAOP Reservation to tear down the MCCAOP Reservation.

The MCCAOP Reservation ID field is an eight bit unsigned integer that represents the ID for the MCCAOP reservation.

The MCCAOP Owner field is an optional field. It is six octets long and indicates the 48-bit MAC address of the MCCAOP owner. This field is only included if the element is transmitted by the MCCAOP responder..

- Mesh Resource Coordination Action frame details

- General

Action frame formats for management of Mesh Resource Coordination are defined in this subclause.

An Action field, in the octet field immediately after the Category field, differentiates the frame format. The Action field values associated with each frame format are defined in Mesh Resource Coordination Action field valuesinput.doc.

| - Mesh Resource Coordination Action field valuesinput.doc |  |
| --- | --- |
| Action field value | Description |
|  | Congestion Control Notification |
|  | MCCA Setup Request |
|  | MCCA Setup Reply |
|  | MCCAOP Advertisement Request |
|  | MCCAOP Advertisements |
|  | MCCAOP Reservation Teardown |
|  | TBTT Adjustment Request |
|  | Mesh Channel Switch Announcement |
| 8-255 | Reserved |

- Congestion Control Notification frame format

The Congestion Control Notification frame uses the Action frame body format and is sent by a mesh STA to its neighbor peer mesh STA(s) to indicate its congestion status. The body is shown in Congestion Control Notification frame bodyinput.doc.

| - Congestion Control Notification frame bodyinput.doc |  |  |
| --- | --- | --- |
| Order | Information | Notes |
|  | Category |  |
|  | Action |  |
|  | Congestion Notification element |  |
| Last | Vendor Specific | One or more vendor-specific elements are optionally present. These elements follow all other elements. |

The Category field is set to the value in Error: Reference source not found for category Mesh Resource Coordination.

The Action field is set to the value in Mesh Resource Coordination Action field valuesinput.doc for this Action frame type.

The Congestion Notification element field is set as described in Error: Reference source not found.

- MCCA Setup Request frame format

The MCCA Setup Request frame is used to set up an MCCAOP Reservation. It is transmitted by a mesh STA with dot11MCCAActivated equal to TRUE to a neighbor peer mesh STA with dot11MCCAActivated equal to TRUE. This frame is transmitted using individual addresses. The frame body of an MCCA Setup Request frame contains the information shown in MCCA Setup Request frame bodyinput.doc.

| - MCCA Setup Request frame bodyinput.doc |  |  |
| --- | --- | --- |
| Order | Information | Notes |
|  | Category |  |
|  | Action |  |
|  | MCCA Setup Request element |  |
| Last | Vendor Specific | One or more vendor-specific elements are optionally present. These elements follow all other elements. |

The Category field is set to the value in Error: Reference source not found for category Mesh Resource Coordination.

The Action field is set to the value in Mesh Resource Coordination Action field valuesinput.doc for this Action frame type.

The MCCA Setup Request element is described in Error: Reference source not found.

- MCCA Setup Reply frame format

The MCCA Setup Reply frame is used to reply to an MCCAOP Setup Request. It is transmitted by a mesh STA with dot11MCCAActivated equal to TRUE to a neighbor peer mesh STA with dot11MCCAActivated equal to TRUE. This frame is transmitted using individual addresses. The frame body of an MCCA Setup Reply frame contains the information shown in MCCA Setup Reply frame bodyinput.doc.

| - MCCA Setup Reply frame bodyinput.doc |  |  |
| --- | --- | --- |
| Order | Information | Notes |
|  | Category |  |
|  | Action |  |
|  | MCCA Setup Reply element |  |
| Last | Vendor Specific | One or more vendor-specific elements are optionally present. These elements follow all other elements. |

The Category field is set to the value in Error: Reference source not found for category Mesh Resource Coordination.

The Action field is set to the value in Mesh Resource Coordination Action field valuesinput.doc for this Action frame type.

The MCCA Setup Reply element is described in Error: Reference source not found.

- MCCAOP Advertisement Request frame format

The MCCAOP Advertisement Request frame is transmitted by a mesh STA with dot11MCCAActivated equal to TRUE in order to request MCCA advertisements from neighbor peer mesh STAs. The frame body of an MCCAOP Advertisement Request frame is shown in MCCAOP Advertisement Request frame bodyinput.doc.

| - MCCAOP Advertisement Request frame bodyinput.doc |  |  |
| --- | --- | --- |
| Order | Information | Notes |
|  | Category |  |
|  | Action |  |
| Last | Vendor Specific | One or more vendor-specific elements are optionally present. These elements follow all other elements. |

The Category field is set to the value in Error: Reference source not found for category Mesh Resource Coordination.

The Action field is set to the value in Mesh Resource Coordination Action field valuesinput.doc for this Action frame type.

- MCCAOP Advertisements frame format

The MCCAOP Advertisements frame is transmitted by a mesh STA with dot11MCCAActivated equal to TRUE to one or more neighbor peer mesh STAs with dot11MCCAActivated equal to TRUE. This frame is transmitted using group addresses or individual addresses. The frame body of an MCCAOP Advertisements frame contains the information shown in MCCAOP Advertisements frame bodyinput.doc.

| - MCCAOP Advertisements frame bodyinput.doc |  |  |
| --- | --- | --- |
| Order | Information | Notes |
|  | Category |  |
|  | Action |  |
|  | MCCA Advertisements element |  |
| Last | Vendor Specific | One or more vendor-specific elements are optionally present. These elements follow all other elements. |

The Category field is set to the value in Error: Reference source not found for category Mesh Resource Coordination.

The Action field is set to the value in Mesh Resource Coordination Action field valuesinput.doc for this Action frame type.

The MCCAOP Advertisements element is described in Error: Reference source not found.

- MCCAOP Reservation Teardown frame format

The MCCAOP Reservation Teardown frame is transmitted by a mesh STA with dot11MCCAActivated equal to TRUE to one or more neighbor peer mesh STAs with dot11MCCAActivated equal to TRUE. This frame is transmitted using group addresses or individual addresses. The frame body of an MCCAOP Reservation Teardown frame contains the information shown in MCCAOP Reservation Teardown frame bodyinput.doc.

| - MCCAOP Reservation Teardown frame bodyinput.doc |  |  |
| --- | --- | --- |
| Order | Information | Notes |
|  | Category |  |
|  | Action |  |
|  | MCCA Reservation Teardown element |  |
| Last | Vendor Specific | One or more vendor-specific elements are optionally present. These elements follow all other elements. |

The Category field is set to the value in Error: Reference source not found for category Mesh Resource Coordination.

The Action field is set to the value in Mesh Resource Coordination Action field valuesinput.doc for this Action frame type.

The MCCAOP Reservation Teardown element is described in Error: Reference source not found.

- MAC sublayer functional description

Change the first paragraph of Clause 9 as following:

The MAC functional description is presented in this clause. The architecture of the MAC sublayer, including the distributed coordination function (DCF), the point coordination function (PCF), the hybrid coordination function (HCF), the Mesh Coordination Function (MCF), and their coexistence in an IEEE 802.11 LAN are introduced in 9.1. These functions are expanded on in 9.2 (DCF), 9.3 (PCF), and 9.9 (HCF), and 9.9a (MCF). Fragmentation and defragmentation are defined in 9.4 and 9.5. Multirate support is addressed in 9.6. A number of additional restrictions to limit the cases in which MSDUs are reordered or discarded are described in 9.7. Operation across regulatory domains is defined in 9.8. The Block Ack mechanism is described in 9.10. The No Ack mechanism is described in 9.11. The protection mechanism is described in 9.13.

- MAC architecture

Insert the following text to the end of 9.1:

Due to the distributed nature of the MBSS, only the MCF is present in a mesh STA.

Replace MAC architecture with the following figure:

- The changes comprise the addition of MCF component over DCF;

- MAC architecture

- Hybrid coordination function (HCF)

- HCF contention-based channel access (EDCA)

Change the second paragraph in 9.1.3.1 as shown:

- The following text is based on the amendment by TGp.

For each AC, an enhanced variant of the DCF, called an enhanced distributed channel access function (EDCAF), contends for TXOPs using a set of EDCA parameters. When communicating data frames outside the context of a BSS (dot11OCBEnabled is true), the EDCA parameters are the corresponding default values or are as set by the SME in the MIB attribute table dot11EDCATable (except for TXOP limit values, which shall be set to zero for each AC). When communicating within a BSS, the EDCA parameters used are from the EDCA Parameter Set element, or from the default values for the parameters when no EDCA Parameter Set element is received from the AP of the BSS with which the STA is associated or when the STA is a mesh STA. The parameters used by the EDCAF to control its operation are defined by MIB attribute table dot11QAPEDCATable at the AP and by MIB attribute table dot11EDCATable at the non-AP STA.

Change the second to last paragraph in 9.1.3.1 as shown:

The management frames shall be sent using the access category AC_VO without being restricted by admission control procedures. A QoS STA shall also send management frames using the access category AC_VO before associating with any BSS or before establishing mesh peerings with MBSS, even if there is no QoS facility available in that BSS. BlockAckReq and BlockAck control frames shall be sent using the same QoS parameters as the corresponding QoS data frames. PS-Poll control frames shall be sent using the access category AC_BE to reduce the likelihood of collision following a Beacon frame. For the purpose of determining the proper AC for an RTS frame, the RTS frame shall inherit the UP of the data or management frame(s) that are included in the frame exchange sequence where the RTS is the first frame.

Insert the following new subclause immediately after the 9.1.3:

- Mesh coordination function (MCF)

The mesh facility includes an additional coordination function called MCF that is usable only in an MBSS. Mesh STAs shall implement the MCF only. MCF has both a contention-based channel access and contention free channel access mechanism. The contention based mechanism is EDCA and the contention free mechanism is called the MCF Coordinated Channel Access (MCCA) mechanism that optimizes frame exchanges in the MBSS. MCF uses the default values for PTKSA, GTKSA and STKSA Replay Counters. The operation rules of the EDCA are defined in 9.9.1. The operation rules of the MCCA are defined in MCF coordinated channel access (MCCA).

- MCF

- General

Under MCF, the basic unit of allocation of the right to transmit onto the WM is the TXOP. Each TXOP is defined by a starting time and a defined maximum length.

Under MCF, there are two types of TXOPs: EDCA TXOPs and MCCA TXOPs. The EDCA TXOP is obtained by a mesh STA winning an instance of EDCA contention (see Error: Reference source not found). The MCCA TXOP is obtained by a mesh STA gaining control of the WM during an MCCAOP. The MCCAOP is an advance reservation of a time interval for its transmission by means of an MCCA Opportunity Reservation (see MCF coordinated channel access (MCCA)). Neither EDCA TXOPs nor MCCA TXOPs shall exceed dot11MaxDwellTime (if using an FH PHY).

- MCF contention-based channel access

In an MBSS, EDCA implements the contention-based channel access. Error: Reference source not found defines EDCA.

- MCF coordinated channel access (MCCA)

- General

MCF Coordinated Channel Access (MCCA) is an optional access method that allows mesh STAs that implement MCCA to access the WM at selected times with lower contention than would otherwise be possible. This standard does not require all mesh STAs to use MCCA. MCCA can be used by a subset of mesh STAs in an MBSS. However, MCCA reservations can only be set up among mesh STAs that operate on the same channel and that are in MCCA enabled mode. The performance of MCCA may be impacted by devices that do not respect MCCAOP reservations.

MCCA enabled mesh STAs use management frames to make an advance reservation for transmissions. They advertise these advance reservations to their neighbors via MCCA Advertisements. The MCCA enabled neighbor mesh STAs that could cause interference to transmissions during these reserved time periods, or that would experience interference from them, shall not initiate a transmission during these reserved time periods. During its MCCAOP, the MCCAOP owning mesh STA obtains a TXOP by winning an instance of EDCA contention. Because of its advance reservation, the MCCAOP owning mesh STA experiences no competition from other MCCA capable neighbor mesh STAs. To win an instance of EDCA contention during its MCCAOP, the EDCAF of the MCCAOP owning mesh STA replaces the AIFSN and CWmin value of its dot11EDCATable with MCCA access parameters.

In order to use MCCA, a mesh STA maintains synchronization with its neighboring mesh STAs. Mesh STAs that use MCCA share a DTIM interval of common duration. Additionally, a mesh STA shall track the reservations of its neighboring mesh STAs.

NOTE— In this clause, the frame whose RA field is set to individually address is referred to as an individually addressed frame. The DA field of the individually addressed frame in this context may not be set to an individual address, if the frame uses 4 address or 6 address format. Similarly, frames whose RA are set to a group address are referred to as group addressed frames, in this subclause.

- MCCA operation process

A mesh STA that decides to use MCCA shall scan the current operating channel for dot11MCCAScanDuration. During the scan operation, the mesh STA shall not initiate or accept new MCCAOPs, but the mesh STA shall set the MCCA Enabled subfield of the Mesh Capability field in the Mesh Configuration element to 1 in Beacon and Probe Response frames it transmits. The mesh STA shall transmit an MCCAOP Advertisements frame to neighbor mesh STAs that have dot11MCCAActivated equal to TRUE and do not listen to Beacon frames from this mesh STA. After the mesh STA has scanned the channel for dot11MCCAScanDuration it may initiate and accept MCCAOP Reservations.

A mesh STA with dot11MCCAActivated equal to TRUE may discover neighboring mesh STAs with dot11MCCAActivated equal to TRUE by receiving a Beacon, Probe Response, or MCCAOP Advertisements frame from the neighboring mesh STAs.

- MCCAOP Reservations

An MCCAOP Reservation specifies a schedule for one or more frame transmissions, called MCCAOPs. The schedule is set up between an MCCAOP owner and one (for individually addressed frames) or more (for group addressed frames) MCCAOP responders. MCCAOPs are set up by means of the procedure defined in MCCAOP setup procedure. Once an MCCAOP Reservation is set up,

- access to the channel by MCCA-supporting mesh STAs is governed by the procedures in Access during MCCAOP.

- the MCCAOP Reservation is advertised according to the procedures in MCCAOP Advertisements .

The schedule is signaled by means of the MCCAOP Reservation field defined in MCCAOP Reservation field and schedules one or more MCCAOPs. The starting times of these MCCAOPs are specified relative to the DTIM interval of the reporting mesh STA. These MCCAOPs have a common duration.

If the MCCAOP Periodicity subfield in the MCCAOP Reservation field equals 0 then the MCCAOP Reservation schedules one MCCAOP in the DTIM interval of the MCCAOP owner following the successful completion of the MCCAOP setup procedure described in MCCAOP setup procedure.

If the MCCAOP Periodicity subfield in the MCCAOP Reservation field is positive then the MCCAOP Reservation specifies a series of MCCAOPs. This series is started after the successful completion of the MCCAOP setup procedure and terminated when the MCCAOP Reservation is torn down.

The MCCAOP Offset subfield specifies the offset of the first scheduled transmission period of the transmission schedule relative to the beginning of the DTIM interval. The value is specified in multiples of 32 µs. If the MCCAOP Periodicity subfield is positive, additional transmission periods are scheduled and the transmission periods are separated by a distance equal to the length of the DTIM period divided by the value in the MCCAOP Periodicity subfield.

If a mesh STA adjusts its TBTT, e.g., in response to a TBTT Adjustment Request, it shall adjust the MCCAOP Reservations accordingly by modifying the MCCAOP Offset of each MCCAOP Reservation.

The set of MCCAOP Reservations in which a mesh STA is involved as an MCCAOP owner or an MCCAOP responder are referred to as the TX-RX times of this mesh STA.

An MCCAOP Reservation is identified by an MCCAOP Reservation ID. The MCCAOP owner shall select an MCCAOP Reservation ID that is unique among all of its MCCAOP Reservations. The MCCAOP Reservation ID and MCCAOP owner’s MAC address uniquely identify the MCCAOP Reservation in the mesh BSS. The MCCAOP Reservation ID is an 8-bit unsigned integer. If this MCCAOP Setup Request is for the individually addressed transmission, the MCCAOP Reservation ID is between 0 and 127. If this MCCAOP Setup Request is for the group addressed transmission, the MCCAOP Reservation ID is between 128 and 254. The special value of MCCAOP Reservation ID, when all bits are 1, is used to indicate all MCCAOPs.

- Neighborhood MCCAOP times at a mesh STA

In a mesh STA’s mesh neighborhood, all the TX-RX Times and Broadcast Times reported by its neighbors, as described in MCCAOP Advertisements , form a set of MCCAOPs that are already being used in the neighborhood. These times are referred to as Neighborhood MCCAOP times for the mesh STA. In effect, Neighborhood MCCAOP Times at a mesh STA include all MCCAOPs for which the mesh STA or one of its neighbors, including neighbors from other MBSSs, is either transmittes or receives.

A mesh STA shall track at least MaxTrack MCCAOP Reservations, including its own. The value for Max-Track is advertised in the MCCA Information field of the MCCAOP Advertisements. These tracked MCCAOP Reservations are advertised as described in MCCAOP Advertisements .. Clause Access during MCCAOP specifies how to acceto the medium during the tracked Neighborhood MCCAOP Times.

- Interfering Times for a mesh STA

Through the Interfering Times Report in its MCCAOP Advertisements, a mesh STA reports MCCAOP Reservations that its neighbors have advertised in their TX-RX Times Reports and in their Broadcast Times Reports and in which the mesh STA is not involved itself.. These times shall not be used for a new MCCAOP with the reporting mesh STA as they may experience interference MCCAOP setup procedurefrom the transmissions scheduled to take place during the reported MCCAOPs or may cause interference to them.

The Interfering Times are directly derived from the TX-RX Times Reports and Broadcast Times Reports of the neighbor peer mesh STAs.The Interfering Times Report reflects the latest TX-RX Times Reports and Braodcast Times Reports from the neighbor peer mesh STAs.

- MCCA access fraction (MAF)

The MCCA Access Fraction at a mesh STA is the ratio of the total duration of its Neighborhood MCCAOP Times, defined in Neighborhood MCCAOP times at a mesh STA, in a DTIM interval to the duration of the DTIM interval. This parameter is reported in the MCCA Information field in the MCCAOP Advertisements element. The maximum value for the MAF that is allowed at a mesh STA is specified by dot11MAFlimit. The dot11MAFlimit is copied into the MCCA Access Fraction Limit subfield of the MCCA Information field in the MCCAOP Advertisements element.

The MAF and the MAF Limit may be used to limit the use of MCCA in the mesh neighborhood of a mesh STA, as specified in MCCAOP setup procedure. Before attempting to set up an MCCAOP Reservation with a peer mesh STA, a mesh STA is required to ensure that the new MCCAOP Reservation does not cause the MAF of itself or any of its peer neighbor mesh STAs to exceed the MAF Limit. An MCCAOP Setup Request shall be refused by the intended MCCAOP responder if the MAF limit of its own neighbors is exceeded due to the new setup.

- MCCAOP setup procedure

The setup of an MCCAOP Reservation is initiated by the MCCAOP owner, and is accepted, countered, or rejected by the MCCAOP responder. The setup procedure for an MCCAOP Reservation is as follows:

- The MCCAOP owner builds a map of Neighborhood MCCAOP times in the DTIM interval after hearing Advertisements from all of its neighbor mesh STAs with dot11MCCAActivated equal to TRUE. If no advertisement was heard from a neighbor mesh STA in the last dot11MCCAAdvertPeriodMax DTIM intervals or if the Partial Information bitwas equal to 1 in the MCCA Informationfield of the most recent advertisement, the mesh STA may request for an MCCAOP Advertisement from the neighbor mesh STA.

- Based on traffic characteristics, the MCCAOP owner determines the MCCAOP Reservation. The MCCAOP parameters shall be chosen in such a way that they satisfy the following conditions:

- The reservation does not overlap with either the Neighborhood MCCAOP Times of the MCCAOP owner.

- The reservation does not overlap with the Interfering Times of the intended MCCAOP responder or responders.

- It also avoids using times that are known to it as being used by Beacon transmission times of itself or one of its neighbor peer mesh STAs for other activities such as Beacon transmissions.

- The reservation does not overlap with known Beacon transmission times or HCCA times of neighbor APs.

- The reservation does not cause the MAF limit to be exceeded for either itself or its peer mesh STAs.

- If the conditions in b) are satisfied and if the MCCA Reservation is intended for individually addressed frames, the MCCAOP owner transmits an MCCAOP Setup Request element to the intended MCCAOP responder with chosen MCCAOP starting times and duration.

- If the conditions in b) are satisfied and if the MCCA Reservation is intended for group addressed frames, the MCCAOP owner transmits an MCCAOP Setup Request to the intended MCCAOP responders. Such MCCAOP Setup Requests may be sent in individually addressed frames or group addressed frames.

- The MCCAOP responder verifies the following conditions:

- The MCCAOP Reservation does not overlap with its Neighborhood MCCAOP Times excluding those owned by the intended MCCAOP owner.

- The MCCAOP Reservation does not overlap with other known transmission times of itself or its neighbor peer mesh STAs, e.g. for Beacon transmissions..

- The MCCAOP Reservation does not overlap with known Beacon transmission times or HCCA times of neighbor APs

- The MCCAOP Reservation does not cause the MAF limit to be exceeded for itself or its peer mesh STAs.

- If the conditions in e) are satisfied, the responder shall send an MCCAOP Setup Reply frame to the MCCAOP owner with the MCCA Reply Code field in MCCAOP Setup Request elementequal to Accept.

- If the conditions in e) are not satisfied and the MCCAOP request is intended for individually addressed transmissions,, the responder may calculate an alternative MCCAOP Reservation and include it in the MCCAOP Reservation field of the MCCAOP Setup Reply element.

- If the conditions in e) are not satisfied and the MCCAOP request is intended for group addressed transmissions, the responder sends an MCCAOP Setup Reply frame to the MCCAOP owner with the MCCA Reply Code field in MCCAOP Setup Request elementequal to rejection of the MCCAOP Setup Request.

- If the MCCAOP responder accepts a Setup Request for a group addressed Reservation, it shall include the reservation into its MCCAOP Advertisements only after the MCCAOP Advertisements from the owner of the Broadcast MCCAOP is received.

- If the MCCAOP owner receives an MCCA Setup Reply frame with MCCAOP Setup Reply code equal to Accept, the MCCAOP Reservation is established. Otherwise, the mesh STA may repeat the MCCAOP Setup procedure using a modified MCCAOP Setup Request. If an alternative MCCAOP Reservation is included in the MCCAOP Setup Reply, the mesh STA may consider this alternative in its modified MCCAOP Setup Request.

- MCCAOP Advertisements

A mesh STA with dot11MCCAActivated equal to TRUE shall track up to MaxTrack Reservations including its own reservations. It shall advertise the tracked reservations in the TX-RX, Broadcast, and Interfering Times using the MCCAOP Advertisements element, at least once in dot11MCCAAdvertPeriodMax DTIM intervals. It need not report all reservations in each MCCAOP Advertisements element. If it reports only a subset of the reservations, then this shall be indicated by setting the Partial Report subfield in the MCCAOP Information field to 1. This can be useful, e.g. when the mesh STA advertises new reservations only.

These advertisements shall be transmitted in Beacon frames, Probe Response frames or MCCA Advertisements frames. The advertised times may include:

- TX-RX Times Report:

- All MCCAOP times for individually addressed frames for which the mesh STA is the MCCAOP owner or the MCCAOP responder.

- HCCA times for which the mesh STA is the transmitter.

- Broadcast Times Report:

- All MCCAOP times for group addressed frames for which the mesh STA is the MCCAOP owner or the MCCAOP responder.

- Known Beacon transmissions Times for which the mesh STA is either the transmitter or the receiver.

- Interfering Times Report:

- TX-RX and Broadcast times reported by the neighbor peer mesh STAs of the mesh STA excluding those times for which the mesh STA is either the MCCAOP owner or the MCCAOP responder.

Each of the reports shall consist of a number of reported MCCAOP Reservations, as described in Error: Reference source not found. If a mesh STA adjusts its TBTT, e.g., in response to a TBTT Adjustment Request, it shall adjust the Reports accordingly by modifying the MCCAOP Offset of each of the reported MCCAOP Reservations. If a mesh STA adjusts its timing offset value with respect to a peer mesh STA, as specified in Error: Reference source not found, it shall adjust the Reports accordingly by modifying the MCCAOP Offset of each of the re-ported MCCAOP Reservations.

- MCCAOP Reservation teardown

The MCCAOP owner and the MCCAOP responder may initiate a teardown of an MCCAOP Reservation by transmitting an MCCAOP Reservation Teardown frame to the communicating partner. A teardown shall be initiated when either the MCCAOP owner or the MCCAOP responder learn of a conflicting MCCAOP Reservation and if the MAC address of the owner of this conflicting Reservation, in reverse order, evaluates to a higher value than the MAC address of the MCCAOP owner, in reverse order. After a teardown, the MCCAOP owner and MCCAOP responder stop advertising the MCCAOP Reservation in their TX-RX Times Report or Broadcast Times Report.

The transmitter of the MCCAOP Reservation Teardown frame assumes a successful teardown and stops advertising the MCCAOP Reservation in its TX-RX Times Report or Broadcast Times Report if any of the following happens:

- Its MCCAOP Reservation Teardown frame is successfully Acknowledged.

- The advertisement of the receiver of the MCCAOP Reservation Teardown frame does not include the MCCAOP Reservation

- The intended receiver of the MCCAOP Reservation Teardown frame is unreachable for greater than dot11MCCAOPtimeout time

The receiver of the MCCAOP Reservation Teardown frame assumes a successful teardown and stops advertising the MCCAOP Reservation in its TX-RX Times Report or Broadcast Times Report if any of the following happens:

- The advertisement of the transmitter of the MCCAOP Reservation Teardown frame does not include the MCCAOP Reservation.

- The transmitter of the MCCAOP Reservation Teardown frame is inactive for greater than dot11MCCAOPtimeout time.

A teardown shall be assumed by the MCCAOP owner when the MCCAOP responder has been unreachable for greater than dot11MCCAOPtimeout time. A teardown shall be assumed by the MCCAOP responder when the MCCAOP owner has been inactive for greater than dot11MCCAOPtimeout time.

- Access during MCCAOP

- Access by MCCAOP Owners

The EDCAF of the MCCAOP owning mesh STA shall set its AIFSN equal to dot11MCCAAIFSN and its CWmin equal to dot11MCCACWmin during its MCCAOP.

An MCCAOP owner successfully obtains an MCCA TXOP when it completes a frame exchange with the MCCAOP responder. If the mesh STA successfully obtains a TXOP, it may transmit until the end of the MCCAOP, using the rules specified in 9.9.1.4 and 9.9.1.6.

If an MCCAOP owner has no data to transmit, it shall transmit a QoS Null frame during the MCCAOP to end the MCCAOP.

If a mesh STA accesses the channel during the MCCAOP but fails to obtain either an initial or a subsequent TXOP, the mesh STA shall perform the backoff procedure specified in 9.9.1.5.

- Access during an MCCAOP by mesh STAs that are neither the MCCAOP owner nor the MCCAOP responder

A mesh STA with dot11MCCAActivated is true shall maintain a Reservation Allocation Vector (RAV) for each reservation in its Interfering Times. A RAV is set when a corresponding MCCAOP starts and it is initialized to the duration of the MCCAOP given in the MCCAOP Duration subfield of the MCCAOP Reservation. The RAVs are implemented as timers. They expire when they reach 0.

RAVs are set in addition to the NAV. At any time, both the NAV and the non-zero RAVs inhibit access to the medium.

A mesh STA may update its RAV for an MCCAOP upon receipt of a frame that originates from either the MCCAOP owner or responder. Upon receipt of a frame from either the MCCAOP owner or responder, the mesh STA may update the RAV timer to the value in the Duration/ID field of this frame. The RAV for an MCCAOP is not adjusted upon receipt of frames originating from stations that are not the MCCAOP owner or responder.

An EDCA TXOP of a mesh STA with dot11MCCAActivated equal to TRUE shall not extend across any MCCAOP in its neighborhood MCCAOP times.

- (normative) ASN.1 encoding of the MAC and PHY MIB

Insert the following objects to the Major sections:

-- dot11MeshStationConfigTable 						::= { dot11smt 18 }

-- dot11MeshHWMPConfigTable 						::= { dot11smt 19 }

Change the “Dot11StationConfigEntry” of the “dotStationConfigTable” as follows:

Dot11StationConfigEntry ::=

SEQUENCE {

dot11StationID 								MacAddress,

dot11MediumOccupancyLimit 								INTEGER,

dot11CFPollable 								TruthValue,

dot11CFPeriod								INTEGER,

dot11CFPMaxDuration 								INTEGER,

dot11AuthenticationResponseTimeOut 								Unsigned32,

dot11PrivacyOptionImplemented 								TruthValue,

dot11PowerManagementMode 								INTEGER,

dot11DesiredSSID OCTET 								STRING,

dot11DesiredBSSType 								INTEGER,

dot11OperationalRateSet 								OCTET STRING,

dot11BeaconPeriod 								INTEGER,

dot11DTIMPeriod 								INTEGER,

dot11AssociationResponseTimeOut 								Unsigned32,

dot11DisassociateReason 								INTEGER,

dot11DisassociateStation 								MacAddress,

dot11DeauthenticateReason 								INTEGER,

dot11DeauthenticateStation 								MacAddress,

dot11AuthenticateFailStatus 								INTEGER,

dot11AuthenticateFailStation 								MacAddress,

dot11MultiDomainCapabilityImplemented 								TruthValue,

dot11MultiDomainCapabilityEnabled 								TruthValue,

dot11CountryString 								OCTET STRING,

dot11SpectrumManagementImplemented 								TruthValue,

dot11SpectrumManagementRequired 								TruthValue,

dot11RSNAOptionImplemented 								TruthValue,

dot11RSNAPreauthenticationImplemented 								TruthValue,

dot11RegulatoryClassesImplemented	 							TruthValue,

dot11RegulatoryClassesRequired 								TruthValue,

dot11QosOptionImplemented 								TruthValue,

dot11ImmediateBlockAckOptionImplemented 								TruthValue,

dot11DelayedBlockAckOptionImplemented 								TruthValue,

dot11DirectOptionImplemented 								TruthValue,

dot11APSDOptionImplemented 								TruthValue,

dot11QAckOptionImplemented 								TruthValue,

dot11QBSSLoadOptionImplemented 								TruthValue,

dot11QueueRequestOptionImplemented 								TruthValue,

dot11TXOPRequestOptionImplemented 								TruthValue,

dot11MoreDataAckOptionImplemented 								TruthValue,

dot11AssociatedinNQBSS 								TruthValue,

dot11DLSAllowdInQBSS 								TruthValue,

dot11DLSAllowed 								TruthValue,

dot11AssociateStation 								MacAddress,

dot11AssociateID 								INTEGER,

dot11AssociateFailStation 								MacAddress,

dot11AssociateFailStatus 								INTEGER,

dot11ReassociateStation 								MacAddress,

dot11ReassociateID 								INTEGER,

dot11ReassociateFailStation 								MacAddress,

dot11ReassociateFailStatus 								INTEGER,

dot11RadioMeasurementCapable 								TruthValue,

dot11RadioMeasurementEnabled 								TruthValue,

dot11RRMMeasurementProbeDelay 								INTEGER,

dot11RRMMeasurementPilotPeriod 								INTEGER,

dot11RRMLinkMeasurementEnabled 								TruthValue,

dot11RRMNeighborReportEnabled 								TruthValue,

dot11RRMParallelMeasurementsEnabled								TruthValue,

dot11RRMRepeatedMeasurementsEnabled								TruthValue,

dot11RRMBeaconPassiveMeasurementEnabled								TruthValue,

dot11RRMBeaconActiveMeasurementEnabled								TruthValue,

dot11RRMBeaconTableMeasurementEnabled 								TruthValue,

dot11RRMBeaconMeasurementReportingConditionsEnabled								TruthValue,

dot11RRMFrameMeasurementEnabled 								TruthValue,

dot11RRMChannelLoadMeasurementEnabled 								TruthValue,

dot11RRMNoiseHistogramMeasurementEnabled 								TruthValue

dot11RRMStatisticsMeasaurementEnabled 								TruthValue,

dot11RRMLCIMeasurementEnabled 								TruthValue,

dot11RRMLCIAzimuthEnabled 								TruthValue,

dot11RRMTransmitStreamCategoryMeasurementEnabled								TruthValue,

dot11RRMTriggeredTransmitStreamCategoryMeasurementEnabled TruthValue,

dot11RRMAPChannelReportEnabled 								TruthValue,

dot11RRMMIBEnabled 								TruthValue,

dot11RRMMaxMeasurementDuration 								Unsigned32,

dot11RRMNonOperatingChannelMaxMeasurementDuration								Unsigned32,

dot11RRMMeasurementPilotTransmissionInformationEnabled TruthValue,

dot11RRMMeasurementPilotCapability 								Unsigned32,

dot11RRMNeighborReportTSFOffsetEnabled 								TruthValue,

dot11RRMRCPIMeasurementEnabled 								TruthValue,

dot11RRMRSNIMeasurementEnabled 								TruthValue,

dot11RRMBSSAverageAccessDelayEnabled 								TruthValue,

dot11RRMBSSAvailableAdmissionCapacityEnabled 								TruthValue,

dot11RRMAntennaInformationEnabled 								TruthValue,

dot11FastBSSTransitionImplemented 								TruthValue,

dot11LCIDSEImplemented 								TruthValue,

dot11LCIDSERequired 								TruthValue,

dot11DSERequired 								TruthValue,

dot11ExtendedChannelSwitchEnabled 								TruthValue,

dot11RSNAProtectedManagementFramesEnabled 								TruthValue,

dot11RSNAUnprotectedManagementFramesAllowed 								TruthValue,

dot11AssociationPingResponseTimeout 								Unsigned32,

dot11AssociationMaximumPingAttempts 								INTEGER,

dot11HighThroughputOptionImplemented 								TruthValue,

dot11RSNAPBACRequired 								TruthValue},

dot11MeshActivated								TruthValue }

Insert “dot11MeshActivated” to the end of  “dotStationConfigTable”:

dot11MeshActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This is a control variable.

It is written by an external management entity.

This attribute shall specify whether or not mesh services are supported by a station.”

::= { dot11StationConfigEntry 94 }

Change the dot11OperationEntity as shown:

dot11OperationEntry ::=

SEQUENCE {dot11MACAddress 							MacAddress,

dot11RTSThreshold 						INTEGER,

dot11ShortRetryLimit 						INTEGER,

dot11LongRetryLimit 						INTEGER,

dot11FragmentationThreshold 						INTEGER,

dot11MaxTransmitMSDULifetime 						Unsigned32,

dot11MaxReceiveLifetime 						Unsigned32,

dot11ManufacturerID 						DisplayString,

dot11ProductID 						DisplayString,

dot11CAPLimit 						INTEGER,

dot11HCCWmin 						INTEGER,

dot11HCCWmax 						INTEGER,

dot11HCCAIFSN 						INTEGER,

dot11ADDBAResponseTimeout 						INTEGER,

dot11ADDTSResponseTimeout 						INTEGER,

dot11ChannelUtilizationBeaconInterval 						INTEGER,

dot11ScheduleTimeout 						INTEGER,

dot11DLSResponseTimeout 						INTEGER,

dot11QAPMissingAckRetryLimit 						INTEGER,

dot11EDCAAveragingPeriod 						INTEGER, }

dot11SAERetransPeriod						INTEGER,

dot11SAEThresh						INTEGER,

dot11SAESync						INTEGER }

Insert the following to the end of dot11OperationEntry Table in Annex D:

dot11SAERetransPeriod OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when joining or establishing a BSS.

This object specifies the initial retry timeout, in millisecond units, used by the SAE authentication and key establishment protocol."

DEFVAL {40}

::= { dot11OperationEntry 21}

dot11SAEThresh OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

This object specifies the maximum number of SAE protocol instances allowed to simultaneously be in either Commit or Confirmed state."

DEFVAL {5}

::= { dot11OperationEntry 22}

dot11SAESync OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

This object specifies the maximum number of synchronization errors that are allowed to happen prior to disassociation of the offending SAE peer."

DEFVAL {5}

::= { dot11OperationEntry 23}

Change the “dot11RSNAConfigNumberOfPTKSAReplayCounters” in Annex D as follows:

dot11RSNAConfigNumberOfPTKSAReplayCounters OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

Specifies the number of PTKSA replay counters per association:

0 –> 1 replay counters,

1 -> 2 replay counters,

2 –> 4 replay counters,

3 –> 16 replay counters"

DEFVAL { 3 }

::= { dot11RSNAConfigEntry 18 }

Change the “dot11RSNAConfigNumberOfGTKSAReplayCounters” in Annex D as follows:

dot11RSNAConfigNumberOfGTKSAReplayCounters OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

Specifies the number of GTKSA replay counters per association:

0 –> 1 replay counter,

1 –> 2 replay counters,

2 –> 4 replay counters,

3 –> 16 replay counters"

DEFVAL { 3 }

::= { dot11RSNAConfigEntry 29 }

Change the “dot11RSNAConfigNumberOfSTKSAReplayCounters” in Annex D as follows:

dot11RSNAConfigNumberOfSTKSAReplayCounters OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

Specifies the number of STKSA replay counters per association:

0 -> 1 replay counter,

1 -> 2 replay counters,

2 -> 4 replay counters,

3 -> 16 replay counters"

DEFVAL { 3 }

::= { dot11RSNAConfigEntry 37 }

Insert the following dot11 MeshSTATable to the end of Annex D:

**********************************************************************

* dot11MeshSTAConfig TABLE

**********************************************************************

dot11MeshStationConfigTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11MeshStationConfigEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Mesh Station Configuration attributes. In tabular form to allow for multiple instances on an agent."

dot11MeshStationConfigEntry  OBJECT-TYPE

SYNTAX Dot11MeshStationConfigEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11MeshStationConfigTable. It is possible for there to be multiple IEEE 802.11 interfaces on one agent, each with its unique MAC address. The relationship between an IEEE 802.11 interface and an interface in the context of the Internet-standard MIB is one-to-one. As such, the value of an ifIndex object instance can be directly used to identify corresponding instances of the objects defined herein.

ifIndex - Each IEEE 802.11 interface is represented by an ifEntry. Interface tables in this MIB module are indexed by ifIndex."

INDEX { ifIndex }

::= { dot11MeshStationConfigTable 1 }

Dot11MeshStationConfigEntry ::=

SEQUENCE {

dot11MeshID							OCTET STRING,

dot11MeshSecurityActivated							TruthValue,

dot11MeshConfigGroupUpdateCount							INTEGER,

dot11MeshActivePathSelectionProtocol							INTEGER,

dot11MeshForwarding							TruthValue,

dot11MeshMaxRetries							INTEGER,

dot11MeshRetryTimeout							INTEGER,

dot11MeshTTL							INTEGER,

dot11MeshPortalAnnouncementProtocol 							TruthValue,

dot11MeshPortalAnnouncementInterval							INTEGER,

dot11MeshConfirmTimeout							INTEGER,

dot11MeshHoldingTimeout							INTEGER,

dot11MeshNbrOffsetActivated							TruthValue,

dot11MeshNbrOffsetMaxNeighbor							INTEGER,

dot11MeshBeaconTimingReportInterval							INTEGER,

dot11MeshBeaconTimingReportMaxNum							INTEGER,

dot11MeshTSFAdjustmentActivated							TruthValue,

dot11MeshTbttSelectionActivated							TruthValue,

dot11MeshDelayedBeaconTxInterval							INTEGER,

dot11MeshDelayedBeaconDelayAmount							INTEGER,

dot11MeshAverageBeaconFrameDuration							INTEGER,

dot11MeshSTAMissingAckRetryLimit							INTEGER,

dot11MCCAImplemented 							TruthValue,

dot11MCCAActivated 							TruthValue,

dot11MAFlimit 							INTEGER,

dot11MCCAScanDuration							INTEGER,

dot11MCCAAdvertPeriodMax							INTEGER,

dot11MCCATrackStates OBJECT-TYPE							INTEGER,

dot11MCCAOPtimeout							INTEGER,

dot11MCCACWmin							INTEGER,

dot11MCCAAIFSN							INTEGER

}

dot11MeshID OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..32))

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This attribute reflects the Mesh ID configured in this entity."

dot11MeshSecurityActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This object shall specify whether or not the station is security enabled."

dot11MeshConfigGroupUpdateCount OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

The number of times Mesh Group Key Inform frame in Mesh Group Key Handshake will be retried per Mesh Group Key Handshake attempt."

DEFVAL { 3 }

dot11MeshActivePathSelectionProtocol OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This object shall specify the active path selection protocol in the mesh BSS. The default value for this attribute is 0 (HWMP)."

dot11MeshForwarding OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This attribute shall specify the ability of a mesh STA to forward frames. The default value for this attribute is true."

dot11MeshMaxRetries OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This object specifies the maximum number of Mesh Peering Open retries that can be sent to establish a new mesh peering instance in a mesh. The default value for this attribute is 2."

dot11MeshRetryTimeout OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This object specifies the initial retry timeout, in millisecond units, used by the Mesh Peering Open message. The default value for this attribute is 40."

dot11MeshTTL OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This attribute shall specify the value of TTL field set at a source mesh STA. The default value for this attribute is 31."

dot11MeshPortalAnnouncementProtocol OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This attribute shall specify whether or not the mesh STA is using the Portal Announcement protocol. The default value for this attribute is false."

dot11MeshPortalAnnouncementInterval OBJECT-TYPE

SYNTAX INTEGER (1..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

The Portal Announcement Interval is the number of seconds between the transmission of two Portal Announcements. The default value for this attribute is 10."

dot11MeshConfirmTimeout OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This object specifies the initial retry timeout, in millisecond units, used by the Mesh Peering Open message. The default value for this attribute is 40."

dot11MeshHoldingTimeout OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This object specifies the confirm timeout, in millisecond units, used by the mesh peering management to close a mesh peering. The default value for this attribute is 40."

dot11MeshNbrOffsetActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This object shall specify whether or not the station is neighbor offset protocol activated. The default value for this attribute is true. "

dot11MeshNbrOffsetMaxNeighbor OBJECT-TYPE

SYNTAX INTEGER  (1..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This object shall specify the maximum number of neighbor STAs with which the mesh STA maintains the synchronization using the neighbor offset protocol. The default value for this attribute is 16. "

dot11MeshBeaconTimingReportInterval OBJECT-TYPE

SYNTAX INTEGER  (0..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This object shall specify when the Beacon Timing element is present in Beacon frames. When set to 0, Beacon Timing element is present in neither the Beacon frame nor the Probe Response frames. In other cases, Beacon Timing element is present when the DTIM Count value in the Beacon frame is zero or equal to integer multiply of set value. The default value for this attribute is 0. "

dot11MeshBeaconTimingReportMaxNum OBJECT-TYPE

SYNTAX INTEGER  (0..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This object shall specify the maximum number of beacon timing information contained in a Beacon Timing element in the transmitting Beacon frames. The default value for this attribute is 16. "

dot11MeshTSFAdjustmentActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This object shall specify whether or not the station activates TSF adjustment functions. When set to true, the mesh STA adjusts its local TSF timer according to the TSF offset value among neighbor STAs with which the STA maintains synchronization. The default value for this attribute is false. "

dot11MeshTbttSelectionActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This object shall specify whether or not the station activates Mesh TBTT selection functions, which is utilized to avoid beacon collisions. When set to true, the mesh STA selects its TBTT not to collide with neighbor’s and neighbor’s neighbor’s TBTT. The default value for this attribute is false. "

dot11MeshDelayedBeaconTxInterval OBJECT-TYPE

SYNTAX INTEGER  (0..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This attribute shall specify the interval for the delayed beacon transmission from TBTT, for MBCA purpose. The value is expressed in unit of beacon interval, whereas the value 0 indicates that the delayed beacon transmission is disabled exceptionally. The default value for this attribute is 0."

dot11MeshDelayedBeaconDelayAmount OBJECT-TYPE

SYNTAX INTEGER  (0..4023)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This attribute shall specify the delay amount for the delayed beacon transmission from TBTT, for MBCA purpose. The value is expressed in units of microseconds. The default value for this attribute is 0."

dot11MeshAverageBeaconFrameDuration OBJECT-TYPE

SYNTAX INTEGER  (0..4023)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This attribute shall specify the expected averaged duration of last 16 Beacon frames received by the neighboring mesh STAs. The value is expressed in units of microseconds."

dot11MeshSTAMissingAckRetryLimit OBJECT-TYPE

SYNTAX INTEGER (0..100)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This attribute indicates the number of times the mesh STA may retry a frame for which it does not receive an ACK for a STA in power-save mode after the mesh STA does not receive an ACK to a directed MPDU sent with the EOSP set to 1."

dot11MCCAImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

This object shall specify whether or not the MCCA is implemented in this station."

dot11MCCAActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This object shall specify whether or not the station is MCCA enabled."

dot11MAFlimit OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This object shall specify the maximum MCCA Access Fraction allowed at the mesh STA. This number is always a multiple of (1/255)."

dot11MCCAScanDuration OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This attribute specifies the duration in TUs that a mesh STA scans on the current operating channel before it may accept or setup MCCAOPs."

dot11MCCAAdvertPeriodMax OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This object shall specify the maximum interval that a mesh STA with dot11MCCAActivated equal to TRUE waits for an MCCAOP Advertisement update.  It is expressed in number of DTIM intervals."

dot11MCCATrackStates OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This object shall specify the minimum number of MCCAOP reservations which a mesh STA with dot11MCCAActivated equal to TRUE must track; it equals 83, which is the maximum number of MCCAOP reservations that can be reported in one MCCAOP Advertisement element."

dot11MCCAOPtimeout OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This object specifies the timeout value for an MCCAOP teardown. It is expressed in TU units."

dot11MCCACWmin OBJECT-TYPE

SYNTAX INTEGER (0..aCWmin)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This attribute shall specify the value of the minimum size of the window that shall be used by the mesh STA during the MCCAOP for which it is the MCCAOP ownerfor generating a random number for the backoff. The default value of this attribute shall be 0."

dot11MCCAAIFSN OBJECT-TYPE

SYNTAX INTEGER (0..15)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This attribute shall specify the number of slots, after a SIFS duration, that the mesh STA shall sense the medium idle either before transmitting or executing a backoff during an MCCAOP for which it is the MCCAOP owner. The default value of this attribute shall be 1."

**********************************************************************

* End of dot11MeshSTAConfig TABLE

**********************************************************************

**********************************************************************

* dot11MeshHWMPConfig TABLE

**********************************************************************

dot11MeshHWMPConfigTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11MeshHWMPConfigEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Mesh Station HWMP Configuration attributes. In tabular form to allow for multiple instances on an agent."

dot11MeshHWMPConfigEntry  OBJECT-TYPE

SYNTAX Dot11MeshHWMPConfigEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11MeshHWMPConfigTable. It is possible for there to be multiple IEEE 802.11 interfaces on one agent, each with its unique MAC address. The relationship between an IEEE 802.11 interface and an interface in the context of the Internet-standard MIB is one-to-one. As such, the value of an ifIndex object instance can be directly used to identify corresponding instances of the objects defined herein.

ifIndex - Each IEEE 802.11 interface is represented by an ifEntry. Interface tables in this MIB module are indexed by ifIndex."

INDEX { ifIndex }

::= { dot11MeshStationConfigTable 1 }

Dot11MeshHWMPConfigEntry ::=

SEQUENCE {

dot11MeshHWMPmaxPREQretries							INTEGER,

dot11MeshHWMPnetDiameter							INTEGER,

dot11MeshHWMPnetDiameterTraversalTime							INGETER,

dot11MeshHWMPpreqMinInterval							INTEGER,

dot11MeshHWMPperrMinInterval							INTEGER,

dot11MeshHWMPactiveRootTimeout							INTEGER,

dot11MeshHWMPactivePathTimeout							INTEGER,

dot11MeshHWMProotMode							INTEGER,

dot11MeshHWMPpathToRootTimeout							INTEGER,

dot11MeshHWMProotInterval							INTEGER,

dot11MeshHWMPrannInterval							INTEGER,

dot11MeshHWMPtargetOnly							INTEGER,

dot11MeshHWMPreplyAndForward							INTEGER,

dot11MeshHWMPmaintenanceInterval							INTEGER,

dot11MeshHWMPconfirmationInterval							INTEGER

}

dot11MeshHWMPmaxPREQretries OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This object shall specify the number of Action frames containing a PREQ that an originator mesh STA can send to a particular path target for a specific path discovery. The default value of this attribute is 3"

::= { dot11MeshHWMPConfigEntry 1}

dot11MeshHWMPnetDiameter OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This object shall specify the estimate of the maximum number of hops that it takes for an HWMP element to propagate across the mesh. The default value of this attribute is 31"

::= { dot11MeshHWMPConfigEntry 2}

dot11MeshHWMPnetDiameterTraversalTime OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This object shall specify the estimate of the interval of time (in TUs) that it takes for an HWMP element to propagate across the mesh. The default value of this attribute is 500"

::= { dot11MeshHWMPConfigEntry 3}

dot11MeshHWMPpreqMinInterval OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This object shall specify the minimum interval of time (in TUs) during which a mesh STA can send only one Action frame containing a PREQ element. The default value of this attribute is 100"

::= { dot11MeshHWMPConfigEntry 4}

dot11MeshHWMPperrMinInterval OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This object shall specify the minimum interval of time (in TUs) during which a mesh STA can send only one Action frame containing a PERR element. The default value of this attribute is 100"

::= { dot11MeshHWMPConfigEntry 5}

dot11MeshHWMPactiveRootTimeout OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This object shall specify the time (in TUs) for which mesh STAs receiving a PREQ to individual target(s) shall consider the forwarding information from the root to be valid. The default value of this attribute is 5000"

::= { dot11MeshHWMPConfigEntry 6}

dot11MeshHWMPactivePathTimeout OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This object shall specify the time (in TUs) for which mesh STAs receiving a PREQ to individual target(s) shall consider the forwarding information to be valid. The default value of this attribute is 5000"

::= { dot11MeshHWMPConfigEntry 7}

dot11MeshHWMProotMode OBJECT-TYPE

SYNTAX INTEGER {

noRoot(0),

proactivePREQnoPREP(2),

proactivePREQwithPREP(3),

rann(4) }

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This attribute controls the configuration of a mesh STA as root mesh STA. A mesh STA is configured as a root mesh STA if dot11MeshHWMProotMode is set to 2, 3 or 4. Different values correspond to different modes of the root mesh STA. The mesh STA is not a root mesh STA when the attribute is set to 0. The default value of this attribute is 0"

::= { dot11MeshHWMPConfigEntry 8}

dot11MeshHWMPpathToRootTimeout OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This object shall specify the time (in TUs) for which mesh STAs receiving a proactive PREQ shall consider the forwarding information to the root mesh STA to be valid; it must be greater than dot11MeshHWMProotInterval. The default value of this attribute is 5000"

::= { dot11MeshHWMPConfigEntry 9}

dot11MeshHWMProotInterval OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This object shall specify the minimum interval of time (in TUs) during which a root mesh STA can send only one Action frame containing a proactive PREQ element. The default value of this attribute is 2000"

::= { dot11MeshHWMPConfigEntry 10}

dot11MeshHWMPrannInterval OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This object shall specify the minimum interval of time (in TUs) during which a mesh STA can send only one Action frame containing a RANN element. The default value of this attribute is 1000"

::= { dot11MeshHWMPConfigEntry 11}

dot11MeshHWMPtargetOnly OBJECT-TYPE

SYNTAX INTEGER { intermediateMSTA(0), targetOnly(1)}

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This attribute, when set to 0, shall allow intermediate mesh STAs to respond with a PREP to a PREQ if they have valid forwarding information to the requested target. When set to 1, only the target MSTA is allowed to respond with a PREP to a PREQ. The default value of this attribute is 1"

::= { dot11MeshHWMPConfigEntry 12}

dot11MeshHWMPreplyAndForward OBJECT-TYPE

SYNTAX INTEGER { noPropagation(0), propagation(1)}

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This attribute, when set to 1, shall require intermediate mesh STAs that respond with a PREP to a PREQ to propagate the PREQ further towards the target MSTA. The default value of this attribute is 1"

::= { dot11MeshHWMPConfigEntry 13}

dot11MeshHWMPmaintenanceInterval OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This object shall specify the minimum interval of time (in TUs) during which a mesh STA can send only one Action frame containing a PREQ element for path maintenance. The default value of this attribute is 2000"

::= { dot11MeshHWMPConfigEntry 14}

dot11MeshHWMPconfirmationInterval OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

This object shall specify the minimum interval of time (in TUs) during which a mesh STA can send only one Action frame containing a PREQ element for root path confirmation. The default value of this attribute is 2000"

::= { dot11MeshHWMPConfigEntry 15}