IEEE P802.11
Wireless LANs

| Some More SAE Comments Resolved |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-01-21 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Dan Harkins | Aruba Networks | 1322 Crossman ave, Sunnyvale, CA. | +1 408 227 4500 | dharkins at arubanetworks dot com |

Abstract

This document provides resolution to the following CIDS : 2566, 2284, 2285, 2569, 2570. Please disregard the change bars in this Abstract. :-(

- Authentication frame format

Change the second sentence of 7.2.3.10 as shown:

Only Authentication frames with the authentication algorithm set to Open System authentication or, Fast BSS Transition, or SAE authentication may can be used within an RSNA.

Insert the following new rows into table 7-16:

| - Authentication frame bodyinput.doc |  |  |
| --- | --- | --- |
| Order | Information | Notes |
|  |  | Error: Reference source not found |
| 10 | Finite Cyclic Group | An unsigned integer indicating a finite cyclic group as described in Error: Reference source not found. This is present in SAE authentication frames. |
| 11 | Anti-Clogging Token | A random bit-string used for anti-clogging purposes as described in Error: Reference source not found. This is present in SAE authentication frames. |
| 12 | Send-Confirm | A binary encoding of an integer used for anti-replay purposes as described in Error: Reference source not found. This is present in SAE authentication frames. |
| 13 | Scalar | An unsigned integer encoded as described in Error: Reference source not found. This is present in SAE authentication frames. |
| 14 | Element | A field element from a finite field encoded as described in Error: Reference source not found. This is present in SAE authentication frames. |
| 15 | Confirm | An unsigned integer encoded as described in Error: Reference source not found. This is present in SAE authentication frames. |

Insert the following new rows into table 7-17 and change the title of the fourth column:

| - Presence of information elements in Authentication framesinput.doc |  |  |  |
| --- | --- | --- | --- |
| Authentication Algorithm | Authentication transaction sequence number | Status Code | Presence of fields 4-915 |
| SAE | 1 | Status | Scalar is present if Status is zero. Element is present if Status is zero. Anti-Clogging Token is present if status is 52 or if frame is in response to a previous rejection with Status 52. Finite Cyclic Group is present if Status is zero. |
| SAE | 2 | Status | Send-Confirm counter is present. Confirm is present. |

- Status Code field

Please make the new status code have <ANA 17> and increment all the rest of the <ANA *> numbers.

Insert the following rows into Status codesinput.doc and change the last row (Reserved) as shown.

| - Status codesinput.doc |  |
| --- | --- |
| Status code | Meaning |
|  | Authentication is rejected because an anti-clogging token is required |
|  | Authentication is rejected because the offered finite cyclic group is not supported. |
| +1 42-65535 | Reserved |

- Scalar field

The Scalar field is used with SAE authentication to communicate cryptographic material and is described in Scalar field . Itsr construction and encoding is described in Error: Reference source not found.

|  | Scalar |
| --- | --- |
| Octets: | variable |
| - Scalar field |  |

7.3.1.37 Element field

The Element field is used with SAE authentication to communicate an element in a finite field and is described in Figure s9 (Element field). It’s construction and encoding is described in 8.2A.5.4 (Encoding of Commit Messages).

|  | Element |
| --- | --- |
| Octets: | variable |
| - Element field |  |

- Finite Cyclic Group field

The Finite Cyclic Group is used in SAE to indicate which cryptographic group to use in the SAE exchange. The number to group mapping is handled by the Diffie-Hellman Group Transform ID registry managed by IANA for the Internet Key Exchange (IKE), RFC 2409

|  | Finite Cyclic Group |
| --- | --- |
| Octets: | 2 |
| - Finite Cyclic Group field |  |

Increment existing sections 7.3.1.37 and 7.3.1.39

References: