IEEE P802.11
Wireless LANs

Comment Resolution for CID#125

Date: 2009-01-20

| Name | Company | Address | Phone | email |
| --- | --- | --- | --- | --- |
| Henry Ptasinski | Broadcom Corporation | 150 Mathilda Place Sunnyvale CA | (408) 543-3316 | henryp@broadcom.com |
| Emily Qi | Intel Corporation | 2111 NE 25th avenue Hillsboro, OR | 5032647799 | Emily.h.qi@intel.com |

Abstract

This proposal proposes comment resolutions for TGv SB1 CID #125.

The proposed text changes are based on TGv draft D7.04.

TGv editor: In Figure 8-40, Change “Sleep Mode” to “WNM-Sleep Mode”.

Change 8.5.6.1.3 as follows:

8.5.6.1.3 Authenticator state machine: Group Key Handshake (global)

The following list summarizes the states the Authenticator state machine uses to coordinate a group key

update of all STAs:

— GTK_INIT: This state is entered on system initialization.

— SETKEYS: This state is entered if the GTK is to be updated on all Supplicants.

— SETKEYSDONE: This state is entered if the GTK has been updated on all Supplicants.

NOTE—SETKEYSDONE calls SetGTK to set the GTK for all associated STAs that are not in WNM-Sleep Mode. If this fails, all communication via this key will fail, and the AP needs to detect and recover from this situation. A STA that is in WNM-Sleep Mode will not have the current GTK installed when it wakes up and will need to get new GTK as described in the WNM-Sleep Mode procedures in 11.2.1.16.

# Insert a new variable at the end of 8.5.6.2 as follows:

WNM-SleepMode - This variable is set to TRUE when the non-AP STA is in the WNM-Sleep Mode, as described in 11.2.1.16. Otherwise, it is set to FALSE.